3GPP TSG-RAN5 Meeting #109	R5-255716

# Dallas, Texas, USA, 17th - 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-5 | CR | 0239 | rev |  | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of DL-TDOA Assistance data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | For test cases 14.2.1 and 14.2.2 Sequence data values are currently configured for up to 13 “dummy cells” in Section 11.1.2. In RAN5#107 (R5-252908) it was concluded that up to 4 cells (Including the reference cells) are to be provided in the assistence data. The configuration of the “dummy cells” is currently incomplete and only for test cases 14.2.1 and 14.2.2, whereas several other test cases have “dummy cells” configured (Number of cells provided in assistance data > Number of cells in the test case) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Tables with configuration for up to 13 cells have been voided. New tables with detailed configuration, including NR PRS-Assistance Data perTRP, and sequence data have been added for Rel-16 test cases 14.2.1, 14.2.2, 15.2.1, 15.2.2, 16.2.1, 16.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | “dummy cell” configurations will remain incorrect for test cases 14.2.1, 14.2.2 and incomplete for test cases 15.2.1, 15.2.2, 16.2.1, 16.2.2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS 37.571-1 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

# 11	DL-TDOA information

## 11.1	DL-TDOA Assistance data for DL-TDOA measurement tests

### 11.1.1	General

This subclause defines the DL-TDOA assistance data that shall be used for the DL-TDOA measurement tests defined in TS 37.571-1 [6].

### 11.1.2	DL-TDOA Assistance Data

This subclause defines the DL-TDOA assistance data elements which shall be provided to the UE in the DL-TDOA measurement tests defined in TS 37.571-1 [6].

Table 11.1.2-1: Void

Table 11.1.2-2: Void

Table 11.1.2-3: Void

Table 11.1.2-4: NR PRS-Assistance Data perTRP configuration for test cases 14.2.1, 14.2.2, 15.2.1, 15.2.2, 16.2.1

| Derivation Path: TS 37.355 [8] clause 6.4.3 |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Comment |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  |  |
| dl-PRS-ID-r16 | 3 | Test Cases 14.2.1, 14.2.2: Dummy cell 2, Test Cases 15.2.1, 15.2.2, 16.2.1, 16.2.2 |
|  | 2 | Dummy cell 1, Test Cases 15.2.1, 15.2.2, 16.2.1, 16.2.2 |
| nr-PhysCellID-r16 | As specified in Table 11.1.2-7 |  |
| nr-CellGlobalID-r16 | As specified in Table 11.1.2-7 |  |
| nr-ARFCN-r16 | Same as 1st Cell present in current Frequency Layer |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |
| sfn-Offset-r16 | Same as 1st Cell present in current Frequency Layer |  |
| integerSubframeOffset-r16 | Same as 1st Cell present in current Frequency Layer |  |
| } |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 | Test Cases 14.2.1, 14.2.2 Dummy cell 1, Test Cases 15.2.1, 15.2.2, 16.2.1, 16.2.2 |
|  | 23 | Dummy cell 2, Test Cases 15.2.1, 15.2.2, 16.2.1, 16.2.2 |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 39 | Test Cases 14.2.1, 14.2.2, 15.2.1, 15.2.2, 16.2.1 |
|  | 154 | Test Case 16.2.2 |
| nr-DL-PRS-Info-r16 | As specified in Table 11.1.2-5 | Test Cases 14.2.1, 14.2.2, 15.2.1, 15.2.2 |
|  | As specified in Table 11.1.2-6 | Test Cases 16.2.1, 16.2.2 |
| } |  |  |

Table 11.1.2-5: NR-DL-PRS-Info for test cases 14.2.1, 14.2.2, 15.2.1, 15.2.2

| Derivation Path: TS 37.355 [8] clause 6.4.3 |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Comment |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS- ResourceSet-r16 { | Same as 1st Cell present in current Frequency Layer |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 CHOICE { |  |  |
| scs15-r16 CHOICE { |  |  |
| n160-r16 | Same as 1st Cell present in current Frequency Layer |  |
| } |  |  |
| scs30-r16 CHOICE { |  |  |
| n320-r16 | Same as 1st Cell present in current Frequency Layer |  |
| } |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-ResourceTimeGap-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-NumSymbols-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-MutingOption1-r16 SEQUENCE { |  |  |
| dl-prs-MutingBitRepetitionFactor-r16 | Same as 1st Cell present in current Frequency Layer |  |
| nr-option1-muting-r16 CHOICE { |  |  |
| po2-r16 | 01 | Test Cases 14.2.1, 14.2.2 Dummy cell 2, Test Cases 15.2.1, 15.2.2 |
|  | 10 | Dummy cell 1, Test Cases 15.2.1, 15.2.2 |
| } |  |  |
| } |  |  |
| dl-PRS-MutingOption2-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-ResourcePower-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRSResource-r16 { | Same as 1st Cell present in current Frequency Layer |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  |  |
| nr-DL-PRS-ResourceID-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-SequenceID-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |
| n2-r16 | 1 | In 1st Cell present in current Frequency Layer dl-PRS-CombSizeN-AndReOffset-r16 is n2-r16 |
| n4-r16 | 1 | In 1st Cell present in current Frequency Layer dl-PRS-CombSizeN-AndReOffset-r16 is n4-r16 |
| } |  |  |
| dl-PRS-ResourceSlotOffset-r16 | Same as 1st Cell present in current Frequency Layer | Test Cases 14.2.1, 14.2.2 15.2.1, 15.2.2 |
| dl-PRS-ResourceSymbolOffset-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-QCL-Info-r16 | Same as 1st Cell present in current Frequency Layer |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Table 11.1.2-6: NR-DL-PRS-Info for test cases 16.2.1, 16.2.2

| Derivation Path: TS 37.355 [8] clause 6.4.3 |  |  |
| --- | --- | --- |
| Information Element | Value/remark | Comment |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRSResourceSet-r16 { | Same as 1st Cell present in current Frequency Layer |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 CHOICE { |  |  |
| scs15-r16 CHOICE { |  |  |
| n160-r16 | Same as 1st Cell present in current Frequency Layer |  |
| } |  |  |
| scs30-r16 CHOICE { |  |  |
| n320-r16 | Same as 1st Cell present in current Frequency Layer |  |
| } |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-ResourceTimeGap-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-NumSymbols-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-MutingOption1-r16 SEQUENCE { |  |  |
| dl-prs-MutingBitRepetitionFactor-r16 | Same as 1st Cell present in current Frequency Layer |  |
| nr-option1-muting-r16 CHOICE { |  |  |
| po2-r16 | 01 | Dummy cell 1, Test Cases 16.2.1, 16.2.2 |
|  | 10 | Dummy cell 2, Test Cases 16.2.1, 16.2.2 |
| } |  |  |
| } |  |  |
| dl-PRS-MutingOption2-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-ResourcePower-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRSResource-r16 { | Same as 1st Cell present in current Frequency Layer |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  |  |
| nr-DL-PRS-ResourceID-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-SequenceID-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |
| n2-r16 | 0 | In 1st Cell present in current Frequency Layer dl-PRS-CombSizeN-AndReOffset-r16 is n2-r16 |
| n4-r16 | 0 | In 1st Cell present in current Frequency Layer dl-PRS-CombSizeN-AndReOffset-r16 is n4-r16 |
| } |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 0 | Dummy cell 2, Test Cases 16.2.1, 16.2.2 |
|  | 2 | Dummy cell 1, Test Cases 16.2.1, 16.2.2 |
| dl-PRS-ResourceSymbolOffset-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-QCL-Info-r16 | Same as 1st Cell present in current Frequency Layer |  |
| } |  |  |
| NR-DL-PRS-Resource-r16[2] SEQUENCE { |  |  |
| nr-DL-PRS-ResourceID-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-SequenceID-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |
| n2-r16 | 1 | In 1st Cell present in current Frequency Layer dl-PRS-CombSizeN-AndReOffset-r16 is n2-r16 |
| n4-r16 | 1 | In 1st Cell present in current Frequency Layer dl-PRS-CombSizeN-AndReOffset-r16 is n4-r16 |
| } |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 0 | Dummy cell 2, Test Cases 16.2.1, 16.2.2 |
|  | 2 | Dummy cell 1, Test Cases 16.2.1, 16.2.2 |
| dl-PRS-ResourceSymbolOffset-r16 | Same as 1st Cell present in current Frequency Layer |  |
| dl-PRS-QCL-Info-r16 | Same as 1st Cell present in current Frequency Layer |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |

Table 11.1.2-7: Sequence data values for test cases 14.2.1, 14.2.2, 15.2.1, 15.2.2, 16.2.1, 16.2.2

| Cell | Value physCellId | Value cellidentity |  | PFL per Test Case |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Value gNB ID | Value Cell Identity | Add to 1st Frequency Layer | Add to 2nd Frequency Layer |  |
| Dummy cell | 2 | '00 0000 0000 0000 0000 0000 0001'B | '00 0000 0010'B | 14.2.1, 15.2.1, 15.2.2, 16.2.1, 16.2.2: | 14.2.2 |  |
| Dummy cell | 3 | '00 0000 0000 0000 0000 0000 0010'B | '00 0000 0011'B | 15.2.1, 16.2.1 | 15.2.2, 16.2.2: |  |

## 11.2	ReferencePoint and RelativeLocation

This subclause defines the reference point and relative location information elements which shall be provided to the UE in the UE-based DL-TDOA measurement tests defined in TS 37.571-1 [6] and signalling tests defined in TS 37.571-2 [7].

Table 11.2-1: ReferencePoint

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReferencePoint-r16 ::= SEQUENCE { |  |  |  |
| referencePointGeographicLocation-r16 CHOICE { |  |  |  |
| ha-location3D-r16 SEQUENCE { |  |  |  |
| degreesLatitude-r15 | 852891909 | 35.744287 |  |
| degreesLongitude-r15 | 1666449411 | 139.680176 |  |
| altitude-r15 | 38400 | 300 m |  |
| uncertaintySemiMajor-r15 | 2 |  |  |
| uncertaintySemiMinor-r15 | 2 |  |  |
| orientationMajorAxis-r15 | 0 |  |  |
| horizontalConfidence-r15 | 68 |  |  |
| uncertaintyAltitude-r15 | 2 |  |  |
| verticalConfidence-r15 | 68 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.2-2: RelativeLocation for NR Cell 2

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RelativeLocation-r16 ::= SEQUENCE { |  |  |  |
| milli-arc-second-units-r16 | mas30 |  |  |
| height-units-r16 | m |  |  |
| delta-latitude-r16 SEQUENCE { |  |  |  |
| delta-Latitude-r16 | 250 |  |  |
| coarse-delta-Latitude-r16 | 0 |  |  |
| } |  |  |  |
| delta-longitude-r16 SEQUENCE { |  |  |  |
| delta-Longitude-r16 | 250 |  |  |
| coarse-delta-Longitude-r16 | 0 |  |  |
| } |  |  |  |
| delta-height-r16 SEQUENCE { |  |  |  |
| delta-Height-r16 | 0 |  |  |
| coarse-delta-Height-r16 | 0 |  |  |
| } |  |  |  |
| locationUNC-r16 SEQUENCE { |  |  |  |
| horizontalUncertainty-r16 | 2 |  |  |
| horizontalConfidence-r16 | 68 |  |  |
| verticalUncertainty-r16 | 2 |  |  |
| verticalConfidence-r16 | 68 |  |  |
| } |  |  |  |
| } |  |  |  |

Table 11.2-3: RelativeLocation for NR Cell 3

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RelativeLocation-r16 ::= SEQUENCE { |  |  |  |
| milli-arc-second-units-r16 | mas30 |  |  |
| height-units-r16 | m |  |  |
| delta-latitude-r16 SEQUENCE { |  |  |  |
| delta-Latitude-r16 | 250 |  |  |
| coarse-delta-Latitude-r16 | 0 |  |  |
| } |  |  |  |
| delta-longitude-r16 SEQUENCE { |  |  |  |
| delta-Longitude-r16 | 250 |  |  |
| coarse-delta-Longitude-r16 | 0 |  |  |
| } |  |  |  |
| delta-height-r16 SEQUENCE { |  |  |  |
| delta-Height-r16 | 0 |  |  |
| coarse-delta-Height-r16 | 0 |  |  |
| } |  |  |  |
| locationUNC-r16 SEQUENCE { |  |  |  |
| horizontalUncertainty-r16 | 2 |  |  |
| horizontalConfidence-r16 | 68 |  |  |
| verticalUncertainty-r16 | 2 |  |  |
| verticalConfidence-r16 | 68 |  |  |
| } |  |  |  |
| } |  |  |  |

<End of changes >