3GPP TSG-RAN5 Meeting #109	R5-255798

# Dallas, Texas, USA, 17th - 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.579-2 | CR | 0013 | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MCPTT regroup test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, MCProtoc16_enh2MCPTT_eMCData2-ConTest |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - "application/vnd.3gpp.mcvideo-regroup+xml" has been added to the list of XML MIME bodies with integrity protection in clause 6.6.3.1 of 24.379. - Editorial issues |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Editor's notes removed from specific message contents of regroup test cases. - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 6.5	Group regroup using a preconfigured group

### 6.5.1	On-network / Regroup using a preconfigured group / Requesting a group regroup using a preconfigured group / Removing a regroup using preconfigured group / Client Originated (CO)

6.5.1.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorised for MCPTT Service }

ensure that {

when { UE (MCPTT Client) receives a request to establish an MCPTT group regroup using a preconfigured group }

then { UE (MCPTT Client) sends a SIP MESSAGE message to establish an MCPTT group regroup using a preconfigured group }

}

(2)

with { UE (MCPTT Client) having established an MCPTT group regroup using a preconfigured group }

ensure that {

when { UE (MCPTT Client) receives a request to establish an MCPTT On-demand Pre-arranged Group Call using the group regroup }

then { UE (MCPTT Client) sends a SIP INVITE message using the group regroup to establish an MCPTT On-demand Pre-arranged Group Call }

}

(3)

with { UE (MCPTT Client) having established an MCPTT On-demand Pre-arranged Group Call }

ensure that {

when { the MCPTT User engages in communication with the invited MCPTT User(s) }

then { UE (MCPTT Client) respects the floor control imposed by the MCPTT Server }

}

(4)

with { UE (MCPTT Client) having an ongoing On-demand Pre-arranged Group Call }

ensure that {

when { MCPTT User requests to terminate the ongoing MCPTT Group Call }

then { UE (MCPTT Client) sends a SIP BYE message and leaves the MCPTT session }

}

(5)

with { UE (MCPTT Client) having established an MCPTT group regroup using a preconfigured group }

ensure that {

when { UE (MCPTT Client) receives a request to remove the group regroup }

then { UE (MCPTT Client) sends a SIP MESSAGE message to remove the group regroup }            }

6.5.1.2	Conformance Requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.379 clauses 16.2.1.1, 16.2.1.2. Unless otherwise stated these are Rel-16 requirements.

[TS 24.379, clause 16.2.1.1]

Upon receiving a request from an MCPTT user to establish an MCPTT group regroup using a preconfigured group, the MCPTT client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33] and:

1)	shall include an Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

2)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

3)	shall set the Request-URI to the public service identity identifying the originating participating MCPTT function serving the MCPTT user;

4)	may include a P-Preferred-Identity header field in the SIP MESSAGE request containing a public user identity as specified in 3GPP TS 24.229 [4];

5)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service-Id header field according to IETF RFC 6050 [9];

6)	shall contain an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with:

a)	the <mcptt-client-id> element set to the MCPTT client ID of the originating MCPTT client; and

b)	if the MCPTT client is aware of active functional aliases, and an active functional alias is to be included in the SIP MESSAGE request, the <functional-alias-URI> set to the URI of the used functional alias; and

7)	shall contain an application/vnd.3gpp.mcptt-regroup+xml MIME body with:

a)	the <regroup-action> element set to the value "create";

b)	the <mcptt-regroup-uri> element set to a unique temporary group identity URI;

NOTE:	How the unique temporary group identity URI is formed is an implementation decision.

c)	the <preconfigured-group> element set to the group identity of the preconfigured group; and

d)	the <groups-for-regroup> element set to the list of MCPTT group identities of groups that are to be included in the regroup; and

8)	shall send the SIP MESSAGE request according to 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP MESSAGE request, the MCPTT client:

1)	should notify the MCPTT user of the successful creation of the group regroup using preconfigured group.

On receiving a SIP 4xx response, a SIP 5xx response or a SIP 6xx response to the SIP INVITE request:

1)	should notify the MCPTT user of the failure to create the group regroup using preconfigured group.

[TS 24.379, clause 16.2.1.2]

Upon receiving a request from an MCPTT user to remove a user or group regroup using a preconfigured group, the MCPTT client:

1)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33]:

2)	shall include an Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

3)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

4)	shall set the Request-URI to the public service identity identifying the originating participating MCPTT function serving the MCPTT user;

5)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [4];

6)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service-Id header field according to IETF RFC 6050 [9];

7)	shall contain an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with:

a)	the <mcptt-client-id> element set to the MCPTT client ID of the originating MCPTT client; and

b)	if the MCPTT client is aware of active functional aliases, and an active functional alias is to be included in the SIP MESSAGE request, the <functional-alias-URI> set to the URI of the used functional alias; and

8)	shall contain an application/vnd.3gpp.mcptt-regroup+xml MIME body with:

a)	the <mcptt-regroup-uri> element set to the unique temporary group identity URI representing the regroup to be removed; and

b)	the <regroup-action> element set to "remove"; and

9)	shall send the SIP MESSAGE request according to 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP MESSAGE request, the MCPTT client:

1)	should notify the MCPTT user of the successful removal of the regroup using preconfigured group.

On receiving a SIP 4xx response, a SIP 5xx response or a SIP 6xx response to the SIP INVITE request:

1)	should notify the MCPTT user of the failure to remove the regroup using preconfigured group.

6.5.1.3	Test description

6.5.1.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 37.579-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 37.579-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCPTT.

6.5.1.3.2	Test procedure sequence

Table 6.5.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Procedure 'MCX CO communication' as described in TS 37.579-1 [2] clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 1 | Make the UE (MCPTT client) request creation of a new group (group B). (NOTE 1) | - | - | - | - |
| 2 | The UE (MCPTT client) correctly performs procedure 'MCX CO Group Creation' as described in TS 37.579-1 [2] Table 5.3.26.3-1 to create group B. | - | - | - | - |
| 3 | The procedure 'MCX communication release' as described in TS 37.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| 4 | Make the UE (MCPTT client) request to establish an MCPTT group regroup using a preconfigured group. (NOTE 1) | - | - |  |  |
| 5 | Check: Does the UE (MCPTT client) correctly perform procedure ‘MCX SIP MESSAGE CO' as described in TS 37.579-1 [2] Table 5.3.32.3-1 to establish an MCPTT group regroup using a preconfigured group? | - | - | 1 | P |
| 6 | Make the UE (MCPTT client) request the establishment of a group call with automatic commencement mode and implicit floor request using the group regroup. (NOTE 1) | - | - | - | - |
| 7 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO session establishment/modification without provisional responses other than 100 Trying' as described in TS 37.579-1 [2] Table 5.3A.1.3-1 to establish a group call with automatic commencement mode and implicit floor control according to option b.i of NOTE 1 in TS 37.579.1 [2] Table 5.3A.1.3-1 using the group regroup? | - | - | 2 | P |
| 8 | Check: Does the UE (MCPTT client) provide floor granted notification to the user? (NOTE 1) | - | - | 3 | P |
| 9 | Make the UE (MCPTT client) release the floor. (NOTE 1) | - | - | - | - |
| 10 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT Floor release – Floor Idle' as described in TS 37.579-1 [2] Table 5.3A.15.3-1? | - | - | 3 | P |
| 11 | Make the UE (MCPTT client) release the call. (NOTE 1) | - | - | - | - |
| 12 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCX CO call release' as described in TS 37.579-1 [2] Table 5.3.10.3-1 to release the call? | - | - | 4 | P |
| 13 | Make the UE (MCPTT client) request to remove the group regroup. (NOTE 1) | - | - | - | - |
| 14 | Check: Does the UE (MCPTT client) correctly perform procedure ‘MCX SIP MESSAGE CO' as described in TS 37.579-1 [2] Table 5.3.32.3-1 to remove the group regroup? | - | - | 5 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.5.1.3.3	Specific message contents

Table 6.5.1.3.3-1: HTTP PUT (step 2, Table 6.5.1.3.2-1;
step 1, TS 37.579-1 [2], Table 5.3.26.3-1)

Derivation Path: TS 37.579-1 [2], Table 5.5.4.4-1, condition GROUPCREATE

Table 6.5.1.3.3-2: SIP MESSAGE from the UE (Step 5, Table 6.5.1.3.2-1;
Step 2, TS 37.579-1 [2], Table 5.3.32.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  |  |  |
| ac-value[2] |  |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  | TS 24.379 [9] clause 16.2.1.1 |  |
| req-param | "require" |  | TS 24.379 [9] clause 16.2.1.1 |  |
| explicit-param | "explicit" |  | TS 24.379 [9] clause 16.2.1.1 |  |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  | TS 24.379 [9] clause 16.2.1.1 |  |
| P-Preferred-Service | not present |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.5.1.3.3-3 |  | TS 24.379 [9] clause 16.2.1.1 |  |
| MIME body part |  | MCPTT-Regroup |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-regroup+xml" |  |  |  |
| Content-ID | any value | Unique URL identifying the MCPTT-Regroup XML MIME body; used as reference in the signature MIME body |  |  |
| MIME-part-body | MCPTT-Regroup as described in Table 6.5.1.3.3-4 |  | TS 24.379 [9] clause 16.2.1.1 |  |

Table 6.5.1.3.3-3: MCPTT-Info in SIP MESSAGE (Table 6.5.1.3.3-2)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-calling-user-id | not present |  |  |  |
| mcptt-client-id | Encrypted <mcptt-client-id> with mcpttString set to the client-id of the UE | Encryption according to NOTE 1 and NOTE 2 in TS 37.579-1 [2] Table 5.5.3.2.1-1 | TS 24.379 [9] clause 16.2.1.1 |  |

Table 6.5.1.3.3-4: MCPTT-Regroup in SIP MESSAGE (Table 6.5.1.3.3-2)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.16.2-1, condition GROUP_REGROUP

Table 6.5.1.3.3-5: SIP INVITE from the UE (step 7, Table 6.5.1.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3A.1.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.5.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | SDP Message as described in Table 6.5.1.3.3-6 |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.5.1.3.3-7 |  |  |  |

Table 6.5.1.3.3-6: SDP in SIP INVITE (Table 6.5.1.3.3-5)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.1.1-1, condition INITIAL_SDP_OFFER, IMPLICIT_GRANT_REQUESTED

Table 6.5.1.3.3-7: MCPTT-Info in SIP INVITE (Table 6.5.1.3.3-5)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.1-1, condition GROUP-CALL, INVITE_REFER |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | Encrypted <mcptt-request-uri> with mcpttURI set to px_MCPTT_Group_T_ID | Encryption according to NOTE 1 and NOTE 2 in TS 37.579-1 [2] Table 5.5.3.2.1-1 |  |  |

Table 6.5.1.3.3-8: SIP 200 (OK) from the SS (step 7, Table 6.5.1.3.2-1; 
step 4, TS 37.579-1 [2] Table 5.3A.1.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.17.1.2-1, condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP Message | As described in Table 6.5.1.3.3-9 |  |  |  |

Table 6.5.1.3.3-9: SDP in SIP 200 (OK) (Table 6.5.1.3.3-8)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.1.2-1, condition SDP_ANSWER, IMPLICIT_GRANT_REQUESTED, IMPLICIT_FLOOR_GRANTED

Table 6.5.1.3.3-10: SIP MESSAGE from the UE (Step 14, Table 6.5.1.3.2-1;
Step 2, TS 37.579-1 [2], Table 5.3.32.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  |  |  |
| ac-value[2] |  |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  | TS 24.379 [9] clause 16.2.1.2 |  |
| req-param | "require" |  | TS 24.379 [9] clause 16.2.1.2 |  |
| explicit-param | "explicit" |  | TS 24.379 [9] clause 16.2.1.2 |  |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  | TS 24.379 [9] clause 16.2.1.2 |  |
| P-Preferred-Service | not present |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.5.1.3.3-3 |  | TS 24.379 [9] clause 16.2.1.2 |  |
| MIME body part |  | MCPTT-Regroup |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-regroup+xml" |  |  |  |
| Content-ID | any value | Unique URL identifying the MCPTT-Regroup XML MIME body; used as reference in the signature MIME body |  |  |
| MIME-part-body | MCPTT-Regroup as described in Table 6.5.1.3.3-11 |  | TS 24.379 [9] clause 16.2.1.2 |  |

Table 6.5.1.3.3-11: MCPTT-Regroup in SIP MESSAGE (Table 6.5.1.3.3-10)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.16.2-1, condition REMOVE

### 6.5.2	On-network / Regroup using a preconfigured group / Receiving a notification of creation of a regroup using preconfigured group / Receiving notification of removal of a regroup using preconfigured group / Client Terminated (CT)

6.5.2.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorised for MCPTT Service }

ensure that {

when { UE (MCPTT Client) receives a request via a SIP MESSAGE message to create an MCPTT group regroup using a preconfigured group }

then { UE (MCPTT Client) responds with a SIP 200 (OK) message }

}

(2)

with { UE (MCPTT Client) having established an MCPTT group regroup using a preconfigured group }

ensure that {

when { UE (MCPTT Client) receives a request to establish an MCPTT On-demand Pre-arranged Group Call using the group regroup }

then { UE (MCPTT Client) sends a SIP INVITE message using the group regroup to establish an MCPTT On-demand Pre-arranged Group Call }

}

(3)

with { UE (MCPTT Client) having established an MCPTT On-demand Pre-arranged Group Call }

ensure that {

when { the MCPTT User engages in communication with the invited MCPTT User(s) }

then { UE (MCPTT Client) respects the floor control imposed by the MCPTT Server }

}

(4)

with { UE (MCPTT Client) having an ongoing On-demand Pre-arranged Group Call }

ensure that {

when { MCPTT User requests to terminate the ongoing MCPTT Group Call }

then { UE (MCPTT Client) sends a SIP BYE message and leaves the MCPTT session }

}

(5)

with { UE (MCPTT Client) having an affiliation with an established MCPTT group regroup }

ensure that {

when { UE (MCPTT Client) receives a request to remove the group regroup via a SIP MESSAGE message }

then { UE (MCPTT Client) responds with a SIP 200 (OK) message }            }

6.5.2.2	Conformance Requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.379 clauses 16.2.1.3, 16.2.1.4. Unless otherwise stated these are Rel-16 requirements.

[TS 24.379, clause 16.2.1.3]

Upon receiving a "SIP MESSAGE request to the MCPTT client to request creation of a regroup using preconfigured group", the MCPTT client:

1)	should notify the MCPTT user of the creation of the regroup using preconfigured group;

2)	shall send a 200 (OK) response to the MCPTT server according to 3GPP TS 24.229 [4];

3)	in the application/vnd.3gpp.mcptt-regroup+xml MIME body is contained in the incoming SIP MESSAGE request:

a)	if a <users-for-regroup> element is included in that MIME body, shall store the value of the <mcptt-regroup-uri> element as the temporary group identity and associate that with the group identity received in the <mcptt-regroup-uri> element, along with the information that the created regroup is a user regroup and should store the contents of the <users-for-regroup> element as the list of users that are part of that user regroup: or

b)	if a <groups-for-regroup> element is included in that MIME body, shall store the value of the <mcptt-regroup-uri> element as the temporary group identity and associate that with the group identity received in the <mcptt-regroup-uri> element, along with the information that the created regroup is a group regroup and should store the contents of the <groups-for-regroup> element as the list of groups that are part of that group regroup:

4)	shall consider that the MCPTT Client is affiliated with the regroup;

5)	should not initiate calls targeting any of the constituent groups but instead target the regroup for the duration of a group regroup; and

6)	if the regroup is a chat group, the MCPTT client should join the regroup when this notification of creation is received.

[TS 24.379, clause 16.2.1.4]

Upon receiving a "SIP MESSAGE request to the MCPTT client to request removal of a regroup using preconfigured group", the MCPTT client:

1)	should notify the MCPTT user of the removal of the regroup using preconfigured group;

2)	shall send a 200 (OK) response to the MCPTT server according to 3GPP TS 24.229 [4]; and

3)	shall consider that the MCPTT client is de-affiliated from the regroup.

6.5.2.3	Test description

6.5.2.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 37.579-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 37.579-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCPTT.

6.5.2.3.2	Test procedure sequence

Table 6.5.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Procedure 'MCX CO communication' as described in TS 37.579-1 [2] clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 1 | Make the UE (MCPTT client) request creation of a new group (group B). (NOTE 1) | - | - | - | - |
| 2 | The UE (MCPTT client) correctly performs procedure 'MCX CO Group Creation' as described in TS 37.579-1 [2] Table 5.3.26.3-1 to create group B. | - | - | - | - |
| 3 | The procedure 'MCX communication release' as described in TS 37.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| 4 | Check: Does the UE (MCPTT client) correctly perform procedure ‘MCX SIP MESSAGE CT' as described in TS 37.579-1 [2] Table 5.3.33.3-1 to establish an MCPTT group regroup using a preconfigured group? | - | - | 1 | P |
| 5 | Make the UE (MCPTT client) request the establishment of a group call with automatic commencement mode and implicit floor request using the group regroup. (NOTE 1) | - | - | - | - |
| 6 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO session establishment/modification without provisional responses other than 100 Trying' as described in TS 37.579-1 [2] Table 5.3A.1.3-1 to establish a group call with automatic commencement mode and implicit floor control according to option b.i of NOTE 1 in TS 37.579.1 [2] Table 5.3A.1.3-1 using the group regroup? | - | - | 2 | P |
| 7 | Check: Does the UE (MCPTT client) provide floor granted notification to the user? (NOTE 1) | - | - | 3 | P |
| 8 | Make the UE (MCPTT client) release the floor. (NOTE 1) | - | - | - | - |
| 9 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT Floor release – Floor Idle' as described in TS 37.579-1 [2] Table 5.3A.15.3-1? | - | - | 3 | P |
| 10 | Make the UE (MCPTT client) release the call. (NOTE 1) | - | - | - | - |
| 11 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCX CO call release' as described in TS 37.579-1 [2] Table 5.3.10.3-1 to release the call? | - | - | 4 | P |
| 12 | Check: Does the UE (MCPTT client) correctly perform procedure ‘MCX SIP MESSAGE CT' as described in TS 37.579-1 [2] Table 5.3.33.3-1 to remove the group regroup? | - | - | 5 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.5.2.3.3	Specific message contents

Table 6.5.2.3.3-1: HTTP PUT (step 2, Table 6.5.2.3.2-1;
step 1, TS 37.579-1 [2], Table 5.3.26.3-1)

Derivation Path: TS 37.579-1 [2], Table 5.5.4.4-1, condition GROUPCREATE

Table 6.5.2.3.3-2: SIP MESSAGE from the SS (Step 4, Table 6.5.2.3.2-1;
Step 2, TS 37.579-1 [2], Table 5.3.33.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.2-1, condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| P-Asserted-Identity |  |  |  |  |
| name-addr | px_MCX_SIP_PublicUserId_B |  | TS 24.379 [9] clause 16.2.2.4 |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.5.2.3.3-3 |  | TS 24.379 [9] clause 16.2.2.4 |  |
| MIME body part |  | MCPTT-Regroup |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-regroup+xml" |  |  |  |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the MCPTT-Regroup XML MIME body; used as reference in the signature MIME body |  |  |
| MIME-part-body | MCPTT-Regroup as described in Table 6.5.2.3.3-4 |  | TS 24.379 [9] clause 16.2.2.4 |  |

Table 6.5.2.3.3-3: MCPTT-Info in SIP MESSAGE (Table 6.5.2.3.3-2)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | not present |  |  |  |
| mcptt-calling-user-id | not present |  |  |  |
| mcptt-client-id | Encrypted <mcptt-client-id> with mcpttString set to px_MCX_Client_B_ID | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.2.2-1 | TS 24.379 [9] clause 16.2.2.4 |  |

Table 6.5.2.3.3-4: MCPTT-Regroup in SIP MESSAGE (Table 6.5.2.3.3-2)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.16.3-1, condition GROUP_REGROUP

Table 6.5.2.3.3-5: SIP INVITE from the UE (step 6, Table 6.5.2.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3A.1.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.5.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | SDP Message as described in Table 6.5.2.3.3-6 |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.5.2.3.3-7 |  |  |  |

Table 6.5.2.3.3-6: SDP in SIP INVITE (Table 6.5.2.3.3-5)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.1.1-1, condition INITIAL_SDP_OFFER, IMPLICIT_GRANT_REQUESTED

Table 6.5.2.3.3-7: MCPTT-Info in SIP INVITE (Table 6.5.2.3.3-5)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.1-1, condition GROUP-CALL, INVITE_REFER |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | Encrypted <mcptt-request-uri> with mcpttURI set to px_MCPTT_Group_T_ID | Encryption according to NOTE 1 and NOTE 2 in TS 37.579-1 [2] Table 5.5.3.2.1-1 |  |  |

Table 6.5.2.3.3-8: SIP 200 (OK) from the SS (step 6, Table 6.5.2.3.2-1; 
step 4, TS 37.579-1 [2] Table 5.3A.1.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.17.1.2-1, condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP Message | As described in Table 6.5.2.3.3-9 |  |  |  |

Table 6.5.2.3.3-9: SDP in SIP 200 (OK) (Table 6.5.2.3.3-8)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.1.2-1, condition SDP_ANSWER, IMPLICIT_GRANT_REQUESTED, IMPLICIT_FLOOR_GRANTED

Table 6.5.2.3.3-10: SIP MESSAGE from the SS (Step 12, Table 6.5.2.3.2-1;
Step 2, TS 37.579-1 [2], Table 5.3.33.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.2-1, condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| P-Asserted-Identity |  |  |  |  |
| name-addr | px_MCX_SIP_PublicUserId_B |  | TS 24.379 [9] clause 16.2.2.5 |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.5.2.3.3-3 |  | TS 24.379 [9] clause 16.2.2.5 |  |
| MIME body part |  | MCPTT-Regroup |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-regroup+xml" |  |  |  |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the MCPTT-Regroup XML MIME body; used as reference in the signature MIME body |  |  |
| MIME-part-body | MCPTT-Regroup as described in Table 6.5.2.3.3-11 |  | TS 24.379 [9] clause 16.2.2.5 |  |

Table 6.5.2.3.3-11: MCPTT-Regroup in SIP MESSAGE (Table 6.5.2.3.3-10)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.16.3-1, condition REMOVE

### 6.5.3	On-network / Regroup using a preconfigured group / Requesting a user regroup using a preconfigured group / Removing a regroup using preconfigured group / Client Originated (CO)

6.5.3.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorised for MCPTT Service }

ensure that {

when { UE (MCPTT Client) receives a request to establish an MCPTT user regroup using a preconfigured group }

then { UE (MCPTT Client) sends a SIP MESSAGE message to establish an MCPTT user regroup using a preconfigured group }

}

(2)

with { UE (MCPTT Client) having established an MCPTT user regroup using a preconfigured group }

ensure that {

when { UE (MCPTT Client) receives a request to establish an MCPTT On-demand Pre-arranged Group Call using the user regroup }

then { UE (MCPTT Client) sends a SIP INVITE message using the user regroup to establish an MCPTT On-demand Pre-arranged Group Call }

}

(3)

with { UE (MCPTT Client) having established an MCPTT On-demand Pre-arranged Group Call }

ensure that {

when { the MCPTT User engages in communication with the invited MCPTT User(s) }

then { UE (MCPTT Client) respects the floor control imposed by the MCPTT Server }

}

(4)

with { UE (MCPTT Client) having an ongoing On-demand Pre-arranged Group Call }

ensure that {

when { MCPTT User requests to terminate the ongoing MCPTT Group Call }

then { UE (MCPTT Client) sends a SIP BYE message and leaves the MCPTT session }

}

(5)

with { UE (MCPTT Client) having established an MCPTT user regroup using a preconfigured group {

when { UE (MCPTT Client) receives a request to remove the user regroup }

then { UE (MCPTT Client) sends a SIP MESSAGE message to remove a user regroup }            }

6.5.3.2	Conformance Requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.379 clauses 16.3.1.1, 16.3.1.2, 16.2.1.2. Unless otherwise stated these are Rel-16 requirements.

[TS 24.379, clause 16.3.1.1]

Upon receiving a request from an MCPTT user to establish an MCPTT user regroup using a preconfigured group, the MCPTT client shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33] and:

1)	shall include an Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

2)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

3)	shall set the Request-URI to the public service identity identifying the originating participating MCPTT function serving the MCPTT user;

4)	may include a P-Preferred-Identity header field in the SIP MESSAGE request containing a public user identity as specified in 3GPP TS 24.229 [4];

5)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service-Id header field according to IETF RFC 6050 [9];

6)	shall contain an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with:

a)	the <mcptt-client-id> element set to the MCPTT client ID of the originating MCPTT client; and

b)	if the MCPTT client is aware of active functional aliases, and an active functional alias is to be included in the SIP MESSAGE request, the <functional-alias-URI> set to the URI of the used functional alias;

7)	shall contain an application/vnd.3gpp.mcptt-regroup+xml MIME body with:

a)	the <mcptt-regroup-uri> element set to a unique temporary group identity URI;

NOTE:	How the unique temporary group identity URI is formed is an implementation decision.

b)	the <preconfigured-group> element set to the group identity of the preconfigured group;

c)	the <regroup-action> element set to "create"; and

d)	the <users-for-regroup> element set to the list of MCPTT IDs of users that are to be included in the regroup; and

8)	shall send the SIP MESSAGE request according to 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP MESSAGE request, the MCPTT client:

1)	should notify the MCPTT user of the successful creation of the user regroup using preconfigured group.

On receiving a SIP 4xx response, a SIP 5xx response or a SIP 6xx response to the SIP INVITE request:

1)	should notify the MCPTT user of the failure to create the user regroup using preconfigured group.

[TS 24.379, clause 16.3.1.2]

When the user requests the MCPTT client to remove a user regroup, the MCPTT client uses the procedure in clause 16.2.1.2.

[TS 24.379, clause 16.2.1.2]

Upon receiving a request from an MCPTT user to remove a user or group regroup using a preconfigured group, the MCPTT client:

1)	shall generate a SIP MESSAGE request in accordance with 3GPP TS 24.229 [4] and IETF RFC 3428 [33]:

2)	shall include an Accept-Contact header field containing the g.3gpp.mcptt media feature tag along with the "require" and "explicit" header field parameters according to IETF RFC 3841 [6];

3)	shall include an Accept-Contact header field with the media feature tag g.3gpp.icsi-ref with the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" along with parameters "require" and "explicit" according to IETF RFC 3841 [6];

4)	shall set the Request-URI to the public service identity identifying the originating participating MCPTT function serving the MCPTT user;

5)	may include a P-Preferred-Identity header field in the SIP INVITE request containing a public user identity as specified in 3GPP TS 24.229 [4];

6)	shall include the ICSI value "urn:urn-7:3gpp-service.ims.icsi.mcptt" (coded as specified in 3GPP TS 24.229 [4]), in a P-Asserted-Service-Id header field according to IETF RFC 6050 [9];

7)	shall contain an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with:

a)	the <mcptt-client-id> element set to the MCPTT client ID of the originating MCPTT client; and

b)	if the MCPTT client is aware of active functional aliases, and an active functional alias is to be included in the SIP MESSAGE request, the <functional-alias-URI> set to the URI of the used functional alias; and

8)	shall contain an application/vnd.3gpp.mcptt-regroup+xml MIME body with:

a)	the <mcptt-regroup-uri> element set to the unique temporary group identity URI representing the regroup to be removed; and

b)	the <regroup-action> element set to "remove"; and

9)	shall send the SIP MESSAGE request according to 3GPP TS 24.229 [4].

On receiving a SIP 2xx response to the SIP MESSAGE request, the MCPTT client:

1)	should notify the MCPTT user of the successful removal of the regroup using preconfigured group.

On receiving a SIP 4xx response, a SIP 5xx response or a SIP 6xx response to the SIP INVITE request:

1)	should notify the MCPTT user of the failure to remove the regroup using preconfigured group.

6.5.3.3	Test description

6.5.3.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 37.579-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 37.579-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCPTT.

6.5.3.3.2	Test procedure sequence

Table 6.5.3.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Procedure 'MCX CO communication' as described in TS 37.579-1 [2] clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 1 | Make the UE (MCPTT client) request creation of a new group (group B). (NOTE 1) | - | - | - | - |
| 2 | The UE (MCPTT client) correctly performs procedure 'MCX CO Group Creation' as described in TS 37.579-1 [2] Table 5.3.26.3-1 to create group B. | - | - | - | - |
| 3 | The procedure 'MCX communication release' as described in TS 37.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| 4 | Make the UE (MCPTT client) request to establish an MCPTT user regroup using a preconfigured group. (NOTE 1) | - | - |  |  |
| 5 | Check: Does the UE (MCPTT client) correctly perform procedure ‘MCX SIP MESSAGE CO' as described in TS 37.579-1 [2] Table 5.3.32.3-1 to establish an MCPTT user regroup using a preconfigured group? | - | - | 1 | P |
| 6 | Make the UE (MCPTT client) request the establishment of a group call with automatic commencement mode and implicit floor request using the user regroup. (NOTE 1) | - | - | - | - |
| 7 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO session establishment/modification without provisional responses other than 100 Trying' as described in TS 37.579-1 [2] Table 5.3A.1.3-1 to establish a group call with automatic commencement mode and implicit floor control according to option b.i of NOTE 1 in TS 37.579.1 [2] Table 5.3A.1.3-1 using the user regroup? | - | - | 2 | P |
| 8 | Check: Does the UE (MCPTT client) provide floor granted notification to the user? (NOTE 1) | - | - | 3 | P |
| 9 | Make the UE (MCPTT client) release the floor. (NOTE 1) | - | - | - | - |
| 10 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT Floor release – Floor Idle' as described in TS 37.579-1 [2] Table 5.3A.15.3-1? | - | - | 3 | P |
| 11 | Make the UE (MCPTT client) release the call. (NOTE 1) | - | - | - | - |
| 12 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCX CO call release' as described in TS 37.579-1 [2] Table 5.3.10.3-1 to release the call? | - | - | 4 | P |
| 13 | Make the UE (MCPTT client) request to remove the user regroup. (NOTE 1) | - | - | - | - |
| 14 | Check: Does the UE (MCPTT client) correctly perform procedure ‘MCX SIP MESSAGE CO' as described in TS 37.579-1 [2] Table 5.3.32.3-1 to remove user regroup? | - | - | 5 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.5.3.3.3	Specific message contents

Table 6.5.3.3.3-1: HTTP PUT (step 2, Table 6.5.3.3.2-1;
step 1, TS 37.579-1 [2], Table 5.3.26.3-1)

Derivation Path: TS 37.579-1 [2], Table 5.5.4.4-1, condition GROUPCREATE

Table 6.5.3.3.3-2: SIP MESSAGE from the UE (Step 5, Table 6.5.3.3.2-1;
Step 2, TS 37.579-1 [2], Table 5.3.32.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  |  |  |
| ac-value[2] |  |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  | TS 24.379 [9] clause 16.3.1.1 |  |
| req-param | "require" |  | TS 24.379 [9] clause 16.3.1.1 |  |
| explicit-param | "explicit" |  | TS 24.379 [9] clause 16.3.1.1 |  |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  | TS 24.379 [9] clause 16.3.1.1 |  |
| P-Preferred-Service | not present |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.5.3.3.3-3 |  | TS 24.379 [9] clause 16.3.1.1 |  |
| MIME body part |  | MCPTT-Regroup |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-regroup+xml" |  |  |  |
| Content-ID | any value | Unique URL identifying the MCPTT-Regroup XML MIME body; used as reference in the signature MIME body |  |  |
| MIME-part-body | MCPTT-Regroup as described in Table 6.5.3.3.3-4 |  | TS 24.379 [9] clause 16.3.1.1 |  |

Table 6.5.3.3.3-3: MCPTT-Info in SIP MESSAGE (Table 6.5.3.3.3-2)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-calling-user-id | not present |  |  |  |
| mcptt-client-id | Encrypted <mcptt-client-id> with mcpttString set to the client-id of the UE | Encryption according to NOTE 1 and NOTE 2 in TS 37.579-1 [2] Table 5.5.3.2.1-1 | TS 24.379 [9] clause 16.3.1.1 |  |

Table 6.5.3.3.3-4: MCPTT-Regroup in SIP MESSAGE (Table 6.5.3.3.3-2)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.16.2-1, condition USER_REGROUP

Table 6.5.3.3.3-5: SIP INVITE from the UE (step 7, Table 6.5.3.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3A.1.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.5.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | SDP Message as described in Table 6.5.3.3.3-6 |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.5.3.3.3-7 |  |  |  |

Table 6.5.3.3.3-6: SDP in SIP INVITE (Table 6.5.3.3.3-5)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.1.1-1, condition INITIAL_SDP_OFFER, IMPLICIT_GRANT_REQUESTED

Table 6.5.3.3.3-7: MCPTT-Info in SIP INVITE (Table 6.5.3.3.3-5)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.1-1, condition GROUP-CALL, INVITE_REFER |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | Encrypted <mcptt-request-uri> with mcpttURI set to px_MCPTT_Group_T_ID | Encryption according to NOTE 1 and NOTE 2 in TS 37.579-1 [2] Table 5.5.3.2.1-1 |  |  |

Table 6.5.3.3.3-8: SIP 200 (OK) from the SS (step 7, Table 6.5.3.3.2-1; 
step 4, TS 37.579-1 [2] Table 5.3A.1.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.17.1.2-1, condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP Message | As described in Table 6.5.3.3.3-9 |  |  |  |

Table 6.5.3.3.3-9: SDP in SIP 200 (OK) (Table 6.5.3.3.3-8)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.1.2-1, condition SDP_ANSWER, IMPLICIT_GRANT_REQUESTED, IMPLICIT_FLOOR_GRANTED

Table 6.5.3.3.3-10: SIP MESSAGE from the UE (Step 14, Table 6.5.3.3.2-1;
Step 2, TS 37.579-1 [2], Table 5.3.32.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Accept-Contact |  |  |  |  |
| ac-value[2] |  |  |  |  |
| feature-param | "+g.3gpp.mcptt" |  | TS 24.379 [9] clause 16.3.1.2 |  |
| req-param | "require" |  | TS 24.379 [9] clause 16.3.1.2 |  |
| explicit-param | "explicit" |  | TS 24.379 [9] clause 16.3.1.2 |  |
| P-Asserted-Service |  |  |  |  |
| Service-ID | "urn:urn-7:3gpp-service.ims.icsi.mcptt" |  | TS 24.379 [9] clause 16.3.1.2 |  |
| P-Preferred-Service | not present |  |  |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.5.3.3.3-3 |  | TS 24.379 [9] clause 16.3.1.2 |  |
| MIME body part |  | MCPTT-Regroup |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-regroup+xml" |  |  |  |
| Content-ID | any value | Unique URL identifying the MCPTT-Regroup XML MIME body; used as reference in the signature MIME body |  |  |
| MIME-part-body | MCPTT-Regroup as described in Table 6.5.3.3.3-11 |  | TS 24.379 [9] clause 16.3.1.2 |  |

Table 6.5.3.3.3-11: MCPTT-Regroup in SIP MESSAGE (Table 6.5.3.3.3-10)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.16.2-1, condition REMOVE

### 6.5.4	On-network / Regroup using a preconfigured group / Creating a user regroup using preconfigured group / Removing a user regroup using preconfigured group / Client Terminated (CT)

6.5.4.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorised for MCPTT Service }

ensure that {

when { UE (MCPTT Client) receives a request via a SIP MESSAGE message to create an MCPTT user regroup using a preconfigured group }

then { UE (MCPTT Client) responds with a SIP 200 (OK) message }

}

(2)

with { UE (MCPTT Client) having established an MCPTT user regroup using a preconfigured group }

ensure that {

when { UE (MCPTT Client) receives a request to establish an MCPTT On-demand Pre-arranged Group Call using the user regroup }

then { UE (MCPTT Client) sends a SIP INVITE message using the user regroup to establish an MCPTT On-demand Pre-arranged Group Call }

}

(3)

with { UE (MCPTT Client) having established an MCPTT On-demand Pre-arranged Group Call }

ensure that {

when { the MCPTT User engages in communication with the invited MCPTT User(s) }

then { UE (MCPTT Client) respects the floor control imposed by the MCPTT Server }

}

(4)

with { UE (MCPTT Client) having an ongoing On-demand Pre-arranged Group Call }

ensure that {

when { MCPTT User requests to terminate the ongoing MCPTT Group Call }

then { UE (MCPTT Client) sends a SIP BYE message and leaves the MCPTT session }

}

(5)

with { UE (MCPTT Client) having an affiliation with an established MCPTT user regroup }

ensure that {

when { UE (MCPTT Client) receives a request to remove the user regroup via a SIP MESSAGE message }

then { UE (MCPTT Client) responds with a SIP 200 (OK) message }            }

6.5.4.2	Conformance Requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.379 clauses 16.3.1.3, 16.2.1.3, 16.3.1.4, 16.2.1.4. Unless otherwise stated these are Rel-16 requirements.

[TS 24.379, clause 16.3.1.3]

The procedure in clause 16.2.1.3 is used by the MCPTT client when the MCPTT server notifies the MCPTT client of the creation of a user regroup using preconfigured group.

[TS 24.379, clause 16.2.1.3]

Upon receiving a "SIP MESSAGE request to the MCPTT client to request creation of a regroup using preconfigured group", the MCPTT client:

1)	should notify the MCPTT user of the creation of the regroup using preconfigured group;

2)	shall send a 200 (OK) response to the MCPTT server according to 3GPP TS 24.229 [4];

3)	in the application/vnd.3gpp.mcptt-regroup+xml MIME body is contained in the incoming SIP MESSAGE request:

a)	if a <users-for-regroup> element is included in that MIME body, shall store the value of the <mcptt-regroup-uri> element as the temporary group identity and associate that with the group identity received in the <mcptt-regroup-uri> element, along with the information that the created regroup is a user regroup and should store the contents of the <users-for-regroup> element as the list of users that are part of that user regroup: or

b)	if a <groups-for-regroup> element is included in that MIME body, shall store the value of the <mcptt-regroup-uri> element as the temporary group identity and associate that with the group identity received in the <mcptt-regroup-uri> element, along with the information that the created regroup is a group regroup and should store the contents of the <groups-for-regroup> element as the list of groups that are part of that group regroup:

4)	shall consider that the MCPTT Client is affiliated with the regroup;

5)	should not initiate calls targeting any of the constituent groups but instead target the regroup for the duration of a group regroup; and

6)	if the regroup is a chat group, the MCPTT client should join the regroup when this notification of creation is received.

[TS 24.379, clause 16.3.1.4]

The procedure in clause 16.2.1.4 is used by the MCPTT client when the MCPTT server notifies the MCPTT client of the removal of a user regroup using preconfigured group.

[TS 24.379, clause 16.2.1.4]

Upon receiving a "SIP MESSAGE request to the MCPTT client to request removal of a regroup using preconfigured group", the MCPTT client:

1)	should notify the MCPTT user of the removal of the regroup using preconfigured group;

2)	shall send a 200 (OK) response to the MCPTT server according to 3GPP TS 24.229 [4]; and

3)	shall consider that the MCPTT client is de-affiliated from the regroup.

6.5.4.3	Test description

6.5.4.3.1	Pre-test conditions

Test configuration:

-	Single cell configuration according to TS 37.579-1 [2] clause 5.2.2.2.1.

Preamble:

-	The UE has performed procedure 'Initial registration' as described in TS 37.579-1 [2] clause 5.4.2.

-	UE States at the end of the preamble:

-	The UE is in RRC_IDLE state.

-	The client is registered for MCPTT.

6.5.4.3.2	Test procedure sequence

Table 6.5.4.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| - | EXCEPTION: Procedure 'MCX CO communication' as described in TS 37.579-1 [2] clause 5.4.3 is started to establish an RRC connection. | - | - | - | - |
| 1 | Make the UE (MCPTT client) request creation of a new group (group B). (NOTE 1) | - | - | - | - |
| 2 | The UE (MCPTT client) correctly performs procedure 'MCX CO Group Creation' as described in TS 37.579-1 [2] Table 5.3.26.3-1 to create group B. | - | - | - | - |
| 3 | The procedure 'MCX communication release' as described in TS 37.579-1 [2] clause 5.4.14 is performed to release the RRC connection. | - | - | - | - |
| 4 | Check: Does the UE (MCPTT client) correctly perform procedure ‘MCX SIP MESSAGE CT' as described in TS 37.579-1 [2] Table 5.3.33.3-1 to establish an MCPTT user regroup using a preconfigured group? | - | - | 1 | P |
| 5 | Make the UE (MCPTT client) request the establishment of a group call with automatic commencement mode and implicit floor request using the user regroup. (NOTE 1) | - | - | - | - |
| 6 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT CO session establishment/modification without provisional responses other than 100 Trying' as described in TS 37.579-1 [2] Table 5.3A.1.3-1 to establish a group call with automatic commencement mode and implicit floor control according to option b.i of NOTE 1 in TS 37.579.1 [2] Table 5.3A.1.3-1 using the user regroup? | - | - | 2 | P |
| 7 | Check: Does the UE (MCPTT client) provide floor granted notification to the user? (NOTE 1) | - | - | 3 | P |
| 8 | Make the UE (MCPTT client) release the floor. (NOTE 1) | - | - | - | - |
| 9 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCPTT Floor release – Floor Idle' as described in TS 37.579-1 [2] Table 5.3A.15.3-1? | - | - | 3 | P |
| 10 | Make the UE (MCPTT client) release the call. (NOTE 1) | - | - | - | - |
| 11 | Check: Does the UE (MCPTT client) correctly perform procedure 'MCX CO call release' as described in TS 37.579-1 [2] Table 5.3.10.3-1 to release the call? | - | - | 4 | P |
| 12 | Check: Does the UE (MCPTT client) correctly perform procedure ‘MCX SIP MESSAGE CT' as described in TS 37.579-1 [2] Table 5.3.33.3-1 to remove the user regroup? | - | - | 5 | P |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

6.5.4.3.3	Specific message contents

Table 6.5.4.3.3-1: HTTP PUT (step 2, Table 6.5.4.3.2-1;
step 1, TS 37.579-1 [2], Table 5.3.26.3-1)

Derivation Path: TS 37.579-1 [2], Table 5.5.4.4-1, condition GROUPCREATE

Table 6.5.4.3.3-2: SIP MESSAGE from the SS (Step 4, Table 6.5.4.3.2-1;
Step 2, TS 37.579-1 [2], Table 5.3.33.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.2-1, condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| P-Asserted-Identity |  |  |  |  |
| name-addr | px_MCX_SIP_PublicUserId_B |  | TS 24.379 [9] clause 16.3.2.4 |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.5.4.3.3-3 |  | TS 24.379 [9] clause 16.3.2.4 |  |
| MIME body part |  | MCPTT-Regroup |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-regroup+xml" |  |  |  |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the MCPTT-Regroup XML MIME body; used as reference in the signature MIME body |  |  |
| MIME-part-body | MCPTT-Regroup as described in Table 6.5.4.3.3-4 |  | TS 24.379 [9] clause 16.3.2.4 |  |

Table 6.5.4.3.3-3: MCPTT-Info in SIP MESSAGE (Table 6.5.4.3.3-2)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | not present |  |  |  |
| mcptt-calling-user-id | not present |  |  |  |
| mcptt-client-id | Encrypted <mcptt-client-id> with mcpttString set to px_MCX_Client_B_ID | Encryption according to NOTE 1 in TS 37.579-1 [2] Table 5.5.3.2.2-1 | TS 24.379 [9] clause 16.3.2.4 |  |

Table 6.5.4.3.3-4: MCPTT-Regroup in SIP MESSAGE (Table 6.5.4.3.3-2)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.16.3-1, condition USER_REGROUP

Table 6.5.4.3.3-5: SIP INVITE from the UE (step 6, Table 6.5.4.3.2-1;
step 2, TS 37.579-1 [2] Table 5.3A.1.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.5.1-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | SDP message |  |  |
| MIME-part-body | SDP Message as described in Table 6.5.4.3.3-6 |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.5.4.3.3-7 |  |  |  |

Table 6.5.4.3.3-6: SDP in SIP INVITE (Table 6.5.4.3.3-5)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.1.1-1, condition INITIAL_SDP_OFFER, IMPLICIT_GRANT_REQUESTED

Table 6.5.4.3.3-7: MCPTT-Info in SIP INVITE (Table 6.5.4.3.3-5)

| Derivation Path: TS 37.579-1 [2], Table 5.5.3.2.1-1, condition GROUP-CALL, INVITE_REFER |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| mcptt-request-uri | Encrypted <mcptt-request-uri> with mcpttURI set to px_MCPTT_Group_T_ID | Encryption according to NOTE 1 and NOTE 2 in TS 37.579-1 [2] Table 5.5.3.2.1-1 |  |  |

Table 6.5.4.3.3-8: SIP 200 (OK) from the SS (step 6, Table 6.5.4.3.2-1; 
step 4, TS 37.579-1 [2] Table 5.3A.1.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.17.1.2-1, condition INVITE-RSP |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| SDP Message | As described in Table 6.5.4.3.3-9 |  |  |  |

Table 6.5.4.3.3-9: SDP in SIP 200 (OK) (Table 6.5.4.3.3-8)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.1.2-1, condition SDP_ANSWER, IMPLICIT_GRANT_REQUESTED, IMPLICIT_FLOOR_GRANTED

Table 6.5.4.3.3-10: SIP MESSAGE from the SS (Step 12, Table 6.5.4.3.2-1;
Step 2, TS 37.579-1 [2], Table 5.3.33.3-1)

| Derivation Path: TS 37.579-1 [2], Table 5.5.2.7.2-1, condition ACCEPT-CONTACT-WITH-MEDIA-FEATURE-TAG |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| P-Asserted-Identity |  |  |  |  |
| name-addr | px_MCX_SIP_PublicUserId_B |  | TS 24.379 [9] clause 16.3.2.5 |  |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT-Info |  |  |
| MIME-part-body | MCPTT-Info as described in Table 6.5.4.3.3-3 |  | TS 24.379 [9] clause 16.3.2.5 |  |
| MIME body part |  | MCPTT-Regroup |  |  |
| MIME-part-headers |  |  |  |  |
| Content-Type | "application/vnd.3gpp.mcptt-regroup+xml" |  |  |  |
| Content-ID | Unique id in format of a Message-ID assigned by the SS | Unique URL identifying the MCPTT-Regroup XML MIME body; used as reference in the signature MIME body |  |  |
| MIME-part-body | MCPTT-Regroup as described in Table 6.5.4.3.3-11 |  | TS 24.379 [9] clause 16.3.2.5 |  |

Table 6.5.4.3.3-11: MCPTT-Regroup in SIP MESSAGE (Table 6.5.4.3.3-10)

Derivation Path: TS 37.579-1 [2], Table 5.5.3.16.3-1, condition REMOVE