3GPP TSG-RAN5 Meeting #109 	R5-256955

# Dallas, Texas, USA, 17th November 2025 – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0906 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICS for UL Tx Switching |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Innovation Center Inc |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_RF_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2025-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. According to 38.306, UL Tx Switching supported option is signaled by the UE using either uplinkTxSwitching-OptionSupport-r16 and/or uplinkTxSwitching-OptionSupport2T2T-r17. According to 38.331 these two capabilities can be set to one of three possible values {switchedUL, dualUL, both}. Depending on which UL Tx switching option is supported by the UE, the tests might require a different configuration, therefore a PICS is required to identify the supported option. 2. Simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA is a Rel.15 feature, but it is currently marked as Rel.17 in Table A.4.3.6-1/80. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added PICS to control which UL Tx switching option is supported by the UE. - Corrected the release for A.4.3.6-1/80. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case applicability and configuration would be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R5-256506 > R5-256506r1 > R5-256955 R1: Removed PICS for ‘both’ as this capability can be derived from ‘switchedUL’ AND ‘dualUL’. Renumered PICS indices accordingly. Updated r17 PICS mnemonics to include ‘2T2T’ wording. |  |  |  |  |  |  |  |  |

<< Start of Changes >>

### A.4.3.2	Physical Layer Baseline Implementation Capabilities

Table A.4.3.2-1: UE Physical Layer Baseline Implementation Capabilities

| Item | UE Physical Layer Baseline Implementation Capabilities | Ref. | Release | Mnemonic | M | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support PDSCH reception based on semi-persistent scheduling | 38.306, 4.2.7.10 | Rel-15 | pc_downlinkSPS | No |  |  |
| 2 | Support 256QAM for PDSCH for FR1 | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_256QAM_FR1 | CY |  | Mandatory for non-(e)RedCap UEs and optional for (e)RedCap UEs. |
| 3 | Support 256QAM for PDSCH for at least one NR FR2 band | 38.306, 4.2.7.2 | Rel-15 | pc_pdsch_256QAM_FR2 | No |  |  |
| 4 | Support 256QAM for PUSCH for at least one NR FR1 band | 38.306, 4.2.7.2 | Rel-15 | pc_pusch_256QAM_FR1 | No |  |  |
| 4a | Support 256QAM for PUSCH for at least one NR FR2 band | 38.306, 4.2.7.2 | Rel-15 | pc_pusch_256QAM_FR2 | No |  |  |
| 5 | Support receiving PDSCH using PDSCH mapping type A with less than seven symbols | 38.306, 4.2.7.10 | Rel-15 | N/A | Yes | Yes | pdsch-MappingTypeA is purely mandatory |
| 6 | Support receiving PDSCH using PDSCH mapping type B | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_MappingTypeB | Yes |  |  |
| 7 | Support resource allocation Type 0 for PUSCH | 38.306, 4.2.7.10 | Rel-15 | pc_ra_Type0_PUSCH | No |  |  |
| 8 | Support scaling factor 0.75 is applied to the band in the max data rate calculation | 38.306, 4.2.7 | Rel-15 | pc_scalingFactor0dot75 |  |  |  |
| 9 | Support reconfiguration with sync using a contention free random access on PRACH resources that are associated with CSI-RS resources of the target cell | 38.306, 4.2.7.10 | Rel-15 | pc_csi_RS_CFRA_ForHO | No |  |  |
| 10 | Support Type 1 PUSCH transmissions with configured grant | 38.306, 4.2.7.10 | Rel-15 | pc_configuredUL_GrantType1 | No |  |  |
| 11 | Support Type 2 PUSCH transmissions with configured grant | 38.306, 4.2.7.10 | Rel-15 | pc_configuredUL_GrantType2 | No |  |  |
| 12 | Support PDSCH Reception when configured with higher layer parameter aggregationFactorDL > 1 | 38.306, 4.2.7.10 | Rel-15 | pc_pdsch_RepetitionMultiSlots | No |  |  |
| 13 | Supports supplemental uplink with dynamic switch (DCI based selection of PUSCH carrier) | 38.306, 4.2.7.7 | Rel-15 | pc_dynamicSwitch_SUL | No |  |  |
| 14 | Supports codebook based PUSCH MIMO Transmission. UE indicating support of this feature shall also indicate support of PUSCH codebook coherency subset | 38.306, 4.2.7.8 | Rel-15 | pc_nrMIMO_CB_PUSCH | No |  |  |
| 14A | The supported maximum number of MIMO layers at the UE for PUSCH transmission with codebook precoding is four layers. UE indicating support of this feature shall also indicate support of PUSCH codebook coherency subset | 38.306, 4.2.7.8 | Rel-15 | pc_maxNumberMIMO_LayersCB_PUSCH_fourlayer | No |  | Set to true if maxNumberMIMO-LayersCB-PUSCH Is Four Layers. |
| 14B | The supported maximum number of MIMO layers at the UE for PUSCH transmission with codebook precoding is two layers. UE indicating support of this feature shall also indicate support of PUSCH codebook coherency subset | 38.306, 4.2.7.8 | Rel-15 | pc_maxNumberMIMO_LayersCB_PUSCH_twolayer | No |  | Set to true if maxNumberMIMO-LayersCB-PUSCH Is Two Layers. |
| 15 | Void |  |  |  |  |  |  |
| 16 | Support receiving PDSCH with interleaved VRB-to-PRB mapping | 38.306, 4.2.7.10 | Rel-15 | pc_interleavingVRB_ToPRB_PDSCH | Yes |  |  |
| 17 | Support dynamic EN-DC power sharing for at least one EN-DC band combination_FR1 only | 38.306, 4.2.7.9 | Rel-15 | pc_dynamicPowerSharingENDC | Yes |  | If the UE supports this capability it will dynamically share the power between NR and LTE if P_LTE + P_NR > Pcmax. |
| 18 | Supports up to 10 search spaces in a SCell per BWP | 38.306, 4.2.7.10 | Rel-15 | pc_maxNumberSearchSpaces | No |  |  |
| 19 | Supports spatial bundling of HARQ-ACK bits carried on PUCCH or PUSCH per PUCCH group. With spatial bundling, two HARQ-ACK bits for a DL MIMO data is bundled into a single bit by logical "AND" operation | 38.306, 4.2.7.10 | Rel-15 | pc_spatialBundlingHARQ_ACK | Yes |  |  |
| 20 | Support alternative additional DMRS position for co-existence with LTE CRS | 38.306, 4.2.7.5 | Rel-15 | pc_additionalDMRS_DL_Alt | No |  |  |
| 21 | Supports transmitting PUSCH scheduled by DCI format 0_0 or 0_1 when configured with higher layer parameter aggregationFactorIUL > 1 | 38.306, 4.2.7.10 | Rel-15 | pc_pusch_RepetitionMultiSlots | Yes |  |  |
| 22 | Support beam correspondence without UL beam sweeping | 38.306, 4.2.7.2 | Rel-15 | pc_beamCorrespondenceWithoutUL_BeamSweeping | Yes |  | A UE that can fulfil the requirements without UL beam sweeping then set the bit to 1. A UE that can fulfil the requirements with UL beam sweeping then set the bit to 0. |
| 22A | Support beam correspondence based on SSB | 38.306, 4.2.7.2 | Rel-16 | pc_beamCorrespondence_SSBbased | No |  |  |
| 22B | Support beam correspondence based on CSI-RS | 38.306, 4.2.7.2 | Rel-16 | pc_beamCorrespondence_CSI_RSbased | No |  |  |
| 23 | The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 8 Layers. For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. If absent, the UE doesn’t support MIMO on this carrier | 38.306, 4.2.7.6 | Rel-15 | pc_maxNumberMIMO_LayersPDSCH_eightLayers | CY |  | Set to false if Table A.4.3.2-1/23A or 23B set to true. |
| 23A | The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 4 Layers. | 38.306, 4.2.7.6 | Rel-15 | pc_maxNumberMIMO_LayersPDSCH_fourLayers | CY |  | Set to false if Table A.4.3.2-1/23 or 23B set to true. |
| 23B | The maximum number of spatial multiplexing layer(s) supported by the UE for DL reception is 2 Layers. | 38.306, 4.2.7.6 | Rel-15 | pc_maxNumberMIMO_LayersPDSCH_twoLayers | CY |  | Set to false if Table A.4.3.2-1/23 or 23A set to true. |
| 24 | Supports DCI and timer based active BWP switching delay type1 | 38.306, 4.2.7.10 | Rel-15 | pc_bwp_SwitchingDelay_Type1 | No |  | It is mandatory to report one among BWP switching delay type1 or type 2 as supported |
| 24A | Supports DCI and timer based active BWP switching delay type2 | 38.306, 4.2.7.10 | Rel-15 | pc_bwp_SwitchingDelay_Type2 | No |  | It is mandatory to report one among BWP switching delay type1 or type 2 as supported |
| 24B | Supports incremental delay for DCI and timer based active BWP switching type 1 on multiple CCs simultaneously | 38.306, 4.2.7.10 | Rel-16 | pc_bwp_SwitchingMultiCCs_type1_r16 | No |  | It is mandatory to report one among incremental delay for DCI and timer based active BWP switching type 1 or type 2on multiple CCs simultaneously as supported |
| 24C | Supports incremental delay for DCI and timer based active BWP switching type 2 on multiple CCs simultaneously | 38.306, 4.2.7.10 | Rel-16 | pc_bwp_SwitchingMultiCCs_type2_r16 | No |  | It is mandatory to report one among incremental delay for DCI and timer based active BWP switching type 1 or type 2on multiple CCs simultaneously as supported |
| 25A | Support modified MPR behaviour bit 0 | 38.306 4.2.7.2 | Rel-15 | pc_modifiedMPR_behaviour_bit0 | No |  | Applicable to FR2 bands n257, n258, n260 and n261 |
| 25 | Support modified MPR behaviour | 38.306 4.2.7.2 | Rel-15 | pc_modifiedMPR_behaviour | No |  |  |
| 26 | Support dynamic switching between resource allocation Types 0 and 1 for PDSCH | 38.306, 4.2.7.10 | Rel-15 | pc_dynamicSwitchRA_Type0_1_PDSCH | No |  |  |
| 27 | Support dynamic switching between resource allocation Types 0 and 1 for PUSCH | 38.306, 4.2.7.10 | Rel-15 | pc_dynamicSwitchRA_Type0_1_PUSCH | No |  |  |
| 28 | Support almost contiguous UL CP-OFDM transmissions in FR1 | 38.306, 4.2.7.10 | Rel-15 | pc_almostContiguousCP_OFDM_UL_FR1 | No |  |  |
| 29 | Support almost contiguous UL CP-OFDM transmissions in FR2 | 38.306, 4.2.7.10 | Rel-15 | pc_almostContiguousCP_OFDM_UL_FR2 | No |  |  |
| 30 | Support dynamic indication of applicable minimum scheduling restriction by DCI format 0_1 and 1_1, and the minimum scheduling offset for PDSCH and aperiodic CSI-RS triggering offset (K0), and PUSCH (K2), and the extended value range for aperiodic CSI-RS triggering offset | 38.306, 4.2.7.10 | Rel-16 | pc_crossSlotScheduling | No |  |  |
| 31 | Supports pi/2-BPSK modulation scheme for PUSCH in FR1 | 38.306, 4.2.7.10 | Rel-15 | pc_pusch_halfpiBPSK_FR1 | Yes |  |  |
| 31a | Supports pi/2-BPSK modulation scheme for PUSCH in FR2 | 38.306, 4.2.7.10 | Rel-15 | pc_pusch_halfpiBPSK_FR2 | Yes |  |  |
| 32 | Support multi-DCI based multi-TRP and support of fully/partially overlapping PDSCHs in time and non-overlapping in frequency | 38.306, 4.2.7.6 | Rel-16 | pc_multiDCI_MultiTRP_r16 | No |  |  |
| 33 | Support receiving PDSCH with resource mapping that excludes the REs determined by the higher layer configuration LTE-carrier configuring common RS | 38.306, 4.2.7.2 | Rel-15 | pc_rateMatchingLTE_CRS | Yes |  |  |
| 34 | Support of BWP operation without bandwidth restriction | 38.306, 4.2.7.2 | Rel-15 | pc_bwp_WithoutRestriction | No |  |  |
| 35 | Support of receiving SCell dormancy indication on SPCell using DCI format 2_6 outside the active time | 38.306, 4.2.7.4 | Rel-16 | pc_scellDormancyOutsideActiveTime_r16 | No |  |  |
| 36 | Supports pi/2-BPSK modulation scheme for power boosting in FR1 | 38.306, 4.2.7.2 | Rel-15 | pc_powerBoosting_pi2BPSK | No |  |  |
| 37 | Support of dynamic 1Tx-2Tx UL Tx switching | 38.306, 4.2.7.1 | Rel-16 | pc_ULTxSwitchingBandPair | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2B.2.3.1-2 and Table A.4.3.2C.2-1 |
| 37a | Support of 35us Tx switching period for 1Tx-2Tx UL Tx switching | 38.306 4.2.7.1 | Rel-16 | pc_switchingPeriod_35us_r16 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2B.2.3.1-2 and Table A.4.3.2C.2-1 |
| 37b | Support of 140us Tx switching period for 1Tx-2Tx UL Tx switching | 38.306 4.2.7.1 | Rel-16 | pc_switchingPeriod_140us_r16 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2B.2.3.1-2 and Table A.4.3.2C.2-1 |
| 37c | Support of 210us Tx switching period for 1Tx-2Tx UL Tx switching | 38.306 4.2.7.1 | Rel-16 | pc_switchingPeriod_210us_r16 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2B.2.3.1-2 and Table A.4.3.2C.2-1 |
| 37d | Support of DL interruption during 1Tx-2Tx UL Tx switching | 38.306 4.2.7.1 | Rel-18 | pc_uplinkTxSwitching_DL_Interruption_r16 | No |  | Not allowed to be setted to TRUE for the band combination of SUL band+TDD band, for which no DL interruption is allowed. |
| 37e | Support of switchedUL Option for r16 UL Tx Switching | 38.306 4.2.7.1 | Rel-16 | pc_uplinkTxSwitching_OptionSupport_switchedUL_r16 | No |  | Corresponds to IE uplinkTxSwitching-OptionSupport-r16 set to switchtedUL |
| 37f | Support of dualUL Option for r16 UL Tx Switching | 38.306 4.2.7.1 | Rel-16 | pc_uplinkTxSwitching_OptionSupport_dualUL_r16 | No |  | Corresponds to IE uplinkTxSwitching-OptionSupport-r16 set to dualUL |
| 37g | Support of switchedUL Option for r17 UL Tx Switching | 38.306 4.2.7.1 | Rel-17 | pc_uplinkTxSwitching_OptionSupport2T2T_switchedUL_r17 | No |  | Corresponds to IE uplinkTxSwitching-OptionSupport2T2T-r17 set to switchtedUL |
| 37h | Support of dualUL Option for r17 UL Tx Switching | 38.306 4.2.7.1 | Rel-17 | pc_uplinkTxSwitching_OptionSupport2T2T_dualUL_r17 | No |  | Corresponds to IE uplinkTxSwitching-OptionSupport2T2T-r17 set to dualUL |
| 38 | Support uplink transmission power boost by suspension of in-band emission (IBE) in FR2 | 38.306 4.2.7.2 | Rel-16 | pc_mpr_PowerBoost_FR2 | No |  |  |
| 39 | Supports the alternative 64QAM MCS table for PDSCH | 38.306, 4.2.7.10 | Rel-15 | pc_dl_64qam_mcs_tableAlt | No |  |  |
| 40 | Supports the CQI table with target BLER of 10^-5 | 38.306, 4.2.7.10 | Rel-15 | pc_cqi_tableAlt | No |  |  |
| 41 | Supports of single DCI based spatial division multiplexing scheme | 38.306, 4.2.7.5 | Rel-16 | pc_singledci_sdm_scheme_r16 | No |  |  |
| 42 | Support of BWP adaptation (up to 2 BWPs) with the same numerology for FR1 FDD bands | 38.306, 4.2.7.2 | Rel-15 | pc_bwp_SameNumerology_upto2_FR1_FDD | No |  | FR1 FDD bands |
| 42a | Support of BWP adaptation (up to 2 BWPs) with the same numerology for FR1 TDD bands | 38.306, 4.2.7.2 | Rel-15 | pc_bwp_SameNumerology_upto2_FR1_TDD | No |  | FR1 TDD bands |
| 42b | Support of BWP adaptation (up to 2 BWPs) with the same numerology for FR2 bands | 38.306, 4.2.7.2 | Rel-15 | pc_bwp_SameNumerology_upto2_FR2 | No |  | FR2 bands |
| 43 | Support of BWP adaptation (up to 4 BWPs) with the same numerology for FR1 FDD bands | 38.306, 4.2.7.2 | Rel-15 | pc_bwp_SameNumerology_upto4_FR1_FDD | No |  | FR1 FDD bands |
| 43a | Support of BWP adaptation (up to 4 BWPs) with the same numerology for FR1 TDD bands | 38.306, 4.2.7.2 | Rel-15 | pc_bwp_SameNumerology_upto4_FR1_TDD | No |  | FR1 TDD bands |
| 43b | Support of BWP adaptation (up to 4 BWPs) with the same numerology for FR2 bands | 38.306, 4.2.7.2 | Rel-15 | pc_bwp_SameNumerology_upto4_FR2 | No |  | FR2 bands |
| 44 | Support BWP adaptation up to 4 BWPs with the different numerologies, via DCI and timer for FR1 FDD bands | 38.306 4.2.7.2 | Rel-15 | pc_bwp_DiffNumerology_FR1_FDD | No |  | FR1 FDD bands |
| 44a | Support BWP adaptation up to 4 BWPs with the different numerologies, via DCI and timer for FR1 TDD bands | 38.306 4.2.7.2 | Rel-15 | pc_bwp_DiffNumerology_FR1_TDD | No |  | FR1 TDD bands |
| 44b | Support BWP adaptation up to 4 BWPs with the different numerologies, via DCI and timer for FR2 bands | 38.306 4.2.7.2 | Rel-15 | pc_bwp_DiffNumerology_FR2 | No |  | FR2 bands |
| 45 | Support PUSCH repetition type B | 38.306, 4.2.7.7 | Rel-16 | pc_pusch_RepetitionTypeB_r16 | No |  |  |
| 46 | Support of 2-Step RACH | 38.306, 4.2.7.10 | Rel-16 | pc_twoStepRACH_r16 | No |  |  |
| 47 | Void |  |  |  |  |  |  |
| 48 | Void |  |  |  |  |  |  |
| 49 | Void |  |  |  |  |  |  |
| 50 | Void |  |  |  |  |  |  |
| 51 | Void |  |  |  |  |  |  |
| 52 | Support monitoring DCI format 1_2 for DL scheduling and monitoring DCI format 0_2 for UL scheduling | 38.306, 4.2.7.10 | Rel-16 | pc_dci_Format1_2And0_2_r16 | No |  |  |
| 53 | Support of multi-DCI based multi-TRP | 38.306, 4.2.7.6 | Rel-16 | pc_multi_dci_multi_trp | No |  |  |
| 54 | Support of single DCI based FDMSchemeA | 38.306, 4.2.7.2 | Rel-16 | pc_single_dci_fdmschemeA | No |  |  |
| 55 | Support of single-DCI based inter-slot TDM | 38.306, 4.2.7.2 | Rel-16 | pc_single_dci_interslot_tdm | No |  |  |
| 56 | Support of maximum number of TRS resource sets per CC which the UE can track simultaneously is at least 2 | 38.306, 4.2.7.2 | Rel-16 | pc_simultaneous_TRS | No |  | Corresponds to IE maxSimultaneousResourceSetsPerCC within csi-RS-ForTracking |
| 57 | Support of low PAPR DMRS | 38.306 4.2.7.2 | Rel-16 | pc_lowPAPR_DMRS_pusch_precoding | No |  |  |
| 58 | Support of UL full power transmission mode of full power | 38.306, 4.2.7.7 | Rel-16 | pc_ul_FullPwrMode_r16 | No |  |  |
| 59 | Support of UL full power transmission mode of fullpowerMode1 | 38.306, 4.2.7.7 | Rel-16 | pc_ul_FullPwrMode1_r16 | No |  |  |
| 60 | Void |  |  |  |  |  |  |
| 61 | Support of PDSCH processing capability 2 | 38.306, 4.2.7.5 | Rel-16 | pc_pdsch_processing_cap2 | No |  |  |
| 62 | Support Pre-Emption Indication | 38.306, 4.2.7.10 | Rel-15 | pc_preEmptIndication_DL | No |  |  |
| 63 | Support of SSB based BFD | 38.306, 4.2.7.2 | Rel-15 | pc_maxNumberSSB_BFD | CY |  |  |
| 64 | Support of CSI-RS based BFD | 38.306, 4.2.7.2 | Rel-15 | pc_maxNumberCSI_RS_BFD | CY |  |  |
| 65 | Support of SSB and/or CSI-RS based Link Recovery | 38.306, 4.2.7.2 | Rel-15 | pc_maxNumberCSI_RS_SSB_CBD | CY |  |  |
| 66 | Support of type II codebook | 38.306, 4.2.7.2 | Rel-15 | pc_typeIICodebook | No |  | Applicable for UE support at least 16 ports per CSI-RS resource based on 38.306 IE maxNumberTxPortsPerResource under CodebookParameters Type2 |
| 67 | Support of Enhanced Type II codebook with at least 16 ports per CSI-RS resource | 38.306, 4.2.7.2 | Rel-16 | pc_enhanced_typeII_codebook | No |  | Applicable for UE support at least 16 ports per CSI-RS resource based on 38.306 IE maxNumberTxPortsPerResource under CodebookParameters eType2 |
| 68 | Support of TDD NR UL transmission with a 7.5 kHz shift to the LTE raster | 38.101-1, 5.4.2 | Rel-15 | pc_frequencyShift7p5khz_TDD | No |  | Mandatory since Rel-16 |
| 69 | Support of FDD NR UL transmission with a 7.5 kHz shift to the LTE raster | 38.101-1, 5.4.2 | Rel-15 | pc_frequencyShift7p5khz_FDD | Yes |  |  |
| 70 | Void |  |  |  |  |  |  |
| 71 | Support of density of CSI-RS for Channel Measurement Report | 38.306, 4.2.7.2 | Rel-16 | pc_supportedCSI_RS_Density_CMR | No |  |  |
| 72 | Support of SSB/CSI-RS for L1-SINR measurement | 38.306, 4.2.7.2 | Rel-16 | pc_ssb_csirs_SINR_measurement | No |  |  |
| 73 | Support of SSB as CMR with dedicated CSI-IM for L1-SINR measurement | 38.306, 4.2.7.2 | Rel-16 | pc_supportedSINR_meas_ssbWithCSI_IM | No |  |  |
| 74 | Support of SSB as CMR with dedicated NZP IMR for L1-SINR measurement | 38.306, 4.2.7.2 | Rel-16 | pc_supportedSINR_meas_ssbWithNZP_IMR | No |  |  |
| 75 | Support of CSI-RS as CMR with dedicated NZP IMR configured for L1-SINR measurement | 38.306, 4.2.7.2 | Rel-16 | pc_supportedSINR_meas_csirsWithNZP_IMR | No |  |  |
| 76 | Support of CSI-RS as CMR without dedicated IMR configured for L1-SINR measurement | 38.306, 4.2.7.2 | Rel-16 | pc_supportedSINR_meas_csi_RSWithoutIMR | No |  |  |
| 77 | Support of SCell beam failure recovery | 38.306, 4.2.7.2 | Rel-16 | pc_scellBFR | No |  |  |
| 78 | Support of the maximum number of activated TCI states per BWP per CC is other than n1, including control and data | 38.306, 4.2.7.2 | Rel-15 | pc_maxNumberActiveTCI_PerBWP | No |  |  |
| 79 | Support enhanced UL performance for the transient period | 38.306, 4.2.7.2 | Rel-16 | pc_enhancedUL_TransientPeriod_r16 | No |  |  |
| 80 | Supports the priority indicator field configured in DCI formats 1_1 and 1_2 in a BWP when configured to monitor both DCI formats 1_1 and 1_2 in the BWP | 38.306, 4.2.7.10 | Rel-16 | pc_dci_DL_PriorityIndicator_r16 | No |  |  |
| 81 | Supports the priority indicator field configured in DCI formats 0_1 and 0_2 in a BWP when configured to monitor both DCI formats 0_1 and 0_2 in the BWP | 38.306, 4.2.7.10 | Rel-16 | pc_dci_UL_PriorityIndicator_r16 | No |  | A UE supporting this feature shall also support ul-IntraUE-Mux-r16 and dci-Format1-2And0-2-r16 |
| 82 | Supports restricting data transmission from a given LCH to a configured (sub-) set of dynamic grant priority levels | 38.306, 4.2.6 | Rel-16 | pc_lch_ToGrantPriorityRestriction_r16 | No |  |  |
| 83 | Supports two PUCCH group in CA with a same numerology across CCs for data and control channel. | 38.306 4.2.7.7 | Rel-15 | pc_twoPUCCH_group | No |  |  |
| 84 | Support of transparent Tx diversity requirements for at least one NR FR1 band for single band (non-CA) case | 38.306, 4.2.7.238.331, Annex C | Rel-16 | pc_txDiversity_r16 | No |  | FR1 only This capability has been introduced in Rel-16 and is early implementable from Rel-15 onwards. |
| 84A | Support of 2Tx Tx diversity for the band configured | 38.306, 4.2.7.7 | Rel-18 | pc_txDiversity2Tx_r18 | No |  |  |
| 84B | Support of 4Tx Tx diversity for the band configured | 38.306, 4.2.7.7 | Rel-18 | pc_txDiversity4Tx_r18 | No |  |  |
| 85 | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | 38.306, 4.2.7.2 | Rel-17 | pc_pusch_RepetitionMsg3_r17 | No |  |  |
| 86 | Support of DL scheduling slot offset (K0) greater than 0 for PDSCH mapping type A. | 38.306, 4.2.7.10 | Rel-15 | pc_dl_SchedulingOffset_PDSCH_TypeA | Yes |  |  |
| 87 | Support of CQI reporting with 4 bits per subband for NTN and shared spectrum channel access | 38.306, 4.2.7.2 | Rel-17 | pc_cqi_4_BitsSubbandNTN_SharedSpectrumChAccess_r17 | No |  | UE supports CQI reporting with 4 bits per subband for NTN and shared spectrum channel access |
| 88 | Support of propagation delay compensation based on legacy TA procedure for NTN and shared spectrum channel access | 38.306, 4.2.7.2 | Rel-17 | pc_ta_BasedPDC_NTN_SharedSpectrumChAccess_r17 | No |  | UE supports propagation delay compensation based on legacy TA procedure for NTN and shared spectrum channel access |
| 89 | Support of 8 dynamic slot-level repetitions for group-common PDSCH for multicast for NTN and shared spectrum channel access | 38.306, 4.2.7.2 | Rel-17 | pc_dynamicSlotRepetitionMulticastNTN_SharedSpectrumChAccess_r17_n8 | No |  | UE supports 8 dynamic slot-level repetitions for group-common PDSCH for multicast for NTN and shared spectrum channel access |
| 90 | Support of 16 dynamic slot-level repetitions for group-common PDSCH for multicast for NTN and shared spectrum channel access | 38.306, 4.2.7.2 | Rel-17 | pc_dynamicSlotRepetitionMulticastNTN_SharedSpectrumChAccess_r17_n16 | No |  | UE supports 16 dynamic slot-level repetitions for group-common PDSCH for multicast for NTN and shared spectrum channel access |
| 91 | Support of NTN features in GSO scenario | 38.306, 4.2.2 | Rel-17 | pc_ntn_ScenarioSupport_r17_GSO | No |  | UE supports NTN features in GSO scenario |
| 92 | Support of NTN features in NGSO scenario | 38.306, 4.2.2 | Rel-17 | pc_ntn_ScenarioSupport_r17_NGSO | No |  | UE supports NTN features in NGSO scenario |
| 93 | Void |  |  |  |  |  |  |
| 94 | Supports the restriction to 3450 - 3550 MHz and 3700 - 3980 MHz ranges of band n77 | 38.306, 4.2.7.11 | Rel-16 | pc_extendedBand_n77_r16 | No |  | Applicable for UE support band n77 and in the USA this band is restricted to 3450 – 3550 MHz and 3700 – 3980 MHz. |
| 95 | Supports the restriction to 3450 - 3650 MHz and 3650 - 3980 ranges of band n77 | 38.306, 4.2.7.11 | Rel-17 | pc_extendedBand_n77_2_r17 | No |  | Applicable for UE support band n77 and in Canada this band is restricted to 3450 – 3650 MHz and 3650 – 3980 MHz. |
| 96 | Void |  |  |  |  |  |  |
| 97 | Void |  |  |  |  |  |  |
| 98 | Void |  |  |  |  |  |  |
| 99 | Void |  |  |  |  |  |  |
| 100 | Void |  |  |  |  |  |  |
| 101 | Void |  |  |  |  |  |  |
| 102 | Void |  |  |  |  |  |  |
| 103 | Void |  |  |  |  |  |  |
| 104 | Support of 1024QAM modulation scheme for PDSCH with maximum 2 MIMO layers for FR1 FDD bands | 38.306, 4.2.7.2 | Rel-17 | pc_pdsch_1024QAM_2MIMO_FR1_r17_FDD | No |  | FR1 FDD bands |
| 105 | Support of 1024QAM modulation scheme for PDSCH with maximum 2 MIMO layers for FR1 TDD bands | 38.306, 4.2.7.2 | Rel-17 | pc_pdsch_1024QAM_2MIMO_FR1_r17_TDD | No |  | FR1 TDD bands |
| 106 | Support of 1024QAM modulation scheme for PDSCH for FR1_FDD bands | 38.306, 4.2.7.2 | Rel-17 | pc_pdsch_1024QAM_FR1_r17_FDD | No |  | FR1 FDD bands |
| 107 | Support of 1024QAM modulation scheme for PDSCH for FR1_TDD bands | 38.306, 4.2.7.2 | Rel-17 | pc_pdsch_1024QAM_FR1_r17_TDD | No |  | FR1 TDD bands |
| 108 | Support RTT-based propagation delay compensation for time synchronization of the Uu interface based on CSI-RS for tracking and SRS. | 38.306, 4.2.7.5 | Rel-17 | pc_rtt_BasedPDC_CSI_RS_ForTracking_r17 | No |  | A UE supporting this feature shall also indicate support of csi-RS-ForTracking and supportedSRS-Resources as specified in TS 38.331. |
| 109 | Support RTT-based Propagation delay compensation for time synchronization of the Uu interface based on DL PRS and SRS. | 38.306, 4.2.7.5 | Rel-17 | pc_rtt_BasedPDC_PRS_r17 | No |  | If UE provides parameter maxNumberPRS-Resource-r17 and optionally parameter maxNumberPRS-ResourceProcessedPerSlot-r17 as described in TS 38.331, consider this as supported, otherwise not supported. A UE supporting this feature shall also indicate support of supportedSRS-Resources as specified in TS 38.331. |
| 110 | Support propagation delay compensation based on legacy TA procedure for TN and non-shared spectrum channel access. | 38.306, 4.2.7.10 | Rel-17 | pc_ta_BasedPDC_TN_NSSChAccess_r17 | No |  |  |
| 111 | Support Type 1 PUSCH transmissions with configured grant | 38.306 4.2.7.10 | Rel-15 | pc_type1_PUSCH_RepetitionMultiSlots | No |  |  |
| 112 | Support Type 2 PUSCH transmissions with configured grant | 38.306 4.2.7.10 | Rel-15 | pc_type2_PUSCH_RepetitionMultiSlots | No |  |  |
| 113 | Support the dynamic indication of the number of repetitions for PUSCH transmission | 38.306 4.2.7.10 | Rel-16 | pc_pusch_RepetitionTypeA_r16 | No |  |  |
| 114 | Support repetitions for PUCCH format 0, 1, 2, 3 and 4 over multiple PUCCH subslots based on dynamic repetition indication | 38.306 4.2.7.7 | Rel-17 | pc_pucch_Repetition_F0_1_2_3_4_DynamicIndication_r17 | No |  |  |
| 115 | Support the increased maximum number of PUSCH Type A repetitions up to 32 | 38.306 4.2.7.2 | Rel-17 | pc_maxNumberPUSCH_TypeA_Repetition_r17 | No |  |  |
| 116 | Support PUSCH repetitions based on available slots | 38.306 4.2.7.2 | Rel-17 | pc_puschTypeA_RepetitionsAvailSlot_r17 | No |  |  |
| 117 | Supports TB processing over multi-slot PUSCH without repetition | 38.306 4.2.7.2 | Rel-17 | pc_tb_ProcessingMultiSlotPUSCH_r17 | No |  |  |
| 118 | Supports repetition of TB processing over multi-slot PUSCH | 38.306 4.2.7.2 | Rel-17 | pc_tb_ProcessingRepMultiSlotPUSCH_r17 | No |  |  |
| 119 | Support SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH | 38.306, 4.2.7.5 | Rel-17 | pc_sfn_schemeA_r17 | No |  |  |
| 120 | Support SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH | 38.306, 4.2.7.5 | Rel-17 | pc_sfn_schemeB_r17 | No |  |  |
| 121 | Support of up to 8 configured SPS configurations in a BWP of a serving cell and up to 32 configured SPS configurations in a cell group | 38.306, 4.2.7.2 | Rel-16 | pc_multi_sps_r16 | No |  | The UE can include this feature only if the UE indicates support of downlinkSPS. |
| 122 | Support transmission of two PUCCH formats in TDM in the same slot | 38.306, 4.2.7.10 | Rel-15 | pc_twoPUCCH_AnyOthersInSlot | No |  |  |
| 123 | Support of unified TCI state operation with joint DL/UL TCI update for intra-cell beam management | 38.306, 4.2.7.2 | Rel-17 | pc_unifiedJointTCI_r17 | No |  |  |
| 124 | Support of unified TCI state operation with separate DL/UL TCI update for intra-cell beam management | 38.306, 4.2.7.2 | Rel-17 | pc_unifiedSeparateTCI_r17 | No |  |  |
| 125 | Support of RRC configuration of additional PCI different from serving cell associated with the TCI state and/or QCL-info | 38.306, 4.2.7.2 | Rel-17 | pc_mTRP_InterCell_r17 | No |  |  |
| 126 | Support of unified TCI with separate DL/UL TCI update for inter-cell beam management | 38.306, 4.2.7.2 | Rel-17 | pc_unifiedSeparateTCI_InterCell_r17 | No |  |  |
| 127 | Support of dynamic 2Tx-2Tx UL Tx switching | 38.306, 4.2.7.1 | Rel-17 | pc_2Tx_ULTxSwitchingBandPair_r17 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 |
| 127a | Support of R18 dynamic UL Tx switching across up to 4 bands in case of inter-band CA, SUL | 38.306 4.2.7.1 | Rel-18 | pc_ULTxSwitchingBandPair_r18 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2C.3-3 |
| 127b | Support of R18 dynamic UL Tx switching with 35us Tx switching period | 38.306 4.2.7.1 | Rel-18 | pc_switchingPeriod_35us_r18 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2C.3-3 |
| 127c | Support of R18 dynamic UL Tx switching with 140us Tx switching period | 38.306 4.2.7.1 | Rel-18 | pc_switchingPeriod_140us_r18 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2C.3-3 |
| 127d | Support of R18 dynamic UL Tx switching with 210us Tx switching period | 38.306 4.2.7.1 | Rel-18 | pc_switchingPeriod_210us_r18 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3, Table A.4.3.2C.3-3 |
| 127e | Support of R18 dynamic UL Tx switching with DL interruption occurring during Tx switching | 38.306 4.2.7.1 | Rel-18 | pc_uplinkTxSwitching_DL_Interruption_r18 | No |  | Not allowed to be setted to TRUE for the band combination of SUL band+TDD band, for which no DL interruption is allowed. |
| 128 | Indicates whether the UE supports aperiodic CSI-RS for tracking for fast SCell activation, i.e., 1) Aperiodic CSI-RS for tracking for fast SCell activation is triggered by enhanced SCell activation/deactivation MAC CE; 2) Aperiodic CSI-RS for tracking for fast SCell activation is triggered within the BWP indicated by firstActiveDownlinkBWP-Id for the SCell. | 38.306, 4.2.7.2 | Rel-17 | pc_aperiodicCSIRS_FastScellActivation_r17 | No |  | Includes maxNumberAperiodicCSI-RS-PerCC-r17 and maxNumberAperiodicCSI-RS-AcrossCCs-r17 to indicate the number of RS configurations for fast SCell activation that can be indicated by the MAC CE |
| 129 | Indicates whether the UE supports conditional PSCell addition in EN-DC. The UE supporting this feature shall also support 2 trigger events for same execution condition in conditional PSCell addition in EN-DC. | 38.306, 4.2.7.9 | Rel-17 | pc_condPSCellAdditionENDC_r17 | No |  |  |
| 130 | Support of conditional PSCell addition in NR-DC | 38.306, 4.2.7.12 | Rel-17 | pc_condPSCellAdditionNRDC_r17 | No |  |  |
| 131 | Support of activation (with or without RACH) and deactivation on SCG in EN-DC | 38.306, 4.2.7.9 | Rel-17 | pc_scg_ActivationDeactivationENDC_r17 | No |  |  |
| 132 | Support of activation (with or without RACH) and deactivation on SCG in NR-DC | 38.306, 4.2.7.12 | Rel-17 | pc_scg_ActivationDeactivationNRDC_r17 | No |  |  |
| 133 | Support of TCI state update and activation by a) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 with DL assignment) And b) MAC-CE+DCI-based TCI state indication (use of DCI formats 1_1/1_2 without DL assignment) | 38.306, 4.2.7.2 | Rel-17 | pc_unifiedSeparateTCI_multiMAC_CE_r17 | No |  | A UE supporting this feature shall also support unifiedSeparateTCI-r17. |
| 134 | Indicates whether UE supports partial frequency sounding for SRS with frequency hopping. | 38.306, 4.2.7.2 | Rel-17 | pc_srs_partialFrequencySounding_r17 | No |  |  |
| 135 | Indicates whether the UE supports partial frequency sounding for SRS with frequency hopping and start RB location hopping. | 38.306, 4.2.7.2 | Rel-17 | pc_srs_startRB_locationHoppingPartial_r17 | No |  | A UE supporting this feature shall also indicate support of pc_srs_partialFequencySounding_r17 as specified in TS 38.306. |
| 136 | Indicates whether the UE supports PDCCH search space monitoring occasions in any symbol of the slot (pdcch-MonitoringAnyOccasions) | 38.306, 4.2.7.5 | Rel-15 | pc_pdcchMonitoringAnyOccasions | No |  |  |
| 137 | Indicates whether the UE supports PDCCH search space monitoring occasions in any symbol of the slot with minimum time separation between two consecutive transmissions of PDCCH with span (pdcch-MonitoringAnyOccasionsWithSpanGap) | 38.306, 4.2.7.5 | Rel-15 | pc_pdcchMonitoringAnyOccasions_SpanGap | No |  |  |
| 138 | Support of UE reporting of information related to TA pre-compensation | 38.306, 4.2.7.2 | Rel-17 | pc_uplink_TA_Reporting_r17 | No |  | UE supports UE reporting of information related to TA pre-compensation. |
| 139 | Indicates whether the UE supports mTRP BFR based on two BFD-RS sets. | 38.306, 4.2.7.5 | Rel-17 | pc_mTRP_BFR_twoBFD_RS_Set_r17 | No |  |  |
| 140 | Indicates the support of intra-slot PDCCH repetition based on two linked SS sets associated with corresponding CORESETs. | 38.306, 4.2.7.5 | Rel-17 | pc_mTRP_PDCCH_Repetition_r17 | No |  |  |
| 141 | Support of reception of UE-specific K_offset | 38.306, 4.2.7.2 | Rel-17 | pc_ue_specific_K_Offset_r17 | No |  |  |
| 142 | Support of at least 2 TAGs | 38.306, 4.2.7.4 | Rel-15 | pc_supportedNumberTAG_n2_n3_n4 | No |  |  |
| 143 | Indicates whether UE supports multi-TRP PUSCH repetition based on codebook with PUSCH repetition type A. | 38.306, 4.2.7.7 | Rel-17 | pc_mtrpPuschTypeACb_r17 | No |  |  |
| 144 | Indicates whether UE supports multi-TRP PUSCH repetition based on codebook with PUSCH repetition type B. | 38.306, 4.2.7.8 | Rel-17 | pc_mtrpPuschTypeBCb_r17 | No |  |  |
| 145 | Indicates whether UE supports cyclic mapping when the number of repetitions is larger than 2 with repetition type. | 38.306, 4.2.7.2 | Rel-17 | pc_mtrpPuschCyclicMapping_r17 | No |  |  |
| 146 | Indicates whether UE supports PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion corresponding to each SRS resource set, and report two PHRs). | 38.306, 4.2.7.2 | Rel-17 | pc_mtrpPuschTwoPhrReporting_r17 | No |  |  |
| 147 | Indicates whether UE supports DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4 over consecutive symbols | 38.306 4.2.7.2 | Rel-17 | pc_dmrs_BundlingPUCCH_Rep_r17 | No |  | UE supports DMRS bundling over PUCCH |
| 148 | Indicates whether UE supports DM-RS bundling for TB processing over multi-slot PUSCH over consecutive symbols | 38.306 4.2.7.2 | Rel-17 | pc_dmrs_BundlingPUSCH_multiSlot_r17 | No |  | UE supports DMRS bundling over PUSCH |
| 149 | Indicates whether the UE supports DM-RS bundling for PUSCH repetition type A over consecutive symbols | 38.306 4.2.7.2 | Rel-17 | pc_dmrs_BundlingPUSCH_RepTypeA_r17 | No |  | UE supports DMRS bundling over PUSCH |
| 150 | Indicates whether the UE supports DM-RS bundling for PUSCH repetition type B over consecutive symbols | 38.306 4.2.7.2 | Rel-17 | pc_dmrs_BundlingPUSCH_RepTypeB_r17 | No |  | UE supports DMRS bundling over PUSCH |
| 151 | Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: -Reception of 12 PRB PBCH based on RB-level puncturing; -Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; -Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only. It is applicable when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2]. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL. | 38.306, 4.2.7.2 | Rel-18 | pc_support3MHz_ChannelBW_Symmetric_r18 | No |  | FR1 FDD bands Not applicable for (e)RedCap UEs |
| 152 | Indicates whether the UE supports reception of 12 PRB CORESET0 with an associated SS/PBCH block that is located according to Table 5.4.3.1-2 in TS 38.101-1 [2]. A UE supporting this feature shall also indicate support of support-3MHz-ChannelBW-Symmetric-r18. This feature is supported for 15kHz SCS only. NOTE: The UE supporting this capability supports configuration of 12 PRB BWP operation. | 38.306, 4.2.7.2 | Rel-18 | pc_support12PRB_CORESET0_r18 | No |  | FR1 FDD bands Not applicable for (e)RedCap UEs |
| 153 | Support of 35us Tx switching period for 2Tx-2Tx dynamic UL Tx switching | 38.306 4.2.7.1 | Rel-17 | pc_switchingPeriod_35us_r17 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 |
| 154 | Support of 140us Tx switching period for 2Tx-2Tx dynamic UL Tx switching | 38.306 4.2.7.1 | Rel-17 | pc_switchingPeriod_140us_r17 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 |
| 155 | Support of 210us Tx switching period for 2Tx-2Tx dynamic UL Tx switching | 38.306 4.2.7.1 | Rel-17 | pc_switchingPeriod_210us_r17 | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 |
| 156 | Support of DL interruption during 2Tx-2Tx dynamic UL Tx switching | 38.306 4.2.7.1 | Rel-17 | pc_uplinkTxSwitching_DL_Interruption_r17 | No |  | Not allowed to be set to TRUE for the band combination of SUL band+TDD band, for which no DL interruption is allowed. |
| 157 | Indicates the interruption time on DL/UL reception within a NR band pair during the RF retuning for switching between a carrier on one band and another (PUSCH-less) carrier on the other band to transmit SRS | 38.306, 4.2.7.1 | Rel-16 | pc_srs_SwitchingTimeNR | No |  |  |
| 158 | Indicates the interruption time on DL/UL reception within a EUTRA band pair during the RF retuning for switching between a carrier on one band and another (PUSCH-less) carrier on the other band to transmit SRS | 38.306, 4.2.7.1 | Rel-16 | pc_srs_SwitchingTimeEUTRA | No |  |  |
| 159 | Indicates whether the UE supports multi-PUSCHs for configured grant by indicating whether the UE supports the determination of time-domain resource allocation for CG-PUSCHs associated to a multi-PUSCHs CG | 38.306, 4.2.7.2 | Rel-18 | pc_multiPUSCH_CG_r18 | No |  | A UE supporting this feature shall also indicate support of at least one of configuredUL-GrantType1, configuredUL-GrantType1-v1650, configuredUL-GrantType2, and configuredUL-GrantType2-v1650. |
| 160 | Supports {2, 4, 8} for the number of multiple PRACH transmissions with same Tx spatial filter. | 38.306 4.2.7.2 | Rel-18 | pc_prach_CoverageEnh_r18 | No |  |  |
| 161 | UE is 2Rx XR UE | 38.306, 4.2.7.2 | Rel-18 | pc_supportOf2RxXR_r18 | No |  | Not applicable for (e)RedCap UEs |
| 162 | Indicates whether the UE supports monitoring DCI format 1_3 for DL scheduling with same SCS between scheduling cell and cells in the set and supports Type-2 for 'Antenna port(s)' field. | 38.306, 4.2.7.4 | Rel-18 | pc_multiCell_PDSCH_DCI_1_3_SameSCS_r18 | No |  |  |
| 163 | Indicates whether the UE supports monitoring DCI format 0_3 for UL scheduling with same SCS between scheduling cell and cells in the set and supports Type-2 for 'Antenna port(s)', 'Precoding information and number of layers' and 'SRS resource indicator' fields. | 38.306, 4.2.7.4 | Rel-18 | pc_multiCell_PUSCH_DCI_0_3_SameSCS_r18 | No |  |  |
| 164 | Indicates whether the UE supports co-scheduled cell indication scheme based on FDRA field of DCI format 1_3. | 38.306, 4.2.7.4 38.331 6.3.3 | Rel-18 | pc_coScheduledCellIndicationScheme_r18_fdra_DCI_1_3 | No |  |  |
| 165 | Indicates whether the UE supports co-scheduled cell indication scheme based on Scheduled cell set indicator field of DCI format 1_3 | 38.306, 4.2.7.4 38.331 6.3.3 | Rel-18 | pc_coScheduledCellIndicationScheme_r18_cellInd_DCI_1_3 | No |  |  |
| 166 | Indicates whether the UE supports co-scheduled cell indication scheme based on FDRA field of DCI format 0_3. | 38.306, 4.2.7.4 38.331 6.3.3 | Rel-18 | pc_coScheduledCellIndicationScheme_r18_fdra_DCI_0_3 | No |  |  |
| 167 | Indicates whether the UE supports co-scheduled cell indication scheme based on Scheduled cell set indicator field of DCI format 0_3 | 38.306, 4.2.7.4 38.331 6.3.3 | Rel-18 | pc_coScheduledCellIndicationScheme_r18_cellInd_DCI_0_3 | No |  |  |
| 168 | Indicate whether the UE supports search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells. | 38.306, 4.2.7.4 38.331 6.3.3 | Rel-18 | pc_supportOfSearchSpace_r18_DCI_1_3 | No |  |  |
| 169 | Indicate whether the UE supports search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells. | 38.306, 4.2.7.4 38.331 6.3.3 | Rel-18 | pc_supportOfSearchSpace_r18_DCI_0_3 | No |  |  |
| 170 | Indicates whether the UE supports harq feedback type1 for DCI format 1_3 | 38.306, 4.2.7.4 38.331 6.3.3 | Rel-18 | pc_harqFeedbackType1_DCI_1_3 | No |  |  |
| 171 | Indicates whether the UE supports harq feedback type2 for DCI format 1_3 | 38.306, 4.2.7.4 38.331 6.3.3 | Rel-18 | pc_harqFeedbackType2_DCI_1_3 | No |  |  |
| 172 | Indicates whether the UE supports basic feature of Rel-18 enhanced DMRS ports for PDSCH for scheduling of mapping type A. | 38.306 4.2.7.5 | Rel-18 | pc_pdsch_TypeA_DMRS_r18 | No |  |  |
| 173 | Void |  |  |  |  |  |  |
| 174 | Indicates whether the UE supports two TA enhancement for multi-DCI based intra-cell Multi-TRP operation. A UE supporting this feature shall also indicate support of multiDCI-MultiTRP-r16. | 38.306, 4.2.7.6 | Rel-18 | pc_multiDCI_IntraCellMultiTRP_TwoTA_r18 | No |  |  |
| 175 | Support unified TCI with joint DL/UL LTM TCI-state indication for LTM procedure, indicating and activating a single joint LTM TCI state in a cell switch command | 38.306 4.2.7.2 | Rel-18 | pc_ltm_BeamIndicationJointTCI_r18 | No |  | A UE supporting this feature shall also indicate support of unifiedJointTCI-r17 and at least one of ltm-MCG-IntraFreq-r18 (pc_ltm_MCG_IntraFreq_r18) or ltm-SCG-IntraFreq-r18 (pc_ltm_SCG_IntraFreq_r18). |
| 176 | Support unified TCI with separate DL/UL TCI-state indication for LTM procedure and indicating/activating a pair of UL/DL TCI-state in a cell switch command | 38.306 4.2.7.2 | Rel-18 | pc_ltm_BeamIndicationSeparateTCI_r18 | No |  | A UE supporting this feature shall also indicate support of unifiedSeparateTCI-r17 and at least one of ltm-MCG-IntraFreq-r18 (pc_ltm_MCG_IntraFreq_r18) or ltm-SCG-IntraFreq-r18 (pc_ltm_SCG_IntraFreq_r18). |
| 177 | Support fast processing of LTM candidate cell RRC configuration | 38.306 4.2.7.2 | Rel-18 | pc_ltm_FastProcessingConfig_r18 | No |  | A UE supporting this capability shall also indicate support of ltm-MAC-CE-JointTCI-r18 (pc_ltm_MAC_CE_JointTCI_r18) or ltm-MAC-CE-SeparateTCI-r18 (pc_ltm_MAC_CE_SeparateTCI_r18). |
| 178 | Support MAC-CE activated joint LTM TCI states | 38.306 4.2.7.2 | Rel-18 | pc_ltm_MAC_CE_JointTCI_r18 | No |  | A UE supporting this feature shall also indicate support of ltm-BeamIndicationJointTCI-r18 (pc_ltm_BeamIndicationJointTCI_r18). |
| 179 | Support MAC-CE activated DL/UL LTM TCI states | 38.306 4.2.7.2 | Rel-18 | pc_ltm_MAC_CE_SeparateTCI_r18 | No |  | A UE supporting this feature shall also indicate support of ltm-BeamIndicationSeparateTCI-r18 (pc_ltm_BeamIndicationSeparateTCI_r18). |
| 180 | Support RACH-based early TA acquisition | 38.306 4.2.7.2 | Rel-18 | pc_rach_EarlyTA_Measurement_r18 | No |  | A UE supporting this feature shall also indicate support of ta-IndicationCellSwitch-r18 (pc_ta_IndicationCellSwitch_r18) and at least one of ltm-MCG-IntraFreq-r18 (pc_ltm_MCG_IntraFreq_r18) or ltm-SCG-IntraFreq-r18 (pc_ltm_SCG_IntraFreq_r18). |
| 181 | Support TA indication in cell switch command | 38.306 4.2.7.2 | Rel-18 | pc_ta_IndicationCellSwitch_r18 | No |  | A UE supporting this feature shall also indicate support of at least one of ltm-MCG-IntraFreq-r18 (pc_ltm_MCG_IntraFreq_r18) or ltm-SCG-IntraFreq-r18 (pc_ltm_SCG_IntraFreq_r18). |
| 182 | Support UE-based TA measurement by indicating the maximum number of candidate cells that the UE maintains the TA for | 38.306 4.2.7.2 | Rel-18 | pc_ue_TA_Measurement_r18 | No |  | A UE supporting this feature shall also indicate the support of at least one of ltm-MCG-IntraFreq-r18 (pc_ltm_MCG_IntraFreq_r18) or ltm-SCG-IntraFreq-r18 (pc_ltm_SCG_IntraFreq_r18). |
| 183 | Support PDCCH-ordered RACH transmission for the corresponding band pair indicating whether UE may cause interruption on DL slot(s) on serving cells due to PDCCH-ordered RACH transmission | 38.306 4.2.7.5 | Rel-18 | pc_pdcch_RACH_AffectedBandsList_r18 | No |  | A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18 (pc_rach_EarlyTA_Measurement_r18). |
| 184 | Support PDCCH-ordered RACH transmission for the corresponding band pair indicating the RF/BB preparation time for PDCCH ordered RACH of which the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells | 38.306 4.2.7.5 | Rel-18 | pc_pdcch_RACH_SwitchingTimeList_r18 | No |  | A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18 (pc_rach_EarlyTA_Measurement_r18). |
| 185 | Support PDCCH-ordered RACH transmission for the corresponding band pair indicating the interruption length (Y ms) due to RF re-tuning for PDCCH ordered RACH of which the resources are not fully contained in any of UE's configured UL BWP(s) of active serving cells | 38.306 4.2.7.5 | Rel-18 | pc_pdcch_RACH_PrepTimeList_r18 | No |  | A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18 (pc_rach_EarlyTA_Measurement_r18). |
| 186 | Support simultaneous transmission to handle the overlap between UL transmission on serving cell(s) and PRACH on candidate cell(s) | 38.306 4.2.7.7 | Rel-18 | pc_rach_EarlyTA_BandList_r18 | No |  | A UE supporting this feature shall also indicate support of rach-EarlyTA-Measurement-r18 (pc_rach_EarlyTA_Measurement_r18). |
| 187 | Indicates whether the UE supports cell DTX operation by RRC configuration | 38.306 4.2.7.2 | Rel-18 | pc_nes_CellDTX_r18 | No |  | nes-CellDTX-DRX-r18 is set to the value 'cellDTXonly' or 'both' and UE shall also indicate support of longDRX-Cycle |
| 188 | Indicates whether the UE supports cell DRX operation by RRC configuration | 38.306 4.2.7.2 | Rel-18 | pc_nes_CellDRX_r18 | No |  | nes-CellDTX-DRX-r18 is set to the value 'cellDRXonly' or 'both' |
| 189 | Indicates whether the UE supports cell DTX/DRX configuration activation and deactivation via DCI 2_9 | 38.306 4.2.7.2 | Rel-18 | pc_nes_CellDTX_DRX_DCI2_9_r18 | No |  | A UE supporting this feature shall also indicate support of nes-CellDTX-DRX-r18 |
| 190 | Indicates whether the UE supports R-ML (reduced complexity ML) receivers with enhanced inter-user interference suppression, when co-scheduled UE(s)' modulation order is explicitly signalled by DCI. | 38.306 4.2.7.10 | Rel-18 | pc_advReceiver_MU_MIMO_r18 | No |  |  |
| 191 | Support intra-frequency LTM for MCG with RACH without NR-DC configured | 38.306 4.2.7.2 | Rel-18 | pc_ltm_MCG_IntraFreq_r18 | No |  | A UE supporting this feature shall also indicate support for ltm-BeamIndicationJointTCI-r18 (pc_ltm_BeamIndicationJointTCI_r18) or ltm-BeamIndicationSeparateTCI-r18 (pc_ltm_BeamIndicationSeparateTCI_r18). |
| 192 | Support intra-frequency LTM for SCG with RACH | 38.306 4.2.7.2 | Rel-18 | pc_ltm_SCG_IntraFreq_r18 | No |  | A UE supporting this feature shall also indicate support for ltm-BeamIndicationJointTCI-r18 (pc_ltm_BeamIndicationJointTCI_r18) or ltm-BeamIndicationSeparateTCI-r18 (pc_ltm_BeamIndicationSeparateTCI_r18). |
| 193 | Support of always including the current SpCell in the L1 measurement report | 38.306 4.2.7.5 | Rel-18 | pc_currentSpCellInclL1_Report_r18 | No |  | A UE supporting this feature shall also indicate support of intraFreqL1-MeasConfig-r18 (pc_intraFreqL1_MeasConfig_r18). |
| 194 | Support of inter-frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s), regardless whether the candidate cell(s) are inside or outside of the BC (unless the UE also indicates support of ltm-interFreqL1-OnlyInBC-r18) | 38.306 4.2.7.5 | Rel-18 | pc_interFreqL1_MeasConfig_r18 | No |  | A UE supporting this feature shall also indicate support of intraFreqL1-MeasConfig-r18 (pc_intraFreqL1_MeasConfig_r18). |
| 195 | Support of SSB based inter-frequency L1-RSRP measurements on SSBs within active DL BWP without measurement gaps (without interruption on serving cell(s)) for LTM | 38.306 4.2.7.5 | Rel-18 | pc_interFreqSSB_L1_MeasWithoutGaps_r18 | No |  | A UE supporting this feature shall also indicate support of interFreqL1-MeasConfig-r18 (pc_interFreqL1_MeasConfig_r18). |
| 196 | Support of intra-frequency L1- RSRP measurement and reporting based on SSB(s) of candidate cell(s) | 38.306 4.2.7.5 | Rel-18 | pc_intraFreqL1_MeasConfig_r18 | No |  | A UE supporting this feature shall also indicate support of periodicBeamReport or aperiodicBeamReport or sp-BeamReportPUCCH or sp-BeamReportPUSCH. |
| 197 | Support of reporting the maximum number of frequency layers UE can measure for intra- and inter-frequency without measurement gaps L1-RSRP measurement | 38.306 4.2.7.5 | Rel-18 | pc_supportedMaxIntraInterFreqLayersWithoutGaps_r18 | No |  | A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 (pc_intraFreqL1_MeasConfig_r18) and/or interFreqSSB-L1-MeasWithoutGaps-r18 (pc_interFreqSSB_L1_MeasWithoutGaps_r18). |
| 198 | Support of reporting the maximum number of frequency layers UE can measure for inter-frequency L1-RSRP measurement with measurement gaps | 38.306 4.2.7.5 | Rel-18 | pc_supportedMaxInterFreqLayersWithGaps_r18 | No |  | A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18 (pc_ltm_InterFreqMeasGap_r18). |
| 199 | Support of reporting the max number of neighbour cells UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps | 38.306 4.2.7.5 | Rel-18 | pc_supportedMaxNeighCellsPerFreqLayersWithoutGaps_r18 | No |  | A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 (pc_intraFreqL1_MeasConfig_r18) or interFreqSSB-L1-MeasWithoutGaps-r18 (pc_interFreqSSB_L1_MeasWithoutGaps_r18). |
| 200 | Support of reporting the max number of neighbour cells UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps | 38.306 4.2.7.5 | Rel-18 | pc_supportedMaxNeighCellsPerFreqLayersWithGaps_r18 | No |  | A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18 (pc_ltm_InterFreqMeasGap_r18). |
| 201 | Support of reporting the max number of SSB resources UE can measure for L1-RSRP per frequency layer for intra-frequency or inter-frequency without measurement gaps | 38.306 4.2.7.5 | Rel-18 | pc_supportedMaxSSB_PerFreqLayersWithoutGaps_r18 | No |  | A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 (pc_intraFreqL1_MeasConfig_r18) or interFreqSSB-L1-MeasWithoutGaps-r18 (pc_interFreqSSB_L1_MeasWithoutGaps_r18). |
| 202 | Support of reporting the max number of SSB resources UE can measure for L1-RSRP per frequency layer for inter-frequency with measurement gaps | 38.306 4.2.7.5 | Rel-18 | pc_supportedMaxSSB_PerFreqLayersWithGaps_r18 | No |  | A UE indicating support for this component shall also indicate support for ltm-InterFreqMeasGap-r18 (pc_ltm_InterFreqMeasGap_r18). |
| 203 | Support of simultaneous L1-RSRP measurements for more than one cell when the max RTD among the cells on the same frequency layer or in the same active BWP is larger than CP length of the cell on the frequency layer or in the same active BWP | 38.306 4.2.7.5 | Rel-18 | pc_multiCellL1_measRTD_greaterThan_CP_r18 | No |  | A UE supporting this feature shall also indicate support of either intraFreqL1-MeasConfig-r18 (pc_intraFreqL1_MeasConfig_r18), interFreqSSB-L1-MeasWithoutGaps-r18 (pc_interFreqSSB_L1_MeasWithoutGaps_r18) or ltm-InterFreqMeasGap-r18 (pc_ltm_InterFreqMeasGap_r18). |
| 204 | Support of reporting the max number of total cells of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement. | 38.306 4.2.7.5 | Rel-18 | pc_supportedMaxCellsWithoutGapsL1_Meas_r18 | No |  | A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 (pc_intraFreqL1_MeasConfig_r18) or interFreqSSB-L1-MeasWithoutGaps-r18 (pc_interFreqSSB_L1_MeasWithoutGaps_r18). |
| 205 | Support of reporting the max number of total SSB resources of serving cells and neighboring cells across all frequency layers of intra-frequency and inter-frequency without measurement gaps for L1 measurement | 38.306 4.2.7.5 | Rel-18 | pc_supportedMaxSSB_L1_Meas_r18 | No |  | A UE indicating support for this feature shall also indicate support for intraFreqL1-MeasConfig-r18 (pc_intraFreqL1_MeasConfig_r18) or interFreqSSB-L1-MeasWithoutGaps-r18 (pc_interFreqSSB_L1_MeasWithoutGaps_r18). |
| 206 | Support of reporting the max number of SSB resources for L1-RSRP measurement that UE can measure within a slot across candidate cells for intra- and inter-frequency without gap L1-RSRP measurement | 38.306 4.2.7.5 | Rel-18 | pc_supportedMaxSSB_WithinSlotL1_Meas_r18 | No |  | A UE indicating support for this component shall also indicate support for intraFreqL1-MeasConfig-r18 (pc_intraFreqL1_MeasConfig_r18) or interFreqSSB-L1-MeasWithoutGaps-r18 (pc_interFreqSSB_L1_MeasWithoutGaps_r18). |
| 207 | Indicates whether the UE supports DCI-based enabling/disabling NES-specific CHO execution condition | 38.306 4.2.7.2 | Rel-18 | pc_nesBasedCondHandoverWithDCI_r18 | No |  | A UE supporting this feature shall also indicate the support of condHandover-r16. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. |
| 208 | Indicates whether the UE supports eType-II codebook with refinement for multi-TRP CJT | 38.306 4.2.7.2 | Rel-18 | pc_eType2CJT_r18 | No |  | The UE indicating support of eType2CJT-r18 shall also indicate support of csi-ReportFramework and simultaneousCSI-ReportsAllCC. |
| 209 | Indicates whether the UE supports providing multi-Rx operation preference for FR2 | 38.306 4.2.7.10 | Rel-18 | pc_multiRxPreferenceIndication_r18 | No |  | It is only supported for power class 3. |
| 210 | Indicates whether the UE supports simultaneous reception of CSI-RS for layer 1 measurement and PDSCH with different QCL Type-D on overlapping OFDM symbols and simultaneous layer 1 measurement of CSI-RS overlapping with another CSI-RS with different QCL Type-D on overlapping OFDM symbol(s). | 38.306 4.2.7.6 | Rel-18 | pc_schedulingMeasurementRelaxation_r18 | No |  | A UE supporting this feature shall also indicate support of simultaneousReceptionDiffTypeD-r16, mTRP-GroupBasedL1-RSRP-r17, and at least one of multiDCI-MultiTRP-r16, singleDCI-SDM-scheme-r16, supportFDM-SchemeA-r16 and supportFDM-SchemeB-r16 It can be supported for PC3 only |
| 211 | Indicates whether the UE supports beam sweeping factor reduction for SSB-based layer-1 measurement for activated serving cell when the UE is in multi-Rx operation. | 38.306 4.2.7.2 | Rel-18 | pc_fastBeamSweepingMultiRx_r18 | No |  | It is only supported for power class 3 |
| 212 | Indicates whether the UE supports neighboring LTE cell CRS-IM in DSS scenario with NR 15 kHz SCS | 38.306 4.2.7.6 | Rel-17 | pc_crs_IM_DSS_15kHzSCS_r17 | No |  | FR1 only UE can indicate support of this capability on the CC(s) in a band only if the UE indicates support of rateMatchingLTE-CRS on that band. |
| 213 | Indicates whether the UE supports neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, without the assistance of network signalling on LTE channel bandwidth. | 38.306 4.2.7.6 | Rel-17 | pc_crs_IM_nonDSS_15kHzSCS_r17 | No |  | FR1 only |
| 214 | Indicates whether the UE supports neighboring LTE cell CRS-IM in non-DSS and 15 kHz NR SCS scenario, with the assistance of network signalling on LTE channel bandwidth. | 38.306 4.2.7.6 | Rel-17 | pc_crs_IM_nonDSS_NWA_15kHzSCS_r17 | No |  | FR1 only |
| 215 | Indicates whether the UE supports neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, without the assistance of network signalling on LTE channel bandwidth. | 38.306 4.2.7.6 | Rel-17 | pc_crs_IM_nonDSS_30kHzSCS_r17 | No |  | FR1 only |
| 216 | Indicates whether the UE supports neighboring LTE cell CRS-IM in non-DSS and 30 kHz NR SCS scenario, with the assistance of network signalling on LTE channel bandwidth. | 38.306 4.2.7.6 | Rel-17 | pc_crs_IM_nonDSS_NWA_30kHzSCS_r17 | No |  | FR1 only |
| 217 | Indicates whether the UE supports multiplexing of the unused transmission occasions UCI (UTO-UCI) on a CG-PUSCH. | 38.306 4.2.7.2 | Rel-18 | pc_cg_PUSCH_UTO_UCI_Ind_r18 | No |  | UE supporting this feature shall also indicate support of at least one of configuredUL-GrantType1, configuredUL-GrantType1-v1650, configuredUL-GrantType2, configuredUL-GrantType2-v1650. |
| 218 | Indicates whether the UE supports spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting and single-panel type 1 codebook. | 38.306 4.2.7.2 | Rel-18 | pc_spatialAdaptation_CSI_Feedback_r18 | No |  | A UE reports sdType1 or both. A UE supporting of this feature shall also indicate support of csi-ReportFramework and spatialAdaptation-CSI-FeedbackPerBC-r18. |
| 219 | Indicates whether the UE supports power domain adaptation with CSI feedback based on CSI report sub-configuration(s) for periodic CSI reporting and single-panel type 1 codebook. | 38.306 4.2.7.2 | Rel-18 | pc_powerAdaptation_CSI_FeedbackPUCCH_r18 | No |  | A UE supporting of this feature shall also indicate support of csi-ReportFramework and powerAdaptation-CSI-FeedbackPerBC-r18. |
| 220 | Support of DM-RS bundling for PUSCH over consecutive slots in NGSO scenarios and pre-compensation to keep phase rotation due to timing drift within the phase difference limit. | 38.306 4.2.7 | Rel-18 | pc_ntn_DMRS_BundlingNGSO_r18 | No |  | A UE supporting this feature shall indicate support of uplinkPreCompensation-r17 and at least one of dmrs-BundlingPUSCH-RepTypeA-r17, dmrs-BundlingPUSCH-RepTypeB-r17 or dmrs-BundlingPUSCH-RepTypeC-r17. |
| 221 | Indicates whether the UE supports unified TCI with separate DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC. | 38.306, 4.2.7.2 | Rel-18 | pc_tci_SeparateTCI_UpdateSingleActiveTCI_PerCC_r18 | No |  |  |
| 222 | Indicates whether UE supports increase in maximum output power above the power class indication for inter-band UL CA and NR-DC band combinations as defined in clause 6.2A of TS 38.101-1 [23]. | 38.306 4.2.7.4 | Rel-17 | pc_higherPowerLimit_r17 | No |  | Only applicable to FR1 |
| 223 | Indicates whether UE supports increase in maximum output power above the power class indication for inter-band UL (NG)EN-DC band combinations as defined in clause 6.2B of TS 38.101-3 [25]. | 38.306 4.2.7.9 | Rel-17 | pc_higherPowerLimitMRDC_r17 | No |  | Only applicable to FR1 |
| 224 | Indicates the maximal supported HARQ process numbers for UL and for DL respectively. For each value of max-HARQ-ProcessNumber-r17, value u16d32 indicates the maximal supported HARQ process number is 16 for UL and 32 for DL, value u32d16 indicates the maximal supported HARQ process number is 32 for UL and 16 for DL, value u32d32 indicates the maximal supported HARQ process number is 32 for UL and 32 for DL | 38.306 4.2.7.2 | Rel-17 | pc_max_HARQ_ProcessNumber_r17 | No |  | This field is only applicable for NR-NTN bands and HAPS operation bands |
| 225 | Void |  |  |  |  |  |  |
| 226 | Indicates whether the UE supports simultaneous reception with different QCL Type D reference signal | 38.306, 4.2.7.2 | Rel-16 | pc_simultaneousReceptionDiffTypeD_r16 | No |  | FR2 only |
| 227 | Indicates whether the UE supports the group based L1-RSRP reporting enhancements. | 38.306, 4.2.7.2 | Rel-17 | pc_mTRP_GroupBasedL1_RSRP_r17 | No |  |  |
| 228 | Indicates the maximal number of PDSCH scrambling sequences per serving cell when the UE supports PDSCHs with fully overlapping Resource Elements. The UE that indicates support of this feature shall support multiDCI-MultiTRP-r16 | 38.306 4.2.7.2 | Rel-16 | pc_overlapPDSCHsFullyFreqTime_r16 | No |  | Note: A UE may assume that its maximum receive timing difference between the DL transmissions from two TRPs is within a Cyclic Prefix |
| 229 | Indicates whether the UE supports PDSCHs with partially overlapping Resource Elements | 38.306 4.2.7.2 | Rel-16 | pc_overlapPDSCHsInTimePartiallyFreq_r16 | No |  | The UE that indicates support of this feature shall support overlapPDSCHsFullyFreqTime-r16 |
| 230 | Indicates whether the UE supports up to 12 configured/active configured grant configurations in a BWP of a serving cell. | 38.306 4.2.7.2 | Rel-16 | pc_activeConfiguredGrant_r16 | No |  | A UE supporting this feature shall also indicate support of either configuredUL-GrantType1 or configuredUL-GrantType1-v1650 or configuredUL-GrantType2 or configuredUL-GrantType2-v1650. |
| 231 | Indicates whether the UE supports SCell without SS/PBCH block for inter-band CA. | 38.306 4.2.7.5 | Rel-18 | pc_scellWithoutSSB_InterBandCA_r18 | No |  | FR1 only. For supportOfSingleGroup, UE should indicate ‘both’ for two bands or indicate 'referenceBand' for one band and indicate 'scellWithoutSSB' for the other band. |
| 232 | Indicates the UE support of the maximum number of SRS resources in one SRS resource set with usage set to 'codebook' for uplink full power Mode 2 operation. | 38.306 4.2.7.7 | Rel-16 | pc_ul_FullPwrMode2_MaxSRS_ResInSet_r16 | No |  |  |
| 233 | Indicates the UE supported TPMI group(s) which delivers full power | 38.306 4.2.7.7 | Rel-16 | pc_ul_FullPwrMode2_TPMIGroup_r16 | No |  | UE indicates support of this feature shall also indicate support of ul-FullPwrMode2-MaxSRS-ResInSet. |
| 234 | Support of PUCCH Msg4 HARQ-ACK repetition | 38.306 5.4 | Rel-18 | pc_msg4_harq_Repetition_r18 | No |  |  |
| 235 | Support reporting of information related to TA pre-compensation for ATG | 38.306 4.2.7.10 | Rel-18 | pc_uplinkTA_ReportingATG_r18 | No |  | The UE indicating support of this feature shall also indicate support of uplinkPreCompensationATG-r18 |
| 236 | Indicates whether the UE supports spatial domain adaptation with CSI feedback based on CSI report sub-configuration(s) for aperiodic CSI reporting and single-panel type 1 codebook | 38.306 4.2.7.2 | Rel-18 | pc_spatialAdaptation_CSI_FeedbackAperiodic_r18 | No |  | A UE reports sdType2 or both. A UE indicating support of this feature shall also indicate support of csi-ReportFramework and spatialAdaptation-CSI-FeedbackAperiodicPerBC-r18 |
| 237 | Support of ΔPPowerClass /ΔPPowerClass, CA/ΔPPowerClass, EN-DC/ΔPPowerClass, NR-DC reporting which is triggered upon uplink duty cycle exceedance or upon return to the power class after the duty cycle exceedance, as specified in TS 38.101-1 [2] and TS 38.101-3 [4] | 38.306 4.2.7.10 | Rel-18 | pc_deltaPowerClassReporting_r18 | No |  |  |
| 238 | Support reporting of power headroom information for an assumed PUSCH using target waveform different from waveform of actual PUSCH | 38.306 4.2.7.2 | Rel-18 | pc_dynamicWaveformSwitchPHR_r18 | No |  |  |
| 239 | Indicates whether the UE supports co-scheduled cell indication scheme based on Scheduled cells indicator field of DCI format 0_3 | 38.306, 4.2.7.4 38.331 6.3.3 | Rel-18 | pc_coScheduledCellIndicationScheme_r18_SchedCellInd_DCI_0_3 | No |  |  |
| 240 | Indicates whether the UE supports unified TCI with joint DL/UL TCI update for multi-DCI based multi-TRP with single activated TCI codepoint per CORESETPoolIndex per CC | 38.306, 4.2.7.2 | Rel-18 | pc_tci_JointTCI_UpdateSingleActiveTCI_PerCC_PerCORESET_r18 | No |  | A UE supporting this feature shall also indicate support of unifiedJointTCI-r17 |
| 241 | Indicates whether the UE supports Unified TCI with joint DL/UL TCI update for single-DCI based intra-cell multi-TRP with single activated TCI codepoint per CC | 38.306, 4.2.7.2 | Rel-18 | pc_tci_JointTCI_UpdateSingleActiveTCI_PerCC_r18 | No |  | A UE supporting this feature shall also indicate support of unifiedJointTCI-r17 |
| 242 | Support of type I codebook with at least 16 ports per CSI-RS resource | 38.306, 4.2.7.2 | Rel-15 | pc_typeI_codebook_with_16ports | No |  | Applicable for UE support at least 16 ports per CSI-RS resource based on 38.306 IE maxNumberTxPortsPerResource under CodebookParameters Type1 |
| 243 | Support of type I codebook with at least 32 ports per CSI-RS resource | 38.306, 4.2.7.2 | Rel-15 | pc_typeI_codebook_with_32ports | No |  | Applicable for UE support at least 32 ports per CSI-RS resource based on 38.306 IE maxNumberTxPortsPerResource under CodebookParameters Type1 |
| 244 | Supports dynamic waveform switching for DCI format 0_1/0_2 when configured with only 1 UL carrier in the band | 38.306 4.2.7.2 | Rel-18 | pc_dynamicWaveformSwitch_r18 | No |  |  |
| 245 | Support of measurement validation and report based on EMR measurement during connection setup/resume for fast CA/DC setup | 38.306 4.2.7.2 | Rel-18 | pc_measValidationReportEMR_r18 | No |  |  |
| 246 | Support of measurement validation based on reselection measurements during IDLE/INACTIVE state and reporting for fast CA/DC setup | 38.306 4.2.7.2 | Rel-18 | Pc_measValidationReportReselectionMeasurements_r18 | No |  |  |
| 247 | Support of the Rx timing difference between the two DL reference timings is larger than CP length. | 38.306 4.2.7.6 | Rel-18 | pc_rxTimingDiff_r18 | No |  |  |
| 248 | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based SSSG switching, if pdcch-SkippingDurationList is not configured as specified in TS 38.213 [30], clause 10.4 | 38.306, 4.2.7.2 | Rel-17 | pc_sssg_Switching_1BitInd_r17 | No |  |  |
| 249 | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based SSSG switching, if pdcch-SkippingDurationList is not configured as specified in TS 38.213 [30], clause 10.4 | 38.306, 4.2.7.2 | Rel-17 | pc_sssg_Switching_2BitInd_r17 | No |  | UE indicating support of this feature shall also indicate support of sssg-Switching-1bitInd-r17. |
| 250 | Support of search space set group switching capability 2 for FR1 according to Table 10.4-1 of TS 38.213 [30] for SSSG switching | 38.306, 4.2.7.2 | Rel-17 | pc_searchSpaceSetGrp_switchCap2_r17 | No |  | FR1 only UE indicating support of this feature shall also indicate support of sssg-Switching-1bitInd-r17. |
| 251 | Support of RSRP reporting for a group of two reference signals | 38.306, 4.2.7.2 | Rel-15 | pc_groupBeamReporting | No |  |  |

<< Skipped sections >>

### A.4.3.6	Measurement Capabilities

Table A.4.3.6-1: UE Measurement Capabilities

| Item | UE Measurement Capabilities | Ref. | Release | Mnemonic | M | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release | Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Support NR measurements and events A triggered reporting | 38.306, 4.2.9 | Rel-15 | pc_eventA_MeasAndReport | Yes |  |  |
| 2 | Support two independent measurement gap configurations for FR1 and FR2 | 38.306, 4.2.9 | Rel-15 | pc_independentGapConfig | No |  |  |
| 2A | Support two independent measurement gap configurations between LTE and NR for EN-DC | 38.306, 4.2.9a | Rel-15 | pc_independentGapConfig_ENDC | No |  |  |
| 2B | Support two independent measurement gap configurations for NR-DC | 38.306, 4.2.9a | Rel-15 | pc_independentGapConfig_NRDC | No |  |  |
| 3 | Support NR intra-frequency and inter-frequency measurements and at least periodical reporting | 38.306, 4.2.9 | Rel-15 | pc_intraAndInterF_MeasAndReport | Yes |  |  |
| 4 | Support CSI-RSRP and CSI-RSRQ measurement as specified in TS38.215 [21], where CSI-RS resource is configured with an associated SS/PBCH | 38.306, 4.2.9 | Rel-15 | pc_csi_RSRP_AndRSRQ_MeasWithSSB | No |  |  |
| 5 | Support inter-RAT E-UTRA measurements and events B triggered reporting | 38.306, 4.2.9 | Rel-15 | pc_eventB_MeasAndReport | Yes |  |  |
| 6 | Support SS-SINR measurents | 38.306, 4.2.9 | Rel-15 | pc_ss_SINR_Meas | No |  |  |
| 6a | Support CSI-SINR measurents | 38.306, 4.2.9 | Rel-15 | pc_csi_SINR_Meas | No |  |  |
| 7 | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is not configured. | 38.306, 4.2.9 | Rel-15 | pc_eutra_CGI_Reporting | Yes |  |  |
| 7a | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell using autonomous gap and reporting the acquired information to the network as specified in TS 38.331 [9] when MR-DC is not configured | 38.306,4.2.9 | Rel-16 | pc_eutra_AutonomousGaps_r16 | No |  |  |
| 8 | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when EN-DC is not configured. | 38.306, 4.2.9 | Rel-15 | pc_nr_CGI_Reporting | Yes |  |  |
| 8a | Support acquisition of relevant information from a neighbouring NR cell by reading the SI of the neighbouring cell using autonomous gap and reporting the acquired information to the network as specified in TS 38.331 [9] when MR-DC is not configured. | 38.306, 4.2.9 | Rel-16 | pc_nr_AutonomousGaps_r16 | No |  |  |
| 9 | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is configured. | 38.306, 4.2.9 | Rel-15 | pc_nr_CGI_Reporting_ENDC | Yes |  |  |
| 10 | Support shorter measurement gap length (i.e. gp2 and gp3) for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC. | 36.331, 6.3.6 | Rel-15 | pc_gp2_gp3_en_dc | No |  |  |
| 11 | Support NR supports gap pattern 4 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC | 36.331, 6.3.6 | Rel-15 | pc_gp4_en_dc | No |  |  |
| 12 | Support NR supports gap pattern 5 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC | 36.331, 6.3.6 | Rel-15 | pc_gp5_en_dc | No |  |  |
| 13 | Support NR supports gap pattern 6 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC | 36.331, 6.3.6 | Rel-15 | pc_gp6_en_dc | No |  |  |
| 14 | Support NR supports gap pattern 7 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC | 36.331, 6.3.6 | Rel-15 | pc_gp7_en_dc | No |  |  |
| 15 | Support NR supports gap pattern 8 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC | 36.331, 6.3.6 | Rel-15 | pc_gp8_en_dc | No |  |  |
| 16 | Support NR supports gap pattern 9 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC | 36.331, 6.3.6 | Rel-15 | pc_gp9_en_dc | No |  |  |
| 17 | Support NR supports gap pattern 10 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC | 36.331, 6.3.6 | Rel-15 | pc_gp10_en_dc | No |  |  |
| 18 | Support NR supports gap pattern 11 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC | 36.331, 6.3.6 | Rel-15 | pc_gp11_en_dc | No |  |  |
| 19 | Support measurement gap pattern 2 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp2_nr | No |  |  |
| 20 | Support measurement gap pattern 3 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp3_nr | No |  |  |
| 21 | Support measurement gap pattern 4 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp4_nr | No |  |  |
| 22 | Support measurement gap pattern 5 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp5_nr | No |  |  |
| 23 | Support measurement gap pattern 6 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp6_nr | No |  |  |
| 24 | Support measurement gap pattern 7 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp7_nr | No |  |  |
| 25 | Support measurement gap pattern 8 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp8_nr | No |  |  |
| 26 | Support measurement gap pattern 9 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp9_nr | No |  |  |
| 27 | Support measurement gap pattern 10 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp10_nr | No |  |  |
| 28 | Support measurement gap pattern 11 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp11_nr | No |  |  |
| 29 | Support measurement gap pattern 12 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp12_nr | No |  |  |
| 30 | Support measurement gap pattern 15 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp15_nr | No |  |  |
| 31 | Support measurement gap pattern 16 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp16_nr | No |  |  |
| 32 | Support measurement gap pattern 17 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp17_nr | No |  | This feature shall be supported if the UE is an NR standalone capable UE that supports a band in FR2, or if the UE is an (NG)EN-DC capable UE that supports independentGapConfig and supports a band in FR2 |
| 34 | Support measurement gap pattern 18 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp18_nr | No |  | This feature shall be supported if the UE is an NR standalone capable UE that supports a band in FR2, or if the UE is an (NG)EN-DC capable UE that supports independentGapConfig and supports a band in FR2 |
| 35 | Support measurement gap pattern 19 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp19_nr | No |  | This feature shall be supported if the UE is an NR standalone capable UE that supports a band in FR2, or if the UE is an (NG)EN-DC capable UE that supports independentGapConfig and supports a band in FR2 |
| 36 | Support measurement gap pattern 20 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp20_nr | No |  |  |
| 37 | Support measurement gap pattern 21 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp21_nr | No |  |  |
| 38 | Support measurement gap pattern 22 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp22_nr | No |  |  |
| 39 | Support measurement gap pattern 23 configured by NR RRC. | 38.306, 4.2.9 | Rel-15 | pc_gp23_nr | No |  |  |
| 40 | Support CSI-RSRP and CSI-RSRQ measurement as specified in TS38.215 [21], where CSI-RS resource is configured without an associated SS/PBCH | 38.306, 4.2.9 | Rel-15 | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB | No |  |  |
| 41 | Support CSI-RS based Radio Link Monitoring for FR1 | 38.306, 4.2.9 | Rel-15 | pc_CSI_RS_RLM_FR1 | Yes |  | If the UE supports this feature, the UE needs to report maxNumberResource-CSI-RS-RLM in its capability report. If the UE doesn’t support CSI-RS based RLM, it will not include this IE in its capability report. |
| 41a | Support CSI-RS based Radio Link Monitoring for FR2 | 38.306, 4.2.9 | Rel-15 | pc_CSI_RS_RLM_FR2 | Yes |  | If the UE supports this feature, the UE needs to report maxNumberResource-CSI-RS-RLM in its capability report. If the UE doesn’t support CSI-RS based RLM, it will not include this IE in its capability report. |
| 42 | Support of E-UTRA RS-SINR measurements | 38.306, 4.2.10 | Rel-15 | pc_RS_SINR_MeasEUTRA | No |  |  |
| 43 | Support of SFTD measurements between a E-UTRA PCell and an NR PSCell in FDD | 38.306, 4.2.9 | Rel-15 | pc_SFTD_MeasPSCell_MRDC_FDD | No |  | The SFTD measurement support should be indicated in MRDC capabilities for EN-DC. The support needs to be declared for FDD and TDD separately |
| 44 | Support of SFTD measurements between a E-UTRA PCell and an NR PSCell in TDD | 38.306, 4.2.9 | Rel-15 | pc_SFTD_MeasPSCell_MRDC_TDD | No |  | The SFTD measurement support should be indicated in MRDC capabilities for EN-DC. The support needs to be declared for FDD and TDD separately |
| 45 | Support of relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE | 38.306, 5.6 | Rel-16 | pc_Relaxed_Measurement | No |  |  |
| 46 | Support of SFTD measurements between a E-UTRA PCell and an NR neighbour cell in FDD | 38.306, 4.2.9 | Rel-15 | pc_SFTD_MeasNR_Cell_FDD | No |  | The support needs to be declared for FDD and TDD separately The SFTD measurement support can only be indicated in MRDC capabilities for EN-DC |
| 47 | Support of SFTD measurements between a E-UTRA PCell and an NR neighbour cell in TDD | 38.306, 4.2.9 | Rel-15 | pc_SFTD_MeasNR_Cell_TDD | No |  | The support needs to be declared for FDD and TDD separately The SFTD measurement support can only be indicated in MRDC capabilities for EN-DC |
| 48 | Support of SFTD measurements between a NR PCell and an NR neighbour cell in FDD | 38.306, 4.2.9 | Rel-15 | pc_SFTD_MeasNR_Neigh_FDD | No |  | The support needs to be declared for FDD and TDD separately |
| 49 | Support of SFTD measurements between a NR PCell and an NR neighbour cell in TDD | 38.306, 4.2.9 | Rel-15 | pc_SFTD_MeasNR_Neigh_TDD | No |  | The support needs to be declared for FDD and TDD separately |
| 50 | Support of SFTD measurements between a NR PCell and an NR PSCell in FDD | 38.306, 4.2.9 | Rel-15 | pc_SFTD_MeasPSCell_NRDC_FDD | No |  | The SFTD measurement support should be indicated in UE-NR-Capability |
| 51 | Support of SFTD measurements between a NR PCell and an NR PSCell in TDD | 38.306, 4.2.9 | Rel-15 | pc_SFTD_MeasPSCell_NRDC_TDD | No |  | The SFTD measurement support should be indicated in UE-NR-Capability |
| 52 | Support of acquisition of CGI related information from a neighbouring intra-frequency or inter-frequency NPN CAG cell | 38.306, 4.2.9 | Rel-16 | pc_nr_CGI_Reporting_NPN_r16 | No |  |  |
| 53 | Supports periodic EUTRA measurement and reporting. | 38.306, 4.2.9 | Rel-15 | pc_periodicEUTRA_MeasAndReport | Yes |  |  |
| 54 | Support configuration of NR SSB measurements in RRC_IDLE/RRC_INACTIVE and reporting of the corresponding results upon network request as specified in TS 38.331 [9] | 38.306, 4.2.9 | Rel-16 | pc_idleInactiveNR_MeasReport | No |  |  |
| 55 | Support configuration of E-UTRA measurements in RRC_IDLE/RRC_INACTIVE and reporting of the corresponding results upon network request as specified in TS 38.331 [9] | 38.306, 4.2.9 | Rel-16 | pc_idleInactiveEUTRA_MeasReport | No |  |  |
| 56 | Support SRS-RSRP measurements between a NR Pcell and an interfering UE, upon network request as specified in 38.331 [9] | 38.306, 4.2.9 | Rel-16 | pc_nr_CLI_Reporting_r16 | No |  | If the UE supports this feature, the UE needs to report maxNumberCLI-SRS-RSRP-r16 and maxNumberPerSlotCLI-SRS-RSRP-r16 If the UE doesn’t support CLI SRS-RSRP measurement, it will not include this IE in its capability report. |
| 56A | Support SRS-RSRP measurements and periodical reporting and measurement event triggering based on SRS-RSRP | 38.306, 4.2.9 | Rel-16 | pc_cli_SRS_RSRP_Meas_r16 | No |  | If the UE supports this feature, the UE needs to report maxNumberCLI-SRS-RSRP-r16 and maxNumberPerSlotCLI-SRS-RSRP-r16 |
| 56B | Support CLI RSSI measurements and periodical reporting and measurement event triggering | 38.306, 4.2.9 | Rel-16 | pc_cli_RSSI_Meas_r16 | No |  | If the UE supports this feature, the UE needs to report maxNumberCLI-RSSI-r16 |
| 57 | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NE-DC is configured. | 38.306, 4.2.9 | Rel-15 | pc_eutra_CGI_Reporting_NEDC | No |  |  |
| 58 | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NR-DC is configured wherein MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if the DRX cycles are the same. | 38.306, 4.2.9 | Rel-15 | pc_eutra_CGI_Reporting_NRDC | No |  |  |
| 59 | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NE-DC is configured. | 38.306, 4.2.9 | Rel-15 | pc_nr_CGI_Reporting_NEDC | Yes |  |  |
| 60 | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the NR-DC is configured wherein MN and SN have different DRX cycles, or on-duration configured by MN does not contain on-duration configured by SN if the DRX cycles are the same. | 38.306, 4.2.9 | Rel-15 | pc_nr_CGI_Reporting_NRDC | Yes |  |  |
| 61 | Supports performing eNB-configured SSB-based RRM measurements for EN-DC configured NR FR1 carrier(s) in RRC_IDLE and reporting them when requested by the eNB while resuming from RRC_IDLE or in RRC_CONNECTED, as specified in TS 36.331 [5]. | 36.306, 4.3.6.41 | Rel-16 | pc_nrIdleInactiveNRFR1_MeasReport | No |  |  |
| 62 | Supports performing eNB-configured SSB-based RRM measurements for EN-DC configured NR FR2 carrier(s) in RRC_IDLE and reporting them when requested by the eNB while resuming from RRC_IDLE or in RRC_CONNECTED, as specified in TS 36.331 [5]. | 36.306, 4.3.6.42 | Rel-16 | pc_nrIdleInactiveNRFR2_MeasReport | No |  |  |
| 63 | Support more than 1 per-UE measurement gap configurations. | 38.306, 4.2.9 | Rel-17 | pc_concurrentPerUE_OnlyMeasGap_r17 | No |  |  |
| 64 | Support all concurrent gap combination configurations as specified in TS 38.133 [5] including support of more than 1 per-UE measurement gap configurations. | 38.306, 4.2.9 | Rel-17 | pc_concurrentPerUE_PerFRCombMeasGap_r17 | No |  |  |
| 65 | Support the configurations of E-UTRAN measurement objectives associated with more than 1 concurrent measurement gaps. | 38.306, 4.2.9 | Rel-17 | pc_concurrentMeasGapEUTRA_r17 | No |  |  |
| 66 | Support reporting of the NCSG and measurement gap requirement information for E-UTRA target bands in the UE response to a network configuration RRC message as specified in TS 38.331 [9]. | 38.306, 4.2.9 | Rel-17 | pc_eutra_NeedForGapNCSG_Reporting_r17 | No |  |  |
| 67 | Support two independent measurement gap configurations for FR1 and FR2 for PRS measurement. | 38.306, 4.2.9 | Rel-17 | pc_independentGapConfigPRS_r17 | No |  |  |
| 68 | Support NR-only NCSG patterns. | 38.306, 4.2.9 | Rel-17 | pc_ncsg_MeasGapNR_Patterns_r17 | No |  |  |
| 69 | Support NCSG patterns. | 38.306, 4.2.9 | Rel-17 | pc_ncsg_MeasGapPatterns_r17 | No |  |  |
| 70 | Support per-FR NCSG. | 38.306, 4.2.9 | Rel-17 | pc_ncsg_MeasGapPerFR_r17 | No |  |  |
| 71 | Support performing measurement with NCSG based on flag deriveSSB-IndexFromCell-inter and meeting the following requirements that the scheduling restriction in FR2 serving cell during NCSG ML is on SSB symbol level. | 38.306, 4.2.9 | Rel-17 | pc_ncsg_SymbolLevelScheduleRestrictionInter_r17 | No |  |  |
| 72 | Support reporting of the NCSG and measurement gap requirement information for SSB based measurement in the UE response to a network configuration RRC message. | 38.306, 4.2.9 | Rel-17 | pc_nr_NeedForGapNCSG_Reporting_r17 | No |  |  |
| 73 | Support the preconfigured measurement gap with UE-autonomous mechanism for activation and deactivation. | 38.306, 4.2.9 | Rel-17 | pc_preconfiguredUE_AutonomousMeasGap_r17 | No |  |  |
| 74 | Support the preconfigured measurement gap with network-controlled mechanism for activation and deactivation. | 38.306, 4.2.9 | Rel-17 | pc_preconfiguredNW_ControlledMeasGap_r17 | No |  |  |
| 75 | Support of SFTD measurements between the NR PCell and a configured E-UTRA PSCell | 38.306, 4.2.9 | Rel-15 | pc_SFTD_MeasPSCell_NEDC | No |  |  |
| 76 | Support of location-based triggered measurement reporting (i.e., event D1) as specified in TS 38.331 [9]. | 38.306, 4.2.9 | Rel-17 | pc_eventD1_MeasReportTrigger_r17 | CY |  | If UE supports locationBasedCondHandover-r17 in any NTN band then UE needs to support this feature. |
| 77 | Void |  |  |  |  |  |  |
| 78 | Support of Rel-17 BFD relaxation criteria | 38.306, 4.2.7.2 | Rel-17 | pc_bfd_Relaxation_r17 | No |  |  |
| 79 | Support of Rel-17 RLM relaxation criteria | 38.306, 4.2.7.2 | Rel-17 | pc_rlm_Relaxation_r17 | No |  |  |
| 80 | Indicates the support of simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA. If this field is included in ca-ParametersNR-ForDC, it indicates the UE supports simultaneous transmission and reception between any UL/DL band pair within a cell group and across MCG and SCG in TDD-TDD and TDD-FDD inter-band NR-DC. | 38.306, 4.2.7 | Rel-15 | pc_simultaneous_RxTx_InterBandCA | No |  | If the capability is supported then the band pair(s) for which it is supported shall be indicated in Table A.4.3.2A.4.1-3 |
| 81 | Indicates whether the UE can perform inter-frequency SSB based measurements without measurement gaps if the SSB is completely contained in the active BWP of the UE as specified in TS 38.133 [5]. If this parameter is indicated for FR1 and FR2 differently, each indication corresponds to the frequency range of cells to be measured. | 38.306, 4.2.9 | Rel-16 | pc_interFrequencyMeas_NoGap_r16 | No |  |  |
| 82 | Indicates whether the UE supports 2 parallel measurement gaps for NTN SSB based RRM measurements | 38.306, 4.2.9 | Rel-17 | pc_parallelMeasGap_r17 | No |  | If a UE supports this feature, then it supports r17 NTN |
| 83 | Indicates whether the UE supports Time-based measurement initiation for NTN while in RRC_IDLE/RRC_INACTIVE | 38.306, 5.6 | Rel-17 | pc_timeBasedMeasInit_r17 | No |  | If a UE supports this feature, then it supports r17 NTN |
| 84 | Indicates whether the UE supports Location-based measurement initiation for NTN while in RRC_IDLE/RRC_INACTIVE | 38.306, 5.6 | Rel-17 | pc_locationBasedMeasInit_r17 | No |  | If a UE supports this feature, then it supports r17 NTN |
| 85 | Indicates whether the UE supports enhanced RRM requirements for measurements in NTN bands while in RRC_IDLE/RRC_INACTIVE | 38.306, 5.6 | Rel-17 | pc_enhRRMreqMeas_r17 | No |  | If a UE supports this feature, then it supports r17 NTN |
| 86 | Void |  |  |  |  |  |  |
| 87 | Indicates whether the UE supports measurement of 1 LEO satellite in parallel within an SMTC for NTN (maxNumber-NGSO-SatellitesWithinOneSMTC-r17) | 38.306, 4.2.7.2 | Rel-17 | pc_maxNumNGSOsatPerSMTC_1_r17 | No |  | If a UE supports this feature, then it supports r17 NTN |
| 88 | Indicates whether the UE supports measurement of 2 LEO satellite in parallel within an SMTC for NTN (maxNumber-NGSO-SatellitesWithinOneSMTC-r17) | 38.306, 4.2.7.2 | Rel-17 | pc_maxNumNGSOsatPerSMTC_2_r17 | No |  | If a UE supports this feature, then it supports r17 NTN |
| 89 | Indicates whether the UE supports measurement of 3 LEO satellite in parallel within an SMTC for NTN (maxNumber-NGSO-SatellitesWithinOneSMTC-r17) | 38.306, 4.2.7.2 | Rel-17 | pc_maxNumNGSOsatPerSMTC_3_r17 | No |  | If a UE supports this feature, then it supports r17 NTN |
| 90 | Indicates whether the UE supports measurement of 4 LEO satellite in parallel within an SMTC for NTN (maxNumber-NGSO-SatellitesWithinOneSMTC-r17) | 38.306, 4.2.7.2 | Rel-17 | pc_maxNumNGSOsatPerSMTC_4_r17 | No |  | If a UE supports this feature, then it supports r17 NTN |
| 91 | Indicates whether the UE supports inter-satellite measurements for NTN (interSatMeas-r17) | 38.306, 4.2.9 | Rel-17 | pc_interSatMeas_r17 | No |  | If a UE supports this feature, then it supports r17 NTN |
| 92 | Support of measurement gap pattern 13 configured by NR RRC | 38.306, 4.2.9 | Rel-15 | pc_gp13_nr | No |  | This feature shall be supported if the UE is an NR standalone capable UE that supports a band in FR2, or if the UE is an (NG)EN-DC capable UE that supports independentGapConfig and supports a band in FR2 |
| 93 | Support of measurement gap pattern 14 configured by NR RRC | 38.306, 4.2.9 | Rel-15 | pc_gp14_nr | No |  | This feature shall be supported if the UE is an NR standalone capable UE that supports a band in FR2, or if the UE is an (NG)EN-DC capable UE that supports independentGapConfig and supports a band in FR2 |
| 94 | Support reduced TLTM_processing for cell switch from FR1 to FR1 | 38.306, 4.2.9 | Rel-18 | pc_ltm_FastUE_Processing_fr1_r18 | No |  |  |
| 95 | Support reduced TLTM_processing for cell switch from FR2 to FR2 | 38.306, 4.2.9 | Rel-18 | pc_ltm_FastUE_Processing_fr2_r18 | No |  |  |
| 96 | Support reduced TLTM_processing for cell switch from FR1/FR2 to FR2/FR1 | 38.306, 4.2.9 | Rel-18 | pc_ltm_FastUE_Processing_fr1_AndFR2_r18 | No |  |  |
| 97 | Support SSB based inter-frequency L1-RSRP measurements with measurement gaps for LTM | 38.306, 4.2.9 | Rel-18 | pc_ltm_InterFreqMeasGap_r18 | No |  | A UE supporting this feature shall also indicate support of interFreqL1-MeasConfig-r18 (pc_interFreqL1_MeasConfig_r18). |
| 98 | Support LTM for MCG with RACH with NR-DC configured | 38.306, 4.2.9 | Rel-18 | pc_ltm_MCG_NRDC_r18 | No |  | A UE indicating support for this feature shall also indicate support of ltm-MCG-IntraFreq-r18 (pc_ltm_MCG_IntraFreq_r18). |
| 99 | Support LTM for MCG with the release of NR-DC configuration as part of LTM execution when LTM cell switch command MAC CE is received | 38.306, 4.2.9 | Rel-18 | pc_ltm_MCG_NRDC_Release_r18 | No |  | A UE indicating support for this feature shall also indicate support of ltm-MCG-IntraFreq-r18 (pc_ltm_MCG_IntraFreq_r18). |
| 100 | Support inter-frequency MCG LTM on all the bands where the UE indicates support of ltm-MCG-IntraFreq-r18 or inter-frequency SCG LTM on all the bands where the UE indicates support of ltm-SCG-IntraFreq-r18 respectively | 38.306, 4.2.9 | Rel-18 | pc_ltm_InterFreq_r18 | No |  | A UE supporting this feature shall also indicate support of ltm-MCG-IntraFreq-r18 (pc_ltm_MCG_IntraFreq_r18) or ltm-SCG-IntraFreq-r18 (pc_ltm_SCG_IntraFreq_r18). |
| 100A | Support of inter-frequency L1-RSRP measurement and reporting based on SSB(s) of LTM candidate cell(s) that are inside the BC only in which the UE indicates support of interFreqL1-MeasConfig-r18 | 38.306, 4.2.9 | Rel-18 | pc_ltm_interFreqL1_OnlyInBC_r18 | No |  | A UE supporting this feature shall also indicate support of interFreqL1-MeasConfig-r18. (pc_interFreqL1_MeasConfig_r18). |
| 101 | Support RACH-less LTM with configured grant for MCG LTM if the UE indicates support of ltm-MCG-IntraFreq-r18 or for SCG LTM if the UE indicates support of ltm-SCG-IntraFreq-r18 respectively | 38.306, 4.2.9 | Rel-18 | pc_ltm_RACH_LessCG_r18 | No |  | A UE indicating support for this feature shall also indicate support of either ltm-BeamIndicationJointTCI-r18 (pc_ltm_BeamIndicationJointTCI_r18) or ltm-BeamIndicationSeparateTCI-r18 (pc_ltm_BeamIndicationSeparateTCI_r18) for at least one band and either ta-IndicationCellSwitch-r18 (pc_ta_IndicationCellSwitch_r18) or ue-TA-Measurement-r18 (pc_ue_TA_Measurement_r18). |
| 102 | Support RACH-Less LTM with dynamic grant, for MCG LTM if the UE indicates support of ltm-MCG-IntraFreq-r18 or for SCG LTM if the UE indicates support of ltm-SCG-IntraFreq-r18 respectively | 38.306, 4.2.9 | Rel-18 | pc_ltm_RACH_LessDG_r18 | No |  | A UE indicating support for this feature shall also indicate support of either ltm-BeamIndicationJointTCI-r18 (pc_ltm_BeamIndicationJointTCI_r18) or ltm-BeamIndicationSeparateTCI-r18 (pc_ltm_BeamIndicationSeparateTCI_r18) for at least one band and TA indication in ta-IndicationCellSwitch-r18 (pc_ta_IndicationCellSwitch_r18) or ue-TA-Measurement-r18 (pc_ue_TA_Measurement_r18). |
| 103 | Support recovery procedure for MCG LTM execution when the selected cell in RRC re-establishment procedure is a LTM candidate | 38.306, 4.2.9 | Rel-18 | pc_ltm_Recovery_r18 | No |  | A UE indicating support for this feature shall also indicate support of ltm-MCG-IntraFreq-r18 (pc_ltm_MCG_IntraFreq_r18) for at least one band. |
| 104 | Support a reference configuration for LTM | 38.306, 4.2.9 | Rel-18 | pc_ltm_ReferenceConfig_r18 | No |  | A UE indicating support for this feature shall also indicate support of either ltm-MCG-IntraFreq-r18 (pc_ltm_MCG_IntraFreq_r18) or ltm-SCG-IntraFreq-r18 (pc_ltm_SCG_IntraFreq_r18) for at least one band. |
| 105 | Support reporting the measurement gap requirement information for NR target in the UE response to a network configuration RRC message. | 38.306, 4.2.9 | Rel-16 | pc_nr_NeedForGap_Reporting_r16 | No |  |  |
| 105A | Support performing SSB based inter-RAT measurements on each supported NR band without measurement gaps | 36.306, 4.3.6.38 | Rel-16 | pc_interRAT_NeedForGapsNR_r16 | No |  |  |
| 106 | Support reporting the interruption requirement information for SSB based measurement towards NR target without gap in the UE response to a network configuration RRC message | 38.306, 4.2.9 | Rel-18 | pc_nr_NeedForInterruptionReport_r18 | No |  | A UE supporting this feature shall also indicate support of nr-NeedForGap-Reporting-r16 (pc_nr_NeedForGap_Reporting_r16). |
| 107 | Support concurrent inter-RAT measurement on EUTRAN cell in non-DSS and PDCCH or PDSCH reception from the serving cell with a different numerology | 38.306, 4.2.9 | Rel-18 | pc_concurrentMeasCRS_InsideBWP_EUTRA_r18 | No |  | A UE supporting this feature shall also indicate support of eutra-NoGapMeasurementInsideBWP-r18 (pc_eutra_NoGapMeasurementInsideBWP_r18) or eutra-NoGapMeasurementOutsideBWP-r18 (pc_eutra_NoGapMeasurementOutsideBWP_r18). |
| 108 | Support multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG as specified in TS 38.133 | 38.306, 4.2.9 | Rel-18 | pc_concurrentMeasGapsNCSG_r18 | No |  | A UE supporting this feature shall also indicate support of nr-NeedForGapNCSG-Reporting-r17 (pc_nr_NeedForGapNCSG_Reporting_r17) and concurrentMeasGap-r17. |
| 109 | Support multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG as specified in TS 38.133 | 38.306, 4.2.9 | Rel-18 | pc_concurrentMeasGapsPreMG_r18 | No |  | A UE supporting this feature shall also indicate support of concurrentMeasGap-r17 and one of preconfiguredNW-ControlledMeasGap-r17 (pc_preconfiguredNW_ControlledMeasGap_r17) and preconfiguredUE-AutonomousMeasGap-r17 (pc_preconfiguredUE_AutonomousMeasGap_r17). |
| 110 | Support RRM requirements for handling dynamic collisions between a Pre-MG and another measurement gap or Pre-MG | 38.306, 4.2.9 | Rel-18 | pc_dynamicCollision_r18 | No |  | A UE supporting this feature shall also indicate support of concurrentMeasGapsPreMG-r18 (pc_concurrentMeasGapsPreMG_r18). |
| 111 | Support configuration of effective measurement window for inter-RAT EUTRAN measurements, including offset, duration and periodicity | 38.306, 4.2.9 | Rel-18 | pc_eutra_MeasEMW_r18 | No |  | A UE supporting this feature shall also indicate support of eutra-NoGapMeasurementOutsideBWP-r18 (pc_eutra_NoGapMeasurementOutsideBWP_r18) or eutra-NoGapMeasurementInsideBWP-r18 (pc_eutra_NoGapMeasurementInsideBWP_r18). |
| 112 | Support inter-RAT EUTRAN measurements without gap when CRS is completely contained within UE's active DL BWP | 38.306, 4.2.9 | Rel-18 | pc_eutra_NoGapMeasurementInsideBWP_r18 | No |  |  |
| 113 | Support inter-RAT EUTRAN measurements outside active DL BWP for nogap-noncsg | 38.306, 4.2.9 | Rel-18 | pc_eutra_NoGapMeasurementOutsideBWP_r18 | No |  | A UE supporting this feature shall also indicate support of eutra-NeedForGapNCSG-Reporting-r17 (pc_eutra_NeedForGapNCSG_Reporting_r17). |
| 114 | Indicates measurement gap pattern(s) optionally supported by the UE for NR SA and NR-DC when the frequencies to be measured within this measurement gap are all NR frequencies | 38.306, 4.2.9 | Rel-16 | pc_meas_gap_nr_only_r16 | No |  | The UE shall set the bits corresponding to the measurement gap pattern 2, 3 and 11 to 1 |
| 115 | Support of SSB based inter-RAT measurements without measurement gap, but with interruption (no-gap-with-interruption) on each supported NR band. | 36.306, 4.3.6.53 | Rel-18 | pc_interRAT_NeedForInterruptionNR_NGWI_r18 | No |  | A UE includes this field only if it indicates measurement gap is not required in the corresponding interRAT-NeedForGapsNR-r16 field. |
| 116 | Support of SSB based inter-RAT measurements without measurement gap nor interruption (no-gap-no-interruption) on each supported NR band. | 36.306, 4.3.6.53 | Rel-18 | pc_interRAT_NeedForInterruptionNR_NGNI_r18 | No |  | A UE includes this field only if it indicates measurement gap is not required in the corresponding interRAT-NeedForGapsNR-r16 field. |
| 117 | Support of concurrent SSB-based inter-RAT measurement on NR FR1 cell and PDCCH or PDSCH reception from the serving cell with a different numerology | 36.306, 4.3.6.54 | Rel-18 | pc_simultaneousRxDataSSB_DiffNumerology_FR1_r18 | No |  | A UE includes this field only if it indicates support of interRAT-NeedForInterruptionNR-r18 for at least one target band in at least one band combination. |
| 118 | Support location based RRM measurements of neighbour cells in NTN quasi-Earth fixed cell in RRC_IDLE/RRC_INACTIVE | 38.306, 5.6 | Rel-18 | pc_meas_ntn_quasi_earth_fixed_cell | No |  | If a UE supports this feature, then it supports r18 NTN Enhanced |
| 119 | Support location based RRM measurements of neighbour cells in NTN Earth-moving cell in RRC_IDLE/RRC_INACTIVE | 38.306, 5.6 | Rel-18 | pc_meas_ntn_earth_moving_cell | No |  | If a UE supports this feature, then it supports r18 NTN Enhanced |
| 120 | Support Skipping TN measurements | 38.306, 5.6 | Rel-18 | pc_skipping_tn_measurements | No |  |  |

<< End of Changes >>