3GPP TSG-RAN4 Meeting # 117	R4-2523241

# Dallas, USA, Nov.17 – 21, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6285 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Big CR for RRM core requirements of NR MIMO Phase 5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, MediaTek |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-11-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This is the Big CR to capture the endorsed draft CRs in RAN4#117 for core maintenance for Rel-19 MIMO RRM: - R4-2522773, CR for UEIBM reporting delay, Nokia - R4-2522774, CR on UEIBR Requirements, Apple - R4-2521095, CR on core requirements maintenance for Rel-19 MIMO UEIEBM, vivo - R4-2522096, CR MTTD requirements for UL only TRP, Nokia - R4-2522097, CR on transmit timing requirements for asymmetric DL sTRP/UL mTRP deployment, Nokia |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the RRM core requirements correction for Rel-19 MIMO RRM |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The correction for Rel-19 MIMO RRM core requirements are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.3.4 9.13.3.4 7.5.7 7.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==First change R4-2522773&R4-2522774&R4-2521095==

#### 9.5.3.4	Event Triggered Reporting for the UE initiated beam management

For a UE configured with a CSI-ReportConfig with csi-ReportUE-IBR-r19 and with dl-OrJointTCI-StateList，with or without eventCountWindow-r19 configuration, the reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2, respectively.

The UE shall not send any event triggered measurement reports if no reporting criteria is fulfilled.

The event triggered L1-RSRP measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE transmits first PUCCH over the air interface.

When eventCountWindow-r19 is not configured,

The event triggered measurement reporting delay shall be no larger than TL1-meas_basic + Tfirst_UL_channel, where:

TL1-meas_basic is the maximum of L1-RSRP measurement periods of the reference signals of the beams corresponding to the event

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventTypeUE-IBR-r19 set to ‘event2-r19’, UE shall perform L1-RSRP measurement based on the L1-RSRP measurement period of current beam and new beam(s), respectively.

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventTypeUE-IBR-r19 set to ‘event1-r19’, UE shall perform L1-RSRP measurement based on current beam L1-RSRP measurement period.

As defined in clause 5.2.1.5.4 TS 38.214 [26], if newBeamResourceSet is a CSI-SSB-ResourceSet:

the L1-RSRP measurement period of the current beam refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-1 or 9.5.4.1-2 where TReport. = 0, and TSSB is the periodicity of the SSB-Index of the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state for the current beam.

the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-1 or 9.5.4.1-2 where TReport. = 0, and TSSB is the periodicity of the SSB-Index of the reference signal configured by the newBeamResourceSet-r19.

As defined in clause 5.2.1.5.4 TS 38.214 [26], if newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition:

the L1-RSRP measurement period of the current beam refers to TL1-RSRP_Measurement_Period_CSI-RS as specified in table 9.5.4.2-1 or 9.5.4.2-2 where TReport. = 0, and TCSI-RS is the periodicity of the reference signal in the indicated TCI state for the current beam.

the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_CSI-RS as specified in table 9.5.4.2-1 or 9.5.4.2-2 where TReport. = 0, and TCSI-RS is the periodicity of the the reference signal configured by the newBeamResourceSet-r19 for new beams.

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventTypeUE-IBR-r19 set to ‘event7-r19’, UE shall perform L1-RSRP measurement based on L1-RSRP measurement period of the beams in active TCI state list and new beams.

As defined in clause 5.2.1.5.4 TS 38.214 [26], if newBeamResourceSet is a CSI-SSB-ResourceSet:

the L1-RSRP measurement period of each beam of activated TCI states refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-1 or 9.5.4.1-2 where TReport. = 0, and TSSB is the periodicity of the SSB-Index of the reference signal in the activated TCI states or the SS/PBCH block which is QCLed with the reference signal in the activated TCI states.

the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_SSB as specified in table 9.5.4.1-1 or 9.5.4.1-2 where TReport. = 0, and TSSB is the periodicity of the SSB-Index of the reference signal configured by the newBeamResourceSet-r19.

As defined in clause 5.2.1.5.4 TS 38.214 [26], if newBeamResourceSet is a NZP-CSI-RS-ResourceSet configured with repetition:

the L1-RSRP measurement period of each beam of activated TCI states refers to TL1-RSRP_Measurement_Period_CSI-RS as specified in table 9.5.4.2-1 or 9.5.4.2-2 where TReport. = 0, and TCSI-RS is the periodicity of the reference signal in the activated TCI states.

the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_CSI-RS as specified in table 9.5.4.2-1 or 9.5.4.2-2 where TReport. = 0, and TCSI-RS is the periodicity of the the reference signal configured by the newBeamResourceSet-r19 for new beams.

Tfirst UL channel is from the time point at RS which triggered the L1 reporting to the time point at next PUCCH transmission occasion

Otherwise, if eventCountWindow-r19 is provided, the event triggered measurement reporting delay shall be no larger than (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel, where

- eventInstanceCount-r19 is configured by the network

- TL1-meas_basic is the measurment period above based on the configuration,

- L is the number of measurement instances (TL1-meas_basic) that do not satisfy the event condition, counted after the event counter is started for the window that will trigger the measurement report and before the required number of events (eventInstanceCount-r19) are detected within the specified time window (eventDetectionTimeWindow-r19).

- The above requriement is applicable when eventDetectionTimeWindow-r19 is not less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic . The requirement does not apply when eventDetectionTimeWindow-r19 is less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic.

#### 9.13.3.4	Event triggered reporting for UE initiated beam management

For a UE configured with a CSI-ReportConfig with csi-ReportUE-IBR-r19 and with dl-OrJointTCI-StateList, with or without eventCountWindow-r19 configuration the reported L1-RSRP measurements contained in an event triggered L1-RSRP measurement report shall meet the requirements in clauses 10.1.19 for FR1 and 10.1.20 for FR2.

The UE shall not send any event triggered measurement reports if no reporting criteria is fulfilled.

The event triggered L1-RSRP measurement reporting delay is defined as the time between an event that will trigger a measurement report after condition is met over the air and the point when the UE transmits first PUCCH over the air interface.

When eventCountWindow-r19 is not configured,

The event triggered L1-RSRP measurement reporting delay is defined as the time between an event that will trigger a measurement report after condition is met over the air and the point when the UE transmits first PUCCH over the air interface. The event triggered measurement reporting delay shall be no larger than TL1-meas_basic + Tfirst_UL_channel.

TL1-meas_basic is the maximum of L1-RSRP measurement periods of the reference signals of beams corresponding to the event

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventTypeUE-IBR-r19 set to ‘event2-r19’, UE shall perform L1-RSRP measurement based on the L1-RSRP measurement period of current beam and new beam(s) respectively.

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventTypeUE-IBR-r19 set to ‘event1-r19’, UE shall perform L1-RSRP measurement based on the L1-RSRP measurement period of current beam.

If the SS/PBCH block is QCLed with the reference signal in the indicated TCI state, the L1-RSRP measurement period of the current beam refers to TL1-RSRP_Measurement_Period_SSB_CDP in Table 9.13.4.1-1 or 9.13.4.1-2 where TReport. = 0, and TSSB_CDP is the periodicity of the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state for the current beam

If the reference signals in newBeamResourceSet-r19 are based on SSB for a cell with additional PCI, the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_SSB_CDP in Table 9.13.4.1-1 or 9.13.4.1-2 where TReport. = 0, and TSSB_CDP is the periodicity of the SSB-Index of the reference signal configured by the newBeamResourceSet-r19.

For a UE configured with a CSI-ReportConfig with the higher layer parameter eventTypeUE-IBR-r19 set to ‘event7-r19’, UE shall perform L1-RSRP measurement based on L1-RSRP measurement period of current beam and new beam respectively.

If the SS/PBCH block is QCLed with the reference signal corresponding to the activated TCI states with the highest valueOfQ-r19  the L1-RSRP measurement period of the beam refers to TL1-RSRP_Measurement_Period_SSB_CDP as specified in table 9.13.4.1-1 or 9.13.4.1-2 where TReport. = 0, and TSSB is the periodicity of the SSB-Index of the reference signal corresponding to the activated TCI states with the highest valueOfQ-r19

If the reference signals in newBeamResourceSet-r19 are based on SSB for a cell with additional PCI, the L1-RSRP measurement period of the new beam refers to TL1-RSRP_Measurement_Period_SSB_CDP in Table 9.13.4.1-1 or 9.13.4.1-2 where TReport. = 0, and TSSB_CDP is the periodicity of the SSB-Index of the reference signal configured by the newBeamResourceSet-r19.

Tfirst UL channel is the time to first PUCCH after L1 reporting condition is triggered

Otherwise, if eventCountWindow-r19 is provided, the event triggered measurement reporting delay shall be no larger than (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel, where

- eventInstanceCount-r19 is configured by the network

- TL1-meas_basic is the measurement period above based on the configuration

- L is the number of measurement instances (TL1-meas_basic) that do not satisfy the event condition, counted after the event counter is started for the window that will trigger the measurement report and before the required number of events (eventInstanceCount-r19) are detected within the specified time window (eventDetectionTimeWindow-r19).

- The above requriement is applicable when eventDetectionTimeWindow-r19 is not less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic . The requirement does not apply when eventDetectionTimeWindow-r19 is less than (eventInstanceCount-r19 + L-1) * TL1-meas_basic.

==============Next change R4-2522096==============

### 7.5.7	Minimum requirements for multi-TRP

A UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18 shall be capable of handling at least a relative transmission timing difference between slot timing of a pair of TAGs configured on the same carrier as shown in tables 7.5.7-1, if the UE does not indicate support of rxTimingDiff-r18.

A UE supporting multiDCI-IntraCellMultiTRP-TwoTA-r18 or multiDCI-InterCellMultiTRP-TwoTA-r18 shall be capable of handling at least a relative transmission timing difference between slot timing of the pair of TAGs configured on the same carrier as shown in table 7.5.7-2, provided that the UE indicates that it is capable of rxTimingDiff-r18.

A UE supporting twoTA-InterCellBM-r19 or twoTA-IntraCellBM-r19 shall be capable of handling at least a relative transmission timing difference between slot timing of a pair of TAGs configured on the same carrier with single DCI as shown in Table 7.5.7-1, if the UE is configured with PL_offset or if the UE does not indicate support of rxTimingDiff-r18, and in Table 7.5.7-2, if the UE indicates that it is capable of rxTimingDiff-r18 and PL_offset is not configured.

Table 7.5.7-1: Maximum uplink transmission timing difference requirement for multi-TRP for UE not supporting rxTimingDiff-r18

| Frequency Range of the pair of TAGs | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| FR1 | CP lengthnote 1+1.6us |
| FR2-1 | CP lengthnote 1+0.5us |
| NOTE 1: CP length of the maximum SCS on the carrier. NOTE 2: This requirement applies to the UE capable of [STxMP]. |  |

Table 7.5.7-2: Maximum uplink transmission timing difference requirement for multi-TRP for UE supporting rxTimingDiff-r18

| Frequency Range of the pair of TAGs | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| FR1 | 34.6 |
| FR2-1 | 8.5 |

==============Next change R4-2522097==============

# 7	Timing

## 7.1	UE transmit timing

### 7.1.1	Introduction

The UE shall have capability to follow the frame timing change of the reference cell in RRC_CONNECTED state or when transmitting PUSCH on CG resources for SDT in RRC_INACTIVE. The uplink frame transmission takes place NTA+NTA offset×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. For serving cell(s) in pTAG, UE shall use the SpCell as the reference cell for deriving the UE transmit timing for cells in the pTAG. For serving cell(s) in sTAG, UE shall use any of the activated SCells as the reference cell for deriving the UE transmit timing for the cells in the sTAG. For UL-WUS transmission for OD-SIB1, UE shall use the target NES cell as the reference cell for deriving the UE transmit timing. UE initial transmit timing accuracy and gradual timing adjustment requirements are defined in the following requirements.

In the requirements of clause 7.1.2, the term reference cell on a carrier frequency subject to CCA is not available at the UE refers to when at least one SSB is configured by gNB, but the first two successive candidate SSB positions for the same SSB index within the discovery burst transmission window are not available during at least one discovery burst transmission window, at the UE due to DL CCA failures at gNB during the last 1280 ms; otherwise the reference cell on the carrier frequency subject to CCA is considered as available at the UE.

For UE supporting rach-EarlyTA-Measurement-r18 for LTM, and if the candidate cell is a neighbour cell or a secondary serving cell without uplink carrier,

-	UE shall have capability to follow the frame timing of the reference cell. The PRACH transmission for early TA acquisition takes place NTA+NTA offset×Tc before the reception of the first detected path (in time) of the corresponding downlink frame from the reference cell. UE shall use this candidate cell as the reference cell for deriving transmit timing. UE initial transmit timing accuracy is defined in the following requirements.

For multi-DCI based multi-TRP operation with two TAs and for asymmetric DL sTRP/UL mTRP deployment with two TAs, for each TAG, the uplink transmission timing takes place (NTA+NTA offset)×Tc before the reception of the downlink reference point.

For multi-DCI based multi-TRP operation with two TAs, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState TS 38.331 [2] having the same TAG as the uplink signal.

For asymmetric DL sTRP/UL mTRP deployment with two TAs, for the scenario that UE is configured with PL offset in joint/UL TCI state(s) and UE does not expect to receive SSB from UL TRP(s),  the reference point for PRACH/PUCCH/PUSCH/SRS is the first detected path in time of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DL or Joint TCI state, and is the same for both TAGs.

For  asymmetric DL sTRP/UL mTRP deployment with two TAs, for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), for the case UE is configured with SSB-MTC-additionalPCI, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState or -UL TCI states TS 38.331 [2] having the same TAG as the uplink signal.

For multi-DCI based multi-TRP operation with two TAs if the UE is not provided SSB-MTC-AdditionalPCI, the reference point for PRACH transmission triggered by PDCCH order, is the first detected path (in time) of one of the downlink reference signal(s) in the DL or joint TCI state of the reference cell associated with the same coresetPoolIndex as PDCCH carrying PDCCH order if PRACH association indicator is 0, and that of the other coresetPoolIndex if PRACH association indicator is 1.

For multi-DCI based multi-TRP operation with two TAs if the UE is provided SSB-MTC-AdditionalPCI, the reference point for PRACH transmission triggered by PDCCH order,

-	if “PRACH association indicator” is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

-	if “PRACH association indicator” is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the second TAG.

For asymmetric DL sTRP/UL mTRP deployment with two TAs, for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), for the case UE is configured with SSB-MTC-additionalPCI, the reference point for PRACH transmission triggered by PDCCH order,

-	if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG.

-	if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL or UL TCI state(s) of the reference cell associated with the second TAG.

Where NTA and NTA offset are commanded by the network independently for each TAG TS 38.331 [2]. NTA offset for first TAG and second TAG are as defined in n-TimingAdvanceOffset and n-TimingAdvanceOffset2 respectively, if configured, otherwise it is NTA offset defined in table 7.1.2-2.

In the requirements of clause 7.1.2, the SSB applies for both CD-SSB and NCD-SSB if the UE supports ncd-SSB-BWP-Wor-r18.

==============End of change==============