3GPP TSG-RAN4 Meeting #117	R4-2521706

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-1 | CR | 0355 | rev | - | Current version: | 17.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (AASenh_BS_LTE_UTRA-Perf) CR to 37.145-1: test procedure for additional transmitter intermodulation for MSR operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  | Date: |  |  | 2025-11-21 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Interfering signal generation for additional transmitter intermodulation test are missing in the test procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The interfering signal generation for additional co-locattion (BC1 and BC2) are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interfering signal generation for additional transmitter intermodulation test are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 37.145-2 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

6.7.4.2	Procedure

The minimum requirement is applied to all TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in clause D.1.1. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

1)	Connect TAB connector to measurement equipment as shown in annex D.1.2. All TAB connectors not under test shall be terminated.

2)	The measurement device characteristics shall be:

-	Detection mode: True RMS.

The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table 4.1.2.2-1.

3)	Set the set the TAB connector to transmit:

a)	For MSR:

-	Set the TAB connector to transmit maximum power according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in clause 4.11.

b)	For UTRA FDD:

-	For a TAB connector declared to be capable of single carrier operation only, set the TAB connector to transmit a signal according to TM1, clause 4.12.2, at the manufacturer's declared rated output power, Prated,c,TABC.

-	For a TAB connector declared to be capable of multi-carrier operation, set the set the TAB connector to transmit according to TM1 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 4.11.

c)	For UTRA TDD:

-	For a TAB connector declared to be capable of single carrier operation only, set the parameters of the transmitted signal according to table 6.7.4.2-1.

-	For a TAB connector declared to be capable of multi-carrier operation, set to transmit according to table 6.7.4.2-1on all carriers.

Table 6.7.4.2-1: Parameters of the transmitted signal for transmit intermodulation testing
for 1,28 Mcps TDD

| Parameter | Value/description |
| --- | --- |
| TDD Duty Cycle | TS i; I = 0, 1, 2, 3, 4, 5, 6: transmit, if i is 0,4,5,6; receive, if i is 1,2,3. |
| Time slots under test | TS4, TS5 and TS6 |
| Number of DPCH in each time slot under test | 8 |
| Power of each DPCH | 1/8 of Base Station output power |
| Data content of DPCH | real life (sufficient irregular) |

d)	For E-UTRA:

-	TAB connector declared to be capable of single carrier operation only, set the TAB connector to transmit a signal according to E- TM1.1 in clause 4.12.2, at manufacturer's declared rated output power Prated,c,TABC.

-	For a TAB connector declared to be capable of multi-carrier and/or CA operation, set the set the TAB connector to transmit according to E-TM1.1 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 4.11.

4)	Generate the interfering signal:

a)	For MSR:

-	for general co-location using E-TM1.1 as defined in clause 4.12.2, with 5 MHz channel bandwidth, at a centre frequency offset according to the conditions in table 6.7.2.1-1 in TS 37.105 [8], but exclude interfering frequencies that are outside of the allocated downlink operating band or interfering frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

-	for additional co-location (BC1 and BC2) using CW signal with a frequency offset according to the conditions in table 6.7.2.2-1 in TS 37.105 [8], but exclude interfering frequencies that are outside of the allocated downlink operating band or interfering frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

b)	For UTRA FDD:

-	in accordance to TM1, clause 4.12.2 with a frequency offset of according to the conditions of table 6.7.3.1-1 in TS 37.105 [8], but exclude interfering signal frequencies that are outside of the allocated downlink operating band or interfering signal frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

c)	For UTRA TDD:

-	The signal shall be like-modulated as the transmit signal and the active time slots of both signals shall be synchronized, with a frequency offset of according to the conditions of table 6.7.3.2-1 in TS 37.105 [8], but exclude interfering signal frequencies that are outside of the allocated downlink operating band or interfering signal frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

d)	For E-UTRA:

-	according to E-TM1.1, as defined in clause 4.12.2, with 5 MHz channel bandwidth and a centre frequency offset according to the conditions of table 6.7.4.1-1 in TS 37.105 [8], but exclude interfering frequencies that are outside of the allocated downlink operating band or interfering frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

5)	Adjust ATT1 so that level of the interfering signal is as defined in:

a)	For MSR:

i.	General co-location table 6.7.2.1-1 in TS 37.105 [8].

ii.	Additional co-location (BC1 and BC2) table 6.7.2.2-1 in TS 37.105 [8].

iii.	Additional co-location (BC3) table 6.7.2.3-1 in TS 37.105 [8].

iv.	Intra-system table 6.7.2.5-1 in TS 37.105 [8].

b)	For UTRA FDD:

i.	General co-location table 6.7.3.1-1 in TS 37.105 [8].

ii.	Intra-system table 6.7.3.3-1 in TS 37.105 [8].

c)	For UTRA TDD:

i.	General co-location for 1,28 Mcps TDD UTRA table 6.7.3.2-1 in TS 37.105 [8].

ii.	Intra-system table 6.7.3.3-1 in TS 37.105 [8].

d)	For E-UTRA:

i.	General co-location table 6.7.4.1-1 in TS 37.105 [8].

ii.	Void

iii.	Intra-system table 6.7.4.131 in TS 37.105 [8].

6)	If the test signal is applicable according to clause 5, perform the unwanted emission tests specified in clauses 6.6.3, 6.6.4 and 6.6.5, for all third and fifth order intermodulation products which appear in the frequency ranges defined in clauses 6.6.3, 6.6.4 and 6.6.5. The width of the intermodulation products shall be taken into account.

7)	If the test signal is applicable according to clause 5, perform the Transmitter spurious emissions test as specified in clause 6.6.6, for all third and fifth order intermodulation products which appear in the frequency ranges defined in clause 6.6.6. The width of the intermodulation products shall be taken into account.

8)	Verify that the emission level does not exceed the required level in clause 6.7.5 with the exception of interfering signal frequencies.

9)	Repeat the test for the remaining interfering signal centre frequency offsets according to the conditions of:

a)	For MSR:

i.	General co-location table 6.7.2.1-1 in TS 37.105 [8].

ii.	Additional co-location (BC1 and BC2) table 6.7.2.2-1 in TS 37.105 [8].

iii.	Additional co-location (BC3) table 6.7.2.3-1 in TS 37.105 [8].

iv.	Intra-system table 6.7.2.5-1 in TS 37.105 [8].

b)	For UTRA FDD:

i.	General co-location table 6.7.3.1-1 in TS 37.105 [8].

ii.	Intra-system table 6.7.3.3-1 in TS 37.105 [8].

c)	For UTRA TDD:

i.	General co-location for 1,28 Mcps TDD UTRA table 6.7.3.2-1 in TS 37.105 [8].

ii.	Intra-system table 6.7.3.3-1 in TS 37.105 [8].

d)	For E-UTRA:

i.	General co-location table 6.7.4.1-1 in TS 37.105 [8].

ii.	Void

iii.	Intra-system table 6.7.4.131 in TS 37.105 [8].

10)	Repeat the test for the remaining test signals defined in clause 5 for requirements 6.6.1, 6.6.2 and 6.6.4.

In addition, for multi-band TAB connector(s), the following steps shall apply:

11)	For multi-band TAB connectors and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

NOTE:	The third order intermodulation products are centred at 2F1F2 and 2F2F1. The fifth order intermodulation products are centred at 3F12F2, 3F22F1, 4F1F2, and 4F2F1 where F1 represents the test signal centre frequency or centre frequency of each sub-block and F2 represents the interfering signal centre frequency. The widths of intermodulation products are:

-	(n*BWF1 + m*BWF2) for the nF1mF2 products;

-	(n*BWF2 + m*BWF1) for the nF2mF1 products;

where BWF1 represents the test signal RF bandwidth or channel bandwidth in case of single carrier, or sub-block bandwidth, and BWF2 represents interfering signal bandwidth.

==============End of change==============