3GPP TSG-RAN4 Meeting #117	R4-2523000

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR | 0399 | rev | 1 | Current version: | 15.19.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (AASenh_BS_LTE_UTRA-Perf) CR to 37.145-2: test procedure for additional transmitter intermodulation for MSR operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | AASenh_BS_LTE_UTRA-Perf |  |  |  |  |  | Date: |  |  | 2025-11-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | 1. Interfering signal generation for additional transmitter intermodulation test are missing in the test procedure. 2. The frequency offsets for co-location transmitter intermodulation (BC1 and BC2) is to be added as agreed in R4-2511303 for core specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. The interfering signal generation for additional co-locattion (BC1 and BC2) are added. 2. The frequency offsets are added for test requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interfering signal generation and some frequency offsets for additional transmitter intermodulation test are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.8.4.2, 6.8.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 37.145-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

6.8.4.2	Procedure

1)	Select a CLTA according to parameters given in Table 4.15.2.2-1.

2)	Place the CLTA according to parameters given in Table 4.15.2.3-1.

3)	The test antenna(s) shall be dual (or single) polarized covering the same frequency range as the AAS BS and the emission frequencies.

4)	Several test antennas are required to cover both the AAS BS and the whole emission frequency range.

5)	Connect the test antenna and CLTA to the measurement equipment as shown in Annex D1.5, Figures D.1.5-1.

6)	During the OTA emission measurements at the test antenna conducted output(s), both AAS BS and CLTA are rotated around same axis.

7)	The OTA unwanted emissions measurement method shall be TRP, according to the procedure described in Annex F.

8)	The measurement device (signal analyzer) characteristics shall be:

-	Detection mode: True RMS.

The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table 4.1.2.2-1.

9)	Set the AAS BS to transmit:

a)	For MSR:

-	Set the AAS BS to transmit maximum power according to the applicable test configuration in clause 5 using the corresponding test models or set of physical channels in clause 4.11.

b)	For UTRA FDD:

-	For a AAS BS declared to be capable of single carrier operation only, set the AAS BS to transmit maximum power according to TM1, clause 4.12.2, at the manufacturer's declared rated carrier TRP, Prated,c,TRP.

-	For a AAS BS declared to be capable of multi-carrier operation, set the AAS BS to transmit maximum power according to TM1 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 4.11.

c)	For E-UTRA:

-	For AAS BS declared to be capable of single carrier operation only, set the AAS BS to transmit maximum power according to E-TM1.1 in clause 4.12.2, at manufacturer's declared rated carrier TRP, Prated,c,TRP.

-	For a AAS BS declared to be capable of multi-carrier and/or CA operation, set the AAS BS to transmit maximum power according to E-TM1.1 on all carriers configured using the applicable test configuration and corresponding power setting specified in clause 4.11.

10)	Generate the interfering signal:

a)	For MSR:

-	for general co-location using E-TM1.1 as defined in clause 4.12.2, with 5 MHz channel bandwidth, at a centre frequency offset according to the conditions in table 6.8.5.1.1-1, but exclude interfering frequencies that are outside of the allocated downlink operating band or interfering frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

-	for additional co-location (BC1 and BC2) using CW signal with a frequency offset according to the conditions in table 6.8.5.1.2-1, but exclude interfering frequencies that are outside of the allocated downlink operating band or interfering frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

b)	For UTRA FDD:

-	in accordance to TM1, clause 4.12.2 with a frequency offset according to the conditions of table 6.8.5.2.1-1, but exclude interfering signal frequencies that are outside of the allocated downlink operating band or interfering signal frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

c)	For E-UTRA:

-	according to E-TM1.1, as defined in clause 4.12.2, with 5 MHz channel bandwidth and a centre frequency offset according to the conditions of table 6.8.5.3.1-1, but exclude interfering frequencies that are outside of the allocated downlink operating band or interfering frequencies that are not completely within the sub-block gap or within the Inter RF Bandwidth gap.

11)	Connect the interfering signal to the CLTA input interfaces, equally dividing the power among supported polarizations. Adjust the interfering signal level at the CLTA conducted input(s) as defined in:

a)	For MSR:

i.	General co-location table 6.8.5.1.1-1.

ii.	Additional co-location (BC1 and BC2) table 6.8.5.1.2-1.

iii.	Additional co-location (BC3) table 6.8.5.1.3-1.

b)	For UTRA FDD:

i.	General co-location table 6.8.5.2.1-1 .

c)	For E-UTRA:

i.	General co-location table 6.8.5.3.1-1.

ii.	Void

12)	If the interfering signal is applicable according to clause 5, perform the unwanted emission tests specified in clauses 6.7.3 (OTA ACLR), 6.7.4 (OTA spectrum mask) and 6.7.5 (OTA OBUE), for all third and fifth order intermodulation products which appear in the frequency ranges defined in clauses 6.7.3, 6.7.4 and 6.7.5 (NOTE 2). The width of the intermodulation products shall be taken into account.

13)	If the interfering signal is applicable according to clause 5, perform the transmitter spurious emissions test as specified in clause 6.7.6 (OTA spurious emission), except OTA co-location spurious emission, for all third and fifth order intermodulation products which appear in the frequency ranges defined in clause 6.7.6 (NOTE 2). The width of the intermodulation products shall be taken into account.

14)	Verify that the emission level does not exceed the required level in clause 6.8.5 (Test requirements) with the exception of interfering signal frequencies.

15)	Repeat the test for the remaining interfering signal centre frequency offsets according to the conditions of:

a)	For MSR:

i.	General co-location table 6.8.5.1.1-1.

ii.	Additional co-location (BC1 and BC2) table 6.8.5.1.2-1.

iii.	Additional co-location (BC3) table 6.8.5.1.3-1.

b)	For UTRA FDD:

i.	General co-location table 6.8.5.2.1-1 .

c)	For E-UTRA:

i.	General co-location table 6.8.5.3.1-1.

ii.	Void

16)	Repeat the test for the remaining interfering signals defined in clause 5 for requirements 6.7.3 (OTA ACLR), 6.7.4 (OTA spectrum mask), 6.7.5 (OTA OBUE) and 6.7.6 (OTA spurious emission), except OTA co-location spurious emission.

In addition, for multi-band AAS BS, the following steps shall apply:

17)	For multi-band AAS BS and single band tests, repeat the steps above per involved band where single band test configurations and test models shall apply with no carrier activated in the other band.

NOTE 1:	The third order intermodulation products are centred at 2F1F2 and 2F2F1. The fifth order intermodulation products are centred at 3F12F2, 3F22F1, 4F1F2, and 4F2F1 where F1 represents the test signal centre frequency or centre frequency of each sub-block and F2 represents the interfering signal centre frequency. The widths of intermodulation products are:

-	(n*BWF1 + m*BWF2) for the nF1mF2 products;

-	(n*BWF2 + m*BWF1) for the nF2mF1 products;

where BWF1 represents the test signal RF bandwidth or channel bandwidth in case of single carrier, or sub-block bandwidth, and BWF2 represents the interfering signal bandwidth.

NOTE 2:	During the conformance test the interfering signal can be applied on one side of the wanted signal, while the transmitter intermodulation emission is measured only on the opposite side of the wanted signal. This applies for intermodulation products which are within the operating band or OBUE region.

==============Next change==============

6.8.5.1	MSR test requirements

6.8.5.1.1	General test requirement

In the frequency range relevant for this test the transmitter intermodulation level shall not exceed the unwanted emission limits specified for transmitter spurious emission in clause 6.7.6 (except co-location spurious emission), operating band unwanted emission in clause 6.7.5 and ACLR in clause 6.7.3 in the presence of a wanted signal and an interfering signal according to table 6.8.5.1.1-1 for an OTA AAS BS operating in BC1, BC2 and BC3.

The requirement is applicable outside the edges of the Base Station RF Bandwidth. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges or radio bandwidth edges.

For RIB supporting operation in non-contiguous spectrum, the requirement is also applicable inside a sub-block gap for interfering signal offsets where the interfering signal falls completely within the sub-block gap. The interfering signal offset is defined relative to the sub-block edges.

For multi-band RIBs, the requirement applies relative to the Base Station RF Bandwidth edges of each operating band. In case the inter Base Station RF Bandwidth gap is less than 15 MHz, the requirement in the gap applies only for interfering signal offsets where the interfering signal falls completely within the inter Base Station RF Bandwidth gap.

Table 6.8.5.1.1-1: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal type | E-UTRA or NR signal |
| Interfering signal type | E-UTRA signal of channel bandwidth 5 MHz |
| Interfering signal power level applied to the CLTA | min(46 dBm, Prated,t,TRP) |
| Interfering signal centre frequency offset from Base Station RF Bandwidth edge or edge of sub-block inside a gap | ±2.5 MHz ±7.5 MHz ±12.5 MHz |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the RIB is excluded from the requirement, unless the interfering signal positions fall within the frequency range of adjacent downlink operating bands in the same geographical area. In case that none of the interfering signal positions fall completely within the frequency range of the downlink operating band, TS 37.141 provides further guidance regarding appropriate test requirements. NOTE 2: In certain regions, NOTE 1 is not applied in Band 1, 3, 8, 9, 11, 18, 19, 21, 28, 32 operating within 1 475.9 MHz to 1 495.9 MHz, 34. NOTE 3: For OTA AAS BS with dual polarization, the interfering signal power shall be equally divided between the supported polarizations at the CLTA. |  |

6.8.5.1.2	Additional test requirement (BC1 and BC2)

In the frequency range relevant for this test the transmitter intermodulation level shall not exceed the unwanted emission limits specified for transmitter spurious emission in clause 6.7.6 (except co-location spurious emission), operating band unwanted emission in clause 6.7.5 and ACLR in clause 6.7.3 in the presence of a wanted signal and an interfering signal according to table 6.8.5.1.2-1 for an OTA AAS BS operating in BC2.

The requirement is applicable outside the edges of the Base Station RF Bandwidth for BC2. The interfering signal offset is defined relative to the Base Station RF Bandwidth edges.

For RIBs supporting operation in non-contiguous spectrum in BC1 or BC2, the requirement is also applicable inside a sub-block gap with a gap size larger than or equal to two times the interfering signal centre frequency offset. For RIBs supporting operation in non-contiguous spectrum in BC1, the requirement is not applicable inside a sub-block gap with a gap size equal to or larger than 5 MHz. The interfering signal offset is defined relative to the sub-block edges.

For multi-band RIBs, the requirement applies relative to the Base Station RF Bandwidth edges of a BC2 operating band. The requirement is also applicable for BC1 and BC2 inside an inter Base Station RF Bandwidth gap equal to or larger than two times the interfering signal centre frequency offset. For RIBs supporting operation in multiple operating bands, the requirement is not applicable for BC1 band inside an inter Base Station RF Bandwidth gap with a gap size equal to or larger than 5 MHz.

Table 6.8.5.1.2-1: Interfering and wanted signals for the OTA transmitter intermodulation requirement

| Parameter | Value |
| --- | --- |
| Wanted signal type | E-UTRA and/or NR UTRA signal |
| Interfering signal type | CW |
| Interfering signal power level applied to the CLTA | min(46 dBm, Prated,t,TRP)) |
| Interfering signal centre frequency offset from Base Station RF Bandwidth edge or edge of sub-block inside a gap | ± 0.8 MHz ± 2.0 MHz ± 3.2 MHz ± 6.2 MHz |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the RIB are excluded from the requirement. NOTE 2: For OTA AAS BS with dual polarization, the interfering signal power shall be equally divided between the supported polarizations at the CLTA. |  |

6.8.5.1.3	Additional test requirement (BC3)

In the frequency range relevant for this test, the transmitter intermodulation level shall not exceed the unwanted emission limits specified for transmitter spurious emission in clause 6.7.6 (except co-location spurious emission), operating band unwanted emission in clause 6.7.5 and ACLR in clause 6.7.3 in the presence of a wanted signal and an interfering signal according table 6.8.5.1.3-1 an OTA AAS BS operating in BC3.

For multi-band RIBs, the requirement applies relative to the Base Station RF Bandwidth edges of each operating band. In case the Inter RF Bandwidth gap is less than 3.2 MHz, the requirement in the gap applies only for interfering signal offsets where the interfering signal falls completely within the inter Base Station RF Bandwidth gap.

Table 6.8.5.1.3-1: Interfering and wanted signals for the OTA transmitter intermodulation requirement (BC3)

| Parameter | Value |
| --- | --- |
| Wanted signal type | E-UTRA and/or UTRA and/or NR signal |
| Interfering signal type | 1,28 Mcps UTRA TDD signal of channel bandwidth 1,6 MHz |
| Interfering signal power level applied to the CLTA | min(46 dBm, Prated,t,TRP) |
| Interfering signal centre frequency offset from Base Station RF Bandwidth edge or edge of sub-block inside a gap | ±0,8 MHz ±1,6 MHz ±2,4 MHz |
| NOTE 1: Interfering signal positions that are partially or completely outside of any downlink operating band of the base station are excluded from the requirement. NOTE 2: For OTA AAS BS with dual polarization, the interfering signal power shall be equally divided between the supported polarizations at the CLTA. |  |

==============End of change==============