3GPP TSG-RAN4 Meeting #117	R4-2520363

# Dallas, United States, 17th Nov 2025 - 21st Nov 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 3062 | rev | - | Current version: | 18.11.1 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core) Correction CR for TS 38.101-1 general description of MSD requirements for 3Tx CA_R18 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, ZTE, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | 4Rx_low_NR_band_handheld_3Tx_NR_CA_ENDC-Core |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The paragraph proposed to be removed in clause 7.3A.2.3 was introduced in Rel-18 for inter-band ULCA with 3Tx (supporting UL MIMO or TxD in one of the bands), as the MSD requirements were agreed to be reused from inter-band ULCA with 2Tx(1Tx per band), except PC1.5 MSD due to IMD. Note this paragraph was added in clause 7.2A.2.3 (i.e., suffix A) as suffix H or L doesnot have clause 7. Which is to reflect the following R18 agreement made for 3Tx CA: However, this agreement doesnot fit DLCA with single UL (with PC2 FDD band) which applies different requirements between UL aggressor with 1Tx and 2Tx. Hence, we propose to remove this paragraph, as it is more straightforwd to directly check the MSD tables, in which either one set of requriements or two set of requirements can be easily found. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the paragraph as shown in this CR. (This CR is a re-submission. It was agreed in RAN4#116 as R4-2510370, but has not been implemented due to cover page issue. ) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec is not accurate |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | × | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | × |  | Test specifications |  |  |  | TS/TR ... CR ... TS 38.521 |  |  |
| (show related CRs) |  |  | × | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This CR is a re-submission. It was agreed in RAN4 116 as R4-2510370, but has not been implemented due to cover page issue. There is no mirror cat A CR for R19 as it has already been implemented in Rel-19 spec. |  |  |  |  |  |  |  |  |

==============First change==============

7.3A.2.3	Reference sensitivity power level for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 with parameters specified in  Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, Table 7.3.2-2a, Table 7.3.2-3, and in Table 7.3F.2-1, Table 7.3F.2-2, Table 7.3F.2-3 for inter-band CA with one shared spectrum channel access band, modified in accordance with clause 7.3A.3.2. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. Exceptions to reference sensitivity are allowed in accordance with clause 7.3A.4, 7.3A.5 and 7.3A.6.

For the combination of intra-band and inter-band carrier aggregation, the intra-band CA relaxation, ΔRIBC and ΔRIBNC, are also applied according to the clause 7.3A.2.1 and 7.3A.2.2.

==============End of change==============