3GPP TSG-RAN WG4 Meeting #117	R4-2522262

# Dallas, USA, November 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6281 | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: |  |  | UICC apps |  |  |  |  | ME | X | Radio Access Network |  |  |  |  |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | CR 38.133 Correction to interruption requirements for PRS BWA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  |  |  |  |  | Date: |  |  | 2025-11-07 |  |  |  |  |
| Category: | A |  |  |  |  |  |  |  |  |  | Release: |  |  | Rel-19 |  |  |  |  |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |  |  |  |  |
| Reason for change: |  | Interruption requirements for PRS BWA for aggregated SRS transmission with 4 OFDM symbols are incorrect. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | Interruption requirements for PRS BWA for aggregated SRS transmission with 4 OFDM symbols are corrected. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Interruption requirements for SRS BWA are incorrect. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.2.2.21 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Other specs |  |  |  | X | Other core specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |
| affected: |  |  |  | X | Test specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |
| (show related CRs) |  |  |  | X | O&M Specifications |  |  |  |  |  |  | TS/TR ... CR ... |  |  |  |  |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

##### 8.2.2.2.21	Interruptions at NR SRS bandwidth aggregation for positioning

The requirements in this clause are applicable to SRS transmission for positioning with bandwidth aggregation, when SRS resource configured in a CC without PUSCH/PUCCH is linked for aggregation with an SRS resource configured in an UL active BWP of a CC with PUSCH/PUCCH.

SRS resource sets across two or three CCs are aggregated for positioning when UE is configured with linkage information SRS-PosResourceSetLinkedForAggBW specified in TS 38.331 [2], provided that:

-	the aggregated SRS transmission satisfy all the conditions defined in clause 6.2.1.4.2 in TS 38.214 [26];

-	the aggregated SRS transmission and the guard period before and after the SRS transmission are not colliding with any other UL transmission or/and DL reception with higher priority in victim cells defined in TS 38.214 [26];

-	the aggregated SRS transmission and the guard period before and after does not collide with any NR L3 or L1 measurements on victim cells.

For interruption caused by SRS transmission for positioning with bandwidth aggregation, the victim cell is identified based on the entry number of the band indicated by UE in posSRS-BWA-AffectedBandList-r18 regardless of per-FR MG capability.

NOTE 1: UE may indicate no other bands in the band combination are affected by the SRS carrier switching, in which case, only the band with the aggregated SRS transmissions is affected.

NOTE 2: If the capability is not indicated, interruption is allowed on all serving cells.

For each SRS transmission, interruption with up to X OFDM symbols is allowed on any active serving cell on the band as indicated in posSRS-BWA-AffectedBandList-r18, during the guard periods before and after aggregated SRS transmission and the aggregated SRS transmission duration, where X equals to (Y+Z+1). Interruption length of one extra OFDM symbol is added to cover the impact of receive timing difference between aggressor and victim cells.

The supported guard period is reported by UE in guardPeriod in PosSRS-BWA-IndependentCA-RRC-Connected-r18 [2] and the total duration Y of guard periods before and after aggregated SRS transmission is converted into number of OFDM symbols in table 8.2.2.2.21-1.

Table 8.2.2.2.21-1: Total duration Y of guard periods before and after aggregated SRS transmission (OFDM symbols)

|  | Guard period (s) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SCS of victim cell (kHz) | 0 | 30 | 100 | 140 | 200 |
| 15 | 0 | 1 | 3 | 4 | 6 |
| 30 |  | 2 | 6 | 8 | 12 |
| 60 |  | 4 | 12 | 16 | 23 |
| 120 |  | 7 | 23 | 32 | 45 |
| 480 |  | 27 | 87 | 122 | 174 |
| 960 |  | 53 | 174 | 244 | 348 |

The duration of aggregated SRS transmission is given by nrofSymbols in corresponding SRS-Resource configurations [2]. This duration in number of nrofSymbols OFDM symbols is converted to Z OFDM symbols in victim cells according to the SCS configurations among the aggressor cell and victim cell(s). Z is specified in table 8.2.2.2.21-2, table 8.2.2.2.21-3, table 8.2.2.2.21-4, table 8.2.2.2.21-5 and table 8.2.2.2.21-6 for nrofSymbols equal to {1, 2, 4, 8, 12} respectively with different SCS configurations.

Table 8.2.2.2.21-2: Duration Z of aggregated SRS transmission with nrofSymbols = 1 (OFDM symbols)

|  | SCS of aggressor cell (kHz) |  |  |  |
| --- | --- | --- | --- | --- |
| SCS of victim cell (kHz) | 15 | 30 | 60 | 120 |
| 15 | 1 | 1 | 1 | 1 |
| 30 | 2 | 1 | 1 | 1 |
| 60 | 4 | 2 | 1 | 1 |
| 120 | 8 | 4 | 2 | 1 |
| 480 | 32 | 16 | 8 | 4 |
| 960 | 64 | 32 | 16 | 8 |

Table 8.2.2.2.21-3: Duration Z of aggregated SRS transmission with nrofSymbols = 2 (OFDM symbols)

|  | SCS of aggressor cell (kHz) |  |  |  |
| --- | --- | --- | --- | --- |
| SCS of victim cell (kHz) | 15 | 30 | 60 | 120 |
| 15 | 2 | 1 | 1 | 1 |
| 30 | 4 | 2 | 1 | 1 |
| 60 | 8 | 4 | 2 | 1 |
| 120 | 16 | 8 | 4 | 2 |
| 480 | 64 | 32 | 16 | 8 |
| 960 | 128 | 64 | 32 | 16 |

Table 8.2.2.2.21-4: Duration Z of aggregated SRS transmission with nrofSymbols = 4 (OFDM symbols)

|  | SCS of aggressor cell (kHz) |  |  |  |
| --- | --- | --- | --- | --- |
| SCS of victim cell (kHz) | 15 | 30 | 60 | 120 |
| 15 | 4 | 2 | 1 | 1 |
| 30 | 8 | 4 | 2 | 1 |
| 60 | 16 | 8 | 4 | 2 |
| 120 | 32 | 16 | 8 | 4 |
| 480 | 128 | 64 | 32 | 16 |
| 960 | 256 | 128 | 64 | 32 |

Table 8.2.2.2.21-5: Duration Z of aggregated SRS transmission with nrofSymbols = 8 (OFDM symbols)

|  | SCS of aggressor cell (kHz) |  |  |  |
| --- | --- | --- | --- | --- |
| SCS of victim cell (kHz) | 15 | 30 | 60 | 120 |
| 15 | 8 | 4 | 2 | 1 |
| 30 | 16 | 8 | 4 | 2 |
| 60 | 32 | 16 | 8 | 4 |
| 120 | 64 | 32 | 16 | 8 |
| 480 | 256 | 128 | 64 | 32 |
| 960 | 512 | 256 | 128 | 64 |

Table 8.2.2.2.21-6: Duration Z of aggregated SRS transmission with nrofSymbols = 12 (OFDM symbols)

|  | SCS of aggressor cell (kHz) |  |  |  |
| --- | --- | --- | --- | --- |
| SCS of victim cell (kHz) | 15 | 30 | 60 | 120 |
| 15 | 12 | 6 | 3 | 2 |
| 30 | 24 | 12 | 6 | 3 |
| 60 | 48 | 24 | 12 | 6 |
| 120 | 96 | 48 | 24 | 12 |
| 480 | 384 | 192 | 96 | 48 |
| 960 | 768 | 384 | 192 | 96 |

--- End of change 1 ---