3GPP TSG RAN#110 Meeting 	RP-253704

Baltimore, United States of America, December 8-11, 2025

Title:	Observations on the status of WRC-27 Agenda Item 1.13 Studies

Source: 	Sateliot

Type:	Discussion

Document for:	Information

Agenda Item:	9.1.5

# 1. Introduction

Sateliot thanks the proponents of RP-251803 at RAN#108 for initiating a useful discussion on potential NR-NTN operations in Mobile Service (MS) allocations. Given the relevance of these frequency bands to both terrestrial IMT and satellite systems, Sateliot wishes to provide contextual information on the ongoing ITU-R studies under WRC-27 Agenda Item 1.13, which are currently being developed within ITU-R Study Group 4 (Working Party 4C). These ITU-R activities are examining possible new or modified MSS allocations (operating secondary to terrestrial IMT/MS), coexistence conditions between satellite and IMT user equipment, and the regulatory framework required for any future direct-to-device. The latter is now defined as Direct Connectivity from MSS to IMT UE (DC-MSS-IMT) in ITU-R nomenclature.

In this context, Sateliot considers it important to ensure that this proposal for an 3GPP scope of work remains aligned with the regulatory processes already underway in ITU-R, and to avoid conducting parallel or divergent technical studies that may not be consistent with the eventual WRC-27 outcomes. The timing and scope of NR-NTN work in MS bands should therefore consider that ITU-R will define the internationally harmonised regulatory conditions that govern satellite operation in these bands within 24 months.

Sateliot also notes the proposal in RP-252932 at RAN#109 for defining a single-country NR-NTN operating configuration in Band n7, and highlights that satellite use of adjacent MSS allocations in parts of Region 3 is subject to long-standing power flux-density (PFD) and out-of-band emission (OOBE) limits under ITU RR Article 21. These constraints, together with the need to ensure competitive neutrality between systems bound by ITU-R obligations and those operating under national MS rules, further reinforce the importance of relying on the WRC-27 AI 1.13 study cycle as the appropriate venue for determining future satellite use of MS bands today.

# 2. Discussion

During the discussions arising from RP-251803 at RAN#108 and the follow-up exchanges at RAN#109, a wide range of observations were made by participating companies. Several participants noted that introducing new NR-NTN operating configurations in MS allocations may be complex in jurisdictions where such use is not yet authorised, and where internationally harmonised regulatory guidance is still under development. Others highlighted that any potential new use of these bands would require careful examination of coexistence conditions to ensure that incumbent services (particularly those operating under MSS allocations) are not adversely affected.

Delegations also noted the close relationship between this topic and the work underway in ITU-R for WRC-27, emphasising the importance of taking regional and international regulatory frameworks into account when scoping NTN activities in 3GPP. In earlier meetings, some parties expressed concern regarding terminology and the potential for confusion between NTN and terrestrial operation, while others pointed to existing proprietary “direct-to-device” deployments in MSS-allocated bands as evidence of emerging market interest in standardised solutions. It was also observed that certain proponents are planning to deploy services in MSS bands in specific countries, including Australia, which further highlights the need for clarity around the regulatory environment governing such operations.

At RAN#109, a significant number of companies expressed reservations regarding the initiation of a WID at this stage, indicating that additional alignment and deeper study would be beneficial before commencing normative work. The RAN Chair has also noted that although no WID has been established, proponents gained a clear understanding of the types of considerations that would arise should such work proceed.

2.1 Regulatory context

The issue of satellite use within Mobile Service (MS) allocations is currently being examined in detail within the ITU-R study cycle established under WRC-27 Agenda Item 1.13 (Resolution 253 (WRC-23)). This agenda item tasks ITU-R Study Group 4 and Working Party 4C with assessing possible new or modified MSS allocations, the conditions for direct connectivity between space stations and IMT user equipment (DC -MSS-IMT), and the regulatory and technical framework required to support such use cases. These studies form the basis for any potential international decisions at WRC-27 concerning secondary MSS operation in MS bands.

In this context, the following points are of relevance:

a. There is clear industry interest in exploring NR-NTN use of MS bands, particularly for emerging D2D, NTN-IoT, and supplemental coverage from space. These use cases highlight the long-term potential for satellite and terrestrial IMT systems to operate in a complementary manner, subject to appropriate regulatory conditions.

b. Several regulatory, coexistence, and cross-border issues raised during previous RAN meetings remain unresolved. These include the relationship with existing MSS allocations, compatibility with adjacent-band IMT systems, variations in national UL/DL arrangements, and scenarios where MSS directionality differs from potential NTN configurations proposed for MS bands.

c. All bands currently mentioned in RAN discussions (n25, n7, n1) fall directly within the scope of the ITU-R studies mandated under Agenda Item 1.13. The outcomes of these studies will determine whether satellite systems may operate in these MS bands, and under what regulatory and technical conditions such operation would be internationally harmonised.

d. Given this ongoing study cycle, any conclusion within 3GPP on new NTN band definitions may risk misalignment with the international regulatory framework.

e. Aligning the sequencing of 3GPP work with the results of WRC-27 can help prevent country-by-country fragmentation, avoid the need for subsequent revision, and ensure that any NR-NTN solution reflects the conditions ultimately adopted by ITU-R.

f. Continued discussion in 3GPP is valuable.

g. Initiating normative work such as a WID would be most productive once the international regulatory framework is clarified through WRC-27, consistent with the long-standing cooperation and alignment between ITU-R and 3GPP processes.

ITU-R SG4 relevant documents on this topic are:

- Annex 11 to WP4C Chair´s Report

- Annex 8 - Working document on sharing and compatibility studies under WRC-27 agenda item 1.13

- Annex 9 Working document on the possible description and functionality of MSS systems for direct connectivity between space stations and IMT user equipment

- Annex 17 - Supporting material on the MSS Protection Criteria to be used in sharing and compatibility studies within the context of WRC-27 agenda items 1.12, 1.13, and 1.14

2.2 Technical coexistence and directionality

Coexistence between MSS and MS systems supporting complementary NTN services can be technically achievable provided that appropriate protection mechanisms are applied.

At the same time, coexistence scenarios involving two MSS systems operating with opposite UL and DL directions in the same or adjacent bands present significantly greater challenges.

This is particularly relevant in parts of ITU Region 3, where the MSS allocations overlap with portions of the frequency range corresponding to Band n7. In these cases, the coexistence problem spans satellite link budgets, cross-border emissions, regulated MSS protection requirements, and interaction with IMT systems in adjacent allocations. These considerations may extend well beyond what can be resolved within minimal 3GPP technical specifications alone.

For determining the necessary regulatory and technical conditions for such coexistence (including UL/DL directionality constraints and the shared use of MS bands by IMT and satellite systems), the ITU via Resolution 253 (WRC-23) mandated SG4 to study this matter under Agenda Item 1.13. This process is specifically mandated to assess the feasibility of MSS or satellite-IMT coexistence in these bands and to establish harmonised international conditions where appropriate.

Additional factors that warrant careful consideration when discussing potential NTN operation in a single national context include:

- differing regional UL/DL arrangements,

- existing MSS operations in adjacent territories,

- in relation to the above: the global nature of satellite beams,

- international harmonisation of IMT/NTN standards,

- and the long-term evolution of D2D and NTN under WRC-27.

In light of these elements, this information document notes that defining a country-specific NTN band configuration could unintentionally lead to similar nationally motivated standards, or de facto specialised or proprietary profile that diverges from the global interoperability objectives of 3GPP, and may require substantial revision once the outcomes of WRC-27 become available.

Technical considerations for non-conforming NTN in Band n7

Within the ITU Radio Regulations, portions of the 2500–2690 MHz range corresponding to 3GPP Band 7 include a primary Mobile-Satellite Service (MSS) allocation in Region 3 (2500–2520 MHz space-to-Earth and 2670–2690 MHz Earth-to-space). While several Region 3 administrations implement this MSS allocation, others, including Australia, have designated the entire 2500–2690 MHz range for IMT under national regulations.

Consequently, any NTN use of Band 7 would have to remain compatible not only with national IMT frameworks such as Australia’s, but also with the MSS protection and coordination obligations that continue to apply for neighbouring Region 3 administrations where MSS is deployed.

Existing MSS systems operating under this allocation are required to comply with the full set of ITU-R regulatory constraints, including: protection of other primary services, adherence to coordination and notification procedures under Articles 9 and 11, compliance with applicable PFD and OOBE limits, as well as cross-border compatibility with adjacent IMT allocations.

Table 5 below is an extract of ITU´s WP4C Annex 8 on Working document on sharing and compatibility studies under WRC-27 agenda item 1.13, which illustrates the multiple interference scenarios under consideration for the NTN use of n7. Where DC-MSS-IMT reflects the service typology proposed for a WID.

TABLE 5

The interference scenarios in the frequency range 2500-2690 MHz

| № | Scenarios |
| --- | --- |
| In-band |  |
| 1 | From DC-MSS-IMT space station transmitters into HIBS BSs receivers |
| 2 | From DC-MSS-IMT space station transmitters into FS station receivers |
| 3 | From DC-MSS-IMT space station transmitters into MSS space station receivers in the opposite direction of transmission |
| 4 | From DC-MSS-IMT IMT UE transmitters into MSS earth station receives in the opposite direction of transmission under maritime scenarios |
| 5 | From DC-MSS-IMT space station transmitters into BSS earth station receivers |
| 6 | From DC-MSS-IMT space station transmitters into BSS space station receivers |
| Adjacent-band |  |
| 7 | From DC-MSS-IMT space station transmitters into ARNS station receivers |
| 8 | From DC-MSS-IMT space station transmitters into RAS station receivers |
| 9 | From DC-MSS-IMT space station transmitters into MSS Earth station receivers |

By contrast, introducing a new non-conforming (Art.4.4 of RR) satellite-based NTN service (de facto MSS) in the same frequency range on the basis of a national Mobile Service allocation, without relying on the MSS allocation or bearing equivalent ITU-R obligations, would create a regulatory asymmetry, and operate contrary to the provisions in Article 21 of ITU´s Radio Regulations.

Such an arrangement would effectively modify the operational nature of satellite use in this band for a subset of privileged systems, while existing MSS operators remain bound by stricter international constraints. This may raise concerns of potential anti-competitive effects and an uneven regulatory playing field between MSS systems operating under the Radio Regulations and NTN/MS deployments authorised only at national level.

2.3 Current national frameworks

Today, regulatory frameworks enabling NTN in MS bands are highly varied: in some cases experimental and – in most – operating under no-protection/no-interference conditions (RR No. 4.4) which risks commercial and regulatory certainty (this unplanned use of Art. 4.4 in also being considered within ITU Satellite issues in the current cycle).

In others, this type of deployment is explicitly restricted or forbidden. These frameworks and associated concerns may become obsolete once ITU-R develops harmonized regulatory recommendations at WRC-27 that every country and associated technical groups may be guided by.

2.4 ITU and 3GPP cooperation

There is an existing and productive coordination between ITU and 3GPP on IMT development. Most recently, 3GPP Release 17 NR-NTN was formally accepted into the IMT-2020 family as the satellite component of IMT (see ITU-R M.2543 (2024). This precedent highlights the importance of ensuring that 3GPP work follows, and complements, ITU-R decisions. Equally, Release 17 (NTN) and posterior standards have been developed to leverage globally harmonized MSS allocations.

# 3. Conclusion and Observations

This contribution is aimed to provide contextual information to assist RAN in understanding the broader technical and regulatory context as work on this topic continues to evolve. The main points raised in the contribution are captured under the following observations:

Observation 1: Recent discussions in 3GPP RAN TSG and the ongoing work in ITU-R under WRC-27 Agenda Item 1.13 show that NR-NTN use of MS bands intersects with several regulatory and technical considerations that are still under study internationally.

Observation 2: National regulatory frameworks enabling satellite or NTN operation in these bands vary significantly across administrations, with many relying on temporary, experimental or no-protection conditions (limiting the services to be provided) that may change following the completion of WRC-27 studies.

Observation 3: The timing of related work has also been noted in 3GPP RAN TSG discussions, given that the ITU-R study cycle under WRC-27 Agenda Item 1.13 is still in progress and is expected to clarify the regulatory framework for potential satellite use of MS bands.

Observation 4: The following elements reflect the broader context observed today:

- A detailed regulatory examination is currently underway in ITU-R, through the study cycle associated with WRC-27 Agenda Item 1.13, which is assessing potential new or modified MSS allocations and the conditions for direct satellite-to-IMT operation in Mobile Service bands.

- Temporary national frameworks governing satellite use of these bands remain heterogeneous, with differing authorisation conditions, allocation structures and protection requirements, and may evolve as ITU-R studies progress.

- Coexistence considerations involving MSS, IMT and NTN systems are being analysed in ITU-R, including directionality, adjacent-band protection and the relationship between existing MSS allocations and future NTN configurations.

- Different uplink and downlink arrangements across ITU regions and administrations have been highlighted in RAN, together with the relevance of ensuring compatibility with existing MSS deployments in adjacent territories.

- The long-standing cooperation between 3GPP and ITU-R has been noted, particularly in relation to NR-NTN’s integration into the IMT-2020 ecosystem and the role of ITU-R M.1036 in defining harmonised band arrangements.

- The cross-border nature of satellite systems and the emerging global market for NTN and D2D services have also been referenced as contextual factors relevant to ongoing international studies.

4. References

[1] ITU-R Resolution 245 (WRC-23): “Studies on the use of Mobile Service allocations for the satellite component of IMT (Agenda Item 1.13, WRC-27)”
[2] ITU Radio Regulations, RR No. 4.4
[3] ITU-R Resolution 253 (WRC-19, revised WRC-23): “Satellite radio interface(s) for IMT-2020”
[4] ITU-R Report M.2543 (2023): Outcome of evaluation and decision of the IMT-2020 satellite process

[5] ITU Radio Regulations, Articles 5, 9, 11 and 21

[6] 3GPP RP-251803 “NR-NTN support in terrestrial spectrum bands” Telstra, SpaceX, T-Mobile, KDDI

[7] RAN#108 Meeting Report, 13th June 2025, RP-250613, TSG-RAN Chair

[8] WP4C Chair´s Report (Fall 2025)