3GPP TSG RAN Meeting #110		RP-253706

Baltimore, USA, 8th – 11th December 2025

Agenda item:	9.1.5.1

Source:	Apple, Viasat, Terrestar, Globalstar, Skyworks Solutions, Iridium, Echostar, MediaTek

Title:	Motivation for the introduction of the higher power classes, 3MHz channel bandwidth and eRedCap for the existing NR/IoT NTN bands

Release:	Rel-20

Document for:	Information

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. That time the only available power class for NTN was PC3 offering the maximum nominal output power of 23dBm. During the RAN#104 meeting a new WI was agreed [2] with one of the objectives to enable higher power classes for NTN. The corresponding WI was completed in August 2025 introducing core RF requirements for the PC2 (26dBm) and PC1 (31dBm) power classes for both NR and IOT NTN bands. However, only two exemplary bands – L-band n255 and S-band n256 – were considered for the band specific requirements.

Furthermore, Rel-19 also introduced support for the 3MHz channel for NTN, but it was enabled only for several bands, such as n254, n255 and n256, which were specified before Rel-19. As a result, certain bands, such as n252, n251 and n253 do not have support to 3MHz channel BW. The corresponding A-MPR values were defined only for the PC3 power class. In other words, for some bands such as n255, PC2/PC1 is defined for the 5MHz and larger channels, but not for 3MHz.

Thus, in this discussion paper we present a general overview on which existing NR and IOT NTN bands need further specification efforts to enable missing features for the existing bands, such as high-power classes and the 3MHz channel bandwidth.

# 2	Missing features for the existing NTN bands

Table 2-1 below summarises existing NTN bands, both NR and IOT, with further information on the release when the band was introduced and which power classes are defined. As can be seen from the table, the Rel-19 NTN high-power class WI enabled PC2 and PC1 only for the "exemplary" bands n255 and n256, which were introduced back in Rel-17. Other Rel-18 and brand new Rel-19 bands (such as n252 or b249) are not covered by the 3GPP specifications.

Table 2-1: Summary of the existing NTN NR/IOT bands and the NTN power classes.

| NTN band |  | Release | PC3 | PC2 | PC1 |
| --- | --- | --- | --- | --- | --- |
| NR | n255 | Rel-17 | X | X | X |
|  | n256 | Rel-17 | X | X | X |
|  | n254 | Rel-18 | X |  |  |
|  | n252 | Rel-19 | X |  |  |
|  | n253 | Rel-19 | X |  |  |
|  | n250 | Rel-19 | X |  |  |
|  | n251 | Rel-19 | X |  |  |
| IOT | b255 | Rel-17 | X | X | X |
|  | b256 | Rel-17 | X | X | X |
|  | b254 | Rel-18 | X |  |  |
|  | b253 | Rel-18 | X |  |  |
|  | b252 | Rel-19 | X |  |  |
|  | b249 | Rel-19 | X |  |  |

As a somewhat related aspect, Rel-19 also introduced support for the 3MHz channel for NTN, but it was enabled only for several bands, such as n254, n255 and n256, which were specified before Rel-19. Furthermore, the corresponding A-MPR values, when needed, were defined only for the PC3 power class. In other words, for some bands such as n255, PC2/PC1 is defined for the 5MHz and larger channels, but not for 3MHz.

Table 2-2: Summary of the 3MHz channel support for the NTN NR bands and power classes.

| NTN band |  | Release | PC3 | PC2 | PC1 |
| --- | --- | --- | --- | --- | --- |
| NR | n255 | Rel-17 | X |  |  |
|  | n256 | Rel-17 | X |  |  |
|  | n254 | Rel-18 | X |  |  |
|  | n252 | Rel-19 |  |  |  |
|  | n253 | Rel-19 |  |  |  |
|  | n250 | Rel-19 |  |  |  |
|  | n251 | Rel-19 |  |  |  |

The table below also summarises current NTN bands with further clarifications on the scope of work for each band. For instance, bands 255/n255 and 256/n256 already have HPUE for basic channel bandwidths and 3MHz, but HPUE for 3MHz is not defined. As for the other bands, HPUE needs to be defined for all applicable channel bandwidths including baseline PC3 3MHz.

Table 2-3: Summary of considered bands and supporting companies.

| NTN band | Supporting operator | Detailed scope |
| --- | --- | --- |
| 249 | Iridium | IOT HPUE |
| 252/n252 | Terrestar, Echostar | IOT/NR HPUE (with PC3 3MHz), eRedCap |
| 253/n253 | Viasat/Inmarsat | IOT/NR HPUE (with PC3 3MHz), eRedCap |
| 254/n254 | Globalstar | IOT/NR HPUE (with PC3 3MHz) |
| 255/n255 | Viasat/Inmarsat | HPUE 3MHz |
| n250 | Viasat/Inmarsat | HPUE 3MHz, eRedCap |
| n251 | Viasat/Inmarsat | HPUE 3MHz, eRedCap |
| 256/n256 |  | HPUE 3MHz |

Thus, we suggest initiating a new Rel-20 spectrum "basket" WI aiming at enabling missing NTN power classes in the existing NTN bands. The potential objectives of the WI may look as presented in [3], where the exact list of bands will be clarified further with interested companies and satellite operators.

Proposal:	Initiate a new Rel-20 basket WI to enable missing NTN features for the existing NTN NR/IOT bands covering the 3MHz channel, high power classes and eRedCap.

# 3	References

RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"

RP-253707, "Draft WID for the Introduction of the higher power classes, 3MHz channel bandwidth and eRedCap for the existing NR/IoT NTN bands", Apple, Viasat, Terrestar, Globalstar, Skyworks Solutions, Iridium, Echostar, MediaTek