3GPP RAN TSG Meeting #110		RP-253708

Baltimore, USA, December 8th – 11th, 2025

Agenda item:	9.1.5.1

Source:	Apple

Title:	Views on HPUE band combinations scope for Rel-20 basket work items

WI/SI:

Release:	Rel-20

Document for:	Discussion and decision

# 1	Introduction

The Rel-19 spectrum basket work items had been concluded in last RAN4 meeting (#117). In preparation for the Rel-20 spectrum basket work items which will be approved in this plenary meeting, RAN4 has discussed the planning in last RAN4 meeting and concluded with a list of recommended basket work items for Rel-20 [1]. Compared to the Rel-19 basket work items, some of the continuing baskets were decided to be merged together to improve the handling efficiency, in particular, the HPUE band combinations which would be merged into the corresponding legacy PC3 band combination baskets for NR CA/DC/SUL and EN-DC respectively. On the other hand, the scope of the Rel-20 HPUE band combinations basket may potentially be reduced as compared to its Rel-19 counterparts, thanks to the introduction of the HPUE MSD LUTs (Look-Up-Tables) in Rel-19. In this contribution, we share our views on the HPUE band combinations scope for Rel-20 basket work items.

# 2	Discussion

HPUE in principle can be supported for any band combinations provided the corresponding UL configurations have been specified in Clause 6 of the technical specifications [2,3]. However, when considering the UL impact to the crossed DL bands REFSENS due to the higher transmission power for HPUE than PC3, the combination cannot be specified to support HPUE till the corresponding PC2 or PC1.5 MSD requirement is defined. And such HPUE MSD evaluations have been the major efforts in HPUE band combinations basket work items in the past few releases. On the other hand, to ensure the HPUE MSD requirements are specified for any DL band combinations intended to support HPUE, RAN4 has introduced the so-called HPUE applicability notes in the band combination configuration tables since Rel-17. Though the intention for the HPUE applicability notes is to indicate that the required PC2 or PC1.5 MSD requirement has been specified, to maintain the specifications consistency, the notes are also needed for the DL combinations even without MSD impact. Furthermore, it is well understood that for single UL with three or more DL bands, there is no need to specify MSD requirements as they have been covered in the fallback 2-band DL combinations. Similarly, for 2-band UL with four or more DL bands, there is also no need to specify MSD requirements as they have been covered in the fallback 3-band DL combinations. Nevertheless, there were still many redundant efforts in the past HPUE basket work items to simply add the HPUE applicability notes in the higher order band combination configuration tables.

Observation 1: HPUE in principle can be supported for any band combinations provided the corresponding UL configurations have been specified in Clause 6 of the technical specifications.

Observation 2: Considering the UL impact to the crossed DL bands REFSENS due to the higher transmission power for HPUE than PC3, the combination cannot be specified to support HPUE till the corresponding PC2 or PC1.5 MSD requirement is defined.

Observation 3: HPUE MSD evaluations have been the major efforts in HPUE band combinations basket work items in the past few releases.

Observation 4: To ensure HPUE MSD requirements are specified for any DL band combinations intended to support HPUE, RAN4 has introduced the so-called HPUE applicability notes in the band combination configuration tables since Rel-17.

Observation 5: There were many redundant efforts in the past HPUE basket work items to simply add the HPUE applicability notes in the higher order band combination configuration tables.

While RAN4 may have seemed accustomed to the process of adding the PC2 and PC1.5 applicability notes and MSD requirements for the band combinations intended to support HPUE, some deficiency was also identified in the current HPUE introduction process which has been alluded in [4] and also led to a RAN task to mitigate part of the issues which is for DL CA combinations with single UL configured only. To resolve all the issues due to the lack of HPUE MSD requirements hindering the HPUE band combinations support, RAN4 made a great effort in Rel-19 to introduce HPUE MSD LUTs which have been completed in RAN4 #116 meeting [5]. With HPUE MSD LUTs, the existing HPUE applicability notes in band combination configuration tables in clause 5 of the RAN4 UE RF specifications [2,3] in principle are no longer needed. In addition, RAN4 should start moving away from the legacy HPUE basket approach after the HPUE MSD LUTs are introduced as the existing process is rather inefficient.

Observation 6: To resolve all the issues due to the lack of HPUE MSD requirements hindering the HPUE band combinations support, RAN4 made a great effort in Rel-19 to introduce HPUE MSD LUTs.

Observation 7: With HPUE MSD LUTs, the existing HPUE applicability notes in band combination configuration tables in clause 5 of the RAN4 UE RF specifications in principle are no longer needed.

Observation 8: RAN4 should start moving away from the legacy HPUE basket approach after the HPUE MSD LUTs are introduced as the existing process is rather inefficient.

Based on the above assessment, we can envision that the HPUE requests for three or more DL bands are no longer needed and the HPUE band combinations scope for Rel-20 basket work items can be concluded as below:

For NR CA/DC/SUL basket work item:

- Single-band intra-band UL/DL CA for both TDD bands and FDD bands

- A-MPR as necessary

- MSD for FDD bands

- Inter-band UL combinations with up to two DL bands

- Two UL bands up to 3 CCs

- Note: Only capture the UL configurations in Clause 6.

For EN-DC basket work item:

- Single-band intra-band EN-DC

- A-MPR as necessary

- MSD for FDD bands

- Inter-band EN-DC with up to two DL bands

- Two UL bands up to 3 CCs

- Note: Only capture the UL configurations in Clause 6.

# 3	Conclusion

In this contribution, we share our views on the HPUE band combinations scope for Rel-20 basket work items.

Observation 1: HPUE in principle can be supported for any band combinations provided the corresponding UL configurations have been specified in Clause 6 of the technical specifications.

Observation 2: Considering the UL impact to the crossed DL bands REFSENS due to the higher transmission power for HPUE than PC3, the combination cannot be specified to support HPUE till the corresponding PC2 or PC1.5 MSD requirement is defined.

Observation 3: HPUE MSD evaluations have been the major efforts in HPUE band combinations basket work items in the past few releases.

Observation 4: To ensure HPUE MSD requirements are specified for any DL band combinations intended to support HPUE, RAN4 has introduced the so-called HPUE applicability notes in the band combination configuration tables since Rel-17.

Observation 5: There were many redundant efforts in the past HPUE basket work items to simply add the HPUE applicability notes in the higher order band combination configuration tables.

Observation 6: To resolve all the issues due to the lack of HPUE MSD requirements hindering the HPUE band combinations support, RAN4 made a great effort in Rel-19 to introduce HPUE MSD LUTs.

Observation 7: With HPUE MSD LUTs, the existing HPUE applicability notes in band combination configuration tables in clause 5 of the RAN4 UE RF specifications in principle are no longer needed.

Observation 8: RAN4 should start moving away from the legacy HPUE basket approach after the HPUE MSD LUTs are introduced as the existing process is rather inefficient.

Proposal: RAN to consider the following HPUE band combinations scope for Rel-20 basket work items:

For NR CA/DC/SUL basket work item:

- Single-band intra-band UL/DL CA for both TDD bands and FDD bands

- A-MPR as necessary

- MSD for FDD bands

- Inter-band UL combinations with up to two DL bands

- Two UL bands with up to 3 CCs

- Note: Only capture the UL configurations in Clause 6.

For EN-DC basket work item:

- Single-band intra-band EN-DC

- A-MPR as necessary

- MSD for FDD bands

- Inter-band EN-DC with up to two DL bands

- Two UL bands with up to 3 CCs

- Note: Only capture the UL configurations in Clause 6.

# 4	References

R4-2522422 “Way forward on Rel-20 basket work items”, Moderator (Apple), 3GPP RAN WG4 Meeting #117, Dallas, USA, November 17th – 21st, 2025

3GPP TS 38.101-1 V19.3.1 (2025-09)

3GPP TS 38.101-3 V19.3.0 (2025-09)

R4-2400191 “On PC2 and PC1.5 indications in BC configuration tables”, Apple, 3GPP RAN WG4 Meeting #110, Athens, Greece, February 26th – March 1st, 2024

R4-2511752 “Draft CR to 38101-1-j20 on introducing the HPUE MSD requirements simplification”, Skyworks Solutions Inc., et al, 3GPP RAN WG4 Meeting #116, Bengaluru, India, August 25th – 29th, 2025