3GPP TSG-RAN WG2 Meeting #132		R2-2508256

Dallas, USA, 17th – 21st Nov., 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 2139 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on DSR triggering |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo, LG, Nokia, Nokia Shanghai Bell, Apple, Ericsson, Lenovo, Qualcomm, Sharp, OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_enh-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | According to the MAC spec, DSR triggering is determined by the smallest remaining time value of PDCP SDUs buffered for the logical channel that have not been transmitted in any MAC PDU. However, there may be cases where a PDCP SDU is segmented at the RLC layer, with some RLC SDU segments already transmitted in a MAC PDU while others remain in the buffer. According to the current specification, these remaining RLC SDU segments cannot trigger DSR because they are “PDCP SDUs that have been transmitted partially” while not qualitied as "PDCP SDUs that have not been transmitted in any MAC PDU." We think this kind of RLC SDU segments should also be able to trigger DSR if the remaining time is below the threshold. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify in the MAC that DSR can be triggerd for the PDCP SDUs with some segments have been transmitted in MAC PDU and some segments have not. Impact Analysis Impacted 5G architecture options: NR SA and MR-DC Impacted functionality: DSR Inter-operability: There is no interoperability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Some RLC SDU segments with remaining time below the remainingTimeThreshold cannot trigger DSR and results in latency. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.9, 6.1.3.72 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of change

### 5.4.9	Delay status reporting

The Delay Status Reporting (DSR) procedure is used to provide the serving gNB with delay status of LCGs. This delay status for an LCG includes remaining time, which is the smallest remaining value of the running PDCP discardTimers among PDCP SDUs, as specified in clause 7.3 in TS 38.323 [4], that are buffered for the LCG but have not been transmitted in MAC PDU(s), and the total amount of delay-critical UL data for the LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively.

RRC controls the DSR procedure by configuring the following parameter:

-	remainingTimeThreshold (per LCG): the threshold on remaining time for triggering a DSR for a logical channel within an LCG.

If an LCG is configured for delay status reporting, the MAC entity shall for each logical channel within the LCG:

1>	if the smallest remaining value of the running PDCP discardTimers among all the PDCP SDUs buffered for the logical channel that have not been transmitted in MAC PDU(s) and have not been reported as data volume in a DSR MAC CE becomes below remainingTimeThreshold of the LCG; and

1>	if there is no DSR pending for the logical channel:

2>	trigger a DSR for the logical channel.

If there is at least one DSR pending, the MAC entity shall:

1>	if UL-SCH resources are available for a new transmission and the UL-SCH resources can accommodate the DSR MAC CE plus its subheader as a result of logical channel prioritization:

2>	instruct the Multiplexing and Assembly procedure to generate the DSR MAC CE as specified in clause 6.1.3.72.

1>	else if there is no pending SR already triggered by the DSR procedure for the same logical channel as of this DSR:

2>	trigger a Scheduling Request.

NOTE 1:	The availability of UL-SCH resources for the transmission of the DSR MAC CE follows the same criteria specified in clause 5.4.5.

A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in MAC PDU(s) and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR.

A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain a DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs.

After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

NOTE 2:	It is up to UE implementation whether the MAC entity includes a DSR MAC CE in a MAC PDU if the MAC PDU can accommodate all PDCP SDUs associated with all the pending DSRs but is not sufficient to additionally accommodate the DSR MAC CE plus its subheader.

NOTE X:	For DSR, a PDCP SDU is not considered to have been transmitted in MAC PDU(s) if at least one of its segments has not been transmitted in any MAC PDU.

Next change

#### 6.1.3.72	Delay Status Report MAC CE

The Delay Status Report (DSR) MAC CE is identified by MAC subheader with an eLCID as specified in Table 6.2.1-2b.

The fields in the DSR MAC CE are defined as follows:

-	LCGi: This field indicates the presence of delay information (i.e. the Remaining Time and Buffer Size fields) for the LCG i. The LCGi field set to 1 indicates that the delay information for the LCG i is reported. The LCGi field set to 0 indicates that the delay information for the LCG i is not reported;

-	Remaining Time: This field indicates the shortest remaining value of running PDCP discardTimer (described in clause 7.3 in TS 38.323 [4]) among all PDCP SDUs that are buffered for an LCG but have not been transmitted in MAC PDU(s), at the time of the first symbol of the first PUSCH transmission that includes this DSR MAC CE. The length of this field is 6 bits. This field is present only if the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the value r in this field indicates a remaining time within the range of (r, r + 1] msec;

-	BT: This field is present only if the corresponding LCG is configured with additionalBS-TableAllowed and the buffer size indicated by the corresponding Buffer Size field is not zero; otherwise, this field is reserved and set to 0. If present, the BT field set to 1 indicates that the buffer sizes specified in Table 6.1.3.1-3 are used to set the value of the Buffer Size field, while the BT field set to 0 indicates that the buffer sizes specified in Table 6.1.3.1-2 are used instead;

-	Buffer Size: The Buffer Size field indicates the total amount of delay-critical UL data for an LCG according to the data volume calculation procedure specified in clause 5.5 in TS 38.322 [3] and clause 5.15 in TS 38.323 [4] for the associated RLC and PDCP entities, respectively, after the MAC PDU has been built. If the corresponding LCG is configured with additionalBS-TableAllowed and the amount of delay-critical UL data for an LCG is within the buffer sizes specified in Table 6.1.3.1-3, the MAC entity shall use the buffer sizes specified in Table 6.1.3.1-3 to set the value of this field; otherwise, the MAC entity shall use Table 6.1.3.1-2 instead. This field is indicated in number of bytes. The length of this field is 8 bits;

-	R: Reserved bit, set to 0.

The DSR MAC CE shall include delay information of all LCGs which have pending DSRs when the MAC PDU containing this DSR MAC CE is to be built. The Remaining Time, the BT, and the Buffer Size fields for an LCG shall be reported in two consecutive octets. These three fields for different LCGs shall be included in a DSR MAC CE in ascending order based on the LCGi.

Figure 6.1.3.72-1: DSR MAC CE

End of change