3GPP TSG-RAN WG2 Meeting #132	R2-2508800

# Dallas, USA, Nov 17th – 21st, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.331 | CR | 5605 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on uplink power control for Type-1 CG-PUSCH [PL RS Type 1 CG] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ofinno |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-11-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In TEI18, RAN1 introduced a new RRC parameter (i.e. enablePL-RS-UpdateForType1CG-PUSCH) to support MAC CE based PL-RS updates for Type 1 CG-PUSCH. In RAN2#129, this new parameter enablePL-RS-UpdateForType1CG-PUSCH in ServingCellConfig was introduced. In 38.213, when enablePL-RS-UpdateForType1CG-PUSCH is provided, srs-ResourceIndicator value included in rrc-ConfiguredUplinkGrant selects an element among the list of SRI-PUSCH-PowerControl elements. However, in the current TS 38.331, only SRI field in DCI can select an element among the list of SRI-PUSCH-PowerControl elements. It needs to be clarified that an element among the list of SRI-PUSCH-PowerControl elements can also be selected using srs-ResourceIndicator value included in rrc-ConfiguredUplinkGrant when UE is configured with enablePL-RS-UpdateForType1CG-PUSCH. Impact analysis Impacted 5G architecture options: NR SA, (NG)EN-DC, NE-DC, NR-DC Impacted functionality: UL power control for PL RS Type 1 CG Inter-operability: If the network is implemented according to the CR and the UE is not, the network configures the UE with RS selection parameter (e.g., srs-ResourceIndicator) to select an element among the list of SRI-PUSCH-PowerControl elements, but the UE cannot select an element among the list of SRI-PUSCH-PowerControl elements. If the UE is implemented according to the CR and the network is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the limitation that an element among the list of SRI-PUSCH-PowerControl elements is selected only by the SRI field in DCI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE may be configured with UL power control parameters for PL RS determination for Type 1 CG that are inconsistent from a UE perspective, possibly resulting in RRC reestablishment or unclear UE behaviour. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Changes

#### –	PUSCH-PowerControl

The IE PUSCH-PowerControl is used to configure UE specific power control parameter for PUSCH.

PUSCH-PowerControl information element

-- ASN1START

-- TAG-PUSCH-POWERCONTROL-START

PUSCH-PowerControl ::=              SEQUENCE {

tpc-Accumulation                    ENUMERATED { disabled }                                                 OPTIONAL, -- Need S

msg3-Alpha                          Alpha                                                                   OPTIONAL, -- Need S

p0-NominalWithoutGrant              INTEGER (-202..24)                                                      OPTIONAL, -- Need M

p0-AlphaSets                        SEQUENCE (SIZE (1..maxNrofP0-PUSCH-AlphaSets)) OF P0-PUSCH-AlphaSet     OPTIONAL, -- Need M

pathlossReferenceRSToAddModList     SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS

OPTIONAL, -- Need N

pathlossReferenceRSToReleaseList    SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs)) OF PUSCH-PathlossReferenceRS-Id

OPTIONAL,  -- Need N

twoPUSCH-PC-AdjustmentStates        ENUMERATED {twoStates}                                                  OPTIONAL, -- Need S

deltaMCS                            ENUMERATED {enabled}                                                    OPTIONAL, -- Need S

sri-PUSCH-MappingToAddModList       SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControl

OPTIONAL, -- Need N

sri-PUSCH-MappingToReleaseList      SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControlId

OPTIONAL  -- Need N

}

P0-PUSCH-AlphaSet ::=               SEQUENCE {

p0-PUSCH-AlphaSetId                 P0-PUSCH-AlphaSetId,

p0                                  INTEGER (-16..15)                                                       OPTIONAL, -- Need S

alpha                               Alpha                                                                   OPTIONAL  -- Need S

}

P0-PUSCH-AlphaSetId ::=             INTEGER (0..maxNrofP0-PUSCH-AlphaSets-1)

PUSCH-PathlossReferenceRS ::=       SEQUENCE {

pusch-PathlossReferenceRS-Id        PUSCH-PathlossReferenceRS-Id,

referenceSignal                     CHOICE {

ssb-Index                           SSB-Index,

csi-RS-Index                        NZP-CSI-RS-ResourceId

}

}

PUSCH-PathlossReferenceRS-r16 ::=   SEQUENCE {

pusch-PathlossReferenceRS-Id-r16    PUSCH-PathlossReferenceRS-Id-v1610,

referenceSignal-r16                 CHOICE {

ssb-Index-r16                       SSB-Index,

csi-RS-Index-r16                    NZP-CSI-RS-ResourceId

}

}

DummyPathlossReferenceRS-v1710 ::= SEQUENCE {

pusch-PathlossReferenceRS-Id-r17    PUSCH-PathlossReferenceRS-Id-r17,

additionalPCI-r17                   AdditionalPCIIndex-r17                                         OPTIONAL  -- Need R

}

PUSCH-PathlossReferenceRS-Id ::=    INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1)

PUSCH-PathlossReferenceRS-Id-v1610 ::= INTEGER (maxNrofPUSCH-PathlossReferenceRSs..maxNrofPUSCH-PathlossReferenceRSs-1-r16)

PUSCH-PathlossReferenceRS-Id-r17 ::= INTEGER (0..maxNrofPUSCH-PathlossReferenceRSs-1-r16)

SRI-PUSCH-PowerControl ::=          SEQUENCE {

sri-PUSCH-PowerControlId            SRI-PUSCH-PowerControlId,

sri-PUSCH-PathlossReferenceRS-Id    PUSCH-PathlossReferenceRS-Id,

sri-P0-PUSCH-AlphaSetId             P0-PUSCH-AlphaSetId,

sri-PUSCH-ClosedLoopIndex           ENUMERATED { i0, i1 }

}

SRI-PUSCH-PowerControlId ::=        INTEGER (0..maxNrofSRI-PUSCH-Mappings-1)

PUSCH-PowerControl-v1610 ::=        SEQUENCE {

pathlossReferenceRSToAddModListSizeExt-v1610   SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSsDiff-r16)) OF PUSCH-PathlossReferenceRS-r16

OPTIONAL, -- Need N

pathlossReferenceRSToReleaseListSizeExt-v1610  SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSsDiff-r16)) OF PUSCH-PathlossReferenceRS-Id-v1610

OPTIONAL, -- Need N

p0-PUSCH-SetList-r16                SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF P0-PUSCH-Set-r16      OPTIONAL, -- Need R

olpc-ParameterSet                   SEQUENCE {

olpc-ParameterSetDCI-0-1-r16        INTEGER (1..2)                                                      OPTIONAL, -- Need R

olpc-ParameterSetDCI-0-2-r16        INTEGER (1..2)                                                      OPTIONAL  -- Need R

}                                                                                                           OPTIONAL, -- Need M

...,

[[

sri-PUSCH-MappingToAddModList2-r17  SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControl

OPTIONAL, -- Need N

sri-PUSCH-MappingToReleaseList2-r17 SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF SRI-PUSCH-PowerControlId OPTIONAL, -- Need N

p0-PUSCH-SetList2-r17               SEQUENCE (SIZE (1..maxNrofSRI-PUSCH-Mappings)) OF P0-PUSCH-Set-r16      OPTIONAL, -- Need R

dummy SEQUENCE (SIZE (1..maxNrofPUSCH-PathlossReferenceRSs-r16)) OF DummyPathlossReferenceRS-v1710          OPTIONAL  -- Need N

]]

}

P0-PUSCH-Set-r16 ::=                SEQUENCE {

p0-PUSCH-SetId-r16                  P0-PUSCH-SetId-r16,

p0-List-r16                         SEQUENCE (SIZE (1..maxNrofP0-PUSCH-Set-r16)) OF P0-PUSCH-r16            OPTIONAL, -- Need R

...

}

P0-PUSCH-SetId-r16 ::=              INTEGER (0..maxNrofSRI-PUSCH-Mappings-1)

P0-PUSCH-r16 ::=                    INTEGER (-16..15)

-- TAG-PUSCH-POWERCONTROL-STOP

-- ASN1STOP

P0-PUSCH-AlphaSet field descriptions

alpha alpha value for PUSCH with grant (except msg3) (see TS 38.213 [13], clause 7.1). When the field is absent the UE applies the value 1.

p0 P0 value for PUSCH with grant (except msg3) in steps of 1dB (see TS 38.213 [13], clause 7.1). When the field is absent the UE applies the value 0.

P0-PUSCH-Set field descriptions

p0-List Configuration of {p0-PUSCH, p0-PUSCH} sets for PUSCH. If SRI is present in the DCI, then one p0-PUSCH can be configured in P0-PUSCH-Set. If SRI is not present in the DCI, and both olpc-ParameterSetDCI-0-1 and olpc-ParameterSetDCI-0-2 are configured to be 1 bit, then one p0-PUSCH can be configured in P0-PUSCH-Set. If SRI is not present in the DCI, and if any of olpc-ParameterSetDCI-0-1 and olpc-ParameterSetDCI-0-2 is configured to be 2 bits, then two p0-PUSCH values can be configured in P0-PUSCH-Set (see TS 38.213 [13] clause 7 and TS 38.212 [17] clause 7.3.1).

p0-PUSCH-SetId Configure the index of a p0-PUSCH-Set (see TS 38.213 [13] clause 7 and TS 38.212 [17] clause 7.3.1).

PUSCH-PowerControl field descriptions

deltaMCS Indicates whether to apply delta MCS. When the field is absent, the UE applies Ks = 0 in delta_TFC formula for PUSCH (see TS 38.213 [13], clause 7.1).

dummy This field is not used in the specification. If received it shall be ignored by the UE.

msg3-Alpha Dedicated alpha value for msg3 PUSCH (see TS 38.213 [13], clause 7.1). When the field is absent the UE applies the value 1.

olpc-ParameterSetDCI-0-1, olpc-ParameterSetDCI-0-2 Configures the number of bits for Open-loop power control parameter set indication for DCI format 0_1/0_2 in case SRI is not configured in the DCI. 2 bits is applicable only if SRI is not present in the DCI format 0_1. The field olpc-ParameterSetDCI-0-1 applies to DCI format 0_1 and the field olpc-ParameterSetDCI-0-2 applies to DCI format 0_2 (see TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 11).

p0-AlphaSets Configuration {p0-pusch, alpha} sets for PUSCH (except msg3 and msgA PUSCH), i.e., { {p0,alpha,index1}, {p0,alpha,index2},...} (see TS 38.213 [13], clause 7.1). When no set is configured, the UE uses the P0-nominal for msg3/msgA PUSCH, P0-UE is set to 0 and alpha is set according to either msg3-Alpha or msgA-Alpha (see TS 38.213 [13], clause 7.1). This field is not configured if unifiedTCI-StateType is configured for the serving cell.

p0-NominalWithoutGrant P0 value for UL grant-free/SPS based PUSCH. Value in dBm. Only even values (step size 2) allowed (see TS 38.213 [13], clause 7.1).

p0-PUSCH-SetList Configure one additional P0-PUSCH-Set per SRI. If present, the one bit or 2 bits in the DCI is used to dynamically indicate among the P0 value from the existing P0-PUSCH-AlphaSet and the P0 value(s) from the P0-PUSCH-Set (See TS 38.212 [17], clause 7.3.1 and TS 38.213 [13], clause 17).

p0-PUSCH-SetList2 For indicating per-TRP OLPC set in DCI format 0_1/0_2 with the open-loop power control parameter set indication field, a second p0-PUSCH-SetList-r16 is used. When this field is present the p0-PUSCH-SetList-r16 corresponds to the first SRS resource set (see TS 38.213 [13]).

pathlossReferenceRSToAddModList, pathlossReferenceRSToAddModListSizeExt A set of Reference Signals (e.g. a CSI-RS config or a SS block) to be used for PUSCH path loss estimation. The set consists of Reference Signals configured using pathLossReferenceRSToAddModList and Reference Signals configured using pathlossReferenceRSToAddModListSizeExt. Up to maxNrofPUSCH-PathlossReferenceRSs may be configured (see TS 38.213 [13], clause 7.1). No elements in the list pathLossReferenceRSToAddModList nor pathlossReferenceRSToAddModListSizeExt are configured if unifiedTCI-StateType is configured for the serving cell.

pathlossReferenceRSToReleaseList, pathlossReferenceRSToReleaseListSizeExt Lists of reference signals for PUSCH path loss estimation to be released by the UE.

sri-PUSCH-MappingToAddModList A list of SRI-PUSCH-PowerControl elements among which one is selected (see TS 38.213 [13], clause 7.1). No elements in this list are configured if unifiedTCI-StateType is configured for the serving cell.

sri-PUSCH-MappingToAddModList2 A list of SRI-PUSCH-PowerControl elements for second SRS-resource set, among which one is selected by the SRI field in DCI (see TS 38.213 [13], clause 7.1). When this field is present the sri-PUSCH-MappingToAddModList corresponds to the first SRS resource set for PUSCH. No elements in this list are configured if unifiedTCI-StateType is configured for the serving cell.

tpc-Accumulation If enabled, UE applies TPC commands via accumulation. If not enabled, UE applies the TPC command without accumulation. If the field is absent, TPC accumulation is enabled (see TS 38.213 [13], clause 7.1).

twoPUSCH-PC-AdjustmentStates Number of PUSCH power control adjustment states maintained by the UE (i.e., fc(i)). If the field is present (n2) the UE maintains two power control states (i.e., fc(i,0) and fc(i,1)). If the field is absent, it maintains one power control state (i.e., fc(i,0)) (see TS 38.213 [13], clause 7.1).

SRI-PUSCH-PowerControl field descriptions

sri-P0-PUSCH-AlphaSetId The ID of a P0-PUSCH-AlphaSet as configured in p0-AlphaSets in PUSCH-PowerControl.

sri-PUSCH-ClosedLoopIndex The index of the closed power control loop associated with this SRI-PUSCH-PowerControl.

sri-PUSCH-PathlossReferenceRS-Id The ID of PUSCH-PathlossReferenceRS as configured in the pathlossReferenceRSToAddModList in PUSCH-PowerControl.

sri-PUSCH-PowerControlId The ID of this SRI-PUSCH-PowerControl configuration. It is used as the codepoint (payload) in the SRI DCI field.

End of Changes