3GPP TSG RAN Meeting #110														draft RP-253799

Baltimore, USA, December 8-11, 2025

Title:	Draft Reply LS to ECC_PT1(25)171_Annex04 = RP-252992 on Parameters for AAS BS operating in bands below 1 GHz (to: ECC PT1; cc: ECC WG SE, ECC WG SE21, ETSI TC ERM, ETSI TC MSG/TFES, RAN4; cc: -; contact: Huawei)

Response to:	LS RP-252992 on Parameters for AAS BS operating in bands below 1 GHz from CEPT ECC PT1

Release:	Rel-15

Work Item:	-

Source:	TSG RAN

To:	CEPT ECC PT1

Cc:	ECC WG SE, ECC WG SE21, ETSI TC ERM, ETSI TC MSG/TFES, RAN4

Contact person:	Michal Szydelko

michal.szydelko@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

3GPP TSG RAN would like to thank CEPT ECC PT1 for their LS in RP-252992 (R4-2513058) on the parameters for AAS BS operating in bands below 1 GHz. 3GPP TSG RAN has received initial feedback from RAN WG4, and would like to provide the following feedback to the questions in RP-252992:

- Unwanted emissions characteristics of AAS BSs operating in the 700 MHz, 800 MHz and 900 MHz bands.

RAN4: The unwanted emissions characteristics of AAS BS and NR BS operating in bands below 1 GHz (including 700 MHz, 800 MHz and 900 MHz bands) are captured since Rel-15 in AAS BS specifications (i.e., TS 37.105, TS 37.145-1, TS 37.145-2), and NR BS specifications (i.e., TS 38.104, TS 38.141-1, TS 38.141-2). The unwanted emissions characteristics of AAS BS were derived in Rel-15 based on 2 GHz carrier frequency simulations assumption and were implemented in technical specifications for AAS BS and NR BS in a band-agnostic manner. The low band AAS products were not explicitly considered for AAS BS use during Rel-15 work. Therefore, updates to the unwanted emissions characteristics may need reconsideration, but currently there is no conclusion when and if this would happen in RAN4.

- Whether 3GPP TS 37.105 and TS 38.104 BS RF requirements are also applicable for AAS BS operation in bands below 1 GHz under consideration.

RAN4: 3GPP TS 37.105 and TS 38.104 BS RF requirements are also applicable for AAS BS operation in bands below 1 GHz, including operating bands in the 700 MHz, 800 MHz and 900 MHz. Those requirement levels were originally derived based on conducted limits for non-AAS BS and may need reconsideration, but currently there is no conclusion when and if this would happen in RAN4.

- A model and associated parameters for modelling AAS BS antenna arrays in frequencies below 1 GHz, both in-band and out of band for frequency bands under consideration.

RAN4: The antenna model in section 7.1 of TR 38.922 can be used in 700 MHz, 800 MHz and 900 MHz operating bands for further discussion in RAN4. However, the associated parameters for the AAS BS antenna model need to be provided.

Additional information related to the further work for question 3:

- In-band: Feedback on antenna array model and associated parameters for in-band AAS BS modelling below 1 GHz is expected to be provided in an LS sent directly from RAN4#118bis meeting in Apr. 2026 to CEPT ECC PT1. In case work for antenna array model and associated parameters is still ongoing at the time of RAN4#118bis meeting then RAN4 will inform CEPT ECC PT1 of the status accordingly.

- Out of band: So far, RAN4 was using a simplified out of band model (assuming either zero antenna correlation, or full antenna correlation). Further details can be provided by RAN4 in the above planned LS, if necessary. More advanced modelling approach(es) will be considered under 6G SI (FS_6G_Radio; WID in RP-252912), including at least consideration of parameterized linear roll-off model as captured in TR 38.922.

# 2	Actions

To CEPT ECC PT1

ACTION: 	3GPP TSG RAN kindly asks CEPT ECC PT1 to consider the above information in their further work on suitability of Active Antenna Systems (AAS) operation in the frequency bands 694-791 MHz (700 MHz), 791-862 MHz (800 MHz) and 880-960 MHz (900 MHz).

3GPP TSG RAN kindly asks ETSI ECC PT1 to provide clarifications on the following issues:

- Expected timeline to complete related co-existence studies in CEPT ECC PT1, and the latest possible deadline for RAN4 to provide requested feedback.

- Provide more details on the prioritized co-existence scenarios for 700 MHz, 800 MHz and 900 MHz operating bands.

# 3	Dates of next TSG RAN meetings

TSG-RAN Meeting #111			Fukuoka, Japan				09-12 March, 2026

TSG-RAN Meeting #112			Singapore						08-11 June, 2026