3GPP TSG-RAN WG1 Meeting #122	 R1-2505343

Bengaluru, India, August 25 – 29, 2025

Agenda Item:	9.1

Source:	Huawei, HiSilicon

Title:	UE features for AI/ML for NR air interface

Document for:	Discussion and Decision

# Introduction

In RAN1#121 meeting, UE features for AI/ML for NR air interface was updated as [1]. Regarding on the remaining issues to be discussed, we provide our views in this contribution.

# UE feature for beam management

The basic FGs of AI/ML for beam management are updated in last meeting as summarized in the Annex. Regarding on the yellow highlighting parts, we provide our views in the following.

## 58-1-2 and 58-1-3

58-1-2/3 correspond to UE side inference for BM-Case 1.

Notation of 58-1-2/58-1-3

Regarding the notation of the two FGs, there is square bracket on [for inference]. We think [for inference] can be removed, since there is no previous BM UE feature for beam prediction.

In the following, we provide our views to the components.

58-1-2 components

- Component 4/4a/5/5a: In legacy FG 2-35, it already incorporates the max number of CSI reports for A-CSI, SP-CSI and P-CSI reports. Regarding AI/ML based CSI reports, the number of AI/ML calculation/memory resources can be reflected by PU mechanism, which will not impact the number of CSI reports. Therefore, we do not support Component 4/4a/5/5a.

- Component 7/7a/7b: Component 7 can fully reflect the information of 7a/7b and is more flexible than 7a/7b, e.g., Component 7 can preclude some unreasonable combinations of Set B size and Set A size, e.g., {4, 64}, {24, 32}. Therefore, we prefer to keep Component 7 over 7a/7b.

- Component 8/9: OK to support.

- Component 10: Can be discussed after the monitoring FG is clear.

- Component 11: Not sure whether setB-different-from-setA only refers to wide-to-narrow, or it can also be narrow-to-narrow. Can be further discussed.

- Component 13 (new): In RAN1#121 meeting, it has been agreed that for inference for BM-Case 1, legacy timeline is extended by adding an additional delay (d/d’) reported by UE. Since the inference is performed only after receiving CSI-RS/SSB resources, the same inference delay should be added to both Z3 and Z3’, i.e., d=d’. Therefore, we propose that UE reports only one value of additional delay per SCS for both Z3 and Z3’. Considering that for relatively small model size of beam management, the inference delay could be ms level.

Agreement For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1 and BM-Case 2, when applicable, extend legacy Z3/Z3’ to Z3+d / Z3’+d’, where d and d’ are reported by UE per SCS for BM-Case 1 and BM-Case 2 respectively - Detailed values of d and d’ can be further discussed in UE feature.

Proposal 1: Update FG 58-1-2 and FG 58-1-3 as the cyan-highlighted parts in Table 1.

Table 1 Updates for FG 58-1-2 and FG 58-1-3

| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs [4. Maximum number of inference report(s) activated for BM-Case1 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case1 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case1 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case1 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] [8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 1 with UE side model] [11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both}] 12. Supported maximum number of predicted beams in each reporting instance 13. The number of symbols di introduced for the timeline of inference in addition to legacy Z3/Z3’, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS. FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection | FFS | yes | n/a | UE-sided beam prediction for BM Case 1 [for inference] is not supported | FFS: CPU/AIMLPU related information FFS: Component 13 candidate values | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-3 | UE-side beam prediction for BM Case1 with predicted RSRP [for inference] | 1. Support of beam prediction, reporting of predicted beam index and predicted RSRP, for BM-Case1 [for inference] 2. Supported maximum number of predicted beams with RSRP in each reporting instance | 58-1-2 | yes | n/a | UE-side beam prediction for BM Case 1 with predicted RSRP [for inference] is not supported | FFS: CPU/AIMLPU related information | Optional with capability signalling |

## 58-1-4 and 58-1-5

58-1-4/5 correspond to UE side inference for BM-Case 2.

Notation of 58-1-4/58-1-5

Similar to our analysis to 58-1-2/3, the “for inference” can be removed.

In the following, we provide our views to the components.

58-1-4 components

- Component 4/4a/5/5a/7/7a/7b/8/9/10: They are same to BM-Case 1, so same handling to 58-1-2 can be applied.

- Component 13: Whether the total number of predicted beams will impact the UE capability can be further clarified. Straightforwardly, it can be represented by the combination of Component 11 and 12.

- Component 20: Not sure whether setB-different-from-setA only refers to wide-to-narrow, or it can also be narrow-to-narrow. Can be further discussed.

- Component 21 (new): Same as analyzed for BM-Case 1, a new component is added.

Proposal 2: Update FG 58-1-4 and FG 58-1-5 as the cyan-highlighted parts in Table 2.

Table 2 Updates for FG 58-1-4 and FG 58-1-5

| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs [4. Maximum number of inference report(s) activated for BM-Case2 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case2 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case2 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case2 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] [8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 2 with UE side model] 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances [13. Supported maximum total number of reported predicted beams for predicted time instances in one report] [20. Supported BM-Case 2 sub usecase(s): e.g., setB-equals-to-setA, setB-subset-of-setA, setB-different-from-setA, or merged version(s)] 21. The number of symbols di introduced for the timeline of inference in addition to legacy Z3/Z3’, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS. | FFS | yes | n/a | UE-side beam prediction for BM-Case2 [for inference] is not supported | FFS: CPU/AIMLPU related information FFS: candidate values for components FFS: Component 21 candidate values | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-5 | UE-side beam prediction for BM-Case2 with predicted RSRP [for inference] | 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (spatial and time domain beam prediction) [for inference] | 58-1-4 | yes | n/a | UE-side beam prediction for BM-Case2 [for inference] is not supported | FFS: CPU/AIMLPU related information | Optional with capability signalling |

## 58-1-7

Components

- Component 6/7: For data collection, UE needs to measure Set B to obtain model input and measure Set A to obtain the label. For Component 7, to enable sufficient number of overall Tx beams, it is proposed to set the max number of CSI-RS resources for Set A as up to 256. Accordingly, for Set B, it is proposed to set the max number of CSI-RS resources for Set B as up to 64.

- Component 3: The candidate cases include {‘Set B equal to Set A’, ‘Set B subset of Set A’,’Set B not a subset of Set A’}. From our view, UE capability of supporting BM-Case 1 and BM-Case 2 should be separated also from the data collection perspective, since it may impact the configuration on the resources of Set B and Set A by gNB. E.g., from NW perspective, the flavors on the resources of Set B and Set A (e.g., number of beams, periodicity of Set B/Set A, time offset between Set B and Set A, etc.) may be different between BM-Case 1 and BM-Case 2. To ensure the aligned configuration between training and inference, gNB should be clear on the intended BM-Case 1 and BM-Case 2 by UE early in the training phase.

According to the discussion for Component 3, FG 58-1-7 should be applied for BM-Case 1, and an additional FG, namely 58-1-7a, should be introduced for BM-Case 2.

Proposal 3: Update FG 58-1-7 as the cyan-highlighted parts in Table 3.

Table 3 Updates for FG 58-1-7

| 58. NR_AIML_air | 58-1-7 | Data collection for UE-side beam prediction [for BM case 1] | 1. Support of data collection for UE-side beam prediction [for BM case 1] 2. Support of SS/PBCH block and 1-port CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection [3. Supported sub-use cases: {‘Set B equal to Set A’, ‘Set B subset of Set A’,’Set B not a subset of Set A’}] [6: Supported maximum number of resources for Set B] [7: Supported maximum number of resources for Set A] 8. Support of SSB as RS type for Set B 9. Support of CSI-RS as RS type for Set B 10. Support of SSB as RS type for Set A 11. Support of CSI-RS as RS type for Set A | FFS | yes | n/a | Data collection for UE-side beam prediction is not supported [for BM case 1] | Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2 Component 6 candidate values: {16, 32, 64} Component 7 candidate values: {32, 64, 128, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-1-7a | Data collection for UE-side beam prediction for BM case 2 | 1. Support of data collection for UE-side beam prediction for BM case 2 2. Support of SS/PBCH block and 1-port CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection [3. Supported sub-use cases: {‘Set B equal to Set A’, ‘Set B subset of Set A’,’Set B not a subset of Set A’}] 4: Supported maximum number of resources for Set B 5: Supported maximum number of resources for Set A | FFS | yes | n/a | Data collection for UE-side beam prediction is not supported for BM case 2 | Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2 Component 6 candidate values: {16, 32, 64} Component 7 candidate values: {32, 64, 128, 256} | Optional with capability signalling |

# UE feature for positioning accuracy enhancement

The basic FGs of AI/ML for positioning accuracy enhancement have been updated in last meeting as summarized in the Annex. Regarding the yellow highlighted parts, we provide our views in the following.

Notation of 58-2-1

Since UE-based positioning Case 1 is the only positioning UE feature in Rel-19, it is fine not to add “for inference”. We are also fine to add it to be aligned with other UE features.

Prerequisite

Regarding the prerequisite, the UE needs to support the reception of DL-PRS resources and be able to process DL-PRS. But the common DL PRS Processing Capability in FG 13-1 has the Field name and Parent IE in TS 38.331 as listed in the below table:

| Field name in TS 38.331 | Parent IE in TS 38.331 |
| --- | --- |
| 1 supportedBandwidthPRS-r16 2 dl-PRS-BufferType-r16 3 durationOfPRS-Processing-r16 4 maxNumOfDL-PRS-ResProcessedPerSlot-r16 | PRS-ProcessingCapabilityPerBand-r16 LPP |

Since UE-based positioning Case 1 is a UE feature independent of other positioning method, it should also be independent of PRS capability designed for other Rel-16 related positioning methods. This means that a UE supporting UE-based positioning Case 1 does not need to support other Rel-16 related positioning methods, i.e., FG 13-1, as the prerequisite.

Proposal 4: Update FG 58-2-1 to FG 58-2-6 as the cyan-highlighted parts in Table 4.

Table 4 Updates for FG 58-2-1 to FG 58-2-6

| 58. NR_AIML_Air | 58-2-1 | UE-based positioning Case 1 [for inference] | Indicates support of UE-based positioning Case 1 [for inference] | FFS N/A | N/A | N/A | UE-based positioning Case 1 [for inference] is not supported | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-2-2 | Support reception of AD for UE-based positioning Case 1 | Indicates support of reception of AD for UE-based positioning Case 1 | FFS N/A | N/A | N/A | Reception of AD for UE-based positioning Case 1 is not supported | Need for location server to know if the feature is supported. Note: For specific ADs, UE indicate supported AD that can be received as in [FFS: FGs] | Optional with capability signalling |
| 58. NR_AIML_Air | 58-2-3 | DL PRS Resources for UE-based positioning Case 1 | 1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE 2. Max number of TRPs across all positioning frequency layers per UE 3. Max number of positioning frequency layers UE supports | [13-1] N/A | N/A | N/A |  | Component 1 candidate values: {1, 2} Component 2 candidate values: {4, 6, 12, 16, 24, 32, 64, 128, 256} Component 3 candidate values: {1, 2, 3, 4} Need for location server to know if the feature is supported. | Optional with capability signaling |
| 58. NR_AIML_Air | 58-2-3a | DL PRS Resources for UE-based positioning Case 1 on a band | [1. Max number of DL PRS Resources per DL PRS Resource Set 2. Max number of DL PRS Resources per positioning frequency layer | [13-1] N/A | N/A | N/A |  | Component 1 candidate values: {1, 2, 4, 8, 16, 32, 64} Component 2 candidate values: {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: For component 1, the values 16, 32, 64 are only applicable to FR2 bands Note: For component 2, the value 6 is only applicable to FR1 bands Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
| 58. NR_AIML_Air | 58-2-3b | DL PRS Resources for UE-based positioning Case 1 on a band combination | 1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation | [13-1] N/A | N/A | N/A |  | Component 1 candidate values: {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC Component 2 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC Component 3 candidate values: {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands Component 4 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands] Need for location server to know if the feature is supported. Note: the reported value is the total number across all bands in the corresponding BC Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
| 58. NR_AIML_Air | 58-2-4 | DL PRS Processing Capability for UE-based positioning Case 1 | 1. Maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 2. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 3. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 4. Max number of DL PRS resources that UE can process in a slot under it Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than 30%. | FFS N/A | N/A | N/A |  | Need for location server to know if the feature is supported. Component 1 candidate values: -FR1 bands values {5, 10, 20, 40, 50, 80, 100} -FR2 bands values {50, 100, 200, 400} Component 3 candidate values: -T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms -N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 4 candidate values: -FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz -FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Notes for component 3: a. UE reports one combination of (N, T) values per band, where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE b. UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value c. UE DL PRS processing capability is defined for a single positioning frequency layer. UE capability for simultaneous DL PRS processing across positioning frequency layers is not supported in Rel.16 (i.e. for a UE supporting multiple positioning frequency layers, a UE is expected to process one frequency layer at a time) d. UE DL PRS processing capability is agnostic to DL PRS comb factor configuration e. The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS Note: if the UE does not indicate this capability for a band or band combination, the UE does not support PRS processing in this band or band combination. [Note: If the UE reports both FG 13-1 and [this FG], then the UE is not expected to perform simultaneous processing of legacy methods and Case 1 positioning method. Note: If UE does not provide [this FG] but the UE supports Case 1, FG 13-1 indicates the DL PRS processing capabilities common across all positioning methods including UE-based positioning Case 1.] | Optional with capability signaling |
| 58. NR_AIML_Air | 58-2-5 | Support of SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 | 1. Support of SSB from neighbour cell as QCL source of a DL PRS 2. Support of reuse SSB measurement from RRM for receiving PRS Note: Refers to Type-C for FR1 and Type-C & Type-D support for FR2 | [58-2-4; otherwise N/A 13-1] | N/A | N/A |  | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 58. NR_AIML_Air | 58-2-6 | Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 | 1. Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS Note 1: Refers to Type-D support for FR2 Note 2: A PRS from a PRS-only TP is treated as PRS from a non-serving cell | [58-2-4; otherwise N/A 13-1] | N/A | N/A |  | Need for location server to know if the feature is supported. DL PRSs are in the same band | Optional with capability signaling |

# UE feature for CSI prediction

The FGs of AI/ML for CSI prediction are updated in last meeting as summarized in the Annex. Regarding on the yellow highlighting parts, we provide our views in the following.

Components for 58-3-2

- Component 8: It needs more clarification: for A-CSI-RS, the max number of CSI-RS resources can be reflected by an additional FG, similar to FG 40-3-2-1b of Rel-18 CSI prediction. For P/SP-CSI-RS, there seems no need to reflect this UE capability since it is UE implementation.

Proposal 5: Update FG 58-3-2 as the cyan-highlighted parts in Table 5.

Table 5 Updates for FG 58-3-2

| 58. NR_AIML_Air | 58-3-2 | CSI prediction for UE-sided inference when N4>1 | 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 5. Support for the size of DD-basis, N4>1 7. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting [8. Supported values of the maximum number of observation number] | 58-3-1 | yes | n/a | CSI prediction for N4>1 for inference is not supported | [Per band and Per BC] |  | FFS: CPU/AIMLPU related information Candidate values: FFS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# UE feature for APU reporting

The basic FGs of AI/ML for CSI report framework are updated in last meeting as summarized in the Annex. We provide our views in the following.

Components for 58-0-1

- Component 1/2: In RAN#108 meeting, the following proposal has been endorsed where UE can report support for N (up to 2) AI/ML PU pools for AI/ML features. For each of the N AI/ML PU pools, UE reports the maximum number of simultaneous AI/ML PUs, respectively. In other words, UE can report the maximum number of up to 2 types of APU (denoted as APU1 and APU2). In addition, when both APU1 and APU2 are reported, UE reports the AI/ML feature(s) that each type of APU can be used for.

Proposal - A UE can report support for N (up to 2) AI/ML PU pools for AI/ML features - For each of the N AI/ML PU pools, UE reports the maximum number of simultaneous AI/ML PUs, respectively. - If N = 2, for each AI/ML use case feature, UE reports which AI/ML PU pool it belongs to.

Proposal 6: Update FG 58-0-1 as the cyan-highlighted parts in Table 6.

Table 6 Updates for FG 58-0-1

| 58. NR_AIML_Air | 58-0-1 | CSI report framework for UE-side inference | 1. The feature(s) applied with APU1 1a. Maximum number of APU1s for all types of UE-sided inference for CSI report(s) for simultaneously in a CC 1b2. Maximum number of APU1s for all types of UE-sided inference for CSI report(s) simultaneously across all CCs 2. The feature(s) applied with APU2 2a. Maximum number of APU2s for UE-sided inference for CSI report(s) for simultaneously in a CC 2b. Maximum number of APU2s for UE-sided inference for CSI report(s) for simultaneously across all CCs | FFS | yes | n/a | Maximum number of APUs for UE-sided inference is unknown to the network | FFS | FFS | FFS | FFS | Component 1 candidate values: FFS {UE-side beam prediction, UE-side CSI prediction, UE-side beam prediction and UE-side CSI prediction } Component 1a candidate values: {from 1 to 8} Component 1b candidate values: {from 5 to 32} Component 2 candidate values: FFS {UE-side beam prediction, UE-side CSI prediction} Component 2a candidate values: {from 1 to 8} Component 2b candidate values: {from 5 to 32} Note: UE can report Component 1,1a,1b only, or report both Component 1,1a,1b and 2,2a,2b. Note: The features applied with APU1 and APU2 are not overlapped. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# Conclusions

In this contribution, we discussed the FGs for Rel-19 NR_AIML_air. The following proposals are provided.

Proposal 1: Update FG 58-1-2 and FG 58-1-3 as the cyan-highlighted parts in Table 1.

Proposal 2: Update FG 58-1-4 and FG 58-1-5 as the cyan-highlighted parts in Table 2.

Proposal 3: Update FG 58-1-7 as the cyan-highlighted parts in Table 3.

Proposal 4: Update FG 58-2-1 to FG 58-2-6 as the cyan-highlighted parts in Table 4.

Proposal 5: Update FG 58-3-2 as the cyan-highlighted parts in Table 5.

Proposal 6: Update FG 58-0-1 as the cyan-highlighted parts in Table 6.

# References

R1-2504673, “Updated RAN1 UE features list for Rel-19 NR after RAN1 #121”, RAN1#121, May 19 – 23, 2025.

RP-251823, “Way forward on the support of separate AI/ML processing pools for different AI use cases”, RAN#108, June 9 – 13, 2025.

# Annex: UE feature list of NR_AIML_air updated in RAN1#121

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_air | 58-0-1 | CSI report framework for UE-side inference | 1. Maximum number of APUs for all types of UE-sided inference for CSI report(s) for simultaneously in a CC 2. Maximum number of APUs for all types of UE-sided inference for CSI report(s) simultaneously across all CCs | FFS | yes | n/a | Maximum number of APUs for UE-sided inference is unknown to the network | FFS | FFS | FFS | FFS | Component 1 candidate values: FFS Component 2 candidate values: FFS | Optional with capability signalling |
| 58. NR_AIML_air | 58-1-1 | Increased number of reported RSs for beam management | 1. Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4 | FFS | yes | n/a | Increased number of reported beams for beam management is not supported | FFS | FFS | FFS | FFS | Component 3 candidate values: {6,8} | Optional with capability signalling |
| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs [4. Maximum number of inference report(s) activated for BM-Case1 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case1 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case1 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case1 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A][8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 1 with UE side model] [11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both}] 12. Supported maximum number of predicted beams in each reporting instanceFFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection | FFS | yes | n/a | UE-sided beam prediction for BM Case 1 [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: CPU/AIMLPU related information | Optional with capability signalling |
| 58. NR_AIML_air | 58-1-3 | UE-side beam prediction for BM Case1 with predicted RSRP [for inference] | 1. Support of beam prediction, reporting of predicted beam index and predicted RSRP, for BM-Case1 [for inference] 2. Supported maximum number of predicted beams with RSRP in each reporting instance | 58-1-2 | yes | n/a | UE-side beam prediction for BM Case 1 with predicted RSRP [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: CPU/AIMLPU related information | Optional with capability signalling |
| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 [for inference] | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs [4. Maximum number of inference report(s) activated for BM-Case2 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case2 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case2 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case2 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A][8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS] [9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report] [10. Supported options for performance monitoring for beam case 2 with UE side model] 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances [13. Supported maximum total number of reported predicted beams for predicted time instances in one report] [20. Supported BM-Case 2 sub usecase(s): e.g., setB-equals-to-setA, setB-subset-of-setA, setB-different-from-setA, or merged version(s)] | FFS | yes | n/a | UE-side beam prediction for BM-Case2 [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: CPU/AIMLPU related information FFS: candidate values for components | Optional with capability signalling |
| 58. NR_AIML_air | 58-1-5 | UE-side beam prediction for BM-Case2 with predicted RSRP [for inference] | 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (spatial and time domain beam prediction) [for inference] | 58-1-4 | yes | n/a | UE-side beam prediction for BM-Case2 [for inference] is not supported | FFS | FFS | FFS | FFS | FFS: CPU/AIMLPU related information | Optional with capability signalling |
| 58. NR_AIML_air | 58-1-7 | Data collection for UE-side beam prediction [for BM case 1] | 1. Support of data collection for UE-side beam prediction [for BM case 1] 2. Support of SS/PBCH block and 1-port CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection [3. Supported sub-use cases: {‘Set B equal to Set A’, ‘Set B subset of Set A’,’Set B not a subset of Set A’}] [6: Supported maximum number of resources for Set B] [7: Supported maximum number of resources for Set A] 8. Support of SSB as RS type for Set B 9. Support of CSI-RS as RS type for Set B 10. Support of SSB as RS type for Set A 11. Support of CSI-RS as RS type for Set A | FFS | yes | n/a | Data collection for UE-side beam prediction is not supported [for BM case 1] | FFS | FFS | FFS | FFS | Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2 | Optional with capability signalling |
| 58. NR_AIML_Air | 58-2-1 | UE-based positioning Case 1 [for inference] | Indicates support of UE-based positioning Case 1 [for inference] | FFS | N/A | N/A | UE-based positioning Case 1 [for inference] is not supported | FFS | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| 58. NR_AIML_Air | 58-2-2 | Support reception of AD for UE-based positioning Case 1 | Indicates support of reception of AD for UE-based positioning Case 1 | FFS | N/A | N/A | Reception of AD for UE-based positioning Case 1 is not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: For specific ADs, UE indicate supported AD that can be received as in [FFS: FGs] | Optional with capability signalling |
| 58. NR_AIML_Air | 58-2-3 | DL PRS Resources for UE-based positioning Case 1 | 1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE 2. Max number of TRPs across all positioning frequency layers per UE 3. Max number of positioning frequency layers UE supports | [13-1] | N/A | N/A |  | Per UE | No | No | No | Component 1 candidate values: {1, 2} Component 2 candidate values: {4, 6, 12, 16, 24, 32, 64, 128, 256} Component 3 candidate values: {1, 2, 3, 4} Need for location server to know if the feature is supported. | Optional with capability signaling |
| 58. NR_AIML_Air | 58-2-3a | DL PRS Resources for UE-based positioning Case 1 on a band | [1. Max number of DL PRS Resources per DL PRS Resource Set 2. Max number of DL PRS Resources per positioning frequency layer | [13-1] | N/A | N/A |  | Per band | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 4, 8, 16, 32, 64} Component 2 candidate values: {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: For component 1, the values 16, 32, 64 are only applicable to FR2 bands Note: For component 2, the value 6 is only applicable to FR1 bands Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
| 58. NR_AIML_Air | 58-2-3b | DL PRS Resources for UE-based positioning Case 1 on a band combination | 1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation | [13-1] | N/A | N/A |  | Per BC | n/a | n/a | n/a | Component 1 candidate values: {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC Component 2 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC Component 3 candidate values: {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands Component 4 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands] Need for location server to know if the feature is supported. Note: the reported value is the total number across all bands in the corresponding BC Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
| 58. NR_AIML_Air | 58-2-4 | DL PRS Processing Capability for UE-based positioning Case 1 | 1. Maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 2. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 3. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 4. Max number of DL PRS resources that UE can process in a slot under it Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than 30%. | FFS | No | N/A |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Component 1 candidate values: -FR1 bands values {5, 10, 20, 40, 50, 80, 100} -FR2 bands values {50, 100, 200, 400} Component 3 candidate values: -T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms -N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 4 candidate values: -FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz -FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Notes for component 3: a. UE reports one combination of (N, T) values per band, where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE b. UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value c. UE DL PRS processing capability is defined for a single positioning frequency layer. UE capability for simultaneous DL PRS processing across positioning frequency layers is not supported in Rel.16 (i.e. for a UE supporting multiple positioning frequency layers, a UE is expected to process one frequency layer at a time) d. UE DL PRS processing capability is agnostic to DL PRS comb factor configuration e. The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS Note: if the UE does not indicate this capability for a band or band combination, the UE does not support PRS processing in this band or band combination. [Note: If the UE reports both FG 13-1 and [this FG], then the UE is not expected to perform simultaneous processing of legacy methods and Case 1 positioning method. Note: If UE does not provide [this FG] but the UE supports Case 1, FG 13-1 indicates the DL PRS processing capabilities common across all positioning methods including UE-based positioning Case 1.] | Optional with capability signaling |
| 58. NR_AIML_Air | 58-2-5 | Support of SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 | 1. Support of SSB from neighbour cell as QCL source of a DL PRS 2. Support of reuse SSB measurement from RRM for receiving PRS Note: Refers to Type-C for FR1 and Type-C & Type-D support for FR2 | [58-2-4; otherwise 13-1] | No | N/A | SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| 58. NR_AIML_Air | 58-2-6 | Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 | 1. Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS Note 1: Refers to Type-D support for FR2 Note 2: A PRS from a PRS-only TP is treated as PRS from a non-serving cell | [58-2-4; otherwise 13-1] | No | N/A | DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. DL PRSs are in the same band | Optional with capability signaling |
| 58. NR_AIML_Air | 58-3-1 | CSI prediction for UE-sided inference when N4=1 | 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) [9. Value for CPU occupation, when P/SP-CSI-RS is configured for CMR] [10. Value for CPU occupation, when A-CSI-RS is configured for CMR] 11. Scaling factor for active resource counting Kp | 2-35 | yes | n/a | CSI prediction for N4=1 for inference is not supported | [Per band and Per BC] | n/a | n/a | n/a |  | Optional with capability signalling |
| 58. NR_AIML_Air | 58-3-1a-1 | DD unit size when A-CSI-RS is configured for CMR N4>1 for UE side inference of CSI prediction | Value of d=1 for the DD unit size when A-CSI-RS for inference is configured for CMR | 58-3-1 | Yes | N/A | DD unit size d=1 is not supported when A-CSI-RS is configured for CMR N4>1 for UE side inference of CSI prediction | Per-band and Per-BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 58. NR_AIML_Air | 58-3-1b | Maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for Rel-16-based doppler measurement for UE side inference of CSI prediction | 1. Maximum number of aperiodic CSI-RS resources for inference that can be configured in the same CSI report setting for Rel-16-based doppler measurement | 58-3-1 | Yes | N/A | Maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for Rel-16-based doppler measurement for UE side inference of CSI prediction is not known | Per-band and Per-BC | N/A | N/A | N/A | Component 1 candidate values: {4, 8, 12} | Optional with capability signaling |
| 58. NR_AIML_Air | 58-3-1-2 | Support R=2 for Rel-16-based doppler codebook for UE side inference of CSI prediction | A list of supported combinations {Max # of Tx ports in one resource for inference, Max # of resources for inference and total # of Tx ports}, across all CCs simultaneously, with R=2 | 58-3-1 | Yes | N/A | R=2 for Rel-16-based doppler codebook for UE side inference of CSI prediction is not supported | Per band and per BC | N/A | N/A | N/A | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signaling |
| 58. NR_AIML_Air | 58-3-1-3 | Support X=1 based on first and last slot of WCSI, for Rel-16-based doppler codebook for UE side inference of CSI prediction | Support of TDCQI = ‘1-2’ | 58-3-1 | Yes | N/A | X=1 based on first and last slot of WCSI, for Rel-16-based doppler codebook for UE side inference of CSI prediction is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 58. NR_AIML_Air | 58-3-1-3a | Support X=2 CQI based on 2 slots for Rel-16-based doppler codebook for UE side inference of CSI prediction | Support of TDCQI = ‘2’ | 58-3-1 | Yes | N/A | X=2 CQI based on 2 slots for Rel-16-based doppler for UE side inference of CSI prediction is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 58. NR_AIML_Air | 58-3-1-4 | support of l = (n – nCSI,ref ) for CSI reference slot for Rel-16 based doppler codebook for UE side inference of CSI prediction | 1. Support of l = (n – nCSI,ref ) for CSI reference slot when N4=1 and d>1 2. Support of l = (n – nCSI,ref ) for CSI reference slot when N4>1 | 58-3-1 | Yes | N/A | l = (n – nCSI,ref ) for CSI reference slot for Rel-16 based doppler codebook for UE side inference of CSI prediction is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 58. NR_AIML_Air | 58-3-1-5 | Support of L=6 for Rel-16 based doppler codebook for UE side inference of CSI prediction | Support of L=6 for Rel-16 based doppler codebook | 58-3-1 | Yes | N/A | L=6 for Rel-16-based doppler codebook for UE side inference of CSI prediction is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| X. NR_AIML_Air | 58-3-1-6 | Support of rank equals 3 and 4 for Rel-16 based doppler codebook for UE side inference of CSI prediction | Support of rank equals 3 and 4 for Rel-16 based doppler codebook | 58-3-1 | Yes | N/A | Rank equals 3 and 4 for Rel-16-based doppler codebook for UE side inference of CSI prediction is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| 58. NR_AIML_Air | 58-3-1-7 | Active CSI-RS resources and ports for mixed R16 based doppler codebook for CSI prediction via UE side model with other codebooks in any slot | 1. List of codebook combinations of two types 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 23-9-1, 16-3a, 2-36, 2-40, 2-41, 23-9-2, 23-9-4, 40-3-2-1, 40-3-2-1a, 40-3-2-2, X-1-1, X-1-1a, X-1-2 | Yes | n/a | Active CSI-RS resources and ports for mixed R16 based doppler codebook for CSI prediction via UE side model with other codebooks in any slot is not supported | Per band and Per BC | n/a | n/a | n/a | Component 1 candidate values: FFS Component 2 candidate values: FFS Note: if a UE reports one or more codebook combinations in 58-3-1-7, then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations Note: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-9-5, 58-3-1-7 and per-codebook capability 2-36/40/41, 16-3a, and 23-9-1/23-9-2/23-9-4. [Note: Up to 4 combinations for component 1] | Optional with capability signaling |
| 58. NR_AIML_Air | 58-3-2 | CSI prediction for UE-sided inference when N4>1 | 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 5. Support for the size of DD-basis, N4>1 7. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting [8. Supported values of the maximum number of observation number] | 58-3-1 | yes | n/a | CSI prediction for N4>1 for inference is not supported | [Per band and Per BC] | FFS | FFS | FFS | FFS: CPU/AIMLPU related information Candidate values: FFS | Optional with capability signalling |
| 58. NR_AIML_Air | 58-3-4 | UE side data collection for CSI prediction | 1. Support of data collection for CSI prediction | FFS | yes | n/a | UE side data collection for CSI prediction is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signalling |