3GPP TSG RAN WG1 #122bis	R1-2506789

Prague, Czech Republic, Oct 13th - 17th, 2025

Source:	Nokia

Title:	Air interface for Device 2b/C for Rel-20 AIoT

Agenda item:	10.3.2

Document for:	Discussion and Decision

# Introduction

At RAN#108, a new study item “Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices” (FS_Ambient_IoT_Outdoor_Active) was approved; the SID was most recently revised at RAN#109 [1].

The following objective(s) is/are relevant for the present agenda item:

1. Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]:  Deployment scenario 4 with topology 1 with no external carrier wave.  Traffic types DO-DTT, DT, DO-A.  Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command).  Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs.  FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum.  Assumption of the same air interface for Device 2b and Device C. o Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 ....  Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios o Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios  Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage.

# Discussion

Rel-19 AIoT laid the foundational groundwork for A-IoT communications. The primary target of Rel-19 was the initial standardization of device 1 which is a purely passive device characterized by a power consumption of approximately 1 µW. This device features a simple RF envelope detector for R2D reception and employs backscatter communication for D2R transmissions, modulating an externally provided carrier wave (CW). Critically, This passive type of AIoT device lacks any internal RF amplification for either the reader-to-device (R2D) or D2R links and is designed to operate with low-cost oscillators exhibiting a high initial sampling frequency offset (SFO) of up to 105 ppm.

The scope of the Rel-19 work was constrained to ensure feasibility, focusing on Deployment Scenario 1 with Topology 1 (D1T1). This scenario involves an indoor A-IoT device communicating directly with an indoor base station (BS) reader. The target representative use cases were indoor inventory (rUC1) and indoor command (rUC4), supporting Device-terminated (DT) and Device-originated device-terminated triggered (DO-DTT) traffic types. A key limitation of the Rel-19 framework was its inability to support device-originated autonomous (DO-A) traffic. This was a direct consequence of passive nature of device 1, as it lacks the active transmission capability required to initiate communication without a prior trigger from the network. This limitation motivated us to study the expanded scope in Release 20.

The introduction of active devices (Device 2b, Device C) and new topologies in Rel-20 necessitates the development of necessary physical layer solutions. Unlike the passive, energy-limited paradigm of Rel-19, the Rel-20 framework must contend with interferences, more complex access procedures, and the inherent imperfections of low-cost transceivers.

## Device Transmit Power Control

In the previous RAN1 meeting, the following agreement was made:

Agreement Study necessity and feasibility of D2R Tx power control for Device 2b and for Device C, including the following aspects - Open loop and/or closed loop power control - Required measurements (e.g., signal strength) - Required information at the device - Required information at reader side - Signaling overhead and power consumption

Unlike Device 1 which rely on RF energy harvester and backscattering module, modulating and reflecting an external carrier wave, D2R signal amplification is done by Device 2b and Device C. Based on [1] and [2], they are capable of performing D2R transmission generated internally by the device. That is, they have an RF component enabling them to generate their own transmission signal, powered by their internal energy source.

Rel-20 studies outdoor scenario with the maximum distance up to 500 meters between a reader and a AIoT device, which means there would be huge difference pathloss depending on the distance. Furthermore, the maximum power difference between Device C and Device 2b is quite substantial. As a worst-case example, if a Device C nearby a reader transmits D2R signal with 5 dBm while a Device 2b transmits D2R signal with -20 dBm, it is expected to far exceed what AGC and ADC can accommodate. The bottom line is that power control should be considered to address the near-far problem.

Observation 1: D2R power control is necessary to address near-far problems. The received signal power difference from a Device 2b and Device C may be too large depending on their distance with respect to a reader and their transmission power.

We need to build a comprehensive power control framework with two approaches such as open-loop power control with an initial power setting for device-initiated data transmission and closed-loop power control to adjust device power with being aware of device’s energy status. The closed-loop power control should not be meant to introduce dynamic power control like NR UE. If device reports any information such as RSRP measurement or energy status, it is a closed loop power control.

Device 2b/C would be able to perform measurement such as RSRP to estimate a received signal strength. As a simple power control mechanism, the network can indicate whether to use the power amplifier (bypass PA by turning off) for a specific received RSRP level. Based on this approach, a certain group of devices, which is located in a certain range of distances, may transmit a D2F signal for a specific R2D.

Another approach is that a reader tries to communicate with nearby devices. The reader may transmit a R2D signal with a very low power so that only nearby devices can detect the R2D signal. The devices may be indicated to not transmit a D2R signal response to another R2D under some conditions such as a detection of a specific sequence, and then the reader will increase the transmission power for a R2D transmission targeting farther devices.

Proposal 1: Study the way to enable a certain group of A-IoT devices to transmit D2R responses to a R2D signal so that received signal power of the D2R could be within a certain range, based on indications from network and measurements made at the A-IoT devices.

In addition, it may be feasible for a reader to indicate an increase or decrease of D2R transmission power based on a specific granularity, which may be effective to control the power. However, low power device characteristic should be considered at the reader. Certain A-IoT devices would be mainly relying on a energy source from energy harvesting such as solar energy. The reader should be aware of the energy status of the device for the effective power control. Even if the reader requests a device to increase D2R transmission power, the device would not be able to follow it depending on its energy status, but the reader cannot clear expect such behavior unless it has a relevant information.

Proposal 2: For D2R Tx power control, the required information at reader side at least includes energy level of A-IoT device.

## Impact of CFO

One of the most significant PHY layer challenges for active A-IoT devices may be their poor frequency stability. The core design principle of ultra-low cost mandates the use of simple, inexpensive oscillators (e.g., RC/LC oscillators) instead of the precise crystal oscillators found in conventional NR UEs. This design choice has profound implications for the receiver and requires dedicated compensation mechanisms.
The active devices (2b and C) are expected to use more advanced IF-ED/ZIF receivers to achieve better sensitivity. These receivers are highly vulnerable to frequency offsets. Even with improved oscillators, initial CFO values are expected to be in the range of [100-200] ppm, far greater than the <[20 ppm] tolerance of standard NR UEs.

A large CFO has two primary detrimental effects. First, it introduces a continuous phase rotation on the received signal, which can corrupt the demodulation of phase-based modulation schemes. Second, it can cause the signal spectrum to drift, potentially moving it partially or entirely out of the receiver's filter passband or causing saturation of the ADC. In short, without effective compensation, the link performance of Device 2b/C would be severely degraded, negating the benefits of their active architecture.

Proposal 3: RAN1 study CFO mitigation mechanisms (e.g., preamble-based scheme and/or calibration signal design)

## DO-A

The representative use cases include various use cases which demand device-initiated data transmission from AIoT devices. For example, an event from a forest fire monitoring or water leak monitor may not occur frequently, but once it happens, the data should be transmitted within specified latency bounds. The SA1 latency requirements for event-triggered DO-A use cases are not particularly stringent, see Table 1

Table 1

| Use case | Max. allowed end-to-end latency | Reference |
| --- | --- | --- |
| Base Station Machine Room Environmental Supervision | 30 s | TR 22.840 Table 5.13.6-1 |
| Forest Fire Monitoring | 10 s | TR 22.840 Table 5.19.6-1 |

The main impact is on how the device gets permission to transmit data.

A preliminary investigation of DO-A was performed during the Rel-19 study item and the conclusion can be found in [2]:

From the RAN1 perspective, at least the following aspect of the air interface for DO-DTT and DT traffic types is not sufficient for the DO-A traffic type: For DO-DTT and DT traffic types, the D2R resource(s) for D2R transmission is/are indicated in a R2D transmission, but this is not applicable at least for the first D2R transmission for DO-A traffic.

The transition from network-triggered to device-initiated communication introduces a technical challenge, which is the loss of network-enforced time synchronization. In cellular systems, UE is synchronized with the gNB’s timing through continuous monitoring downlink signals such as SS/PBCH block. Also, existing contention-based random access procedures in NR systems are designed for UEs that are already synchronized to the downlink timing. However, Rel-19 A-IoT devices has no knowledge of timing resource grid information.

At RAN1#122, we reached the following agreement:

Agreement Study the following aspects to support DO-A traffic (periodic and event-triggered) for Device 2b and for Device C in Rel-20. - Timing and frequency synchronization for DO-A transmission - Resource allocation/determination method for DO-A transmission - Configuration/signaling of necessary information

### Resource allocation/determination for DO-A transmission

#### Polling by the reader (Polling-based method)

The most straightforward approach is a polling-based method built on the Rel-19 paging and inventory mechanisms. For example.

- Rel-19 paging could be extended by including a flag to indicate that the paging addresses devices with pending DO-A transmission; the existing “filtering information” mechanism (TS 23.369 clause 5.8) can be used to optionally restrict the set of devices that are addressed.

- Rel-19 contention-based random access procedure can then be re-used to either directly schedule DO-A transmissions in Msg3 or first collect device identities and then schedule DO-A transmissions in subsequent messages.

As is clear from this short description, the specification effort to support the polling-based approach would be minimal. In particular, no new or dedicated “Timing and frequency synchronization for DO-A transmission” would be required. Moreover, it could be implemented for all device types, including device type 1.

On the other hand, one might wonder if a pure polling-based approach strictly deserves the name DO-A, which stands for “device-originated autonomous” transmission, since the procedure is triggered and initiated by the reader rather than the device and could be considered DO-DTT rather than DO-A. Moreover, if DO-A transmissions are rare then this approach could be inefficient; the rate at which the “DO-A paging” is transmitted by the reader would be dictated by the latency requirement; so, if the required latency is short, but DO-A events are very infrequent, then most transmissions of “DO-A paging” would be wasted.

#### Recurring resource configuration provided by the reader

Another approach is for the reader to configure a device or a set of devices with a resource configuration that repeats in time (e.g. periodic resources). There are many variations of this approach, according to the following aspects:

- Are resources dedicated to one device or shared among a set of devices?

- Are the recurring resources intended for transmission of the DO-A data or are they for transmission of a “DO-A scheduling request”?

- How is resource configuration information provided to the device(s) by the network?

If the recurring resources are dedicated to one device and intended for transmission of the actual DO-A data, then this approach could be similar to a Uu configured grant. This could be suitable for devices generating periodic traffic.

If the recurring resources are dedicated to one device and intended for transmission of a “DO-A scheduling request” then this approach could be similar to Uu dynamic scheduling. This could be suitable if there is a small number of devices which may generate urgent event-triggered traffic.

If the recurring resources are shared among multiple devices:

Rel-19 contention-based random access could be re-used – the main difference would be that the Rel-19 procedure is triggered by an A-IoT Paging message and/or Access Trigger message, while for DO-A the procedure would be triggered autonomously by a device with pending DO-A on reaching an “DO-A occasion”.

Alternatively, the shared resource could be used for transmission of a joint “DO-A scheduling request”, with all devices with pending DO-A transmitting the same waveform on the same resource. This would indicate to the reader that at least one device has DO-A to transmit and the reader could then trigger collection of the DO-A transmissions, e.g. by using the procedure outlined in the polling-based method of section 2.3.1.1.

#### Resource allocation/determination for DO-A transmission - Summary

Proposal 4: Regarding resource allocation/determination methods for DO-A, further study the following methods, including combinations of these methods: - Polling by the reader - Recurring resource configuration provided by the reader

## Device Energy Management - Device (Un)availability

As far as energy/power is concerned, Device C is defined as follows:

- ≤ 1 mW to ≤ 10 mW peak power consumption

- has energy storage

This means that for device C the peak power consumption is much larger than the energy harvesting power: According to the SID [1], “the output power of energy harvester is typically from 1µW to a few hundreds of µW”. It follows that a period of device activity (e.g. transmission at high power) may be followed by an extended time during which the device C is not available or only partially available (e.g. it is available to receive R2D transmissions, but not available to transmit D2R at sufficient TX power for the required range).

During the Rel-19 study item, some potential solutions to address device (un)availability were studied [2]; the Rel-19 work item, however, only considered device 1 (~1 µW peak power consumption), and the device (un)availability issue was not critical for a device with such low power consumption. In Rel-20, the issue should be revisited.

Observation 2: For device 2b and especially for device C, peak power consumption is much higher than typical energy harvester output power. This could result in extended periods of full or partial device unavailability.

Proposal 5: Study device energy management, in particular solutions to address the issue of device (un)availability due to energy harvesting.

It could be beneficial for the reader to be aware of the device’s current energy storage level, achieved energy harvester output power and required power for RX and TX activities. That would avoid e.g. the case that the reader requests the device to transmit D2R when the device is unable to do so due to its depleted energy storage.

Proposal 6: Study solutions to assist the reader in predicting device (un)availability, e.g. based on device reporting the following information to the reader: device’s energy storage level, achievable energy harvester output power, power/energy required for RX and TX.

## M values for R2D

Agreement For R2D, study necessity of addition/removal of M values from Rel-19 M value set for Device 2b and for Device C for at least the following M values: - Addition of M=1 - Addition of M=32 - Removal of M=24

RAN1 made an agreement to study additional M values such as M=1 and M=32 for R2D transmissions. Introduction of M=1 may be beneficial to increase coverage distance as RAN1 is considering outdoor scenarios. Also, it may be easy to handle CP at the device side, and if device C is able to perform correlation operation, the benefit would be clear, but the benefit may be limited to device 2b.

If Rel-20 supports M=32, the peak data rate is increased. Also, it may be able to reduce latency as data transmission would be quickly done. However, with large M such as M=32, the considered fading channel model with large delay spread with 300 ns is around 10 % of a chip duration, may lead to inter-chip interference and error floors. In such case, M=32 performance would not be always better than M=24. It should be noted that motivation of removing M=24 is from the same issue, and some companies showed error flow for M=24 and M=32. If RAN1 removes M=24, addition of M=32 would not be properly justified.

We should be careful to remove M=24, which has been already introduced. For performance evaluation of Rel-20 A-IoT SI, TDL-C with RMS delay spread of 300 ns is a good fit for outdoor with NLOS scenario. Under this assumption, it may be hard to justify the necessity of such a large M value, but it should be noted that the air interface for device 2b/C studied for outdoor will be reused in indoor scenarios, based on SID. In addition, if the device is located to a reader, M=24 is still useful. Thus, we should not remove M=24.

Proposal 7: RAN1 should not remove existing M values such as M=24.

Proposal 8: RAN1 to capture pros and cons of addition of M=1 and M=32 for TR without a conclusion or recommendation for WI.

## FEC for R2D

RAN1 discussed an issue on FEC for R2D and the following is the meeting outcome of the previous meeting.

Agreement Study necessity and feasibility of FEC for R2D for Device 2b and for Device C including the following aspects: - Coding schemes: convolutional code (e.g TBCC), RM code - Note: focus on coding schemes already specified in LTE or NR - FEC code rates: at least 1/2, 1/3 - Interleaver or LTE bit collection scheme for TBCC

In Rel-19, RAN1 tentatively assumed that device 1 cannot perform decoding of convolutional coding scheme, which is clear. Now we are considering active devices defined as device 2b and device C. It is clear that introduction of FEC is beneficial to improve coverage performance for outdoor scenarios. However, the feasibility of FEC should be studied especially for Device 2b.

Based on an assumption of device C with a similar power budget (mW-level) of active RFID and BLE devices although the range is quite diverse, device C may be able to perform decoding convolutional code, as BLE 5.0 chips running on coin cells can do convolutional decoding. However, sub-mW level power budget should be considered for device 2b. There are research which demonstrated Viterbi decoding with less than 100 uW power budget such as [4], but it is questionable if it is implementable in a tiny wireless device. Also, we are assuming a few hundred uW as a peak power of device 2b, so the available power consumption for FEC decoding would be more limited. In addition, if it is feasible, how long the device 2b could be alive is also critical.

Proposal 9: RAN1 should study the implementation feasibility of FEC decoding of convolutional code for Device 2b with consideration of the power budget. E.g., implementation feasibility of ultra-low power Viterbi decoder at device 2b (e.g., < 100uW) and battery lifetime if it is applied.

## R2D/D2R Timing Offset

Agreement At least for DT and DO-DTT, study necessity and feasibility of changes to existing timing offsets for Device 2b and for Device C, e.g., - timing offset value between R2D and corresponding subsequent D2R (e.g., Toffset1/2/3/4) - timing offset value between D2R and corresponding subsequent R2D (e.g., T_D2R_min) - Note: RAN1 only discusses potential reasons that may impact the value of T_D2R_min, and if any provides those reasons to RAN4 - timing offset between R2D and corresponding subsequent D2R provided by a reader - timing offset between D2R and corresponding subsequent R2D provided by a reader. At least for DT and DO-DTT, study necessity and feasibility of introduction of new timing offsets for Device 2b and for Device C, e.g., - timing offset between two consecutive R2D or D2R transmissions for the same Device, if such scenario exists.

RAN1 discussed timing offsets, which have been defined in Rel-19. When RAN1 defined the Toffset parameters, the considered scenario is indoor and only for device 1. A device 1 performs backscattering without carrier generation, which means there is very tight coupling between R2D and D2R. In Rel-20, device 2b/C performs active transmission with higher Tx power. In addition, timing error due to SFO and processing time were also reflected to the defined offset for device 1. For device 2b/C, SFO is different, and a processing time is also expected to be different than that of device 1. For example, if we introduce R2D FEC is introduced, the device needs enough time to decode a PRDCH. In our understanding, depending on introducing features, the required timeline should be reconsidered. Thus, we propose that RAN1 identifies factors affecting the R2D/D2R transmission timeline considering expected features in the study item, and discuss the necessity change of the timing offset.

Proposal 10: RAN1 identifies factors which potentially affect the existing timing offset such as R2D FEC, SFO, longer RTT, by supporting Device 2b/C especially for outdoor scenarios.

# Conclusion

In this contribution we have made the following observations and proposals:

Observation 1: D2R power control is necessary to address near-far problems. The received signal power difference from a Device 2b and Device C may be too large depending on their distance with respect to a reader and their transmission power.

Proposal 1: Study the way to enable a certain group of A-IoT devices to transmit D2R responses to a R2D signal so that received signal power of the D2R could be within a certain range, based on indications from network and measurements made at the A-IoT devices.

Proposal 2: For D2R Tx power control, the required information at reader side at least includes energy level of A-IoT device.

Proposal 3: RAN1 study CFO mitigation mechanisms (e.g., preamble-based scheme and/or calibration signal design)

Proposal 4: Regarding resource allocation/determination methods for DO-A, further study the following methods, including combinations of these methods: - Polling by the reader - Recurring resource configuration provided by the reader

Observation 2: For device 2b and especially for device C, peak power consumption is much higher than typical energy harvester output power. This could result in extended periods of full or partial device unavailability.

Proposal 5: Study device energy management, in particular solutions to address the issue of device (un)availability due to energy harvesting.

Proposal 6: Study solutions to assist the reader in predicting device (un)availability, e.g. based on device reporting the following information to the reader: device’s energy storage level, achievable energy harvester output power, power/energy required for RX and TX.

Proposal 7: RAN1 should not remove existing M values such as M=24.

Proposal 8: RAN1 to capture pros and cons of addition of M=1 and M=32 for TR without a conclusion or recommendation for WI.

Proposal 9: RAN1 should study the implementation feasibility of FEC decoding of convolutional code for Device 2b with consideration of the power budget. E.g., implementation feasibility of ultra-low power Viterbi decoder at device 2b (e.g., < 100uW) and battery lifetime if it is applied.

Proposal 10: RAN1 identifies factors which potentially affect the existing timing offset such as R2D FEC, SFO, longer RTT, by supporting Device 2b/C especially for outdoor scenarios.

# References

- RP-252964, ”Revised SID: Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices”, RAN#109

- TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR”

- TR 38.848, “Study on Ambient IoT (Internet of Things) in RAN”

- S L Haridas and Dr. N. K. Choudhari. “Very Low Power Viterbi Decoder Employing Minimum Transition and Exchangeless Algorithms for Multimedia Mobile Communication”. International Journal of Advanced Computer Science and Applications (IJACSA) 2.12 (2011).