3GPP TSG RAN WG1 #122 bis	R1-2507620

Prague, Czech Republic, Oct 13th - 17th, 2025

Source: 	MediaTek Inc.

Title:	Study of air interface for Device 2b/C

Agenda item:	10.3.2

Document for:	Discussion/Decision

- Introduction

In RAN #109, the revised SID on enhancements for solutions for Ambient IoT in NR outdoor for active devices was described in the [1]:

Objectives - Study necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT in outdoor scenarios under the following conditions [RAN1-led]: - Deployment scenario 4 with topology 1 with no external carrier wave. - Traffic types DO-DTT, DT, DO-A. - Representative use cases rUC5 (outdoor inventory), rUC6 (outdoor sensor), and rUC8 (outdoor command). - Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. - Assumption of the same air interface for Device 2b and Device C. - Take into account the aspects reported in Clause 6.10 “DO-A assessment” and Clause 6.1.1.7 “CFO calibration signal for device 2b” of TR 38.769 - Positioning for outdoor scenarios is under objective 2. This objective begins after RAN#108 (June 2025) RAN#111 (March 2026) will make a decision on whether to include outdoor scenarios in Rel-20 normative work. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage. - Identify D2R RSRP-like measurement, and the involved A-IoT signal(s)/channel(s), which are feasible for Device 2b/Device C for more accurate outdoor Device localization than based on Reader-ID [RAN1]. - RAN1 is not expected to conclude on the positioning accuracy as part of the identification of feasible D2R RSRP-like measurement - Note: Device localization based on Reader ID as specified in Rel-19 can be supported for active device(s) in Rel-20 with no additional work. - The design of D2R signal(s)/channel(s) for active devices is not impacted by the identification of the D2R RSRP-like measurement - This objective does not include the study of dedicated Device procedures for Device localization - The objective assumes no dedicated positioning architecture will be specified in Rel-20 - The objective assumes no specification of inter-Reader coordination for Device localization in Rel-20 - This objective will start at RAN1#124

In this contribution, we discuss the necessary and feasible changes to the Rel-19 A-IoT specifications to support A-IoT active devices in outdoor scenarios.

- General aspects

The current air interface specifications for ambient IoT NR have been primarily designed with indoor coverage assumptions, which may not translate effectively to outdoor scenarios without appropriate modifications. According to the link budget coverage evaluation for R19 A-IoT in outdoor scenario in Table 1, the air interface enhancement may be various for different scenarios and assumptions.

Table 1. Coverage evaluation

| Direction | device type | PL model (NLOS) | NR ISD*0.5 [D0] /m | Distance (2D) /m with penetration margin of 20 dB | Distance (2D) /m with penetration margin of 0 dB | Distance (2D) /m with penetration margin of 20 dB | Distance (2D) /m with penetration margin of 0 dB |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | BLER 10% |  | BLER 1% |  |
| D2R | Device 2b | UMa | 250 | 154 | 505 | 100 | 334 |
|  |  | RMa | 866 | 262 | 869 | 171 | 572 |
|  |  | UMi | 100 | 133 | 492 | 84 | 312 |
| D2R | Device C | UMa | 250 | 375 | 1222 | 248 | 809 |
|  |  | Rma | 866 | 645 | 2126 | 424 | 1400 |
|  |  | UMi | 100 | 355 | 1309 | 225 | 829 |

By establishing clear coverage targets as a foundational element of the study, the necessary modifications to the air interface can be systematically identified, evaluated, and specified in a manner that ensures both technical feasibility and commercial viability for outdoor ambient IoT deployments. Thus, we have the following proposal:

Observation 1: The necessary and feasible changes to the Rel-19 air interface to support outdoor scenarios are highly related with the coverage target.

Proposal 1: The coverage target need to be considered to study necessary and feasible changes to air interface to support outdoor scenarios.

According to the coverage evaluation results, the UMa and UMi deployment scenarios seems to be more feasible. And to achieve a continuous coverage from the device perspective, the A-IoT outdoor coverage of 250m and 100m should be achieved, which the 250m and 100m target is derived from the ISD*0.5. Thus, an enhancement of approximate 9 dB of the required SNR is to be assumed at BLER 10%.

Proposal 2: Support the enhancement of D2R required SNR of 9 dB at BLER 10%

- D2R aspects

Due to the presence of substantial frequency and sampling offsets—most notably in device 2b—a marked degradation in the performance of BPSK modulation was observed. As depicted in Figure 1, the performance curve for BPSK reaches a floor at around 10% BLER, indicating that further improvements in SNR do not yield significant reductions in block error rate under these conditions. In contrast, the performance of DBPSK under the same circumstances is considerably more robust; its curve is able to achieve a BLER as low as 1%, demonstrating a clear advantage over conventional BPSK in environments with large frequency and sampling mismatches, especially for large message size cases, e.g., 96 bits or 400 bits.

This improved resilience of DBPSK can be attributed to its method of data bit extraction, which relies on detecting the relative upward or downward trends between consecutive symbols, rather than absolute phase information. As a result, DBPSK is inherently less sensitive to the impairments caused by frequency and sampling offsets, which tend to disrupt absolute phase coherence. These findings suggest that, in scenarios where such offsets are significant and difficult to compensate for, DBPSK offers a more reliable alternative to BPSK.

In light of these results, we propose the following recommendations to enhance system robustness under challenging frequency and sampling conditions:

Figure 1. PDRCH performance with BPSK and DBPSK modulation method

Observation 2: D2R DBPSK modulation method has better performance than BPSK for large SFO and large payload size.

Proposal 3: Support D2R DBPSK modulation method

From the link budget and coverage evaluations, we can see that D2R distance is shorter than R2D at the same circumstances (e.g., BLER assumptions). This observation indicates that, from a holistic system perspective, the overall coverage limitation—or bottleneck—will manifest in the D2R direction. In other words, the maximum achievable coverage area for the system is constrained by the D2R link, rather than the R2D link.

For the physical layer design in Release 19, the block repetition factor specified for the block repetition procedure is R = {1, 2}. However, our analysis shows that with a maximum repetition number of only 2, it is not possible to achieve continuous coverage in the D2R direction, especially in scenarios where signal attenuation or interference is significant. This limitation could result in coverage gaps and reduced reliability for devices located at the edge of the coverage area.

Therefore, to address this issue and enhance the robustness of the D2R link, we propose supporting a larger repetition factor specifically for the D2R direction. By allowing for higher repetition numbers, the system can improve signal reliability and extend the effective coverage range, thereby mitigating the coverage bottleneck and ensuring more consistent connectivity throughout the service area.

Proposal 4: Support larger block repetition number for D2R, e.g., R = {1,2,4,8,16}

- R2D aspects

From the perspective of coverage enhancement, there has been interest in investigating the potential benefits of adopting a lower M value, specifically M = 1, to potentially extend coverage or improve system flexibility. However, simulation results indicate that utilizing M = 1 leads to inferior performance compared to M = 2.

The primary reason for this performance degradation is that an M value of 2 introduces a power boosting effect in the time domain signal amplitude. This effect is particularly beneficial for signal detection, where signal strength is typically weakest. For instance, as illustrated in Figure 2, the non-zero amplitude observed with M = 2 is twice as large as that with M = 1. This increased amplitude directly contributes to improved detection performance.

Therefore, while the idea of lowering the M value for coverage enhancement was initially considered if using edge detection method, the empirical evidence suggests that maintaining M = 2 is preferable for the R2D direction. Based on these findings, we recommend retaining the minimum M value of 2 for R2D transmissions in future system designs.

Figure 2. the time domain signal amplitude of M = 1 and M = 2

Observation 3: The performance of M=2 is better than M=1 due to power boosting effect

- DO-A related aspects

In the last R122 meeting, the following agreement is supported:

Agreement Study the following aspects to support DO-A traffic (periodic and event-triggered) for Device 2b and for Device C in Rel-20. - Timing and frequency synchronization for DO-A transmission - Resource allocation/determination method for DO-A transmission - Configuration/signaling of necessary information

From our perspective, it is essential to further discuss and clarify the specific characteristics of DO-A traffic. In particular, the transmission interval and message payload size for DO-A traffic are critical factors that are closely tied to the design of the DO-A procedure, and by extension, to the detailed physical layer design. For instance, the physical layer requirements and design considerations may differ significantly between periodic DO-A transmissions and event-triggered DO-A transmissions. Furthermore, if both periodic and event-triggered DO-A traffic types are to be supported simultaneously, additional complexity may arise in the physical layer design to accommodate the differing requirements of each traffic type.

Therefore, we recommend a comprehensive analysis of the DO-A traffic characteristics, including typical transmission intervals and payload sizes and etc., to inform the development of efficient and robust physical layer solutions. This will ensure that the system can effectively support the diverse requirements of DO-A traffic in future releases.

Observation 4: The DO-A procedure design is highly related with the DO-A traffic characteristics.

Proposal 5: Support further study of DO-A traffic characteristics, e.g., DO-A interval, typical message size, etc.

DO-A traffic is not supported with Release 19 physical layer design. For other traffic types, such as Device-originated by device-terminated trigger (DO-DTT) and device-terminated (DT) traffic, the resources required for D2R transmission are indicated within an R2D transmission. However, this resource indication mechanism is not applicable, at least for the initial D2R transmission in the case of DO-A traffic since there is no prior R2D transmission to convey the necessary resource information.

Prior to the initial D2R transmission of DO-A traffic, there is no established synchronization between the device and the reader. This absence of synchronization means that critical parameters, such as CFO and SFO, can be extremely large. Specifically, CFO may reach values as high as 10^4 parts per million (ppm), and SFO may be as high as 10^3 ppm before any calibration procedures is performed. To mitigate these issues, it is essential for the device to first receive a R2D signal before attempting its initial D2R DO-A transmission. The R2D signal acts as a reference, allowing the device to perform necessary synchronization operations, including the calibration of both CFO and SFO. By aligning its timing and frequency parameters with those of the reader, the device can substantially reduce these offsets, thereby establishing a stable and reliable communication link for subsequent D2R transmissions.

Given the critical role of robust synchronization in enabling successful DO-A traffic transmission, it is recommended that further studies be conducted to determine the optimal design of the R2D synchronization signal.

Observation 5: A R2D synchronization signal is needed before the DO-A transmission.

Proposal 6: Support of a signal for DO-A R2D synchronization.

- Conclusion

In this contribution, the following observation and proposals are made:

Observation 1: The necessary and feasible changes to the Rel-19 air interface to support outdoor scenarios are highly related with the coverage target

Observation 2: D2R DBPSK modulation method has better performance than BPSK for large SFO and large payload size.

Observation 3: The performance of M=2 is better than M=1 due to power boosting effect

Observation 4: The DO-A procedure design is highly related with the DO-A traffic characteristics.

Observation 5: A R2D synchronization signal is needed before the DO-A transmission.

Proposal 1: The coverage target need to be considered to study necessary and feasible changes to air interface to support outdoor scenarios

Proposal 2: Support the enhancement of D2R required SNR of 9 dB at BLER 10%

Proposal 3: Support D2R DBPSK modulation method

Proposal 4: Support larger block repetition number for D2R, e.g., R = {1,2,4,8,16}

Proposal 5: Support further study of DO-A traffic characteristics, e.g., DO-A interval, typical message size, etc.

Proposal 6: Support of a signal for DO-A R2D synchronization.

- Reference

- RP-252964, Revised SID: Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices, LG Electronics, RAN #109, September 15-18, 2025.