3GPP TSG RAN WG1 #122bis			R1-2507792

Prague, Czech, Oct 13th – 17th, 2025

Source:	NTT DOCOMO, INC.

Title:	Maintenance on solutions for Ambient IoT in NR

Agenda Item:	8.4

Document for: Discussion and Decision

- Introduction

At the previous RAN1 meetings, CRs have been agreed for TS38.291.

In this contribution, remaining issues on solutions for ambient IoT in Rel-19 are discussed.

- Discussion

  - Overlapped Msg1 in time domain when padding chips exist for R2D

According to the current specification[1], time offset between the end of R2D which triggers Msg1 and second time resource for Msg1 is specified as follows;

If the D2R transmission is for a Random ID message - if after chip χR2D= χendR2D there are potential access occasion(s), as defined in TS 38.391 [3], for the transmission which are earlier in time than the access occasion selected for the transmission - the device shall set TR→D=1.25Toffset+MchipD2RTchipD2R

For TR→D=1.25Toffset+MchipD2RTchipD2R, it is derived as Toffset + Toffset2, where Toffset is time offset between the end of R2D which triggers Msg1 and the start of first Msg1, and Toffset2 is D2R duration for first Msg1 (=MchipD2RTchipD2R) and time margin as guard period considering 10% SFO during Toffset and first Msg1.

In the current specification[1], the time margin included in Toffset2 above only considers 10% SFO during Toffset and MchipD2RTchipD2R. This is based on the assumption that device get synchronized until the end of R2D, e.g., based on Manchester coding which has rising/falling edge at the middle of every single codeword, and the time drift starts from the end of R2D.

For R2D, it was also specified that padding chips are appended at the end of R2D transmission when the end of postamble is not aligned with OFDM symbol boundary. According to the current specification[1], padding chip can be any value other than R-TAS SIP or two consecutive ON chips at the end for M=24. Given that ON/OFF pattern or location on rising/falling edge is unclear from device perspective during padding chips, it is expected that it would be difficult for device to get synchronized during padding chips.

Following postamble bit P3, the smallest integer Npad≥0 padding chips are inserted, if needed, until MchipR2D -NSIP modulo Mchipsymb=0. The padding chips are set to any values which do not result in another R-TAS SIP, and if Mchipsymb=24 values of 1 are mapped to the final two padding chips.

In addition, R2D postamble is specified as 4 consecutive ON chips and it is unclear whether device can keep synchronized during postamble.

Due to the time drift during padding chips and possibly postamble, it is concerned that accumulative timing error can be larger than what is assumed in current specification, then Msg1 transmissions from different devices may overlap in time which affect to the decoding performance at reader.

Furthermore, according to RAN4 specification[2], device transmission timing error is specified as Te as follows.

For D2R transmission, the reference point for the device transmission timing is TR→D after the end of the corresponding R2D transmission for the device as define in TS 38.291. The device transmission timing error shall be less than or equal to Te. The value of Te is equal to 10%*TSFO, where TSFO is the length in microseconds from the last transition edge of PRDCH intended for the device in the corresponding R2D transmission to the starting time of the D2R transmission.

TSFO starts from the last transition edge of PRDCH. According to the current RAN1 specification[1], no transition edge exists in postamble and also it is not promising that padding has transition edge. This means that TSFO can start from before postamble and can be at most 10%*(Tpostamble+Tpadding+Toffset+MchipD2RTchipD2R). Even if device fulfil the timing error requirement, Msg1 from different devices may overlap.

Observation 1: For time offset for second Msg1 transmission occasion in time domain, potential time drift during padding chips and postamble are not considered for TR→D.

In this context, it should be discussed how to address the issue on the time drift during padding chips (and potentially postamble) for R2D which triggers Msg1 transmission if padding chips exist, otherwise, Msg1 from different device may overlap in time domain. More specifically, following alternatives can be further considered;

- Alt.1: 10% SFO during padding chips (and postamble) are taken into account for Toffset2.

- Alt.2: pattern on padding chips are specified so that it can be used for time synchronization

For Alt.1, Toffset2 should be larger than 2e1-eTpadding+  2e1-eToffset+ 1+e1-e Tmsg1, where Tpadding is duration for padding chips, Tmsg1 is duration for Msg1 and e is 10% SFO. Therefore, time offset for second time resource for Msg1 is expected to be TR→D=1.25Toffset+MchipD2RTchipD2R+0.25Tpadding, where Tpadding is duration for padding chips for end of R2D. In addition, if the time drift for postamble is considered as well, the time offset is expected to be TR→D=1.25Toffset+MchipD2RTchipD2R+0.25Tpadding+Tpostamble, where Tpostamble is time duration for R2D postamble, i.e., 4 ON chips.

For Alt.2, in our understanding, it is not necessary to specify a specific pattern for padding chips and it may be sufficient to specify a rule such as “rising/falling edge exist per two chips” like applying Manchester coding etc.

Proposal 1: For time offset for second time resource for Msg1 transmission, discuss how to address the issue on the time drift during padding chips (and potentially postamble) for R2D which triggers Msg1 transmission if padding chips exist.

- Alt.1: 10% SFO during padding chips (and postamble) are taken into account for Toffset2.

  - i.e., TR→D=1.25Toffset+MchipD2RTchipD2R+0.25Tpadding+0.25Tpostamble

- Alt.2: pattern on padding chips is specified so that it can be used for time synchronization

Fig.1: Definition on TR→D for Msg1 Tx according to current specification TS38.291 V19.0.0.

- Conclusion

In this contribution, we discussed remaining issue on solutions for A-IoT in Rel-19. Based on the discussion, we made following observations and proposal.

Observation 1: For time offset for second Msg1 transmission occasion in time domain, potential time drift during padding chips and postamble are not considered for TR→D.

Proposal 1: For time offset for second time resource for Msg1 transmission, discuss how to address the issue on the time drift during padding chips (and potentially postamble) for R2D which triggers Msg1 transmission if padding chips exist.

- Alt.1: 10% SFO during padding chips (and postamble) are taken into account for Toffset2.

  - i.e., TR→D=1.25Toffset+MchipD2RTchipD2R+0.25Tpadding+0.25Tpostamble

- Alt.2: pattern on padding chips is specified so that it can be used for time synchronization

Reference

[1] TS38.291 V19.0.0, Ambient IoT Physical layer (Release 19), July 2025.

[2] TS38.191 V1.0.0, Ambient IoT device radio transmission and reception (Release 19), Sep. 2025.