3GPP TSG RAN WG1 #123                                                            R1-2508428

Dallas, USA, November 17th – 21th, 2025

Source:	vivo

Title:	Evaluation methodology and performance evaluation for 5G-A ISAC

Agenda Item:	10.5.1

Document for:	Discussion and Decision

Introduction

A new study on integrated sensing and communications (ISAC) has been approved in RAN #108 [1], and in the RAN1 #122b meeting [4], we have certain constructive agreements as follows:

Agreement-1 On False alarm probability Type 2 (targets dropped in simulation area), the parameter K is the number of drops with at least one detected object. Conclusion-1 Sensing resolution is not considered as a performance metric for evaluation of NR ISAC. Agreement-2 The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. BS antenna configuration; For 4GHz, 4.9GHz: - Option 1 as optional - Tx: (12,16, 2,1,1;2,16) - Rx: (12,16,2,1,1;2,16) - Option 2 - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8); For 6GHz: - Option 1 as optional - Tx: (16,16, 2,1,1;4,16) - Rx: (16,16,2,1,1;4,16) - Option 2: - Tx: (8,8,2,1,1;4,8) - Rx: (8,8,2,1,1;4,8). Mandatory: dH,dV = (0.5, 0.8)λ, +45°/-45° polarization Optional: dH,dV = (0.5, 0.5)λ, +45°/-45° polarization. - Note: Tx antenna elements and Rx antenna elements are operating simultaneously Agreement-3 The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. BS antenna radiation pattern; Table 9 in Report ITU-R M.2412. BS antenna mechanic tilt (downtilt angle without electrical tilt); 90° in GCS (pointing to horizontal direction). BS antenna electrical tilt (downtilt angle after mechanic and electrical tilt); Option 1: no electrical tilt Option 2: 102° in GCS. Polarized antenna model; Model-2 in clause 7.3.2 in TR 38.901. Agreement-4 The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. gNB-target link; gNB-aerial UE of UMa/RMa parameters in 36.777 with Alternative 3. Concatenation of TX-target and target-RX links; Up to company choice between two options for concatenation defined in Step 9 in section 7.9.4.1, TR 38.901. The power threshold for path dropping after concatenation for target channel; -25dB and -40dB are respectively used for the two options for concatenation. Agreement-5 The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. Co-site inter-sector interference; Not modelled. Adjacent channel interference; Not modelled. BS receiver noise figure; 5dB. Agreement-6 When Tx/Rx operates simultaneously (baseline case), the following parameters are used for the evaluation on NR ISAC: Antenna isolation; - Option 1: 65dB - Option 2: 80dB. Maximum BS Tx power; - Option 1: 37dBm - Option 2: 52dBm The above options are calculated with BS_maxpower = BS Rx saturation power + antenna isolation by assuming the BS Rx saturation power = -28dBm and the antenna isolation = 65dB and 80dB, respectively. Optionally: companies should report the maximum BS Tx power when it is assumed that Tx and Rx don’t operate simultaneously. Companies should report how the maximum BS Tx power is derived. Agreement-7 The following evaluation parameters are agreed for the evaluation on NR ISAC. Parameters; Assumptions. CPI (coherent processing interval); Companies to report the CPI value assumed. Waveform; CP-OFDM as baseline, other waveform is up to companies report. - Agreement-8 When Tx/Rx operates simultaneously, the following evaluation parameters are agreed for the evaluation on NR ISAC. Self-interference; The residual leakage interference/noise is modelled e.g. by additional additive white Gaussian noise, -94+X dBm in 100 MHz, X is up to company report. Companies to provide details on their modelling. Note: X = -Infinity corresponds to not modelling self-interference. Agreement-9 For the evaluation on NR ISAC, - 3 kinds of resources are defined - Type 1: resources that are used for sensing signal transmission - Type 2: part of Type 1 resources that are used for communication purpose - Note: it is possible the resource type 2 doesn’t exist - Type 3: resources that are not used for sensing signal transmission, and cannot be used for communication purpose due to sensing operation - Both options should be reported for sensing resource ratio - Option 1: (Type_1 + Type_3) resources over all radio DL and UL resources - Option 2: (Type_1 - Type_2 + Type_3) resources over all radio DL and UL resources - Note: if Type 2 resource doesn’t exist, two options are the same For the evaluation on NR ISAC, - Company should report T/F RE mapping of sensing RS, assumed TDD UL/DL configuration if applicable. - Company should report which sensing RS resources are considered as Type 2 resource and related reason. Agreement-10 Companies are encouraged to report the high-level sensing signal/data processing method used in the evaluation, e.g., 2D FFT, MUSIC, and any other methods. Agreement-11 Up to company to model low power cluster in the evaluation of NR ISAC Agreement-12 Companies should report details of the Tx beam information (number of Tx beams, wide/narrow Tx beam) being used at TRP. - The details of the Tx beam information (number of Tx beams, wide/narrow Tx beam) being used at TRP will be captured along with the corresponding evaluation results in the Rel-20 TR Agreement-13 The following performance objectives are adopted for evaluation purpose of NR ISAC. Metric; Value. Missed detection Probability; [5]%. False Alarm Probability Type 1; [5]%. False Alarm Probability Type 2; [5]%. Horizontal Positioning Accuracy; [10] m with confidence level 90%. Vertical Positioning Accuracy; [10] m with confidence level 90%. Velocity Accuracy; [5] m/s with confidence level 90%.

Although the agreements listed above may pursue our discussion further, there are still some missing issues that we need to be confirmed and verified in the coming meeting. In this contribution, we express our views on the evaluation methodology, signal configuration, measurement quantities, and evaluation parameters, and provide our initial evaluation results.

Evaluation Methodology

Remaining issues on evaluation assumptions

Some issues have remained on evaluation assumptions, such as Tx/Rx selection, antenna configuration, and sensing information fusing and metric calculation, that we need to be confirmed and verified.

Tx/Rx selection

In the RAN1#122b meeting, three Tx/Rx selection methodologies were mentioned as follows:

- Direct selection of all 21 TRPs for sensing

- TRP selection based on sensing targets

- TRP selection based on sensing area

The direct selection of 21 TRPs for sensing would require orthogonal sensing resources among TRPs due to the absence of inter-TRP interference modeling, resulting in significant resource overhead and computational complexity. For target-based TRP selection, where optimal Tx/Rx pairs are selected for each target, several issues have been identified. This approach may not facilitate aligned simulation performance across different companies. For instance, with 15 targets in 3 sectors, per-target Tx/Rx selection could result in extreme cases where each target is associated with a different Tx/Rx pair or all 15 targets are associated with the same Tx/Rx pair, leading to significant performance variations. Consequently, different Tx/Rx pair selection methodologies may lead to different target distributions (varying numbers of targets per Tx/Rx pair), making simulation performance alignment challenging even with identical target number and Tx/Rx pairs per drop. Furthermore, without prior knowledge, no deterministic criteria exist for selecting appropriate TRPs per target.

Considering these factors, we prefer the area-based TRP selection approach. In our simulation, we assume that all sensing targets (i.e., 15 sensing targets across 3 sectors) are associated with each TRP. The area to be sensed comprises the three central sectors, and TRPs are selected based on geographical location: the TRP associated with the target-dropping central sector, along with TRPs from adjacent sectors, can be selected for sensing participation. This significantly simplifies the Tx/Rx pair selection procedure, and ensures uniformity in the distribution of sensing targets, meaning that the number of sensing targets associated with each Tx/Rx pair is consistent, which is advantageous for aligning simulation performance. To ensure effective coverage of the sensing area, we propose two TRP selection schemes, as illustrated in Figure 1.

- Scheme 1: Select TRPs associated with the three target-dropping sectors (marked in red) plus the 9 nearest neighboring sectors (marked in green and blue), resulting in 12 participating TRPs (marked in green, blue and red).

- Scheme 2: Consider both geographical location and TRP coverage performance, selecting 3 TRPs (marked in green) from the 9 nearest neighboring sectors that provide better coverage of the target area, resulting in 6 participating TRPs (marked in green and red).

Figure 1: TRP selection based on sensing area

TRP selection based on sensing area should be supported to reduce resource overhead and computational complexity.

Antenna configuration

The agreement-2 on antenna configuration listed in Section 1 was made in RAN1 #122b.

For antenna configuration, Option 2 is employed in our simulation, and the beamforming methodology is as follows: sensing signals are first mapped to 4×8×2 TxRUs through digital precoding, then to 8×8×2 antenna elements via analog beamforming. The digital precoder generates a wide beam, while the analog beamforming employs a DFT codebook of 2 antenna elements with directional beams.

Regarding the beam set configuration, if prior target information is available, the N beams in the set may be steered toward N respective sensing targets. If such prior information is unavailable, the beams should provide coverage for the specified sensing area. In this case, the number and directions of beams should be determined by balancing sensing performance and simulation complexity.

In the simulation, a fixed horizontal analog beam and several differently pointed vertical analog beams could be employed. Beam sweeping achieves full coverage of the UAV airspace (zenith angle from 0° to 90°), using the strategy shown in Figure 2, where candidate beams are swept slowly with dwell time Tp per beam corresponding to the coherent processing interval (CPI). This beam sweeping approach requires no increase in time-domain resource density for sensing signals within each CPI.

Figure 2: Beam configuration for the sensing evaluation.

To achieve full coverage of the UAV airspace, an appropriate beam set configuration combined with an effective beam sweeping strategy is required.

Sensing information fusing and metric calculation

For single TRP monostatic sensing, no sensing information fusion is required as metrics are directly calculated using the current TRP's sensing results and corresponding ground truth. When multiple TRPs operate independently, metrics are computed based on each TRP's individual sensing results and their respective ground truth. In the case of multi-TRP monostatic sensing that requires result fusion from all participating TRPs, i.e., cooperative sensing, different TRPs' sensing information can be clustered and merged based on the detected targets' position coordinates, or other sensing data. The metrics are then calculated by comparison between the fused sensing results and the ground truth.

Specifically, it should be clarified whether the metric is calculated based solely on single-TRP monostatic sensing results or jointly from multiple TRP monostatic sensing results, where the former typically yields inferior performance and thus warrants relatively lower requirements.

The metric of sensing can be calculated based either solely on single-TRP monostatic sensing results or jointly from multiple TRP monostatic sensing results, where the former usually yields inferior performance and thus has relatively low requirements, while the latter usually offers superior performance and thus has relatively high requirements.

Companies report which option is used in the simulation:

- Option1:  The sensing results are calculated based on single TRP's information.

- Option2: The sensing results are calculated based on fusion from multiple TRP’s sensing information, where each TRP performs monostatic sensing.

Micro-Doppler Modeling for UAV

Sensing targets or their components often exhibit micro-motion such as vibration, rotation, and acceleration. These micro-motions modulate the phase of the echo signal, resulting in corresponding frequency modulation, which is known as the micro-Doppler effect. To support the micro-motion detecting and tracking, the micro-Doppler model in current ISAC channel model should be further enhanced since the Doppler frequency is defined as:

fD,n',m',n,mk,pt=rrx,k,p,n',m'Ttvrxt+rk,p,n',m'Ttvk,ptλ0+rtx,k,p,n,mTtvtxt+rk,p,n,mTtvk,ptλ0

where the velocity of micro movement has not been defined. However, different micro movement type may lead to different mathematical model. For instance, the rotational local movement, such as the rotation of the wings from UAV, which is moving regularly, can be modeled as the circular model and the micro velocity can be modeled by the circular law. On the other hand, the non-circular movement with obvious regular movement like swinging of human arms or the chest cavity movements caused by breathing can be modeled as the mathematical model of trigonometric functions. Therefore, in order to provide the evaluation basic for the detection or tracking in the use case of micro motion, the detail velocity model of micro movement can be modeled based on the sensing use case with several typical motion characteristics. The detailed discussion and the proposals about micro-Doppler formula can be found in [3].

For UAV use cases, micro-Doppler is primarily caused by the rotation of UAV blades. When modeling the Doppler shift of UAVs, in addition to considering the UAV's translational motion, it is necessary to model the micro-Doppler characteristics based on the angular velocity and rotational radius of the UAV blades. Micro-Doppler detection involves extracting time-domain signal sequences associated to detected paths (sampling points), followed by time-frequency analysis to derive corresponding micro-Doppler information, as illustrated in Figure 3. The spectrum reflects combined Doppler frequency related to the translational motion of the UAV and micro-Doppler induced by its two rotating blades.

Figure 3: Illustration of UAV Micro-Doppler spectrum.

Micro-Doppler modeling is beneficial not only for identifying characteristics of different UAVs but also enhances UAV detection performance, particularly when the radial velocities of the UAV is low and its echo may be obscured by static clutter. Modeling the blade rotation-induced micro-Doppler improves detection probability as illustrated in Figure 4. The left subfigure without micro-Doppler modeling shows no detectable target in the delay-Doppler spectrum after static clutter suppression, whereas the right subfigure with micro-Doppler modeling demonstrates distinguishable target echoes from static clutter due to the micro-Doppler signature.

Figure 4: Delay-Doppler spectrum with/without micro-Doppler modeling.

Micro-Doppler modeling improves detection probability of UAVs under low radial velocity conditions and facilitates identification of UAV-specific characteristics, enabling differentiation of UAV types and discrimination from other flying objects such as birds.

Signal Configuration and Resource Overhead Calculation

For TRP monostatic sensing, NR downlink reference signals such as CSI-RS could be prioritized for sensing purposes. Sensing unambiguous range and resolution performance depend on signal resource configuration. Table 1 summarizes the relationship between the resource configuration of sensing signals and their unambiguous range and resolution performance for monostatic sensing.

Table 1: Sensing performance and calculation formulas for monostatic sensing.

| Sensing Performance | Calculation Formulas |
| --- | --- |
| Unambiguous range | Rmax=c2∆F |
| Unambiguous velocity | vmax=±λ4∆T |
| Range resolution | ∆R=c2B |
| Velocity resolution | ∆v=λ2TCPI |

where B is the bandwidth, ∆F is the frequency-domain resource spacing, ∆T is the time-domain resource spacing, TCPI is the coherent processing interval, c is the speed of light, and λ is the wavelength.

In NR systems, CSI-RS is primarily used for downlink channel measurements, beam management, time-frequency tracking, and mobility management. It offers flexible signal resource configuration, allowing for either periodic, or semi-persistent, or aperiodic transmissions, with time-frequency domain resources dynamically allocated by the system based on its intended use. For the evaluation of UAV use case, periodic CSI-RS configurations enable adaptable Coherent Processing Interval (CPI) selection at the receiver to satisfy velocity resolution requirements, while configurable bandwidth addresses range resolution constraints. Current specifications limit a CSI-RS resource to a maximum frequency density of 3 resource elements per resource block and a minimum periodicity of 4 slots. At 6GHz carrier frequency with 30 kHz subcarrier spacing, this configuration yields a maximum unambiguous range of 1250 meters and maximum unambiguous velocity of 22.5 km/h. In Urban Macro (UMa) deployments with 500-meter inter-site distance, the supportable maximum unambiguous range is sufficient for the measurement requirements; however, UAV velocities exceeding 22.5 km/h surpass measurable unambiguous velocity limits. The maximum horizontal speed of UAV agreed in RAN1 #122 is 180 km/h, and the minimum periodicity of CSI-RS should be 0.5 slot to strictly satisfy the maximum unambiguous velocity requirement at 6GHz carrier frequency with 30 kHz subcarrier spacing. Consequently, meeting sensing measurement requirements may necessitate either configuring multiple CSI-RS resources or extending the minimum transmission periodicity supported by CSI-RS.

To address the unambiguous velocity requirement without prior knowledge of UAV position/velocity for ambiguity resolution, the following CSI-RS resource configuration is proposed:

- Number of CSI-RS resources: 6 single-port NZP CSI-RS resources.

- Frequency density: 3 REs per RB.

- Time-domain periodicity: 5 slots.

- Slot offsets within a period:

  - Resources #0 & #1: Offset = 0 slot

  - Resources #2 & #3: Offset = 1 slot

  - Resources #4 & #5: Offset = 2 slots

- Symbol locations within a slot:

  - Resources in the same slot are mapped to symbol 0 and symbol 7.

This configuration yields a sensing signal with a frequency density of 3 and an effective transmission period of 0.5 slot. The resource pattern for the sensing signal is illustrated in Figure 5. To align with TDD frame structure constraints, the sensing signal is transmitted only in downlink slots (D slots) or in the downlink symbols of special slots (S slots).

Figure 5: Illustration of sensing RS resource mapping.

To address the unambiguous velocity requirement without prior knowledge of UAV position/velocity for ambiguity resolution, a sensing signal with an effective transmission period of 0.5 slot is required.

Based on the aforementioned sensing signal configuration, the resource overhead for sensing signals per participating TRP is calculated as follows:

- If no power boosting is applied to OFDM symbols carrying sensing signals, the sensing signal resource overhead per TRP is given by:

O1TRP=ρNscRB×Nsymb-sensslotNsymbslot×Nslot-sensframeNslotframe

- If power boosting is applied to OFDM symbols carrying sensing signals, the subcarriers that do not carry sensing signals within these OFDM symbols cannot be used for communication, and the sensing signal resource overhead per TRP is calculated as:

O1TRP=Nsymb-sensslotNsymbslot×Nslot-sensframeNslotframe

Where ρ denotes the RS frequency density, NscRB denotes the number of subcarriers per resource block, Nsymb-sensslot denotes the number of OFDM symbols carrying sensing signals per slot, Nsymbslot is the number of OFDM symbols per slot, Nslot-sensframe denotes the number of slots carrying sensing signals per radio frame, Nslotframe denotes the number of slots per radio frame.

When NmultiTRP (NmultiTRP>1) TRPs participate in sensing without inter-TRP interference modeling, orthogonal time-frequency resources are required among TRPs. In this case, the total sensing signal resource overhead for NmultiTRP TRPs is:

OmultiTRP=O1TRP×Ntrp

It should be noted that the sensing RS adopted in the simulation reuses NR CSI-RS, meaning that the CSI-RS resources used for sensing can also be utilized for communication purposes, as Type 2 resource defined in Agreement-9. Therefore, the actual resource overhead dedicated to sensing could be further reduced.

Additionally, TR 22.137 defines the sensing result refresh period for Sensing Service Category 1 (Detection of UAV) as 1s [2]. Furthermore, considering the beam sweeping requirements illustrated in Figure 2, completing one UAV detection may require multiple CPIs. This implies that within each sensing refresh period, sensing signals only need to be transmitted for the duration required to meet CPI requirements, while no sensing signals are transmitted during the remaining time, as illustrated in Figure 6. Therefore, the effective overhead is reduced proportionally to the ratio of total CPI duration versus refresh period, and the actual resource overhead dedicated to sensing could be further reduced.

Figure 6: Illustration of sensing overhead calculation considering sensing refresh period.

The application of power boosting to sensing signal symbols impacts resource overhead calculation. When power boosting is enabled, the entire symbol resources are considered occupied for sensing purposes.

The resource overhead scales linearly with the number of participating TRPs when orthogonal resources are required.

Since sensing signals only need to be transmitted during CPI intervals within each refresh period, the effective overhead is reduced proportionally to the ratio of total CPI duration versus refresh period.

Reusing NR CSI-RS for sensing purposes enables shared resource utilization between communication and sensing functions, which significantly reduces the effective resource overhead dedicated to sensing operations.

Proponents should report the factors impacted to resource overhead calculation, such as power boosting, orthogonality of different TRP resources, sensing refresh period, and Type 2 resource.

Measurement Quantities

In the RAN1#122b meeting, several definitions of sensing measurement quantities were proposed, including a hierarchical framework spanning from Level 1 (raw data) to Level 6 (tracked object information) [5]. For the Rel-20 Study Item, the following levels were identified for potential down-selection: Level 3vs. Level 4, and Level 5 vs. Level 6, and sensing-specific SINR/RSRP measurements.

While Level 3 measurement quantities are preferred as the baseline, certain modifications are required to better align with sensing-specific requirements. The revised definition is proposed as follows:

- Option 1: Delay, Doppler, (micro-Doppler), angle, and power per detected path for a given time stamp

  - Definition: One path is associated with one delay, one Doppler, one or multiple micro-Doppler components, and one or multiple angles

- Option 2: Position, velocity, (micro-Doppler), and power per detected path for a given time stamp

  - Definition: One path is associated with one velocity (radial velocity), one or multiple micro-Doppler components, and one or multiple positions

And the technical justification for this definition is provided below. For sensing applications, target detection (e.g., via CFAR) is typically performed in the delay-Doppler domain. Therefore, detected paths should be defined within this domain, with each path associated with a specific delay and Doppler component. Consequently, the definition of path(s) in sensing requires extension, and measurements can be defined/reported based on path(s). Path(s) should be defined in the delay-Doppler domain, for instance, the i-th path in the delay-Doppler domain is characterized by <delay xi Doppler shift (or radial velocity) yi>, as shown in Figure 7.

Figure 7: Path(s) defined in delay-Doppler domain.

During angle estimation for detected paths, large antenna spacing may introduce angular ambiguity, and a single detected path may correspond to multiple targets with similar delay and Doppler characteristics, both potentially yielding multiple AoA and ZoA estimates. The angular ambiguity introduced by large antenna spacing for a given path is illustrated in Figure 8. Position coordinates can be derived from path delay and angle measurements, while radial velocity can be calculated from the Doppler component. When a path is associated with multiple angles, it consequently corresponds to multiple position coordinates.

Figure 8: Illustration of angular ambiguity introduced by large antenna spacing for a given path.

If each path is associated with one delay, one Doppler, and one angle (or equivalently, one velocity and one position), then path-based measurements (Level 3) become equivalent to point-based measurements (Level 4). However, if a path corresponds to multiple points, reporting point-based measurements would result in redundant reporting of delay/range or Doppler/velocity information.

Furthermore, in UAV scenarios, micro-Doppler components induced by blade rotations should be considered alongside translational Doppler shifts. Following CFAR detection in the delay-Doppler domain, micro-Doppler estimation can be performed for each detected path, potentially yielding one or multiple micro-Doppler components associated with given time stamps.

It is proposed to adopt the following modified Level 3 measurement quantity definition:

- Option 1: Delay, Doppler, (micro-Doppler), angle, and power per detected path for a given time stamp

  - Definition: One path is associated with one delay, one Doppler, one or multiple micro-Doppler components, and one or multiple angles

- Option 2: Position, velocity, (micro-Doppler), and power per detected path for a given time stamp

  - Definition: One path is associated with one velocity (radial velocity), one or multiple micro-Doppler components, and one or multiple positions

Evaluation Parameters and Initial Evaluation Results for UAV Sensing Scenario

For 5GA UAV sensing, the evaluation parameters listed in Table 2 are supported.

Table 2: Evaluation parameters for UAV sensing scenario.

| Parameters |  | Value |
| --- | --- | --- |
| Scenarios |  | UMa-AV[TR36.777] |
| Sensing mode |  | Mono-static |
| Carrier Frequency |  | 4.9GHz, 6GHz |
| Bandwidth |  | 100MHz |
| SCS |  | 30KHz |
| Sensing transmitters and receivers properties | STX/SRX Locations | STX/SRX locations are selected among the TRPs |
| Sensing target | LOS/NLOS | LOS and NLOS |
|  | Outdoor/indoor | Outdoor |
|  | 3D mobility | Horizontal velocity: Uniform distribution between 0 and 180km/h Vertical velocity: 0km/h |
|  | 3D distribution | N = 5 targets per sector in the center site. Horizontal plane: uniformly distributed in a sector Vertical plane: Uniformly distributed between 25m and 300m. |
|  | Orientation | Random in horizontal domain |
|  | Physical characteristics (e.g., size) | Size: 0.3m x 0.4m x 0.2m |
|  | RCS model | Single point model |
| Minimum 3D distances between pairs of STX/SRX and sensing target |  | 10m in TR36.777 |
| Minimum 3D distance between sensing targets |  | 10 m |
| BS TX power |  | 52dBm (antenna isolation=80dB), 37dBm (antenna isolation=65dB) |
| BS noise figure |  | 5dB |
| Residual leakage interference/noise |  | -94+5 dBm in 100 MHz |
| Wrapping Method |  | No wrapping method is used |
| Channel model |  | - Convolution method for NLOS and NLOS ray in target channel: One by one random coupling convolution with power normalization - The power threshold for path dropping after concatenation for target channel: -25dB |
| BS antenna configuration |  | Tx: (Mg, Ng, M, N, P) = (1,1,8,8,2;4,8) Rx: (Mg, Ng, M, N, P) = (1,1,8,8,2;4,8) dH,dV = (0.5, 0.8)λ |
| Analog beam set for both Tx and Rx |  | Azimuth angle φi = 0 Zenith angle θj = {pi/4, pi*7/16} |
| Sensing RS type and pattern |  | CSI-RS as baseline, frequency density = 3, period = 0.5 slot CPI (Coherent process interval) = 80ms, 160ms |
| Evaluation metric |  | Horizontal/vertical positioning accuracy, radial velocity accuracy, missed detection probability, false alarm probability (Type1 & Type2) |

In the simulation, we select the TRPs associated with the target dropped 3 sectors and 3 nearby TRPs as the sensing TRPs (totally 6 TRPs), as illustrated in Figure 1. The metrics of position and velocity are calculated by comparison between the fused sensing results and the ground truth. The detailed simulation results under different configurations are presented in Annex-1 of Section 8.

Table 3 and Table 4 summarize the comprehensive simulation results, including position accuracy, velocity accuracy, detection probabilities, and resource overhead for all 6 TRPs, for 4.9GHz and 6GHz respectively. Power boosting is applied to OFDM symbols carrying sensing signals for the 37dBm TX power configuration, while no power boosting is applied for the 52dBm configuration. Resource sharing between sensing and communication is not considered for overhead calculation. Furthermore, considering a sensing refresh period of 1s and the requirement for 2 CPIs (corresponding to 2 vertical beams) to complete one sensing instance in the current configuration, the actual sensing overhead is further reduced as reflected in Resource overhead (Option 2). This reduction accounts for the fact that sensing signals are only transmitted during active CPI intervals within each refresh period.

- Resource overhead (Option 1): sensing refresh period is not considered

- Resource overhead (Option 2): a sensing refresh period of 1 second and 2 CPIs (corresponding to 2 vertical beams) in 1 second are assumed

Table 3: Simulation results summary for 4.9GHz.

| Metrics | 52dBm，CPI=80ms | 52dBm，CPI=160ms | 37dBm，CPI=80ms | 37dBm，CPI=160ms |
| --- | --- | --- | --- | --- |
| Horizontal localization error [m] @90% | 1.31 | 1.08 | 1.9 | 1.84 |
| Vertical localization error [m] @90% | 0.86 | 0.59 | 1.42 | 0.99 |
| Radial velocity error [m/s] @90% | 0.18 | 0.09 | 0.18 | 0.09 |
| Missed detection probability | 4.33% | 3% | 16.67% | 6.33% |
| False alarm probability (Type1) | 0 | 0 | 0 | 0 |
| False alarm probability (Type2) | 0.35% | 0 | 0.4% | 0 |
| Resource overhead (Option 1) | 12.86% | 12.86% | 51.43% | 51.43% |
| Resource overhead (Option 2) | 2.06% | 4.11% | 8.23% | 16.46% |

Table 4: Simulation results summary for 6GHz.

| Metrics | 52dBm，CPI=80ms | 52dBm，CPI=160ms | 37dBm，CPI=80ms | 37dBm，CPI=160ms |
| --- | --- | --- | --- | --- |
| Horizontal localization error [m] @90% | 1.32 | 1.08 | 1.9 | 1.84 |
| Vertical localization error [m] @90% | 0.86 | 0.6 | 1.42 | 0.99 |
| Radial velocity error [m/s] @90% | 0.14 | 0.07 | 0.14 | 0.07 |
| Missed detection probability | 4.33% | 3% | 16.67% | 6.33% |
| False alarm probability (Type1) | 0 | 0 | 0 | 0 |
| False alarm probability (Type2) | 0.35% | 0 | 0.4% | 0 |
| Resource overhead (Option 1) | 12.86% | 12.86% | 51.43% | 51.43% |
| Resource overhead (Option 2) | 2.06% | 4.11% | 8.23% | 16.46% |

According to the evaluation results shown above, it can be observed that for configurations with 52dBm TX power, multi-TRP cooperative sensing achieves performance that meets the UAV detection requirements defined in TR 22.137. The total resource overhead for 6 participating TRPs remains at approximately 10%. When considering that sensing signals are only transmitted during active CPI intervals within each sensing refresh period, the effective resource overhead can be further reduced. For configurations with 37dBm TX power, meeting UAV detection requirements necessitates employing longer CPI durations and applying power boosting techniques. These compensatory measures result in significantly higher resource overhead compared to the 52dBm configurations.

Multi-TRP cooperative sensing with 52dBm Tx power meets TR 22.137 UAV detection requirements with approximately 10% resource overhead, while 37dBm configurations require longer CPI and power boosting with significantly higher overhead.

Conclusions

In this contribution, we have expressed our views on the evaluation metrics and methodology, signal configuration, measurement quantities, and evaluation parameters, and demonstrated our initial evaluation results. The observations and proposals are summarized as follows.

Observation 1: To achieve full coverage of the UAV airspace, an appropriate beam set configuration combined with an effective beam sweeping strategy is required.

Observation 2: The metric of sensing can be calculated based either solely on single-TRP monostatic sensing results or jointly from multiple TRP monostatic sensing results, where the former usually yields inferior performance and thus has relatively low requirements, while the latter usually offers superior performance and thus has relatively high requirements.

Observation 3: Micro-Doppler modeling improves detection probability of UAVs under low radial velocity conditions and facilitates identification of UAV-specific characteristics, enabling differentiation of UAV types and discrimination from other flying objects such as birds.

Observation 4: To address the unambiguous velocity requirement without prior knowledge of UAV position/velocity for ambiguity resolution, a sensing signal with an effective transmission period of 0.5 slot is required.

Observation 5: The application of power boosting to sensing signal symbols impacts resource overhead calculation. When power boosting is enabled, the entire symbol resources are considered occupied for sensing purposes.

Observation 6: The resource overhead scales linearly with the number of participating TRPs when orthogonal resources are required.

Observation 7: Since sensing signals only need to be transmitted during CPI intervals within each refresh period, the effective overhead is reduced proportionally to the ratio of total CPI duration versus refresh period.

Observation 8: Reusing NR CSI-RS for sensing purposes enables shared resource utilization between communication and sensing functions, which significantly reduces the effective resource overhead dedicated to sensing operations.

Observation 9: Multi-TRP cooperative sensing with 52dBm Tx power meets TR 22.137 UAV detection requirements with approximately 10% resource overhead, while 37dBm configurations require longer CPI and power boosting with significantly higher overhead.

Proposal 1: TRP selection based on sensing area should be supported to reduce resource overhead and computational complexity.

Proposal 2: Companies report which option is used in the simulation:

- Option1:  The sensing results are calculated based on single TRP's information.

- Option2: The sensing results are calculated based on fusion from multiple TRP’s sensing information, where each TRP performs monostatic sensing.

Proposal 3: Proponents should report the factors impacted to resource overhead calculation, such as power boosting, orthogonality of different TRP resources, sensing refresh period, and Type 2 resource.

Proposal 4: It is proposed to adopt the following modified Level 3 measurement quantity definition:

- Option 1: Delay, Doppler, (micro-Doppler), angle, and power per detected path for a given time stamp

  - Definition: One path is associated with one delay, one Doppler, one or multiple micro-Doppler components, and one or multiple angles

- Option 2: Position, velocity, (micro-Doppler), and power per detected path for a given time stamp

  - Definition: One path is associated with one velocity (radial velocity), one or multiple micro-Doppler components, and one or multiple positions

References

- RAN#108, New SID: Study on Integrated Sensing And Communication (ISAC) for NR, RP-251861.

- 3GPP TS22.137, “Service requirements for Integrated Sensing and Communication”, March, 2024.

- R1-2505767, Considerations on channel modeling and evaluation assumtions for 6GR air interface, BUPT, vivo.

- 3GPP RAN1 #122b, Chair notes RAN1#122b, Prague, Czech, October 13th – 17th, 2025.

- 3GPP RAN1 #122b, Summary #5 on evaluations for NR ISAC, R1-2507427.

Annex-1

4.9GHz with 52dBm TX Power

Figure 9 and Figure 10 present position and velocity accuracy results for the 80ms CPI configuration.

Figure 9: Horizontal/vertical positioning accuracy for 4.9GHz with 52dBm TX power and 80ms CPI.

Figure 10: Radial velocity accuracy for 4.9GHz with 52dBm TX power and 80ms CPI.

Figure 11 and 12 present position and velocity accuracy results for the 160ms CPI configuration.

Figure 11: Horizontal/vertical positioning accuracy for 4.9GHz with 52dBm TX power and 160ms CPI.

Figure 12: Radial velocity accuracy for 4.9GHz with 52dBm TX power and 160ms CPI.

6GHz with 52dBm TX Power

Figure 13 and Figure 14 present position and velocity accuracy results for the 80ms CPI configuration.

Figure 13: Horizontal/vertical positioning accuracy for 6GHz with 52dBm TX power and 80ms CPI.

Figure 7: Radial velocity accuracy for 6GHz with 52dBm TX power and 80ms CPI.

Figure 15 and Figure 16 present position and velocity accuracy results for the 160ms CPI configuration.

Figure 15: Horizontal/vertical positioning accuracy for 6GHz with 52dBm TX power and 160ms CPI.

Figure 16: Radial velocity accuracy for 6GHz with 52dBm TX power and 160ms CPI.

4.9GHz with 37dBm TX Power

Figure 17 and Figure 18 present position and velocity accuracy results for the 80ms CPI configuration.

Figure 17: Horizontal/vertical positioning accuracy for 4.9GHz with 37dBm TX power and 80ms CPI.

Figure 18: Radial velocity accuracy for 4.9GHz with 37dBm TX power and 80ms CPI.

Figure 19 and Figure 20 present position and velocity accuracy results for the 160ms CPI configuration.

Figure 8: Horizontal/vertical positioning accuracy for 4.9GHz with 37dBm TX power and 160ms CPI.

Figure 20: Radial velocity accuracy for 4.9GHz with 37dBm TX power and 160ms CPI.

6GHz with 37dBm TX Power

Figure 21 and Figure 22 present position and velocity accuracy results for the 80ms CPI configuration.

Figure 21: Horizontal/vertical positioning accuracy for 6GHz with 37dBm TX power and 80ms CPI.

Figure 22: Radial velocity accuracy for 6GHz with 37dBm TX power and 80ms CPI.

Figure 23 and Figure 24 present position and velocity accuracy results for the 160ms CPI configuration.

Figure 23: Horizontal/vertical positioning accuracy for 6GHz with 37dBm TX power and 160ms CPI.

Figure 24: Radial velocity accuracy for 6GHz with 37dBm TX power and 160ms CPI.