3GPP TSG RAN WG1 #123                                              R1-2508458

Dallas, USA, Nov 17th – 21st, 2025

Source: 	CMCC

Title:	Discussion on modulation schemes for 6GR interface

Agenda item:	11.4.2

Document for:	Discussion & Decision

- Introduction

During the 3GPP RAN#108 meeting, the SID on the 6G SI “Study on 6G Radio” was approved, including the following objectives on physical layer design for 6G radio (6GR) [1]:

… (2) Physical Layer structure for 6GR, - Waveforms (OFDM-based) and modulations. 5G NR Waveforms and modulation should be considered for 6GR and is also the benchmark for other potential proposals. [RAN1, RAN4] - Frame structure, including compatibility with 5G NR to allow for efficient 5G-6G Multi-RAT Spectrum Sharing (MRSS). [RAN1] - Channel coding, using LDPC and Polar Code as baseline, considering applicable extensions to satisfy 6G requirements and characteristics with acceptable performance/complexity trade-off [RAN1] - Channel Bandwidth (at least minimum and maximum), Numerology, avoiding multiple numerologies for the same band / sub-range (e.g., enabling synergies among frequency bands in the ~7GHz range) [RAN1, RAN4] - Physical layer control, data scheduling and HARQ operation [RAN1, RAN2] - MIMO operation [RAN1, RAN4] - Duplexing [RAN1, RAN4] - Initial access [RAN1, RAN2, RAN4] - Studies on synchronization signal and raster, broadcast signals/channel and physical random access channel [RAN1, RAN4] - Studies on initial access procedure, random access procedures, system information and paging [RAN2, RAN1, RAN4] - 6GR spectrum utilization and aggregation. [RAN1, RAN2, RAN4] - Other physical layer signals, channels and procedures [RAN1, RAN2, RAN4] - Evaluate performance of at least energy efficiency, spectrum efficiency, and coverage compared to 5G NR, and deliver the initial result at the end of study [RAN1]. - RAN4 can be involved, if necessary, based on the LS from RAN1 …

In the RAN1#122b meeting, some agreements have been reached on modulation schemes for 6GR interface. In this contribution, some considerations on modulation schemes for 6GR are provided based on RAN1#122b agreements.

- Considerations on modulation schemes for 6GR

2.1 High-order QAM modulation

In the RAN1#122b meeting, the following agreement was reached regarding the uniform high-order QAM scheme for 6GR:

Agreement For the study of uniform 4096QAM for DL and uniform 1024QAM for UL, need to study performance (assuming realistic channel estimation, time/freq synchronization assumption, phase noise assumption, etc), complexity/power consumption, requirements, benefit/necessity under applicable scenarios, associated restrictions, and challenges (such as EVM requirement, PAPR increase, MPR or A-MPR increase under realistic PA model). - FFS: How to involve RAN4 early - FFS: Shaping of higher order modulation - System level evaluation can be done after link level evaluation.

During the RAN1#122 meeting, it was agreed to adopt up to 1024QAM in DL and 256QAM in UL as the baseline for further study. For higher-order QAM such as uniform 4096QAM for DL and uniform 1024QAM for UL, more stringent EVM requirements are imposed on both base stations and user equipment. This poses a significant implementation challenge, as the core hardware components (such as digital-to-analog converters (DACs), power amplifiers (PAs), and oscillators) face inherent physical limitations and cost barriers. Moreover, higher-order QAM signals exhibit a higher Peak-to-Average Power Ratio (PAPR), which requires power amplifiers to operate with a larger linear dynamic range to prevent distortion. This, in turn, exacerbates the impact on MPR/A-MPR. To comply with emission standards under non-ideal PA models, significant power back-off is needed, which can paradoxically erode the data rate gains expected from the more advanced modulation scheme.

Observation 1: For higher-order QAM, such as uniform 4096QAM for DL and uniform 1024QAM for UL, the required power back-off undermines the data rate advantage, and the extreme hardware demands make implementation impractical.

2.2 Shaping modulation

Compared with high-order uniform QAM, shaping modulation is generally more hardware-friendly, as it significantly reduces the PAPR by either utilizing low-power symbols more frequently (Probabilistic Shaping) or directly optimizing the constellation geometry (Geometric Shaping). A lower PAPR, in turn, relaxes the stringent linearity requirements.

- Geometric Shaping

Geometric shaping uses a non-uniform constellation with equiprobable symbols [2]. For the additive white Gaussian noise (AWGN) channel, the goal of geometric shaping is to maximize the achievable information rate for a given signal-to-noise ratio (SNR) by designing a constellation shape that better matches the Gaussian capacity-achieving distribution. It means that, the constellation design may vary depending on the channel characteristics and SNR conditions. While this is true in theory, a key practical challenge is designing a constellation that remains universal and robust across varying channel conditions without sacrificing low complexity.

Observation 2: For geometric shaping, the constellation design should ensure universal performance across diverse channel conditions.

The key difference between legacy 2m-QAM modulation and geometric shaping lies in the predefined constellation map. Once this constellation is defined, the procedures for symbol mapping at the transmitter and symbol demapping at the receiver follow the same fundamental steps as in legacy 2m-QAM modulation.

Observation 3: Geometric shaping can reuse the 5G procedures for symbol mapping at the transmitter and symbol demapping at the receiver.

Although geometric shaping avoids major changes to the transceiver chain, it shifts complexity to the constellation map itself. For geometrically-shaped constellations, the mapping from input bits to non-uniform symbols typically cannot be described by a simple, closed-form formula. This is because the constellation is a product of numerical optimization, which aims to approximate a continuous, non-uniform probability distribution (e.g., Gaussian). Thus, the constellation map is irregular 2D point arrangements with no simple algebraic relationship. Consequently, the constellation map typically relies on a pre-defined lookup table.

- Transmitter Complexity: The transmitter must store the constellation map table, and perform a table lookup for every symbol to be transmitted. The table size grows linearly with the modulation order, directly increasing memory and access overhead. This linear growth presents a major bottleneck for practical implementations of 1024-QAM and higher-order schemes.

- Receiver Complexity: While the demapping procedure is similar to that of legacy QAM, the non-uniform spacing between constellation points complicates the calculation of log-likelihood ratios (LLRs). The receiver cannot rely on simple, minimum-distance calculations to a regular grid, and may instead compute distances to all relevant non-uniform neighbors, significantly increasing the computational load per received symbol.

Observation 4: For geometrically-shaped constellations, if the constellation map relies on a pre-defined lookup table, the memory and access overhead at the transmitter and the demapping complexity at the receiver should be considered, especially for 1024 and higher-order schemes.

- Probabilistic Shaping

In probabilistic shaping, the constellation is on a uniform grid with differing probabilities per constellation point [3]. Unlike legacy 2m-QAM where all symbols are equally likely, probabilistic shaping intentionally uses some high-energy symbols less frequently and some low-energy symbols more often, in order to shape the transmitted signal’s distribution to better match the Gaussian characteristic that achieves channel capacity. The probabilistic shaping requires a distribution matcher (DM) to map uniform input data bits into amplitude bits that have the desired non-uniform probability distribution. However, these amplitude bits alone cannot be directly mapped to a legacy QAM symbol, as they only carry amplitude-related information and lack the phase or sign (positive/negative) information required to fully define a constellation point. To form a complete symbol, additional bits (called sign bits) are required to determine the phase quadrant. In the most common probabilistic shaping architecture, Probabilistic Amplitude Shaping (PAS), these sign bits are typically derived from the parity bits generated by the FEC encoder. Therefore, introducing probabilistic shaping involves far more than simply adding a DM module. It requires a redesign of the channel coding and modulation chain. This enables the joint optimization of both shaping gain and coding gain, allowing the system to operate closer to the Shannon limit.

Observation 5: Probabilistic shaping can reuse the 5G constellation map.

Observation 6: Probabilistic shaping requires a redesign of the channel coding and modulation chain.

From our understanding, the bit-to-symbol mapping rule in probabilistic shaping remains identical to that of legacy QAM. Taking PAS as an example: although the DM and the FEC encoder work together to generate non-uniformly distributed amplitude bits and derive the sign bits, the complete set of bits (amplitude + sign) is ultimately fed into a standard constellation mapper. The logic by which this mapper converts any given bit sequence into a specific I/Q symbol is unchanged. Since the transceiver's symbol mapping/demapping interfaces are preserved, the fundamental change is the coding subsystem that prepares the bits. Therefore, we propose that design and performance simulation of probabilistic shaping should be included in the channel coding agenda 11.4.1.

Observation 7: The core innovation of probabilistic shaping is the novel bit generation process, while the final step of mapping bits to a constellation point remains identical to legacy QAM.

2.3 MCS adaptation

In the RAN1#122b meeting, the following agreement was reached:

Agreement For 6GR constellation shaping study, proponent is encouraged to provide details for the PS/GS schemes considered for evaluation and comparison, including at least the following - Probabilistic shaping for CP-OFDM and DFT-s-OFDM - Use the list of spectrum efficiencies in NR MCS table as starting point, and provide constellation (including normalization), coding rate and target probabilistic distribution for each SE - If multiple coding rate and target probabilistic distribution pairs are provided for each SE, how to switch between them - Relationship between shaping and FEC, coded bits to modulation symbol mapping, and other modules (such as scrambling, interleaving), in transmit and receive chains. How to handle HARQ retransmission - PS algorithm details (for example, source coding based, channel coding based, etc) and parameters (such as block length, rate loss) - Geometric shaping for CP-OFDM and DFT-s-OFDM - Use the list of spectrum efficiencies in NR MCS table as starting point, and provide target constellation shape (including normalization) (1D-NUC, 2D-NUC, QAM-CS, etc) for each SE - If multiple constellation shapes are provided for each SE, how to switch between them - GS mapping details, such as bit to constellation point mapping and shape - Relationship with other blocks (such as scrambling, interleaving). How to handle HARQ retransmission

Due to variations in channel conditions, the shaping modulation designed for a specific channel model may not be applicable to other channel models under a given spectral efficiency (SE). Therefore, adaptive Modulation and Coding Scheme (MCS) selection for each SE can be considered, and this approach is likely to impact the design of the MCS table.

Proposal 1: Adaptive MCS selection for each SE can be considered.

- Conclusions

In this contribution, we provide some considerations of modulation schemes for 6GR. The following observations and proposals are made.

Observation 1: For higher-order QAM, such as uniform 4096QAM for DL and uniform 1024QAM for UL, the required power back-off undermines the data rate advantage, and the extreme hardware demands make implementation impractical.

Observation 2: For geometric shaping, the constellation design should ensure universal performance across diverse channel conditions.

Observation 3: Geometric shaping can reuse the 5G procedures for symbol mapping at the transmitter and symbol demapping at the receiver.

Observation 4: For geometrically-shaped constellations, if the constellation map relies on a pre-defined lookup table, the memory and access overhead at the transmitter and the demapping complexity at the receiver should be considered, especially for 1024 and higher-order schemes.

Observation 5: Probabilistic shaping can reuse the 5G constellation map.

Observation 6: Probabilistic shaping requires a redesign of the channel coding and modulation chain.

Observation 7: The core innovation of probabilistic shaping is the novel bit generation process, while the final step of mapping bits to a constellation point remains identical to legacy QAM.

Proposal 1: Adaptive MCS selection for each SE can be considered.

- References

- RP-251881, New SID: Study on 6G Radio, RAN#108, Prague, Czech Republic, June 9-13, 2025.

- Feng-Wen Sun and H. C. A. van Tilborg, “Approaching capacity by equiprobable signaling on the Gaussian channel,” in IEEE Transactions on Information Theory, vol. 39, no. 5, pp. 1714-1716, Sept. 1993.

- F. R. Kschischang and S. Pasupathy, “Optimal nonuniform signaling for Gaussian channels,” in IEEE Transactions on Information Theory, vol. 39, no. 3, pp. 913-929, May 1993.