3GPP TSG RAN WG1 #123		R1-2508708

Dallas, USA, Nov. 17th-21st, 2025

Source:	OPPO

Title:	Maintenance on low-band CA via switching

Agenda Item:	8.8

Document for:	Discussion and Decision

Introduction

In this contribution, we provide maintenance discussion and the related TP for the following maintenance issues for Rel-19 LBCA.

- It is unclear whether the collision handling in Clause 24 of TS38.213 is applied per single repetition of a channel or per all repetitions of a channel.

Discussion

Issue #1: lack of clarity whether the collision handling in Clause 24 of TS38.213 is applied per single repetition of a channel or per all repetitions of a channel.

Reasons for change: The current Clause 24 of TS38.213 lists a number of collision handling behaviors for downlink/uplink channels that are semi-statically configured in a duration available for transmission/reception in FDD cell or SDL cell. Take the following as an example,

The UE does not transmit a PUSCH, or PUCCH, or SRS, or PRACH configured by higher layers in a set of symbols when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’, assuming TTA=0.

However, it is unclear whether “a PUSCH” here refers to a PUSCH channel including all its repetitions or just a repetition of a PUSCH channel. On one hand, it is non-harmful and even beneficial if the collision handling is performed per single repetition of a channel. On the other hand, the existing TS38.213 already established some precedents that, if a PUSCH is configured with repetition, the wording “a PUSCH transmission” refers to “repetitions of the PUSCH transmission”. One of such precedents is the collision handling on PUSCH repetition Type B given as following for an example.

“If a UE transmits a PUSCH with repetition Type B and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots, the UE expects all actual repetitions of the PUSCH transmission [6, TS 38.214] that would overlap with the PUCCH transmission to fulfill the conditions in clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, and the UE multiplexes the HARQ-ACK and/or CSI information in the earliest actual PUSCH repetition of the PUSCH transmission that would overlap with the PUCCH transmission and includes more than one symbol. The UE does not expect that all actual repetitions that would overlap with the PUCCH transmission do not include more than one symbol”

So, it is necessary to clarify that collision handling in LBCA is per repetition instead of per channel (all repetitions) to maintain the channel efficiency.

Summary of changes: Add clarification in Clause 24 for the case where a channel or signal is configured with repetitions.

Consequences if not approved: Some repetitions of a channel/signal that are allocated in the cell-available duration are unnecessarily banned from transmission/reception.

Proposal 1: The collision handling for semi-statically configured channel in LBCA is per repetition.

Proposal 2: Adopt the following TP for Clause 24 in TS38.213.

24 Downlink carrier aggregation via switching ************** Unchanged parts omitted************** When the UE is provided LBCA-SwitchingPattern, the UE is provided - a first duration, by LBCA-SwitchingGap-Duration-PCelltoSCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘0’, for the UE to switch operation from the PCell to the SCell over the first duration, and - a second duration, by LBCA-SwitchingGap-SCelltoPCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘1’, for the UE to switch operation from the SCell to the PCell over the second duration The following applies to a channel or signal if the channel or signal is not configured with repetitions, and applies to each repetition of a channel or signal otherwise. The UE does not transmit a PUSCH, or PUCCH, or SRS, or PRACH configured by higher layers in a set of symbols when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’, assuming TTA=0. The UE does not receive a PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers in a set of symbols on the PCell when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’. The UE does not receive a PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers in a set of symbols on the SCell when the set of symbols includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’. The UE does not expect to detect a DCI format scheduling or triggering a PUSCH, or PUCCH, or SRS, or PRACH transmission in a set of symbols when the set of symbols includes at least one symbol that is in the first duration, or is in a slot with bit value ‘1’ and is not in the second duration. The UE does not expect to detect a DCI format scheduling or triggering a PDSCH, or PDCCH, or CSI-RS, or PRS reception in a set of symbols on the PCell when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value of ‘1’. The UE does not expect to detect a DCI format scheduling or triggering a PDSCH, or PDCCH, or CSI-RS, or PRS reception in a set of symbols on the SCell when the set of symbols includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’.

Conclusions

This contribution is concluded with the following proposals:

Proposal 1: The collision handling for semi-statically configured channel in LBCA is per repetition.

Proposal 2: Adopt the following TP for Clause 24 in TS38.213.

24 Downlink carrier aggregation via switching ************** Unchanged parts omitted************** When the UE is provided LBCA-SwitchingPattern, the UE is provided - a first duration, by LBCA-SwitchingGap-Duration-PCelltoSCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘0’, for the UE to switch operation from the PCell to the SCell over the first duration, and - a second duration, by LBCA-SwitchingGap-SCelltoPCell, ending at the end of the last slot from a number of consecutive slots with bit value ‘1’, for the UE to switch operation from the SCell to the PCell over the second duration The following applies to a channel or signal if the channel or signal is not configured with repetitions, and applies to each repetition of a channel or signal otherwise. The UE does not transmit a PUSCH, or PUCCH, or SRS, or PRACH configured by higher layers in a set of symbols when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’, assuming TTA=0. The UE does not receive a PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers in a set of symbols on the PCell when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value ‘1’. The UE does not receive a PDSCH, or PDCCH, or CSI-RS, or PRS configured by higher layers in a set of symbols on the SCell when the set of symbols includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’. The UE does not expect to detect a DCI format scheduling or triggering a PUSCH, or PUCCH, or SRS, or PRACH transmission in a set of symbols when the set of symbols includes at least one symbol that is in the first duration, or is in a slot with bit value ‘1’ and is not in the second duration. The UE does not expect to detect a DCI format scheduling or triggering a PDSCH, or PDCCH, or CSI-RS, or PRS reception in a set of symbols on the PCell when the set of symbols includes at least one symbol that is in the first duration or is in a slot with bit value of ‘1’. The UE does not expect to detect a DCI format scheduling or triggering a PDSCH, or PDCCH, or CSI-RS, or PRS reception in a set of symbols on the SCell when the set of symbols includes at least one symbol that is in the second duration or is in a slot with bit value ‘0’.

Reference

- RAN1 chairman notes in RAN1 #122 meeting

- RP-250247 Revised WID on Low NR band carrier aggregation via switching