3GPP TSG RAN WG1 #123   	                              R1-2508750

Dallas, USA, Nov 17th – 21st, 2025

Agenda item:		10.1.1.2

Source:		LG Electronics

Title:			Discussion on other aspects for CSI compression

Document for:	Discussion and Decision

Introduction

In RAN#108 meeting, the WID on “Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface enhancements” [1] was approved. The following objectives related to AI/ML-based CSI compression were included in the WID.

CSI feedback enhancement, encompassing (two-sided model) [RAN1/2]: - CSI spatial/frequency compression without temporal aspects (“Case 0”), - Specify necessary signalling/mechanism(s) as per the identified potential specification impacts in FS_NR_AIML_Air , including - Model pairing procedure including ID and applicability reporting - Inference aspects including target CSI type, measurement and report configuration, CQI RI determination, payload determination, quantization configuration codebook, UCI mapping, CSI processing criteria and timeline, priority rules for CSI reports - NW and UE side data collection for training, - Target CSI format - Note The framework defined in BM and CSI prediction use cases could be reused - Specify performance monitoring

In this contribution, we provide our view on other aspects including model pairing, performance monitoring, and data collection.

Discussion

Model pairing

In AI/ML-based CSI compression (two-sided model), the model pairing procedure is required to ensure compatibility between the UE-side CSI generation model(s) and the NW-side CSI reconstruction model(s). Compatibility depends on factors such as the dataset on which the models were trained, the model architecture/parameters, etc. Direction A of inter-vendor collaboration allows flexible model structures/parameters, whereas Direction C is based on a fixed reference model. Therefore, Direction A requires more detailed pairing information to ensure compatibility. During the study phase, the following agreements were made regarding the pairing related ID for directions A and C, respectively.

Agreement In CSI compression using two-sided model use case, feasibility and procedure to align the information that enables the UE to select a CSI generation model(s) compatible with the CSI reconstruction model(s) used by the gNB is studied. At least the following options have been proposed by companies to define the pairing information used to enable the UE to select a CSI generation model(s) that is compatible with the CSI reconstruction model(s) used by the gNB: - Option 1: The pairing information is in the forms of the CSI reconstruction model ID that NW will use. - Option 2: The pairing information is in the forms of the CSI generation model ID that the UE will use. - Option 3: The pairing information is in the forms of the paired CSI generation model and CSI reconstruction model ID. - Option 4: The pairing information is in the forms of by the dataset ID during type 3 sequential training. - Option 5: The pairing information is in the forms of a training session ID to a prior training session (e.g., API) between NW and UE. - Option 6: The pairing information is up to UE/NW offline co-engineering alignment, transparent to 3GPP specification. - Note: the disclosure of the vendor information during the model pairing procedure and model identification procedure should be considered. - Note: If each UE side model is compatible with all NW side model, the information is not needed for the UE. - Note: Above does not imply there is a need for a central entity for defining/storing/maintaining the IDs. Agreement In Options 3a-1 and 4-1, the exchanged dataset or the model parameters can be associated with an ID for pairing related discussion, then - The same ID can be used for UE to collect UE-side target CSI for UE-side training - The same ID can be used for applicability inquiry and reporting - The same ID can be used for inference configuration - The same ID can be used for NW-side data collection - FFS: whether ID/even same ID is needed for monitoring configuration - FFS: where the ID is assigned - Note: whether the purpose for pair will be specified will be discussed separately. Agreement In Direction C, the fully standardized reference model is associated with an ID for pairing related discussion, then - The same ID can be used for UE to collect UE-side target CSI for UE-side training - The same ID can be used for applicability inquiry and reporting - The same ID can be used for inference configuration - The same ID can be used for NW-side data collection - FFS: whether ID/even same ID is needed for monitoring configuration - FFS: where the ID is assigned or how the ID is specified - Note: whether the purpose for pair will be specified will be discussed separately.

Based on the above agreements of study phase, a pairing related ID (i.e., pairing ID) can be associated with the exchanged dataset (Sub-option 4-1, Direction A), the model parameter (Sub-option 3a-1, Direction A), and the standardized reference model (Direction C). The same ID can be used for UE/NW-side data collection, applicability inquiry and reporting, and inference configuration. Related to usages of pairing ID, there was controversy in the previous meeting on whether all potential usages agreed in Rel-19 study phase can be supported in Rel-20 work item. The most controversial part is whether pairing ID can be used for UE-side data collection and NW-side data collection. It is known that a model trained under specific conditions can have better performance than a generalized model in those conditions. For training with specific conditions, data categorization may be needed for the training entity. For this reason, UE may receive the NW-side additional conditions (e.g., scenario, cell ID) during UE-side data collection. For NW-side data collection, NW may receive UE-side additional conditions (e.g., UE mobility) and/or NW-side additional condition related to the collected data, since these conditions can change during data collection. For this reason, some ID (e.g., pairing ID) may be used to indicate UE-side/NW-side (additional) conditions. However, in our view, if certain ID(s) (e.g., pairing ID) is used for data categorization during data collection, further discussion seems to be needed on which NW-side additional condition and/or UE-side additional condition can be inferred from that ID.

Agreement @ RAN1#122 For model pairing procedure for inference, consider the applicability report procedure of Rel-19 AI/ML beam management for UE side model as a starting point. - Pairing ID(s) (i.e., ID for pairing related discussion) is indicated (e.g., in CSI-reportConfig) in Step 3. - FFS whether/how to link the Pairing ID with the inter-vendor collaboration - FFS whether/how Paring ID(s) are reported in Step 4 - FFS whether to support both Option A (CSI-ReportConfig for inference configuration) and Option B (sets of inference related parameters for applicability report only). - FFS CSI compression specific inference configuration or inference related parameters.

In RAN1 #122 meeting, for model pairing procedure, it was agreed to consider the applicability report procedure as a starting point. FFS point is whether to adopt Option A only or both Option A and Option B. In our view, considering both option A and B can provide greater flexibility in inference configuration compared to adopting Option A only. In addition, Option B can allow the NW to reduce the signaling overhead of inference configurations. Regarding the potential inference related information for Option B, at least quantization related information (e.g., quantization parameter combination {D, Q, L}, VQ/SQ, layer/rank specific/common) and scalability related information (e.g., sub-bands, TX ports, payload sizes) can be considered.

For applicability report of CSI compression, consider both option A and option B.

Regarding the CSI compression specific inference related parameters for Option B, quantization related information (e.g., quantization parameter combination {D, Q, L}, VQ/SQ, layer/rank specific/common) and scalability related information (e.g., sub-bands, TX ports, payload sizes) can be considered

It is noted that LCM framework for two-side model will be a reference of future two-sided model case in 6GR. Therefore, this framework should be designed to be both flexible and future proof. In this context, it is necessary to specify the pairing ID in a clear and future-proof manner. So, the following consideration points need to be discussed: i) functionalities of the pairing ID ii) information included in the ID, iii) the format of the ID, iv) uniqueness of ID and v) whether the pairing ID can be common or separated for sub options of inter-vendor collaboration.

Performance monitoring

In RAN1#117 meeting, the following was agreed regarding monitoring options. Also, some observation and conclusions were made in RAN1#120bis meeting. In this section, we discuss and compare the various monitoring options in terms of monitoring accuracy, signaling overhead, UE complexity/capability, and latency. To reduce normative workload, it is recommended focus on some popular options.

Agreement Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook Regarding monitoring metrics: - Monitoring accuracy also includes generalization considerations, if applicable. - Complexity also includes LCM complexity, if applicable. - Monitoring overhead, latency, complexity, and accuracy analysis may have to consider using at N>1 CSI feedback occasions. - Testability of UE reported metrics Discussion may include the following aspects: - Consideration of Options 1-5 and their sub-options for alleviating / resolving the issues related to inter-vendor training collaboration - Temporal domain aspects of CSI compression - How the above monitoring approaches or combination of them may help identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches. Note: implementation-based monitoring solutions can be considered in assessing the necessity of the above monitoring approaches. Conclusion @RAN1#120bis For NW-side monitoring with target CSI reporting - Target CSI reporting via legacy CSI codebooks can be used for NW-side monitoring - Target CSI reporting with CSI codebook enhancement via higher-resolution parameter combination may be beneficial for improving NW-side monitoring with additional cost of complexity and overhead at UE side. Conclusion @RAN1#120bis UE-side monitoring is feasible. Observation @RAN1#120bis Some companies think that, at least in some UE-side monitoring options, NW-side monitoring with target CSI reporting is needed to check the reliability of UE-side monitoring reports.

NW-side monitoring

As captured above, two options of NW-side monitoring are under consideration: target CSI reporting-based and SRS-based. In both options, the NW compares the inference result generated by the CSI reconstruction model with the corresponding ground-truth CSI, and then NW calculated the monitoring metric and decide its LCM action (e.g., fallback, switching, deactivation, etc.).

During the study, a majority of companies prefer target CSI reporting for NW-side performance monitoring. Target CSI reporting can identify root causes (e.g., model issues, dataset shifts) of performance degradation and can also be leveraged for NW-side data collection. However, the associated signaling overhead remains a concern. Therefore, methods to reduce signaling overhead should be considered when adopting target-CSI reporting for performance monitoring. For a simple solution, legacy CSI reporting can be reused. In fact, several evaluations [2,3] presented in previous meetings indicated that the legacy eType-II codebook via L1 signaling is sufficient for NW-side performance monitoring.

Another issue can be whether dedicated report configuration is needed for target CSI reporting for NW-side monitoring. For resource configuration, since inference measurement resource and monitoring resource can be the same, configuring dedicated resource for monitoring report seems not be needed. For NW-side monitoring, NW also needs inference report. If dedicated monitoring report is supported, there is case that one of reports can be dropped due to collision. In that case, NW cannot calculate performance metric/output. Thus, it would be good to consider reporting both results of inference and monitoring together when monitoring report occasion is occurred.

For NW-side monitoring, support target CSI reporting via L1 signaling with paired Target CSI and CSI feedback.

Meanwhile, some companies prefer SRS-based monitoring approach that may reduce signaling overhead as target CSI can be obtained by SRS measurement. But it may result in an additional latency issue in case that UE performs antenna switching (i.e., if the number of Tx antennas is less than the number of Rx antennas within the UE). In addition, in FDD, full channel reciprocity cannot be achieved because the UL band and DL band are different and timing mismatch between CSI-RS and SRS, so additional compensation at NW side or some specification support may be required. Even though the channel reciprocity is somewhat valid, monitoring accuracy may highly depend on the channel estimation performance, which, in turn, counts upon the level of the channel reciprocity validity. Based on above discussion, following is proposed.

UE-side monitoring

UE-side monitoring options can be grouped into two categories: model-based and reconstructed CSI delivery. Both categories can mitigate the uplink resource overhead issues inherent in NW-side monitoring because the size of reporting contents (e.g., monitoring outcome or metric) of these options is very small compared to that of NW-side monitoring. However, model-based options require additional models on top of the UE’s CSI generation model, thereby imposing extra storage and computational burdens on the UE. Because the models added for monitoring are also AI-based, complementary non-AI monitoring methods may also be needed to validate these models. Moreover, these monitoring models are typically smaller than the actual CSI reconstruction model (actual CSI reconstruction model transfer is excluded in this context due to inter-vendor training collaboration discussions), their generalization capability may be limited. This means that these models may not function properly on datasets that differ from the training datasets. For example, if the UE’s proxy or intermediate/eventual KPI models encounter channel conditions outside of the distribution of their training dataset, they may fail to mimic the actual reconstruction model or estimate intermediate/eventual KPIs accurately. In addition, eventual KPI-based monitoring can be influenced not only by CSI feedback but also by various factors such as scheduling strategy and environmental interference.

In the reconstructed CSI delivery options, the NW sends the inference results generated by its CSI generation model to the UE, via precoded RS or codebook, and the UE compares these with the corresponding ground-truth CSI. The resulting metrics or monitoring decision-related information are then fed back to the NW. Because the monitoring-related information traverses the air interface three times before the NW can perform monitoring-related operation (e.g., fallback/switching/(de)activation), the overall latency can become significant.

Precoded RS-based monitoring can reduce the signaling overhead compared with codebook-based monitoring. However, as with eventual KPI-based monitoring, factors unrelated to the CSI compression model, such as the channel conditions which the RS is transmitted, can be included in the monitoring results. For example, if channel aging occurs, monitoring performance inevitably degrades because the channel assumed in the reconstructed CSI no longer matches the current downlink channel.

For the option of indicating reconstructed CSI via codebook, although it may reduce UE complexity for calculating intermediate KPI, there are several challenges. First challenge point is latency issue discussed in above. The second issue is signaling overhead to indicate reconstructed CSI via codebook, where 100 ~ 1000bits for indication is required if Rel-16 Type-II codebook is considered. Lastly, indication mechanism for reconstructed CSI, e.g., via DCI, MAC-CE, needs to be discussed.

Deprioritize UE-side performance monitoring.

Data collection

In RAN1#122 meeting, the following was agreed regarding data collection for model training.

Agreement For NW side data collection of CSI compression, discuss the content and format of the collected data by assuming higher layer based signalling. - Details on signaling is up to RAN2. Agreement To enable UE side data collection, CSI-ReportConfig can be used for configuring the resources for data collection purpose without CSI report - P-CSI-RS and /SP CSI-RS are supported Agreement For NW side data collection, study the solution for quantizing Target CSI, from the aspects of overhead, quantization loss/performance, and complexity. E.g.,: - Option 0: Reusing legacy e-Type II codebook - Option 1: Scalar quantization to the Target CSI. FFS number of bits for real/imaginary, whether coefficients for quantization are obtained from transformation of Target CSI. - Option 2: Enhanced methods to reduce the SVD/EVD decomposition complexity, reduce frequency/spatial/beam domain basis searching, with potentially relaxed overhead, e.g., Type II like codebook with scalar quantized W2 similar to Option 1 scalar quantization. - Option 3: Enhanced method for improved overhead saving, e.g., reporting the dominant eigenvalues/eigenvectors of per layer precoding matrix. - Option 4: eType II like codebook with new parameter values (e.g. larger L, pv, beta, amplitude, phase) - Note: the quantizaton format should consider the maximum payload size per reporting (e.g., 9KBytes) of the current higher layer signaling. - Note: At least take precoding matrix as Target CSI type. FFS channel matrix. - FFS: EVM, metric, benchmark (e.g., FP32, eT2 with PC6/8), by reusing R18 study EVM as baseline.

Based on the above agreements, CSI-ReportConfig without CSI report can be used for UE-side data collection. For NW-side data collection, it was agreed to discuss the content and format of the collected data by assuming higher layer based signaling, and several target CSI format options were identified for further discussion.

Target CSI format

In RAN1#122bis meeting, the details for further study of Target CSI format Option 1 and Option 4 and evaluation assumptions were agreed.

Agreement: For NW side data collection with higher layer reporting, for further study of Option 1 (scalar quantization), consider the following definition for evaluation: - Content: for a number of S=1 (as baseline) Target CSI samples each with the dimensions of 2N1N2 ports, N3 subbands, v layers for precoding matrix, and real+imaginary (or amplitude+phase) parts, which has overall 2N1N2*N3*v complex elements, and any complex element has a real (or amplitude) part of x1 and an imaginary (or phase) part of x2. - Note: Companies to report whether real value and imaginary value are quantized, or amplitude value and phase value are quantized. - Format: x1 is quantized to k1 bits within range of [T1min, T1max], and x2 is quantized to k2 bits within range of [T2min, T2max]. E.g., for Scalar8, k1=8 bits for quantizing the real part, and k2=8 bits for quantizing the imaginary part. Uniform quantization is assumed. - Note: Companies to report whether/how the normalization is performed, and whether it is normalized within or across 2N1N2 ports/ N3 subbands/ v layers/ real and imaginary (or amplitude and phase) parts. - Fixed parameter values of k1/k2/T1min/T1max/T2min/T2max across complex elements, as a baseline for evaluations. Companies to report the values. - Note: It is up to company to use number of samples S>1 in their evaluations. In this case, companies need to provide information how S samples are quantized in their evaluation. Agreement: For NW side data collection with higher layer reporting, regarding Option 4 (eType II like codebook with new parameter), further study the following candidate enhanced parameter values with potential down selection. - L= 4/6/12/16/(N1*N2), pv=0.6/0.75/1, Beta = 0.5/0.8/1. Reference amplitude = 4bits/6bits, differential amplitude = 3bits/4bits, and phase = 4bits/6bits - Company to report the quantization step for reference amplitude for 6bits. - No restriction to the total number of non-zero coefficients for a specific rank. - Other combinations can be considered and reported by companies. Agreement: For the study of NW side data collection with higher layer reporting, following evaluation assumptions are considered: - Benchmark#1: FP32 (k1=k2=32 bits) - Benchmark#2: legacy eType II with PC6 (for layer 1/2/3/4), PC8(for layer 1/2) - CSI payload size: at least consider large CSI payload Z ≥ 230 bits per layer, optionally consider low/medium CSI payload size X/Y per layer. - Tx port: 32, 64, or 128 up to companies. - Subband number: 13, or 19 up to companies. - Performance metric: - Metric#1: SGCS for specific layer(s) between Target CSI of FP32 and recovery CSI, for a model trained with quantized Target CSI with a specific quantization option. - Metric#2: SGCS for specific layer(s) between the Target CSI of FP32 with its quantized version subject to a specific quantization solution - Overhead per sample and per layer SGCS results are reported - Note: Complexity should be analysed. - For Option 1, consider k1+k2=4, 6 ,7, 8, 16, 32 - Other assumptions follow R18 study EVM

Regarding the target CSI format, the enhancement of legacy codebook seems out of scope of this CSI compression agenda item. Therefore, Option 0 can be a baseline for the target CSI format. In addition, Option 1 can be further considered, since it is a simple and scalable method. As model training requires a large amount of data, it is important to generate target CSI using simple method at the UE-side. Even though Option1 may have larger payload sizes than Option 4, the payload size issue is not a critical problem if higher layer signaling is used for NW-side data collection. If a single Target CSI sample cannot be reported within one RRC message due to exceeding 9KBytes, it can be handled by reporting multiple messages.

Regarding the target CSI format for NW-side data collection,

  - Support legacy eType-II codebook (option 0) as a baseline.

  - Support scalar quantization (option 1)

    - If one Target CSI sample exceeds 9KBytes, multiple RRC messages can be considered.

Based on the agreement in the previous meeting, performance evaluation results for Option 1 with different k1+k2 bits are presented in Table I. In this evaluation, eigenvectors are considered as the Target CSI type, and the quantization ranges for int k1 and k2 are set to T1min=T2min=-1, T1max=T2max= 1 for real and imaginary elements. If dedicated T1min/T1max/T2min/T2max per sample are applied based on minimum and maximum values of real/imaginary elements, training performance may increase, with increasing payload overhead which is very marginal. Regarding the normalization scope, normalization within one sample (2N1N2*N3*v) for each real/imaginary or amplitude/phase part is preferred, since the total size of normalization scope information can be fixed regardless of the number of sub-bands, TX ports, or rank, which may depend on the NW configuration.

Regarding the normalization scope of Target CSI format Option 1, consider normalization within one sample (2N1N2*N3*v) for each real/imaginary or amplitude/phase part.

Table I. Option 1 SGCS results for Target CSI format (32 TX ports, 13 sub-bands)

| (k1, k2) | Size of Target CSI per layer (Bytes) | Metric#1: SGCS between Target CSI of FP32 and recovery CSI, for a model trained with quantized Target CSI |  |  |  |  | Metric#2: SGCS between the Target CSI of FP32 with its quantized version |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Layer 1 | Layer 2 | Layer 3 | Layer 4 | Average 4 Layers |  |
| (FP32, FP32) | 3328 | 0.9055 | 0.8457 | 0.5561 | 0.434 | 0.6853 | 1 |
| (FP16, FP16) | 1664 (-50%) | 0.9038 (-0.18%) | 0.8743 (+3.38%) | 0.5535 (-0.46%) | 0.4314 (-0.59%) | 0.6907 (+0.78%) | 1(0%) |
| (Int8, int8) | 832 (-75%) | 0.9052 (-0.03%) | 0.8458 (+0.01%) | 0.5566 (+0.08%)) | 0.434 (0%) | 0.6854 (-0.01%) | 0.9996(-0.04%) |
| (Int4, int4) | 416 (-87.5%) | 0.9036 (-0.20%) | 0.8741 (+3.35%) | 0.5558 (-0.05%) | 0.434 (0%) | 0.6918 (+0.98%) | 0.9043 (-9.57%) |
| (int3, int3) | 312 (-90.625%) | 0.6856 (-24.28%) | 0.6652 (-21.3%) | 0.4578 (-17.67%) | 0.371 (-14.51%) | 0.5449 (-20.48%) | 0.5708(-42.92%) |

Based on the evaluation results, Option 1 provides substantial overhead reduction compared to FP32 baseline. Specifically, the size of Target CSI per layer can be reduced by up to 87.5% with k1=k2=int4, while maintaining model performance (Metric#1). However, when the quantization bits k1 and k2 is further reduced to int 3, the SGCS noticeably decreases across all layers, indicating that int 3 quantization is too coarse to preserve the Target CSI information. Overall, these results demonstrate that Option 1 can achieve significant payload reduction with maintaining model performance when using int 4 or higher resolution. Therefore, the int 4 quantization is recommended as a practical trade-off between overhead reduction and model performance.

Observation 1: For Option 1, k1=k2=Int4 quantization provides more than 87.5% overhead reduction while maintaining the model performance (Metric#1) compared to the FP32 baseline. When the quantization level is further reduced to k1=k2=Int3, the model performance drops significantly across all layers, showing the limitation of coarse quantization.

General aspects on data collection

Since an AI/ML model is data-driven, its performance can vary depending on the characteristics of the dataset used for training, retraining, or update/fine-tuning. Training with a low-quality dataset (e.g., severe distortion in the downlink channel) or datasets consisting of diverse NW/UE-side conditions/additional conditions may degrade model performance. Also, for NW-side data collection, reporting all target CSI to the NW can increase uplink resource overhead. Compared to the BM cases where L1-RSRP and corresponding beam ids are reported for NW-side data collection, CSI compression requires more larger payload to include target CSI in certain range of SBs. Therefore, how to efficiently construct dataset for training is one of the important discussion points. For example, collected dataset can be selected based on predefined criteria (e.g., data quality metrics threshold)

For data collection for training, study how to efficiently construct dataset (e.g. data omission/drop, filtering)

Conclusion

In this contribution, we provided our views on the other aspects of the CSI compression, including model pairing, performance monitoring, and data collection. The summary of our views is listed below.

Observation 1: For Option 1, k1=k2=Int4 quantization provides more than 87.5% overhead reduction while maintaining the model performance (Metric#1) compared to the FP32 baseline. When the quantization level is further reduced to k1=k2=Int3, the model performance drops significantly across all layers, showing the limitation of coarse quantization.

Proposal #1:	For applicability report of CSI compression, consider both option A and option B.

Proposal #2:	Regarding the CSI compression specific inference related parameters for Option B, quantization related information (e.g., quantization parameter combination {D, Q, L}, VQ/SQ, layer/rank specific/common) and scalability related information (e.g., sub-bands, TX ports, payload sizes) can be considered

Proposal #3:	For NW-side monitoring, support target CSI reporting via L1 signaling with paired Target CSI and CSI feedback.

Proposal #4:	Deprioritize UE-side performance monitoring.

Proposal #5:	Regarding the target CSI format for NW-side data collection,

- Support legacy eType-II codebook (option 0) as a baseline.

- Support scalar quantization (option 1)

- If one Target CSI sample exceeds 9KBytes, multiple RRC messages can be considered.

Proposal #6:	Regarding the normalization scope of Target CSI format Option 1, consider normalization within one sample (2N1N2*N3*v) for each real/imaginary or amplitude/phase part.

Proposal #7:	For data collection for training, study how to efficiently construct dataset (e.g. data omission/drop, filtering)

- Reference

[1] RP-251870, “New WI: AI/ML for NR air interface enhancements”, Qualcomm

[2] R1-2502832, “Additional study on CSI compression”, Qualcomm

[3] R1-2502912, “Discussion on AI/ML for CSI compression”, CEWiT