3GPP TSG-RAN WG1 #123	R1-2508997

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:	8.1

Source:	Panasonic

Title:	Discussion on AI/ML maintenance in 5G NR interface

Document for:	Discussion

# Introduction

In Rel. 19, the following use cases have been specified for 5G NR AI/ML as follows [1]:

- Specification support for beam management (BM)

  - Spatial-domain DL Tx beam prediction for Set A of beams based on measurement results of Set B of beams (“BM-Case1”),

  - Temporal DL Tx beam prediction for Set A of beams based on the historic measurement results of Set B of beams (“BM-Case2”).

- Specification support for positioning accuracy enhancements

- Specification support for CSI prediction

In the following, we provide our views on maintenances on use cases of BM and CSI prediction.

# Discussion on specification support for BM

## The determination of updated CSI reports based on CPU and APU

Regarding an aspect of CPU and APU, it has been made the following agreements.

Agreement (RAN1#120bis) For UE-sided model, regarding a CSI report corresponding to CSI-ReportConfig for Type 1 option 2 monitoring, . Note: the occupation duration is a separate discussion. Agreement (RAN1#121) For UE-sided model, regarding a CSI-ReportConfig for data collection, - Reuse the existing CPU occupation time for a CSI report with CSI-ReportConfig with reportQuantity set to 'none' and TRS-info not configured Agreement (RAN1#121) For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case1, - Rel-15 CPU occupation time is reused for CPU occupation time of the CSI report - Rel-15 CPU occupation time is reused for AI/ML PU occupation time of the CSI report - Note: this is applicable to all types of CSI reports (i.e., AP/SP/P CSI report) Agreement (RAN1#121) For UE-sided model, regarding a CSI report with CSI-ReportConfig for inference for BM-Case2, for occupancy duration of CPU and APU, same occupation time for AI/ML PU and legacy CPU. - If the CSI report is aperiodic, for AI/ML PU, and for CPU, Rel-15 CPU occupation time for AP CSI report is reused - If the CSI report is semi-persistent or periodic, - From the 1st symbol of the latest CSI-RS/SSB transmission occasion no later than CSI reference resource, until the last symbol of the PUCCH/PUSCH carrying the report. Agreement (RAN1#121) For UE-sided model, regarding a CSI-ReportConfig for data collection, - Reuse the existing CPU occupation time for a CSI report with CSI-ReportConfig with reportQuantity set to 'none' and TRS-info not configured - For PU occupancy, for the number of AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied, - OAPU= 0 or X1/X2 is reported by UE in UE capability report for BM-Case 1 and BM-Case 2 respectively - OCPU=0 or Y1/Y2 is reported by UE in UE capability report for BM-Case 1 and BM-Case 2 respectively - Note: Detailed values of X1/X2 and Y1/Y2 can be further discussed in UE feature. - Note: Combination of OAPU= 0 and OCPU=0 is not allowed - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behavior of exceeding the CPU limit, neither of the PUs are occupied

In last RAN1#122bis, it has been discussed how to capture a note to specification because it is not clear in current CR provided by Editor. Based on the discussion, 3 alternatives are provided in below FL’s proposal [2].

FL proposal: For procedure for the determination of updated CSI reports based on CPU and APU, down-select the following: Alt 1: Handle the procedure for CPU and APU separately. If any condition for CPU and APU is not met, the CSI report is not required to be updated. Alt 2: handle the procedure for CPU and APU in sequential, and release the occupation based on the outcome. - Alt 2-1: CPU first. If the condition for CPU is not met, the corresponding APU is considered to be zero for subsequent procedure for APU. Alt 3: Handle the procedure for CPU and APU together. - Step 1: Determinate M, where M is the set of CSI reports that require to occupy CPU (i.e., require CPU only or CPU+APU), and the condition for CPU is met; - other CSI reports requiring to occupy CPU are not updated - Step 2: Determinate Mx, where Mx is the set of CSI reports that require to occupy APU (i.e., require APU only or CPU+APU), and the condition for APU is met. - other CSI reports requiring to occupy APU are not updated - Step 3: Determine a set of CSI reports that require to occupy CPU and APU, and are not within M (i.e., the condition for CPU is not met) or not within Mx (i.e., the condition for APU is not met) - The corresponding CPU and APU are considered to be zero for the set of CSI reports

We think that Alt. 1 is a simple solution to handle CPU and APU, Alt. 2 is too restrictive because it requires to handle sequentially. Alt. 3 is to utilize pools of CPU and GPU, but it requires most discussion effort to have consensus on 3 steps. With consideration of very limited TU, we would prefer to support Alt. 1 for a sake of progress.

Proposal 1: For procedure for the determination of updated CSI reports based on CPU and APU, support to handle the procedure for CPU and APU separately. If any condition for CPU and APU is not met, the CSI report is not required to be updated.

# Conclusions

In this document, we have discussed some details on AI/ML. We have the following proposals:

## Specification support for BM

Proposal 1: For procedure for the determination of updated CSI reports based on CPU and APU, support to handle the procedure for CPU and APU separately. If any condition for CPU and APU is not met, the CSI report is not required to be updated.

# Reference

[1]. RP-234039, “New WID on AI/ ML for NR Air Interface”, 3GPP, RAN#102.

[2]. R1-2508000, “FL summary #2 for AIML in beam management”, RAN1#122bis, Prague, Czech Republic, 2025.