3GPP TSG RAN WG1 #123	R1- 2509010

Dallas, USA, 17 – 21 November 2025

Agenda item:		8.3

Source:				Nokia, Nokia Shanghai Bell

Title:					Maintenance of SBFD

Document for:		Discussion and Decision

# Introduction

This contribution presents proposals for the maintenance of the Rel-19 work on SBFD. As per the RAN1 Chairman guidelines, the contribution focuses on essential corrections.

# Remaining issues on UE Tx/Rx behaviours

## Issue 1: SPS PDSCH and PUCCH across SBFD symbols and non-SBFD symbols

The following was agreed in RAN1#120-bis:

Conclusion For SPS HARQ-ACK transmission when Configuration 1 is configured for the UL BWP, the symbol type can be different for different PUCCH transmission occasions.

The above conclusion means that the symbol type can be different for different PUCCH transmission occasions triggered by a SPS PDSCH reception (regardless of whether Configuration 1 or 2 is configured for the UL BWP).

From our perspective, this agreement prevents the use of SPS PDSCH with short periodicities in SBFD operation. Taking Figure 1 as an example, the UE is configured with SPS PDSCH reception with a periodicity of 1 slot. In the left figure, the SPS HARQ-ACK (in a PUCCH) and the SPS PDSCH overlap in time during SBFD symbols. This is regarded as an error case due to collision of semi-statically configured UL transmission and DL reception (Case 3 of collision handling). The only option to solve this is to shorten the duration of the SPS PDSCH to ensure the PUCCH and PDSCH do not overlap, while still allowing sufficient time for UE Tx-Rx and Rx-Tx transitions. As shown in the right side of Figure 1 this results in a large resource waste due to many, frequent, and unnecessary UE Tx-Rx transitions. Note that all SPS PDSCHs use the same K1 value, so the same problem would also occur for other values of K1.

Allowing PUCCH transmission triggered by a SPS PDSCH reception to be valid in both SBFD and non-SBFD symbol types prevents the use of SPS PDSCH with short periodicities.

Figure 1: SPS PDSCH with short periodicities in SBFD

Recall that 3GPP Rel-17 standardized SPS HARQ-ACK deferral for TDD unpaired spectrum operation, where the UE postpones the SPS HARQ-ACK to the next ‘valid’ slot, e.g. the ‘U’ slot in case of TDD DDDSU operation. However, SPS HARQ-ACK deferral does not apply for SBFD operation since SBFD slots are always ‘valid’ for SPS HARQ-ACK PUCCH transmissions based on the latest RAN1 agreement.

RAN1 must therefore discuss whether to support SPS PDSCH with short periodicities in SBFD operation. As one simple solution, RAN1 could additionally agree to support configuring a valid symbol type for PUCCH containing HARQ-ACK for SPS PDSCH. By configuring SPS HARQ-ACK in non-SBFD symbols only, SPS HARQ-ACK deferral can be used to postpone all the SPS HARQ-ACK to the UL-slot. This is illustrated in Figure 2.

Figure 2: Deferral of SPS PDSCH with short periodicities in SBFD

Introduce configuration of a valid symbol type for PUCCH corresponding to SPS PDSCH HARQ-ACK in order to support SPS PDSCH with short periodicities in SBFD operation.

| Reason for change: | Current specifications do not support SPS PDSCH with short periodicities (e.g. 1 slot) in SBFD operation |
| --- | --- |
| Summary of change: | Introduce configuration of a valid symbol type for PUCCH corresponding to SPS PDSCH HARQ-ACK. |
| Consequences if not approved: | Limited support for SPS PDSCH with short periodicities (e.g. 1 slot) in SBFD operation. |

## 11.1 Slot configuration <omitted text> When the UE is provided sbfd-Configuration2-Transmission, the UE can - transmit a first PUCCH or PUSCH or a first repetition of a PUCCH or PUSCH, or receive a first PDSCH or a first repetition of a PDSCH, in non-SBFD symbols, and - transmit a second PUCCH or PUSCH or a second repetition of the PUCCH or PUSCH, or receive a second PDSCH or a second repetition of the PDSCH, in SBFD symbols When a UE is not provided sbfd-Configuration2-Transmission - for a PUCCH or PUSCH transmission with repetitions, or for a PDSCH reception with repetitions, if the first repetition is either in SBFD symbols or in non-SBFD symbols, the remaining repetitions are also either in SBFD symbols or in non-SBFD symbols, respectively - for a Type 2 CG PUSCH transmission, or for a PUSCH transmission with SP-CSI reports, or for a SPS PDSCH reception, if the first transmission or reception after a latest activation is either in SBFD symbols or in non-SBFD symbols, the remaining transmissions or receptions associated with the latest activation are also either in SBFD symbols or in non-SBFD symbols, respectively - for a Type 1 CG PUSCH, or for a PUCCH transmission with SP-CSI reports, or for a PUCCH with SR, [LRR, UEIRI], or for PUCCH corresponding to SPS HARQ-ACK with sps-PUCCH-symbolType provided in the SPS configuration, or P-CSI, or SRS, all respective transmissions are either only in SBFD symbols or only in non-SBFD symbols as indicated by the respective configurations [12, TS 38.331] <omitted text>

## Issue 2: Available slot counting for PUCCH

From the following specs excerpt from Section 9.2.6 of TS 38.213, it can be observed that a slot is counted/accumulated in the number of PUCCH repetitions NPUCCHrepeat if it has an UL/consecutive UL symbols or an SBFD/consecutive SBFD symbols. This would mean that, a slot with SBFD or non-SBFD symbols is counted/accumulated in NPUCCHrepeat regardless of whether Configuration 1 or Configuration 2 is applied.

9.2.6 PUCCH repetition procedure <<omitted text>> For unpaired spectrum, the UE determines the NPUCCHrepeat slots for a PUCCH transmission starting from a slot indicated to the UE as described in clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in clause 9.2.4 for SR reporting or in clause 5.2.1.4 of [6, TS 38.214] for CSI reporting and having - an UL symbol, as described in clause 11.1, or flexible symbol, or an SBFD symbol as described in clause 11.1, that is not SS/PBCH block symbol provided by startingSymbolIndex as a first symbol, and - consecutive UL symbols, as described in clause 11.1, or flexible symbols, or consecutive SBFD symbols as described in clause 11.1, respectively, that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols <<omitted text>>

In addition to that, the determination of a valid slot for UL PUCCH repetitions shall also take into account the overlapping with SSB or DL (non-SBFD) symbols, as noted in the following agreement:

Agreement (#120bis) For PUCCH repetition, PUSCH repetition with available counting and TBoMS with Configuration 1, - In case the valid symbol type is SBFD symbol, a slot is determined as available if the symbols allocated for PUCCH/PUSCH in the slot are all SBFD symbols and not include a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - In case the valid symbol type is non-SBFD symbol, a slot is determined as available if the symbols allocated for PUCCH/PUSCH in the slot are all non-SBFD symbols and not include a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

This issue was discussed in RAN1#122 and RAN1#122bis but without agreement. We propose to adopt the latest proposal from the moderator in R1-2508106:

The following change in Section 9.2.6 of TS 38.213 is proposed regarding PUCCH repetitions with a valid symbol type:

| Reason for change: | For PUCCH repetitions, the current specs do not capture that the repetitions are limited to a same valid symbol type. Also, the spec cannot guarantee that a slot with invalid symbol type will not be counted in NPUCCHrepeat repetitions. |
| --- | --- |
| Summary of change: | For unpaired spectrum, when a UE is not provided sbfd-Configuration2-Transmission, the repetitions are limited to a same valid symbol type. Clarify how to determine the valid symbol type for PUCCH repetitions depending on the valid symbol type. |
| Consequences if not approved: | Unclear behaviour if a slot with invalid symbol type will be counted in NPUCCHrepeat repetitions or not. |

9.2.6 PUCCH repetition procedure <<omitted text>> For unpaired spectrum, the UE determines the NPUCCHrepeat slots for a PUCCH transmission starting from a slot indicated to the UE as described in clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in clause 9.2.4 for SR reporting or in clause 5.2.1.4 of [6, TS 38.214] for CSI reporting and having - an UL symbol, as described in clause 11.1, or flexible symbol, or an SBFD symbol as described in clause 11.1, that is not SS/PBCH block symbol provided by startingSymbolIndex as a first symbol, and - consecutive UL symbols, as described in clause 11.1, or flexible symbols, or consecutive SBFD symbols as described in clause 11.1, respectively, that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols - if the UE is not provided sbfd-Config2-Transmission, and - if the first repetition is in SBFD symbols, the UE shall only consider slots containing consecutive SBFD symbols starting from the first symbol, with a length equal to or larger than the number of symbols indicated by nrofsymbols and do not include a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. - if the first repetition is in non-SBFD symbols, the UE shall only consider slots containing consecutive non-SBFD symbols starting from the first symbol, with a length equal to or larger than the number of symbols indicated by nrofsymbols, and do not include a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst. <<omitted text>>

## Issue 3: HPN for multi-PXSCH

This issue was discussed in RAN1#122bis under Issue 2-9 in the FL summary R1-2508106. One of the concerns on the moderator’s proposed TP was its length as well and the large amount of repeated text. In the following, we propose a simpler version of the TP discussed in RAN1#122bis with equivalent functionality:

Adopt the following change to TS 38.214 regarding HARQ Process Numbering for multi-PXSCH:

| Reason for change: | For multi-PXSCH, determination of the HARQ process number (HPN) for the first and subsequent PXSCH transmissions/receptions do not account for SBFD operation |
| --- | --- |
| Summary of change: | For multi-PXSCH transmissions/receptions, HPN is determined taking into account whether the PXSCH is in the valid symbol type and/or not across SBFD and non-SBFD symbols |
| Consequences if not approved: | Improper determination of HPN for multi-PXSCH with SBFD operation. |

# 5 Physical downlink shared channel related procedures ## 5.1 UE procedure for receiving the physical downlink shared channel ## <<omitted text>> A UE shall upon detection of a PDCCH with a configured DCI format 1_0, 1_1, 1_2, 1_3, 4_0, 4_1, or 4_2 decode the corresponding PDSCHs as indicated by that DCI. When the UE is scheduled with multiple PDSCHs on a serving cell by a DCI, HARQ process ID indicated by this DCI applies to the first PDSCH not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, and, if applicable, overlapping with the valid symbol type and not across SBFD symbols and non-SBFD symbols according to clause 5.1.2.1a., HARQ process ID is then incremented by 1 for each subsequent PDSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPDSCH applied if nrofHARQ-ProcessesForPDSCH is provided, or with modulo operation of nrofHARQ-ProcessesForPDSCH-v1700 applied if or nrofHARQ-ProcessesForPDSCH-v1700 is provided, or with modulo operation of 8 applied, otherwise. HARQ process ID is not incremented for PDSCH(s) not received if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or, if applicable, the symbols indicated by the indexed row overlap with a symbol that is not the valid symbol type or include SBFD symbols and non-SBFD symbols according to clause 5.1.2.1a. When a UE is configured by the higher layer parameter repetitionScheme set to 'tdmSchemeA', the PDSCH includes two PDSCH transmission occasions. For each PDSCH, if either PDSCH occasion overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, the PDSCH is not received and HARQ process ID is not increment for the PDSCH. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to receive a PDSCH that overlaps in time with another PDSCH if the UE is not capable of receiving FDMed unicast and multicast PDSCH per slot per carrier. When HARQ feedback for the HARQ process ID is not disabled, or for the HARQ process associated with the first SPS PDSCH when HARQ-feedbackEnablingforSPSactive is provided and enabled, the UE is not expected to receive another PDSCH for a given HARQ process until after the end of the expected transmission of HARQ-ACK for that HARQ process, where the timing is given by Clause 9.2.3 of [6, TS 38.213]. For HARQ-ACK subject to HARQ-ACK deferral described in Clause 9.2.5.4 of [6 TS 38.213], the expected transmission of HARQ-ACK corresponds to the expected transmission HARQ-ACK in a first slot. When HARQ feedback for the HARQ process ID is disabled, the UE is not expected to receive another PDCCH carrying a DCI scheduling a PDSCH or set of slot-aggregated PDSCH scheduled for the given HARQ process or to receive another PDSCH without corresponding PDCCH for the given HARQ process that starts until Tproc,1 after the end of the reception of the last PDSCH or slot-aggregated PDSCH for that HARQ process. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex and the UE reports its capability of outOfOrderOperationDL-r16, in a given scheduled cell, the UE is not expected to receive a first PDSCH and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH, where the two resources are in different slots for the associated HARQ-ACK transmissions, each slot is composed of symbols [4] or a number of symbols indicated by subslotLengthForPUCCH if provided, and the HARQ-ACK for the two PDSCHs are associated with the HARQ-ACK codebook of the same priority. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet and PDCCHs that schedule two PDSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex and the UE reports its capability of outOfOrderOperationDL-r16, in a given scheduled cell, the UE is not expected to receive a first PDSCH and a second PDSCH, starting later than the first PDSCH, with its corresponding HARQ-ACK assigned to be transmitted on a resource ending before the start of a different resource for the HARQ-ACK assigned to be transmitted for the first PDSCH if the HARQ-ACK for the two PDSCHs are associated with HARQ-ACK codebooks of different priorities. For any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start receiving a first PDSCH starting in symbol j by a PDCCH ending in symbol i on a scheduling cell, the UE is not expected to be scheduled to receive a PDSCH starting earlier than the end of the first PDSCH with a PDCCH that ends later than symbol i of a scheduling cell,. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], the PDCCH ending in symbol i is determined based on the PDCCH candidate that ends later in time. In a given scheduled cell, for any PDSCH corresponding to SI-RNTI, the UE is not expected to decode a re-transmission of an earlier PDSCH with a starting symbol less than N symbols after the last symbol of that PDSCH, where the value of N depends on the PDSCH subcarrier spacing configuration m, with N=13 for m=0, N=13 for m=1, N=20 for m=2, N=24 for m=3, N=96 for m=5, and N=192 for m=6. ## <<omitted text>> # 6 Physical uplink shared channel related procedure ## 6.1 UE procedure for transmitting the physical uplink shared channel ## <<omitted text>> When the UE is configured dl-OrJointTCI-StateList or ul-TCI-StateList, the UE shall perform PUSCH transmission corresponding to a Type 1 configured grant or a Type 2 configured grant or a dynamic grant according to the spatial relation, if applicable, with a reference to the RS for determining UL Tx spatial filter. The RS is determined based on an RS configured with qcl-Type set to 'typeD' of the indicated TCI-State or an RS in the indicated TCI-UL-State. The reference RS in the indicated TCI-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, or a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info. The reference RS in the indicated TCI-UL-State can be a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition, a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info, an SRS resource in an SRS resource set with the higher layer parameter usage set to 'beamManagement', or SS/PBCH block associated with the same or different PCI from the PCI of the serving cell. When nrofSlotsInCG-Period is configured for Type 1 configured grant or Type 2 configured grant, HARQ process ID for the first configured PUSCH grant and each subsequent valid configured PUSCH grant within a periodicity of the configuration is determined as in clause 5.4.1 of [10, TS 38.321], where a valid configured PUSCH grant is the one not colliding with the DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, and not colliding with a symbol(s) of an SS/PBCH block with index provided by ssb-PositionsInBurst as described in clause 11.1 of [6, TS 38.213] and, if applicable, not colliding with a symbol that is not the valid symbol type and not across SBFD symbols and non-SBFD symbols according to clause 6.1.2.1a. <<omitted text>> When the UE is scheduled with multiple PUSCHs on a serving cell by a DCI, HARQ process ID indicated by this DCI applies to the first PUSCH not overlapping with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, or, if applicable, a symbol that is not the valid symbol type and not across SBFD symbols and non-SBFD symbols according to clause 6.1.2.1a., HARQ process ID is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPUSCH applied if nrofHARQ-ProcessesForPUSCH is provided, or with modulo operation of 16 applied, otherwise. HARQ process ID is not incremented for PUSCH(s) not transmitted if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, or, if applicable, the symbols indicated by the indexed row overlap with a symbol that is not the valid symbol type or include SBFD symbols and non-SBFD symbols according to clause 6.1.2.1a. For any HARQ process ID(s) in a given scheduled cell, the UE is not expected to transmit a PUSCH that overlaps in time with another PUSCH. Except for the case when a UE is configured by higher layer parameter PDCCH-Config that contains two different values of coresetPoolIndex in ControlResourceSet for the active BWP of a serving cell and PDCCHs that schedule two PUSCHs are associated to different ControlResourceSets having different values of coresetPoolIndex and the UE reports its capability of outOfOrderOperationUL-r16 or outOfOrderOperationUL-r18, for any two HARQ process IDs in a given scheduled cell, if the UE is scheduled to start a first PUSCH transmission starting in symbol j by a PDCCH ending in symbol i on a scheduling cell,, the UE is not expected to be scheduled to transmit a PUSCH starting earlier than the end of the first PUSCH by a PDCCH that ends later than symbol i of the scheduling cell. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining the PDCCH ending in symbol i, the PDCCH candidate that ends later in time is used. The UE is not expected to be scheduled to transmit another PUSCH by a DCI format 0_0 with CRC scrambled by TC-RNTI, for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI format 0_0 with CRC scrambled by TC-RNTI or by an UL grant in RA Response. The UE is not expected to be scheduled to transmit another PUSCH by DCI format 0_0, 0_1, 0_2 or 0_3 scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI for a given HARQ process with the DCI received before the end of the expected transmission of the last PUSCH for that HARQ process if the latter is scheduled by a DCI with CRC scrambled by C-RNTI, CS-RNTI or MCS-C-RNTI. <<omitted text>>

## Issue 4: PUCCH resource outside UL subband

Some of the agreements reached for PUCCH transmissions in SBFD symbols are as follows:

Agreement (RAN1#118) Support separate frequency configurations for SBFD symbols and non-SBFD symbols in the same PUCCH-Resource. - pucch-ResourceId is not separately configured for SBFD and non-SBFD symbols - Support separate configurations of startingPRB and secondHopPRB for SBFD symbols and non-SBFD symbols - Introduce new RRC parameters in PUCCH-Resource to configure starting PRB and second hop PRB for SBFD symbols - FFS whether to support separate configurations of intraSlotFrequencyHopping for Configuration 1 or for both Configuration 1 and 2 - No change on the maximum number of PUCCH resources supported by a UE - Above PUCCH resources with the same pucch-ResourceId is counted as 1 resource FFS: UE behaviour when no separate configuration is provided for SBFD symbols, e.g. PUCCH transmissions in SBFD symbols for this pucch-ResourceId is not expected, or configurations for non-SBFD symbols are applied for SBFD symbols (in which case it is not expected that the configurations would lead to unexpected transmissions) etc.

Agreement (RAN1#120) If starting PRB is not configured for SBFD symbols for a PUCCH-Resource, starting PRB configured for non-SBFD symbols for the PUCCH-Resource is used for PUCCH transmissions in SBFD symbols associated with this pucch-ResourceId.

Agreement (RAN1#118) For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots with Configuration 1, - For PUSCH repetition type A with available slot counting, TBoMS and PUCCH repetitions, UE postpones transmissions in the invalid symbol type. For CG PUSCH and SPS PDSCH, P/SP SRS, P/SP CSI-RS, P/SP PUCCH, SP-CSI on PUSCH, PUSCH repetition type A without available slot counting, multi-PUSCH/PDSCH scheduled by a single DCI, and PDSCH repetitions, transmissions/receptions in the invalid symbol type are dropped.

As noted above, a new parameter startingPRB-SBFD has been introduced to define the starting PRB of a PUCCH resource in an SBFD symbol. In case such parameter is not provided, the legacy startingPRB is used instead for the PUCCH transmission in SBFD symbols. However, the current specifications do not properly describe the UE behaviour when PUCCH PRBs in SBFD symbols (determined from startingPRB-SBFD, or from startingPRB if startingPRB-SBFD is not provided) are outside the UL subband. Obviously, a PUCCH whose PRBs are outside the UL subband should not be transmitted as specified by the sentence “(…) the UE transmits or receives only in RBs that are both in the active UL BWP and in the UL sub-band, or both in the active DL BWP and the DL sub-band(s), respectively.” in TS 38.213, Clause 11.1; however, RAN1 should still discuss whether such event should be allowed or not by the specifications and, if allowed, whether e.g. such PUCCHs that are not transmitted should be counted or not as part of the NPUCCHrepeat repetitions.

Clarifying this also improves the consistency with respect to the UE behaviour for other channels, e.g. for the PUSCH channel, it is clear that the UE does not expect that the PRBs for PUSCH transmissions in SBFD symbols are outside the UL subband after applying RBSBFDoffset offset; while for the PDSCH, assigned PRBs that fall outside DL usable PRBs are considered to be invalid. Logically speaking, we think that a PUCCH that is not transmitted due to RBs falling outside the UL subband should not be counted as part of the NPUCCHrepeat repetitions.

Based on this, the following observations and proposals are provided:

According to current specifications, a PUCCH whose PRBs (determined from startingPRB-SBFD if provided, or from startingPRB otherwise) are outside the UL subband is not transmitted. However, RAN1 needs to discuss whether indicating the UE to transmit a PUCCH with PRBs outside the UL subband event should be allowed or not by the specifications. If allowed, it is not clear whether the PUCCHs that are not transmitted should be counted or not as part of the NPUCCHrepeat repetitions.

A PUCCH that is not transmitted due to RBs falling outside the UL subband should not be counted as part of the NPUCCHrepeat repetitions. Adopt the following TP to TS 38.213:

| Reason for change: | It is not clear whether indicating the UE to transmit a PUCCH with PRBs outside the UL subband event is allowed or not by the specifications. If allowed, it is not clear whether the PUCCHs that are not transmitted should be counted or not as part of the NPUCCHrepeat repetitions |
| --- | --- |
| Summary of change: | A PUCCH that is not transmitted due to RBs falling outside the UL subband should not be counted as part of the NPUCCHrepeat repetitions |
| Consequences if not approved: | Unclear behaviour in case the PRBs for PUCCH are not within the UL subband. |

9.2.6 PUCCH repetition procedure <<omitted text>> For unpaired spectrum, the UE determines the NPUCCHrepeat slots for a PUCCH transmission starting from a slot indicated to the UE as described in clause 9.2.3 for HARQ-ACK reporting, or a slot determined as described in clause 9.2.4 for SR reporting or in clause 5.2.1.4 of [6, TS 38.214] for CSI reporting and having - an UL symbol, as described in clause 11.1, or flexible symbol, or an SBFD symbol as described in clause 11.1, that is not SS/PBCH block symbol provided by startingSymbolIndex as a first symbol, and - consecutive UL symbols, as described in clause 11.1, or flexible symbols, or consecutive SBFD symbols as described in clause 11.1, respectively, that are not SS/PBCH block symbols, starting from the first symbol, equal to or larger than a number of symbols provided by nrofsymbols - If the PUCCH transmission is over SBFD symbols, the PRBs for the PUCCH as described in clause 9.2.1 are in the active UL BWP and in the UL sub-band. <<omitted text>>

In addition, a similar issue of PUCCH PRBs outside the UL subband was also discussed in R1-2507791 in the context of the SPS deferral. In current specifications, when a PUCCH for SPS HARQ-ACK overlaps with semi-static DL symbol or SSB symbol, the UE can defer the PUCCH until the first available ‘valid’ slot (i.e. a slot where PUCCH resource does not overlap with DL or SSB symbols). We support the proposal in R1-2507791 to extend the rules for determining the ‘valid’ slot to also include the i) possibility to transmit PUCCH with SPS HARQ-ACK in SBFD symbols and ii) considerations on whether the PUCCH PRBs are within the UL subband for the determination of the ‘valid’ slot for deferring the HARQ-ACK feedback. As noted in Figure 3, such additions provide an alternative option to solve the issue raised in Section 2.1 of this document by e.g. determining the initial PUCCH with SPS HARQ-ACK as ‘invalid’ due to the PUCCH PRBs being outside the UL subband in SBFD symbols, and then deferring the SPS HARQ-ACK information to the UL (non-SBFD) slot.

RAN1 to update the SPS HARQ-ACK deferral procedure (TS 38.213, 9.2.5.4) to account for SBFD operation, including consideration on whether the PUCCH PRBs are within the UL subband for the determination of the ‘valid’ slot for deferring the SPS HARQ-ACK. Use TP#13 in R1-2507791 as starting point for the discussion.

Figure 3: SPS HARQ-ACK deferral due to PUCCH PRBs outside UL subband

## Issue 5: CSI report can be transmitted in SBFD or UL slots

A UE configured with SBFD symbols can transmit a CSI report in uplink and SBFD symbols depending on the PUCCH or PUSCH allocation. However, the text in TS 38.214 does not reflect this and instead, only describes that CSI reports are expected in uplink slots. This can be seen within the definition of timeRestrictionForChannelMeasurements for L1-RSRP as highlighted in yellow below. Same observations are found for the L1-SINR reporting and others.

If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "notConfigured", the UE shall derive the channel measurements for computing L1-RSRP value reported in uplink slot n based on only the SS/PBCH or NZP CSI-RS, no later than the CSI reference resource, (defined in [4, TS 38.211]) associated with the CSI resource setting. If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the channel measurements for computing L1-RSRP reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SS/PBCH or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting.

Given this we suggest updating the text in TS 38.214 to allow for CSI reporting in SBFD slots, if configured.

Adopt the following TP to TS 38.214 to allow CSI report transmissions SBFD slots in addition to UL slots:

| Reason for change: | CSI reports are only considered to be transmitted in uplink slots |
| --- | --- |
| Summary of change: | Allow for CSI report transmission in uplink or SBFD slots |
| Consequences if not approved: | CSI reporting in SBFD slots is not specified |

5.2.1.4.3 L1-RSRP Reporting <<omitted text>> If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "notConfigured", the UE shall derive the channel measurements for computing L1-RSRP value reported in uplink or SBFD slot n based on only the SS/PBCH or NZP CSI-RS, no later than the CSI reference resource, (defined in [4, TS 38.211]) associated with the CSI resource setting. If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the channel measurements for computing L1-RSRP reported in uplink or SBFD slot n based on only the most recent, no later than the CSI reference resource, occasion of SS/PBCH or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting. <<omitted text>> 5.2.1.4.4 L1-SINR Reporting <<omitted text>> When one or two resource settings are configured for L1-SINR measurement - If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to 'notConfigured', the UE shall derive the channel measurements for computing L1-SINR reported in uplink or SBFD slot n based on only the SSB or NZP CSI-RS, no later than the CSI reference resource, (defined in TS 38.211[4]) associated with the CSI resource setting. - If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to 'configured', the UE shall derive the channel measurements for computing L1-SINR reported in uplink or SBFD slot n based on only the most recent, no later than the CSI reference resource, occasion of SSB or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting. - If the higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is set to 'notConfigured', the UE shall derive the interference measurements for computing L1-SINR reported in uplink or SBFD slot n based on only the CSI-IM or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) or NZP CSI-RS for channel and interference measurement no later than the CSI reference resource associated with the CSI resource setting. - If the higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is set to 'configured', the UE shall derive the interference measurements for computing the L1-SINR reported in uplink or SBFD slot n based on the most recent, no later than the CSI reference resource, occasion of CSI-IM or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) or NZP CSI-RS for channel and interference measurement associated with the CSI resource setting. <<omitted text>> 5.2.2 Channel state information 5.2.2.1 Channel quality indicator (CQI) <<omitted text>> Based on an unrestricted observation interval in time unless specified otherwise in this Clause, and an unrestricted observation interval in frequency, the UE shall derive for each CQI value reported in uplink or SBFD slot n the highest CQI index which satisfies the following condition: <<omitted text>> If the higher layer parameter timeRestrictionForChannelMeasurements is set to "notConfigured", the UE shall derive the channel measurements for computing CSI value reported in uplink or SBFD slot n based on only the NZP CSI-RS, no later than the CSI reference resource, (defined in [4, TS 38.211]) associated with the CSI resource setting. If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the channel measurements for computing CSI reported in uplink or SBFD slot n based on only the most recent, no later than the CSI reference resource, in cell DTX active period of a serving cell if cell DTX is activated, occasion of NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting on the serving cell. If the higher layer parameter timeRestrictionForInterferenceMeasurements is set to "notConfigured", the UE shall derive the interference measurements for computing CSI value reported in uplink or SBFD slot n based on only the CSI-IM and/or NZP CSI-RS for interference measurement no later than the CSI reference resource associated with the CSI resource setting. If the higher layer parameter timeRestrictionForInterferenceMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the interference measurements for computing the CSI value reported in uplink or SBFD slot n based on the most recent, no later than the CSI reference resource, in cell DTX active period of a serving cell if cell DTX is activated, occasion of CSI-IM and/or NZP CSI-RS for interference measurement (defined in [4, TS 38.211]) associated with the CSI resource setting on the serving cell.

## Issue 6: Collision handling order

There has been discussion of collision handling order for several RAN1 meetings while without consensus. There were 3 options from companies’ view about the collision handling for multiplexed PUCCH and for unused resource of SBFD configuration 1.

Actually the collision handling for multiplexed PUCCH is not a new issue. From current spec, it has already been solved. For dynamic TDD, there were also multiplexed PUCCH in collision handling. While the multiplexed PUCCH is seen as one type of resource but not discussed in detail on what is multiplexed. The PUCCH will be handled as a whole in the collision handling, i.e. the PUCCH multiplexing will be handled in processing of PUCCH and collision handling will not consider any content of the PUCCH in the collision [11, 11.1, 38.213].

Collision handling of multiplexed PUCCH is not new issue, which has already been solved in legacy spec for dynamic TDD, i.e. the PUCCH multiplexing will be handled in processing of PUCCH and collision handling will not consider any content of the PUCCH in the collision.

No new impact to spec is needed on collision handling of multiplexed PUCCH, because collision handling will consider PUCCH transmission as one type but not considering detail content of the PUCCH.

The SBFD configuration 1 is the new issue introduced in Rel19 in collision handling. But still, similar principle has already been handled in legacy spec for e.g. actual repetition. It means the unused resource is not considered as actual transmission or reception and should not be considered in the collision handling. Another way to consider the unused resource is as the resource is not used for transmission or reception of PUSCH, PDSCH, etc, then the resource is for other usage, then no need to be considered in the collision handing as in the spec only the used resource for transmission or reception should be considered. From this point of view, for each channel, when SBFD configuration 1 is applied, firstly the unused resource within the entire channel transmission or reception should be handled and then the channel will be considered as one type in the collision handling.

In legacy spec, the unused resource of a channel will be excluded and not seen as part of the channel in the collision handling, i.e. if a resource is not used for a channel, then no need to consider that resource in collision handling of that channel.

Consider the following two ways options for collision handling order in SBFD:

  - Option 1) firstly the unused resource in SBFD configuration 1 is excluded from the transmission or reception, then the legacy collision handling is conducted

  - Option 2) no new impact to spec for collision handling order of SBFD, i.e. as legacy principle, only used resource will be considered as part of the transmission/reception.

Based on above discussion, if option 1 can be a compromise in RAN1, then we think the following TP for 38.213 can be discussed.

# 9	UE procedure for reporting control information

<<omitted text>>

When PUCCH or PUSCH with UCI is across SBFD symbols and non-SBFD symbols within a slot or in different slots, and UE is not configured with sbfd-Configuration2-Transmission, the UE determines not to transmit PUCCH or PUSCH in invalid type of symbol(s) before resolving the overlapping between PUCCH transmissions or between PUCCH and PUSCH transmissions.

<<omitted text>>

RAN1 can discuss the above TP for 38.213 on collision handling order.

## Issue 7: SBFD in half duplex CA operation

There was discussion of SBFD in half duplex CA scenario in RAN1 #122 meeting and there were two TP options discussed, while actually both of them deem SBFD symbol as similar processing as flexible symbol in dynamic TDD and no essential difference between the two TP options. In legacy dynamic TDD operation, there was already discussion in detail on how to handle the flexible symbol in half duplex CA operation. While for half duplex CA, it is same impact by SBFD as the symbol may be used by UE as uplink or downlink considering the transmission/reception direction. By this understanding, we think there can be simple impact/update to spec to treat the SBFD symbol as flexible type as in dynamic TDD in legacy.

SBFD symbol should be treated the same as flexible symbol in half duplex CA operation as legacy.

## Issue 8: CSI reference resource definition

The concept of CSI reference resource is used to determine the UE behaviour for performing CSI measurements according to the parameter timeRestrictionForChannelMeasurements/timeRestrictionForInterferenceMeasurements. Moreover, it is also used as condition for whether a UE shall report CSI after CSI report (re)configuration, serving cell activation, BWP change or activation of SP-CSI or not.

The CSI reference resource is defined in time domain by a single downlink slot calculated based on a time-offset with respect to the uplink slot n ' where the CSI report is expected to be transmitted. According to latest TS 38.214, a downlink slot is considered as valid from a CSI reference resource definition point of view if:

The slot comprises at least one higher layer configured downlink or flexible symbol, and

The slot does not fall within a configured measurement gap for the UE.

Now with the introduction of SBFD symbols, we think the conditions to determine if a slot can be considered as valid downlink slot shall be extended. We propose that a slot comprising at least a SBFD symbol should also be considered as valid downlink slot.

Adopt the following TP to TS 38.214 to extend the conditions to determine a valid downlink slot for the CSI reference resource definition:

| Reason for change: | CSI reference resource definition needs to be updated |
| --- | --- |
| Summary of change: | Allow a slot with SBFD symbols to also be considered as valid downlink slot |
| Consequences if not approved: | Incorrect UE behaviour for CSI derivation |

#### 5.2.2.5 CSI reference resource definition << omitted text >> A slot in a serving cell shall be considered to be a valid downlink slot if: - it comprises at least one higher layer configured downlink or flexible or SBFD symbol, and - it does not fall within a configured measurement gap for that UE << omitted text >>

# Remaining issues on Cross-link interference handling

## Issue 1: Clarification of DL signals/channels in FG 60-9a

During RAN1 #122 meeting, we discussed the following proposal about the UE behaviour when configured with overlapping CLI-RSSI and SRS-RSRP measurement resources:

Proposal (RAN1 #122): UE does not expect to be configured with CLI-RSSI measurement resources in the DL subband and SRS-RSRP measurement resources in UL subband in the same OFDM symbol.

Also, regarding to this issue, RAN4 has made following agreement:

Agreement (RAN4 #116) - UE does not expect to be configured with L1-CLI-RSSI measurements resources within DL subband(s) and L1-SRS-RSRP measurement resources on the same symbol. - Note: this can be revised if RAN1 have different agreement for this Collision between Method#1 and Method#2.

As shown above, RAN4 has made the decision of preventing the configuration of LI-CLI-RSSI measurements resources within DL subband(s) and L1-SRS-RSRP measurements resources on the same OFDM symbol. However, this agreement could be revised if RAN1 agrees a different behaviour.

From our point of view, we prefer to not enforce such configuration restriction. Note that RAN1 earlier agreed on a capability that a UE can receive DL signals and perform SRS-RSRP measurements if separated in frequency (see RAN1 latest UE capabilities below). Thus, for UEs supporting such capability, measuring L1-CLI-RSSI within DL subband(s) and L1-SRS-RSRP within UL subband should be feasible. With this said, we think that setting a CLI measurement resources configuration restriction will go against the mentioned UE capability.

Updated RAN1 UE features list for Rel-19 NR after RAN1 #122 60. NR_duplex_evo; 60-9a; FDM-ed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 1. Support of FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource; 60-9; YES; n/a; FDMed reception of DL signals/channels and L1 SRS-RSRP measurement resource is not supported; Per band; TDD only; n/a; n/a; Optional with capability signalling.

RAN1 agreed to support a UE capability to support FDM-ed reception of DL signals and SRS-RSRP measurements.

Allow the configuration of L1-CLI-RSSI within DL subband(s) and L1-SRS-RSRP measurements resources within UL subband on the same symbol.

For UEs not supporting the FDM-ed DL reception and SRS measurement, we prefer to define collision handling rules. These rules shall be used by the UE to prioritize L1-CLI-RSSI or L1-SRS-RSRP depending on the specific case.

Introduce collision handling rules to handle the prioritization of L1-CLI-RSSI and L1-SRS-RSRP resources for UEs not supporting simultaneous FDM-ed DL reception and SRS-RSRP measurements.

Depending on the measurement resource type and reporting type, the following collision/cases between CLI-RSSI and SRS-RSRP are expected:

|  |  | AP reporting of L1-CLI-RSSI |  |  | P reporting of L1-CLI-RSSI |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | P CLI-RSSI resources | SP CLI-RSSI resources | AP CLI-RSSI resources | P CLI-RSSI resources | SP CLI-RSSI resources | AP CLI-RSSI resources |
| AP reporting of L1-SRS-RSRP | P L1-SRS-RSRP resources | Prioritize SRS-RSRP | Prioritize CLI-RSSI | Prioritize CLI-RSSI | Prioritize SRS-RSRP | N/A | N/A |
|  | SP L1-SRS-RSRP resources | Prioritize SRS-RSRP | Prioritize SRS-RSRP | Prioritize CLI-RSSI | Prioritize SRS-RSRP | N/A | N/A |
|  | AP L1-SRS-RSRP resources | Prioritize SRS-RSRP | Prioritize SRS-RSRP | Prioritize SRS-RSRP | Prioritize SRS-RSRP | N/A | N/A |

The table above includes the outcome after applying the proposed collision handling rules. The following criteria are used to determine whether CLI-RSSI or SRS-RSRP shall be prioritized:

- Aperiodic reporting shall be prioritized over periodic reporting

- Aperiodic resources shall be prioritized over periodic and semi-persistent resources

- Semi-persistent resources shall be prioritized over periodic resource

- SRS-RSRP resources shall be prioritized over CLI-RSSI resources when both have the same resource type

Use the following rules for collision handling between L1-CLI-RSSI and L1-SRS-RSRP measurement resources:

  - Aperiodic reporting shall be prioritized over periodic reporting

  - Aperiodic resources shall be prioritized over periodic and semi-persistent resources

  - Semi-persistent resources shall be prioritized over periodic resource

  - SRS-RSRP resources shall be prioritized over CLI-RSSI resources when both have the same resource type

# Conclusion

In this contribution we have discussed the maintenance of SBFD TX/RX/measurement procedures. The following observations and proposals have been made:

Observation 1:	Allowing PUCCH transmission triggered by a SPS PDSCH reception to be valid in both SBFD and non-SBFD symbol types prevents the use of SPS PDSCH with short periodicities.

Proposal 1:	Introduce configuration of a valid symbol type for PUCCH corresponding to SPS PDSCH HARQ-ACK in order to support SPS PDSCH with short periodicities in SBFD operation.

Proposal 2:	The following change in Section 9.2.6 of TS 38.213 is proposed regarding PUCCH repetitions with a valid symbol type:

Proposal 3:	Adopt the following change to TS 38.214 regarding HARQ Process Numbering for multi-PXSCH:

Observation 2:	According to current specifications, a PUCCH whose PRBs (determined from startingPRB-SBFD if provided, or from startingPRB otherwise) are outside the UL subband is not transmitted. However, RAN1 needs to discuss whether indicating the UE to transmit a PUCCH with PRBs outside the UL subband event should be allowed or not by the specifications. If allowed, it is not clear whether the PUCCHs that are not transmitted should be counted or not as part of the NPUCCHrepeat repetitions.

Proposal 4:	A PUCCH that is not transmitted due to RBs falling outside the UL subband should not be counted as part of the NPUCCHrepeat repetitions. Adopt the following TP to TS 38.213:

Proposal 5:	RAN1 to update the SPS HARQ-ACK deferral procedure (TS 38.213, 9.2.5.4) to account for SBFD operation, including consideration on whether the PUCCH PRBs are within the UL subband for the determination of the ‘valid’ slot for deferring the SPS HARQ-ACK. Use TP#13 in R1-2507791 as starting point for the discussion.

Proposal 6:	Adopt the following TP to TS 38.214 to allow CSI report transmissions SBFD slots in addition to UL slots:

Observation 3:	Collision handling of multiplexed PUCCH is not new issue, which has already been solved in legacy spec for dynamic TDD, i.e. the PUCCH multiplexing will be handled in processing of PUCCH and collision handling will not consider any content of the PUCCH in the collision.

Proposal 7:	No new impact to spec is needed on collision handling of multiplexed PUCCH, because collision handling will consider PUCCH transmission as one type but not considering detail content of the PUCCH.

Observation 4:	In legacy spec, the unused resource of a channel will be excluded and not seen as part of the channel in the collision handling, i.e. if a resource is not used for a channel, then no need to consider that resource in collision handling of that channel.

Proposal 8:	Consider the following two ways options for collision handling order in SBFD:

	Option 1) firstly the unused resource in SBFD configuration 1 is excluded from the transmission or reception, then the legacy collision handling is conducted

	Option 2) no new impact to spec for collision handling order of SBFD, i.e. as legacy principle, only used resource will be considered as part of the transmission/reception.

Proposal 9:	RAN1 can discuss the above TP for 38.213 on collision handling order.

Proposal 10:	SBFD symbol should be treated the same as flexible symbol in half duplex CA operation as legacy.

Proposal 11:	Adopt the following TP to TS 38.214 to extend the conditions to determine a valid downlink slot for the CSI reference resource definition:

Observation 5:	RAN1 agreed to support a UE capability to support FDM-ed reception of DL signals and SRS-RSRP measurements.

Proposal 12:	Allow the configuration of L1-CLI-RSSI within DL subband(s) and L1-SRS-RSRP measurements resources within UL subband on the same symbol.

Proposal 13:	Introduce collision handling rules to handle the prioritization of L1-CLI-RSSI and L1-SRS-RSRP resources for UEs not supporting simultaneous FDM-ed DL reception and SRS-RSRP measurements.

Proposal 14:	Use the following rules for collision handling between L1-CLI-RSSI and L1-SRS-RSRP measurement resources:

	Aperiodic reporting shall be prioritized over periodic reporting

	Aperiodic resources shall be prioritized over periodic and semi-persistent resources

	Semi-persistent resources shall be prioritized over periodic resource

	SRS-RSRP resources shall be prioritized over CLI-RSSI resources when both have the same resource type