3GPP TSG RAN WG1 #123			   R1-2509201

Dallas, USA, November 17th  – 21st, 2025

Agenda item:	8.3

Source: 	Qualcomm Incorporated

Title: 	Maintenance for NR duplex evolution

Document for:	Discussion/Decision

# Introduction

In this contribution we provide our views on the maintenance and remaining issues for NR duplex evolution for SBFD operation and CLI enhancement.

# SBFD Tx/Rx operation and SBFD configuration

## Collision handling of Half-duplex CA

It was agreed to support multi-carrier operation where one or more carrier support SBFD operation within the carrier. One remaining issue was FFS point on whether/how to handle the collision for half-duplex UE operating in multi-carrier scenario. In current specification, HD-CA collision rules first identify a reference cell for a symbol and determine the direction (uplink or downlink) either based on the TDD-UL-DL pattern or based on the configured uplink transmission or downlink reception by higher layer in the flexible symbol. The SBFD symbol can be treated as flexible symbol. When there is configured/scheduled DL/UL transmission or reception in the SBFD symbol, the existing collision rules between configured/scheduled UL (or DL) in one cell and configured/scheduled DL (or UL) in another cell can be reused

Proposal 1: Adopt the following TP with change in red to TS 38.213 for handling the collision in half-duplex CA when one or more carrier supporting SBFD operation, where SBFD symbol is treated as flexible symbol.

## 11.1 Slot configuration ## *** Unchanged parts are omitted *** If a UE - is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the configured multiple serving cells, and - indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, the UE determines a reference cell for a symbol as an active cell with the smallest cell index among - the configured multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and - the cells of each band respectively if the UE is capable of simultaneous transmission and reception by simultaneousRxTxInterBandCA for the configured multiple serving cells, ## where the symbol is configured by higher layers as - downlink, or uplink, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated - uplink, if the symbol is flexible or SBFD, and the UE is configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol - downlink, if the symbol is flexible or SBFD, and the UE is configured by higher layers to receive PDCCH, PDSCH or CSI-RS on the symbol. ## *** Unchanged parts are omitted ***

Reason for change - There is a remaining issue on how to handle the collision in handle half-duplex CA case where one or more cells are operation in SBFD mode. Summary of change - Treat SBFD symbol as flexible symbol where link direction is determined as uplink or downlink based on whether the UE is configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol or to receive PDCCH, PDSCH or CSI-RS on the symbol, respectively.

## Enhancement to Resource allocation in frequency domain in SBFD symbols

### Downlink precoding granularity (PRG)

The PRG size for PDSCH can be 2 PRBs or 4 PRBs or wideband precoding. The PRG size can be either configured semi-statically or dynamically indicated. When dynamic precoding bundling indication, i.e. the higher layer parameter prb-BundlingType is set to 'dynamicBundling’,  gNB configures two sets of bundles where the first set is configured with one or two values among {2,4,wideband} and second set is configured with one value among {2,4,wideband}and bundling size indicator bitfield in DCI determine which PRG set to use.

When DCI bitfield ‘bundling size indicator’ set to ‘1’ and first set configured with two values as ‘n4-wideband’ or ‘n2-wideband’, the condition for determining wideband or narrowband depends on number scheduled RBs with respect to half of the size of the BWP NBWPsize/2. Then, there could be issue when UE is scheduled PDSCH in one of the DL subband, as the number of scheduled PRBs may be always less than half the BWP size which may not allow for wideband precoding. This  issue can be solved by interpreting the NBWPsize as the number of valid DL usable PRB in the DL subband(s).

Proposal 2: Adopt the following TP with change in red to TS 38.214 for dynamic determination of the precoding bundling in SBFD symbols when DCI bitfield (bundling size indicator) indicates ‘1’ and first set configured with two values as ‘n4-wideband’ or ‘n2-wideband’.

5.1.2.3 Physical resource block (PRB) bundling ## *** Unchanged parts are omitted *** If the PRB 'bundling size indicator' signalled in DCI format 1_1 as defined in Clause 7.3.1.2.2 of [5, TS 38.212] - is set to '0', the UE shall use the value from the second set of values when receiving PDSCH scheduled by the same DCI. - is set to '1' and one value is configured for the first set of values, the UE shall use this value when receiving PDSCH scheduled by the same DCI - is set to '1' and two values are configured for the first set of values as 'n2-wideband' (corresponding to two values 2 and wideband) or 'n4-wideband' (corresponding to two values 4 and wideband), the UE shall use the value when receiving PDSCH scheduled by the same DCI as follows: - If the scheduled PRBs are contiguous and the size of the scheduled PRBs is larger than , is the same as the scheduled bandwidth, otherwise is set to the remaining configured value of 2 or 4, respectively. If UE configured with SBFD symbols, NBWP,isize is the number PRBs that are both in the active DL BWP and in the DL sub-band(s).

Reason for change - When DCI bitfield ‘bundling size indicator’ indicates ‘1’ and first set configured with two values as ‘n4-wideband’ or ‘n2-wideband’, the condition for determining wideband or narrowband depends on number scheduled RBs with respect to half of the size of the BWP NBWPsize/2. - At least for DUD configuration when PDSCH is scheduled in one DL subband, the condition for wideband precoding may not be satisfied. Summary of change - The threshold of half BWP size is reinterpreted half number of DL usable PRB in the subband(s) where PDSCH is scheduled.

## Physical channels/signals and procedure across SBFD and non-SBFD symbols in different slots.

### Invalid transmission/reception occasions due to configuration #1

In current specification, UL transmission occasion overlapping with at least DL symbol or SSB symbol is considered invalid. Some examples of these occasions include multiple PUSCHs by single DCI, or multiple PUSCHs in a CG period or valid occasion carrying UTO-UCI.  The definition of valid transmission occasion should take into account the invalid symbol of configuration #1. Similarly, DL reception occasions that overlap at least with UL symbols are considered invalid and not considered for some procedure, e.g. HARQ process ID increment for multiple PDSCHs by single DCI. The definition of valid DL reception occasion should take into account the invalid symbol type due to configuration #1.

#### Multiple PDSCHs by single DCI

When the UE is scheduled with multiple PDSCHs on a serving cell by a DCI, HARQ process ID indicated by this DCI applies to the first valid PDSCH. In TDD system, the first valid PDSCH refers to the PDSCH not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided. In addition, HARQ process ID is not incremented for PDSCH(s) not received if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided.

When UE is provided with configuration #1 for DL reception in a BWP and UE determines the valid symbol type as ‘SBFD’ or ‘non-SBFD’ based on the first PDSCH reception occasion indicated by the scheduling DCI. Then, PDSCHs occasions in the invalid symbol types are not received and UE doesn’t increment the HARQ process ID.

Proposal 3: Adopt the following TP with change in red to TS 38.214 for the multiple PDSCHs scheduled by single DCI format

## 5.1 UE procedure for receiving the physical downlink shared channel ## *** Unchanged parts are omitted *** A UE shall upon detection of a PDCCH with a configured DCI format 1_0, 1_1, 1_2, 1_3, 4_0, 4_1, or 4_2 decode the corresponding PDSCHs as indicated by that DCI. When the UE is scheduled with multiple PDSCHs on a serving cell by a DCI, HARQ process ID indicated by this DCI applies to the first PDSCH not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, HARQ process ID is then incremented by 1 for each subsequent PDSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPDSCH applied if nrofHARQ-ProcessesForPDSCH is provided, or with modulo operation of nrofHARQ-ProcessesForPDSCH-v1700 applied if or nrofHARQ-ProcessesForPDSCH-v1700 is provided, or with modulo of 8 applied, otherwise. HARQ process ID is not incremented for PDSCH(s) not received if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided or invalid symbol type if the UE is configured with SBFD symbols and not configured with [sbfd-Configuration2-Reception]. *** Unchanged parts are omitted ***

Reason for change - For multiple PDSCHs scheduled by single DCI, HARQ process ID is not incremented for PDSCH(s) not received if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a UL symbol. - With SBFD configuration #1 and valid symbol as ‘sbfd’ or ‘non-sbfd’, HARQ process ID is not incremented for invalid PDSCH(s) that overlap with ‘non-SBFD’ or ‘SBFD’. Summary of change - For multiple PDSCHs scheduled by singe DCI and UE is provided with SBFD configuration #1, HARQ-process ID is not incremented for PDSCH(s) not received in the invalid symbol type.

#### Multiple PUSCHs by single DCI

When the UE is scheduled with multiple PUSCHs on a serving cell by a DCI, HARQ process ID indicated by this DCI applies to the first valid PUSCH. In TDD system, the first PUSCH refers to the PUSCH not overlapping with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided or SSB symbol. In addition, HARQ process ID is not incremented for PUSCH(s) not received if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or SSB symbol.

When UE is provided with configuration #1 for UL transmission in a BWP and UE determines the valid symbol type as ‘SBFD’ or ‘non-SBFD’ based on the first PUSCH reception occasion indicated by the scheduling DCI. Then, PUSCHs occasions in the invalid symbol types are not transmitted and UE doesn’t increment the HARQ process ID.

Proposal 4: Adopt the following TP with change in red to TS 38.214 for the multiple PUSCHs scheduled by single DCI format.

## 6.1 UE procedure for transmitting the physical uplink shared channel ## *** Unchanged parts are omitted *** When the UE is scheduled with multiple PUSCHs on a serving cell by a DCI, HARQ process ID indicated by this DCI applies to the first PUSCH not overlapping with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, HARQ process ID is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPUSCH applied if nrofHARQ-ProcessesForPUSCH is provided, or with modulo operation of 16 applied, otherwise. HARQ process ID is not incremented for PUSCH(s) not transmitted if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst or invalid symbol type if the UE is configured with SBFD symbols and not configured with [sbfd-Configuration2-Transmission]. *** Unchanged parts are omitted ***

Reason for change - For multiple PUSCHs scheduled by single DCI, HARQ process ID is not incremented for PUSCH(s) not received if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL or SSB symbol. - With SBFD configuration #1 and valid symbol as ‘sbfd’ or ‘non-sbfd’, HARQ process ID is not incremented for invalid PUSCH(s) that overlap with a ‘non-SBFD’ or ‘SBFD symbol respectively’. Summary of change - For multiple PUSCHs scheduled by singe DCI and UE is provided with SBFD configuration #1, HARQ-process ID is not incremented for PUSCH(s) not transmitted in the invalid symbol type.

#### Multiple PUSCHs in a CG period

In TDD deployment, when UE is configured with multiple CG-PUSCHs in a CG period, a valid CG-PUSCH is the one not colliding with DL symbol(s) or SSB symbols. When UE is configured with SBFD symbols and provided with configuration #1, the CG-PUSCH in the invalid symbols should be considered invalid. For example, for CG-PUSCH type 1, if UE is configured with symbol type = ‘sbfd’, then a valid CG-PUSCH is the one not colliding with non-SBFD symbols or SSB symbols. Similarly, for valid symbol type = ‘non-SBFD’, a valid CG-PUSCCH is the one not colliding with SBFD symbols, DL symbol or SSB.

Proposal 5: Adopt the following TP with change in red to TS 38.214 for the valid CG-PUSCHs in a CG period.

## 6.1 UE procedure for transmitting the physical uplink shared channel ## *** Unchanged parts are omitted *** When nrofSlotsInCG-Period is configured for Type 1 configured grant or Type 2 configured grant, HARQ process ID for the first configured PUSCH grant and each subsequent valid configured PUSCH grant within a periodicity of the configuration is determined as in clause 5.4.1 of [10, TS 38.321], where a valid configured PUSCH grant is the one not colliding with the DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, and not colliding with a symbol(s) of an SS/PBCH block with index provided by ssb-PositionsInBurst as described in clause 11.1 of [6, TS 38.213], and not colliding with invalid symbol type if the UE is configured with SBFD symbols and not configured with [sbfd-Configuration2-Transmission]. ## *** Unchanged parts are omitted ***

Reason for change - In TDD deployment with multiple CG-PUSCHs in CG period, a valid CG-PUSCH is the one not colliding with DL symbol(s) or SSB symbols. - With SBFD configuration #1 and valid symbol as ‘sbfd’ or ‘non-sbfd’, a valid CG-PUSCH shouldn’t collied with invalid symbol type. In other words, a CG-PUSCH occasion colliding with invalid symbol is considered invalid. Summary of change - A valid CG-PUSCH shouldn’t collide with invalid symbol type when UE is provided with SBFD symbols and configuration #1.

### Default valid symbol type under configuration #1

When UE is provided with configuration 1 for UL transmission (or DL reception) per UL-BWP (or DL-BWP). The valid symbol type will be configured semi-statically for CSI-ReportConfig, CG-PUSCH type 1 and SRS resource Set and PUCCH carrying P/SP CSI or SR. if the valid symbol type is not configured, then the default behaviour should be non-SBFD symbols.

Proposal 6: When UE is not provided semi-statically with valid symbol type for SRS, CSI-ReportConfig and PUCCH resource carrying P/SP CSI or SR, then default assumption is non-SBFD symbol as valid symbol type.

### PUSCH TDW and DMRS bundling

When DMRS bundling is enabled, UE keeps power and phase coherency within each actual TDW. Rel-17 introduced procedure for the UE how to determine the nominal and actual TDWs. In general, when UE encounters an event that causes interruption of power and phase coherency, the nominal TDW is reset and new actual TDW is created.

Then, when PUSCH or PUCCH repetitions crosses the boundaries between SBFD slot and non-SBFD slot, the phase continuity and/or power consistency may not be maintained as the UE may change the start-RB of the PUSCH. The UE may retune some of the filters, adjust some settings for the RFFE, adapt the sampling rates and possibly the UL timing. In addition, the gNB may need to adapt the analog filters and switch some panels. Similarly, the gNB may change some of the reception configurations, e.g. panels, filter or beams.  Then, the transition from SBFD to non-SBFD symbol (or vice versa) can break the phase continuity. And then, the actual TDW is terminated at the boundary and new actual TDW is started as shown in the figure below.

Proposal 7: Adopt the following TP with change in red to TS 38.214 for DMRS bundling where phase coherency is not maintained across SBFD and non-SBFD symbols.

#### 6.1.7 UE procedure for determining time domain windows for bundling DM-RS <omitted text> Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1, 0_2 or 0_3, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: - A downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated for unpaired spectrum <omitted text> - A transition from SBFD to non-SBFD, or transition from non-SBFD to SBFD symbols <omitted text>

Reason for change - When DMRS bundling is enabled, the phase coherency can’t be maintained across SBFD and non-SBFD symbols. - When UE is scheduled with PUSCH/PUCCH with DMRS bundling, the TDW Summary of change - Add the switching from SBFD to non-SBFD and vice versa as an event that causes phase continuity not to be maintained for PUSCH/PUCCH transmission in different slots across.

### CSI reference resource

In current spec, the CSI reference resource is defined such that in time domain it is located in a single downlink slot (n-nCSIref - Koffset2μDL2μKoffset) for a CSI reporting in an uplink slot n’, where  nCSIref is determined as follows for periodic/semi-persistent CSI reporting

- If a single CSI-RS/SBB resource is configured for channel measurement, nCSIref is the smallest value greater or equal to 4.2μDL such that it corresponds to a valid downlink slot.

- If multiple CSI-RS/SBB resources are configured for channel measurement, nCSIref is the smallest value greater or equal to 5.2μDL such that it corresponds to a valid downlink slot.

The CSI reference location in time determines which CSI-RS occasions are used for computing the CSI. A slot is considered to be a valid downlink slot if it comprises at least one higher layer configured DL or FL symbol, and it does not fall within a configured measurement gap for that UE. It may need to be clarified that a valid DL slot can be either DL symbol or SBFD symbols and not just limited to non-SBFD symbols.

Proposal 8: Adopt the following TP with change in red to TS 38.214 for clarification of the valid DL slot of the

the CSI reference resource.

## 5.2.2.5 CSI reference resource definition ## *** Unchanged parts are omitted *** A slot in a serving cell shall be considered to be a valid downlink slot if: - It comprises at least one higher layer configured downlink or flexible symbol or SBFD symbol, and - it does not fall within a configured measurement gap for that UE ## *** Unchanged parts are omitted ***

Reason for change - CSI reference resource in time domain is defined to correspond to a valid downlink slot, where a valid downlink slot comprises at least one DL/FL symbol configured by RRC. It is better to clarify that a slot containing an SBFD symbol is also a valid downlink slot, and the definition is not restricted to non-SBFD symbols. Summary of change - Clarify that a valid DL slot may or may not contain SBFD symbols, i.e., CSI reference resource in time can correspond to a slot containing one or more SBFD symbols.

Additionally, for the purpose of deriving CQI and/or PMI, the UE assumes that the hypothetical PDSCH bandwidth is the same as configured in the CQI report. This may need to be clarified for SBFD CSI report, where the PDSCH frequency resources included only in the downlink usable PRBs in an SBFD symbol should be considered.

Proposal 9: Adopt the following TP with change in red to TS 38.214 for the freq. resources of the hypothetical PDSCH bandwidth used for deriving SBFD CSI metrics, in which the UE shall assume that the CSI reference resource contains only the usable DL PRBs in the CSI report bandwidth.

## 5.2.2.5 CSI reference resource definition ## *** Unchanged parts are omitted *** If configured to report CQI index, in the CSI reference resource, the UE shall assume the following for the purpose of deriving the CQI index, and if also configured, for deriving PMI and RI: - The first 2 OFDM symbols are occupied by control signaling. - The number of PDSCH and DM-RS symbols is equal to 12. - The same bandwidth part subcarrier spacing configured as for the PDSCH reception - The bandwidth as configured for the corresponding CQI report - If a UE is configured with SBFD symbols and a CSI-ReportConfig with the higher layer parameter symbolType indicating SBFD symbols, CQI is generated based on the PRBs in the configured CSI report bandwidth that are both in the active DL BWP and in the DL sub-band(s). - The IAB-MT shall only assume the frequency resources as indicated by the DL TX power adjustment MAC CE, if indicated for the slot of the CSI reference resource by DL Tx Power Adjustment MAC CE as described in [10, TS 38.321]. ## *** Unchanged parts are omitted ***

Reason for change - In the definition of the hypothetical PDSCH used in calculating the CQI report, the frequency resources are determined by the configured bandwidth for CQI reporting. In SBFD slots, the definition of the frequency resources needs to be clarified to correspond to only usable DL PRBs within the configured reporting bandwidth. Summary of change - When UE is configured with CSI report indicating symbolType as SBFD, the CQI is calculated based on the valid DL PRBs in the reporting bandwidth.

## Power control in SBFD symbols

RAN1 agreed to support separate UL power control parameters for SRS/PUCCH/PUSCH in SBFD and non-SBFD symbols based on unified TCI framework where Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols. For the case, when unified TCI is not associated with power control parameters, then the UE should be configured with separate ‘default’ power control parameters per the UL-BWP for SRS, PUCCH and PUSCH transmission in SBFD symbols and non-SBFD symbols.

Agreement For a single TRP scenario, support separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols - Decide in RAN1#118 whether separate UL power control for FR2 is supported based on UL spatial relation Info framework and/or unified TCI state framework. Agreement For a single TRP scenario, for separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols based on unified TCI state framework, - Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols - New P0AlphaSets are introduced in Uplink-powerControl for SBFD symbols for PUSCH, PUCCH and SRS respectively Agreement (#119) The maximum number of power control loops is the same as legacy (i.e. no more than 2) for SBFD operation in Rel-19.

Proposal 10: Support separate default power control parameters for SBFD and non-SBFD symbols configured per UL-BWP when the unified TCI state is not associated with power control parameters.

# SBFD Random access operation

## UE transition period for additional RO across SBFD and non-SBFD symbols

For SBFD-aware UE that need to change some baseband/RF filtering and/or settings between SBFD and non-SBFD symbols, these adaptations need to be accommodated either before the transmission PRACH in the SBFD symbols or afterwards in the non-SBFD symbols, as shown in Figure 3-1. The UE would require some timelines to switch from the SBFD to non-SBFD settings as the UE can’t stop the PRACH transmission in the transition period between SBFD and non-SBFD symbols.

Figure 3-1 SBFD-aware switching from SBFD to non-SBFD state (baseband/RF configurations) before preamble transmission (left) or after preamble transmission (right).

Proposal 11:  For preamble transmission in a valid RO starting in SBFD symbols and ending in non-SBFD symbols, SBFD-aware UE is switching from SBFD to non-SBFD mode within a transition period before or after the RO.

# CLI enhancement

## L1 Inter-UE CLI Measurement and Reporting

### Remove text on the maximum number of SRS-RSRP measurement resource sets

In RAN1 121 meeting, UE feature session agreed that add a component for “Maximum number of L1 SRS-RSRP measurement resources across all CCs” in FG 60-9 and the candidate values are still pending to be discussed. However, there is no RAN 1 agreement to agree on the maximum number of SRS-RSRP measurement resource sets and the maximum number of resources within each set. The maximum number of L1 SRS-RSRP measurement resources should be based on the following agreed UE capability. And RAN1 shall continue to discuss the candidate values for this UE capability.

Similar agreement was agreed for CLI-RSSI measurement resources as well.

Agreement: Add a component for “Maximum number of L1 SRS-RSRP measurement resources across all CCs” in FG 60-9 Agreement: Add a component for “Maximum number of configured L1 CLI-RSSI measurement resources (sum of aperiodic and periodic, and semi-persistent if supported) across all CCs” in FG 60-8

Proposal 12:  Adopt the following TP to remove text on the maximum number of SRS-RSRP measurement resource sets and the maximum number of resources within each set. Also remove text on the maximum number of CLI-RSSI measurement resource sets and the maximum number of resources within each set.

## 5.2.1.4.6 L1-SRS-RSRP Reporting ## *** Unchanged parts are omitted *** For L1-SRS-RSRP computation - the UE may be configured with a CSI resource setting with up to 32 SRS-RSRP measurement resource sets having up to 32 resources within each set. For L1-SRS-RSRP reporting, if the higher layer parameter nrofReportedCLIMeasureResources in CSI-ReportConfig is configured to be one, the reported L1-SRS-RSRP value is defined by a 7-bit value in the range [-140, -44] dBm with 1dB step size, if the higher layer parameter nrofReportedCLIMeasureResources is configured to be larger than one, the UE shall use differential L1-SRS-RSRP based reporting, where the largest measured value of L1-SRS-RSRP is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential L1-SRS-RSRP is quantized to a 4-bit value. The differential L1-SRS-RSRP value is computed with 2 dB step size with a reference to the largest measured L1-SRS-RSRP value which is part of the same L1-SRS-RSRP reporting instance. If one or more of the reported L1-SRS-RSRP values are corresponding to [L1-SRS-RSRP_97] (L1-SRS-RSRP >= -44 dBm) or [L1-SRS-RSRP_98] (Infinity), the UE shall assume the largest measured value of L1-SRS-RSRP as -44dBm for computing the differential L1-SRS-RSRP value(s). The mapping between the reported L1-SRS-RSRP value and the measured quantity is described in [11, TS 38.133]. *** Unchanged parts are omitted *** ## 5.2.1.4.7 L1-CLI-RSSI Reporting ## *** Unchanged parts are omitted *** For L1-CLI-RSSI computation - the UE may be configured with a CSI resource setting with up to 64 CLI-RSSI resource sets having up to 64 resources within each set. For L1-CLI-RSSI reporting, if the higher layer parameter nrofReportedCLIMeasureResources in CSI-ReportConfig is configured to be one, the reported L1-CLI-RSSI value is defined by a 7-bit value in the range [-100, -25] dBm with 1dB step size, if the higher layer parameter nrofReportedCLIMeasureResources is configured to be larger than one, the UE shall use differential L1-CLI-RSSI based reporting, where the largest measured value of L1-CLI-RSSI is quantized to a 7-bit value in the range [-100, -25] dBm with 1dB step size, and the differential L1-CLI-RSSI is quantized to a 4-bit value. The differential L1-CLI-RSSI value is computed with 2 dB step size with a reference to the largest measured L1-CLI-RSSI value which is part of the same L1-CLI-RSSI reporting instance. The mapping between the reported L1-CLI-RSSI value and the measured quantity is described in [11, TS 38.133]. *** Unchanged parts are omitted ***

Reason for change - Per RAN1 agreements, UE feature session agreed that add a component for “Maximum number of L1 SRS-RSRP measurement resources across all CCs” in FG 60-9 and the candidate values are still pending to be discussed. However, there is no RAN 1 agreement to agree on the maximum number of SRS-RSRP measurement resource sets and the maximum number of resources within each set. The maximum number of L1 SRS-RSRP measurement resources should be based on the following agreed UE capability. Similar reason is for L1 CLI-RSSI. Summary of change - Remove text on the maximum number of SRS-RSRP measurement resource sets and the maximum number of resources within each set. Also remove text on the maximum number of CLI-RSSI measurement resource sets and the maximum number of resources within each set.

### Add the assumption for the number of SRS antenna ports

RAN1 agreed that for SRS-RSRP measurement resource configuration, the number of SRS antenna ports is 1.

Agreement For SRS-RSRP measurement resource configuration, the number of SRS antenna ports is 1.

Proposal 13:  Adopt the following TP to add the assumption for SRS-RSRP measurement resource configuration that the number of SRS antenna ports is 1.

#### 5.2.2.6 CLI-RSSI measurement resource *** Unchanged parts are omitted *** For SRS-RSRP measurement resource configuration, the number of SRS antenna ports is 1. *** Unchanged parts are omitted ***

Reason for change - Per RAN1 agreements, for SRS-RSRP measurement resource configuration, the number of SRS antenna ports is 1, which shall be captured in TS 38.214. Summary of change - Add the assumption for SRS-RSRP measurement resource configuration that the number of SRS antenna ports is 1.

# Conclusion

In this contribution, we provided our views on the maintenance and remaining issues for SBFD transmission, reception and measurements procedures with the following TP/proposals:

Proposal 1: Adopt the following TP with change in red to TS 38.213 for handling the collision in half-duplex CA when one or more carrier supporting SBFD operation, where SBFD symbol is treated as flexible symbol.

## 11.1 Slot configuration ## *** Unchanged parts are omitted *** If a UE - is configured with multiple serving cells and is provided with directionalCollisionHandling-r16 = 'enabled' for a set of serving cell(s) among the configured multiple serving cells, and - indicates support of half-DuplexTDD-CA-SameSCS-r16 capability, and - is not configured to monitor PDCCH for detection of DCI format 2_0 on any of the multiple serving cells, the UE determines a reference cell for a symbol as an active cell with the smallest cell index among - the configured multiple serving cells if the UE is not capable of simultaneous transmission and reception as indicated by simultaneousRxTxInterBandCA among the multiple serving cells, and - the cells of each band respectively if the UE is capable of simultaneous transmission and reception by simultaneousRxTxInterBandCA for the configured multiple serving cells, ## where the symbol is configured by higher layers as downlink, or uplink, as indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated - uplink, if the symbol is flexible or SBFD, and the UE is configured by higher layers to transmit SRS, PUCCH, PUSCH, or PRACH on the symbol - downlink, if the symbol is flexible or SBFD, and the UE is configured by higher layers to receive PDCCH, PDSCH or CSI-RS on the symbol. ## *** Unchanged parts are omitted ***

Proposal 2: Adopt the following TP with change in red to TS 38.214 for dynamic determination of the precoding bundling in SBFD symbols when DCI bitfield (bundling size indicator) indicates ‘1’ and first set configured with two values as ‘n4-wideband’ or ‘n2-wideband’.

5.1.2.3 Physical resource block (PRB) bundling ## *** Unchanged parts are omitted *** If the PRB 'bundling size indicator' signalled in DCI format 1_1 as defined in Clause 7.3.1.2.2 of [5, TS 38.212] - is set to '0', the UE shall use the value from the second set of values when receiving PDSCH scheduled by the same DCI. - is set to '1' and one value is configured for the first set of values, the UE shall use this value when receiving PDSCH scheduled by the same DCI - is set to '1' and two values are configured for the first set of values as 'n2-wideband' (corresponding to two values 2 and wideband) or 'n4-wideband' (corresponding to two values 4 and wideband), the UE shall use the value when receiving PDSCH scheduled by the same DCI as follows: - If the scheduled PRBs are contiguous and the size of the scheduled PRBs is larger than , is the same as the scheduled bandwidth, otherwise is set to the remaining configured value of 2 or 4, respectively. If UE configured with SBFD symbols, NBWP,isize is the number PRBs that are both in the active DL BWP and in the DL sub-band(s).

Proposal 3: Adopt the following TP with change in red to TS 38.214 for the multiple PDSCHs scheduled by single DCI format

## 5.1 UE procedure for receiving the physical downlink shared channel ## *** Unchanged parts are omitted *** A UE shall upon detection of a PDCCH with a configured DCI format 1_0, 1_1, 1_2, 1_3, 4_0, 4_1, or 4_2 decode the corresponding PDSCHs as indicated by that DCI. When the UE is scheduled with multiple PDSCHs on a serving cell by a DCI, HARQ process ID indicated by this DCI applies to the first PDSCH not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, HARQ process ID is then incremented by 1 for each subsequent PDSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPDSCH applied if nrofHARQ-ProcessesForPDSCH is provided, or with modulo operation of nrofHARQ-ProcessesForPDSCH-v1700 applied if or nrofHARQ-ProcessesForPDSCH-v1700 is provided, or with modulo of 8 applied, otherwise. HARQ process ID is not incremented for PDSCH(s) not received if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided or invalid symbol type if the UE is configured with SBFD symbols and not configured with [sbfd-Configuration2-Reception]. *** Unchanged parts are omitted ***

Proposal 4: Adopt the following TP with change in red to TS 38.214 for the multiple PUSCHs scheduled by single DCI format.

## 6.1 UE procedure for transmitting the physical uplink shared channel ## *** Unchanged parts are omitted *** When the UE is scheduled with multiple PUSCHs on a serving cell by a DCI, HARQ process ID indicated by this DCI applies to the first PUSCH not overlapping with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, HARQ process ID is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPUSCH applied if nrofHARQ-ProcessesForPUSCH is provided, or with modulo operation of 16 applied, otherwise. HARQ process ID is not incremented for PUSCH(s) not transmitted if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst or invalid symbol type if the UE is configured with SBFD symbols and not configured with [sbfd-Configuration2-Transmission]. *** Unchanged parts are omitted ***

Proposal 5: Adopt the following TP with change in red to TS 38.214 for the valid CG-PUSCHs in a CG period.

## 6.1 UE procedure for transmitting the physical uplink shared channel ## *** Unchanged parts are omitted *** When nrofSlotsInCG-Period is configured for Type 1 configured grant or Type 2 configured grant, HARQ process ID for the first configured PUSCH grant and each subsequent valid configured PUSCH grant within a periodicity of the configuration is determined as in clause 5.4.1 of [10, TS 38.321], where a valid configured PUSCH grant is the one not colliding with the DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, and not colliding with a symbol(s) of an SS/PBCH block with index provided by ssb-PositionsInBurst as described in clause 11.1 of [6, TS 38.213], and not colliding with invalid symbol type if the UE is configured with SBFD symbols and not configured with [sbfd-Configuration2-Transmission]. ## *** Unchanged parts are omitted **

Proposal 6: When UE is not provided semi-statically with valid symbol type for SRS, CSI-ReportConfig and PUCCH resource carrying P/SP CSI or SR, then default assumption is non-SBFD symbol as valid symbol type.

Proposal 7: Adopt the following TP with change in red to TS 38.214 for DMRS bundling where phase coherency is not maintained across SBFD and non-SBFD symbols.

#### 6.1.7 UE procedure for determining time domain windows for bundling DM-RS <omitted text> Events which cause power consistency and phase continuity not to be maintained across PUSCH transmissions of PUSCH repetition type A scheduled by DCI format 0_1, 0_2 or 0_3, or PUSCH repetition Type A with a configured grant, or PUSCH repetition type B or TB processing over multiple slots, or PUCCH transmissions of PUCCH repetition, within the nominal TDW, are: - A downlink slot or downlink reception or downlink monitoring based on tdd-UL-DL-ConfigurationCommon and tdd-UL-DL-ConfigurationDedicated for unpaired spectrum <omitted text> - A transition from SBFD to non-SBFD, or transition from non-SBFD to SBFD symbols <omitted text>

Proposal 8: Adopt the following TP with change in red to TS 38.214 for clarification of the valid DL slot of the

the CSI reference resource.

## 5.2.2.5 CSI reference resource definition ## *** Unchanged parts are omitted *** A slot in a serving cell shall be considered to be a valid downlink slot if: - It comprises at least one higher layer configured downlink or flexible symbol or SBFD symbol, and - it does not fall within a configured measurement gap for that UE ## *** Unchanged parts are omitted ***

Proposal 9: Adopt the following TP with change in red to TS 38.214 for the freq. resources of the hypothetical PDSCH bandwidth used for deriving SBFD CSI metrics, in which the UE shall assume that the CSI reference resource contains only the usable DL PRBs in the CSI report bandwidth.

## 5.2.2.5 CSI reference resource definition ## *** Unchanged parts are omitted *** If configured to report CQI index, in the CSI reference resource, the UE shall assume the following for the purpose of deriving the CQI index, and if also configured, for deriving PMI and RI: - The first 2 OFDM symbols are occupied by control signaling. - The number of PDSCH and DM-RS symbols is equal to 12. - The same bandwidth part subcarrier spacing configured as for the PDSCH reception - The bandwidth as configured for the corresponding CQI report - If a UE is configured with SBFD symbols and a CSI-ReportConfig with the higher layer parameter symbolType indicating SBFD symbols, CQI is generated based on the PRBs in the configured CSI report bandwidth that are both in the active DL BWP and in the DL sub-band(s). ## *** Unchanged parts are omitted ***

Proposal 10: Support separate default power control parameters for SBFD and non-SBFD symbols configured per UL-BWP when the unified TCI state is not associated with power control parameters.

Proposal 11:  For preamble transmission in a valid RO starting in SBFD symbols and ending in non-SBFD symbols, SBFD-aware UE is switching from SBFD to non-SBFD mode within a transition period before or after the RO.

Proposal 12:  Adopt the following TP to remove text on the maximum number of SRS-RSRP measurement resource sets and the maximum number of resources within each set. Also remove text on the maximum number of CLI-RSSI measurement resource sets and the maximum number of resources within each set.

## 5.2.1.4.6 L1-SRS-RSRP Reporting ## *** Unchanged parts are omitted *** For L1-SRS-RSRP computation - the UE may be configured with a CSI resource setting with up to 32 SRS-RSRP measurement resource sets having up to 32 resources within each set. For L1-SRS-RSRP reporting, if the higher layer parameter nrofReportedCLIMeasureResources in CSI-ReportConfig is configured to be one, the reported L1-SRS-RSRP value is defined by a 7-bit value in the range [-140, -44] dBm with 1dB step size, if the higher layer parameter nrofReportedCLIMeasureResources is configured to be larger than one, the UE shall use differential L1-SRS-RSRP based reporting, where the largest measured value of L1-SRS-RSRP is quantized to a 7-bit value in the range [-140, -44] dBm with 1dB step size, and the differential L1-SRS-RSRP is quantized to a 4-bit value. The differential L1-SRS-RSRP value is computed with 2 dB step size with a reference to the largest measured L1-SRS-RSRP value which is part of the same L1-SRS-RSRP reporting instance. If one or more of the reported L1-SRS-RSRP values are corresponding to [L1-SRS-RSRP_97] (L1-SRS-RSRP >= -44 dBm) or [L1-SRS-RSRP_98] (Infinity), the UE shall assume the largest measured value of L1-SRS-RSRP as -44dBm for computing the differential L1-SRS-RSRP value(s). The mapping between the reported L1-SRS-RSRP value and the measured quantity is described in [11, TS 38.133]. *** Unchanged parts are omitted *** ## 5.2.1.4.7 L1-CLI-RSSI Reporting ## *** Unchanged parts are omitted *** For L1-CLI-RSSI computation - the UE may be configured with a CSI resource setting with up to 64 CLI-RSSI resource sets having up to 64 resources within each set. For L1-CLI-RSSI reporting, if the higher layer parameter nrofReportedCLIMeasureResources in CSI-ReportConfig is configured to be one, the reported L1-CLI-RSSI value is defined by a 7-bit value in the range [-100, -25] dBm with 1dB step size, if the higher layer parameter nrofReportedCLIMeasureResources is configured to be larger than one, the UE shall use differential L1-CLI-RSSI based reporting, where the largest measured value of L1-CLI-RSSI is quantized to a 7-bit value in the range [-100, -25] dBm with 1dB step size, and the differential L1-CLI-RSSI is quantized to a 4-bit value. The differential L1-CLI-RSSI value is computed with 2 dB step size with a reference to the largest measured L1-CLI-RSSI value which is part of the same L1-CLI-RSSI reporting instance. The mapping between the reported L1-CLI-RSSI value and the measured quantity is described in [11, TS 38.133]. *** Unchanged parts are omitted ***

Proposal 13:  Adopt the following TP to add the assumption for SRS-RSRP measurement resource configuration that the number of SRS antenna ports is 1.

#### 5.2.2.6 CLI-RSSI measurement resource *** Unchanged parts are omitted *** For SRS-RSRP measurement resource configuration, the number of SRS antenna ports is 1. *** Unchanged parts are omitted ***