3GPP TSG RAN WG#123		R1-2509205

Dallas, USA, Nov 17th – 21st, 2025

Agenda item:	8.7.1

Source: 	Qualcomm Incorporated

Title: 	Maintenance for Rel-19 NR NTN

Document for:	Discussion

# Issue#1: Handling of PTRS (OCC)

The specification impact of OCC on PT-RS has been overlooked in the previous meetings. Although typically used in FR2, RAN1 specifications allow usage of PT-RS regardless of the frequency range and, therefore, may be enabled in FR1 NTN. We believe that RAN1 should not specify the joint operation of PT-RS and OCC due to the following reasons:

- Technically, PT-RS is designed for channels in which the phase changes symbol by symbol, requiring estimation of this phase variation. If the channel phase changes very rapidly across symbols, inter-slot OCC will suffer from loss of orthogonality and, therefore, should not be enabled.

- From specification and implementation complexity point of view, enabling PT-RS (while keeping orthogonality across UEs using OCC) would be challenging, with RAN1 needing to discuss aspects such as whether PT-RS is OCC’d across slots.

Therefore, we make the following proposal:

Proposal 1: If the UE is scheduled to transmit PUSCH with OCC, PT-RS is not present in the PUSCH.

We propose the following TP for capturing this limitation in the specifications.

<TP 1, 38.214>

Reason for change: Current specifications do not clarify PT-RS compatibility with OCC. If we follow the current specifications, the PT-RS between multiple UEs will not be orthogonal.

Summary of change: Clarify that PT-RS is not present when OCC is enabled.

Consequences if not approved: Reduced performance due to non-orthogonality of PT-RS.

### 6.2.3	UE PT-RS transmission procedure

The procedures on PT-RS transmission described in this clause as well as clauses 6.2.3.1 and 6.2.3.2 apply to a UE PUSCH transmission scheduled by DCI format 0_2 if the higher layer parameter phaseTrackingRS in dmrs-UplinkForPUSCH-MappingTypeA-DCI-0-2 or dmrs-UplinkForPUSCH-MappingTypeB-DCI-0-2 is configured, to PUSCH transmissions scheduled by DCI format 0_0, 0_1 or 0_3 if the higher layer parameter phaseTrackingRS in dmrs-UplinkForPUSCH-MappingTypeA or dmrs-UplinkForPUSCH-MappingTypeB is configured and PUSCH transmissions corresponding to a configured grant if the higher layer parameter phaseTrackingRS in cg-DMRS-Configuration is configured. If a UE is not configured with the higher layer parameter phaseTrackingRS in the respective DMRS-UplinkConfig, the UE shall not transmit PT-RS. For a PUSCH transmission with repetition type A and with OCC operation enabled, the UE shall not transmit PT-RS. The PT-RS is only present on PUSCH scheduled by PDCCH with CRC scrambled by MCS-C-RNTI, C-RNTI, CS-RNTI, SP-CSI-RNTI and on PUSCH corresponding to a configured grant. For PUSCH repetition Type B, the PT-RS transmission procedure is applied for each actual repetition separately based on the allocation duration of the actual repetition.

</TP 1, 38.214>

Proposal 2: Endorse TP1 (TS 38.214)

# Issue#2: SIB1 repetition (DL CovEnh)

In previous meetings, the issue of supporting SIB1 repetition in TN was discussed:

- RAN Plenary agreed to support PDCCH repetition for TN, but did not agree on supporting SIB1 PDSCH for TN.

- RAN1 specifications are “coupling” the features of SS0 PDCCH repetition and SIB1 PDSCH.

Separating these two features for TN and NTN will require additional specification changes. Additionally, it may create some ambiguity at the UE in case the UE does not know after decoding MIB whether the cell is TN or NTN. Therefore, we propose to allow SIB1 repetition for TN.

Proposal 3: SIB1 repetition is supported for TN, following the exact same implementation as for NTN.

- No RAN1 specification change is needed.

# Summary of proposals

In this document we identified presented our views on remaining Rel-19 NTN issues. We made the following proposals:

Proposal 1: If the UE is scheduled to transmit PUSCH with OCC, PT-RS is not present in the PUSCH.

Proposal 2: Endorse TP1 (TS 38.214).

Proposal 3: SIB1 repetition is supported for TN, following the exact same implementation as for NTN.

- No RAN1 specification change is needed.