3GPP TSG RAN WG1 #123                                                                  R1-2509299

Dallas, USA, Nov 17th – 21st, 2025

Agenda Item:		11.4.1

Source: 		Google

Title:			Discussion on channel coding for 6GR air interface

Document for: 	Discussion and Decision

# Introduction

In this document, we provide our views on channel coding for 6GR air interface.

# Discussion

Working Assumption - Study 6G data channel coding for higher throughput than 5G with acceptable performance-complexity tradeoff for both NW side and UE side, - Target peak data rate is assumed to be 2 times of the target peak data rate defined in TR38.913 Note: The other target throughput is up to company to report. Note: Applicability of the potential channel code will be further discussed. Agreement - For 6G channel coding, LDPC is used for data (including SIBs) and Polar code is used for L1 control information (larger than 11 bits, including PBCH) - For 6G LDPC - Working assumption: For data rate within NR range, reuse of NR LDPC design is supported - For data rate beyond NR range, study LDPC extension with acceptable performance-complexity tradeoff for both NW side and UE side - Note: Applicability of the potential LDPC extension to data rate within NR range will be further discussed - For 6G Polar code - Working assumption: For control information within NR range (larger than 11 bits), reuse of NR Polar code design is supported - For control information beyond NR range, study Polar code extension with acceptable performance-complexity tradeoff for both NW side and UE side - Note: Necessity for control information beyond NR range is to be further discussed - Polar code maximum mother code length is kept as 1024. - FFS: further motivation(s) for potential extension/enhancement until RAN1#123

In the last meeting, the group has agreed to reuse 5G coding schemes for data rate and control information (e.g., DCI and UCI) sizes within the 5G range, and for 6G Polar code, further study motivations for potential extension enhancement. In previous meetings, companies proposed to have larger downlink control information (DCI) and uplink control information (UCI). According to companies’ contributions, larger control information may need a larger interleaving table for DCI and Polar encoding reliability table for UCI.

For UCI size, the main reason of having a larger size is for the potential larger measurement report size of 6GR MIMO. From our perspective, there are several candidate solutions to handle a larger UCI size. For example, the maximum code blocks can be increased to a number larger than 2. The advantage of this solution is the maximum code block length keeps the same (i.e., 1024), thereby the reliability table of 5GNR polar coding can be reused. In another example, during the 6GR workshop, some companies proposed to deliver UCI through other channels, such as PUSCH or MAC CE. With this solution, a larger UCI payload size has no effect to reuse current channel coding schemes.

Observation 1: For larger UCI size, it is not necessary to change the maximum code block size and reliability table of 5G polar coding scheme.

For the DCI size, the reason of having a larger size comes from release 18 multi-carrier enhancement, where a new DCI format can schedule multiple transmissions on multiple carriers. During the study phase of multi-carrier enhancement, the only benefit identified was to reduce the control channel blocking probability, which is from our perspective not a critical issue in the day-one 6GR. On the other hand, there were other solutions that can achieve the same result (e.g., two-stage DCI), which can be further discussed in downlink control channel agenda item.

Observation 2: The main reason of having a larger DCI size is based on multi-carrier enhancement, where a single DCI can schedule transmissions on multiple carriers. The only benefit for this feature is to reduce control channel blocking probability, which might not be a critical issue in the day-one 6GR.

Proposal 1: RAN1 studies DCI and UCI size enhancement for coding schemes, only if requirements have been identified in 6GR data and control channels agenda items.

# Conclusion

Observation 1: For larger UCI size, it is not necessary to change the maximum code block size and reliability table of 5G polar coding scheme.

Observation 2: The main reason of having a larger DCI size is based on multi-carrier enhancement, where a single DCI can schedule transmissions on multiple carriers. The only benefit for this feature is to reduce control channel blocking probability, which might not be a critical issue in the day-one 6GR.

Proposal 1: RAN1 studies DCI and UCI size enhancement for coding schemes, only if requirements have been identified in 6GR data and control channels agenda items.

# References

- “Draft Report of 3GPP TSG RAN WG1 #122b v15,” Prague, Czech Republic, Oct. 13th-17th, 2025.

- R1-2508012, “ FL summary#3 for 6G channel coding,” Moderator (ZTE, Apple), Prague, Czech Republic, Oct. 13th-17th, 2025.