3GPP TSG RAN WG1 #123	    	R1-2509516

Dallas, USA, 17th-21st November 2025

Agenda item:	   11.6

Source: 	Samsung

Title:	AI/ML Use cases and framework for 6GR

Document for: 	Discussion and Decision

# Introduction

Study item for 6G radio was approved in RP-251881[1], wherein, AI/ML is one of objectives for study:

(8) AI/ML for 6GR and Radio Access NW, leveraging 5G AI/ML framework, as appropriate [See TR38.843] [RAN1, RAN2, RAN3, RAN4] - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc… Coordinated discussion needs to be ensured with related design areas, where needed (e.g., MIMO, Mobility, etc…) NOTE: lead WG depends on the use case. - AI/ML framework: Extensible AI/ML enablers based on the identified Use Case(s), including - LCM procedures [RAN2, RAN1, RAN3, RAN4] - Data collection and data management, in coordination with SA WGs [RAN2, RAN3, RAN1] Note: NW for AI is assumed to be covered by new services 6GR and RAN design shall ensure that the 6G System can also operate without AI/ML

In RAN 1 #122, the following agreement was made Agreement For 6GR AI/ML use cases identification/categorization, for each (sub-)use case proposed, proponent companies are encouraged to study and report the following: - Definition of each (sub-)use case, including at least AI/ML model input/output - The evaluation assumption, methodology, KPIs, benchmark, and preliminary simulation results - Assumption on training types, e.g., - offline training, online training/finetuning - Label construction (if applicable), including whether/how to obtain label data for model training - Assumption on model location for inference, e.g., UE-sided model, NW-sided model, and two-sided model - Collaboration/interaction between UE and NW, e.g., - no collaboration/interaction - UE/Network collaboration targeting at separate or joint ML operation - High level potential specification impact.

In RAN 1 #122bis, RAN 1 further categorized some AI/ML use cases.  For further study. This paper discussed the 6GR AI/ML framework, evaluation methodology, and use cases for 6GR AI/ML for air interface study.

# Analysis on idented AI/ML use cases for 6GR

## CSI prediction and CSI-RS overhead reduction

### Sub-case A: Frequency and/or spatial domain CSI prediction with sparse/low overhead CSI-RS with AI/ML

In RAN#122-bis, as one of AI/ML use cases, CSI prediction and CSI-RS overhead reduction were identified. In this use case, especially, frequency and/or spatial domain CSI prediction with sparse/low overhead CSI-RS is considered as a representative sub-case. In specific, in order to reduce the CSI-RS overhead, a transmission of the sampled/compressed measurement of CSI-RS ports can be considered. Using the channel measurement with sparse/low overhead CSI-RS as an input, the AI model predicts a full channel matrix based on the correlation of spatial and/or frequency channels.

#### Further clarification of sub-case A

To clarify this sub-case A, we consider two key density parameters:

- Frequency-domain density (FD): this parameter represents CSI-RS frequency density of each CSI-RS port among partial M ports per PRB. For example, when M=128 and N=256, an FD of 0.5 means that a certain port among M=128 ports is transmitted on every alternate RB (i.e., FD unit is 2).

- Spatial-domain density (SD): this parameter represents the fraction of M ports among N ports (i.e., M/N) that are measured within FD unit. For example, when FD=0.5, an SD of 0.5 means that only 50% of the total ports are measured on 2 RBs.

Based on the FD and SD parameters, we can categorize three different prediction schemes.

- Frequency domain CSI prediction (FD < 1, SD = 1)
: Total N ports are distributed based on the FD value.

- Spatial domain CSI prediction (FD = 1, SD < 1)
: M ports are transmitted across all RBs.

- Frequency-spatial domain CSI prediction (FD < 1, SD < 1)
: M ports are distributed within the FD unit.

In sub-case A, the AI/ML model acts as a highly non-linear interpolator exploiting the learned frequency and/or spatial channel correlation properties. UE-side model is attractive solution to this end as UE can acquire high-resolution raw channel matrices from CSI-RS measurements. Alternatively, NW-side model can be investigated, however, the prediction performance depends highly on the feedback quality. One advantage of NW-side model is that it is simpler to train site-specific models that are trained to specificity of NW-side additional conditions, e.g., deployment, antenna correlation properties, etc. However, if the codebook (CB)-based precoder feedback is used, it would be difficult to reconstruct high-resolution CSI due to the CB quantization loss and phase ambiguity. Hence, in order to achieve the prediction gain with NW-side model, high resolution explicit channel feedback is required. Conversely, two-sided model based approach can be considered where the UE feedbacks the sparse channel while the network reconstructs the CSI with SD/FD domain up-sampling. This approach can be considered as joint CSI prediction and compression, akin to approach studied in Rel-19 for temporal-domain prediction. As the NW-side model and two-sided model-based solutions require explicit channel feedback, they are separately discussed in Section 2.3 , dedicating this section for UE-side model.

Consider the input, output and label of AI model and model location are captured as followings.

- Model input: Measurement of channel with sparse/low overhead CSI-RS

- Model output: Full channel matrix

- Label: Estimated/ideal channel matrix based on full CSI-RS density

- Model location: UE-side model

In order to obtain the label, transmission of CSI-RS with FD=1 and SD=1 is required.

Observation  1: Based on a combination of FD and SD parameters, categorize the frequency domain prediction, spatial domain prediction, frequency-spatial domain prediction separately.

#### Benefits

This sub-case introduces several significant advantages for the 6GR systems.

- Significant reduction in CSI-RS overhead
: The most direct and significant advantage is the substantial reduction in CSI-RS overhead. As an example, in the spatial-domain prediction, by transmitting only a reduced subset of M ports instead of the full N ports, the number of REs allocated for CSI-RS transmission is decreased by approximately a factor of (N-M)/N. Also, in the frequency-domain prediction, by transmitting only a subset of K RBs instead of the full L RBs, the number of REs is decreased by a factor of (L-K)/L. These REs, which would otherwise be used by reference signals, become available for scheduling PDSCH, leading to a direct increase in data throughput and overall cell spectral efficiency.

- gNB energy saving

: The proposed technique enables significant energy savings at the gNB. In conventional systems, the gNB should power all N antenna ports during the CSI-RS transmission symbols. In contrast, the proposal allows the gNB to selectively activate only the M PAs corresponding to the transmitted ports (especially for the spatial-domain CSI prediction). The PAs for the (N-M) non-transmitted ports can be effectively muted or switched to a low-power state during these specific symbols. Given that PAs are a dominant contributor to the total power consumption of the gNB, this selective deactivation leads to a tangible reduction in overall energy usage.

- Inter-cell interference reduction

: A further crucial benefit is the mitigation of inter-cell interference (ICI). When CSI-RSs are cell-specific, they inevitably act as a source of interference to UEs in neighboring cells. This interference becomes particularly detrimental when adjacent UEs are performing interference measurements (e.g., using CSI-IM) or even receiving PDSCH on the same REs. By reducing the number of transmitted CSI-RS ports from N to M, the proposed technique effectively lowers the total interference power projected into the network on those specific resources.

Observation  2: Sub-case A can achieve a significant reduction in CSI-RS overhead, gNB energy saving, and inter-cell interference reduction

#### Aspects for further study

- CSI-RS port pattern design

An essential consideration for the proposal is the dependency on the specific antenna port selection. The efficacy of the AI-based channel reconstruction is not only a function of the number of transmitted ports (M), but also is fundamentally dependent on which M ports are selected for transmission. The chosen frequency and/or spatial domain pattern under a certain combination of FD and/or SD. For instance, whether they are contiguous, interleaved, or sparsely distributed-directly dictates the quality of spatial information available to the AI model. An improperly selected pattern may fail to capture the essential spatial correlation structure of the channel, leading to high prediction errors.

This challenge is further compounded by the fact that the optimal M-port pattern is highly dependent on the site-specific channel environment and implementation, e.g., antenna layout, TxRu mapping. This site-specific dependency introduces a significant generalization issue for the AI model. Consequently, this presents a fundamental system design trade-off. One approach is to allow full flexibility, where the gNB can dynamically select the M-port pattern that is optimal for the current channel conditions. While this offers the highest potential for performance, it places an enormous generalization burden on the UE’s AI model and requires complex downlink signalling to indicate the active pattern. The alternative is to define a small set of fixed or restricted patterns. This approach simplifies AI model training and signalling but sacrifices the ability to perform site-specific optimization, as the predefined patterns will inevitably be suboptimal in many real-world environments. A thorough study is therefore required to evaluate this trade-off between adaptive optimization flexibility and the robustness and implementation simplicity of pre-defined patterns.

Observation  3: A study is required to evaluate the impact of various CSI-RS patterns.

- Model generalization

A critical aspect for the practical deployment of the proposed AI-based CSI scheme is its generalization capability. This focus would be a direct continuation of the extensive studies initiated from Rel-18. These foundational studies immediately identified generalization (model’s ability to perform reliably in environments and conditions not seen during training) as a paramount challenge for standardization. A single, pre-trained model may function correctly across thousands of cells with unique radio characteristics, and any failure in generalization can lead to severe link performance degradation. Ensuring robustness is therefore essential for interoperability and for minimizing the network management complexity associated with LCM.

Key factors highlighted during Rel-18 discussions included the model’s robustness across diverse deployment scenarios (e.g., UMa, UMi, InH), its performance under varying mobility conditions (e.g., different Doppler spreads), frequency-related factors and its adaptability to different gNB/UE antenna configurations (e.g., TxRU mappings). The new sub-case A inherits all these established challenges. Building upon this foundation, specific to our M-to-N prediction scheme, a new generalization challenge arises from the port selection pattern. The model’s reconstruction quality is critically dependent on which M ports are selected. A study is needed to assess how a model trained on a specific, fixed pattern (e.g., an interleaved pattern) generalizes when the gNB is permitted to flexibly select a different, unseen pattern, which may be optimal for the channel conditions.

Observation  4: Sub-case A inherits the generalization factors studied from Rel-18. Building upon these factors, specific to newly-designed prediction schemes, a new generalization challenge (e.g., port pattern, NW’s antenna correlation) arises for both SD and/or FD predictions.

- Model scalability

A critical dimension of generalization that necessitates dedicated study is model scalability. In this context, scalability refers to the AI model’s ability to maintain performance and accuracy when the dimensionality of its input and output tensors changes. A standardized solution may impractical if it requires a unique, dedicated AI model for every possible configuration of bandwidth, antenna ports, or carrier combination. Therefore, the ability of a single model to flexibly scale to different dimensions is a fundamental requirement for deployment.

Further studies are required to evaluate the model’s robustness against the following key scalability factors:

- Frequency domain scalability (Bandwidth)
: The input and output dimensions of a CSI prediction model are a direct function of the system bandwidth (e.g., number of subbands, RBs, or subcarriers). A UE may be configured with different BWP sizes within the same carrier or operate on carriers with different bandwidths. In study, we need to investigate whether a single AI model can perform inference on variable-sized frequency-domain inputs/outputs. Particularly, the generalization performance would be quantified as the BWP size scales from small to large configurations.

- Spatial domain scalability (Number of antenna ports)
: N is a primary driver of the model’s output dimensionality. Furthermore, for the M-to-N prediction case, the input dimension is also a variable. Especially, a solution could be future-proof, capable of handling not only current configurations (e.g., 32 ports) but also future MIMO systems (e.g., 256 ports). Hence, a thorough evaluation is needed to determine if a model trained for one specific port configuration (e.g., M=64 → N=256) can successfully generalize to another (e.g., M=32 → N=128).

Observation  5: At least, frequency domain scalability and spatial domain scalability are considered for the model scalability.

- Coexistence with non-AI UEs

Ensuring co-existence between AI-capable UEs and legacy (non-AI) UEs within the same cell is needed for the practical deployment of the proposed AI-based framework. The sub-case A relies on transmitting a reduced CSI-RS ports under a combination of FD and SD parameters. In case of frequency domain CSI prediction (i.e., FD<1 and SD=1), legacy UEs not equipped with the AI/ML models can generate the CSI of full N-port channel based on CSI support with low density while AI-capable UEs infer the full N-port channel. However, in case of spatial domain and frequency-spatial domain CSI predictions (i.e., UE receives M (<N) ports), legacy UE has no standardized reporting mechanism for M-port subset. To maintain backward compatibility and support co-existence, a new reporting mechanism-such as dedicated M-port codebook-would need to be studied for legacy UEs.

#### Spec impact

For the inference, a configuration for the port compression pattern (or matrix) is required. This configuration may vary depending on the port compression methods such as port subsampling or port combining. Particularly, in these configurations, the NW’s proprietary implementations (e.g., antenna panel structure, antenna implementation) might need to be considered. For this purpose, the Associated ID which is adopted in Rel-19 AI/ML BM to ensure consistency on UE’s assumption on measured and predicted beams can be repurposed. That is, with the same associated ID for data collection and inference, the UE assumes the same spatial domain correlation for measured ports and the reported ports.

Proposal 1: For CSI prediction and CSI-RS overhead reduction study,

- Use case

  - Depending on FD and SD parameters, categorize frequency domain prediction, spatial domain prediction, and frequency-spatial domain prediction separately

  - Input: Measurement of channel with sparse/low overhead CSI-RS

  - Output: Full channel matrix

  - Label: Estimated/ideal channel matrix based on full CSI-RS density

  - Model location: UE-side model

- Evaluation assumption:

  - KPI: SGCS, Throughput, Ratio of CSI-RS overhead

  - Benchmark: Genie-aided approach, traditional channel interpolation algorithm

- Aspects for further study

  - Port pattern design considering the trade-off between the performance, overhead and complexity

  - Generalization for deployment scenarios, antenna configurations, and port patterns

  - Scalability for frequency domain factor (e.g., bandwidth) and spatial domain factor (e.g., number of antenna ports)

  - Coexist with non-AI UEs

- Specification impact: inference configuration, reporting content, signaling and procedure for LCM

### Sub-case B: Temporal-domain CSI prediction

#### Benefits

- CSI aging handling: CSI aging is one of the key challenges in mobile communication systems. Addressing CSI aging becomes increasingly critical in the 6GR due to an ever-growing number of antennas (hence antenna ports) at the base station, e.g., FR3. To be specific, CSI tends to age more rapidly when it corresponds to narrow beams, which are the consequences of high-dimensional precoding in systems with a large number of antennas. Therefore, to fully exploit the beamforming gains offered by a large antenna array, it is crucial to address the issue of CSI aging. In this regard, CSI prediction plays a crucial role. In NR [2], UE-side model-based CSI prediction is observed to provide up to 23% and 26% mean and 5%tile UPT gain, respectively, against non-predictive CSI feedback in moderate UE speed.

- CSI-RS overhead reduction: CSI prediction could also play a role in reducing overhead for CSI measurement and reporting in the low-mobility regimes. The UE may report predicted CSI for finer time granularity as compared to the CSI-RS measurement. This aspect can be modeled with the following setup where the UE performs infrequent CSI-RS measurement, e.g., T2=20ms, and report CSI for finer time granularity, e.g., T1=5ms. In this setup, the CSI-RS overhead saving can be parameterized by the ratio T1/T2.

Figure 1 Temporal-domain CSI prediction for CSI-RS overhead reduction

#### Aspects for further study

The 5G NR supported UE-side model-based CSI prediction. The specification support encompasses aspects such data collection, performance monitoring as well as AI/ML processing unit and timeline management while taking measurement and reporting framework defined in the Rel-18 codebook enhancement for predicted CSI reporting. In particular, the feature is supported 32 ports as maximum. 6GR’s specification support may extend this to higher number of ports. As the aforementioned performance benefits naturally transcend to the 6GR, if not amplified, we propose UE-side model-based time-domain CSI prediction to be supported too. NR discussed the necessity of network’s support to ensure consistency between training and inference with respect to network-side additional conditions that are not explicitly configurable, e.g., antenna layout. This discussion failed to achieve consensus.  In 6GR, considering the possibilities of even more diverse NW’s implementations, further study is needed. The following may summarize the various aspects that may be studied for temporal-domain CSI prediction. This section focuses on UE-side model-based approach. As outlined in the previous sub-sections, NW-side and two-sided model based approaches are subject to the availability of high resolution CSI feedback for explicit channel matrices.

- Extension to more than 32 ports

- Configurations for CSI-RS overhead reduction

- Handling network-side additional conditions, e.g., antenna layout, deployment scenario, site-specific models.

Proposal 2: For sub-case B, i.e., temporal-domain CSI prediction

- Use case

  - Input: channel matrices from measurements and/or predictions earlier than the CSI report

  - Output: channel matrices for time slots later than the CSI report

  - Label: ground-truth channel matrices for time slots later than the CSI report

  - Model location: UE-side model

- Evaluation assumption:

  - KPI: SGCS, Throughput, Ratio of CSI-RS overhead

  - Benchmark: Non-predictive CSI report (sample-and-hold)

- Aspects for further study

  - NR studies can be reconsidered

  - Extensions and enhancement from NR study including extension to more than 32 ports, configurations for CSI-RS overhead reduction, network-side additional conditions handling, e.g., antenna layout, deployment scenario, site-specific models

- Specification impact: measurement and reporting configurations, signaling and procedure for LCM

### Sub-case C: CSI prediction cross carrier/band/frequency band

#### Further clarification of sub-case C

To properly study the AI/ML-based cross-frequency prediction, it is essential to first clarify the applicable use cases. The feasibility, performance, and complexity of an AI model are all fundamentally dependent on the granularity of the CSI being predicted and the frequency separation between the source and target carriers. A strong hypothesis is that the achievable CSI granularity is inversely related to the frequency separation. This creates a spectrum of potential use cases that would be studied and defined. For example, if the goal is to predict fine-grained and frequency-selective CSI (e.g., per-RB channel coefficients), the model relies on the high channel correlation found in small-scale fading. This correlation degrades rapidly with frequency separation. Therefore, this use case might be only feasible for scenarios with a small separation, such as intra-band CA or prediction between adjacent frequency blocks.

In sub-case C, the AI/ML model acts as a highly non-linear extrapolator exploiting the learned frequency and/or spatial channel properties. As we analyzed the model location in 2.1.1.1, UE-sided model is attractive solution since UE can acquire high-resolution raw channel matrix from CSI-RS. As outlined in Section 2.1.1.1, in case of NW-side and two-sided model-based scheme, further study such as explicit channel feedback may be needed.

Consider the input, output and label of AI model and model location are captured as followings.

- Model input: Channel matrix of carrier/band/frequency block A

- Model output: Channel matrix of carrier/band/frequency block B

- Label: Channel matrix of carrier/band/frequency block B

- Model location: UE-side model

In order to obtain the label, transmission of CSI-RS at target carrier/band/frequency block B is required.

#### Benefits

- Fast CSI acquisition

This sub-case facilitates drastically faster CSI acquisition and highly flexible carrier management. Conventionally, acquiring CSI for an SCell requires the gNB to transmit CSI-RS, which the UE should measure, process, and report, incurring significant CSI acquisition latency. The proposed sub-case bypasses this entire physical measurement loop. The UE can perform an instantaneous inference using the already-available source channel (e.g., from the PCell), enabling the gNB to receive CSI for the target band proactively. This capability allows for low-latency SCell activation and real-time scheduling decisions.

- Enhancement of UE power efficiency

By removing the need to measure the target carrier, the UE is no longer required to power on and operate the RF front-end and receiver chain corresponding to that specific band (e.g., the SCell radio). The UE can remain in a low-power state for that carrier, relying solely on the active PCell radio for both its own operation and the channel prediction of other carriers. This drastically reduces the power consumption associated with multi-carrier monitoring, offering substantial battery life extension for UEs operating in CA.

- CSI-RS overhead reduction and network energy saving

The scheme enables the complete elimination of CSI-RS overhead on the target carrier/frequency band. Unlike conventional methods that merely reduce overhead, this approach obviates the need for any CSI-RS transmission on the target band. This zero-overhead operation allows all available REs on the target carrier to be allocated for PDSCH, maximizing its spectral efficiency and data throughput. This is particularly advantageous for supplemental data carriers or in opportunistic spectrum access scenarios.

Observation  6: CSI prediction for cross carrier/band/frequency band can achieve fast CSI acquisition, CSI-RS overhead reduction, and enhancement of UE power efficiency.

#### Aspects for further study

1. Model generalization

Generalization failure in this sub-case C occurs when a model, trained to learn the relationship between the source frequency block and the target frequency block in one environment, fails to predict accurately in a new, unseen environment. Therefore, further study is required to validate the model’s robustness against a comprehensive set of key generalization factors. This evaluation should reuse the established factors from Rel-18 and be expanded to include new factors specific to this sub-case.

The generalization performance would first be re-validated against the baseline established factors. This includes ensuring robustness across all 3GPP TR 38.901 channel scenarios (e.g., UMa, UMi, InH) and LoS/NLoS conditions, especially when the model is trained on only a subset of them. Furthermore, its performance needs to be confirmed across different antenna configurations and UE distributions (e.g., indoor and outdoor).

Beyond these, new sub-case-specific factors can be considered. For the cross-carrier scheme, the model’s accuracy can be evaluated as the frequency separation between the source and target carriers increases, testing its limits from intra-band to inter-band operation. Also, the model can be confirmed to generalize to different subcarrier spacing (SCS) for each carrier frequency, which is especially critical in cross-carrier cases where the source and target carriers may use different numerologies.

2. Model scalability

Scalability for this sub-case C refers to the model’s ability to handle variations in the dimensional parameters such as the bandwidth and frequency separation without requiring a different model for each specific combination.

- Bandwidth

The input and output tensor dimensions in the frequency domain are directly tied to the bandwidth of the source frequency block and the target frequency block, which can vary. Thus, we need to verify that a single model can flexibly handle different BWP/carrier bandwidth combinations.

- Frequency gap

The physical channel correlation between two frequency bands is known to degrade as the frequency separation increases. An AI/ML model trained for a small-gap, intra-band CA scenario (e.g., gap=50 MHz) will likely fail when applied to a large-gap, inter-band scenario (e.g., gap=1 GHz). A thorough study is required to characterize the model’s prediction accuracy as a function of frequency gap to define the effective operational range of a single AI model.

3. Coexistence with non-AI UEs

The co-existence strategy for the cross-frequency prediction scheme is significantly more straightforward than for the spatial-domain (M-to-N) prediction scheme. In a specific, the cross-frequency scheme enables a simple, differentiated operation in a mixed-UE environment. For non-AI UE, the sNB continues to transmit the standard CSI-RS on the target frequency block. These UEs operate exactly as they do today, performing measurements and reporting CSI based on existing, standardized procedures. Backward compatibility is fully maintained. For AI-capable UE, the sNB can configure these UEs to not measure the CSI-RS of target frequency block. Instead, these UEs rely on their AI models for prediction.

#### Spec impact

For the inference, a configuration for the target frequency band for cross frequency CSI prediction is required. This configuration may vary depending on the sub use cases (e.g., inter-CC, inter-BWP, or intra-BWP) and the desired granularity of the predicted channel in the frequency domain. Particularly, a resource link is required between the channel measurement used as the AI input and the predicted channel generated as the AI output. That is, the UE is instructed to measure the channel of a specific frequency band, which is then used to predict the channel of a target frequency band.

Observation  7: For UE-side cross-frequency CSI prediction, the following specification impacts is identified

- Configuration of frequency location for the prediction, e.g., carrier, BWP, subband

- Configuration to link channel measurement resources and resources for prediction

Proposal 3: For CSI prediction and CSI-RS overhead reduction study,

- Use case

  - Input: Channel matrix of carrier/band/frequency block A

  - Output: Channel matrix of carrier/band/frequency block B

  - Label: Channel matrix of carrier/band/frequency block B

  - Model location: UE-side model

- Evaluation assumption:

  - KPI: SGCS, NMSE, Throughput

  - Benchmark: Ground-truth of target frequency block, sample and hold

- Aspects for further study

  - Generalization for channel scenarios, UE distribution, and subcarrier spacing

  - Scalability for bandwidth and frequency gap between the source and target frequency bands

  - Coexist with non-AI UEs

- Specification impact: Inference configuration, signaling and procedure for LCM

### Sub-case D: CSI prediction across analog beams

#### Benefits

- CSI-RS overhead reduction
The CSI-RS overhead reduction can be also extended to massive MIMO systems with hybrid beamforming, where CSI-RS overhead increases with the number of analog beams used by BS. To mitigate the increased CSI-RS overhead, CSI-RS can be transmitted through a reduced number of measurement analog beams and CSI for the data beams, which are used for PDSCH transmission, can be predicted using AI/ML-based processing (Figure 2). In this case, the AI/ML model for CSI prediction is located at the UE side. Comparing to the conventional scheme, where CSI-RS is transmitted through every analog data beam, the proposed CSI prediction across analog beams eliminates excessive CSI-RS transmissions, increasing the number of REs available for PDSCH transmission.

Figure 2: CSI interpolation for different analog beams using AI/ML in sub-case D

- Fast CSI acquisition (i.e., latency reduction)
Typically, CSI-RS transmission with multiple analog beams is achieved via time-domain beam switching, requiring multiple time resources. The proposed scheme reduces time-domain resource usage by transmitting CSI-RS over fewer analog measurement beams than in the conventional approach, thereby accelerating CSI acquisition for the UE in sub-case D.

#### Aspects for further study

In sub-case D, the AI/ML model relies on CSI obtained from measurement beam(s), requiring alignment between the AI/ML model and the actual measurement analog beamforming. Two main approaches can achieve this alignment. The first is to design a generalized AI/ML model capable of operating across a wide range of measurement beamforming weights. The second approach is to train a specific AI/ML model with side information about the actual measurement beamforming weights. However, this approach introduces challenges for model deployment to UEs and potential model updates if the measurement beamforming weights change over time or different BS vendors.

Another aspect of sub-case D involves support of CSI-RS transmission for non-AI/ML capable UEs. One possible solution is to design more general CSI reporting scheme (e.g. based on downloadable codebook) capable of CSI compression for CSI obtained from the measurement beam(s) and use AI/ML deployed at the BS to predict CSI for data beam(s).

This section presents SLS evaluation results for AI-based CSI prediction across analog beams. In the evaluation, the set of analog beams is divided into a measurement beam and six data beams. CSI obtained for the measurement beam is then used by the AI/ML model to predict CSI for the data beams. The data analog beams span the range of azimuth angles from -30° to +30° and zenith angles from 90° to 95°. The results are given in Table 1 for a hybrid MIMO antenna system with 1024 antenna elements (32×16×2) and 64 digital ports (4×8×2). To better demonstrate an impact of the CSI prediction accuracy, the actual CSI-RS overhead is not included in the results.

Table 1 The performance of the AI-based channel prediction across analog beams (SLS evaluation)

| Scenario | UE SE Average, bps/Hz | UE SE 5%-tile, bps/Hz |
| --- | --- | --- |
| MU-MIMO 64 ports, single CRI | 1.2256 (0%) | 0.3969 (0%) |
| MU-MIMO 64 ports, Multi-CRI, non-AI | 1.7698 (44.4%) | 0.4817 (21.4%) |
| MU-MIMO 64 ports, AI-based Multi-CRI | 1.6504 (34.7%) | 0.4745 (19.6%) |

The results demonstrate the gain of hybrid massive MIMO systems with multi-CRI report over the system with a single CRI (i.e., the reporting of the best beam). The scheme with AI-based CSI prediction shows only a small degradation over the conventional (non-AI) scheme while providing significant CSI-RS overhead reduction because, actually, only one CSI-RS is transmitted (for the measurement beam) instead of six (for the data beams).

Observation 8: The proposed AI-based channel interpolation reduces CSI-RS overhead in massive MIMO systems employing hybrid beamforming, while achieving channel prediction performance comparable to the conventional scheme without AI and CSI-RS transmission for every data analog beam.

Proposal 4: For CSI prediction and CSI-RS overhead reduction study,

- Sub-use case D: CSI prediction across analog beams

  - Input: Channel matrix of Set B of beams (measurement beams), target data beam index (optional)

  - Output: Channel matrix of Set A of beams (data beams)

  - Label: Channel matrix of Set A of beams

  - Model location: UE (DL), BS (UL, for SRS overhead reduction)

- Evaluation assumption:

  - KPI: SGCS, Throughput (or SE), Ratio of CSI-RS overhead

  - Benchmark: Ground-truth of Set A of beams

- Aspects for further study

  - Generalization for channel scenarios, antenna configurations, and port patterns

  - Coexist with non-AI UEs

- Specification impact: Inference configuration, signaling and procedure for LCM

### Summary on the CSI prediction and CSI-RS overhead reduction

This section focuses on UE-side model-based solutions. As outlined in the previous sub-sections, NW-side and two-sided model based approaches are possible for all four sub-cases subject to the availability of high resolution CSI feedback for explicit channel matrices.

Observation 9: The feasibility of NW-side CSI prediction for all four sub-cases of CSI prediction is subject to the availability of high-resolution CSI feedback for explicit channel matrices.

With the above analysis, the sub-cases can be summarized as below:

Table 2 Further information for CSI prediction and CSI RS overhead reduction.

| Sub-use case |  | Sub-Case A: Frequency and/or spatial domain CSI prediction with sparse/low overhead CSI-RS with AI/ML | Sub-Case B: CSI time domain prediction (as Rel-19 CSI prediction or extension) | Sub-case C: CSI prediction cross carrier/band/frequency band | Sub-Case D: CSI prediction across analog beams |
| --- | --- | --- | --- | --- | --- |
| KPI |  | SGCS, Throughput, Ratio of CSI-RS | SGCS, Throughput, Ratio of CSI-RS | SGCS, Throughput, | SGCS, Throughput, Ratio of CSI-RS |
| Benchmark |  | Genie-aided approach, traditional channel interpolation algorithm | Non-predictive CSI report (sample-and-hold) | Ground-truth of target frequency block, sample and hold | Ground-truth of Set A of beams |
| Model location |  | UE-side model | UE-side model | UE-side model | UE-side model |
| Benifit | CSI-RS overhead reduction |  |  |  |  |
|  | NW energy saving |  |  |  |  |
|  | Inter-cell interreference reduction |  |  |  |  |
|  | CSI aging handling |  |  |  |  |
|  | Fast CSI acquisition |  |  |  |  |
| Aspects for further study | Generalization | Deployment scenarios, antenna configurations |  |  |  |
|  |  | CSI-RS patterns, e.g., port sampling pattern, frequency-domain mapping. |  | Triplets of {measurement, gap, prediction} bandwidth. |  |
|  | Coexist with non-AI UE | Y | NA | NA | Y |
|  | Scalability | Frequency domain factor (e.g., bandwidth) and spatial domain factor (e.g., number of antenna ports) | extension to >32 port | Frequency domain factor (e.g., bandwidth) and spatial domain factor (e.g., number of antenna ports) | Frequency domain factor (e.g., bandwidth) and spatial domain factor (e.g., number of antenna ports) |
| Spec impact (Other than LCM related) |  | CSI-RS pattern design | As per NR AI/ML CSI prediction. | Configuration of measurement and prediction frequency units | Configuration of measurement |

## DMRS design with AI receiver

In RAN1#122bis, following observation is drawn on DMRS design with AI receiver.

Observation For 6GR AI/ML use cases identification/categorization, [23 sources] provided preliminary simulation results and analysis on low overhead DMRS with AI/ML receiver. - [22 sources] provided preliminary simulation results and analysis on sparse orthogonal DMRS in frequency and/or time domain with AI/ML receiver. - [11 sources] provided preliminary simulation results and analysis on superimposed pilot with AI/ML receiver. - [5 sources] provided preliminary simulation results and analysis on DMRS free with AI/ML receiver. - Detailed evaluation assumptions (model input/output/label/benchmark/KPI/ training type) and initial analysis can be found in Table C. Note: whether/how to capture the observation in the TR is a separate discussion.

### Clarification on use cases

As discussed in RAN1#122bis, DMRS design with AI receiver in 6GR mainly consists of three sub-cases:

1) Sub-case A: Sparse orthogonal DMRS in frequency and/or time domain;

2) Sub-case B: Superimposed pilot;

3) Sub-case C: DMRS free.

Clarification on receiver assumption

Based on companies’ input in RAN1#122b, for each use-case, different receiver types (corresponding to different model inputs/outputs/labels) are used, summarized in the following table. The procedure to obtain different types of labels, and the consideration aspects (e.g., generalization, scalability) are quite different for each receiver type. Therefore, some additional clarification on the receiver assumptions is needed to facilitate further study.

Table 3 Clarification on receiver assumptions for each sub-use case.

| Receiver types | Sub-case A: Sparse orthogonal DMRS | Sub-case B: Superimposed pilot | Sub-case C: DMRS free |
| --- | --- | --- | --- |
| Alt 1: AI-based denoise (on DMRS REs only) | Yes | - | - |
| Alt 2: AI-based channel estimation (for DMRS and data REs) | Yes | - | - |
| Alt 3: AI-based joint {channel estimation +equalization + demodulation} | Yes | Yes | Yes |
| Alt 4: AI-based channel estimation + AI-based joint {equalization + demodulation} (Two AI/ML models) | [TBD] | Yes | [Yes] |

Clarification on label acquisition

As summarized in [2], for Alt 2 and Alt 4, majority companies used ideal channel information as label. In practice, it is not feasible to obtain the ideal channel. Therefore, the evaluation result under this assumption is not meaningful. If those receiver types are considered for further study, ideal channel information should not be considered as label. Instead, the estimated channel at high SNR (e.g., 30dB) shall be considered for labeling.

Observation 10: Ideal channel information is not acquirable in practical, and the simulation result with ideal channel information as label cannot justified the performance gain.

### KPI and study aspects

#### Benchmark and KPIs

Benchmark

There are two aspects for the benchmark—DMRS pattern and channel estimation method.

- DMRS pattern: A certain DMRS pattern/overhead can be selected for baseline. For example, Rel-15 DMRS type 1 can be used as the DMRS pattern benchmark, which was widely assumed in the submitted results. Additional DM-RS position can be considered for certain scenario, for example for high Doppler scenario.

- Non-AI Channel estimation method: Ideal channel estimation and/or conventional channel estimation are used. The complexity of the channel estimation method for benchmark is better to have comparable complexity. At least  LS and linear interpolation shall not be considered for benchmark.

Besides, there are several ways to compare the performance for corresponding KPIs:

- Normal DMRS overhead + Non-AI CE

- Low overhead DMRS+ Non-AI CE

KPI

As summarized in [2], BLER, throughput, and MSE (when applicable) are used as KPIs in the reported results by each company. Besides, when evaluating the performance, there are some other aspects to look into.

- Model complexity and inference latency. The complexity for a given model is computed based on a fixed number of RBs. When the scheduled number of RB is larger than the input size, the overall complexity might be scaled with the number of RB since multiple times of inference are needed. The high complexity also introduces inference latency, and will cause longer time for HARQ-ACK feedback, thus, results in UPT loss.

- DMRS Overhead. DMRS overhead refers to the amount of time-frequency or power domain resources allocated for transmitting the reference signals

- Overhead for fine-tuning, when applicable. Fine-tuning overhead is needed when the receiver adapts online for specific environmental changes.

- Inter-vendor collaboration overhead and complexity for two-sided model, when applicable. When the communication system involves equipment from different vendors, the cost and difficulty of coordinating and integrating their respective AI models and interfaces must be considered.

#### Aspects for further study

Label quality impact

As discussed in previous section, ideal channel information is not practical and shall not be considered as label. Instead, estimated channel information with non-AI scheme can be used as label. However, the quality of the label will have impact on the performance. Some investigation on the label quality impact is needed.  Besides, this approach may introduce several potential challenges in practical deployment:

- Label quality and practical feasibility. While using conventional estimates at high SNR is more realistic than using an ideal channel, acquiring high-quality labels in real-world deployment remains challenges.

  - SNR Dependence: The quality of the label is directly dependent on the SNR conditions under which the conventional algorithm generates it. In practical networks, it is not guaranteed that all UEs for data collection are in high SNR conditions.

  - Cell Edge UE Scenarios: It is challenge for cell-edge UEs to obtain high quality label since they suffer from low-SNR and high-interference. The performance loss when model is trained with lower-quality label is not clear. Also, whether the model trained under high-quality label (e.g., obtained from cell-center UE) can be applied directly for cell edge UE is unclear and worthy of studying.

The results in Annex C shows the impact of label quality. There is performance degradation when using 30dB estimated labels compared with ideal channel information as label. This degradation is minimal in benign channels (CDLC300-11) but becomes more pronounced (Patten 3, @30% BLER, up to 3dB NMSE loss; up to 0.8dB BLER loss) in challenging channels (CDLC300-667). For Alt2 receiver, even that using 30dB estimated labels introduces degradation compared to ideal labels, it still outperforms the legacy receiver. A key finding is that the low density DMRS Pattern 3 combined with the Alt2 receiver can match or even surpass the performance of the traditional DMRS Pattern 1 with a legacy receiver. This validates the feasibility of deploying AI/ML solutions to enhance spectral efficiency with practical label.

DMRS pattern for Sub-case A: Sparse DMRS

The majority of contributions use the Release 15 DMRS Type 1 pattern as the benchmark, e.g., single-symbol DMRS with one additional position to support basic channel estimation needs. To systematically evaluate the robustness of AI receivers when handling sparse DMRS under various practical scenarios, several distinct DMRS pattern designs and corresponding receiver architectures have been proposed.

Different patterns have been assumed in the evaluations, for example,

- Frequency domain:

  - Intra-PRB: comb-like, localized CDM (one or less CDM group per PRB), DMRS with two adjacent REs

  - Inter-PRB: additional sparsity inter-PRB level

- Time domain

  - Double symbols

  - Symbol-level sparse

Different DMRS pattern may benefit for or required by different receiver scheme. Further study is needed.

DMRS sequence/pattern design, power allocation for Sub-case B: Superimposed DMRS

For Sub-Case B: superimposed DMRS, DMRS and data are overlapped. Several aspects need to be determinate for DMRS design:

- DMRS sequence: DMRS sequence may have impact on the orthogonality between different antenna ports, and also the orthogonality to data.

- DMRS pattern: Companies also provided result for FDMed DMRS for different antenna ports. Therefore, DMRS pattern for multiple antenna ports needs to be studied.

- Allocated power: The percentage of allocated power may have impact on the performance.

Besides the above aspects, DMRS design shall consider the potential impact on PAPR when overlap to each other, and/or overlap with data.

Scalability

The scalability of AI models is a critical area of research. If a model exhibits poor scalability, different models may need to be deployed to handle varying scenarios (e.g., different bandwidths or transmission configurations), increasing system complexity. This analysis examines model scalability across several key dimensions for the proposed alternatives (Alt 1-4, as clarified in previous section) and sub-cases (A-C).

- Frequency domain scalability: This addresses whether the AI model can process different numbers of scheduled RBs.

  - Alt1(Denoise on DMRS):  This approach inherently possesses good frequency domain scalability. The denoising operation typically relies on adjacent REs (e.g., within the same PRB) to derive the denoising result on the DMRS REs, so that the model can be trained per RB. This localized processing makes the algorithm agnostic to the total number of RBs scheduled across the bandwidth.

  - Alt 2(CE for all REs)/Alt3(Equalization)/Alt4(Alt 2+Alt3): [or ] Scalability is more complex for models that take a fixed-size frequency-domain input.

    - Handling Large Inputs: If the scheduled number of RBs is larger than the model's fixed input size, the input data must be segmented. Each segment is processed independently by the model.

    - Handling Small Inputs: If the last segment is smaller than the required input size, whether the unmatched model can be used or not needs further investigated.

    - This may or may not be an issue. For example, sliding window can be used. However, this may be an issue if AI receiver also covers decoding function.

- Time domain scalability: This addresses whether the model can handle different numbers of symbols within a time resource, such as variations in the Time Domain Resource Assignment (TDRA).

  - Alt1(Denoise on DMRS):  This solution only concerns DMRS symbols. The handling on time domain resource is based on legacy.

  - Alt 2(CE for all REs): [or ]  Scalability maybe achievable if the assigned number of symbols is less than or equal to the model's maximum input/output capacity. The UE can perform channel estimation across all relevant symbols and simply extract the results corresponding to the scheduled data symbols.

  - Alt3(Equalization)/Alt4(Alt 2+Alt3):  These alternatives typically learn a complex mapping relationship between the input time-frequency grid (including DMRS) and the entire TB. Variations in the TDRA (e.g., changing the number of data symbols or DMRS positions) fundamentally alter this mapping relationship. Therefore, these solutions generally lack scalability across different TDRA configurations and require different models for each specific assignment pattern.

- Spatial Domain Scalability (Layers/Antenna Ports/Precoders): This addresses the model’s ability to handle varying numbers of transmitted layers.

  - Sub-case A:  Due to the preserved orthogonality between DMRS across different layers (e.g., via CDM), the AI model can learn to estimate the channel information for each layer independently. This design offers good scalability with respect to the number of layers.

  - Sub-case B/C:  The DMRS orthogonality does not exist, requiring the model to process multi-layer data collectively. The AI model is designed to inherently learn the complex inter-layer relationships and interference patterns. Therefore, the solution lacks scalability regarding the number of layers; a model trained for two layers may not perform optimally for four layers without retraining/fine-tuning. Moreover, non-ideal precoder introduces cross-layer interference. Current research has not sufficiently addressed whether the model remains robust when the precoder changes.

- Modulation Order Scalability: This addresses whether the model is sensitive to changes in the Modulation and Coding Scheme (MCS) configuration.

  - Alt 1(Denoise on DMRS)/Alt2:(CE for all REs):   Demodulation goes after the AI model's output (denoised DMRS or channel estimate). The model is agnostic to the data modulation scheme, providing good scalability for different modulation orders (e.g., QPSK, 16QAM, 64QAM).

  - Alt3(Equalization)/Alt4(Alt 2+Alt3): Scalability depends on the model's output format:

    - Constellation points or modulated symbols:  The solution is scalable to different modulation orders.

    - Soft bits or LLR:  The definition and number of output bits per symbol changes with the modulation order (e.g., 2 bits for QPSK, 4 for 16QAM). This fundamental change in the output structure means the model lacks scalability; a model trained for QPSK LLR output cannot process 64QAM LLR output without significant architectural modification or retraining/fine-tuning.

Observation 11: The scalability of sub-cases and alts is summarized in Table 4.

- Sub-case A in general shows better scalability in the following aspect: frequency domain, time domain, spatial domain, modulation order, and RS pattern.

  - Alt 1 shows good scalability in all the aspects.

- Sub-case B and sub-case C lack scalability in the following aspect: frequency domain, time domain, spatial domain, modulation order, and RS pattern.

Table 4 Summary of scalability of sub-cases and receiver assumption.

| Sub-cases/ Receiver assumption |  | Frequency domain | Time domain | Spatial Domain (Layer) | Modulation Order |
| --- | --- | --- | --- | --- | --- |
| Sub-case A: Sparse orthogonal DMRS | - Alt1 |  |  |  |  |
|  | - Alt 2 | [or ] | [or ] |  |  |
|  | - Alt 3 | [or ] |  |  |  |
| Sub-case B: Superimposed pilot | - Alt 3 | [or ] |  |  |  |
|  | - Alt 4 | [or ] |  |  | / |
| Sub-case C: DMRS free | - Alt 3 | [or ] |  |  |  |

Performance impact considering non-ideal factors

There are some non-ideal factors in the practical communication system. Besides TO/FO and the interference, some other non-ideal factor may also be assumed when performing evaluation, such as phase noise, IQ imbalance, ADC quantization noise.

Sub-case B and C does not confine with the legacy DMRS framework. Some legacy handling method of non-ideal factor may not work under these sub-cases. For example, when handling TO/FO, Sub-case B and Sub-case C are incompatible with legacy TO/FO estimation methods, while Sub-case A maintains compatibility due to the use of orthogonal DMRS.

Observation 12: Sub-case B and sub-case C fail to work with legacy TO/FO estimation method.

- Sub-case A: with orthogonal DMRS, TO/FO can be estimated (with legacy method).

- Sub-case B: with non-orthogonal DMRS, legacy TO/FO estimation method does not work

- Sub-case C: with no DMRS, legacy TO/FO estimation method does not work

The preliminary results in [1] shows the performance degradation of sub-case A when considering some non-ideal factors. When considering the TO/FO and interference, both AI and non-AI performance degraded, AI still outperforms non-AI. But for Sub-case B and C, the affects from non-ideal factors are not thoroughly analyzed.

Generalization

#1 Performance under different channel

It is crucial to thoroughly evaluate and exploit the generalization performance of the proposed AI receiver models across various channel conditions. Communication channels vary significantly based on the environment (urban, rural, indoor), propagation characteristics, mobility (Doppler spread), and delay spread. There are some challenges in generalization across channel models.

- Training and Inference mismatch: Significant performance degradation can occur if a model trained exclusively under one specific channel profile is deployed in a different environment.

- Practical Channel Issues: Unlike CSI compression or CSI prediction use cases, which often rely on estimated channel state information as input, the AI receiver models operate on raw baseband I/Q samples. This means they are directly susceptible to all real-world physical layer impairments introduced by the propagation channel, making generalization more challenging and critical to study.

Observation 13: The generalization performance of AI receivers when encountering different channel models has not been thoroughly analyzed or validated.

#2 Performance with non-ideal factors

A major concern is that if a model specifically trained under one set of non-ideal factor parameters, it may not function correctly or efficiently when the parameters of these factors change in the operational environment. Moreover, some distortions are highly specific and often vary significantly from one UE to another, and even change over time for a single device due to temperature variations or aging hardware. The generalization performance under non-ideal factors is not thoroughly analyzed in the current study.

Observation 14: The generalization performance under non-ideal factors is not thoroughly analyzed.

### Pros and cons analysis for each sub-case

As per the established 6G AI/ML guidelines, the introduction of AI solutions should ideally not negatively affect or render obsolete existing non-AI (legacy) solutions. Considering this crucial compatibility requirement with non-AI configurations, it is highly beneficial for 6GR to prioritize the investigation of Sub-case A (Sparse orthogonal DMRS) during the initial stages of study. Sub-case A allows for the preservation of legacy DMRS structures while introducing AI processing advantages.

Moreover, DMRS design is intrinsically linked to sophisticated Multiple-Input Multiple-Output (MIMO) functionalities, such as the efficient support of a large number of antenna ports. It is therefore critical to first identify potential sparse DMRS patterns that simultaneously offer reduced overhead and maintain a decent performance level. To ensure seamless integration and standardized benchmarking, it is more beneficial to study Sub-case A at the beginning of the research phase, in parallel with the study of corresponding non-AI/ML based features.

Observation 15: Based on 6G guidelines emphasizing compatibility with legacy solutions and the inherent link to MIMO functionality, it is most beneficial to prioritize the study of Sub-case A (sparse orthogonal DMRS) in the initial research phase.

A comparison of the sub-cases for DMRS overhead reduction is summarized in Table 5. Sub-case B and Sub-case C present numerous drawbacks and practical implementation challenges, making them less suitable for immediate study within the current framework. Key disadvantages of Sub-cases B and C include difficulties in legacy compatibility and potential issues with practical implementation.

Observation 16: Sub-cases B and C have significant drawbacks, including difficulties in practical implementation and legacy incompatibility, making them less suitable for immediate study compared to Sub-case A.

Furthermore, another concern is the complexity of AI models compared to traditional methods. AI receivers processing DMRS introduce additional inference latency. This latency is also heavily dependent on the processing capabilities of the UE. Given that UE computational power is inherently limited, due to power constraints and cost, the overall system impact of deploying complex AI receivers at the UE side has not yet been fully and comprehensively analysed. However, NW side has more potential to be equipped with higher computational resources, and is more suitable to deploy AI receiver.

Observation 17: Due to limited UE computational power, deploying complex AI receivers on the network side (NW side) is more feasible than the UE side.

Table 5 Comparison of sub-cases and alts for DMRS overhead reduction.

|  |  | Pros | Cons |
| --- | --- | --- | --- |
| Sub-case A: Sparse orthogonal DMRS | - Alt1 | - Compatible with non-AI solutions - No cross layer interference on DMRS - Coexistence with non-AI UEs - Flexible scheduling - Minor spec changes |  |
|  | - Alt 2 |  | - Unclear performance with practical CE as label |
|  | - Alt 3 |  | - May have scheduling constraint - High receiver complexity |
| Sub-case B: Superimposed pilot | - Alt 3 | - T/F domain resource overhead reduction: all REs can be used for data | - Not compatible with non-AI solutions - High receiver complexity - Cross layer interference on DMRS - Large spec changes - Scheduling constraint - Coexistence issue with non-AI UEs - Unclear performance with non-ideal factors |
|  | - Alt 4 |  |  |
| Sub-case C: DMRS free | - Alt 3 | - T/F domain resource overhead reduction: all REs can be used for data | - Not compatible with non-AI solutions - Complexity not thoroughly analyzed - Large spec changes - Scheduling constraint - Coexistence issue with non-AI UEs - Unclear performance with non-ideal factors |

Based on the above analysis, the following is proposed:

Proposal 5: For study on DMRS design with AI receiver, consider the following:

- Sub-Case clarification:

  - Receiver assumptions for each sub-use case.

- Label:

  - For the applicable sub-cases and receiving assumption, only estimated channel information is used and do not consider ideal channel information as label.

  - Initial results showed performance loss when using estimated channel as label compared with using ideal channel information as label.

    - With estimated channel @30dB as label, the AI receiver + low density DMRS still outperforms legacy receiver + 1/2 FD density DMRS.

- Benchmark:

  - Both normal DMRS overhead and low density DMRS overhead

  - Non-AI channel estimation method:

    - Ideal channel estimation

    - Conventional channel estimation with reasonable complexity.

    - Note: The complexity of non-AI receiver shall be reported.

- KPIs:

  - Performance related: Throughput, DMRS overhead, BLER, MSE (when applicable)

  - Model complexity and inference latency.

  - Overhead for fine-tuning, when applicable.

  - Inter-vendor collaboration overhead and complexity for two-sided model, when applicable.

- Study aspects:

  - Label quality impact for DMRS design with AI receiver

  - DMRS design

    - Time-domain and frequency-domain sparse DMRS pattern for Sub-case A

    - DMRS sequence/pattern design, power allocation for Sub-case B

  - Scalability on different frequency domain allocation, different time domain allocation, different number of layers, different precoders, different modulation order for DMRS design for DMRS design with AI receiver

  - Performance gain under non-ideal factor, at least including TO, FO, phase noise, interference, other distortions (e.g., PA, IQ imbalance).

  - Generalization

    - Generalization performance of AI receiver over different channels

    - Generalization performance of AI receiver over different non-ideal factors, if the trained model is used to overcome non-ideal factors

Proposal 6: For Prioritize the study on Sub-Case A: sparse orthogonal DMRS in frequency and/or time domain for the study on DMRS design with AI receiver.

## CSI compression and feedback

### Sub-use case analysis

In RAN 1 #122bis, 4 sub-use cases have been identified [2]. This section further clarifies the definition of some use cases with further explanation in theoretical, as well as the training and inference schemes.

#### Sub-case A: CSI compression with JSCC

As in [1], joint source and channel coding was proposed for CSI compression. DL CSI was compressed into a floating-points vector by AI-model for encoder, followed by a quantizer to change it into bit. At NW sided, the de-quantizer is used to recover the floating-point vector as the input to the decoder. In order to provide better performance for uplink channel, during the training, a channel network is used. For example, considering the system encounters the AWGN channel, a binary symmetric channel NW can be deployed, which operates by flipping each feedback bits according to a probability that corresponds to the SINR of the channel.

For inference, the compression outputs (bit sequence) need to be modulated and mapped to corresponding REs for UCI transmission, without channel coding.

Comparing with NR separate source and channel coding (SSCC), JSCC can alleviate the “cliff effect”, i.e., the performance drop-off due to mismatch between the actual SNR and the applied rate adaptation. However, the performance gain (SGCS and UPT) is limited comparing with SSCC because the quantization loss.

#### Sub-case B: CSI compression with JSCM

As analyzed in [1], we proposed CSI compression method based on NW-sided. As shown in Figure 3, UE compresses CSI by projecting L CSI to a matrix, directly maps and transmits the compressed results, i.e., complex values, to the allocated resource elements. This operation can avoid the quantization and deliver more information to receiver side. For instance, assuming the information (entropy) of raw DL channel and compressed channel are Iraw and  Icompress respectively, with a reasonable compress rate, we can assume Iraw≈  Icompress. For AI or non-AI codebook-based CSI compression or SSCC/JSCC-based CSI compression, quantization is required so that there is information loss during compression at transmitter size. Even all quantized bits can be successfully coded, the information loss due to quantization can not be recovered. On the other hand, it also leverages the source coding, i.e., CSI compression, and channel coding, at the transmitter side, so that, this use case was categorized together with CSI compression by JSCM [2]. In addition, JSCM can also work with 2-sided model.

DMRS can be inserted for equalization at NW, to eliminate the distortion of uplink fading for UCI transmission. The equalization output is the unquantized compressed DL channel with noise/distortion (e.g., due to UL channel estimation error). The equalization output contains similar level of information entropy  I'compress.  At receiver, NW-sided model can be used for reconstruction DL CSI based received. With the help of AI/ML, the reconstructed DL channel is able to have I'compress, a similar level of raw DL channel.

Figure 3. JSCM with NW-sided model(inference)

With the help of conventional equalization, the equivalent received signaling can be treated as an AWGN channel, which simplifies the training for the AI model. Shown as Figure 4, a matrix for compression can be trained together with NW-sided model, to achieve better performance.

Figure 4. JSCM with NW-sided model(training)

#### Sub-case C: DLable basis

As explain in [1], the method of downloadable basis relies on NW to train a set of basis vectors and then UE to select W1 from the trained set of basis vectors according to UE specific channel. With AI-trained basis, a significant (e.g. 65%~70% [2]) UCI overhead reduction can be achieved and also an increase in average user throughput can be observed.

The user side operations between the fixed-basis and AI-trained basis are shown in Figure 5 (a). As can be seen, the user side operations remain the same as fixed basis, except that the input needs to be vectorized. For a user, such a change requires a software (S/W) upgrade, hence is easy to implement.

The proposed model architecture is shown in Figure 5 (b). The output of the linear block is quantized via W2 processing block, same as Rel-16 enhanced Type II CSI. The decoder includes a series of sub-blocks, each comprising: linear dense layer A', non-linear activation, and normalization. The received CSI bits at the NW are processed to reconstruct vector Y that is linearly multiplied with the matrix A' whose output is passed through the non-linear activation function followed by a normalization layer. The final reconstructed output is normalized to norm 1. As shown in Figure 5 (b), for a given layer, a precoder (eigen-) matrix with P×N3 can be vectorized to a 2PN3×1 real-value vector, where P is the number of CSI-RS ports and N3 is the number of precoding matrices for the CSI reporting band. Then, the AI-trained basis with size K×2PN3 is multiplied by the 2PN3×1 vector to result in a K×1 projected vector onto the AI-trained basis. Out of the K coefficients of the projected vector, L'≤K coefficients are selected and quantized via amplitude and phase codebooks designed based on Rel-16 eType-II CSI.

For this use case, downloadable basis was trained by NW offline, together with AI-based decoder at NW side, which can be used for reconstructing the CSI based on the feedback CSI. The input of the model can be the amplitudes and phases (of the selected L' coefficients) obtained by a look up table based on feedback CSI bits, where a same table can be used as fixed basis, i.e., the table can be  based on Rel-16 eType-II W2 quantization scheme, shown as Figure 5 (c). During the training phase, eigenvectors can be used for labeling. Correspondingly, the eigenvectors are to be the inference output for the NW-sided model. Other CSI formats (than eigenvectors) are expected to be feasible as well. In addition, whether to use AI-based decoder can be NW implementation.

(a)

(b)

(c)

Figure 5. (a) Fixed vs AI-trained basis & (b) model architecture (c) Dlable basis training

#### Sub-case D: CSI reconstruction with CSI feedback with SRS

Here the SRS measurements are fused with the CSI report to reconstruct explicit CSI, e.g., channel matrices. This approach attempts to combine the unique advantages of SRS-based DL CSI acquisition, e.g., rich information in the spatial domain, as well as CSI-RS based acquisition, e.g., better coverage especially to cover large CSI bandwidth. For a large number of antennas at the network, a configuration of interest is based on spatial domain sparse CSI-RS transmission, i.e., M CSI-RS ports are measured and channel matrices for N antenna ports are reconstructed where M≤N.

Benefits

The benefits of this scheme include

- Addressing the limitations of DL CSI acquisition via SRS

  - Addressing SRS power imbalance, i.e., insertion loss (IL), by using the fused IL-free CSI report as reference for CSI reconstruction.

  - Addressing SRS coverage especially when CSI bandwidth is large, i.e., SRS is sounded for small portion of the CSI bandwidth wherein the additional frequency-domain information is complemented by the CSI report.

  - Addressing SRS overhead, especially in the presence of a large number of UEs, i.e., SRS is sounded for the small portion of the CSI bandwidth and/or subset of SRS ports and complemented by the CSI report.

- Addressing the limitations of DL CSI acquisition via CSI-RS

  - CSI-RS overhead reduction (CSI measurement for M ports and reconstruction for N ports (M≤N))

  - CSI feedback overhead reduction (CSI report for M ports and reconstruction for N ports (M≤N))

The network may combine SRS measurements with the CSI feedback on the explicit (channel matrix) as shown in the Figure 6. The CSI feedback corresponding to spatially sub-sampled measurements, e.g., 64 ports out of 256 ports, carries the channel information for the entire CSI BW. On the other hand, one-shot SRS transmission covers only the fraction of CSI BW, e.g., 3 RBs, to achieve comparable coverage with CSI-RS transmission. In contrast, the SRS measurement provides full spatial domain information, e.g., 256 ports, as it is assumed to be received by all the receive ports. Here, the SRS may also be transmitted from the subset of the received ports as opposed to the entirety of them. In particular, it may be sufficient if the UE transmits SRS on the antenna ports with higher power. Additional advantage of such NW-side up sampling or reconstruction is that the reconstruction models could easily be site-specific, trained for the specificity of deployment scenario and network-side additional conditions, e.g., antenna layout, subsampling patter, TxRU mapping, etc.

Aspects for further study

As illustrated in Figure 6, for the above purpose, two cases can be considered.

- Case1: NW-side model-based approaches. Here, the network-side model may combine CSI feedback and SRS measurement. The CSI feedback may not necessarily rely on AI-based feedback as long as it is for explicit channel matrices, e.g., we may consider CB-based feedback or linear projection-based approach, as outlined in the previous section.

- Case 2: Two-sided model-based approach. Here, the CSI encoding and the corresponding reconstruction must be trained to be compatible, e.g., based on the training collaboration approaches outlined in the NR.

Proper combining of CSI feedback with SRS measurement requires at least the following

- CSI feedback for explicit channel matrices

- Alignment on SRS Tx port and CSI-RS receive port.

Observation  18: For SRS-assisted explicit CSI reconstruction, to properly combine CSI feedback with SRS measurement, the UE should report channel matrices as opposed to precoder matrices.

Figure 6 Two-sided model for SRS-assisted CSI reconstruction from spatially sub-sampled CSI-RS measurement

Proposal 7: With the above discussion, we propose the following changes for sub-case D:

Table D

| Sub-use case | Sub-case D: CSI reconstruction with CSI feedback with SRS (assuming SSCC) |
| --- | --- |
| Reported companies | (3) Qualcomm, vivo, Samsung3 |
| Model input of decoder or model output of encoder, when applicable | 1. Compressed CSI bits - CSI measurement and channel matrix feedback for subset of CSI-RS ports3 2. Estimated channel based on SRS |
| Model output of decoder or model input of encoder, when applicable | (Reconstructed) Eigenvectors, Explicit H3 |
| Label | Eigenvectors Explicit H3 |
| Training types | Offline training |
| KPI | SGCS, UPT |
| Benchmark | eType II NR AI/ML CSI compression without SRS SRS without CSI feedback |
| Model location for inference | Two-sided model NW-side model3 |
| Collaboration/interaction between UE and NW | Similar to two-sided model in NR |
| Potential specification impact | 1. Necessary signaling/procedure to support lower overhead and/or simpler CSI feedback, e.g., feedback for explicit H, signalling to assist SRS and CSI combining. 2. Signalling/ procedure related to LCM for two-sided model including inter-vendor collaboration |

### Additional evaluation results

#### Sub-case B: CSI compression with JSCM with NW-sided model

As shown in [1], for both eigenvector-based and explicit CSI, JSCM with NW-sided model can provide better performance (in SGCS/NMSE/SE) than eType II, SSCC, or JSCC with same overhead, or significantly reduction on the uplink feedback overhead with much lower UE complexity (up to more 90% complexity reduction at UE side). Comparing with JSCM with 2-sided model, some performance loss in SGCS can be observed, however, the complexity at UE side and the inter-vendor collaboration can reduced significantly.

Evaluation results testing under fading channel can be found in Figure 7(a). in the evaluation, the following assumptions are used.

| Parameters | Eigenvector (Rank 1) | Explicit H |
| --- | --- | --- |
| UL channel | 1 Tx, 32 Rx AWGN, TDL-C 300 |  |
| UL channel estimation | Ideal channel estimation, frequency domain LS |  |
| Resource for UCI transmission | 1 PRB with 7 symbols for UCI, and 1 symbol for DMRS | 1 PRB with 12 symbols for UCI, and 2 symbol for DMRS |
| UCI Payload | 72 REs for JSCM and 62 bits for eType II (QPSK, ½ polar) | 144REs for JSCM |
| DL CSI | 32 Tx, 4 Rx Eigenvector, rank 1(32*1), | 32 Tx, 4 Rx Explicit H (32*4) |

From the results we can observe that,

- AI Model can provide good performance for fading channel.

- Channel estimation error based on the inserted DMRS degrades SGCS in very low SNR range, while still better than eType II.

Also note that, LS channel estimation in frequency domain with simple averaging is assumed, which can be treated as lower bound of the performance. With advance channel estimation and interpolation method, the performance can be improved.

Observation  19: With conventional equalization based on the estimated channel of DMRS, JSCM has good performance under uplink fading channel.

Observation  20: JSCM can provide better SGCS performance than eType-II over uplink fading channel with same overhead for feedback.

(a) Eigenvector (Rank 1)                                             (b) Explicit H (32Tx * 4 Rx)

Figure 7. SGCS in AWGN channel and fading channel with idea channel estimation/with channel estimation error.

The UPT performance under realistic (non-full-buffer) traffic model and MU operations is considered for sTRP scenario with Np=64 antenna ports and NSB=13 SBs. The SLS results are shown in Figure 8 for JSCM with a NW-side model, as proposed above. The simulation assumptions are provided in appendix B. The size of the compression matrix for JSCM scheme is K×2NpNSB=K×1664 where four values of K=40, 80, 120, 160 are simulated. After compression, two adjacent values form a complex number that is mapped to an RE. The total number of REs to map K values therefore is K2. For comparison, the UPT vs overhead of SSCC scheme (based on the Rel-16 enhanced Type-II CSI) is also shown where the UL overhead (number of REs) is determined based on an average UL MCS level, which is QPSK and rate =1/2 in this case. It is evident from these plots that the proposed JSCM scheme can achieve better UPT performance as well as lower overhead when compared with the SSCC scheme (legacy Rel-16 enhanced Type II CSI). For instance, up to 3% gain in avg. UPT is achievable when the overhead is 60 REs, and up to 70% overhead saving is possible when the UPT performance is 102%.

Observation  21: When compared with SSCC (based on Rel-16 eType-II CSI), the proposed JSCM scheme exhibits significant gains in terms of performance vs overhead tradeoffs

3%

70%

Figure 8. User throughput vs Rank 1 overhead (number of REs)

#### Sub-case D: CSI reconstruction with CSI feedback with SRS

To evaluate performance of SRS-assisted explicit channel reconstruction, the parameters in Table 12 in Annex are considered. The two-sided model-based approach as illustrated in the Error! Reference source not found. is considered, where the model input of the decoder is estimated channel based on SRS and feedback CSI. It is shown that the two-sided model-based approach achieves a significant gain, e.g., close to 100% SGCS gain for layer 1, as compared to the impaired SRS measurement. The SRS measurement is impaired by channel aging as a result of 30ms interval to cover the CSI bandwidth as well as power imbalance. It is to be noted these results are for non-fixed  power imbalance ratios. The SRS power imbalance coefficients are varied across their mean value with uniform distribution in the range [-1, 1]dB. Therefore, the results can be considered reasonably generalizable across diverse UE-side implementations and variations.

Figure 9 SRS-assisted explicit CSI reconstruction: SGCS gain as compared to impaired SRS-based DL CSI acquisition

Observation 22: Two-sided model for SRS-assisted explicit channel reconstruction from spatially sub-sampled CSI-RS measurement achieves significant SGCS gain over SRS-based DL-CSI acquisition for large channel dimensions.

### Benefits and further study aspects

Table 6 summarized the benefit of each sub-use cases. JSCM with NW-sided model provides the best trade-off for performance, CSI feedback overhead, UE complexity, inter-vendor collaboration. CSI + SRS can be treated as an add-on feature additionally.

Observation 23: JSCM with NW-sided model provides the best trade-off: better performance, less CSI feedback overhead, low UE complexity, no inter-vendor collaboration.

Observation 24: CSI reconstruction with CSI+SRS is applicable for all CSI feedback schemes, at least with feedback of explicit H.

Based on the analysis on the benefits, we propose:

Proposal 8: Prioritize the study on CSI compression with NW-sided model.

Table 6 Summary of benefit of all sub-use cases

| Sub-use cases | Higher DL throughput | Lower CSI Feedback Overhead | Lower UE complexity | Avoid inter-vendor collaboration | Others |
| --- | --- | --- | --- | --- | --- |
| Sub-use case A: JSCC | - Alleviate cliff effect at low SNR and smooth link adaptation - (Slightly) better performance than SSCC/eType-II | Smaller overhead than eTypeII | Comparable/higher than eType-II |  | - |
| Sub-use case B: JSCM (Two-sided model) | - Alleviate cliff effect at low SNR and smooth link adaptation - Better performance than JSCC/ SSCC/eType-II | Smaller overhead than JSCC/SSCC/ eType-II | Comparable/higher than eType-II |  | - |
| Sub-use case B: JSCM (NW-sided model) | + - Alleviate cliff effect at low SNR and smooth link adaptation - Better performance than JSCC/SSCC/ eType-II - Slightly lower than JSCM with two-sided model | Smaller overhead than JSCC/SSCC/eType-II | - Much lower complexity - Short computation latency |  | - |
| Sub-use case C: Downloadable basis | - Better performance than eType-II | Smaller overhead than eTypeII | Reuse precoder as non-AI |  | - |
| Sub-use case D: CSI+ SRS (add-on feature) | - Same as or higher than the corresponding scheme for CSI feedback - Able to handle SRS power imbalance of SRS | Same as or lower than the corresponding scheme for CSI feedback | As the corresponding scheme for CSI feedback | for NW-sided for two-sided | Resolve SRS power imbalance/ Coverage/ Overhead |

For further study CSI compression, the following aspects needs to be considered:

KPIs:

- Intermediate KPIs: SGCS, NMSE

- System KPIs: throughput, overhead

- Model complexity at NW side

- Model/computation complexity at UE side

- Computation latency at UE side

- Inter-vendor collaboration overhead and complexity for two-sided model, if applicable.

Benchmark:

- eTypeII,

- CSI compression with two-sided model with channel coding and modulation with same/similar model complexity, when applicable.

Study aspects:

- Generalization performance

  - Different DL channel

  - Different UL channel, when applicable

- Scalability: e.g., antenna ports, subband number, number of UL resources for UCI

- Spec impact

  - Data collection for training

  - Inter-vender collaboration for two-sided model, when applicable

  - Configuration for inference, e.g., downloadable compression matrix, downloadable basis

  - Performance monitoring including necessity and method

Proposal 9: For study on CSI compression, consider the following aspects:

- KPIs

  - Intermediate KPIs: SGCS, NMSE

  - System KPIs: throughput, overhead

  - Model complexity at NW side

  - Model/computation complexity at UE side

  - Computation latency at UE side

  - Inter-vendor collaboration overhead and complexity for two-sided model, if applicable.

- Benchmark:

  - eTypeII

  - CSI compression with two-sided model with channel coding and modulation with same/similar model complexity, when applicable.

- Study aspects:

  - Generalization performance

    - Different DL channel

    - Different UL channel, when applicable

  - Scalability: e.g., antenna ports, subband number, number of UL resources for UCI

  - Spec impact

    - Data collection for training

    - Inter-vender collaboration for two-sided model, when applicable

    - Configuration for inference, e.g., downloadable compression matrix, downloadable basis

    - Performance monitoring including necessity and method

## AI for beam management and extension

As discussed in RAN1#122bis, AI for beam management in 6GR can be categorized into two parts: 1) the NR use case; 2) extension on the top of NR use case.

The NR use case is the exact Rel-18/19 use case including BM-Case1 and BM-Case2. The standardization practice of NR use case includes a thorough evaluation and standard impact identification. The standard impact includes the support of beam report for DL Tx beam prediction and the support of associated data collection procedure and performance monitoring procedure. In short, the necessity/feasibility and specification impact of NR use case were well studied in 5G NR. Hence, it is proposed to make it a 6GR use case in 6G study phase.

Proposal 10:consider NR AI/ML for beam management as 6GR use case for study [in MIMO agenda item].

In RAN1#122bis, observation is drawn on AI/ML for beam management and extension of NR use case. In particular, six sub-use cases are identified as follows:

- Sub-case A: Inter-Cell/M-TRP DL Tx beam prediction and management

- Sub-Case B: Cross frequency DL Tx beam prediction

- Sub-Case C: Tx-Rx beam pair prediction

- Sub-Case D: Beam prediction for initial access

- Sub-Case E: DL Tx beam prediction for spatial and/or temporal domain with additional local UE information

- Sub-Case F: Reinforcement learning-based approach beam selection

For Sub-case A, at least for non-group based beam reporting, the simulation results from NR [3] can be largely reused assuming beams in Set A / Set B can be from different cells/TRPs. With the capability of inter-Cell/M-TRP DL Tx beam prediction, smooth/swift switch across different cells/TRPs is possible with reduced RS overhead. Considering a more reliable switch and less interruption time during the switch, higher reliability/throughput and reduced latency of UEs can be expected.

Some potential issues need to be studied is the performance metric for training and/or performance monitoring. For example, whether the prediction accuracy of cell/TRP can be used as an performance metric. This performance metric can reflect the link quality in cell/TRP level which facilitate the cell switch decision from NW perspective.

Observation 25:: For Sub-case A: Inter-Cell/M-TRP DL Tx beam prediction and management

- Inter-Cell/M-TRP DL Tx beam prediction enables smooth/swift switch across different cells/TRPs with reduced RS overhead

- Prediction accuracy of cell/TRP can be considered as an additional performance metric

For Sub-case B, the simulation results from NR cannot be reused due to the dramatically change of channel characteristic across frequency. It is understandable that with the capability of cross frequency DL Tx beam prediction, measurement on some frequency can be skipped which results in RS overhead reduction.

However, the feasibility of Sub-case B should be careful studied. For example, for the case of cross frequency range (i.e., cross FR1 and FR2), the height of the panel of FR1 and the height of the panel of FR2 are typically different. It should be studied that whether DL Tx beam prediction can work well with such difference of panel height.

Observation 26: For Sub-case B: Cross frequency DL Tx beam prediction

- With the capability of cross frequency DL Tx beam prediction, UE measurement on some frequency can be skipped which results in RS overhead reduction

- The feasibility of cross frequency DL Tx beam prediction needs to be studied especially for the case that the panel for measurement (e.g., in FR1) and the panel for prediction are not the same and of different height (e.g., in FR2)

For Sub-case C, it was studied in Rel-19 but failed to be justified. In particular, the performance of Tx-Rx beam pair prediction is worse than DL Tx beam prediction. Also, the generalization of the model for Tx-Rx beam pair prediction is challenging as well. Therefore, for 6GR, additional justification is needed to reopen this sub- case.

Observation 27:: For Sub-case C: Tx-Rx beam pair prediction

- Sub-case C was studied in Rel-19 but failed to be justified due to compromised performance

- Additional justification is needed to reopen the study of Sub-case C in 6GR

For Sub-case D, the simulation results from NR [3] can be directly reused assuming initial access procedure happens in the same cell. With the capability of DL Tx beam prediction, it is possible that UE can use beam prediction operation in RACH procedure to reduce the latency of initial access. For example, skipping P1 in the procedure of beam management.

Observation 28:: For Sub-case D: Beam prediction for initial access

- With the capability of DL Tx beam prediction, the latency of initial access procedure can be reduced

For Sub-case E, the simulation results from NR cannot be reused since UE location information is involved. It is understandable that the accuracy of DL Tx beam prediction is more accurate with UE location information. However, it may not be easy to obtain UE location from L1 perspective due to latency and proprietary issues.

Observation 29:: For Sub-case E: DL Tx beam prediction for spatial and/or temporal domain with additional local UE information

- Latency and proprietary issues need to be considered on obtaining local UE information

For Sub-case F, the simulation results from NR cannot be reused since reinforcement learning is involved. However, it not clear that what is the L1 specification impact for the sub-case.

Observation 30: For Sub-case F: Reinforcement learning-based approach beam selection

- More clarification is needed on L1 specification impact

Based on the discussion and observation made on the sub-case A-F, we have the following proposal.

Proposal 11: For AI/ML for beam management and extension, support the study of the following

- Sub-case A: Inter-Cell/M-TRP DL Tx beam prediction and management [in MIMO agenda item]

- Sub-case D: Beam prediction for initial access [in initial access agenda item]

## AI for non-linearity compensation

On top of the observation made in last meeting (as shown in below), in this section, two further evaluations have been conducted to show the performance of AI-NC.

Observation For 6GR AI/ML use cases identification/categorization, [5 sources] provided preliminary simulation results and analysis on AI-based none-linearity handling at transmitter or receiver. - [5 sources] provided preliminary simulation results and analysis on AI-based DPoD/None-linearity compensation at receiver. - [2 sources] provided preliminary simulation results and analysis on AI-based DPD at transmitter. - Detailed evaluation assumptions (model input/output/label/KPI/benchmark) and initial analysis can be found in Table G. Note: whether/how to capture the observation in the TR is a separate discussion. Sub-use case; Sub-use case A: AI-based DPoD/None-linearity compensation; Sub-use case B: AI-based DPD. Reported companies; (5) Samsung1, Ericsson2, OPPO3, vivo4, Huawei5; (2) vivo2, Huawei1. Model input; 1. Received signal1,3,4,5; Time domain samples before pre-distortion. Model output; 1. Compensated signal in time domain1,2,4,5 2. Soft bits2,3; Time domain samples after pre-distortion. Label; 1. DMRS1 2. Known bit sequence2,3,4 3. time domain samples from known sequence5; Time domain samples. Training types; Online training/finetune1 Offline training; Offline training Online training/finetune2. KPI; BLER, MPR, EVM, throughput; BLER, EVM, MPR. Benchmark; Without compensation, GMP model based NC1; No DPD. Model location for inference; NW-sided model; UE-sided model. Collaboration/interaction between UE and NW; Similar to NW-sided model as NR; Similar to UE-sided model as NR. Potential specification impact; 1. RAN 4 requirements, e.g. EVM 2. DMRS/Sequence design/selection, Tx power determination 3. Signalling/ procedure related to LCM for NW-sided model; 1. RAN4 requirements, e.g. EVM 2. Tx power determination 3. Signaling/ procedure related to LCM for UE-sided model.

The two evaluations are on:

- Comparison with non-AI NC (i.e., GMP based NC, GMP-NC) method

- Comparison with different cases when consistent level of PAPR between data and DMRS are different

### Comparison with GMP-NC method

In the existing study of non-linearity handling, there is some non-AI method to compensate the non-linearity. For example, the GMP-NC [4] is trying to modeling the PA nonlinearity. However, based on our study, such kind method highly relies on the purity of the non-linearity source, e.g., only coming from single source that can be modelled.  But in the reality, the situation is not that ideal, many aspects will contribute the overall non-linearity experienced by the signal. Thus, in this evaluation, I/Q imbalance is added on top of typical PA non-linearity [5]. The modelling of I/Q imbalance is listed in the table, which are basically two dimensions of uniformly distributed random value.

Figure 10  BLER performance comparison with non-AI NC (i.e., GMP-NC) method

Table 7  Evaluation setup

| Parameters | Value |
| --- | --- |
| Carrier frequency | 3 GHz |
| SCS | 15 kHz |
| BW | 11.88 MHz |
| Code rate | 0.92 |
| Modulation | 256 QAM |
| Antenna Config. | 1T1R |
| Channel | TDL-D |
| Input buffer size (AINC & MPNC) | 2 |
| # of reservoirs | 30 |
| Order (AINC & MPNC) | 5 |
| Non-linearity factor | PA non-linearity Rx EVM=4.5%, corresponds to additional I/Q imbalance (Amp imbalance ~ [0,0.1dB], phase imbalance ~ [-5, 5 degree]) Rx EVM=6%, corresponds to additional I/Q imbalance (Amp imbalance ~ [0,0.5dB], phase imbalance ~ [-5, 5 degree]) |

Based on BLER performance comparison, it can observe that:

- When nonlinearity is dominated by PA component, e.g., I/Q imbalance is slight (Rx EVM=4.5% case). For a memoryless PA, GMP-NC can already fit PA model so that AI-NC improves only a few over GMP-NC.

- When nonlinearity is contributed by both of I/Q imbalance and nonlinear PA, e.g., Rx EVM=6% case. In such case, GMP-NC exhibits obvious under-fitting while the nonlinearity can still be well-canceled by AI-NC.

The observations prove that the capability of AI based NC to compensate the non-linearity through the whole loop instead of one particular component. Thus, AI NC is more robust to multiple non-linearity sources.

Observation  31: AI-NC is more robust to multiple non-linearity sources comparing to GMP-NC.

Proposal 12: adopted following change in table G in observation from RAN1#122bis:

| Benchmark | Without compensation, GMP model based NC1 | No DPD |
| --- | --- | --- |

### Comparison with different consistent level of PAPR between data and DMRS

To fully utilizing the capability of AI NC, one key point is that whether the training can ‘learn’ full non-linearity that the data symbol will experience. On the other word, the consistent level of PAPR between data and DMRS (used for training) is very important. In the evaluation of this subsection, the performance of different settings of DMRS are exanimated. Follow the evaluation assumptions in subsection 2.5.1 with only PA non-linearity considered. For DFT-s-OFDM, baseline EVM improved to 3.5%

Figure 11  BLER performance comparison with different consistent level of PAPR between data and DMRS

Table 8  Different consistent level of PAPR between data and DMRS

|  | DMRS PAPR | Data PAPR |
| --- | --- | --- |
| Case 1: ZC w/o transform precoding | 3.6 [dB] | 8 [dB] |
| Case 2: PN w/o transform precoding | 8.9 [dB] | 8 [dB] |
| Case 3: ZC w/i transform precoding | 3.3 [dB] | 8 [dB] |
| Case 4: PN w/i transform precoding | 6 [dB] | 8 [dB] |

Based on the simulation results, one observation we can get: the BLER performance is improved proportionally to the increase of PAPR similarity between DATA and DMRS.

- The consistence level of PAPR between data and DMRS from high to low is: case 2>case 4>case 1>case 3;

- The BLER performance from good to bad is: case 2>case 4>case 1>case 3.

PN sequence without transform precoding significantly outperforms others since DMRS and Data have similar PAPR and nonlinearity in such case. AI-NC can learn most of their nonlinearity characteristics. For the two cases of ZC sequence, the PAPR difference is largest, the AI-NC only learns noise so that AI-NC results in no improvement at all. So the information from the simulation is showing that having a consistent PAPR level between DMRS and data is critical for fully utilizing the AI-NC. This observation motivates the spec impact on the DMRS design/selection.

Observation  32 : a consistent PAPR level between DMRS and data is essential for fully utilizing the AI-NC.

Proposal 13: the study of AI-NC in RAN1 should consider DMRS design to have similar PAPR level as corresponding data part in the same channel.

## AI-enabled uplink precoder indication

In RAN1#122bis, following observation is drawn on AI-enabled UL precoder indication.

| Use case | AI-enabled UL precoder indication |
| --- | --- |
| Reported companies | (3) vivo1, Fujitsu2, Samsung3 |
| Model input of decoder or model output of encoder | UL precoder indicator/compressed UL precoder |
| Model output of decoder or model input of encoder | (Reconstructed) eigenvectors of UL channel |
| Label | Estimated eigenvectors of UL channel based on SRS measurement |
| Training types | offline training online finetune1 |
| KPI | SCGS, BLER |
| Benchmark | NR TPMI codebook |
| Model location for inference | No model for inference 1,3 Two-sided model1,2 |
| Collaboration/interaction between UE and NW | Similar to one-sided model in NR 1,3 Similar as two-sided model in NR1,2 |
| Potential specification impact | 1.The signaling/procedure related to the download/upload of UL codebooks/compressed UL precoder 2. LCM procedure to facilitate the training of the downloadable/uploadable UL codebooks when no model for inference, 1,3 3. Signalling/ procedure related to LCM for for two-sided model including inter-vendor collaboration, when applicable1,2 |

### Further simulation results and analysis

Further simulation results in addition to the simulation results in RAN#122bis are provided and analysed in this section. Simulation related parameters are summarized in Table 9. In particular, 4 coherent antenna ports are used for simulation. For the baseline, the precoding matrix specified in TS38.211 are used. Simulation results are generated for both NR UL Codebook and Dlable (downloadable) UL Codebook assuming different geometries for rank 1. In particular, two antenna geometries (Antenna geometry #2 in Figure 13, Antenna geometry #3 in Figure 14 are considered for simulation. Note that NR UL Codebook is designed based on Antenna geometry #1 in Figure 12.

Table 9  Simulation parameters

| Parameters | Values |
| --- | --- |
| Bandwidth, SCS | 10MHz (52RB), 15kHz |
| Tx ports | 4 |
| Channel Model | CDL-C |
| UL precoding granularity | wideband |
| Payload size | 5 bit |
| Performance metric | SGCS |

Figure 12  Antenna geometry #1: 2×1 antenna with dual polarization

Figure 13  Antenna geometry #2: 4×1 antenna with single polarization

Figure 14  . Antenna geometry #3: 4×1 antenna with single polarization

Figure 15  SCGS for NR UL codebook and Dlable UL codebook

Simulation results are provided in Figure 15. For Rank 1, Performance gain can be observed from Dlable codebook in comparison with NR UL Codebook. In particular, 6.9% SCGS gain can be obtained from Antenna geometry #2 and 35.6% SCGS gain can be obtained from Antenna geometry #3. For Antenna geometry #2, the gain of Dlable UL Codebook is from the better match of antenna geometry with the feature of 4-Tx linear antenna array. For Antenna geometry #3,  the gain of Dlable UL Codebook is even more since legacy NR UL Codebook is designed with the assumption that the first two antenna port and the last two antenna port have high spatial correlation. However, this assumption does not hold for Antenna geometry #3. With the use of Dlable UL codebook, the mismatch is addressed and hence with a large performance improvement.

Observation 33: Dlable UL codebook provides performance gain in comparison with NR UL codebook.

### Benefit of the use case

The benefits of Dlable UL codebook are listed as follows:

- Performance optimization

- Future-proofness

- No risk of disclosing UE-side antenna structures

For the first aspect of benefit, performance optimization means that Dlable UL codebook can minimize the performance degradation due to the mismatch between the generic antenna structure or layout assumed for codebook design and the actual antenna structure or layout used for UL transmission. This can be observed from the simulation results above. UE antenna geometries are less-structured considering different UE types such as CPE, mobile device, etc. Even for the same UE type, e.g., mobile device, the implementation of antenna geometry for each UE can be very different due to the different appearance of mobile devices. If Dlable UL codebook is adopted, better performance for some antenna geometry can be expected in comparison with fixed UL codebook.

For the second aspect of benefit, future-proofness means that whenever a new UL antenna structure is used/applied, the corresponding Dlable UL codebook can be configured after offline engineering of UL codebook development. This can largely reduce the standard exercise on the design/optimization of UL codebook based on a particular UE antenna structure in the future.

For the third aspect of benefit, the key idea of Dlable UL codebook is to leverage offline engineering or data-driven method (e.g., training based on AI/ML) to figure out the best set of UL codebooks for UE(s) without disclosing UE implementation on antenna structures.

### Aspects for further study

The key aspect is the configuration and indication of Dlable UL codebooks. An exemplary procedure for the use of Dlable UL codebook can be as follows: 1) Step 1. UL codebook with a set of Dlable precoding matrix; 2) Step 2. UE receive the indication of an precoding matrix, where the precoding matrix can correspond to a Dlable precoding matrix; 3) UE use the indicated precoding matrix for PUSCH transmission.

Another aspect is the necessary procedure to facilitate the development or performance monitoring of Dlable UL codebook. For example, it is necessary to study how the perform data collection and the related procedure for the development (e.g., training or offline engineering) of Dlable UL codebook. Also, for performance monitoring, mechanisms are needed to enable the comparison (performance monitoring) between the UL performance using Dlable UL codebooks and the UL performance using fixed UL codebooks.

Proposal 14: Support the study of Dlable UL codebooks [in MIMO agenda item], including the following

- Signaling/procedure related to the download of UL codebooks

- Signaling/procedure to facilitate development or performance monitoring of the Dlable UL codebooks

## AI for PAPR reduction for waveform

### Further clarification of use case

The proposed use case is based on a two-sided model architecture for baseband (BB) side PAPR reduction, which can be combined with AI-NC for RF side optimization. Some companies have proposed UE-sided model approaches for PAPR reduction, but these raise fundamental questions about their distinction from AI-designed waveforms. If a UE-sided model for BB PAPR reduction operates independently without considering RF nonlinearity, it essentially becomes equivalent to designing a fixed filter coefficient through AI, regardless of symbol combinations, channel conditions, or other dynamic features. This is functionally equivalent to AI-designed waveform rather than adaptive AI/ML-based signal processing.

For UE-sided models to be valid and distinguishable from simple AI-designed waveforms, they must jointly consider both RF and BB characteristics. When RF nonlinearity is incorporated into the optimization, the model must adapt to specific PA characteristics, making it truly device-specific and adaptive rather than a predetermined filter design. However, this approach introduces significant challenges including the need for PA-specific training, device-specific model deployment, and inter-vendor interoperability issues. In contrast, the proposed two-sided model operates at the BB level and can be combined with a separate AI-NC at the RF level. This implies UE-implementation agnostic training and it addresses the concerns for UE-specific training, and the inter-vendor collaboration complexity thereafter, while enabling joint optimization through the overall transmit chain.

Observation  34: The proposed two-sided model-based transform precoder operates in the baseband (BB) level not subject to a specific PA nonlinearity characteristic. This UE-implementation agnostic approach relieves the need for UE-specific training. Conversely, the proposed approach can be applied along AI-based nonlinearity correction (AI-NC) approaches which operate in the RF level.

### Benefits

- UE power saving

The AI/ML-based transform precoding framework achieves approximately 1.2 dB PAPR reduction compared to traditional DFT-s-OFDM systems, enabling the power amplifier to operate closer to its saturation region where efficiency is maximized. This enhanced PA efficiency directly translates to extended battery life for user equipment, as less DC power is wasted as heat and more energy is converted into radiated RF power. The computational complexity analysis reveals only 0.365 MFLOPs at the transmitter, representing merely a 27% increase compared to DFT-s-OFDM, making the power savings implementation practically feasible.

- Uplink coverage enhancement

The proposed framework improves uplink link budget and extended coverage range, particularly for cell-edge users. Lower PAPR enables UE to transmit at higher average power levels while maintaining the same peak power constraints, improving the received signal strength at the sNB and enabling successful demodulation at greater distances. The AI-based transform precoder adaptively optimizes the transformation based on actual signal characteristics, addressing the paradox where cell-edge UEs with smaller resource allocations experience significant PAPR issues. The improved uplink coverage enables larger cell sizes and reduces the density of required sNB, lowering deployment costs and enabling economically viable service in rural and remote areas.

- Backward compatibility and simplified network evolution

The proposed framework maintains comprehensive backward compatibility with existing network infrastructure and protocols, retaining the signal flow of DFT-s-OFDM and ensuring fundamental structure and timing of transmissions remain unchanged. Channel estimation, equalization procedures, reference signal design (e.g., DMRS/SRS), and higher layer procedures require no modification, minimizing standardization complexity and deployment barriers. The standardization approach leverages fully specified reference models for TxNet and RxNet, enabling vendors to train interoperable implementations while optimizing for their specific deployment scenarios and hardware characteristics. Since the training dataset utilizes modulation symbols, additional data collection may not be necessary as the reference model can be trained based on synthesized training data. In this case, no interaction between UE and the NW is required. By avoiding wholesale infrastructure replacement, operators can upgrade network capabilities through software updates and targeted equipment deployment, accelerating technology adoption and enabling rapid realization of benefits.

### Aspects for further study

- Generalization

For two-sided models operating at the BB level, generalization must be demonstrated across various channel conditions, modulation schemes, and resource allocation patterns. The interaction with AI-NC at the RF level could also be studied to ensure the BB-side model generalizes properly regardless of the specific PA nonlinearity compensation approach used. The study should verify that the two-sided model maintains performance when combined with different AI-NC implementations from various vendors.

- Coexist with non-AI UEs

The two-sided model architecture inherently requires both the transmitter (UE) and receiver (sNB) to support AI/ML processing capabilities, creating potential compatibility issues with legacy or low-complexity devices. Network implementations must consider strategies such as separate time-frequency resources for different device types.

- Inter-vendor collaboration aspects

The two-sided model approach offers clearer inter-vendor collaboration paths compared to UE-sided models, as the standardized reference models can define both TxNet and RxNet architectures ensuring interoperability. The specification impact includes defining model architectures as well as parameters, input/output formats, and verification procedures. For deployments combining with AI-NC is possible by clarifying the interface between BB and RF processing to enable independent optimization while maintaining overall performance.

### Specification impact

While the [1] identifies basic specification impacts (reference models for TxNet and RxNet, monitoring methods, data collection), additional specification areas require study. The specification must address UE capability reporting including applicability indication for different deployment scenarios and hardware configurations. Physical layer procedures for waveform switching and handling of AI/ML-based waveform and legacy waveforms must be defined.

Proposal 15: For AI for PAPR reduction for waveform study,

- Use case: transformed symbols symbol in frequency domain/ the modulated symbol in frequency domain /label free

- Evaluation assumption:

  - KPI: BLER, CCDF of PAPR, throughput

  - Benchmark: DFT-s-OFDM

- AI/ML specific assumption/aspects

  - Generalization must be demonstrated across various channel conditions, modulation schemes, and resource allocation patterns

  - Inter-vendor collaboration (fully specified reference model)

- Specification impact: inter-vendor collaboration, waveform switching

## AI for PRACH detection and early contention resolution

### Motivation and use case description

In mobile communication system, the random access (RA) procedure serves as a critical mechanism for establishing uplink synchronization by resolving two key parameters: the preamble index (RAPID) and timing advance (TA) between UE and BS. For contention-based random access (CBRA), UEs autonomously select preambles from a limited pool of preamble sequence, creating inherent collision risks when multiple UEs select the identical preamble. Such collisions force contending UEs to initiate power-ramped preamble retransmissions, resulting in higher power consumption and latency on the UE side, especially for power-constrained or latency-sensitive devices.

The NR PRACH resource partitioning exacerbate this collision challenge during each Random Occasion (RO). While Rel-15 5G NR systems allocated 64 dedicated preambles per RO, emerging services in 5G evolution accelerates preamble resource fragmentation, thus reducing the available contention-based preamble subset for random access and thereby amplifying the challenge of multi-user collision.

Conventional RAPID detection relies on threshold-based peak identification utilizing the power delay profile (PDP), derived by correlating the PRACH preamble with a locally generated ZC sequence. A preamble is declared detection if the maximum peak of PDP in the search window (SW) exceeds the predetermined threshold, simultaneously resolving the RAPID and TA. This approach exhibits three fundamental limitations: (a) Static-setting threshold through empirical methods: Predefined threshold could fail to adapt to dynamic channel condition, particularly in low SNR scenarios. (b) Near-far power disparity: In multi-user environments, strong interferers mask weaker signal, thus complicate the threshold setting for multi-user detection. The analysis underscores the need to develop an effective preamble detection algorithm designed to operate efficiently in scenarios involving multiple users.

Machine learning (ML)-based PRACH detection has merged as a transformative approach and drawn attention in both academic and industrial societies. With AI-based PRACH receiver, early contention resolution can be feasible in which the BS employs the AI-based receiver to detect a preamble and estimate the number of UEs that selected and transmitted the same preamble. Based on the estimated multiplicity for a given preamble, the BS can transmit a random-access response (RAR) associated with the RA-RNTI corresponding to the preamble. The RAR may include multiple PUSCH grants, where the number of grants corresponds to the BS’s estimation of how many UEs transmitted the same preamble in the same PRACH resource. Based on the estimated multiplicity for a given preamble, the BS can transmit a random-access response (RAR) associated with the RA-RNTI corresponding to the preamble. The RAR may include multiple PUSCH grants. The contented UE can choose the corresponding TA and UL grand for msg3 transmission to solve the preamble collision in early stage. By enabling this kind of contention resolution at the Msg1/Msg2 stage for a substantial portion of collisions, it reduces access delay, improves first-attempt success probability, and enhances uplink resource efficiency, making it particularly beneficial for 6G networks with massive and bursty access demands.

To enable the early contention resolution with AI/ML-enabled PRACH receiver, from RAN1 and RAN2 prospective, random-access procedure to solve multi-user collision need to be studied, especially for msg2 to support grants for multi-user corresponding to one preamble, and msg3 transmission in UE side.

In general, assisting by AI/ML technologies, we expect that the following benefits can be achieved

- Detection performance improvement including both missing detection and false alarm detection compared with legacy receiver in both single-user and multi-user collision scenario

- Access delay and power consumption reduction for UE random access

### Assumption on model structure/location and training types

The following is the evaluation assumption for AI-enabled PRACH receiver for early contention resolution

Table 10 : Evaluation assumption of AI-enabled PRACH receiver for early contention resolution

| Model input | Detected PRACH signal with PDP or truncated PDP in time domain for each preamble detection window |
| --- | --- |
| Model output | Predicted number of UEs that transmitted the same preamble for given PRACH resources |
| Label | Ground-truth number of UEs that transmitted the same preamble |
| Training types assumption | offline training |
| KPI | Missing detection rate with UE multiplicity, and false alarm detection rate |
| Benchmark | Legacy receiver |
| Model location for inference | NW-sided model |
| Collaboration/interaction between UE and NW | Similar to NW-sided model in NR |

### Preliminary evaluation results

The following shows a preliminary result of detection performance at BS AI/ML-based PRACH receiver for both single-user and multi-user collision scenario. For simulation, we utilize the typical assumption to assess the performance of the proposed method in relation to both detection probability and false detection probability, referred to as Pd and Pf, respectively. According to the targeting requirements outlined in 3GPP, Pd must be equal to or greater than 99%, while Pf should be less than 0.1%. The conditions for the simulation are delineated in Table 11.

Table 11: Simulation conditions

| Parameter | Values |
| --- | --- |
| Channel type | AWGN/TDLC300-100 |
| PRACH format | C0 |
| Ncs | 0/23 |
| Logical index | 0/1 |
| Preamble ID | 0/5 |
| Number of Preamble | 64 |
| Number of UE | 1/2 |
| Doppler Shift | 2334Hz for restrict set B |
| Number of Tx/Rx | 1/2 |
| Timing offset between 2UE | s 20samples |

Figure 16: Multi-user collision detection performance in both AWGN and fading scenario

The Figure 16 illustrates the detection performance Pd of two UEs in AWGN and TDLC300-100, respectively. -8dB and -2dB correspond to the minimum SNR required by 3GPP specifications to meet the performance for a single user under AWGN and TDLC300-100, respectively. When the two UE have equal power, the corresponding SNR in AWGN is -8dB and in TDLC300-100 it is -2 dB, with the probability of correct multi-user detection being 99.96% and 97.06%, respectively. When the power difference between the two user devices is 3 dB, the probability of multi-user detection reaches 95.6%.

Meanwhile, for single user scenario, the proposed AI based solution can also achieve the outstanding performance for single UE detection under non-HST scenario, almost 2dB gain compared with conventional threshold-based approaches in Figure 17. Also, the outstanding performance can be achieved with around 2dB gain under high-speed scenario in the low SNR region as illustrated in Figure 20.

Figure 17: single-user detection performance in AWGN scenario, (Ncs, RAPID, logical root sequence index) as (0,0,0)

Figure 18: single-user detection performance in HST scenario, (Ncs, RAPID, logical root sequence index) as (23,5,1) for HST restriction setB

### Potential spec impact

With supporting of AI/ML-enabled advanced receiver for PRACH detection, the input and output of AI/ML model should be studied, as well as the corresponding LCM procedure for NW-sided model.

Meanwhile, to enable the early contention resolution with AI/ML-enabled PRACH receiver, the corresponding random-access procedure to solve multi-user collision is needed, especially for signalling/procedure related with msg2 to support grants for multi-user corresponding to one preamble, and the signalling/ procedure related to Msg3 transmission for more than one UEs with selected the same PRACH preamble.

Proposal 16: Study on AI-enable PRACH receiver for early contention resolution with enhanced random access signaling and procedure as well as corresponding LCM procedure. update the Table in observation from RAN1#122bis as following:

| Use case | AI-enable PRACH receiver |
| --- | --- |
| Model input | Detected PRACH signal with PDP or truncated PDP in time domain for each preamble detection window |
| Model output | Predicted number of UEs that transmitted the same preamble for given PRACH resources |
| Label | Ground-truth number of UEs that transmitted the same preamble |
| Training types | offline training |
| KPI | Missing detection rate with UE multiplicity, and false alarm detection rate |
| Benchmark | Legacy receiver |
| Model location for inference | NW-sided model |
| Collaboration/interaction between UE and NW | no collaboration or Similar to NW-sided model in NR |
| Potential specification impact | - signalling/ procedure related to random access for more than one UEs with selected the same PRACH preamble. - Signalling/procedure related to LCM for NW-sided model |

# AI/ML framework and evaluation methodology

The NR AI/ML framework for the air-interface is a compromise between practicality and forward compatibility. The main aspects of the NR AI/ML framework can be summarized with the following points:

Data collection: NR specification support was provisioned to enable UE-side and the NW-side data collection. Particularly, UE-side data collection is enabled through UEs measurements report. For this, the NW may provide configurations and reference signal transmissions for UEs measurement, where the measurements may not entail reporting to the NW. On the other hand, for the NW-side data collection, the specification is updated for the UE to measure, log, and report collected data based on the MDT framework. This architecture supports a harmonized approach to data acquisition, enabling both UE feedback for trend analysis, model training, and also validation.

Applicability Report: The legacy capability report (non-AI/ML-related capabilities) is reported based on a set of explicitly defined conditions, e.g., the set of codebook parameters UE supports. Moreover, UE reports such capability in a consistent manner irrespective to the particular cell it is attached to. In stark contrast, UE’s AI/ML capabilities are additionally conditioned on whether UE’s models are trained based on dataset from certain additional conditions, e.g., a particular scenarios and NW-side settings/implementations, which may not be explicitly defined or configured. Examples of such scenarios and configurations include NW’s deployment scenario, NW’s beam information.  In this case, the UE may additionally report applicability/inapplicability of AI/ML functionalities in response to the NW’s indication. Such (in)applicability indication can be based on indication from candidate inference configurations or candidate inference-related parameters that the NW provides.

Protection for NW-side implementation: To safeguard proprietary NW implementations while enabling effective UE-side AI/ML model training and inference, the NR framework leverages the concept of an Associated ID. Associated ID is to implicitly indicate NW-side additional conditions that may impact performance of a UE-side model. Associated ID is introduced for UE-side BM prediction to ensure consistency on UE’s assumptions on measured (Set B) and candidate (Set A) beams for prediction. It is also introduced in UE-side AI/ML positioning to implicitly represent the sensitive TRP location. As the beam information is proprietary to the NW’s implementation such indication enables the UE-side to train its model based on training data collected under the same indication as inference while protecting sensitive NW-side implementation information.

Model performance monitoring for UE-side model: As compared to non-AI/ML algorithms, the data-driven AI/ML algorithms may not generalize for all deployment scenarios and NW-side implementations, e.g., antenna structure, TX filter, etc. This motivated proactive model performance monitoring. The monitoring outcome may assist the NW to make swift decisions on configurations/functionalities switching including fallback to non-AI/ML approaches. NR considered two performance monitoring approaches: UE-assisted performance monitoring where performance monitoring metric is calculated at the UE and NW-side metric calculation considering ground-truth measurements report from UE as input.

Model training: In the NR offline model training is assumed. Thus, there was no explicit specification support for on-device (online) model training and fine-tuning.

Inter-vendor training collaboration for two-sided model: For two-sided models, ensuring both inter-operability as well as separate training at the NW-side and UE-side was a major challenge. Rel-19 identified three inter-vendor training collaboration options, namely: i) fully specified reference model ii) reference encoder parameters sharing-based approach iii.) encoder-related training dataset sharing-based approach. The encoder parameters and dataset are shared from the NW-side to the UE-side after the NW trains its part of the model. The fully specified reference model, a.k.a. Direction C, provides a common guidance in the input-output mapping to enable vendors to train inter-operable models which can also be optimized to the vendors preferences. The vendors may employ collected field data to optimize their respective models with respect to the relevant additional conditions, e.g., deployment scenarios, antenna layout, and/or model implementation aspects, e.g., complexity and model backbone. In the encoder parameters/dataset sharing based approaches, NW may have the freedom in initializing the training process without a predefined mapping constraints imposed by the reference model- however, it requires the UE to implement multiple encoders to interoperate with multiple NW vendors.

Model Transfer: In the NR model transfer across vendors, e.g., from NW to the UE, has not been supported. It is assumed that the UE-part of the models are trained offline by UE-side vendors and delivered to the UEs in a transparent manner.

Processing unit and timeline: The 5G NR acknowledged AI/ML processing may be carried out by a separate hardware from non-AI/ML processes. In this regard, the AI/ML processing unit (APU) is introduced to quantify the number of concurrently processed AI/ML-based CSI reports. Moreover, a separate CSI processing timeline is considered for AI/ML-based CSI reports.

The aforementioned features of the NR framework provide a strong foundation for 6GR framework. Therefore, we suggest that the discussion for 6GR AI/ML begin by adopting them as a starting point.

Proposal 17Take NR AI/ML framework as a starting point and support/adopt the following in 6GR AI/ML framework

- Terminologies in TR 38.843

- UE-side and NW-side data collection

- Applicability report

- Associated ID to indicate additional conditions that may not be explicitly configured

- Fully specified reference two-side models to address interoperability

- Performance monitoring

- Dedicated AI/ML processing unit (APU) and timeline

## Potential Enhancements from NR AI/ML Framework

One issue that has not been fully addressed in the NR is how to support site/scenario specific UE-side/part models in a scalable manner. As mentioned above, in NR, UE-side or UE-part of two-sided model training and deployment was left to the UE-side vendor, e.g., UE-side vendor may collect site-specific data, train the respective models, and deliver the models (in a transparent manner) to the UE when needed.

This may impose logistical scalability issues for the UE-side vendors as the vendors may not be able to train models for every cell/site, etc. In a yet another approach, the NW may train a model with site-specific dataset and transfer the model to the UE. The latter approach was challenged in NR studies as the NW may not have sufficient information on UE’s computing platform posing interoperability challenges, i.e., a model trained by the NW may not be able to run properly at the UE. Moreover, frequent model transfers entail radio resource overhead which may curtail performance benefits, especially when the entirety of a large model has to be transfer. This is particularly challenging when the model transfer is to be made during handover when the channel quality does not support high transfer rates.

To address these challenges and unlock the potential of site/scenario-specific models, we propose exploring two complementary strategies: non-linear and linear approach. A non-linear approach focuses on targeted parameter transfer, specifically adapting layers within a pre-defined model architecture. This strategy is particularly effective when the core model structure, such as the encoder in a two-sided model (illustrated in Figure. 19), is fixed, allowing for interoperable transfers of only essential adaptation parameters rather than entire models. Alternatively, parameters for linear processing of input can be shared for adaptation to site/scenario specific nuances. The site/cell-specific basis and projection matrices detailed in subsequent sections are examples of this targeted parameter transfer approach. This latter method offers better scalability for deploying and operating UE-side models, as it may not require the standardization of model structures at the UE-side.

Proposal 18: Consider the following approaches to support site/scenario-specific models

- Non-linear approaches: Parameter transfer for input/output adaptation layers of specified model structure

- Linear approaches: downloadable projection/basis matrices for input/output processing

Figure. 19 Parameter transfer for adaptation layers for site/scenario specific models

Current AI/ML investigations within NR air interface design predominantly rely on offline model training. However, advancements in AI processing capabilities are going to enable online training and fine-tuning approaches, particularly for use cases requiring relatively lightweight models. This presents a significant opportunity for 6GR to expand its application scope. For instance, utilizing AI/ML techniques for power amplifier (PA) nonlinearity compensation, as discussed further in Section 3.5, is well-suited to online adaptation strategies due to the inherently limited complexity of such models. Therefore, 6GR architectures should actively accommodate and explore use cases amenable to online training or fine-tuning methodologies.

Proposal 19: Considering online training/fine-tuning in 6GR AI/ML study, at least for the use case that only requires lightweight model.

Figure 20 illustrates a framework for model storage, deployment, and inference. Advanced implementations allow for AI/ML models to be delivered from external entities or network nodes directly to the UE. This process might happen between NW’s provision of candidate configurations/parameters and UE’s applicability report. The AI/ML model could also be stored in the UE’s non-volatile storage; ready to be loaded to the computing platform. Typically, fetching the AI/ML model to the cache memory and configuring it in the processing platforms entails a longer delay than a typical inference would incur, e.g., CSI processing delay. This process may happen between NW’s AI/ML feature/model activation and UE’s confirmation. AI feature/model activation may occupy a certain memory unit - as AI/ML inference may occupy the processing unit. When the NW deactivates a certain AI/ML feature/model, the model can be released from the processing platform freeing the occupied memory space. Consequently, effective AI/ML processing capability is fundamentally limited by two dimensions: available memory and processing capacity.

The memory requirement has not been addressed in the NR. One reason could be memory issues may not be particularly pressing in the NR given that only a few delay-sensitive (CSI-related) AI/ML features are supported. However, in 6GR when a large number of use cases are considered and when the UE is configured with a large number of AI/ML features, managing memory (number of active features/models) would become pertinent. In the NR, the buffering (memory) requirement for CSI measurement and reporting is quantified by active CSI-RS resource and ports counting. This approach can be extended to quantify the number of concurrently activated AI/ML features/models at the UE.

Observation 35: AI/ML processing capability at the UE is composed of two dimensions

- Processing/computing capability

- Memory (buffering/caching) capability

Proposal 20: Study the potential impact of AI/ML memory on the concurrently activated AI/ML features/models, including whether to introduce AI/ML memory unit (MU) taking the NR’s active CSI-RS resource and ports counting as a starting point.

Figure 20 : Framework for model storage, deployment, and inference

## Evaluation methodology

AI/ML studies in NR relied in two types of KPIs, namely, intermediate KPIs and ultimate (eventual/system) KPIs. The intermediate KPIs, such as SGCS, are generally used to assess model-specific performance, hence, to evaluate aspects such as model performance-complexity tradeoff, generalization performance, impacts of parameters and approaches related to training and dataset, monitoring accuracy, etc.  Besides, theoretical spectrum efficiency (SE) calculated based on equivalent SINR can be considered, which can directly show the link performance gain in terms of spectrum efficiency for single-user.  On the other hand, the ultimate KPIs, such as UPT, are used to assess the overall benefit of AI/ML use cases against baseline and make decisions towards normative work.  This approach has been proven to be efficient and it can also be adopted in 6GR AI/ML studies.

Proposal 21: For 6GR use-case studies, adopt intermediate and ultimate (eventual/system) KPIs as in the NR.

- Intermediate KPIs to evaluate model-specific performance, e.g., model performance-complexity trade-off, generalization performance, monitoring accuracy, training and dataset aspects.

- Ultimate KPIs to assess overall performance benefits of AI/ML use cases.

Generalization performance plays a paramount role in the cost-benefit analysis of AI/ML use cases as well as determination of aspects that should be considered/configured in the training, dataset collection and inference. NR has designed EVM to assess model generalization performance which has been used without noticeable limitations. The same EVM can be adopted in 6GR.

Proposal 22: Adopt NR’s AI/ML evaluation methodology for model generalization performance evaluation for 6GR. The following cases for verifying the generalization performance of an AI/ML model over various scenarios/configurations:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

Detailed evaluation assumptions, methodologies, intermediate KPIs, and ultimate KPIs can be discussed separately per AI/ML use cases.

# Conclusion

This paper discussed the 6GR AI/ML framework, evaluation methodology, and use cases for 6GR AI/ML for air interface study. Based on the analysis and evaluation, the following proposals were proposed:

## CSI prediction and CSI-RS overhead reduction:

Proposal 1: For CSI prediction and CSI-RS overhead reduction study,

- Use case

  - Depending on FD and SD parameters, categorize frequency domain prediction, spatial domain prediction, and frequency-spatial domain prediction separately

  - Input: Measurement of channel with sparse/low overhead CSI-RS

  - Output: Full channel matrix

  - Label: Estimated/ideal channel matrix based on full CSI-RS density

  - Model location: UE-side model

- Evaluation assumption:

  - KPI: SGCS, Throughput, Ratio of CSI-RS overhead

  - Benchmark: Genie-aided approach, traditional channel interpolation algorithm

- Aspects for further study

  - Port pattern design considering the trade-off between the performance, overhead and complexity

  - Generalization for deployment scenarios, antenna configurations, and port patterns

  - Scalability for frequency domain factor (e.g., bandwidth) and spatial domain factor (e.g., number of antenna ports)

  - Coexist with non-AI UEs

Specification impact: inference configuration, reporting content, signaling and procedure for LCM

Proposal 2: For sub-case B, i.e., temporal-domain CSI prediction

- Use case

  - Input: channel matrices from measurements and/or predictions earlier than the CSI report

  - Output: channel matrices for time slots later than the CSI report

  - Label: ground-truth channel matrices for time slots later than the CSI report

  - Model location: UE-side model

- Evaluation assumption:

  - KPI: SGCS, Throughput, Ratio of CSI-RS overhead

  - Benchmark: Non-predictive CSI report (sample-and-hold)

- Aspects for further study

  - NR studies can be reconsidered

  - Extensions and enhancement from NR study including extension to more than 32 ports, configurations for CSI-RS overhead reduction, network-side additional conditions handling, e.g., antenna layout, deployment scenario, site-specific models

- Specification impact: measurement and reporting configurations, signaling and procedure for LCM

Proposal 3: For CSI prediction and CSI-RS overhead reduction study,

- Use case

  - Input: Channel matrix of carrier/band/frequency block A

  - Output: Channel matrix of carrier/band/frequency block B

  - Label: Channel matrix of carrier/band/frequency block B

  - Model location: UE-side model

- Evaluation assumption:

  - KPI: SGCS, NMSE, Throughput

  - Benchmark: Ground-truth of target frequency block, sample and hold

- Aspects for further study

  - Generalization for channel scenarios, UE distribution, and subcarrier spacing

  - Scalability for bandwidth and frequency gap between the source and target frequency bands

  - Coexist with non-AI UEs

- Specification impact: Inference configuration, signaling and procedure for LCM

Proposal 4: For CSI prediction and CSI-RS overhead reduction study,

- Sub-use case D: CSI prediction across analog beams

  - Input: Channel matrix of Set B of beams (measurement beams), target data beam index (optional)

  - Output: Channel matrix of Set A of beams (data beams)

  - Label: Channel matrix of Set A of beams

  - Model location: UE (DL), BS (UL, for SRS overhead reduction)

- Evaluation assumption:

  - KPI: SGCS, Throughput (or SE), Ratio of CSI-RS overhead

  - Benchmark: Ground-truth of Set A of beams

- Aspects for further study

  - Generalization for channel scenarios, antenna configurations, and port patterns

  - Coexist with non-AI UEs

- Specification impact: Inference configuration, signaling and procedure for LCM

## DMRS design with AI receiver:

Proposal 23: For study on DMRS design with AI receiver, consider the following:

- Sub-Case clarification:

  - Receiver assumptions for each sub-use case.

- Label:

  - For the applicable sub-cases and receiving assumption, only estimated channel information is used and do not consider ideal channel information as label.

  - Initial results showed performance loss when using estimated channel as label compared with using ideal channel information as label.

    - With estimated channel @30dB as label, the AI receiver + low density DMRS still outperforms legacy receiver + 1/2 FD density DMRS.

- Benchmark:

  - Both normal DMRS overhead and low density DMRS overhead

  - Non-AI channel estimation method:

    - Ideal channel estimation

    - Conventional channel estimation with reasonable complexity.

    - Note: The complexity of non-AI receiver shall be reported.

- KPIs:

  - Performance related: Throughput, DMRS overhead, BLER, MSE (when applicable)

  - Model complexity and inference latency.

  - Overhead for fine-tuning, when applicable.

  - Inter-vendor collaboration overhead and complexity for two-sided model, when applicable.

- Study aspects:

  - Label quality impact for DMRS design with AI receiver

  - DMRS design

    - Time-domain and frequency-domain sparse DMRS pattern for Sub-case A

    - DMRS sequence/pattern design, power allocation for Sub-case B

  - Scalability on different frequency domain allocation, different time domain allocation, different number of layers, different precoders, different modulation order for DMRS design for DMRS design with AI receiver

  - Performance gain under non-ideal factor, at least including TO, FO, phase noise, interference, other distortions (e.g., PA, IQ imbalance).

  - Generalization

    - Generalization performance of AI receiver over different channels

    - Generalization performance of AI receiver over different non-ideal factors, if the trained model is used to overcome non-ideal factors

Proposal 24: For Prioritize the study on Sub-Case A: sparse orthogonal DMRS in frequency and/or time domain for the study on DMRS design with AI receiver.

Proposal 6:  For Prioritize the study on Sub-Case A: sparse orthogonal DMRS in frequency and/or time domain for the study on DMRS design with AI receiver.

## CSI compression and feedback:

Proposal 7: With the above discussion, we propose the following changes for sub-case D:

Table D

| Sub-use case | Sub-case D: CSI reconstruction with CSI feedback with SRS (assuming SSCC) |
| --- | --- |
| Reported companies | (3) Qualcomm, vivo, Samsung3 |
| Model input of decoder or model output of encoder, when applicable | 1. Compressed CSI bits - CSI measurement and channel matrix feedback for subset of CSI-RS ports3 2. Estimated channel based on SRS |
| Model output of decoder or model input of encoder, when applicable | (Reconstructed) Eigenvectors, Explicit H3 |
| Label | Eigenvectors Explicit H3 |
| Training types | Offline training |
| KPI | SGCS, UPT |
| Benchmark | eType II NR AI/ML CSI compression without SRS SRS without CSI feedback |
| Model location for inference | Two-sided model NW-side model3 |
| Collaboration/interaction between UE and NW | Similar to two-sided model in NR |
| Potential specification impact | 1. Necessary signaling/procedure to support lower overhead and/or simpler CSI feedback, e.g., feedback for explicit H, signalling to assist SRS and CSI combining. 2. Signalling/ procedure related to LCM for two-sided model including inter-vendor collaboration |

Proposal 8: Prioritize the study on CSI compression with NW-sided model.

Proposal 9: For study on CSI compression, consider the following aspects:

- KPIs

  - Intermediate KPIs: SGCS, NMSE

  - System KPIs: throughput, overhead

  - Model complexity at NW side

  - Model/computation complexity at UE side

  - Computation latency at UE side

  - Inter-vendor collaboration overhead and complexity for two-sided model, if applicable.

- Benchmark:

  - eTypeII

  - CSI compression with two-sided model with channel coding and modulation with same/similar model complexity, when applicable.

- Study aspects:

  - Generalization performance

    - Different DL channel

    - Different UL channel, when applicable

  - Scalability: e.g., antenna ports, subband number, number of UL resources for UCI

  - Spec impact

    - Data collection for training

    - Inter-vender collaboration for two-sided model, when applicable

    - Configuration for inference, e.g., downloadable compression matrix, downloadable basis

    - Performance monitoring including necessity and method

## AI for beam management and extension

Proposal 10: Consider NR AI/ML for beam management as 6GR use case for study [in MIMO agenda item].

Proposal 11: For AI/ML for beam management and extension, support the study of the following

- Sub-case A: Inter-Cell/M-TRP DL Tx beam prediction and management [in MIMO agenda item]

- Sub-case D: Beam prediction for initial access [in initial access agenda item]

## AI for non-linearity compensation

Proposal 12: adopted following change in table G in observation from RAN1#122bis:

| Benchmark | Without compensation, GMP model based NC1 | No DPD |
| --- | --- | --- |

Proposal 13: the study of AI-NC in RAN1 should consider DMRS design to have similar PAPR level as corresponding data part in the same channel.

## AI-enabled uplink precoding indication

Proposal 14: Support the study of Dlable UL codebooks [in MIMO agenda item], including the following

- Signaling/procedure related to the download of UL codebooks

- Signaling/procedure to facilitate development or performance monitoring of the Dlable UL codebooks

## AI for PAPR reduction

Proposal 15: For AI for PAPR reduction for waveform study,

- Use case: transformed symbols symbol in frequency domain/ the modulated symbol in frequency domain /label free

- Evaluation assumption:

  - KPI: BLER, CCDF of PAPR, throughput

  - Benchmark: DFT-s-OFDM

- AI/ML specific assumption/aspects

  - Generalization must be demonstrated across various channel conditions, modulation schemes, and resource allocation patterns

  - Inter-vendor collaboration (fully specified reference model)

- Specification impact: inter-vendor collaboration, waveform switching

## AI for PRACH detection

Proposal 25: Study on AI-enable PRACH receiver for early contention resolution with enhanced random access signaling and procedure as well as corresponding LCM procedure. update the Table in observation from RAN1#122bis as following:

| Use case | AI-enable PRACH receiver |
| --- | --- |
| Model input | Detected PRACH signal with PDP or truncated PDP in time domain for each preamble detection window |
| Model output | Predicted number of UEs that transmitted the same preamble for given PRACH resources |
| Label | Ground-truth number of UEs that transmitted the same preamble |
| Training types | offline training |
| KPI | Missing detection rate with UE multiplicity, and false alarm detection rate |
| Benchmark | Legacy receiver |
| Model location for inference | NW-sided model |
| Collaboration/interaction between UE and NW | no collaboration or Similar to NW-sided model in NR |
| Potential specification impact | - signalling/ procedure related to random access for more than one UEs with selected the same PRACH preamble. - Signalling/procedure related to LCM for NW-sided model |

## Framework and evaluations

Proposal 17: Take NR AI/ML framework as a starting point and support/adopt the following in 6GR AI/ML framework

- Terminologies in TR 38.843

- UE-side and NW-side data collection

- Applicability report

- Associated ID to indicate additional conditions that may not be explicitly configured

- Fully specified reference two-side models to address interoperability

- Performance monitoring

- Dedicated AI/ML processing unit (APU) and timeline

Proposal 18: Consider the following approaches to support site/scenario-specific models

- Non-linear approaches: Parameter transfer for input/output adaptation layers of specified model structure

- Linear approaches: downloadable projection/basis matrices for input/output processing

Proposal 19: Considering online training/fine-tuning in 6GR AI/ML study, at least for the use case that only requires lightweight model.

Proposal 20: Study the potential impact of AI/ML memory on the concurrently activated AI/ML features/models, including whether to introduce AI/ML memory unit (MU) taking the NR’s active CSI-RS resource and ports counting as a starting point.

Proposal 21: For 6GR use-case studies, adopt intermediate and ultimate (eventual/system) KPIs as in the NR.

- Intermediate KPIs to evaluate model-specific performance, e.g., model performance-complexity trade-off, generalization performance, monitoring accuracy, training and dataset aspects.

- Ultimate KPIs to assess overall performance benefits of AI/ML use cases.

Proposal 22: Adopt NR’s AI/ML evaluation methodology for model generalization performance evaluation for 6GR. The following cases for verifying the generalization performance of an AI/ML model over various scenarios/configurations:

- Case 1: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a dataset from the same Scenario#A/Configuration#A

- Case 2: The AI/ML model is trained based on training dataset from one Scenario#A/Configuration#A, and then the AI/ML model performs inference/test on a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B

- Case 3: The AI/ML model is trained based on training dataset constructed by mixing datasets from multiple scenarios/configurations including Scenario#A/Configuration#A and a different dataset than Scenario#A/Configuration#A, e.g., Scenario#B/Configuration#B, Scenario#A/Configuration#B, and then the AI/ML model performs inference/test on a dataset from a single Scenario/Configuration from the multiple scenarios/configurations, e.g.,  Scenario#A/Configuration#A, Scenario#B/Configuration#B, Scenario#A/Configuration#B.

# Reference

[1] R1-2507259 AI/ML Use cases and framework for 6GR, Samsung

[2] R1-2508118 Feature lead summary #5 of 6GR AI/ML, Samsung (Moderator)

[3] R1-2307670 Evaluation on AI/ML for beam management, Samsung

[4] S. Xiao et. al. “A Digital Correction Method to Reduce the Nonlinearity of Terahertz Transmitters” (GC 21)

[5] 3GPP TR 38.803 V14.3.0 (2022-03), Study on new radio access technology: Radio Frequency (RF) and co-existence aspects (Release 14)

# Annex A: SRS power imbalance

SRS power imbalance, is mainly a result of insertion loss where uneven impedance between the Tx RF chain and the SRS Tx antennas (DL Rx antennas) results in uneven power among SRS ports. To make things worse, the imbalance is time-varying as it is impacted by unpredictable device-side conditions, e.g., status of camera. For the true DL channel matrix, denoted by HDL, the effect of SRS power imbalance can be modeled as Hsrs=ΑHDL, where, Α=diag1,α2…αNTX is a diagonal matrix with diagonal coefficient αi  representing the power ratio of the i-th antenna with respect to the antenna that has highest power. Figure 21 shows the SGCS degradation (as compared to the reference, i.e., unit SGCS) as a result of channel ageing and power imbalance. The results are generated based on simulation parameters listed in Table 12.

Figure 21 Impact of channel ageing and SRS power imbalance in the DL channel acquisition

Table 12 Simulation parameters for results for joint SRS and CSI feedback

| Parameter | Value | Parameter | Value |
| --- | --- | --- | --- |
| M (measured CSI-RS ports) | 64 | Channel | UMa Dense-urban |
| N (NW antenna ports) | 256 | SRS measurement interval | 30ms |
| Nr (number of UE receive ports) | 4 | Mean power imbalance (4 antennas) | [0 -3 -6 -9]dB |
| Carrier frequency/BW | 7 GHz/ 20MHz 13 SBs | Power imbalance distribution | Uniform [-1,1] dB centered on the mean. |

# Annex B: JSCM SLS results

Models have been trained for each cell based on data collected across 10 random drops. The number of SBs NSB=13 with SB size = 4 PRBs.

Table 13. Evaluation assumptions for sTRP

| Parameters | Values |
| --- | --- |
| Carrier Frequency | 4 GHz (sTRP) |
| Bandwidth | 10 MHz |
| Subcarrier Spacing | 15kHz |
| Scenario | Dense Urban, ISD = 200m |
| BS Tx power | 41 dBm (each TRP) |
| BS Antenna Configuration | 64 ports: (16,4,2,1,1,8,4), (dH,dV) = (0.5, 0.8)λ |
| Number of UE antennas | 4 |
| Traffic Model | Non-Full buffer, FTP model 1 High RU (RU 60~70) |
| Scheduling | MU-MIMO PF |
| Max layers for MU | 4 |
| User rank | 1 |
| Number of Rings | 1 Rings, (=21 sectors) |
| Number of UEs per cell | 10 |

# Annex C: Sparse DMRS results with Alt 2 receiver (AI-based channel estimation for DMRS and data REs)

To study the sparse orthogonal RS design with lower overhead, the following DMRS patterns are considered for evaluation with different AI based channel estimation or receivers’ assumption in both frequency domain and time domain.

As benchmark, the existing NR Rel-15 type 1 DMRS structure can be referred as Pattern 1 in Error! Reference source not found. (a), where FD OCC as 2, FD density =1/2, and number of DMRS symbols is configured. It was selected as the typical assumption to specify the PUSCH demodulation performance with fading channel condition. Pattern 3 in Error! Reference source not found. (b), is designed with simply reducing the frequency density, with a FD density =1/6. The power boost setting is 3dB for pattern 1 and 7.78dB for pattern 3. Meanwhile, we also considered the case with no power boosting, where the REs between DMRS REs are filled with data.

(a) DMRS pattern 1, FD OCC=2, FD density =1/2,

Rel-15 NR type 1 with 2 symbols (with power boosting)

(b) DMRS pattern 3, FD OCC=2, FD density =1/6 (with power boosting)

(c) DMRS pattern 1, FD OCC=2, FD density =1/2,

Rel-15 NR type 1 with 2 symbols (without power boosting)

(d) DMRS pattern 3, FD OCC=2, FD density =1/6 (without power boosting)

Figure 22 DMRS patterns (with power boosting)

Label quality impact

To study the label quality impact. Two schemes were utilized for label construction of model training:

- Ideal CE labels: Uses perfect channel information (across both DMRS and data REs) as training labels.

- 30dB estimated labels: LS channel estimation on DMRS REs, followed by Frequency Domain and Time Domain linear interpolation (based on Pattern 1 configuration) to construct training labels.

Figure 23 shows the NMSE performance conducted under channel models CDLC300-11 and CDL300-667. Overall, the Alt2 AI/ML receiver consistently improves NMSE performance under both label assumptions (ideal CE and 30dB estimation labels).

- Performance analysis for DMRS Pattern 1 & 3

For DMRS Pattern 1, under the CDLC300-11 channel condition, there is a minor performance difference between models trained with "30dB estimated labels" versus "ideal CE labels". However, in the more challenging CDL300-667 channel, the model using "30dB estimated labels" shows an approximate 3dB performance loss compared to the "ideal CE labels" model.

For DMRS pattern 3, models trained with "30dB estimated labels" exhibit up to a 3dB performance loss (@30%BLER) compared to those using "ideal CE labels" in both channel conditions. This indicates that label quality is critical for AI/ML training with reduced DMRS density.

- AI/ML vs. Legacy Receiver Comparison

It is worth noting that Alt2 AI/ML receiver achieves better NMSE performance even with the lower-density DMRS Pattern 3, compared to the traditional legacy receiver using the higher-density Pattern 1 configuration (specifically under the CDLC300-11 channel). Under the CDLC300-11 channel, the Pattern 3 + Alt2 receiver only experiences approximately a 1dB loss compared to the Pattern 1 + legacy receiver setup.

- Result summary

The results confirm the potential of AI/ML receivers in improving channel estimation accuracy. It is possible to achieve or even surpass the performance of traditional high-density DMRS schemes using a lower-density DMRS Pattern 3 configuration, thereby improving overall spectral efficiency.

(a) CDLC300-11                                              (b) CDLC300-667

Figure 23 NMSE performance comparison with legacy receiver and AI/ML based receiver (Alt 2), 8RB

Figure 24 shows the BLER performance conducted under channel models CDLC300-11 and CDL300-667.

- Performance analysis for DMRS Pattern 1 & 3

Using the practical "Label from 30dB" approach introduces minor performance degradation compared to the idealistic "Label from ideal CE" assumption. To be specific, under CDLC300-11, approximately 0.1dB loss for both DMRS Pattern 1 and Pattern 3. Under CDLC300-667 Channel, around 0.2dB loss observed for Pattern 1. Up to 0.8dB loss is observed for Pattern 3, indicating higher sensitivity to label quality when DMRS density is lower.

- AI/ML vs. Legacy Receiver Comparison

The Alt2 AI/ML receiver consistently outperforms the traditional legacy receiver when trained with practical (30dB estimated) labels. Up to 2dB gain is observed for alt2 receiver with label from 30dB for DMRS pattern3, and similar performance for DMRS pattern 1 in CDLC300-11 compared to legacy receiver. Up to 3dB gain is observed for alt2 receiver with label from 30dB for DMRS pattern3, and around 0.5dB gain for DMRS pattern1 in CDLC300-667 compared to legacy receiver.

- Result summary

The results demonstrate the significant benefits of the AI/ML (Alt2) receiver architecture. Even when trained with non-ideal labels (30dB CE), the AI/ML receiver provides substantial BLER gains over legacy approaches, particularly when utilizing the lower-density DMRS Pattern 3 in challenging channel conditions (CDLC300-667). This validates the feasibility of deploying AI/ML solutions to enhance spectral efficiency.

(a) CDLC300-11                                              (b) CDLC300-667

Figure 24 BLER performance comparison with legacy receiver and AI/ML based receiver (Alt 2), 8RB

Power boosting impact

To test the gain loss of non-power boosting case, we further evaluate the impact of enabling/disabling power boosting on the Block Error Rate (BLER) performance at the 30% BLER operating point, comparing traditional (Legacy) receivers with the AI/ML (Alt2) receiver architecture.

- Legacy Receiver

The legacy receiver shows noticeable performance degradation when power boosting is removed, demonstrating its reliance on the boosted DMRS signal strength for accurate channel estimation. For Pattern 1, approximately 0.5dB to 0.8dB BLER degradation observed without power boosting in both CDLC300-11 and CDLC300-667 channels. For Pattern 3, under CDLC300-11, around 1dB BLER degradation without power boosting. Under CDLC300-667, only a minor difference observed in this more challenging channel, suggesting other channel impairments might dominate the performance here.

- AI/ML (Alt2) Receiver

The Alt2 receiver exhibits only minor BLER degradation without power boosting compared to the configuration with power boosting for both DMRS Pattern 1 and Pattern 3 across both channels. This highlights the AI/ML model's ability to effectively estimate channels even with non-power boosting DMRS signals.

Under CDLC300-11, a gain of approximately 0.5dB to 1dB is achieved for the Alt2 receiver (for both Pattern 1 and Pattern 3) compared to the legacy receiver. Under CDLC300-667, the Alt2 receiver provides a substantial gain of around 3dB (for both Pattern 1 and Pattern 3) over the legacy receiver in this challenging environment.

- Result summary

The AI/ML (Alt2) receiver demonstrates greater robustness and efficiency, showing minimal impact from the removal of power boosting and superior performance compared to the legacy approach.

(a) CDLC300-11, Pattern 1                            (b) CDLC300-11, Pattern 3

(c) CDLC300-667, Pattern 1                       (d) CDLC300-667, Pattern 3

Figure 25 BLER performance comparison with legacy and AI/ML based receiver w/wo power boosting (Alt 2), 8RB