3GPP TSG RAN WG1 #122			R1-250XXXX

Bengaluru, India, Aug 25th – 29th, 2025

Agenda Item:	8.13

Source:	Ad-Hoc Chair (Huawei)

Title:	Session notes for 8.13 Maintenance on LTE-based 5G Broadcast Phase 2

Document for:	Discussion, Decision

## Maintenance on LTE-based 5G Broadcast Phase 2

[122-R19-Broadcast] Email discussion on Rel-19 LTE-bad 5G Broadcast Phase 2 – Assunta (EBU)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

### Time-frequency interleaver design for MBMS dedicated cells

R1-2505232	Maintenance for LTE based 5G broadcast	Huawei, HiSilicon

R1-2505505	Discussion on Time-frequency interleaver design for MBMS dedicated cells	ZTE Corporation, Sanechips

R1-2505559	Time-frequency interleaver design for MBMS dedicated cells	Samsung

R1-2505645	Maintenance on LTE-based 5G Broadcast	Xiaomi

R1-2506194	Time and Frequency Interleaving for 5G Broadcast	Qualcomm Incorporated

R1-2506499	Feature lead summary #1 on TFI for LTE-based 5G Terrestrial Broadcast (maintenance)	Moderator(EBU)

Agreement (updated on Wednesday)

Adopt the following text proposal for TS 36.211:

- Reason for change: In some cases, the value of d in the frequency interleaver may be negative or zero, in which case the row permutation is incorrect;

- Summary of changes: Clarify that row permutation is only applied if RK-K+1>0;

- Consequence if not approved: Specification is not implementable for cases of RK-K+1≤0.

******** start of TP *************

6.5.2 Frequency Domain interleaving

The input to the interleaver is the block of complex-valued symbols yp0, …, yp(Msymbap-1)  defined in clause 6.3.4.1 and the output from the interleaver is a block of complex-valued symbols yp0, …, yp(Msymbap-1).

<<unchanged text>>

Permute the rows , if RK-K+1>0, such that row ri is moved to row i where

ri=ri-1+dri-1≤R-d-1idRotherwise
d=RRRK-K+1

******** end of TP *************

Note: it can be discussed in a future meeting whether an essential correction is needed for the case where RK-K+1≤0.

Agreement

The contents to be included in the MBMS-ROM-Info-19 are as follows:

MBMS-ROM-Info-r19 ::= SEQUENCE { mbms-ROM-Freq-r19 ARFCN-ValueEUTRA-r9, mbms-ROM-SubcarrierSpacing-r19 ENUMERATED {kHz15, kHz7dot5, kHz2dot5, kHz1dot25}, mbms-Bandwidth-r19 ENUMERATED {n6, n15, n25, n30, n35, n40, n50, n75, n100}

Agreement

To maintain backwards compatibility with legacy non-time-frequency-interleaved PMCHs associated an MCCH, frequency interleaving is not applied to MCCH and MSI.

Agreement

Additional contents to be included in the MBMS-ROM-Info-19 is as follows:

β and Nsoftref as described in TS 36.212, for MCH with time-interleaving

R1-2506544	Feature lead summary #2 on TFI for LTE-based 5G Terrestrial Broadcast (maintenance)	Moderator(EBU)

Agreement

Adopt the following text proposal for TS 36.211, with a separate UE capability for the newly added alternative:

- Reason for change: In some cases, it is observed that an alternative value of the cyclic shift may provide performance improvement;

- Summary of changes: Add two new methods for cyclic shift;

- Consequence if not approved: New values of cyclic shift are not supported.

6.5.1  Cyclic Shift for PMCH

The input to the cyclic shift is the block of bits b0, …, bMbit-1 defined in clause 6.3.1 and the ouput is a block of bits b0, …, bMbit-1.

The block of bits b0, …, bMbit-1 shall be cyclically shifted by Xi bits to form the block b0, …, bMbit-1 according to

bn=b(n-Xi mod Mbit)

where

-	Xi is given by

Xi=r=C-SiC-1Er

Si=iα mod C

-	i∈0, 1, …, N-1 denotes the ith subframe to which the transport block is mapped

-	N is the number of subframes to which the transport block is mapped

-	Er is the number of bits in the rth codeblock within a subframe as defined in TS 36.212 [3]

-	C is the number of codeblocks in the transport block

-	α is given by

-	if the higher-layer parameter XXX is set to alphaOne

α=1

-	if the higher-layer parameter XXX is set to alphaOther

α=CNL

- if the higher-layer parameter XXX is set to alphaOther2

Xi= Ainf ,nSF×floor(NSC ×QmN), 	Ainf ,nSF ∈{0,…N-1}

where Ainf ,nSF= m=07c8nf mod 12810+nSF+m2mmod N where nf is the system frame number and nSF is subframe number within the frame. The pseudo-random sequence ci is defined by clause 7.2 and shall be initialized with cinit=NIDMBSFN at the beginning of each radio frame for which nf mod 128=0, where  NSC  is the number of subcarriers available in 1 OFDM symbol for PMCH,  NSC =Mbit/(QmL), Qm is the modulation order.

- where L is the number of OFDM symbols used for the PMCH transmission.