3GPP TSG RAN WG1 #124			R1-2601501

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Agenda Item:	8.6

Source:	Ad-Hoc Chair (NTT DOCOMO, INC.)

Title:	Session Notes of AI 8.6

Document for:	Endorsement

  - Maintenance on Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

Note: Maximum one contribution per company/organization/university. For efficient review, please use the following sections in your contribution corresponding to the maintenance issues, if any:

- LP-WUS and LP-SS design

- LP-WUS operation in IDLE/INACTIVE modes

- LP-WUS operation in CONNECTED modes

[124-R19-LP-WUS] Email discussion on LP-WUS – Xueming (vivo)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2601501	Session Notes of AI 8.6	Ad-Hoc Chair (NTT DOCOMO, INC.)

R1-2601580	Summary #1 of maintenance on Low-power wake-up signal and receiver for NR (LP-WUS/WUR)	Moderator (vivo)

Agreement:

Adopt the following TP to section 7.4.5.1.1, TS 38.211 as alignment CR

---------------------------------Start of Text Proposal on 3GPP TS 38.211 V19.0.0-----------------------

<Unchanged part is omitted>

7.4.5.1.1	Generation of rOOK(n)

The sequence rOOK0,…,rOOK(NOOK-1) is defined by Tables 7.4.5.1.1-1 to 7.4.5.1.1-3 with the quantity MLPSS given by the higher-layer parameter lpss-MvalueAndSeqConfig XXX , the sequence length by the higher-layer parameter lpss-BinarySeqLen, and the configuration index by the higher-layer parameter lpss-BinarySeqIndex.

<Unchanged part is omitted>

--------------------------------------End of Text Proposal on 3GPP TS 38.211 V19.0.0 ------------------

Agreement:

Adopt the following TP to section 7.4.5.1.2, TS 38.211 as alignment CR

---------------------------------Start of Text Proposal on 3GPP TS 38.211 V19.0.0----------------------- <Unchanged part is omitted> ##### 7.4.5.1.2 Generation of rZC(n) If the quantity qϵ1,…,NZC-1 is configured by the higher-layer parameter XXX lpss-OverlaidSeqRoots, the sequence rZC(n) is defined by <Unchanged part is omitted> --------------------------------------End of Text Proposal on 3GPP TS 38.211 V19.0.0 ------------------

Agreement:

Adopt the following TP to section 7.4, TS 38.212 as alignment CR

-------------------------------Start of Text Proposal on 3GPP TS 38.212 V19.0.0----------------------- <Unchanged part is omitted> - 7.4 Wake-up information The wake-up information is carried by a wake-up signal as defined in clause 7.4.4 of [4, TS 38.211]. - For a UE configured with higher layer parameter LP-WUS_LP-SS_startRB_IDLE_INACTIVE lpwus-LPSS-StartRB and operating in the RRC_IDLE or RRC_INACTIVE state, the wake-up information bit sequence c0,c1,…,cK-1 is the binary sequence of the codepoint as defined by Clause 10.4C of [5, TS38.213], where c0 is the most significant bit and K is - maxlog2NPO, 1 if NsubgroupPO=1, where NPO is configured by higher layer parameter lpwus-PO-NumPerLOPO-to-LO association and NsubgroupPO is configured by higher layer parameter lp-SubgroupsNumPerPO subgroupNumber_PO_LPWUS; - log2NPONsubgroupPO+1 if NsubgroupPO>1, where NsubgroupPO is configured by higher layer parameter lp-SubgroupsNumPerPO subgroupNumber_PO_LPWUS; - For a UE configured with higher layer parameter lpwus-StartRB LP-WUS_startRB_CONNECTED and operating in the RRC_CONNECTED state, the wake-up information bit sequence c0,c1,…,cK-1 is the binary sequence of the codepoint as defined by Clause 10.4D of [5, TS38.213], where c0 is the most significant bit, K is provided by the higher layer parameter lpwus-NumOfBits Num_info_bits_WUS_CONNECTED. <Unchanged part is omitted> --------------------------------------End of Text Proposal on 3GPP TS 38.212 V19.0.0 ------------------

Agreement:

Adopt the following TP to section 7.4.2.1 and 7.4.2.2, TS 38.212 as alignment CR

---------------------------------Start of Text Proposal on 3GPP TS 38.212 V19.0.0----------------------- <Unchanged part is omitted> - 7.4.2.1 Rate matching for OOK modulation The input bit sequence to rate matching is d00,d01,…,d0N0-1. Rate matching is performed according to Clause 5.4.3 by setting the rate matching output sequence length E=E0, where E0=EWUS, EWUS=NOS×MLP/2, where - for a UE configured with higher layer parameter LP-WUS_LP-SS_startRB_IDLE_INACTIVE lpwus-LPSS-StartRB and operating in the RRC_IDLE or RRC_INACTIVE state, NOS is configured by higher layer parameter LP-WUS_ActualMO_duration_IDLE/INACTIVE lpwus-ActualDuration and MLP is configured by higher layer parameter LP-WUS_Mvalue_IDLE/INACTIVE lpwus-MvalueAndSeqConfigFR1 or lpwus-MvalueAndSeqConfigFR2; - for a UE configured with higher layer parameter LP-WUS_startRB_CONNECTED and operating in the RRC_CONNECTED state, NOS is configured by higher layer parameter LP-WUS_ActualMO_duration_CONNECTED lpwus-ActualDuration and MLP is configured by higher layer parameter LP-WUS_Mvalue_CONNECTED lpwus-MvalueAndSeqConfigFR1 or lpwus-MvalueAndSeqConfigFR2. The output bit sequence after rate matching is denoted as f00,f01,f02,…,f0E0-1. 7.4.2.2 Rate matching for sequence modulation Information bits for the second bit block are delivered to the rate matching block. They are denoted by c0,c1,…,cK-1, where K is the number of bits and K≤5. If the number of sequences configured by higher layer parameter lpwus-OverlaidSeqNum or lpwus-OverlaidSeqNum-SCS-60kHz or lpwus-OverlaidSeqNum-SCS-120kHz LP-WUS_num_overlaidSeq_CONNECTED or LP-WUS_num_overlaidSeq_IDLE/INACTIVE, denoted as L1, is larger than one, padding is performed and the bits after padding are denoted by d10,d11,…,d1N1-1, where N1=K+L, L=-K mod log2L1. The relation between ck and d1k is: d1k=0 for k=0,1,…, L-1 d1k=ck-L for k=L,L+1,…, N1-1. <Unchanged part is omitted> --------------------------------------End of Text Proposal on 3GPP TS 38.212 V19.0.0 ------------------

Agreement:

Adopt the following TP to section 10.4C TS 38.213 in principle

| Reason for change: | The conclusion of No explicit gap is introduced between two consecutive LP-WUS nominal MOs is not reflected in TS 38.213. |
| --- | --- |
| Summary of change: | For both idle/inactive and connected mode operation, clarify there is no time gap between two consecutive nominal WUS MOs. |
| Consequences if not approved: | Ambiguity for a UE to determine the location of subsequent WUS MOs. |
| ---------------------------------Start of Text Proposal on 3GPP TS 38.213 V19.0.0----------------------- *** Unchanged parts are omitted *** 10.4C PDCCH monitoring activation by WUS in RRC_IDLE/RRC_INACTIVE *** Unchanged parts are omitted *** A UE assumes that WUS occasions occur with a periodicity equal to the I-DRX cycle in the RRC_IDLE/RRC_INACTIVE state [17, TS 38.304]. The UE determines WUS occasions associated with a paging occasion based on PO-to-LO association. A first frame of a WUS occasion starts a number of frames prior to the first of a number of paging frames associated with the WUS occasion [17, TS 38.304]. The first WUS monitoring occasion of a WUS occasion starts at an offset provided by offset_firstMO_withinLO relative to the start of the first frame. A UE assumes WUS monitoring occasions per periodicity are consecutive in time. *** Unchanged parts are omitted *** 10.4D PDCCH monitoring activation by WUS in RRC_CONNECTED *** Unchanged parts are omitted *** A UE can be provided by WUS-MOCONNECTED-Option1-2 a periodicity, by periodicityMO-Option 1-2, and a time offset, by offsetMO-Option 1-2, relative to the start of a system frame with SFN 0, for the UE to determine first WUS monitoring occasions from a number of WUS monitoring occasions per periodicity, provided by numMO-perPeriodicity-Option 1-2. A UE assumes WUS monitoring occasions per periodicity are consecutive in time. *** Unchanged parts are omitted *** ---------------------------------End of Text Proposal on 3GPP TS 38.213 V19.0.0----------------------- |  |

Draft CR is endorsed in R1-260xxxx

Final CR is endorsed in R1-260xxxx

Agreement:

Adopt the following TP to Clause 5.1.5, TS 38.214.

Reason for change: The UE assumption on QCL information before applying an indicated TCI state is not yet described in specification. Summary of Change: Clarify the assumption for QCL information during WUS monitoring before applying the indicated TCI state by updating the existing clause 5.1.5 to add WUS in addition to CSI-RS and DMRS of PDSCH/PDCCH. Consequences if not approved: Unclear UE behaviour for WUS monitoring prior to applying an indicated TCI state.

---------------------------------Start of Text Proposal on 3GPP TS 38.214 V19.0.0----------------------- <Unchanged part is omitted> After a UE receives an initial higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state and before application of an indicated TCI state from the configured TCI states: - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH that are not received during the RACH procedure, WUS, and the CSI-RS applying the indicated TCI state are quasi co-located with the reference signal(s) in the CandidateTCI-State indicated in the LTM Cell Switch Command MAC CE [10, 38.321] if applicable, otherwise. - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH, WUS, and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block the UE identified during the initial access procedure <Unchanged part is omitted> After a UE receives a higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state as part of a Reconfiguration with sync procedure as described in [12, TS 38.331] and before applying an indicated TCI state from the configured TCI states: - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH, WUS, and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block or the CSI-RS resource the UE identified during the random access procedure initiated by the Reconfiguration with sync procedure as described in [12, TS 38.331]. <Unchanged part is omitted> ---------------------------------End of Text Proposal on 3GPP TS 38.214 V19.0.0-----------------------

Final CR is endorsed in R1-260xxxx

R1-2600082	Maintenance on  LP-WUS/WUR	Huawei, HiSilicon

R1-2600173	WUS monitoring occasions for LP-WUS/WUR	OPPO

R1-2600214	Maintenance on low-power wake-up signal and receiver for NR	Ericsson

R1-2600476	Maintenance on NR Rel-19 LP-WUS	vivo

R1-2600529	Maintenance on LP-WUS operation	InterDigital, Inc.

R1-2600597	Maintenance of LP-WUS Operation	Ofinno

R1-2600730	Maintenance on low-power wake-up signal and receiver for NR	Samsung

R1-2601215	Maintenance on  LP-WUS	ZTE Corporation, Sanechips