3GPP TSG RAN WG1 #124			R1-2601501

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Agenda Item:	8.6

Source:	Ad-Hoc Chair (NTT DOCOMO, INC.)

Title:	Session Notes of AI 8.6

Document for:	Endorsement

  - Maintenance on Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

Note: Maximum one contribution per company/organization/university. For efficient review, please use the following sections in your contribution corresponding to the maintenance issues, if any:

- LP-WUS and LP-SS design

- LP-WUS operation in IDLE/INACTIVE modes

- LP-WUS operation in CONNECTED modes

[124-R19-LP-WUS] Email discussion on LP-WUS – Xueming (vivo)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2601501	Session Notes of AI 8.6	Ad-Hoc Chair (NTT DOCOMO, INC.)

R1-2601580	Summary #1 of maintenance on Low-power wake-up signal and receiver for NR (LP-WUS/WUR)	Moderator (vivo)

R1-2601581	Summary #2 of maintenance on Low-power wake-up signal and receiver for NR (LP-WUS/WUR)	Moderator (vivo)

Agreement:

Adopt the following TP to section 7.4.5.1.1, TS 38.211 as alignment CR

---------------------------------Start of Text Proposal on 3GPP TS 38.211 V19.2.0-----------------------

<Unchanged part is omitted>

7.4.5.1.1	Generation of rOOK(n)

The sequence rOOK0,…,rOOK(NOOK-1) is defined by Tables 7.4.5.1.1-1 to 7.4.5.1.1-3 with the quantity MLPSS given by the higher-layer parameter lpss-MvalueAndSeqConfig XXX , the sequence length by the higher-layer parameter lpss-BinarySeqLen, and the configuration index by the higher-layer parameter lpss-BinarySeqIndex.

<Unchanged part is omitted>

--------------------------------------End of Text Proposal on 3GPP TS 38.211 V19.2.0 ------------------

Agreement:

Adopt the following TP to section 7.4.5.1.2, TS 38.211 as alignment CR

---------------------------------Start of Text Proposal on 3GPP TS 38.211 V19.2.0----------------------- <Unchanged part is omitted> ##### 7.4.5.1.2 Generation of rZC(n) If the quantity qϵ1,…,NZC-1 is configured by the higher-layer parameter XXX lpss-OverlaidSeqRoots, the sequence rZC(n) is defined by <Unchanged part is omitted> --------------------------------------End of Text Proposal on 3GPP TS 38.211 V19.2.0 ------------------

Agreement:

Adopt the following TP to section 7.4, TS 38.212 as alignment CR

-------------------------------Start of Text Proposal on 3GPP TS 38.212 V19.2.0----------------------- <Unchanged part is omitted> - 7.4 Wake-up information The wake-up information is carried by a wake-up signal as defined in clause 7.4.4 of [4, TS 38.211]. - For a UE configured with higher layer parameter LP-WUS_LP-SS_startRB_IDLE_INACTIVE lpwus-LPSS-StartRB and operating in the RRC_IDLE or RRC_INACTIVE state, the wake-up information bit sequence c0,c1,…,cK-1 is the binary sequence of the codepoint as defined by Clause 10.4C of [5, TS38.213], where c0 is the most significant bit and K is - maxlog2NPO, 1 if NsubgroupPO=1, where NPO is configured by higher layer parameter lpwus-PO-NumPerLOPO-to-LO association and NsubgroupPO is configured by higher layer parameter lp-SubgroupsNumPerPO subgroupNumber_PO_LPWUS; - log2NPONsubgroupPO+1 if NsubgroupPO>1, where NsubgroupPO is configured by higher layer parameter lp-SubgroupsNumPerPO subgroupNumber_PO_LPWUS; - For a UE configured with higher layer parameter lpwus-StartRB LP-WUS_startRB_CONNECTED and operating in the RRC_CONNECTED state, the wake-up information bit sequence c0,c1,…,cK-1 is the binary sequence of the codepoint as defined by Clause 10.4D of [5, TS38.213], where c0 is the most significant bit, K is provided by the higher layer parameter lpwus-NumOfBits Num_info_bits_WUS_CONNECTED. <Unchanged part is omitted> --------------------------------------End of Text Proposal on 3GPP TS 38.212 V19.2.0 ------------------

Agreement:

Adopt the following TP to section 7.4.2.1 and 7.4.2.2, TS 38.212 as alignment CR

---------------------------------Start of Text Proposal on 3GPP TS 38.212 V19.2.0----------------------- <Unchanged part is omitted> - 7.4.2.1 Rate matching for OOK modulation The input bit sequence to rate matching is d00,d01,…,d0N0-1. Rate matching is performed according to Clause 5.4.3 by setting the rate matching output sequence length E=E0, where E0=EWUS, EWUS=NOS×MLP/2, where - for a UE configured with higher layer parameter LP-WUS_LP-SS_startRB_IDLE_INACTIVE lpwus-LPSS-StartRB and operating in the RRC_IDLE or RRC_INACTIVE state, NOS is configured by higher layer parameter LP-WUS_ActualMO_duration_IDLE/INACTIVE lpwus-ActualDuration and MLP is configured by higher layer parameter LP-WUS_Mvalue_IDLE/INACTIVE lpwus-MvalueAndSeqConfigFR1 or lpwus-MvalueAndSeqConfigFR2; - for a UE configured with higher layer parameter LP-WUS_startRB_CONNECTED and operating in the RRC_CONNECTED state, NOS is configured by higher layer parameter LP-WUS_ActualMO_duration_CONNECTED lpwus-ActualDuration and MLP is configured by higher layer parameter LP-WUS_Mvalue_CONNECTED lpwus-MvalueAndSeqConfigFR1 or lpwus-MvalueAndSeqConfigFR2. The output bit sequence after rate matching is denoted as f00,f01,f02,…,f0E0-1. 7.4.2.2 Rate matching for sequence modulation Information bits for the second bit block are delivered to the rate matching block. They are denoted by c0,c1,…,cK-1, where K is the number of bits and K≤5. If the number of sequences configured by higher layer parameter lpwus-OverlaidSeqNum or lpwus-OverlaidSeqNum-SCS-60kHz or lpwus-OverlaidSeqNum-SCS-120kHz LP-WUS_num_overlaidSeq_CONNECTED or LP-WUS_num_overlaidSeq_IDLE/INACTIVE, denoted as L1, is larger than one, padding is performed and the bits after padding are denoted by d10,d11,…,d1N1-1, where N1=K+L, L=-K mod log2L1. The relation between ck and d1k is: d1k=0 for k=0,1,…, L-1 d1k=ck-L for k=L,L+1,…, N1-1. <Unchanged part is omitted> --------------------------------------End of Text Proposal on 3GPP TS 38.212 V19.2.0 ------------------

Agreement:

Adopt the following TP to section 10.4C TS 38.213 in principle

| Reason for change: | The conclusion of No explicit gap is introduced between two consecutive LP-WUS nominal MOs is not reflected in TS 38.213. |
| --- | --- |
| Summary of change: | For both idle/inactive and connected mode operation, clarify there is no time gap between two consecutive nominal WUS MOs. |
| Consequences if not approved: | Ambiguity for a UE to determine the location of subsequent WUS MOs. |
| ---------------------------------Start of Text Proposal on 3GPP TS 38.213 V19.2.0----------------------- *** Unchanged parts are omitted *** 10.4C PDCCH monitoring activation by WUS in RRC_IDLE/RRC_INACTIVE *** Unchanged parts are omitted *** A UE assumes that WUS occasions occur with a periodicity equal to the I-DRX cycle in the RRC_IDLE/RRC_INACTIVE state [17, TS 38.304]. The UE determines WUS occasions associated with a paging occasion based on PO-to-LO association. A first frame of a WUS occasion starts a number of frames prior to the first of a number of paging frames associated with the WUS occasion [17, TS 38.304]. The first WUS monitoring occasion of a WUS occasion starts at an offset provided by offset_firstMO_withinLO relative to the start of the first frame. A UE assumes WUS monitoring occasions per periodicity are consecutive in time. *** Unchanged parts are omitted *** 10.4D PDCCH monitoring activation by WUS in RRC_CONNECTED *** Unchanged parts are omitted *** A UE can be provided by WUS-MOCONNECTED-Option1-2 a periodicity, by periodicityMO-Option 1-2, and a time offset, by offsetMO-Option 1-2, relative to the start of a system frame with SFN 0, for the UE to determine first WUS monitoring occasions from a number of WUS monitoring occasions per periodicity, provided by numMO-perPeriodicity-Option 1-2. A UE assumes WUS monitoring occasions per periodicity are consecutive in time. *** Unchanged parts are omitted *** ---------------------------------End of Text Proposal on 3GPP TS 38.213 V19.2.0----------------------- |  |

Draft CR is endorsed in R1-2601589

Final CR is endorsed in R1-2601590

Agreement:

Adopt the following TP to Clause 5.1.5, TS 38.214.

Reason for change: The UE assumption on QCL information before applying an indicated TCI state is not yet described in specification. Summary of Change: Clarify the assumption for QCL information during WUS monitoring before applying the indicated TCI state by updating the existing clause 5.1.5 to add WUS in addition to CSI-RS and DMRS of PDSCH/PDCCH. Consequences if not approved: Unclear UE behaviour for WUS monitoring prior to applying an indicated TCI state.

---------------------------------Start of Text Proposal on 3GPP TS 38.214 V19.2.0----------------------- <Unchanged part is omitted> After a UE receives an initial higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state and before application of an indicated TCI state from the configured TCI states: - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH that are not received during the RACH procedure, WUS, and the CSI-RS applying the indicated TCI state are quasi co-located with the reference signal(s) in the CandidateTCI-State indicated in the LTM Cell Switch Command MAC CE [10, 38.321] if applicable, otherwise. - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH, WUS, and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block the UE identified during the initial access procedure <Unchanged part is omitted> After a UE receives a higher layer configuration of dl-OrJointTCI-StateList where more than one TCI-State can be used as an indicated TCI state as part of a Reconfiguration with sync procedure as described in [12, TS 38.331] and before applying an indicated TCI state from the configured TCI states: - The UE assumes that DM-RS of PDSCH and DM-RS of PDCCH, WUS, and the CSI-RS applying the indicated TCI state are quasi co-located with the SS/PBCH block or the CSI-RS resource the UE identified during the random access procedure initiated by the Reconfiguration with sync procedure as described in [12, TS 38.331]. <Unchanged part is omitted> ---------------------------------End of Text Proposal on 3GPP TS 38.214 V19.2.0-----------------------

Final CR is endorsed in R1-2601591

Agreement:

Adopt the following TP to section 10.4D TS 38.213.

---

========================== TP for TS 38.213 ===================================

Reason for change: The UE behavior for WUS monitoring after receiving waking-up indication is ambiguous.

Summary of changes: Add the UE behavior for WUS monitoring after receiving waking-up indication.

Consequence if not approved: The UE behavior for WUS monitoring after receiving waking-up indication is not specified and unclear.

========================== Start of TP =======================================

10.4D	PDCCH monitoring activation by WUS in RRC_CONNECTED

======================== Unchanged Text Omitted ========

If a UE detects a codepoint in a WUS reception, from the number of codepoints, on the primary cell of the cell group, the UE starts monitoring PDCCH on all applicable serving cells of the cell group, and the UE does not need to monitor WUS before lpwus-PDCCH-MonitoringTimer starts, if lpwus-PDCCH-MonitoringTimer is provided.

=========================== End of TP=======================================

---

Final CR is endorsed in R1-260xxxx

Agreement:

Adopt the following TP to section 10.4C, TS 38.213 as alignment CR:

---

======================== TP for TS 38.213 ===================================

Reason for change: RRC parameter names are not aligned with TS 38.331.

Summary of changes: Align RRC parameter names with TS 38.331.

Consequence if not approved: RRC parameter names are not aligned with TS 38.331.

========================== Start of TP =======================================

10.4C	PDCCH monitoring activation by WUS in RRC_IDLE/RRC_INACTIVE

A UE configured with DRX mode operation and operating in the RRC_IDLE or RRC_INACTIVE state can be provided for LPSS/WUS reception

-	a number of OOK symbols per OFDM symbol, a first RB by lpwus-LPSS-StartRB, and an overlaid OFDM sequence per OOK symbol for LPSS reception, and an EPRE ratio relative to SS/PBCH blocks [4, TS 38.211],

-	a number of OOK symbols per OFDM symbol, the first RB, and one or more overlaid OFDM sequences per OOK symbol for WUS reception, and an EPRE ratio relative to SS/PBCH blocks [4, TS 38.211], and

A UE determines to receive LPSS/WUS based on procedures defined in [17, TS 38.304].

A UE assumes that an SCS configuration for LPSS/WUS receptions is same as an SCS of the initial DL BWP and an SCS configuration of an SS/PBCH block the UE used to obtain SIB1.

A UE receives an LPSS in consecutive symbols within a slot. The UE can be provided one or two first symbols for respective one or two LPSS reception occasions in the slot by lpss-StartSymbol. The UE determines slots for LPSS reception occasions based on a periodicity and a time offset, relative to the start of a system frame with SFN 0, provided by lpss-PeriodicityAndOffset. Within a period of LPSS reception occasions, LPSS reception occasions are in a set of KL consecutive slots that have all symbols indicated as downlink by tdd-UL-DL-ConfigurationCommon, if provided, and start from the first slot provided by the time offset in the period, where K is the number of transmitted SS/PBCH blocks indicated by ssb-PositionsInBurst in SIB1 and L is the number of LPSS reception occasions in a slot.

LPSS reception occasions are indexed sequentially in time. An LPSS reception at the k-th LPSS reception occasion is quasi co-located with the k-th transmitted SS/PBCH block, with respect to quasi co-location 'typeC' or 'typeD' properties when applicable, where 1≤k≤K.

If a UE is provided lpwus-LPSS-BeamSubset, the UE receives LPSS/WUS based on the quasi co-location properties of transmitted SS/PBCH blocks indicated by lpwus-LPSS-BeamSubset [12, TS 38.331]; otherwise, the UE receives LPSS/WUS based on the quasi co-location properties for transmitted SS/PBCH blocks indicated by ssb-PositionsInBurst in SIB1. A WUS occasion includes K⋅M WUS monitoring occasions that are indexed sequentially in time, where

-	K is the number of transmitted SS/PBCH blocks indicated by ssb-PositionsInBurst in SIB1, M is a number of WUS monitoring occasions associated with each of the K transmitted SS/PBCH blocks provided by lpwus-MO-NumPerLO, and

-	a WUS monitoring occasion with index k-1⋅M+m-1, where 1≤m≤M and 1≤k≤K, is quasi co-located with the k-th transmitted SS/PBCH block with respect to quasi co-location 'typeC' or 'typeD' properties, when applicable

A UE can be provided, by lpwus-AvailableSlot in LowPowerConfig, a bitmap that corresponds to a set of time units that repeats continuously and indicates a subset of time units from the set of time units that is available for the UE to monitor WUS [12, TS 38.331]. A time unit includes one slot or two slots. A set of time units includes a total of either 10, or 20, or 40 time units. A duration P, in msec, of the set of time units has maximum value of 40 msec. The first symbol of the set of time units every 40 msec/P periods is a first symbol in frame nf mod 4 = 0. The UE can be additionally provided, by lpwus-AvailableSymbol in LowPowerConfig, an indication of symbols in each time unit from the subset of time units that is available for the UE to monitor WUS. If the UE is not provided lpwus-AvailableSlot, the UE assumes that all time units are available for the UE to monitor WUS. If the UE is not provided lpwus-AvailableSymbol, the UE assumes a time unit of one slot and, for a time unit that is available for the UE to monitor WUS, all symbols in the time unit are available for the UE to monitor WUS. The UE assumes that a symbol is not available to monitor WUS when

-	the symbol is indicated as uplink, by tdd-UL-DL-configurationCommon

-	the symbol is indicated for an SS/PBCH block transmission, by ssb-PositionsInBurst in SIB1, and the SS/PBCH block transmission would overlap in frequency with the WUS transmission

-	the symbol is indicated for PDCCH transmissions, by pdcch-ConfigSIB1, and CORESET 0 for the PDCCH transmissions would overlap in frequency with the WUS transmission

A WUS monitoring occasion is over a first number of symbols, provided by lpwus-NominalMoDuration in LowPowerConfig. If a number of available symbols for the UE to monitor WUS in a WUS monitoring occasion is smaller than a second number of symbols, provided by lpwus-ActualDuration in LowPowerConfig, the UE does not monitor WUS in the WUS monitoring occasion. The UE monitors WUS in a WUS monitoring occasion over the earliest available lpwus-ActualDuration symbols in the WUS monitoring occasion. If a number of available symbols for the UE to monitor WUS in a WUS monitoring occasion includes a symbol for LPSS reception, the UE does not monitor WUS in the WUS monitoring occasion.

A UE assumes that WUS occasions occur with a periodicity equal to the I-DRX cycle in the RRC_IDLE/RRC_INACTIVE state [17, TS 38.304]. The UE determines WUS occasions associated with a paging occasion based on lpwus-PO-NumPerLO. A first frame of a WUS occasion starts a number of frames prior to the first of a number of paging frames associated with the WUS occasion [17, TS 38.304]. The first WUS monitoring occasion of a WUS occasion starts at an offset provided by lpwus-OffsetFirstMoWithinLo relative to the start of the first frame.

A paging occasion associated with a WUS occasion has index iPO=UE_ID mod N⋅NS+i_s mod NPOWO where NPOWO is a number of paging occasions associated with a WUS occasion, N, NS, iSG, and i_s are defined in [17, TS 38.304], and UE_ID is defined in clause 7.1 of [17, TS 38.304]. If a number of NSGPO subgroups per paging occasion, provided by lp-SubgroupsNumPerPO, is NSGPO>1, the codepoint for the subgroup index iSG in a PO iPO is iPO*NSGPO+1+iSG, and the codepoint for all subgroups in the PO is (iPO+1)*NSGPO+1-1; otherwise, the codepoint for the PO iPO is iPO.

If, in a WUS monitoring occasion, a UE determines a codepoint associated with the UE [17, TS 38.304], the UE performs PDCCH monitoring according to Type2-PDCCH CSS sets for the paging occasion associated with the WUS monitoring occasion; otherwise, the UE is not required to perform the PDCCH monitoring. The UE may also perform PDCCH monitoring for Type2A-PDCCH CSS sets for DCI format 2_7, if provided.

========================== End of TP =======================================

---

Agreement:

Adopt the following TP to section 10.4C, TS 38.213 as alignment CR

---

==========================TP for TS 38.213 ===================================

Reason for change: RRC parameter names are not aligned with TS 38.331.

Summary of changes: Align RRC parameter names with TS 38.331.

Consequence if not approved: RRC parameter names are not aligned with TS 38.331.

=================== Start of TP =======================================

10.4C	PDCCH monitoring activation by WUS in RRC_IDLE/RRC_INACTIVE

A UE configured with DRX mode operation and operating in the RRC_IDLE or RRC_INACTIVE state can be provided for LPSS/WUS reception

-	a number of OOK symbols per OFDM symbol, a first RB by lpwus-LPSS-StartRB, and an overlaid OFDM sequence per OOK symbol for LPSS reception, and an EPRE ratio relative to SS/PBCH blocks [4, TS 38.211],

-	a number of OOK symbols per OFDM symbol, the first RB, and one or more overlaid OFDM sequences per OOK symbol for WUS reception, and an EPRE ratio relative to SS/PBCH blocks [4, TS 38.211], and

A UE determines to receive LPSS/WUS based on procedures defined in [17, TS 38.304].

A UE assumes that an SCS configuration for LPSS/WUS receptions is same as an SCS of the initial DL BWP and an SCS configuration of an SS/PBCH block the UE used to obtain SIB1.

A UE receives an LPSS in consecutive symbols within a slot. The UE can be provided one or two first symbols for respective one or two LPSS reception occasions in the slot by lpss-StartSymbol. The UE determines slots for LPSS reception occasions based on a periodicity and a time offset, relative to the start of a system frame with SFN 0, provided by lpss-PeriodicityAndOffset. Within a period of LPSS reception occasions, LPSS reception occasions are in a set of KL consecutive slots that have all symbols indicated as downlink by tdd-UL-DL-ConfigurationCommon, if provided, and start from the first slot provided by the time offset in the period, where K is the number of transmitted SS/PBCH blocks indicated by ssb-PositionsInBurst in SIB1 and L is the number of LPSS reception occasions in a slot.

LPSS reception occasions are indexed sequentially in time. An LPSS reception at the k-th LPSS reception occasion is quasi co-located with the k-th transmitted SS/PBCH block, with respect to quasi co-location 'typeC' or 'typeD' properties when applicable, where 1≤k≤K.

If a UE is provided lpwus-LPSS-BeamSubset, the UE receives LPSS/WUS based on the quasi co-location properties of transmitted SS/PBCH blocks indicated by lpwus-LPSS-BeamSubset [12, TS 38.331]; otherwise, the UE receives LPSS/WUS based on the quasi co-location properties for transmitted SS/PBCH blocks indicated by ssb-PositionsInBurst in SIB1. A WUS occasion includes K⋅M WUS monitoring occasions that are indexed sequentially in time, where

-	K is the number of transmitted SS/PBCH blocks indicated by ssb-PositionsInBurst in SIB1, M is a number of WUS monitoring occasions associated with each of the K transmitted SS/PBCH blocks provided by lpwus-MO-NumPerLO, and

-	a WUS monitoring occasion with index k-1⋅M+m-1, where 1≤m≤M and 1≤k≤K, is quasi co-located with the k-th transmitted SS/PBCH block with respect to quasi co-location 'typeC' or 'typeD' properties, when applicable

A UE can be provided, by lpwus-AvailableSlot in LowPowerConfig, a bitmap that corresponds to a set of time units that repeats continuously and indicates a subset of time units from the set of time units that is available for the UE to monitor WUS [12, TS 38.331]. A time unit includes one slot or two slots. A set of time units includes a total of either 10, or 20, or 40 time units. A duration P, in msec, of the set of time units has maximum value of 40 msec. The first symbol of the set of time units every 40 msec/P periods is a first symbol in frame nf mod 4 = 0. The UE can be additionally provided, by lpwus-AvailableSymbol in LowPowerConfig, an indication of symbols in each time unit from the subset of time units that is available for the UE to monitor WUS. If the UE is not provided lpwus-AvailableSlot, the UE assumes that all time units are available for the UE to monitor WUS. If the UE is not provided lpwus-AvailableSymbol, the UE assumes a time unit of one slot and, for a time unit that is available for the UE to monitor WUS, all symbols in the time unit are available for the UE to monitor WUS. The UE assumes that a symbol is not available to monitor WUS when

-	the symbol is indicated as uplink, by tdd-UL-DL-configurationCommon

-	the symbol is indicated for an SS/PBCH block transmission, by ssb-PositionsInBurst in SIB1, and the SS/PBCH block transmission would overlap in frequency with the WUS transmission

-	the symbol is indicated for PDCCH transmissions, by pdcch-ConfigSIB1, and CORESET 0 for the PDCCH transmissions would overlap in frequency with the WUS transmission

A WUS monitoring occasion is over a first number of symbols, provided by lpwus-NominalMoDuration in LowPowerConfig. If a number of available symbols for the UE to monitor WUS in a WUS monitoring occasion is smaller than a second number of symbols, provided by lpwus-ActualDuration in LowPowerConfig, the UE does not monitor WUS in the WUS monitoring occasion. The UE monitors WUS in a WUS monitoring occasion over the earliest available lpwus-ActualDuration symbols in the WUS monitoring occasion. If a number of available symbols for the UE to monitor WUS in a WUS monitoring occasion includes a symbol for LPSS reception, the UE does not monitor WUS in the WUS monitoring occasion.

A UE assumes that WUS occasions occur with a periodicity equal to the I-DRX cycle in the RRC_IDLE/RRC_INACTIVE state [17, TS 38.304]. The UE determines WUS occasions associated with a paging occasion based on lpwus-PO-NumPerLO. A first frame of a WUS occasion starts a number of frames prior to the first of a number of paging frames associated with the WUS occasion [17, TS 38.304]. The first WUS monitoring occasion of a WUS occasion starts at an offset provided by lpwus-OffsetFirstMoWithinLo relative to the start of the first frame.

A paging occasion associated with a WUS occasion has index iPO=UE_ID mod N⋅NS+i_s mod NPOWO where NPOWO is a number of paging occasions associated with a WUS occasion, N, NS, iSG, and i_s are defined in [17, TS 38.304], and UE_ID is defined in clause 7.1 of [17, TS 38.304]. If a number of NSGPO subgroups per paging occasion, provided by lp-SubgroupsNumPerPO, is NSGPO>1, the codepoint for the subgroup index iSG in a PO iPO is iPO*NSGPO+1+iSG, and the codepoint for all subgroups in the PO is (iPO+1)*NSGPO+1-1; otherwise, the codepoint for the PO iPO is iPO.

If, in a WUS monitoring occasion, a UE determines a codepoint associated with the UE [17, TS 38.304], the UE performs PDCCH monitoring according to Type2-PDCCH CSS sets for the paging occasion associated with the WUS monitoring occasion; otherwise, the UE is not required to perform the PDCCH monitoring. The UE may also perform PDCCH monitoring for Type2A-PDCCH CSS sets for DCI format 2_7, if provided.

=========================== End of TP =======================================

---

R1-2600082	Maintenance on  LP-WUS/WUR	Huawei, HiSilicon

R1-2600173	WUS monitoring occasions for LP-WUS/WUR	OPPO

R1-2600214	Maintenance on low-power wake-up signal and receiver for NR	Ericsson

R1-2600476	Maintenance on NR Rel-19 LP-WUS	vivo

R1-2600529	Maintenance on LP-WUS operation	InterDigital, Inc.

R1-2600597	Maintenance of LP-WUS Operation	Ofinno

R1-2600730	Maintenance on low-power wake-up signal and receiver for NR	Samsung

R1-2601215	Maintenance on  LP-WUS	ZTE Corporation, Sanechips