3GPP TSG RAN WG1 #124			R1-2601509

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Agenda Item:	9.5

Source:	Ad-Hoc Chair (NTT DOCOMO, INC.)

Title:	Session Notes of AI 9.5

Document for:	Endorsement

  - Study on Integrated Sensing And Communication (ISAC) for NR

Please refer to RP-253246 for detailed scope of the SI.

[124-R20-ISAC] Email discussion on Rel-20 ISAC – Yingyang (Xiaomi)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2601509	Session Notes of AI 9.5	Ad-Hoc Chair (NTT DOCOMO, INC.)

    - Evaluation assumptions and performance evaluation

Including study of metrics, measurements, and relevant measurement quantization for UAV use case.

R1-2601377	Evaluation results of NR ISAC	Moderator (Xiaomi)

R1-2601378	Details on evaluation of NR ISAC reported by companies	Moderator (Xiaomi)

R1-2601379	Summary on [Post-123-R20-ISAC-evaluation]	Moderator (Xiaomi)

R1-2601380	Summary #1 on evaluations for NR ISAC	Moderator (Xiaomi)

Agreement:

- The definition on time stamp from existing specifications, e.g., TS 38.455, TS 38.355, TS 37.355 are reused as baseline

- No more discussion on time stamp is expected from RAN1 study perspective

Agreement:

- From RAN1 perspective, the measurement of Level D reported from RAN is per gNB

  - Note: RAN1’s understanding is that gNB can associate with one or multiple TRPs

R1-2601381	Summary #2 on evaluations for NR ISAC	Moderator (Xiaomi)

Agreement:

The Text Proposal to TR 38.765 in R1-2601594 is endorsed.

Agreement:

The Text Proposal to TR 38.765 in R1-2601595 is endorsed.

Agreement:

The Text Proposal to TR 38.765 in R1-2601596 is endorsed

Agreement:

- From RAN1 perspective, on measurement quantization of Level C/D for position/velocity measurement, quantization methods in existing specifications, e.g., TS 23.032, TS 38.455, TS 38.355, TS 37.355 can be considered.

- From RAN1 perspective, on measurement quantization of Level C for delay/angle measurement, quantization methods in existing specifications, e.g., TS 38.133 can be considered.

  - For Doppler measurement, same number of quantized bits as radial velocity can be considered.

- Note: above does not take into account the potential impact from measurement quantization on the sensing performance.

Agreement:

- The velocity measurement of Option C1/C3/C4 is radial velocity.

R1-2601382	Summary #3 on evaluations for NR ISAC	Moderator (Xiaomi)

Agreement:

The following Text Proposal to Conclusion section in TR 38.765 is endorsed.

The performance for UAV sensing use case with gNB monostatic sensing is evaluated based on the evaluation assumptions in Annex A (including UMa-AV, Sensing Tx/Rx operating simultaneously, FR1), with detailed assumptions and modelling reported by the companies as captured in Annex B. Among all the reported evaluation results as captured in Annex B, - Baseline 1 (high BS TX power 52dBm with 80dB antenna isolation) - By utilizing measurements from multiple or all TRPs, results from 9/10 sources achieve the performance objectives. - By utilizing measurements from single TRP, results from 3/11 sources achieve the performance objectives. - Baseline 2 (low BS TX power 37dBm with 65dB antenna isolation) - By utilizing measurements from multiple or all TRPs, results from 7/9 sources achieve the performance objectives. - By utilizing measurements from single TRP, results from 2/9 sources achieve the performance objectives

Conclusion:

- From RAN1 perspective, the issue on ambiguity handling in delay/range, Doppler/radial velocity, angle, position and 3D velocity measurements is identified, but not further discussed in Rel-20 NR ISAC.

  - Note: it does not prevent further discussion in future.

  - Note: this issue may or may not be addressed by NW implementation.

Agreement:

- The velocity measurement of Option D1/D4 is 3D velocity, optional radial velocity.

  - Note: in the deployment where 3D velocity cannot be reported, Level D should not be reported.

Agreement:

- The velocity measurement of Option C2/C5 is radial velocity.

Conclusion:

- No further discussion on the assistance information from SF to RAN is expected in Rel-20 NR ISAC from RAN1 study perspective.

Agreement:

Confirm the definition of measurement Level B by removing the square brackets

- Level B: Amplitude and phase profile of delay, and/or Doppler, and/or angle per TRP for a given time stamp by using window(s) of the [consecutive] samples in at least one profile of delay, and/or Doppler and/or angle domain

R1-2601383	Summary #4 on evaluations for NR ISAC	Moderator (Xiaomi)

Agreement:

- From RAN1 perspective, the following methods can be considered to define the quantization for the measurements of Level A/B,

  - e.g., floating point, uniform scalar, non-uniform scalar

  - other quantization method is not precluded.

- Note: above doesn’t take into account the potential impact from the measurement quantization on sensing performance

- Note: No more discussion on measurement quantization on Level A/B is expected in the study item from RAN1 perspective

Agreement:

- From RAN1 perspective, the metric related to power can be considered for Level C measurement report.

- From RAN1 perspective, the metric related to power (if applicable) can be considered for Level D measurement report.

- Note: metric related to power may be different from those defined for communication and between Level C and Level D

- From RAN1 perspective, confidence metrics for the measurements can be considered for Level C/D measurement report

- No more discussion on the metric related to power and confidence metrics is expected in the study item from RAN1 study perspective

Agreement:

- For Level C, at least the following examples of assistance information are identified as needed to assist the further processing at Sensing Function. The assistance information may or may not be provided by RAN.

  - Location of TRP if at least delay/range, Doppler/radial velocity or angle is reported

  - If a measurement is reported in LCS, LCS to GCS transformation information

- For level D, if radial velocity is not reported, no assistance information is identified from RAN1 perspective.

- No more discussion on the assistance information on Level C/D from RAN to SF is expected in the study item from RAN1 perspective

R1-2601384	Summary #5 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2601385	Summary #6 on evaluations for NR ISAC	Moderator (Xiaomi)

R1-2600081	Performance metric, methodologies, and initial evaluation results for ISAC	Huawei, HiSilicon

R1-2600105	Discussion on evaluation assumptions and performance evaluations on ISAC	Spreadtrum, UNISOC

R1-2600183	Discussion of ISAC evaluation in 5GA	OPPO

R1-2600331	Discussion on evaluation methodology and evaluation results for R20 ISAC for NR	CATT, CICTCI

R1-2600363	Evaluation assumptions and performance evaluation for ISAC	EURECOM

R1-2600381	Discussion on sensing measurement and evaluation result	CMCC

R1-2600446	Discussion on performance evaluation for ISAC	Xiaomi

R1-2600495	Evaluation methodology and performance evaluation  for 5G-A ISAC	vivo

R1-2600538	Discussion on evaluation assumptions and performance evaluation for NR ISAC	Panasonic

R1-2600574	Evaluation assumptions and performance evaluation of ISAC for NR	InterDigital, Inc.

R1-2600620	Evaluation assumptions and performance evaluations for ISAC NR	Tiami Networks

R1-2600622	Performance evaluations of ISAC for NR	Nokia

R1-2600689	Discussion on ISAC evaluation assumptions and performance evaluation	China Telecom

R1-2600708	Discussion on ISAC for NR	Ericsson

R1-2600747	Discussion on ISAC Evaluation assumptions and performance evaluation	Samsung

R1-2600792	Discussion on ISAC performance evaluations	T-Mobile USA Inc.

R1-2600819	On Rel-20 Integrated Sensing And Communication (ISAC) for NR	Apple

R1-2600846	Discussion on ISAC Performance Evaluation	NIST

R1-2600902	Discussion on ISAC evaluation assumptions and performance evaluation	MediaTek Inc.

R1-2600973	Discussion on 5G-A ISAC evaluation	ZTE Corporation, Sanechips, Shanghai Jiao Tong University, Pengcheng Laboratory

R1-2600995	Discussion on evaluation assumptions and performance evaluation for NR ISAC	ETRI

R1-2601031	Discussion on NR ISAC evalution assumption and performance evaluation	LG Electronics

R1-2601055	On ISAC performance evaluations and assumptions	Lenovo

R1-2601125	On 5G NR ISAC evaluation assumptions and performance evaluation	Sony

R1-2601171	Evaluation assumptions and performance evaluation for ISAC	NTT DOCOMO, INC

R1-2601234	Discussion on evaluation assumptions and performance evaluation for NR ISAC	Panasonic

R1-2601264	Evaluation Assumptions and performance evaluation for UAV gNB-monostatic sensing	Qualcomm Incorporated

R1-2601358	Discussion on Performance Evaluations for Rel-20 ISAC	Google Korea LLC

R1-2601390	Discussion on ISAC evaluation assumptions and performance evaluation for 5G NR	CEWiT

R1-2601407	Discussion and Evaluations for NR ISAC	VIAVI, CMCC