3GPP TSG RAN WG1 #122bis						R1-2507976

Prague, Czech, Oct 13th – 17th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.11

Agenda Item:		9.11

Document for: 	Endorsement

  - UE features for MCE for NR Phase 3

Agreement:

For FG 66-3 and 66-4:

- Adopt “Per BC” for Type column

- Adopt “n/a” for FDD/TDD differentiation, FR1/FR2 differentiation and capability interpretation columns

- Add the following note in FG 66-3 Note column

  - “Note: Only cell(s) with {120kHz SCS, FR2-1} among the set of cells [can be scheduled with more than one PDSCH]”

- Add the following note in FG 66-4 Note column

  - “Note: Only cell(s) with {120kHz SCS, FR2-1}, {15kHz SCS, FR1} and/or {30kHz SCS, FR1} [can be scheduled with more than one PUSCH]”

- Remove the following FFS in FG 66-3 and 66-4

  - FFS: Whether to limit this FG to particular FR and/or SCS

  - FFS: Whether to separate this FG per FR and/or SCS

  - FFS: other component(s)

- For FG 66-3, define “at least one of {49-1, 49-1b, 66-1}” as prerequisite FGs

- For FG 66-4, define “at least one of {49-2, 49-2b, 66-2}” as prerequisite FGs

Agreement:

Update FG 66-1 as follows:

- Change FR2 to FR2-1 for Component 2 (as agreed in RAN1#122)

- Introduce the following new components:

  - “Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells.”

  - “Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}”

  - “Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8}”

  - “Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4}”

  - “Supported HARQ feedback types, candidate values: {type 1, type2, type 1 and type 2}, Note: the UE shall report the same value for all supported BC for FG 66-1”

  - “Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both}”

  - “Support Type-2 for ‘Antenna port(s)’ field”

Agreement:

Update FG 66-2 as follows:

- Change FR2 to FR2-1 for Component 2 (as agreed in RAN1#122)

- Introduce the following new components:

  - “Scheduling cell is PCell if set of cells includes PCell, and scheduling cell is PCell or an SCell if set of cells includes only SCells”

  - “Max number of co-scheduled cells per set of cells supported by UE is reported with candidate value set of {2, 3, 4}”

  - “Max number of sets of cells supported by UE across PUCCH groups: Candidate value set of {1, 2, 3, 4, 5, 6, 7, 8}”

  - “Max number of sets of cells supported by UE for a same scheduling cell: Candidate value set of {1, 2, 3, 4}”

  - “Supported co-scheduled cell indication schemes: Candidate value set of {FDRA field based, co-scheduled cell indicator field based, both}”

  - “Support Type-2 for ‘Antenna port(s)’ field”

Agreement:

- Introduce the following new component in FG 66-1:

Monitoring SS set(s) for DCI format 1_3 for a set of cells for the following cases - 1) Search space set configuration for DCI format 1_3 for the set of cells is provided only on the scheduling cell, or; - 2) Search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being NOT in the set of cells - UE supporting FG 66-1 can additionally report whether the UE support following case - 3) Search space set configurations for DCI format 1_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells

- Introduce the following new component in FG 66-2:

Monitoring SS set(s) for DCI format 0_3 for a set of cells for the following cases - 1) Search space set configuration for DCI format 0_3 for the set of cells is provided only on the scheduling cell, or; - 2) Search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being NOT in the set of cells - UE supporting FG 66-2 can additionally report whether the UE support following case - 3) Search space set configurations for DCI format 0_3 for the set of cells with the same searchSpaceId are provided on both the scheduling cell and a serving cell in the set of cells with the scheduling cell being in the set of cells

Proposal 1-2:

Introduce the following new component in FG 66-1:

X. The number of unicast DL DCIs to process per N consecutive slots of scheduling cell for a set of cells configured for multi-cell PDSCH scheduling by DCI format 1_3 - One DCI format 1_3 for the set of cells and, - One unicast DL DCI formats 1_0/1_1/1_2 (if supported) for each of the cells that are not scheduled by DCI 1_3 - SCS1 is the SCS of scheduling CC, and SCS2 is the SCS for [reference CC] - For SCS1 smaller than [or equal to] SCS2, N = 1 - For SCS1 larger than SCS2, N is based on pair of (SCS1, SCS2): N=2 for (30,15)

- FFS: Reference CC in the set of cells to determine N, to be down-selected from the following:

  - Alt-1: “reference CC” refers to the scheduled CC SCS for each pair of scheduling CC and scheduled CC in all the scheduled CCs in the cell set

  - Alt-2: “reference CC” refers to the smallest SCS among all the scheduled cells in the cell set

Proposal 1-5:

Introduce the following new component in FG 66-1:

The number of unicast UL DCIs to process per N consecutive slots of scheduling cell for a set of cells configured for multi-cell PUSCH scheduling by DCI format 0_3 - For FDD scheduling cell - Up to one DCI format 0_3 for the set of cells and, - Up to one unicast UL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells - For a cell in a set of cells, no more than one DCI scheduling PUSCH for the cell - For TDD scheduling cell - Up to two DCI format 0_3 for the set of cells and, - Up to two unicast UL DCI formats 0_0/0_1/0_2 (if supported) for each of the cells - For a cell in a set of cells, no more than two DCI scheduling PUSCH for the cell - For low-to-high SCS, N = 1. - For high-to-low SCS, N is based on pair of (scheduling CC SCS, [scheduled CC] SCS): N=2 for (30,15)

- FFS: Reference CC in the set of cells to determine N, to be down-selected from the following:

  - Alt-1: “Scheduled CC” refers to the scheduled CC SCS for each pair of scheduling CC and scheduled CC in all the scheduled CCs in the cell set

  - Alt-2: “Scheduled CC” refers to the smallest SCS among all the scheduled cells in the cell set

Proposal 1-6:

Adopt the following updates in FG 66-1/2:

- Consequence if the feature is not supported by the UE

  - For FG 66-1: “Multi-cell PDSCH scheduling by DCI format 1_3 with different SCS and/or different carrier type is not supported”

  - For FG 66-2: “Multi-cell PUSCH scheduling by DCI format 0_3 with different SCS and/or different carrier type is not supported”

- Remove “FFS: Other component(s)” in FG 66-1/2 Component column

- Remove FFS in FG 66-1/2 Note column

Proposal 1-7:

Update the FG for “Support of three sets of (carrier type, SCS) for the cells in the set for multi-cell PDSCH scheduling by DCI format 1_3 with different SCS and/or different carrier type” as follows:

| 66-1a | Support of three sets of (carrier type, SCS) for the cells in the set for multi-cell PDSCH scheduling by DCI format 1_3 with different SCS and/or different carrier type | FFS Supported applicable combinations of the following from the band combination: - Three sets of (carrier type, SCS) for the cells in the set: {(FR1 licensed FDD, 15kHz), (FR1 licensed TDD, 30kHz), (FR2-1, 60kHz), (FR2-1, 120kHz)} - A set of (carrier type, SCS) for scheduling cell: {(FR1 licensed FDD, 15kHz), (FR1 licensed TDD, 30kHz)} | FFS 66-1 | FFS Yes | FFS n/a | FFS Three sets of (carrier type, SCS) for the cells in the set for multi-cell PDSCH scheduling by DCI format 1_3 with different SCS and/or different carrier type is not supported | FFS Per BC | FFS n/a | FFS n/a | FFS n/a | FFS | FFS Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 1-8:

Update FG 66-2a as follows:

| 66-2a | Support of three sets of (carrier type, SCS) for the cells in the set for multi-cell PUSCH scheduling by DCI format 0_3 with different SCS and/or different carrier type | FFS Supported applicable combinations of the following from the band combination: - Three sets of (carrier type, SCS) for the cells in the set: {(FR1 licensed FDD, 15kHz), (FR1 licensed TDD, 30kHz), (FR2-1, 60kHz), (FR2-1, 120kHz)} - A set of (carrier type, SCS) for scheduling cell: {(FR1 licensed FDD, 15kHz), (FR1 licensed TDD, 30kHz)} | FFS 66-2 | FFS Yes | FFS n/a | FFS Three sets of (carrier type, SCS) for the cells in the set for multi-cell PUSCH scheduling by DCI format 0_3 with different SCS and/or different carrier type is not supported | FFS Per BC | FFS n/a | FFS n/a | FFS n/a | FFS | FFS Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 1-9:

Introduce the following new FGs (inherit from Rel-18 FG 49-4a/4b/4c/4d):

| 66-5a | Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 1_3 | 66-1 | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 66-5b | Nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3 | 1) Support of nominal RBG size of Configuration 3 for FDRA type 0 for DCI format 0_3 | 66-2 | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| 66-5c | Configurable Type-1A fields for DCI format 0_3/1_3 | 1) Support Type-1A for ‘Antenna port(s)’ field for DCI format 1_3 2) Support Type-1A for ‘Antenna port(s)’, ‘Precoding information and number of layers’ and ‘SRS resource indicator’ fields for DCI format 0_3 | At least one of {66-1, 66-2} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |
| 66-5d | FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3/0_3 | 1) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 0_3 2) Support of FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3 | At least one of {66-1, 66-2} | Yes |  |  | Per UE | No | No | No |  | Optional with capability signaling |

Proposal 1-10:

Inform RAN2 to update prerequisite FG(s) for Rel-18 RAN1 FG 49-5a/5b/6/7/8/9/10/12/12a/13/14 in Rel-19 as follows (red fonts):

| Index | Feature group | Components | Prerequisite feature groups |
| --- | --- | --- | --- |
| 49-5a | Trigger Type 3 HARQ CB based feedback using DCI format 1_3 | 1. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_3 scheduling at least a PDSCH 2. Support feedback of type 3 HARQ-ACK codebook, triggered by a DCI 1_3 without scheduling a PDSCH using a reserved FDRA value | At least one of {49-1, 49-1b, 66-1} |
| 49-5b | Trigger enhanced Type 3 HARQ CB based feedback using DCI format 1_3 | 1. Support feedback of enhanced type 3 HARQ-ACK codebook, triggered by a DCI 1_3 2. Support configuration of up to 8 enhanced type 3 HARQ-ACK codebooks. 3. Support feedback of a dynamically selected enhanced type 3 HARQ-ACK codebook based on triggering information in DCI 1_3 4. Support transmission of enhanced type 3 HARQ-ACK codebook using the first or second PUCCH configuration based on PHY priority indication in the triggering DCI (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 49-6) 5. Supported maximum number of actual PUCCH transmissions for type 3 or enhanced type 3 HARQ-ACK codebook feedback within a slot | At least one of {49-1, 49-1b, 66-1} |
| 49-6 | Two HARQ-ACK codebooks with up to one sub-slot based HARQ-ACK codebook simultaneously constructed for supporting HARQ-ACK codebooks with different priorities by DCI format 1_3 | 1. Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed with the restriction up to one sub-slot based HARQ-ACK codebook. 2. Supports separate PUCCH configuration for different HARQ-ACK codebooks. 3. Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. 4. Supports a DCI format 1_3 scheduling PDSCH with different HARQ-ACK priorities when only DCI format 0_3/1_3 is configured per BWP. 5. Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and 'codeBlockGroupTransmission" for different HARQ-ACK codebooks. 6. Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot - Candidate values for the component 6 of this FG is: For NCP, {4, 5, 6, 7} for 2-symbol*7 sub-slot configuration; For ECP, the candidate value is {4,5,6} for 2-symbol*6 sub-slot configuration 7. Support intra-UE multiplexing/prioritization of UL overlapping channels/signals with two priority levels for HARQ-ACK | At least one of {49-1, 49-1b, 66-1} |
| 49-7 | UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer for DCI format 1_3/0_3 | Support intra-UE multiplexing/prioritization of overlapping PUCCH/PUCCH and PUCCH/PUSCH with two priority levels in physical layer (PHY) for DCI format 1_3/0_3 1) Configuration of PHY priority level for CG PUSCH and SR, and dynamic indication of priority level for dynamic PUSCH with a single DCI format 0_3 2) Multiplexing/prioritization between UL channels/signals with the same PHY priority level 3) Prioritization between UL channels/signals with different PHY priority levels 4) Additional number of symbols (d1) needed beyond the PUSCH preparation time for cancelling a low priority UL transmission. 5) Additional number of symbols (d2) of the preparation time needed for the high priority UL transmission that cancels a low priority UL transmission | At least one of {49-1, 49-1b, 49-2, 49-2b, 66-1, 66-2} |
| 49-8 | Triggered HARQ-ACK codebook re-transmission for DCI format 1_3 | 1. Support HARQ-ACK re-transmission from an earlier PUCCH slot based on the triggering information in DCI format 1_3 2. Support the related PHY priority handling in terms of HARQ-ACK codebook selection and the applicable PUCCH configuration (for a UE supporting two HARQ-ACK codebooks / PUCCH config in 49-6) 3. Supported minimum value M for the HARQ re-tx offset 4. Supported maximum value N for the HARQ re-tx offset | at least one of {49-1, 49-1b, 66-1} |
| 49-9 | SCell dormancy indication within active time in DCI format 0_3/1_3 | Support for SCell dormancy indication sent within the active time on PCell with DCI format 0_3/1_3 | 6-5, at least one of {49-1, 49-1b, 49-2,49-2b, 66-1, 66-2} |
| 49-10 | Dynamic indication of applicable minimum scheduling restriction by DCI format 0_3/1_3 | 1) Dynamic indication of applicable minimum scheduling restriction by DCI format 0_3 and 1_3 2) minimumSchedulingOffset K0 configuration for PDSCH and aperiodic CSI-RS triggering offset 3) minimumSchedulingOffset K2 configuration for PUSCH 4) Support of extended value range for aperiodic CSI-RS triggering offset | At least one of {49-1, 49-1b, 49-2,49-2b, 66-1, 66-2} |
| 49-12 | Unified TCI with joint DL/UL TCI update by DCI format 1_3 for intra-cell and inter-cell beam management with more than one MAC-CE activated joint TCI state per CC | 1: TCI state indication for update and activation - MAC-CE+DCI-based TCI state indication (use of DCI formats 1_3 with DL assignment for at least one serving cell in a scheduledCellListDCI-1-3 to provide indicated unified TCI state(s) for the CC(s) in the scheduledCellListDCI-1-3) 2: The minimum beam application time in Y symbols per SCS 3: The maximum number of MAC-CE activated joint TCI states per CC in a band | 23-1-1, At least one of {49-1, 49-1b, 66-1} |
| 49-12a | Unified TCI with separate DL/UL TCI update by DCI format 1_3 for intra-cell beam management with more than one MAC-CE activated separate TCI state per CC | - TCI state indication for update and activation - MAC-CE+DCI-based TCI state indication (use of DCI formats 1_3 with DL assignment for at least one serving cell in a scheduledCellListDCI-1-3 to provide indicated unified TCI state(s) for the CC(s) in the scheduledCellListDCI-1-3) 2. The minimum beam application time in Y symbols per SCS 3. The maximum number of MAC-CE activated DL TCI states per CC in a band 4. The maximum number of MAC-CE activated UL TCI states per CC in a band | 23-10-1, At least one of {49-1, 49-1b, 66-1} |
| 49-13 | Default QCL assumption for multi-cell scheduling by DCI format 1_3 | Indicates whether the UE can be configured with enabledDefaultBeamFormultiCellScheduling for default QCL assumption for multi-cell scheduling by DCI format 1_3 for same/different numerologies - Candidate values are {different only, both} - When "both" is reported, the UE supports this feature for same SCS and for different SCS combination(s) (low-to-high, high-to-low or both) reported for 49-1b | At least one of {49-1, 49-1b, 66-1} |
| 49-14 | Support of BWP switch indication by DCI format 0_3/1_3 | Support of BWP switch indication by DCI format 0_3/1_3 | At least one of {49-1, 49-1b, 49-2, 49-2b, 66-1, 66-2} for the BC At least one of {6-2, 6-3, 6-4} for at least one band of the BC |

R1-2506886	UE features for MCE for NR Phase 3	vivo

R1-2506929	UE features for Rel-19 Multi-carrier enhancements	Huawei, HiSilicon

R1-2506975	Discussion on UE feature for Rel-19 Multi-carrier enhancements	Xiaomi

R1-2507078	NR Multi-carrier Enhancements Phase 2 UE features	Nokia

R1-2507151	Discussion on UE feature for multi-cell scheduling with a single DCI	OPPO

R1-2507198	Discussion on UE features for MCE for NR Phase 3	ZTE Corporation, Sanechips

R1-2507272	UE features for multi-carrier enhancements	Samsung

R1-2507615	MCE UE features Phase 3	MediaTek Inc.

R1-2507662	Views on UE features for Rel-19 MCE	Apple

R1-2507708	UE features for MCE for NR Phase 2	Qualcomm Incorporated

R1-2507801	Discussion on UE features for multi-cell PUSCH/PDSCH scheduling with a single DCI	NTT DOCOMO, INC.

R1-2507860	UE features for MCE for NR Phase 3	Ericsson Inc.