3GPP TSG RAN WG1 #122bis                                   	R1-2507966

Prague, Czech, Oct 13th – 17th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.3

Agenda Item:		9.3

Document for: 	Endorsement

  - UE features for evolution of NR duplex operation: SBFD

Agreement:

Update FG 60-7b and 60-7c as follows:

| 60-7b | Separate UL resource muting for Type-1 CG PUSCH for CP-OFDM waveform | Support of separate UL resource muting for Type-1 CG PUSCH for CP-OFDM waveform | FFS | [YES] | [n/a] | [Separate UL resource muting for Type-1 CG PUSCH for CP-OFDM waveform is not supported] | [Per Band] | [TDD only] | [n/a] | [n/a] | FFS: Whether/how to capture maximum number of UL muting symbols per slot | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 60-7c | Separate UL resource muting for Type-1 CG PUSCH for DFT-s-OFDM waveform | Support of separate UL resource muting for Type-1 CG PUSCH for DFT-s-OFDM waveform | FFS | [YES] | [n/a] | [Separate UL resource muting for Type-1 CG PUSCH for DFT-s-OFDM waveform is not supported] | [Per Band] | [TDD only] | [n/a] | [n/a] | FFS: Whether/how to capture maximum number of UL muting symbols per slot | [Optional with capability signalling] |

Agreement:

- Adopt FG 60-7 and 5-19 for prerequisite FG(s) of FG 60-7b

- Adopt FG 60-7a and 5-19 for prerequisite FG(s) of FG 60-7c

Proposal 4-1-3:

Regarding “FFS: Whether/how to capture maximum number of UL muting symbols per slot” in FG 60-7b/7c, adopt Alt-X

- Alt-1: Capture maximum number of UL muting symbols per slot

  - Alt-1.1: Add a component “Maximum of 2 UL muting symbols per slot”

  - Alt-1.2: Add a component “Maximum number of UL muting symbols per slot”, with candidate values {2, 4}

- Alt-2: Decision in maintenance needed in prior

- Alt-3: Remove FFS without additional update

Agreement:

Update FG 60-8 as follows:

- Confirm “aperiodic” in FG name, and update the text for “consequence …” column accordingly

- Define no prerequisite FG

- Adopt n/a for FR1/FR2 differentiation and capability interpretation for mixture of FDD/TDD and/or FR1/FR2

- Add “[Candidate values for component 4 are {8, 16, 32, 64}]”

- Remove “FFS: whether each of components within brackets is separate FG or require reporting”

- (TBC) Remove “FFS: other potential component/FG”

| 60-8 | L1 CLI-RSSI measurement and [aperiodic] reporting | 1. Aperiodic L1 CLI-RSSI reporting on PUSCH 2. Support of CLI-RSSI measurement resource configured in one UL subband only, in one DL subband only, or across two DL subbands only. 3. Periodic and aperiodic CLI-RSSI measurement resource 4. Maximum number of configured L1 CLI-RSSI measurement resources (sum of aperiodic and periodic, and semi-persistent if supported) across all CCs FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG | FFS | YES | n/a | [Aperiodic] L1 CLI-RSSI measurement and aperiodic reporting is not supported | Per band | TDD only | [n/a] | [n/a] | [Candidate values for component 4 are {8, 16, 32, 64}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

Update FG 60-8a as follows:

| 60-8a | Support of periodic L1 CLI-RSSI reporting on PUCCH | 1. Periodic L1 CLI-RSSI reporting on PUCCH | 60-8 | YES | n/a | Support of periodic L1 CLI-RSSI reporting on PUCCH is not supported | Per band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

Update FG 60-9 as follows:

- Remove “FFS: whether each of components within brackets is separate FG or require reporting”

- Define no prerequisite FG

- Revise consequence such as “[Aperiodic] L1 SRS-RSRP measurement and aperiodic reporting is not supported”

- Add “[Candidate values for component 2 are {4, 8, 16, 32}]” in Note column

- (TBC) Remove “FFS: other potential component/FG”

| 60-9 | L1 SRS-RSRP measurement and aperiodic reporting | 1. Aperiodic L1 SRS-RSRP reporting on PUSCH, and periodic and aperiodic SRS-RSRP measurement resource 2. Maximum number of L1 SRS-RSRP measurement resources across all CCs FFS: whether each of components within brackets is separate FG or require reporting FFS: other potential component/FG | FFS | YES | n/a | [Aperiodic] L1 SRS-RSRP measurement and aperiodic reporting is not supported | Per band | TDD only | n/a | n/a | Candidate values for component 2 are {4, 8, 16, 32} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

For Msg.3 repetition in SBFD symbols, RAN1 to down-select one of the following alternatives below in RAN1#123:

- Alt-1: A new FG for Msg.3 repetition in SBFD symbols is introduced as follows:

| 60-6 | Msg.3 repetition in SBFD symbols | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI in SBFD symbols with separate power control parameter | 60-3 or 60-4, 30-6 | YES | n/a | Msg.3 repetition in SBFD symbols is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Alt-2: Adopt the following updates in FG 60-3 and 60-4

  - Add a new component: “X. Support of Msg.3 repetition in SBFD symbols”

  - Add in Note column: “Component X applies only when the UE reports FG 30-6 in the same band”

- Alt-3: Neither new component nor FG is added for support of Msg.3 repetition in SBFD symbols

Agreement:

- Remove “[“ from Component 2 in FG 60-1

- Remove “FFS: Whether to include two DL subband support in component 2”

Agreement:

Regarding “FFS: details of counting” in FG 60-1, adopt Alt1.1

- Alt-1: Update candidate values for Component 11

  - Alt-1.1: Component 11 candidate values: {“with factor-of-two relaxation for CSI timeline”, “with factor-of-two relaxation for active CSI-RS resource and port counting”, “no relaxation”}

Question 2-1-2 (for checking the need for separate FG for Q2-1-2-2):

Companies are encouraged to provide views on the need for the following new components:

- Q2-1-2-1: Unaligned boundaries between SBFD subband(s) and RBG, CSI reporting subband, CSI-RS resource, PRG

- Q2-1-2-2: For PUSCH repetition type B, a nominal repetition is segmented into actual repetitions around boundary of SBFD symbols and non-SBFD symbols.

Agreement:

Update FG 60-2 as follows:

| 60-2 | Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of PDSCH reception across SBFD symbols and non-SBFD symbols (Configuration 2) FFS: Other component(s) FFS: whether component 2 is defined as separate FG | 60-1 | YES | n/a | Reception across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported | Per Band | TDD only | n/a | n/a |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Agreement:

Update FG 60-2a as follows:

| 60-2a | Transmission across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) | 1. Support of UL transmission across SBFD symbols and non-SBFD symbols (Configuration 2) | FFS 60-1 | [YES] | [n/a] | [Transmission across SBFD symbols and non-SBFD symbols in different slots (Configuration 2) is not supported] | [Per band] | [TDD only] | [n/a] | [n/a] |  | [Optional with capability signalling] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

R1-2506773	Discussion on UE features for SBFD	ZTE Corporation, Sanechips

R1-2506801	Discussion on UE features for SBFD	Spreadtrum, UNISOC

R1-2506883	UE features for evolution of NR duplex operation: SBFD	vivo

R1-2506949	UE features for evolution of NR duplex operation	Huawei, HiSilicon

R1-2506973	UE features for Rel-19 SBFD operation	Xiaomi

R1-2507074	SBFD UE features	Nokia

R1-2507101	UE features for SBFD	CATT

R1-2507150	Discussion on UE features for Rel-19 NR Duplex	OPPO

R1-2507238	UE features for NR duplex operation	Samsung

R1-2507461	Discussion on UE features for evolution of NR duplex operation: SBFD	Ofinno

R1-2507575	UE features for evolution of NR duplex operation	MediaTek Inc.

R1-2507638	On UE features for evolution of NR duplex operation	Google

R1-2507753	UE features for evolution of NR duplex operation	Qualcomm Incorporated

R1-2507760	UE features for SBFD	Ericsson

R1-2507797	Discussion on UE features for evolution of NR duplex operation	NTT DOCOMO, INC.