3GPP TSG RAN WG1 #122bis                                   	R1-2507972

Prague, Czech, Oct 13th – 17th, 2025

Source:		Ad-Hoc Chair (NTT DOCOMO)

Title:			Session Notes of AI 9.9

Agenda Item:		9.9

Document for: 	Endorsement

  - UE features for NTN for Internet of Things (IoT) Phase 3

Agreement:

| 1-1 | OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED | 1. Support of symbol-level length-2 OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED 2. Support of TDM DMRS over 4 slots where DMRS are transmitted in the first 2 slots and DMRS REs are blanked in the next 2 slots, or vice-versa 3. Dynamic activation/deactivation of OCC feature via DCI 4. Whether to support combined operation of OCC and interleaved multi-TB scheduling | Rel-17 2-1b | Yes | N/A | UE does not support OCC for single-tone NPUSCH format 1 with 3.75 kHz SCS in RRC_CONNECTED | Per UE | FDD only | N/A | Note: This FG is applicable only to IoT-NTN FDD band Component 4 candidate values: {support, not support} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1-2 | OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED | 1. Support of slot-level length-2 OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED 2. Support of CDM DMRS for NPUSCH format 1 with 15 kHz SCS 3. Dynamic activation/deactivation of OCC feature via DCI 4. Whether to support combined operation of OCC and interleaved multi-TB scheduling | Rel-17 2-1b | Yes | N/A | UE does not support OCC for single-tone NPUSCH format 1 with 15 kHz SCS in RRC_CONNECTED | Per UE | FDD only | N/A | Note: This FG is applicable only to IoT-NTN FDD band Component 4 candidate values: {support, not support} | Optional with capability signalling |

R1-2506914	Discussion on the UE feature for NTN for Internet of Things (IoT) Phase 3	ZTE Corporation, Sanechips

R1-2506939	UE features for IoT-NTN Phase 3	Huawei, HiSilicon

R1-2507103	Discussion on UE features for NTN for Internet of Things (IoT) Phase 3	CATT

R1-2507137	Discussion on UE features for NTN for Internet of Things Phase 3	OPPO

R1-2507260	On UE features for IoT-NTN Phase 3	Ericsson

R1-2507276	UE features for IoT NTN Phase 3	Samsung