3GPP TSG RAN WG1 #124			R1-2601497

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Agenda Item:	8.2

Source:	Ad-Hoc Chair (Ericsson)

Title:	Session Notes of AI 8.2

Document for:	Endorsement

## Maintenance on NR MIMO Phase 5

Note: Maximum one contribution per company/organization/university.  For efficient review, please use the following sections in your contribution corresponding to the maintenance issues, if any:

- Enhancements for UE-initiated/event-driven beam management

- CSI enhancements

- Support for 3-antenna-port codebook-based transmissions

- Enhancement for asymmetric DL sTRP/UL mTRP scenarios.

[124-R19-MIMO] Email discussion on MIMO – Eko (Samsung)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2601497	Session Notes of AI 8.2	Ad-Hoc Chair (Ericsson)

R1-2600084	Maintenance of MIMO Phase 5	Huawei, HiSilicon

R1-2600130	FL Summary Support for 3TX CB-based Uplink; First Round	Moderator (InterDigital, Inc.)

R1-2600172	Remaining Issues of NR MIMO Phase 5	OPPO

R1-2600274	Maintenance on NR MIMO Phase 5	ZTE Corporation, Sanechips

R1-2600288	Maintenance on NR MIMO Phase 5	CATT

R1-2600473	Maintenance on NR MIMO phase 5	vivo

R1-2600594	Maintenance on NR MIMO Phase 5	Ofinno

R1-2600724	Remaining issue on NR MIMO Phase 5	Samsung

R1-2600725	Moderator Summary#1 on Rel-19 CSI enhancements: Round 1	Moderator (Samsung)

R1-2600726	Moderator Summary#1 on Rel-19 CSI enhancements: Round 2	Moderator (Samsung)

R1-2600862	Remaining issues on NR MIMO Phase 5	Fujitsu

R1-2601044	Maintenance of NR MIMO Phase 5	Nokia Corporation

R1-2601154	Summary #1 on Rel-19 asymmetric DL sTRP/UL mTRP	Moderator (OPPO)

R1-2601200	Maintenance on NR MIMO Phase 5	China Telecom

R1-2601305	Maintenance of NR MIMO Phase 5	Ericsson

R1-2601348	Remaining issues on UEI CSI report	ASUSTeK

### Enhancements for UE-initiated/event-driven beam management

R1-2601518	Moderator Summary #1 on UE-initiated/event-driven beam management	Moderator (ZTE)

R1-2601519	Moderator Summary #2 on UE-initiated/event-driven beam management	Moderator (ZTE)

Agreement:

Adopt the following changes in [Sections 6.2.7 and 6.3.2.1.4 in TS 38.212.

The corresponding final CR for Rel-19 TS 38.212 in R1-26xxxxx is endorsed.

- Reason for change: On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the multiplexing first PUCCH(s) with UEIRIs collided/overlapped with a PUSCH, the field of UEIRI is jointly encoded with HARQ-ACK by appending the UEIRI to the HARQ-ACK information bits.

- Summary of change: Specify multiplexing of UEIRI into PUSCH by jointly encoding UEIRI bits and HARQ-ACK information bits, which including: 1) coded bits of UEIRI in TS38.212; 2) UEIRI bits mapped to UCI bit sequence in TS 38.212

- Consequences if not approved: UE procedures of multiplexing UEIRI into a PUSCH in the specification are incomplete.

6.2.7 Data and control multiplexing < Unchanged parts are omitted > In case where there are more than one UL-SCH transport blocks for the PUSCH transmission, the UCI information is multiplexed only on the UL-SCH transport block with highest IMCS value for the initial PUSCH, where IMCS is as defined in Clause 6.1.4.1 in [6, TS 38.214]. In case the two transport blocks have the same IMCS value for the initial PUSCH, the UCI information is multiplexed with data only on the first transport block. The PUSCH for UCI multiplexing in this Clause refers to the UL-SCH transport block for UCI multiplexing. If the higher layer parameter nrofBitsInUTO-UCI is configured, the procedure in this clause 6.2.7 applies by replacing CG-UCI with UTO-UCI in all the notations and texts, and replacing "when higher layer parameter cg-UCI-Multiplexing is configured" with "when UTO-UCI and HARQ-ACK are transmitted on a PUSCH". If a UE would multiplex UEIRI and HARQ-ACK in a PUSCH [5, TS 38.213], the procedure in this clause 6.2.7 applies by replacing CG-UCI with UEIRI in all the notations and texts, and replacing "when higher layer parameter cg-UCI-Multiplexing is configured" with "when UEIRI and HARQ-ACK are transmitted on a PUSCH". UE expects that at most one of CG-UCI, UTO-UCI, or UEIRI to overlap with a PUSCH. Denote the coded bits for UL-SCH as . Denote the coded bits for HARQ-ACK or jointly coded bits for HARQ-ACK and CG-UCI when the high layer parameter cg-UCI-Multiplexing is configured, if any, as . Denote the coded bits for CSI part 1, if any, as . Denote the coded bits for CSI part 2, if any, as . Denote the coded bits for CG-UCI without HARQ-ACK, if any, as g0CG-UCI, g1CG-UCI, g2CG-UCI, g3CG-UCI, …, gGCG-UCI-1CG-UCI. Denote the multiplexed data and control coded bit sequence as . < Unchanged parts are omitted >.

##### 6.3.2.1.4 HARQ-ACK and CG-UCI/UTO-UCI/UEIRI If the higher layer parameter nrofBitsInUTO-UCI is configured, the procedure in this clause 6.3.2.1.4 applies by replacing CG-UCI with UTO-UCI in all the notations and texts, replacing "When higher layer parameter cg-UCI-Multiplexing is configured" with "When UTO-UCI and HARQ-ACK have the same priority index and are jointly encoded and transmitted on a PUSCH" and replacing "is given by Table 6.3.2.1.3-1 mapped in the order from upper part to lower part " with "is given by Clause 9.3.1 of [5, TS 38.213]". If a UE would multiplex UEIRI and HARQ-ACK in a PUSCH [5, TS 38.213], the procedure in this clause 6.3.2.1.4 applies by replacing CG-UCI with UEIRI in all the notations and texts assuming higher layer parameter cg-UCI-Multiplexing is configured and replacing "is given by Table 6.3.2.1.3-1 mapped in the order from upper part to lower part " with "is determined as described in clause 6.3.2.1.3B ". When higher layer parameter cg-UCI-Multiplexing is configured, the UCI bit sequence a0, a1, a2, a3, …,aA-1 is determined as follows, where A=OCG-UCI+OACK. - The CG-UCI bits are mapped to the UCI bit sequencea0, a1, a2, a3, …,aOCG-UCI-1 , where ai=oiCG-UCI for i=0,1, …, OCG-UCI-1. The CG-UCI bit sequence o0CG-UCI, o1CG-UCI, …, oOCG-UCI-1CG-UCI is given by Table 6.3.2.1.3-1 mapped in the order from upper part to lower part, and OCG-UCI is number of CG-UCI bits; - The HARQ-ACK bits are mapped to the UCI bit sequence aOCG-UCI, aOCG-UCI+1, …,aOCG-UCI+OACK-1, where ai+OCG-UCI=oiACK for i=0,1, …, OACK-1. The HARQ-ACK bit sequence o0ACK, o1ACK, …, oOACK-1ACK is given by Clause 9.1 of [5, TS38.213], and OACK is number of HARQ-ACK bits.

Agreement:

Adopt the following changes in Section 5.2.1.4.3 in TS38.214:

The corresponding CR for Rel-19 and TS38.214 in R1-26xxxxx is endorsed.

- Reason for change:  On channel measurement for UE-initiated/event-driven beam reporting, the RRC parameter of ‘timeRestrictionForChannelMeasurement’ specified for legacy beam reporting can be reused for UEI beam reporting by adopting the following operations.

  - RRC parameter of timeRestrictionForChannelMeasurement can be applied to Event-2, Event-1 and Event-7.

  - Besides for new beam RS(s), the value of M determined for measurement period of the new beam(s) by timeRestrictionForChannelMeasurements in the UEIBR CSI report configuration can also be used for the current beam RS measurement.

- Summary of change:  Specify the rule of channel measurements for UE-initiated/event-driven beam reporting depending on RRC parameter of timeRestrictionForChannelMeasurement.

- Consequences if not approved: UE procedures of deriving channel measurements for L1-RSRP based UEI beam reporting is unclear.

# 5.2.1.4.3	L1-RSRP Reporting

<Omitted>

If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "notConfigured", the UE shall derive the channel measurements for computing L1-RSRP value reported in uplink slot n based on only the SS/PBCH or NZP CSI-RS, no later than the CSI reference resource, (defined in [4, TS 38.211]) associated with the CSI resource setting. If the UE is configured with a CSI-ReportConfig with eventType, the channel measurements include measurements performed on SS/PBCH or NZP-CSI-RS associated with the indicated TCI state or activated TCI state, which triggers UEIRI transmission.

If the higher layer parameter timeRestrictionForChannelMeasurements in CSI-ReportConfig is set to "Configured", the UE shall derive the channel measurements for computing L1-RSRP reported in uplink slot n based on only the most recent, no later than the CSI reference resource, occasion of SS/PBCH or NZP CSI-RS (defined in [4, TS 38.211]) associated with the CSI resource setting. If the UE is configured with a CSI-ReportConfig with eventType, the channel measurements include measurements performed on SS/PBCH or NZP-CSI-RS associated with the indicated TCI state or activated TCI state, which triggers UEIRI transmission.

Agreement:

Send a reply to the RAN4 LS R4-2511656 clarifying that, for a UE configured with a UEIBM CSI report configuration, UEIBM measurement reporting delay requirements should consider that:

- timeRestrictionForChannelMeasurements can be configured for any event type, i.e., Event-1, Event-2 and Event-7;

- the value of M determined for the measurement period of the new beams by timeRestrictionForChannelMeasurements in the UEIBM CSI report configuration is also used for the current beam in the event determination/evaluation measurements.

Comeback to review the corresponding draft LS to endorse final LS.

Agreement:

Adopt the following changes in Section 5.2.1.5.4 in TS38.214.

- Reason for change: The reason for resetting counters in legacy operations is to define the UE behaviour. If the counter is not reset, the meaning of a report becomes unclear: to what extent has the UE used the old and new configuration? Furthermore, when the NW has reconfigured the UE, it wants the UE to act according to the new configuration, not the old configuration.

- Summary of change: Specify that the UE shall reset the counter of event instances for all new beams when the parameters associated with event-driven reporting are reconfigured.

- Consequences if not approved: UE procedures of resetting event instance counter for the UE-initiated/event-driven beam reporting is unclear.

5.2.1.5.4.1 UE Initiated CSI reporting <Omitted> The counter of the event instances for such reference signal is reset: - if the reference signal in the indicated TCI state or the SS/PBCH block which is QCLed with the reference signal in the indicated TCI state is updated when the UE is configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event2’ or ‘event1’, or - if the reference signal with the valueOfQ-th highest L1-RSRP out of the reference signals among the activated TCI states or the SS/PBCH block with the valueOfQ-th highest L1-RSRP out of the SS/PBCH blocks QCLed with the reference signals among the activated TCI states, which determines event instance(s), is updated when the UE is configured with a CSI-ReportConfig with the higher layer parameter eventType set to ‘event7’, or - if eventThreshold or valueOfQ or eventInstanceCount or eventDetectionTimeWindow or resourcesForChannelMeasurement in the CSI-ReportConfig are reconfigured. The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType.

### CSI enhancements

R1-2600725	Moderator Summary#1 on Rel-19 CSI enhancements: Round 1	Moderator (Samsung)

R1-2600726	Moderator Summary#1 on Rel-19 CSI enhancements: Round 2	Moderator (Samsung)

Agreement:

Adopt the following changes in Section 9.2.5 in TS38.213:

The corresponding CR for Rel-19 and TS38.214 in R1-26xxxxx is endorsed.

Impacted spec(s): TS38.213

Reason for change: The agreement reached in RAN1#119 has not been incorporated into the current 213 (V19.1.0). This modification is necessary because the Rel-19 Type-I SP codebook exhibits significant overhead variation across different measured ranks. Specifically, RANK 7/8 yields the maximum overhead for Scheme-A, whereas RANK 4 for Scheme-B. However, according to the current spec, UE shall determine PUCCH resources under the assumption of feedback rank 1 for each CSI report. This approach is based on observations that the Rel-15 Type-I codebook maintains consistent overhead across ranks. Persisting with rank 1 assumption for Rel-19 Type-I codebook CSI reports will inevitably increase the frequency of UCI dropping incidents. Agreement [RAN1#119] For the Rel-19 Type-I SP codebook refinement for P=48, 64, 128 CSI-RS ports, when the UE reports or multiplexes the CSI that include Part 2 CSI reports on PUCCH, the PUCCH resource, the number of PRBs for the PUCCH resource, and/or the number of Part 2 CSI reports are determined based on the RI value that results in the largest UCI payload. - For Scheme-A, the RI value is: - 8 when the RI value(s) allowed by the configured RI restriction per CSI reporting configuration include 5, 6, 7, and/or 8, - 1 otherwise - For Scheme-B, the RI value is 4.

Summary of the change: Change the rank assumption for determination of the PUCCH resource or the number of PRBs or number of part 2 CSI reports.

Consequences if not approved: Unnecessary UCI dropping will occur for Rel-19 Type-I codebook.

=====================Start of Text Proposal to TS38.213================== ### 9.2.5 UE procedure for reporting multiple UCI types ##### If a UE would multiplex CSI reports that include Part 2 CSI reports in a PUCCH resource, the UE determines the PUCCH resource and a number of PRBs for the PUCCH resource or a number of Part 2 CSI reports assuming that each of the CSI reports and, if any, each CSI sub-report included in a CSI report, indicates rank 1, or rank combination of {1, 1} if applicable. If csi-ReportMode of CSI reports is set to 'Mode2', the UE determines the PUCCH resource and a number of PRBs for the PUCCH resource or a number of Part 2 CSI reports assuming that each CRI in the CSI report is associated with a resource pair. If the UE is provided CSI-ReportConfig that includes valueOfM, the UE determines a PUCCH resource and a number of PRBs for the PUCCH resource, or a number of Part 2 CSI reports, assuming that each CSI sub-report corresponding to each of M CRIs indicates rank 1. If the UE is provided CSI-ReportConfig that includes a codebookType set to 'typeI-SinglePanel-r19', the UE determines a PUCCH resource and a number of PRBs for the PUCCH resource, or a number of Part 2 CSI reports, assuming that the CSI report, or, if applicable, each CSI sub-report of the CSI report indicates rank 8 if codebookMode set to 'modeA' and typeI-SinglePanelRI-Restriction for the CSI report includes an allowed RI of rank 5, 6, 7 or 8, indicates rank 4 if codebookMode set to 'modeB', and indicates rank 1 otherwise. <Unchanged parts are omitted> ##### =====================End of Text Proposal to TS 38.213==================

Agreement:

Adopt the following changes in Section 5.2.2.2.5a in TS38.214:

The corresponding CR for Rel-19 and TS38.214 in R1-26xxxxx is endorsed.

Impacted spec(s): TS38.214

Reason for change: The proposed TP below was already endorsed verbatim in RAN1#122bis but was not captured in the latest version of TS38.214. Agreement (RAN1#122bis) The TP2.A for TS38.214 in R1-2507269 is endorsed.

Summary of the change: Added the missing oversampling parameters

Consequences if not approved: Endorsed TP in RAN1#122bis is not captured in TS38.214 – procedural error

##### 5.2.2.2.5a Refined eType II Codebook < Unchanged parts omitted > The bitmap parameter typeII-CBSR-r19 forms the bit sequence aAc-1,…,a1,a0, where a0 is the LSB and aAc-1 is the MSB and the number of bits is given by Ac=N1N2/X1(c)X2(c), where the values X1(c),X2(c) are configured by the higher layer parameters valueOfX1-typeII-CBSR-r19 and valueOfX2-typeII-CBSR-r19, respectively. The supported combinations of X1(c),X2(c) for a given combination of (N1,N2) are given in Table 5.2.2.2.5a-1. A bit value of zero for bit aN2X2(c)x1+x2 indicates that PMI reporting is not allowed to correspond to any precoder associated with vector group Ga(x1,x2), x1=0,…,N1X1(c)-1, x2=0,…,N2X2(c)-1, given by Gax1,x2=vX1(c)O1x1+l,X2(c)O2x2+m:l=0,1,…,X1cO1-1;m=0,1,…,X2cO2-1. ##### < Unchanged parts omitted >

Agreement:

Adopt the following changes in Section 5.2.1.4.1 in TS38.214:

The corresponding CR for Rel-19 and TS38.214 in R1-26xxxxx is endorsed.

Impacted spec(s): TS38.214

Reason for change: Two conflicting descriptions exist in the current version of the spec: 1) clause 5.2.1.4.1: “if interference measurement is performed on NZP CSI-RS, a UE does not expect to be configured with more than one NZP CSI-RS resource in the associated resource set within the resource setting for channel measurement” 2) clause 5.2.1.4.2: “Ks>1 CSI-RS resources are configured in the corresponding NZP-CSI-RS-ResourceSet for channel measurement” According to the second description, for M-CRI report, one NZP IMR can be configured and Ks>1 CSI-RS resources are configured in the corresponding NZP-CSI-RS-ResourceSet for channel measurement, which conflicts with the first description.

Summary of the change: Also, except for a CSI-ReportConfig with higher layer parameter valueOfM and reportQuantity set to 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI' and codebookType set to 'typeI-SinglePanel', or a CSI-ReportConfig with higher layer parameter valueOfM and reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-r16', if interference measurement is performed on NZP CSI-RS, a UE does not expect to be configured with more than one NZP CSI-RS resource in the associated resource set within the resource setting for channel measurement.

Consequences if not approved: If not clarified, the specification is inconsistent.

##### 5.2.1.4.1 Resource Setting configuration < Unchanged parts are omitted > Except for L1-SINR, codebookType set to 'typeII-CJT-r18', 'typeII-CJT-PortSelection-r18', 'typeII-Doppler-r18', or 'typeII-Doppler-PortSelection-r18', 'typeI-SinglePanel-r19', 'typeI-MultiPanel-r19', 'eTypeII-r19', 'typeII-FePortSelection-r19', or 'typeII-Doppler-r19', a CSI-ReportConfig with higher layer parameter valueOfM and reportQuantity set to 'cri-RI-PMI-CQI' or 'cri-RI-LI-PMI-CQI' and codebookType set to 'typeI-SinglePanel', or a CSI-ReportConfig with higher layer parameter valueOfM and reportQuantity set to 'cri-RI-PMI-CQI' and codebookType set to 'typeII-r16', if interference measurement is performed on NZP CSI-RS, a UE does not expect to be configured with more than one NZP CSI-RS resource in the associated resource set within the resource setting for channel measurement. Except for L1-SINR, the UE configured with the higher layer parameter nzp-CSI-RS-ResourcesForInterference may expect no more than 18 NZP CSI-RS ports configured in a NZP CSI-RS resource set. < Unchanged parts are omitted >

### Enhancement for asymmetric DL sTRP/UL mTRP scenarios

R1-2601154	Summary #1 on Rel-19 asymmetric DL sTRP/UL mTRP	Moderator (OPPO)

Agreement:

Adopt the following TPs for clause 7 and clause 7.4 for TS 38.213

The corresponding CR for Rel-19 for TS 38.213 in R1-26xxxxx is endorsed.

- Reason for change: (1) The RRC parameter for PL offset in 38.213 is not aligned with 38.331. (2) The text describing the determination of PL offset value for the case “if followUnifiedTCI-StateSRS is not provided for a SRS resource set…” is ambiguous. For example, regarding “when TCI-State or a TCI-UL-State includes pl-Offset”: it is not clear which of the TCI state or TCI-UL-State includes the pl-Offset. And furthermore, the pathlossOffset is included in but not associated with a TCI-State or a TCI-UL-State, which is the current description.

- Summary of changes: (1) Update the RRC parameter name for PL offset to align with 38.331. (2) Update the text description for PL offset determination for the case “if followUnifiedTCI-StateSRS is not provided for a SRS resource set…”.

- Consequences if not approved: (1) The RRC parameter in 38.213 is not aligned with 38.331 and (2) And the text describing the determination of PL offset value if followUnifiedTCI-StateSRS is not provided for a SRS resource set and for a SRS resource from the SRS resource set is not clear and there is a misalignment of whether the pathlossOffset is included in or associated with a TCI-State or a TCI-UL-State in the specification.

7 Uplink Power control < Unchanged parts are omitted > - else, if followUnifiedTCI-StateSRS is not provided for a SRS resource set and for a SRS resource from the SRS resource set, the values of PO_UE_SRS,b,f,cqs, αSRS,b,f,cqs, and SRS power control adjustment state l are provided by p0AlphaSetforSRS or, if p0AlphaSetforSRS-SBFD is provided and for an SRS transmission in SBFD symbols, by p0AlphaSetforSRS-SBFD, associated with TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set, a RS index qd for obtaining a pathloss estimate for the SRS transmission is provided by pathlossReferenceRS-Id-r17 associated with or included in the TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set, and a value of a PLb,f,c,offset, when TCI-State or a TCI-UL-State includes pl-Offset, is provided by pl-Offset pathlossOffset, if any, associated with included in TCI-State or TCI-UL-State of an SRS resource with lowest SRS-ResourceId in the SRS resource set PO_SRS,b,f,cqs is the sum of the component PO_UE_SRS,b,f,cqs and a component p0 provided by SRS-ResourceSet corresponding to the SRS resource set. In the remaining of this clause, except for clause 7.7.3, if in a PUSCH, PUCCH, or SRS transmission occasion i a UE applies a TCI-State or a TCI-UL-State that includes pl-Offset pathlossOffset with value PLb,f,c,offset in dB, where PLb,f,c,offset can be updated by a MAC CE [11, TS 38.321], the UE sets PLb,f,cqd=PLb,f,cqd-PLb,f,c,offset. < Unchanged parts are omitted > 7.4 Physical random access channel < Unchanged parts are omitted > If a PRACH transmission from a UE is in response to a detection of a PDCCH order by the UE that triggers a contention-free random access procedure, and if a pathloss offset indicator field in the PDCCH order [5, TS 38.212] indicates to the UE to apply pl-Offset pathlossOffset that is included in an indicated TCI-State or a TCI-UL-State and has value PLb,f,c,offset in dB, where PLb,f,c,offset can be updated by a MAC CE [11, TS 38.321], the UE sets PLb,f,c=PLb,f,c-PLb,f,c,offset for the PRACH transmission. < Unchanged parts are omitted >

Proposal :

Adopt the following TP for TS 38.214:

- Reason for change: Per the current specification, if the UE has two indicated TCI state but is not configured with tci-SelectionPresentInDCI, the UE shall apply both indicated TCI states on PDSCH reception. That is not aligned with the conclusion made in RAN1#117:

Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

- Summary of changes: Add text to specify that when UE is not configured with tci-SelectionPresentInDCI and is indicated with two indicated TCI states, and if one of the indicated TCI states includes PL offset, the UE shall apply the indicated TCI state without PL offset on PDSCH reception.

- Consequences if not approved: Per the current specification, for a UE configured with dl-OrJointTCI-StateList and having two indicated TCI-states, but not configured with tci-SelectionPresentInDCI, when the UE receives a DCI format 1_1/1_2 that schedules/activates PDSCH reception, the UE shall apply both indicated TCI-States to the scheduled or activated PDSCH reception which is still a mTRP transmission. Even if the two indicated TCI states can be same, there are additional scheduling restrictions and UE complexity different from sTRP transmission.

- 5.1.5 Antenna ports quasi co-location < Unchanged parts are omitted > When a UE is configured with dl-OrJointTCI-StateList and is having two indicated TCI-states: - Regardless of the offset between the reception of the scheduling DCI format 1_0/1_1/1_2 and the scheduled/activated PDSCH reception, if the UE is in frequency range 1, or the UE reports its capability of defaultQCL-TwoTCI in frequency range 2, or < Unchanged parts are omitted > - If the UE is not configured with tci-SelectionPresentInDCI and when the UE receives a DCI format 1_1/1_2 that schedules/activates PDSCH reception, if one of the indicated TCI-State or TCI-UL-State includes pathlossOffset, the UE shall apply the TCI-State that does not include the pathlossOffset or the TCI-State whose corresponding TCI-UL-State does not include the pathlossOffset; otherwise, the UE shall apply both indicated TCI-States to the scheduled or activated PDSCH reception. < Unchanged parts are omitted >