3GPP TSG RAN WG1 #124			R1-2601506

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Agenda Item:	9.2

Source:	Ad-Hoc Chair (Ericsson)

Title:	Session Notes of AI 9.2

Document for:	Endorsement

## NR MIMO Phase 6

Please refer to RP-252936 for detailed scope of the WI.

[124-R20-MIMO] Email discussion on Rel-20 MIMO – Darcy (MTK)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2601506	Session Notes of AI 9.2	Ad-Hoc Chair (Ericsson)

### Improvement of SRS capacity and coverage

Including a) Multiple frequency-domain starting positions for SRS repetition, and b) Cross-slot SRS between one U slot and one adjacent S slot.

R1-2600085	Improvement of SRS capacity and coverage	Huawei, HiSilicon

R1-2600098	Discussion on improvement of SRS capacity and coverage	Spreadtrum, UNISOC

R1-2600131	NR MIMO Phase 6: SRS Enhancement	InterDigital, Inc.

R1-2600176	Discussion on enhancement of SRS capacity and coverage for MIMO phase 6	OPPO

R1-2600225	Discussion on enhancement of SRS capacity and coverage for MIMO phase 6	TCL

R1-2600272	Discussion on improvement of SRS capacity and coverage	ZTE Corporation, Sanechips

R1-2600320	Discussion on enhancements for SRS capacity and coverage	CATT

R1-2600337	Improvement of SRS capacity and coverage	Tejas Network Limited

R1-2600350	Improvement of SRS capacity and coverage	MediaTek Inc.

R1-2600414	Discussion on the SRS capacity and coverage enhancement	Xiaomi

R1-2600483	Discussion on improvement of SRS capacity and coverage	vivo

R1-2600618	Improvement of SRS Capacity and Coverage	Nokia

R1-2600675	Discussion on improvement of SRS capacity and coverage	NEC

R1-2600682	Discussion on SRS capacity and coverage improvement	China Telecom

R1-2600710	On Rel-20 improvement of SRS capacity and coverage	Ericsson

R1-2600740	Views on Improvement of SRS capacity and coverage	Samsung

R1-2600779	Improvement of SRS capacity and coverage	Lenovo

R1-2600813	On Rel-20 MIMO SRS capacity and coverage improvement	Apple

R1-2600866	Discussion on improvement of SRS capacity and coverage	Fujitsu

R1-2600881	Discussion on improvement of SRS capacity and coverage	Panasonic

R1-2600888	Discussion on Enhancing DL CSI acquisition	LG Electronics

R1-2600941	Discussion on improvement of SRS capacity and coverage	HONOR

R1-2600990	Discussion on improvement of SRS capacity and coverage for NR MIMO Phase 6	ETRI

R1-2601121	Further discussion on improvements of SRS capacity and coverage	Sony

R1-2601164	Discussion on Improvement of SRS capacity and coverage	NTT DOCOMO, INC.

R1-2601257	SRS enhancements in 5G MIMO Phase 6	Qualcomm Incorporated

R1-2601317	Improvement of SRS capacity and coverage	Sharp

R1-2601328	Discussion on Improvement of SRS Capacity and Coverage	Rakuten Mobile, Inc

R1-2601357	Views on improvement of SRS capacity and coverage	KDDI Corporation

R1-2600321	Moderator summary on improvement of SRS capacity and coverage: Round1	Moderator (CATT)

R1-2600323	Moderator summary on improvement of SRS capacity and coverage: Round3	Moderator (CATT)

R1-2600324	Moderator summary on improvement of SRS capacity and coverage: Round4	Moderator (CATT)

Agreement:

For K<R, the following exact patterns of starting position hopping within a hop across repetition symbols  can be supported for a given K according to the configured PF, as derived from the corresponding basic pattern and kF={0,1,…,PF-1}:

- {0,…,0, 1,…,1} and {1,…,1, 0,…,0} when PF=K=2.

- {0,…,0, 2,…,2}, {1,…,1, 3,…,3}, {2,…,2, 0,…,0} and {3,…,3, 1,…,1} when PF=4 and K=2.

- {0,…,0, 2,…,2, 1,…,1, 3,…,3}, {1,…,1, 3,…,3, 2,…,2, 0,…,0}, {2,…,2, 0,…,0, 3,…,3, 1,…,1} and {3,…,3, 1,…,1, 0,…,0, 2,…,2} when PF=K=4.

Conclusion:

For intra-repetition hopping for SRS repetition symbols within each SRS frequency hop, the case when R is not an integer multiple of K is not supported.

Agreement:

Support PUSCH transmission with a priority index 0 and corresponding DMRS after all the SRS resource(s) of the same resource set with time-domain resource allocation entirely in the U slot for scenario 1 and scenario 2 (if scenario 2 is agreed).

R1-2600322	Moderator summary on improvement of SRS capacity and coverage: Round2	Moderator (CATT)

Agreement:

For the time-domain location determination of aperiodic SRS resources within the cross-slot resource set, introduce per-resource slot offset.

    - FFS: whether offset is included in the resource configuration or resource set configuration

The slot offset configured to an AP cross-slot SRS resource refers to the first of the two slots spanned by the SRS resource.

Agreement:

Scenario 2: an aperiodic SRS resource set which includes at least one SRS resource with time-domain resource transmitted in a first S slot, and at least one another SRS resource with time-domain resource transmitted in a second consecutive U slot.

  - For an aperiodic cross-slot SRS resource set in scenario 2, the slot offset of the SRS resource set refers to the first of the two slots spanned by the SRS resource set.

Agreement:

Support Alt B from the agreement in RAN1 #123 meeting as follows:

There is no restriction on simultaneous enabling of the R17 RPFS start RB index hopping across multiple legacy SRS frequency hopping periods and intra-repetition hopping for SRS repetition symbols within each SRS frequency hop.

For PF=4 and K=2, support RPFS start RB index hopping across multiple legacy SRS frequency hopping periods with new pattern(s) for intra-repetition hopping for SRS repetition symbols within each SRS frequency hop.

- support new starting RB hopping pattern {0, 1}

Note: The cases PF=2, K=2, and PF=4, K=4 can be discussed separately.

### Enhancing DL CSI acquisition

Including a) Early SRS/CSI/CSI-RS triggering, and b) CSI-RS density reduction for 48, 64, and 128 CSI-RS ports.

R1-2600064	Enhancing DL CSI acquisition	FUTUREWEI

R1-2600086	DL CSI acquisition enhancement	Huawei, HiSilicon

R1-2600099	Discussion on enhancing DL CSI acquisition	Spreadtrum, UNISOC

R1-2600132	NR MIMO Phase 6: DL CSI Enhancement	InterDigital, Inc.

R1-2600177	Discussions on Enhancing DL CSI Acquisition	OPPO

R1-2600226	Discussions on Enhancing DL CSI Acquisition	TCL

R1-2600273	Discussion on enhancing DL CSI acquisition	ZTE Corporation, Sanechips

R1-2600325	Discussion on enhancing DL CSI acquisition	CATT

R1-2600338	Enhancing DL CSI acquisition	Tejas Network Limited

R1-2600351	Enhancing DL CSI acquisition	MediaTek Inc.

R1-2600371	On DL CSI Acquisition Enhancements for FR1	Nokia

R1-2600415	Discussion on enhancing DL CSI acquisition	Xiaomi

R1-2600484	Discussion on enhancing DL CSI acquisition	vivo

R1-2600599	Discussion on enhancing DL CSI acquisition	Ofinno

R1-2600655	Discussion on Enhancing DL CSI acquisition	NEC

R1-2600741	Views on Enhancing DL CSI acquisition	Samsung

R1-2600777	Enhancing DL CSI acquisition	Lenovo

R1-2600814	On Rel-20 MIMO CSI enhancement	Apple

R1-2600867	Discussion on enhancing DL CSI acquisition	Fujitsu

R1-2600981	Discussion on early DL CSI acquisition design	Fainity Innovation

R1-2600991	Discussion on enhancing DL CSI acquisition for NR MIMO Phase 6	ETRI

R1-2601067	Discussion on Early DL CSI Acquisition Enhancements	Panasonic

R1-2601122	Further discussion on DL CSI acquisition enhancements	Sony

R1-2601165	Discussion on Enhancing DL CSI acquisition	NTT DOCOMO, INC.

R1-2601204	Discussion on enhancing DL CSI acquisition	Google

R1-2601236	Discussion on DL CSI acquisition	ITRI, Acer Incorporated

R1-2601258	DL CSI acquisition enhancements in 5G MIMO Phase 6	Qualcomm Incorporated

R1-2601289	Enhancing DL CSI acquisition	Sharp

R1-2601327	Enhanced DL CSI acquisition for MIMO Phase 6	Ericsson

R1-2601332	Discussion on Enhancement of DL CSI Acquisition	Rakuten Mobile, Inc

R1-2601405	Discussion on Enhancing DL CSI acquisition	LG Electronics

R1-2601406	Discussion on enhancing DL CSI acquisition	NICT

R1-2600647	Moderator summary on enhancing DL CSI acquisition (Round 0)	MediaTek Inc.

R1-2600648	Moderator summary on enhancing DL CSI acquisition (Round 1)	MediaTek Inc.

R1-2600649	Moderator summary on enhancing DL CSI acquisition (Round 2)	MediaTek Inc.

R1-2600650	Moderator summary on enhancing DL CSI acquisition (Round 3)	MediaTek Inc.

Conclusion:

There is no RAN1 consensus to support periodic TRS triggered by a MAC-CE in MSG4 when UE transition from IDLE/INACTIVE to CONNECTED mode.

Agreement:

Support a MAC-CE in MSG4 PDSCH for triggering aperiodic SRS-AS/CSI-RS indicates at least

- The slot offset used to determine the transmission slot of aperiodic SRS-AS if triggered,

- The slot offset used to determine the transmission slot of aperiodic CSI-RS associated with AP CSI reporting if triggered, and

- The slot offset used to determine the transmission slot of aperiodic CSI-RS of the first TRS burst, if triggered.

- The slot offset used to determine the transmission slot of the PUSCH for aperiodic CSI reporting, if triggered, via the UL grant provided in the MAC-CE

  - FFS: How to indicate the slot offset by the UL grant provided in the MAC-CE

Note: Whether to indicate the other settings (i.e., comb offset, repetition factor, cyclic shift and the number of TRS bursts, the parameters (e.g., slot offset, time gap) to determine the Tx slot(s) of the TRS burst(s), if triggered, other than the first TRS burst) via MAC-CE in MSG4 are separately discussed.

Agreement:

For aperiodic CSI reporting for a SCell(s), triggered based on a legacy SCell activation command activating the SCell(s), the SCell activation command indicates a UL grant for a PUSCH to carry the aperiodic CSI report(s) for the CSI report configuration(s) triggered for the SCell(s)

- Strive to reuse legacy UL grant design in RAR

- The aperiodic CSI report(s) is reported in the same PUSCH.

- FFS: The serving cell where the PUSCH is scheduled in

Agreement:

To determine the transmission/reception slot of the DL/UL signalling triggered/scheduled based on a legacy SCell activation command (i.e., aperiodic SRS-AS, aperiodic CSI-RS and the PUSCH scheduled for aperiodic CSI reporting), for applying a slot offset between a reference slot and a transmission/reception slot of the DL/UL signalling triggered/scheduled based on the legacy SCell activation command, the reference slot is the first slot that is 3ms after UE transmits HARQ-ACK for the PDSCH carrying the SCell activation command

Agreement

To determine the transmission/reception slot of a MSG4-triggered DL/UL signalling (i.e., aperiodic SRS-AS, aperiodic TRS, aperiodic CSI-RS and the PUSCH scheduled for aperiodic CSI reporting), for applying a slot offset between a reference slot and a transmission/reception slot of the MSG4-triggered DL/UL signalling, the reference slot is the first slot that is 3ms after UE transmits HARQ-ACK for the MSG4 PDSCH carrying the MAC-CE.

Conclusion:

There is no RAN1 consensus to support the followings in UE transition from IDLE/INACTIVE to CONNECTED mode:

- Aperiodic SRS-AS must be always triggered along with aperiodic CSI reporting and associated CSI-RS, via a MSG4

- Aperiodic TRS must be always triggered along with aperiodic CSI reporting and associated CSI-RS, via a MSG4