3GPP TSG RAN WG1 #124			R1-2601510

Gothenburg, SE, Feb. 9th ~ 13th, 2026

Agenda Item:	9.6

Source:	Ad-Hoc Chair (Ericsson)

Title:	Session Notes of AI 9.6

Document for:	Endorsement

## Non-Terrestrial Networks (NTN) for NR Phase 4

Please refer to RP-253137 for detailed scope of the SI for NR-NTN Phase 4.

R1-2600253	Work plan for NR NTN Phase 4	Moderator (THALES)

R1-2601321	Draft TR 38.742 Study on GNSS (Global Navigation Satellite System) resilient NR-NTN (Non-Terrestrial Networks) operation	Rapporteur (Thales)

[124-R20-NR-NTN] Email discussion on Rel-20 NR-NTN – Mohamed (Thales)

- To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

R1-2601510	Session Notes of AI 9.6	Ad-Hoc Chair (Ericsson)

### NR-NTN GNSS resilience

Conclusion:

- Companies’ inputs for PRACH performance evaluation results are collected in Excel sheet “PRACH performance evaluation v022” that is embedded in R1-2601483.

- The template for collection of Connected mode results in Excel sheet “Template-Results for Connected mode_v004,” that is embedded in R1-2601483, is endorsed.

R1-2600065	Discussion on Rel-20 GNSS resilient NR NTN operation	FUTUREWEI

R1-2600077	GNSS resilience for NR-NTN	Huawei, HiSilicon

R1-2600106	Discussion on NR-NTN GNSS resilience	Spreadtrum, UNISOC

R1-2600184	Discussion on NR-NTN GNSS resilience	OPPO

R1-2600237	On NR-NTN GNSS resilience	Ericsson

R1-2600248	Considerations on GNSS resilient NR-NTN operation	THALES

R1-2600259	Discussion on NR-NTN GNSS resilience	ZTE Corporation, Sanechips

R1-2600332	Discussion on NR-NTN GNSS resilience	CATT

R1-2600341	GNSS Resilient operation for NR-NTN - 5G Adv	Tejas Network Limited

R1-2600382	Discussion on NR-NTN GNSS resilience	CMCC

R1-2600421	Further discussion on NR NTN GNSS resilience	Xiaomi

R1-2600449	Discussion on the GNSS resilient NR-NTN operation	TCL

R1-2600496	Discussions on NR-NTN GNSS resilience	vivo

R1-2600524	Discussion on NR-NTN GNSS resilience	Fraunhofer IIS, Fraunhofer HHI

R1-2600577	IMU Views on NR-NTN GNSS Resilience	IMU

R1-2600581	NR-NTN GNSS resilience	InterDigital, Inc.

R1-2600619	Discussion on GNSS resilient NR-NTN operation	TOYOTA Info Technology Center

R1-2600677	Discussion on NR-NTN GNSS resilience	NEC

R1-2600690	Discussion on NR-NTN GNSS resilience	China Telecom

R1-2600748	Discussion on NR-NTN GNSS resilience	Samsung

R1-2600784	GNSS-resilient NR-NTN operation	ST Engineering iDirect

R1-2600794	Considerations for GNSS-resilient NR-NTN operation	ESA, Thales

R1-2600806	Discussion on NR NTN GNSS Resilience	Amazon Web Services

R1-2600820	Discussion on NR-NTN GNSS resilience	Apple

R1-2600858	NR NTN GNSS resilient operation	Sharp

R1-2600903	Discussion on NR-NTN GNSS resilience	MediaTek Inc.

R1-2600943	Discussion on NR-NTN GNSS resilience	HONOR

R1-2600962	Discussion on GNSS resilient NR-NTN operation	LG Electronics

R1-2600971	Discussion on GNSS resilient NR-NTN	Panasonic

R1-2600996	Discussion on NR-NTN GNSS resilient operations	ETRI

R1-2601041	Location uncertainty reduction and mitigation techniques	Eutelsat Group

R1-2601058	Discussion on GNSS resilient NR-NTN operation	Lenovo

R1-2601060	Discussion on GNSS resilient operation for NR over NTN	Nokia

R1-2601077	Discussion on NR-NTN GNSS resilience	Airbus

R1-2601090	Discussion on NR-NTN GNSS resilience	Ofinno

R1-2601126	On Solutions for NR-NTN GNSS Resilience	Sony

R1-2601172	Discussion on NR-NTN GNSS resilience	NTT DOCOMO, INC.

R1-2601231	Discussion of Using DL Timestamps for Initial Access	Johns Hopkins University APL

R1-2601265	NR NTN GNSS resilience	Qualcomm Incorporated

R1-2601318	Discussion on NR-NTN GNSS resilience	Google Korea LLC

R1-2601322	Discussion on NR-NTN GNSS resilience	CAICT

R1-2601391	Discussion on GNSS Resilient Operation for NR NTN	CEWiT

R1-2601410	Discussion on GNSS resilience for NR-NTN	CSCN

R1-2601461	GNSS resilience for NR-NTN	Huawei, HiSilicon

(Revision of R1-2600077)

R1-2600249	FL Summary #1: Study on GNSS resilient NR-NTN operation	Moderator (THALES)

R1-2600251	FL Summary #3: Study on GNSS resilient NR-NTN operation	Moderator (THALES)

R1-2600252	FL Summary #4: Study on GNSS resilient NR-NTN operation	Moderator (THALES)

Agreement:

Confirm the following working assumption.

Working assumption: For UL performance evaluation, UL differential Doppler/frequency offset is calculated based on scaled DL one-way differential Doppler/frequency offset with the following scaling factor: Scaling factor = 2*fc,ul / fc,dl fc,ul and fc,dl are the uplink and downlink carrier frequency respectively.

Agreement:

Capture the following formula in the TR 38.742:

dopplerone-wayu=vsatc2fcu-s,xdone-wayu+vUEc2fcu-s,kdone-wayu

Where vsat is the speed of satellite and vUE is the speed of the UE.

Agreement:

Confirm the working assumptions from RAN1#123 regarding differential delay and Doppler.

Agreement:

Confirm the working assumption on theoretical differential Doppler limits as follows:

Theoretical differential Doppler limits of each PRACH preamble with for  long PRACH preamble formats considered for the study are given in the following table:

| Preamble format | Set | ΔfRA [Hz] | γ | fe [ppm] | fc,UL [GHz] | fc,DL [GHz] | scaling factor | Differential one-way Doppler limit [ppm] | Differential round trip Doppler limit in UL [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0,1,2 | unrestricted | 1250 | 0 | 0.1 | 2 | 2 | 2.0 | 0.21 | 0.85 |
| 0,1,2 | type A | 1250 | 1 | 0.1 | 2 | 2 | 2.0 | 0.53 | 2.1 |
| 0,1,2 | type A | 1250 | 2 | 0.1 | 2 | 2 | 2.0 | 0.84 | 3.35 |
| 0,1,2 | type B | 1250 | 3 | 0.1 | 2 | 2 | 2.0 | 1.15 | 4.6 |
| 0,1,2 | type B | 1250 | 4 | 0.1 | 2 | 2 | 2.0 | 1.46 | 5.85 |
| 3 | unrestricted | 5000 | 0 | 0.1 | 2 | 2 | 2.0 | 1.15 | 4.6 |
| 3 | type A | 5000 | 1 | 0.1 | 2 | 2 | 2.0 | 2.40 | 9.6 |
| 3 | type A | 5000 | 2 | 0.1 | 2 | 2 | 2.0 | 3.65 | 14.6 |
| 3 | type B | 5000 | 3 | 0.1 | 2 | 2 | 2.0 | 4.90 | 19.6 |
| 3 | type B | 5000 | 4 | 0.1 | 2 | 2 | 2.0 | 6.15 | 24.6 |

Theoretical differential Doppler limits of each PRACH preamble with for  short PRACH preamble formats considered for the study are given in the following table:

| Preamble format | SCS | ΔfRA [Hz] | γ | fe [ppm] | fc,UL [GHz] | fc,DL [GHz] | scaling factor | Differential one-way Doppler limit [ppm] | Differential round trip Doppler limit in UL [kHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| all short formats | 15 | 15000 | 0 | 0.1 | 2 | 2 | 2.0 | 3.65 | 14.6 |
|  | 30 | 30000 | 0 | 0.1 | 2 | 2 | 2.0 | 7.40 | 29.6 |
|  | 60 | 60000 | 0 | 0.1 | 30 | 20 | 2.5 3.0 | 1.08 0.90 | 54 |
|  | 120 | 120000 | 0 | 0.1 | 30 | 20 | 2.5 3.0 | 2.28 1.90 | 114 |

Note: These limits are assumed to represent theoretical upper bounds.

Agreement:

Confirm the RAN1#123 working assumptions on differential roundtrip delay limits for PRACH preamble formats considered for the study.

R1-2600250	FL Summary #2: Study on GNSS resilient NR-NTN operation	Moderator (THALES)

Observation:

Based on the evaluation results of the study on GNSS resilient operation:

For scenario 1; where the UE cannot rely on its GNSS for timing and frequency compensation on the service link,

- There is no single PRACH preamble with long PRACH format that could tolerate the differential delay and Doppler simultaneously across all elevation angles in LEO-600 S-band, LEO-1200 S-band and GEO in S-band in case a, where the location uncertainty is the area served by beam.

- There is no single PRACH preamble with short PRACH format that could tolerate the differential delay and Doppler simultaneously across all elevation angles in LEO-600 S-band, LEO-1200 S-band and GEO in S-band and Ku/Ka band, in case a, where the location uncertainty is the area served by beam.

For scenario 2; where there is a previously acquired GNSS based position, the workable PRACH formats (a PRACH format is workable if the PRACH format can tolerate the differential delay and Doppler) across all angles are provided in the tables below for different PRACH formats, uncertainty area X (km) and operating bands:

- Long PRACH Formats | S band

| Orbit/X | Workable PRACH formats across all angles |
| --- | --- |
| LEO 600 km, X=1 km | All formats (0–3; Unrestricted/Type A/Type B; all γ) |
| LEO 600 km, X=5 km | Format 0-2 Type A/B all γ; Format 3 Unrestricted |
| LEO 600 km, X=10 km | Format 1 type B, format 2 type B |
| LEO 600 km, X=25 km | None |
| LEO 1200 km, X=1 km | All formats (0–3; Unrestricted/Type A/Type B; all γ) |
| LEO 1200 km, X=5 km | All formats, except Format 3 Type A/B |
| LEO 1200 km, X=10 km | Format 1 type B, format 2 type B |
| LEO 1200 km, X=25 km | None |
| GEO, X=1 km | All formats (0–3; Unrestricted/Type A/Type B; all γ) |
| GEO, X=5 km | All formats, except Format 3 Type A/B |
| GEO, X=10 km | Format 1 Unrestricted/Type A, format 2 Unrestricted/Type A |
| GEO, X=25 km | Format 1 Unrestricted |

- Short PRACH Formats | S band

| Orbit/X | Workable PRACH formats across all angles |
| --- | --- |
| LEO 600 km, X=1 km | A2-15; A3-15/30; B3-15; B4-15/30; C0-15/30; C2-15/30 |
| LEO 600 km, X=5 km | C2-15 only |
| LEO 600 km, X=10 km | None |
| LEO 600 km, X=25 km | None |
| LEO 1200 km, X=1 km | A2-15; A3-15/30; B3-15; B4-15/30; C0-15/30; C2-15/30 |
| LEO 1200 km, X=5 km | C2-15 only |
| LEO 1200 km, X=10 km | None |
| LEO 1200 km, X=25 km | None |
| GEO, X=1 km | A2-15; A3-15/30; B3-15; B4-15/30; C0-15/30; C2-15/30 |
| GEO, X=5 km | C2-15 only |
| GEO, X=10 km | None |
| GEO, X=25 km | None |

- Short PRACH Formats | Ku/Ka band

| Orbit/X | Workable PRACH formats across all angles |
| --- | --- |
| LEO 600 km, X=1 km | Ku: A2-15; A3-15/30; B3-15; B4-15/30; C0-15/30; C2-15/30/60 Ka: C2-60 |
| LEO 600 km, X=5 km | None |
| LEO 600 km, X=10 km | None |
| LEO 600 km, X=25 km | None |
| LEO 1200 km, X=1 km | Ku: A2-15; A3-15/30; B3-15; B4-15/30; C0-15/30; C2-15/30/60 Ka: C2-60 |
| LEO 1200 km, X=5 km | Ku: C2-15 Ka: None |
| LEO 1200 km, X=10 km | None |
| GEO, X=1 km | Ku: A2-15; A3-15/30; B3-15; B4-15/30; C0-15/30; C2-15/30/60 Ka: C2-60 |
| GEO, X=5 km | Ku: C2-15 Ka: None |
| GEO, X=10 km | None |
| GEO, X=25 km | None |

Note 1: The tolerance gaps for non-workable PRACH formats and tolerance margin for workable PRACH can be found in “PRACH performance evaluation v022,” embedded in R1-2601483.

Note 2: These results have been collected from different sources under the following assumptions:

- UE altitude of 0 km is assumed

- UE altitude uncertainty is not taken into account.

- For Set1/2 parameters, it is assumed that the area served by the cell or beam (corresponding to the nadir beam size as defined in 38.821) is fixed within the satellite coverage.

Note 3: Some sources have reported larger TO/FO tolerance gaps than the reported values in R1-2601483 in case of elongated non-nadir beam at edge of the satellite coverage (larger than the nadir beam size as defined in 38.821).

Conclusion/Observation

For GNSS resilient NR NTN operation, at least for scenario 1, enhancements to the existing NR initial access procedures (including Random access procedures) are required to cope with the large residual Doppler shifts and differential delays encountered in various NTN scenarios.