3GPP TSG RAN WG1 #122b	                                	R1-2507992

Prague, Czech Republic, October 13th – 17th, 2025

Agenda item:	5

Source: 	Moderator (Qualcomm Incorporated)

Title: 	Feature lead summary #1 for IMS voice over NB-IoT NTN

Document for:	Discussion

- Background

In R1-2506707, RAN1 received an LS from SA2 with the following question for RAN1:

During the voice call, if enough consecutive packets are lost or erroneously decompressed, the compressor can find itself leave Second Order (SO) state and enter First Order (FO) state, and then if another enough consecutive packets are lost or erroneously decompressed, the compressor can find itself leave First Order state (FO) and enter the Initialization and Refresh (IR) state, causing gaps in the transmission of the voice packets.Some companies indicated in SA2 that the number of consecutive lost packets that will trigger the compressor to fall back from SO state to FO state is around 16 (e.g., when UO-0 header with 4 bits SN is used in SO state), and the number of consecutive lost packets that will trigger the compressor to fall back from FO state to IR state is around 64 (e.g., when UOR-2 header with 6 bits SN is used in FO state). […] Question 2 (To RAN1): Can RAN1 provide any data regarding the probability that such number of consecutive packets e.g. 16 or 64 can be lost or erroneously decompressed? How often such event can occur?

- Discussion

The following input has been received regarding the above LS:

| Tdoc number | Source | Quick summary |
| --- | --- | --- |
| R1-2506863 | vivo, Spreadtrum | Negligible probability, based on LLS |
| R1-2506906 | ZTE | Negligible probability based on LLS, assumes independence of events |
| R1-2506957 | Xiaomi | Negligible probability assuming independence of events for 2% BLER |
| R1-2507085 | CATT | Negligible probability assuming independence of events for 1% BLER |
| R1-2507141 | OPPO | Negligible probability based on LLS. |
| R1-2507218. | Samsung | Negligible probability assuming independence of events for 2% BLER |
| R1-2507297 | Nokia | Highly unlikely to have consecutive losses, no simulations / calculations |
| R1-2507320 | NEC | Need modelling for NLOS/LOS transitions. Need to model “packet collisions” when the PHY data rate is lower than the codec rate. |
| R1-2507919 | Huawei | Probability is negligible based on calculation and independence |
| R1-2507263 | Ericsson | Theoretical calculation based on to Shannon capacity, probability is negligible. |
| R1-2507687 | Qualcomm | For LLS with fixed receiver, probability is negligible. For LOS/NLOS, probability is not negligible. Take into account other issues (GNSS reading / SIB31 acquisition). |

Feature lead observes the following:

- 3 inputs (vivo/SPDR, OPPO, Qualcomm) performed link level simulations with correlated fading channels. Under the typical link level assumptions used in RAN1, all of them observe that the probability of 16/64 packets being consecutively lost is negligible.

- The majority of inputs (ZTE, Xiaomi, CATT, Samsung, Nokia, Ericsson, Huawei) state that the probability of losing 16/64 packets is negligible, based on theoretical calculations / approximations / extrapolations of BLER curves assuming event independence.

- Two inputs (Qualcomm, NEC), state that LOS/NLOS transitions should be considered, and Qualcomm concludes based on simulations of LOS/NLOS transitions that the probability of losing 16/64 packets is not negligible.

- Two inputs (Qualcomm, NEC) propose to consider other factors than channel performance to reply to SA2:

  - Qualcomm observes that GNSS reacquisition and SIB31 reading will cause interruptions in the reception of packets, which can exceed 1s.

  - NEC proposes to study what would be the impact of packet collisions that may happen when the instantaneous transmission rate exceeds the one supported by the channel.

Feature lead proposes to discuss the input around the following 4 bullets for the reply to SA2:

- Assuming line-of-sight condition for the duration of the call, and no change in large scale parameters (e.g. shadowing), RAN1 concludes that the probability of having 16/64 packets consecutively lost is negligible under typical operating conditions (e.g. 2% packet error rate)

  - [TBD if we add more details in this bullet, to be discussed]

- Assuming a mobile UE that transitions between line-of-sight and non-line-of sight conditions (and vice-versa) during the call, RAN1 concludes that the probability of having 16/64 packets consecutively lost is non-negligible.

  - [TBD if we add more details in this bullet, to be discussed]

- If a voice packet needs to be transmitted for a duration longer than the inter-packet arrival rate (due to e.g. RoHC moving to a state that requires more bits to be transmitted), packet drops may be produced. RAN1 would need further input from RAN2/SA4 to further analyze this impact

  - [TBD if we add more details in this bullet, to be discussed]

- Some procedures needed for the operation of NB-IoT NTN, such as GNSS reacquisition and SIB19 reading, may result in interruption of reception / transmission of packets longer than 1 second.

  - [TBD if we add more details in this bullet, to be discussed]

Q4-1: Please provide your comments on the proposed bullets above:

| Company | Comment |
| --- | --- |
| Ericsson | We think that the response to SA2 can focus on the first and second bullets, whereas the aspects mentioned in the last two bullets can be further studied. The responses in the submitted T-docs span from very simple responses to overly detailed responses. On this matter, we think that the response should be something in between (i.e., not too simplistic, and not too detailed) and at the same time clear and concise. Having said, we propose either of the following alternatives as response (which aim at accounting for Companies views): Alt-1: RAN1 concluded that under LOS conditions in an SNR range from -10dB to -2dB, the probability of 16 or 64 consecutive packet losses is very low (~ negligible) for voice codec rates spanning from fractions of Kbps to several Kbps. Error-correcting methods available in NB-IoT NTN including the use of repetitions contribute to meet the BLER target. Assuming a mobile user for which the line-of-sight condition changes during the call (from line-of-sight LOS to non-line-of-sight NLOS and vice-versa) the probability of having consecutive packet losses exceeding 1 second is not negligible (from slightly above 1 second up to several seconds depending on the mobile speed). Alt-2: RAN1 concluded that under LOS conditions (which is the typical scenario for GEO IMS voice over NTN), in an SNR range from around minus ten dBs to around zero dBs, the probability of 16 or 64 consecutive packet losses is very low (~ negligible) for voice codec rates spanning from fractions of Kbps to several Kbps. Error-correcting methods available in NB-IoT NTN including the use of repetitions contribute to meet the BLER target. The probability of having consecutive packet losses exceeding 1 second is not negligible in non-line-of-sight (NLOS) condition. However, the probability of NLOS in GEO is expected to be low, and NLOS is not typical scenario for GEO IMS voice over NTN from RAN1 perspective. |
| CATT | First, we share same view with Ericsson. The response to SA2 can focus on the first and second bullets, which are relevant to SA2 questions. Secondly, for GEO case, in general the SINR range is a bit low, so the working scenario of GEO should focus on the LOS condition. For NLOS condition, the additional pathloss will be increased with more than 10dB. Therefore, for NLOS condition, the performance of BLER is not mandatory target, just with the best effort. We think the prerequisite channel condition for GEO should be discussed firstly. Based on R17 evaluation, the NLOS channel is not the targeted channel. Hence, for RAN1 evaluation, it should focus on the LOS channel. |
| Nokia, NSB | We think the first two bullets can be discussed in RAN1 and RAN1 need to reply SA2 on these two bullets. For the fourth bullet, maybe there will be impact but need discussion in RAN1. First of all, we think the first bullet is the most important case to be discussed. While for the second bullet, we think the probability of changing from LOS to NLOS is very small and even not considered in the previous evaluations in RAN1. Additionally, when changing from LOS to NLOS, the impact may be not only packet error but there may be lost of the connection, thus only discuss the packet error rate is not the motivation. For the fourth bullet, we think it will be good to discuss and make it clear on the impact of the interruption on the packet error rate/performance. |
| OPPO | We prefer to only focus on the LOS scenario in R20 IMS voice over NB-IoT NTN, and NLOS is not our preferable scenario throughout the NTN study, i.e., from R17 NTN to R19 NTN. We note that, during the SI of R17 IoT NTN, the shadow fading and clutter loss in NLOS condition is not considered in the link budget calculation. Therefore, the 2nd bullet should be removed. |
| Vivo1 | We share a similar view as other companies that bullt3/4 are not much relevant to the LS. The most important part is bullet1 about LOS. Bullet1: We have also evaluated 10% BLER target and observed the probability of having 16/64 packets consecutively lost under 10% BLER target is still negligible. Therefore, it can be concluded that the probability is negligible for target BLER up to 10%. In addition, we suggest adding an upper boundary for the data rate based on companies’ inputs, otherwise SA may interpret it as any data rate can be supported. In our evaluation, data rates up to 2.4 kbps were considered, and Ericsson has evaluated a TBS of 328 bits, which results in an L1 data rate of approximately 4.1 kbps. Therefore, it is suggested to set the upper boundary around 4.1 kbps. We would also like to highlight that LOS is the typical case considered for GEO NTN, and no enhancement or special consideration for NLOS scenarios in GEO NTN has been introduced in previous releases. Therefore, we prefer to clarify this aspect to SA2. The bullet is refined as follows: - Assuming line-of-sight condition (which is the typical case for GEO NTN) for the duration of the call, and no change in large scale parameters (e.g. shadowing), RAN1 concludes that the probability of having 16/64 packets consecutively lost for data rate<=4.1 kbps is negligible under typical operating conditions (e.g. <=10% packet error rate 2% packet error rate) - [TBD if we add more details in this bullet, to be discussed] The first paragraph on LS in alt2 from Ericsson is also fine, but we prefer to change the SNR range (because we have evaluated 10 db) and the boundary of the data rate based on inputs. And in our understanding, higher power UE and better NF may also contribute to meet the BLER target. Thus, we proposed to refine the Alt2 as below Alt-2 rev: RAN1 concluded that under LOS conditions (which is the typical scenario for GEO IMS voice over NTN), in an SNR range from around minus ten dBs to around zero ten dBs, the probability of 16 or 64 consecutive packet losses is very low (~ negligible) for voice codec rates spanning from fractions of Kbps to several around 4.1 Kbps. Higher power UE power, better noise figure, and Eerror-correcting methods available in NB-IoT NTN including the use of repetitions contribute to meet the BLER target. Bullet2: First, the precondition of this discussion is that a voice call has been successfully established, which implies that the target BLER is already met. However, under NLOS conditions, the communication link may be completely lost, and even the RRC connection may not be maintained. In that case, evaluating the probability of consecutive packet losses becomes meaningless, as the link itself is no longer available. Second, as also commented under bullet 1, NLOS is not a typical case for GEO NTN discussions. The probability of NLOS in 50 degree elevation angle is only 6.5%. No enhancement or special consideration for NLOS in GEO NTN has been introduced in previous releases. More specifically, in the Rel-18 NTN, there is an agreement saying that NLOS is de-prioritized for GEO. In the Rel-19 NTN WI, NLOS is not the focus area. It is also unclear why this aspect suddenly needs to be considered for voice via GEO NTN. Furthermore, the observation is from single company, and we would like to point out that, the transitions between LOS and NLOS conditions have not been studied for NTN. Based on the above reasons, we suggest focusing on bullet1. - Assuming a mobile UE that transitions between line-of-sight and non-line-of sight conditions (and vice-versa) during the call, RAN1 concludes that the probability of having 16/64 packets consecutively lost is non-negligible. Bullet3: The case where “a voice packet needs to be transmitted for a duration longer than the inter-packet arrival rate” is clearly due to misconfiguration or improper scheduling, which the network should avoid. Therefore, this observation is not correct. It is true that the packet size may change due to an RoHC state change, but how to hand this varied size is part of the ongoing SA/RAN2 discussion. SA has already considered potential solutions, such as configuring multiple SPS for different packet sizes, scheduling more resources to accommodate the varied sizes, or using DG. RAN1 does not need to reiterate this aspect to SA, and SA2 has not requested input from RAN1 on this aspect. Therefore, this bullet is outside the scope of the LS reply and should be removed. - If a voice packet needs to be transmitted for a duration longer than the inter-packet arrival rate (due to e.g. RoHC moving to a state that requires more bits to be transmitted), packet drops may be produced. RAN1 would need further input from RAN2/SA4 to further analyze this impact Bullet4: First, as we are discussing IoT NTN, it should be SIB31 reading instead of SIB19. We do not see how this point is related to Question 2 of the LS. SA2 is asking about the probability of packet loss, which concerns link-level performance of IMS voice packets, while GNSS reacquisition and SIB31 reading are system-level procedures. GNSS reacquisition interrupts all NB-IoT transmissions/receptions for NB-IoT devices, while SIB31 reading suspends all uplink transmissions when t318 is running. Therefore, these aspects are outside the scope of the link-level performance discussion. Furthermore, it should be noted that the typical devices for IoT NTN voice are commercial handheld devices with GNSS capability, which do not need to rely on the existing TDMed GNSS reacquisition procedure. The GNSS reacquisition interruption issue only exists for NB-IoT devices. Regarding SIB31 reading, its timing and duration are controlled by the NW. NW is able to coordinate uplink scheduling to account for these periods and ensure that they do not directly result in multiple consecutive packet losses. Therefore, bullet4 should be removed: Some procedures needed for the operation of NB-IoT NTN, such as GNSS reacquisition and SIB19 reading, may result in interruption of reception / transmission of packets longer than 1 second. |

- Conclusion

TBD

Appendix: Summary of proposals

| R1-2506863 | vivo, Spreadtrum | Proposal 1: Provide the following answer to SA2 in the LS reply. - RAN1 evaluated the probability of 16 or 64 consecutive packet losses based on the typical NTN channel model and evaluation assumptions, and the example frame structure shown in Figure 5.2.2.3-1 and Figure 5.2.2.3-2 of excerpt_S4-251550_FS_ULBC_P-doc in LS R1-2505140 (S4-251584) from SA4. - RAN1 concluded that the probability of 16 or 64 consecutive packet losses is negligible for voice codec rates of 0.8kbps, 1.2kbps, and 2.4kbps, and voice bundling periods of 80ms, 160ms, and 320ms, under target BLERs of up to 10%. Such an event is unlikely to occur in practice. |
| --- | --- | --- |
| R1-2506906 | ZTE | Proposal 1: Reply SA2 that the probability that 16 or 64 consecutive packets loss or error is nearly zero and almost never not happen. |
| R1-2506957 | Xiaomi | Proposal 1: For a given voice data rate that could meet with the link budget of IoT-NTN GEO, the probability of M consecutive packets being in error satisfies the following formula: P=0.02^M |
| R1-2507085 | CATT | Proposal 1: For IMS voice over GEO, it can be considered that there is rare probability of 16 or 64 consecutive packets loss or erroneously decompressed. |
| R1-2507141 | OPPO | Proposal 1: Decompressing touches on higher layer operation and RAN1 cannot provides the probability or frequency that consecutive packets are erroneously decompressed. Proposal 2: The probability that 16 and 64 consecutive packets are lost is very low, and the impact of consecutively lost packets on using RoHC for support of IMS voice over NB-IoT NTN can be negligible. |
| R1-2507218. | Samsung | Observation 1: Given that one NPDSCH or NPUSCH carries one packet, the probability of consecutive packet failures (with a 2% Target BLER) is 0.02^N, making the scenario SA2 is inquiring about extremely rare. |
| R1-2507297 | Nokia | Proposal 1: RAN1 to reply to SA2 that the scenario of losing 16-64 consecutive packets is highly unlikely and that SA2 does not need to consider this scenario any further. |
| R1-2507320 | NEC | Proposal 1: Replay to SA2 that 1) packet loss or erroneous decompression can be due to the wireless channel degradation and/or packet collisions; 2) the wireless channel degradation modelling requires further RAN1 discussion on the evaluation scenario assumption; 3) the packet collision modelling requires further inputs from RAN4 and SA2 to provide inputs on the UE power model, overall required PHY data rates for different IP-based solutions, and multi-party voice service modelling; 4) RAN1 will provide further feedback on the packet loss or erroneous decompression probability analysis based on the inputs from RAN4 and SA4. |
| R1-2507919 | Huawei | Proposal 1: The probability P_N that consecutive N packets are lost or erroneously decompressed is P_N=(P_bler)^N, which is negligible when the number of N is 16 or 64. |
| R1-2507263 | Ericsson | Response to “Question 2 (To RAN1)”: While providing numerical data on the probability of error requires link-level simulations, in principle it is theoretically possible to transmit data with an arbitrarily low error rate using suitable error-correcting methods (presumably the ones already available in NB-IoT):  For SPS assuming a TBS of 328 bits with 15 kHz SCS NPUSCH Format 1 single-tone in an SNR range from -10 dB to -2 dB, it should be theoretically possible to transmit the following estimated data rates with an arbitrarily low error rate: o Data rates: ~ 1.0250 kbps, ~ 1.0933 kbps, and ~ 2.05 kbps from the lowest SNR (i.e., ~ -10 dB) onwards. o Data rate: ~ 8.2 kbps from ~ -3 dB onwards. o Data rate: ~ 16.4 kbps is not achievable in the SNR from -10 dB up to -2 dB.  For SPS assuming a TBS of 328 bits with 3.75 kHz SCS NPUSCH Format 1 single-tone in an SNR range from -10 dB to -2 dB, it should be theoretically possible to transmit the following estimated data rates with an arbitrarily low error rate: o Data rates: ~ 0.3037 kbps, ~ 0.3094 kbps, ~ 0.5857 kbps and ~ 0.6074 kbps from the lowest SNR (i.e., ~ -10 dB) onwards. o Data rate: ~ 4.1 kbps from ~ -6.5 dB onwards.  For SPS assuming a TBS of 328 bits with 15 kHz SCS NPDSCH in an SNR range from -10 dB to -2 dB, it should be theoretically possible to transmit all the estimated data rates (from fractions of kbps up tens of kbps) at any SNR within the SNR range with an arbitrarily low error rate. |
| R1-2507687 | Qualcomm | Proposal 1: Assuming a static user in good channel conditions (i.e., ~2% BLER), the probability of having consecutive packet losses exceeding 1 second is negligible. Proposal 2: Assuming a mobile user for which the line of sight condition changes during the call (from line-of-sight LOS to non-line-of-sight NLOS and vice-versa), there is a non-negligible probability of a large number of packets being consecutively lost. Assuming a rural scenario with 50 degree elevation angle: - For a slow moving receiver (1m/s): - 88% of the NLOS events will result in consecutive packet losses longer than 1.28s - There are 0.25 events / minute exceeding 1.28s, with an average duration of 15.3s - 65% of the NLOS events will result in consecutive packet losses longer than 5.12s - There are 0.17 events / minute exceeding 5.12s, with an average duration of 20.4s. - For a moderate speed receiver (10 m/s): - 40% of the NLOS events will result in consecutive packet losses longer than 1.28s - There is 1 event/minute exceeding 1.28s, with an average duration of 3.17s - 7% of the NLOS events will result in consecutive packet losses longer than 5.12s - There are 0.15 events/minute exceeding 5.12s, with an average duration of 7s - For a high speed receiver (30m/s): - 12% of the NLOS events will result in consecutive packet losses longer than 1.28s - There are 0.83 events/minute exceeding 1.28s, with an average duration of 2s - <0.1% of the NLOS events will result in consecutive packet losses longer than 5.12s - There are 0.004 events/minute exceeding 5.12s, with an average duration of 5.88s Proposal 3: Some procedures needed to operate NB-IoT NTN will result in consecutive packets being dropped: - For GNSS reacquisition, the UE will drop all packets every GNSS-ValidityDuration for a duration of GNSS-PositionFixDuration - GNSS-ValidityDuration is between 10s and infinity, GNSS-PositionFixDuration-r18 is between 1 and 31 seconds - For SIB31 reading, the UE will drop all packets every ul-SyncValidityDuration for a duration of t318 - ul-SyncValidityDuration is between 5 and 900 seconds, t318 is between 0 and 4 seconds. Proposal 4: Reply to the SA2 LS including proposals 1-3. |