3GPP TSG RAN WG1 #122bis	                                   					       												                                R1-2507738

Prague, Czech Republic, October 13th –17th, 2025

Agenda Item:	9.1

Source:	Moderator (AT&T)

Title:	Summary of UE features for AI/ML for NR Air Interface

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion [122bis-R19-UE_features] during RAN1 #122bis. According to the Chair’s Notes:

[122bis-R19-UE_features] Email discussion on Rel-19 UE features – Ralf (AT&T), Naoya (DOCOMO) - To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

The following was discussed during RAN1 #122bis within the scope of [122bis-R19-UE_features]. All proposals are based on the latest RAN1 UE features list for Rel. 19 in [1].

# Summary of Contributions Submitted to RAN1 #122bis

The following is the moderator’s summary of contributions submitted to RAN1 #122bis in this agenda item.

| 58. NR_AIML_air | 58-0-1 | CSI report framework for UE-side inference | 1. Number of APU pools N 2. Maximum number of APUs in each APU pool of UE-sided inference for CSI report(s) for simultaneously in a CC 3. Maximum number of APUs in each APU pool of UE-sided inference for CSI report(s) simultaneously across all CCs | FFS | yes | n/a | Maximum number of APUs for UE-sided inference is unknown to the network | Per UE | No | No | No | Component 1 candidate values: {1,2} Component 2candidate values: {1…8} Component 3 candidate values: {1…32} Note: Component 2 and 3 candidate values are signalled separately for each pool [A UE that does not support this FG reuses the CPU] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] | Adopt FG 2-35 as the prerequisite for FG 58-0-1 |
| ZTE Corporation/Sanechips [3] | Regarding FG 58-0-1, a UE can report its supported number of CPUs and/or APUs for UE-side inference. If a UE supporting this FG does not report APU support, it should report its CPU instead since UE is not allowed to report 0 for both CPU and APU according to the agreement in RAN1#121 meeting [2]. Agreement For UE-side model, for AI/ML based beam management for BM-Case 1 and BM-Case 2, for processing of a CSI report for inference, - For PU occupancy, for the number of AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied, - OAPU= 0 or X1/X2 is reported by UE in UE capability report for BM-Case 1 and BM-Case 2 respectively - OCPU=0 or Y1/Y2 is reported by UE in UE capability report for BM-Case 1 and BM-Case 2 respectively - Note: Detailed values of X1/X2 and Y1/Y2 can be further discussed in UE feature. - Note: Combination of OAPU= 0 and OCPU=0 is not allowed - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behavior of exceeding the CPU limit, neither of the PUs are occupied. Therefore, we propose the following. Proposal 2: Regarding FG 58-0-1, for UE does not support this FG for APU, reuse CPU. |
| Vivo [4] |  |
| Huawei/HiSilicon [5] | 58. NR_AIML_Air; 58-0-1; CSI report framework for UE-side inference; 1. Number of APU pools N 2. Maximum number of APUs in each APU pool of UE-sided inference for CSI report(s) for simultaneously in a CC 3. Maximum number of APUs in each APU pool of UE-sided inference for CSI report(s) simultaneously across all CCs; FFS; yes; n/a; Maximum number of APUs for UE-sided inference is unknown to the network; Per UE; No; No; No; Component 1 candidate values: {1,2} Component 2candidate values: {1…8} Component 3 candidate values: {1…32} Note: Component 2 and 3 candidate values are signalled separately for each pool [A UE that does not support this FG reuses the CPU] Note: “APU” corresponds to “CPU,x” in TS 38.214, x = 2, 3.; Optional with capability signalling. |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] | 58. NR_AIML_air; 58-0-1; CSI report framework for UE-side inference; 1. Number of APU pools N 2. Maximum number of APUs in each APU pool of UE-sided inference for CSI report(s) for simultaneously in a CC 3. Maximum number of APUs in each APU pool of UE-sided inference for CSI report(s) simultaneously across all CCs; 2-35; yes; n/a; Maximum number of APUs for UE-sided inference is unknown to the network; Per UE; No; No; No; Component 1 candidate values: {1,2} Component 2candidate values: {1…8} Component 3 candidate values: {1…32} Note: Component 2 and 3 candidate values are signalled separately for each pool A UE that does not support this FG reuses the CPU; Optional with capability signalling. |
| CATT/CICTCI [9] |  |
| OPPO [10] | A UE that does not support this FG may not be able carry out inference via legacy CPU. In addition, it seems inappropriate to define UE behavior in the component region. Hence, we suggest removing the last sentence in component part. 58. NR_AIML_air; 58-0-1; CSI report framework for UE-side inference; 1. Number of APU pools N 12. Maximum number of APUs in each APU pool for all types of UE-sided inference for CSI report(s) simultaneously in a CC 23. Maximum number of APUs in each APU pool for all types of UE-sided inference for CSI report(s) simultaneously across all CCs; FFS; yes; n/a; Maximum number of APUs for UE-sided inference is unknown to the network; FFS Per UE; FFS N/A; FFS N/A; FFS N/A; Component 1 candidate values: {1,2} Component 12 candidate values: FFS{1…8} Component 23 candidate values: FFS{1…32} Note: Component 2 and 3 candidate values are signalled separately for each pool; Optional with capability signalling. |
| Samsung [11] | For AI/ML CSI-ReportConfigs, processing criteria is largely mirror legacy non-AI CSI-ReportConfigs behavior, but with a focus on CPU and APU budget. If the number of AI/ML CSI-ReportConfigs needing APUs on a single OFDM symbol exceeds available APUs, the UE won't update the lowest priority CSI-ReportConfigs. AI/ML CSI-ReportConfigs inference involves running the corresponding model on the NPU. This includes loading model weights and input into DRAM, performing the inference and obtaining output. Unlike legacy systems, where "CPU" capacity might limit CSI calculations, AI/ML model inference on the NPU introduces a new UE capability limit: the maximum number of simultaneous model inferences, regardless of APU occupancy. To acknowledge this restriction at UE, one approach is to introduce a UE capability on the maximum number of active AI/ML CSI-ReportConfigs at any time. The active state can be defined using the APU occupancy window. That is, an AI/ML CSI-ReportConfigs is active during the time it occupies APU. If UE reports a value of Xactive for this capability, gNB shall not activate more than Xactive AI/ML CSI-ReportConfigs at any time. Proposal 3: Define a UE capability on the maximum number of active AI/ML CSI-ReportConfig at any time. - Add a component to FG 58-0-1 to define the maximum number of active AI/ML CSI-ReportConfigs across use cases |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] | Proposal 4: For FG58-0-1, the FG should be per band and per band combination, following a similar principle as the FGs: csi-ReportFramework Indicates whether the UE supports CSI report framework. This capability signalling comprises the following parameters: - maxNumberPeriodicCSI-PerBWP-ForCSI-Report indicates the maximum number of periodic CSI report setting per BWP for CSI report; - maxNumberPeriodicCSI-PerBWP-ForBeamReport indicates the maximum number of periodic CSI report setting per BWP for beam report. - maxNumberAperiodicCSI-PerBWP-ForCSI-Report indicates the maximum number of aperiodic CSI report setting per BWP for CSI report; - maxNumberAperiodicCSI-PerBWP-ForBeamReport indicates the maximum number of aperiodic CSI report setting per BWP for beam report; - maxNumberAperiodicCSI-triggeringStatePerCC indicates the maximum number of aperiodic CSI triggering states in CSI-AperiodicTriggerStateList per CC; - maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report indicates the maximum number of semi-persistent CSI report setting per BWP for CSI report; - maxNumberSemiPersistentCSI-PerBWP-ForBeamReport indicates the maximum number of semi-persistent CSI report setting per BWP for beam report; - simultaneousCSI-ReportsPerCC indicates the number of CSI report(s) for which the UE can measure and process reference signals simultaneously in a CC of the band for which this capability is provided. The CSI report comprises periodic, semi-persistent and aperiodic CSI and any latency classes and codebook types. The CSI report in simultaneousCSI-ReportsPerCC includes the beam report and CSI report. The UE is mandated to report csi-ReportFramework.; Band; Yes; N/A; N/A. simultaneousCSI-ReportsAllCC Indicates whether the UE supports CSI report framework and the number of CSI report(s) which the UE can simultaneously process across all CCs, and across MCG and SCG in case of NR-DC. The CSI report comprises periodic, semi-persistent and aperiodic CSI and any latency classes and codebook types. The CSI report in simultaneousCSI-ReportsAllCC includes the beam report and CSI report. This parameter may further limit simultaneousCSI-ReportsPerCC in MIMO-ParametersPerBand and Phy-ParametersFRX-Diff for each band in a given band combination.; BC; Yes; N/A; N/A. 58. NR_AIML_air; 58-0-1; CSI report framework for UE-side inference; 1. Number of APU pools N 12. Maximum number of APUs in each APU pool for all types of UE-sided inference for CSI report(s) for simultaneously in a CC 23. Maximum number of APUs in each APU pool for all types of UE-sided inference for CSI report(s) simultaneously across all CCs; FFS; yes; n/a; Maximum number of APUs for UE-sided inference is unknown to the network; FFS Per UE Per band and per BC; FFS N/A; FFS N/A; FFS N/A; Component 1 candidate values: {1,2} Component 12 candidate values: FFS{1…8} Component 23 candidate values: FFS{1…32} Note: Component 2 and 3 candidate values are signalled separately for each pool Note: [A UE that does not support this FG reuses the CPU]; Optional with capability signalling. |
| NTT DOCOMO, INC. [15] | At RAN1 #122 meeting, the issue about the terminology of APU or CPU,2/CPU,3 was discussed. Based on the discussions, there is no need to change the CPU,2/CPU,3 in the current specification. It is better to use the aligned terminology with the specification for the UE feature discussions. Therefore, we propose changing all APUs occurring in FG 58 to the term aligned with the specification. Proposal 1: Revise the following FGs about the usage of APU and change the APU and corresponding items to align with the term CPU,2/CPU,3. - FG 58-0-1, FG 58-1-2, FG 58-1-4, FG 58-3-1, FG 58-3-2. Besides the APU issue, there are two remaining issues for FG 58-0-1 after RAN1 #122 meeting. - The first issue is about the prerequisite FG. Since this FG focuses on the AI/ML dedicated resource pools for the CSI report, the FG on CSI report framework (FG 2-35) should be the prerequisite FG. - The second issue is about the note for UEs that do not have the dedicated processing resources. We believe that this note is necessary to ensure the UE correctly reports its capability. Therefore, it can be further clarified as shown in our proposals. Proposal 2: Update FG 58-0-1 as shown in the following table. 58. NR_AIML_air; 58-0-1; CSI report framework for UE-side inference; 1. Number of APU CPU,x pools N 2. Maximum number of APU CPU,x in each APU CPU,x pool of UE-sided inference for CSI report(s) for simultaneously in a CC 3. Maximum number of APU CPU,x in each APU CPU,x pool of UE-sided inference for CSI report(s) simultaneously across all CCs; FFS 2-35; yes; n/a; Maximum number of APUs for UE-sided inference is unknown to the network; Per UE; N/A; N/A; N/A; Component 1 candidate values: {1,2} Component 12 candidate values: {1…8} Component 23 candidate values: {1…32} Note: Component 2 and 3 candidate values are signalled separately for each pool [A UE that does not support this FG reuses the CPU should not report non-zero occupied CPU,2 or CPU,3 values in any dependency FG.]; Optional with capability signalling. |

## Specification support for beam management

| 58. NR_AIML_air | 58-1-1 | Increased number of reported RSs for beam management | 1. Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4 | FFS | yes | n/a | Increased number of reported beams for beam management is not supported | Per UE | No | No | No | Component 3 candidate values: {6,8} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] | Adopt FGs 2-21, 2-22 or 2-23, 2-23a as the prerequisites for FG 58-1-1 |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] | 58. NR_AIML_air; 58-1-1; Increased number of reported RSs for beam management; 1. Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4; FFS 2-29; yes; n/a; Increased number of reported beams for beam management is not supported; Per UE; No; No; No; Component 3 candidate values: {6,8}; Optional with capability signalling. |
| Apple [13] | Beam prediction (BM Case 1 and BM case 2) may or may not provide values depending carrier frequency and deployment scenarios. Whether to implement beam prediction based on AI on one band or another band should be a choice driven by market needs, engineering/test efforts. There is no justification on mandating its universal support even if only a single band may be found to benefit from one of those features. We have Proposal 1-1: Rectify the type designation for 58-1-1/58-1-2/58-1-3/58-1-4/58-1-5/58-1-6/58-1-7 to "per Band". |
| Qualcomm Incorporated [14] | Proposal 5: FG58-1-1, FG58-1-2, FG58-1-3, FG58-1-4, FG58-1-5, FG58-1-6, and FG58-1-7, the FGs should be per band, following a similar principle as the FGs for legacy beam management and reporting. 58. NR_AIML_air; 58-1-1; Increased number of reported RSs for beam management; 1. Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4; FFS; yes; n/a; Increased number of reported beams for beam management is not supported; FFS Per UE Per band; FFS N/A; FFS N/A; FFS N/A; Component 3 candidate values: {6,8}; Optional with capability signalling. |
| NTT DOCOMO, INC. [15] | The only remaining issue for FG 58-1-1 is the prerequisite FG. Since this FG is for the NW-sided model and UEs only need to report their beam measurements based on the enhanced beam reporting, we suggest FG 2-24 as the prerequisite FG. Proposal 3: Update FG 58-1-1 as shown in the following table. 58. NR_AIML_air; 58-1-1; Increased number of reported RSs for beam management; 1. Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4; FFS 2-24; yes; n/a; Increased number of reported beams for beam management is not supported; Per UE; N/A; N/A; N/A; Component 3 candidate values: {6,8}; Optional with capability signalling. |

| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 for inference | 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 8. Supported CSI-RS resource types 9. Supported inference report types 11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both}] 12. Supported maximum number of predicted beams in each reporting instanceFFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection 13. Supported number of occupied CPU 14. Supported number of occupied APU 15. Supported value of d for the relaxation of Z3 timeline 16. Supported value of d’ for the relaxation of Z’3 timeline 17. Index of the occupied APU pool | FFS | yes | n/a | UE-sided beam prediction for BM Case 1 for inference is not supported | Per UE | No | No | No | Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} FFS: candidate values for components | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] | Adopt FG 2-35 as the prerequisite for FG 58-1-2 |
| ZTE Corporation/Sanechips [3] | Regarding FG 58-1-2, the candidate values for component 3, component 7a and 7b still need to be specified. For component 3, the maximum number of inference report(s) configured for BM-Case1 per BWP, a legacy counterpart in FG 2-35 is observed as follows: Features; Index; Feature group; Components; Prerequisite feature groups; Note; Mandatory/Optional. 2. MIMO; 2-35; CSI report framework; 1) Maximum number of periodic CSI report setting per BWP for CSI report 2) Maximum number of periodic CSI report setting per BWP for beam report 3) Maximum number of aperiodic CSI report setting per BWP for CSI report 4) Maximum number of aperiodic CSI report setting per BWP for beam report 5) Maximum number of configured aperiodic CSI triggering states in CSI-AperiodicTriggerStateList per CC, 6) Maximum number of semi-persistent CSI report setting per BWP for CSI report 7) Maximum number of semi-persistent CSI report setting per BWP for beam report 8) UE can process Y CSI report(s) simultaneously in a CC. CSI reports can be P/SP/A CSI and any latency class and codebook type. 9) UE can process X CSI report(s) simultaneously across all CCs. CSI reports can be P/SP/A CSI and any latency class and codebook type.; 2-32; Other MIMO capabilities than component 5 may further restrict (reduce) the number of simultaneously CSI report that UE is required to update The CSI report in component 4 and 5 includes the beam report and CSI report Each component is independent CSI report setting are counted in the CC indicated by the parameter carrier in CSI-ResourceConfig.; Mandatory with capability signaling Component-1 candidate values: {1, 2, 3, 4} Component-1a candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-2a candidate values {1, 2, 3, 4} Component-2b candidate values {3, 7, 15, 31, 63, 128} Component-3 candidate values: {0, 1, 2, 3, 4} Component-3a candidate values: {0, 1, 2, 3, 4} Component-4 candidate values: {from 1 to 8} Component-5: candidate values: {from 5 to 32}. According to the legacy FG for beam reports, it is straightforward to reuse the legacy candidate values for maximum number of inference report(s) configured for BM-Case1 per BWP. In such cases, we propose the following: Proposal 3: For FG 58-1-2, for component 3, reuse legacy candidate values for inference report(s), - Support candidate values: {1, 2, 3, 4} for maximum number of periodic inference report(s) configured for BM-Case1 per BWP; - Support candidate values: {from 1 to 8} for maximum number of aperiodic inference report(s) configured for BM-Case1 per BWP. For FG 58-1-2, regarding the component 7a and 7b, the supported maximum number of resources for Set B/Set A also need to be specified. Similarly, we obtained the legacy counterpart for component 7a and 7b in the following list: Features; Index; Feature group; Components; Prerequisite feature groups; Note; Mandatory/Optional. 2. MIMO; 2-24; SSB/CSI-RS for beam measurement; 1) The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot shall not exceed MB_1 2) The max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP shall not exceed MC_1 3) The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_2 4) Supported density of CSI-RS 5) The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-RSRP shall not exceed MD_1; 2-21, 2-22 or 2-23, 2-23a; Mandatory with capability signalling Component-1, candidate value set for MB_1 is {0, 8, 16, 32, 64} On FR2, UE is mandated to signal MB_1 >=8 On FR1, MB_1 >=8 is supported mandatory with capability signalling. Component-2, candidate value set for MC_1 is {0, 4, 8, 16, 32, 64} For FR1, UE is mandated to report at least 8. Component-3, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-4: candidate value set: {"not supported", "1 only", "3 only", "both 1 and 3"} On FR2, UE is mandated to signal either "3 only" or "both 1 and 3" On FR1, either "3 only" or "both 1 and 3" is mandatory with UE capability signalling. Component-5, candidate value set for MD_2 is {0, 1, 4, 8, 16, 32, 64} For both FR1 and FR2, UE is mandated to report at least 4. Proposal 4: For FG 58-1-2, for component 7a and 7b, support the following candidate values, - Support candidate values: { 8, 16, 32, 64} for maximum number of of resources for Set B/Set A; |
| Vivo [4] | For FG 58-1-2, it is acceptable to retain the purpose as "for inference", since the UE inference feature is designated exclusively for this function and cannot be repurposed for other purposes such as data collection or monitoring. - For components of FG 58-1-2, FG 58-1-3, FG 58-1-4 and FG 58-1-5, support to retain the purpose as "for inference". Components 7a and 7b of FG 58-1-2 and FG 58-1-4 offer a superior alternative to component 7. Within the UE capability reporting framework, it is mandatory that the UE declares its maximum capability. Critically, the adoption of component 7 would exponentially increase the combinatorial complexity of Set B and Set A dimensions, resulting in prohibitive signaling overhead. By leveraging components 7a and 7b, the UE can actively validate during the applicable report procedure whether the Set B size or Set A size within inference configuration parameters conforms to the deployed AI models. Concurrently, the minimum capability threshold of Set B must be reported to the network to prevent invalid configuration attempts and optimize resource allocation. - For components of FG 58-1-2 and FG 58-1-4 in addition to the agreed components: - Support 7a: supported maximum number of resources for Set B + 7b: supported maximum number of resources for Set A - Support 7c: supported minimum number of resources for Set B 58. NR_AIML_air; 58-1-2; UE-side beam prediction for BM Case1 [for inference] for inference; 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 [for inference] for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs [4. Maximum number of inference report(s) activated for BM-Case1 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case1 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case1 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case1 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 7c: Supported minimum number of resources for Set B 8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS 9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report 11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both} 12. Supported maximum number of predicted beams in each reporting instanceFFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection 13. Supported number of occupied CPU 14. Supported number of occupied APU 15. Supported value of d for the relaxation of Z3 timeline 16. Supported value of d’ for the relaxation of Z’3 timeline 17. Index of the occupied APU pool; FFS; yes; UE-sided beam prediction for BM Case 1[for inference] for inference is not supported. |
| Huawei/HiSilicon [5] | - Component 3/3a: Candidate values refer to the max number of P/AP CSI reports per BWP in legacy UE feature. - Component 7a/7b: Candidate values refer to maximum number of resources in Set B/Set A for 58-1-7, except that 64 for Set B is not supported. Thus, Set B supports {4, 8, 16, 32}, and Set A supports {8, 16, 32, 64}. - Component 12: Delete the FFS part. - Component 13/14: For legacy CSI report for BM, the OCPU is 1. Considering that for relatively small model size of beam management, {0, 1, 2} can considered as candidate values for the number of occupied CPU and APU. - Component 15/16: Considering that for relatively small model size of beam management, the additional inference delay di/di’ (where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS) for Z3/Z3’, respectively, could be up to 1ms level for all of SCS values. Regarding other candidate values smaller than 1ms delay, we can refer to the candidate values of legacy Z3’. - Component 25: Candidate values include Index 1 to represent the first APU pool and Index 2 to represent the second APU pool. - In addition, to align with the notation between ‘APU’ in UE feature and ‘CPU,x’ in TS 38.214 (shown in below), a note is added. The UE indicates the number of supported simultaneous CSI calculations NCPU with parameter simultaneousCSI-ReportsPerCC or simultaneousCSI-SubReportsPerCC-r18 in a component carrier, and simultaneousCSI-ReportsAllCC or simultaneousCSI-SubReportsAllCC-r18 across all component carriers. If UE is configured with at least one CSI report setting with sub-configuration in a component carrier, UE shall use parameter simultaneousCSI-SubReportsPerCC-r18 in the component carrier; otherwise, UE shall use simultaneousCSI-ReportsPerCC in the component carrier. If UE is configured with at least one CSI reporting setting with sub-configuration in any component carrier, UE shall use simultaneousCSI-SubReportsAllCC-r18; otherwise, UE shall use simultaneousCSI-ReportsAllCC. If a UE supports NCPU simultaneous CSI calculations it is said to have NCPU CSI processing units for processing CSI reports. If L CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU-L unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU-L CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU(n), the UE is not required to update the N-M requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤M≤N is the largest value such that n=0M-1OCPU(n)≤NCPU-L holds. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', or CSI reports configured with the higher layer parameter [RRC_name-r19], OCPU= OCPU,1 is considered. For CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19', and CSI reports configured with the higher layer parameter [RRC_name-r19], the UE may indicate a second value for the number of supported simultaneous CSI calculations NCPU,2 with parameter SecondValuesSimultaneousCSI-ReportsPerCC in a component carrier, and SecondValuesSimultaneousCSI-ReportsAllCC across all component carriers, and if applicable, a third value for the number of supported simultaneous CSI calculations NCPU,3 with parameter ThirdValuesSimultaneousCSI-ReportsPerCC in a component carrier, and ThirdValuesSimultaneousCSI-ReportsAllCC across all component carriers, in addition to NCPU. If a UE supports NCPU,x simultaneous CSI calculations it is said to have NCPU,x CSI processing units for processing CSI reports. If Lx CPUs are occupied for calculation of CSI reports in a given OFDM symbol, the UE has NCPU,x-Lx unoccupied CPUs. If N CSI reports start occupying their respective CPUs on the same OFDM symbol on which NCPU,x-Lx CPUs are unoccupied, where each CSI report n=0, …, N-1 corresponds to OCPU,x(n), the UE is not required to update the N-Mx requested CSI reports with lowest priority (according to Clause 5.2.5), where 0≤Mx≤N is the largest value such that n=0Mx-1OCPU,x(n)≤NCPU,x-Lx holds. If only the second value is indicated, x = 2. If both of the second value and the third value are indicated, x = 2, 3 where the CSI reports with reportQuantity set to 'p-cri-r19', 'p-cri-RSRP-r19', 'p-ssb-index-r19', or 'p-ssb-index-RSRP-r19' corresponds to either x = 2 or x = 3 subject to UE capability and the CSI reports configured with the higher layer parameter [RRC_name-r19] corresponds to either x = 2 or x = 3 subject to UE capability.. 58. NR_AIML_air; 58-1-2; UE-side beam prediction for BM Case1 for inference; 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 8. Supported CSI-RS resource types 9. Supported inference report types 11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both}] 12. Supported maximum number of predicted beams in each reporting instance FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection 13. Supported number of occupied CPU 14. Supported number of occupied APU 15. Supported value of di for the relaxation of Z3 timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 16. Supported value of di’ for the relaxation of Z’3 timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 17. Index of the occupied APU pool; FFS; yes; n/a; UE-sided beam prediction for BM Case 1 for inference is not supported; Component 3 candidate values: {1, 2, 3, 4} Component 3a candidate values: {1, 2, 3, 4, 8} Component 7a candidate values: {4, 8, 16, 32} Component 7b candidate values: {8, 16, 32, 64} Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} FFS: candidate values for components Component 13 candidate values: {0, 1, 2} Component 14 candidate values: {0, 1, 2} Note: The values of Component 13 and 14 are not allowed to be 0 simultaneously. Component 15 candidate values: d1 is {2, 4, 8, 14} d2 is {4, 8, 14, 28} d3 is {8,14, 28, 56} d4 is {14, 28, 56, 112} d5 is {56, 112, 224, 448} d6 is {112, 224, 448, 896} Component 17 candidate values: {1, 2} representing the first APU pool (i.e., CPU,2) and the second APU pool (i.e., CPU,3), respectively. Note: “CPU” corresponds to “CPU,1” in TS 38.214, and “APU” corresponds to “CPU,x” in TS 38.214, x = 2, 3.; Optional with capability signalling. |
| Xiaomi [6] | 58. NR_AIML_air; 58-1-2; UE-side beam prediction for BM Case1 for inference; 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs [4. Maximum number of inference report(s) activated for BM-Case1 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case1 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case1 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case1 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7. Supported combinations of the number of resources for Set B and the number of resources for Set A 8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS 9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report 11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both} 12. Supported maximum number of predicted beams in each reporting instance FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection 13. Supported number of occupied CPU 14. Supported number of occupied APU 15. Supported value of d for the relaxation of Z3 timeline 16. Supported value of d’ for the relaxation of Z’3 timeline 17. Index of the occupied APU pool; FFS; yes; n/a; UE-sided beam prediction for BM Case 1 for inference is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| CMCC [7] | It is fine to remain purpose “for inference” since UE feature for inference seems can not apply for other purposes like data collection or monitoring. Based on RAN1#118b meeting agreement below, multiple CSI reports for inference for UE-side model can be configured/activated/triggered. The maximum number of configured/activated/triggered CSI reports across BM-Case 1 and BM-Case 2 should be reported in UE capability. Agreement@118b For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.. Components 7a+7b of FG 58-1-2 are preferred than component 7. In UE capability reporting, UE should report maximum capability. On the other hand, if we go with component 7, there will be too many combinations of set B size and set A size, the overhead is huge. With components 7a+7b, UE can check whether set B size or set A size in inference configuration or inference related parameters can match that of AI models during applicable function reporting procedure. Proposal 1: Take the following modification (in red) for the feature of Rel-19 AI based beam management: 58. NR_AIML_air; 58-1-2; UE-side beam prediction for BM Case1 [for inference]; 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs [4. Maximum number of inference report(s) activated for BM-Case1 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case1 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case1 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case1 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] 8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS 9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report 11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both} 12. Supported maximum number of predicted beams in each reporting instanceFFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection 13. Supported number of occupied CPU 14. Supported number of occupied APU 15. Supported value of d for the relaxation of Z3 timeline 16. Supported value of d’ for the relaxation of Z’3 timeline 17. Index of the occupied APU pool; FFS; yes; n/a; UE-sided beam prediction for BM Case 1 [for inference] is not supported; FFS; FFS; FFS; FFS; FFS: CPU/AIMLPU related information; Optional with capability signalling. |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] | First, for Component 11, three cases could be listed as candidate values, otherwise it means UE always to have to support all of them if this FG is signaled. Second, for Component 12, this is no need to further study whether to merge the FG of inference with other FGs of monitoring or data collection. 58. NR_AIML_air; 58-1-2; UE-side beam prediction for BM Case1 [for inference]; 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per CC 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs [4. Maximum number of inference report(s) activated for BM-Case1 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case1 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case1 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case1 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] 8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS 9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report 11. Supported BM-Case 1 sub-usecase(s) 12. Supported maximum number of predicted beams in each reporting instance 13. Supported number of occupied CPU 14. Supported number of occupied APU 15. Supported value of d for the relaxation of Z3 timeline 16. Supported value of d’ for the relaxation of Z’3 timeline 17. Index of the occupied APU pool; FFS; yes; n/a; UE-sided beam prediction for BM Case 1 [for inference] is not supported; FFS Per UE; FFS No; FFS No; FFS No; Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} Component 11 candidate values: {setB-subset-of-setA, setB-different-from-setA, both} FFS: candidate values for components; Optional with capability signalling. |
| Samsung [11] | For FG 58-1-2, the capability reporting for inference report should be updated from “per band” to “per UE” since inference report is only feasible in bands in FR2. Same update should be applied for FG 58-1-3, FG 58-1-4 and FG 58-1-5. Proposal 1. Update FG 58-1-2, FG 58-1-3, FG 58-1-4 and FG 58-1-5 from “per UE” to “per band”. |
| LG Electronics [12] | 58. NR_AIML_air; 58-1-2; UE-side beam prediction for BM Case1 for inference; 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 8. Supported CSI-RS resource types 9. Supported inference report types 11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both}] 12. Supported maximum number of predicted beams in each reporting instanceFFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection 13. Supported number of occupied CPU 14. Supported number of occupied APU 15. Supported value of d for the relaxation of Z3 timeline 16. Supported value of d’ for the relaxation of Z’3 timeline 17. Index of the occupied APU pool; FFS; yes; n/a; UE-sided beam prediction for BM Case 1 for inference is not supported; Per UE; No; No; No; Component 7a candidate values: {4, 8, 16, 32, 64} Component 7b candidate values: {4, 8, 16, 32, 64} Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} Component 11 candidate values: {setB-subset-of-setA, setB-different-from-setA, both} Component 12 candidate values: {1, 2, 3, 4} Component 13 candidate values: {0, 1, 2} Component 14 candidate values: {0, 1, 2} Component 17 candidate values: {1, 2} FFS: candidate values for components; Optional with capability signalling. |
| Apple [13] | Beam prediction (BM Case 1 and BM case 2) may or may not provide values depending carrier frequency and deployment scenarios. Whether to implement beam prediction based on AI on one band or another band should be a choice driven by market needs, engineering/test efforts. There is no justification on mandating its universal support even if only a single band may be found to benefit from one of those features. We have Proposal 1-1: Rectify the type designation for 58-1-1/58-1-2/58-1-3/58-1-4/58-1-5/58-1-6/58-1-7 to "per Band". |
| Qualcomm Incorporated [14] | Proposal 5: FG58-1-1, FG58-1-2, FG58-1-3, FG58-1-4, FG58-1-5, FG58-1-6, and FG58-1-7, the FGs should be per band, following a similar principle as the FGs for legacy beam management and reporting. 58. NR_AIML_air; 58-1-2; UE-side beam prediction for BM Case1 [for inference]; 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs [4. Maximum number of inference report(s) activated for BM-Case1 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case1 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case1 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case1 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] 8. Supported CSI-RS resource types for Set B: Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS 9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report 11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both} 12. Supported maximum number of predicted beams in each reporting instanceFFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection 13. Supported number of occupied CPU 14. Supported number of occupied APU 15. Supported value of d for the relaxation of Z3 timeline 16. Supported value of d’ for the relaxation of Z’3 timeline 17. Index of the occupied APU pool 18. Supported CSI-RS resource types for Set A; FFS; yes; n/a; UE-sided beam prediction for BM Case 1 [for inference] is not supported; FFS Per UE Per band; FFS; FFS; FFS; Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} FFS: candidate values for components Component 7a candidate values: {4, 8, 16} Component 7b candidate values: {16, 32, 64} Component 12 candidate values: {1, 2, 4} Component 13 candidate values: INTEGER (1..8) Component 14 candidate values: INTEGER (0..8) Component 15 and Component 16 candidate values: for μ={0,1,2,3,4,5,6}, the corresponding values for d and d' are: {56,112,224,448,896,1792,3584}, respectively. Component 18 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS}; Optional with capability signalling. |
| NTT DOCOMO, INC. [15] | At RAN1 #122 meeting, the issue about the terminology of APU or CPU,2/CPU,3 was discussed. Based on the discussions, there is no need to change the CPU,2/CPU,3 in the current specification. It is better to use the aligned terminology with the specification for the UE feature discussions. Therefore, we propose changing all APUs occurring in FG 58 to the term aligned with the specification. Proposal 1: Revise the following FGs about the usage of APU and change the APU and corresponding items to align with the term CPU,2/CPU,3. - FG 58-0-1, FG 58-1-2, FG 58-1-4, FG 58-3-1, FG 58-3-2. Besides the APU issues, the prerequisite FGs for FG 58-1-2 and 58-1-4 have not been decided yet. Similar to FG 58-1-1, the basic beam management framework (FG 2-24) should be one of the prerequisite FGs. One note is also added to clarify the UE behaviors further if FG 58-0-1 is not signalled based on Proposal 2. For FG 58-1-2, we also propose the candidate values for components 7a, 7b, 12, 13, 14, and 17. For component 14, we suggest considering a limited number of candidate values to reduce the NW operation burden in managing UE resources. Besides the value 0, we propose three candidates to reflect the UE choice on small, medium, and large-scale models. The same proposals are also applied to FG 58-1-4. 58. NR_AIML_air; 58-1-2; UE-side beam prediction for BM Case1 for inference]; 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 8. Supported CSI-RS resource types 9. Supported inference report types 11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both} 12. Supported maximum number of predicted beams in each reporting instanceFFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection 13. Supported number of occupied CPU 14. Supported number of occupied APU CPU,2/CPU,3 15. Supported value of d for the relaxation of Z3 timeline 16. Supported value of d’ for the relaxation of Z’3 timeline 17. Occupied resource pool between CPU,2 and CPU,3APU pool; FFS 2-24; yes; n/a; UE-sided beam prediction for BM Case 1 [for inference] is not supported; Per UE; No; No; No; Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} FFS: candidate values for components Component 7a candidate values: {4, 8, 16, 32} Component 7b candidate values: {16, 32, 64, 128} Component 12 candidate values: {4, 6, 8} Component 13 candidate values: {0, 1, 8} Component 14 candidate values: {0, 2, 4, 8} Component 17 candidate values: {CPU,2 or CPU,3} Note: UE should not report non-zero value for Component 14 if FG 58-0-1 is not signalled.; Optional with capability signalling. |

| 58. NR_AIML_air | 58-1-3 | UE-side beam prediction for BM Case1 with predicted RSRP for inference | 1. Support of beam prediction, reporting of predicted beam index and predicted RSRP, for BM-Case1 for inference 2. Supported maximum number of predicted beams with RSRP in each reporting instance | 58-1-2 | yes | n/a | UE-side beam prediction for BM Case 1 with predicted RSRP for inference is not supported | Per UE | No | No | No | Component 2 candidate values: {1, 2, 3, 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] | For FG 58-1-2, it is acceptable to retain the purpose as "for inference", since the UE inference feature is designated exclusively for this function and cannot be repurposed for other purposes such as data collection or monitoring. - For components of FG 58-1-2, FG 58-1-3, FG 58-1-4 and FG 58-1-5, support to retain the purpose as "for inference". 58. NR_AIML_air; 58-1-3; UE-side beam prediction for BM Case1 with predicted RSRP [for inference] for inference; 1. Support of beam prediction, reporting of predicted beam index and predicted RSRP, for BM-Case1 [for inference] for inference 2. Supported maximum number of predicted beams with RSRP in each reporting instance; 58-1-2; yes; UE-side beam prediction for BM Case 1 with predicted RSRP [for inference] for inference is not supported; Component 2 candidate values: {1, 2, 3, 4}. |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] | It is fine to remain purpose “for inference” since UE feature for inference seems can not apply for other purposes like data collection or monitoring. Based on RAN1#118b meeting agreement below, multiple CSI reports for inference for UE-side model can be configured/activated/triggered. The maximum number of configured/activated/triggered CSI reports across BM-Case 1 and BM-Case 2 should be reported in UE capability. Agreement@118b For beam management, multiple CSI reports for inference for UE-side model can be configured/activated/triggered, which is up to UE capability.. Components 7a+7b of FG 58-1-2 are preferred than component 7. In UE capability reporting, UE should report maximum capability. On the other hand, if we go with component 7, there will be too many combinations of set B size and set A size, the overhead is huge. With components 7a+7b, UE can check whether set B size or set A size in inference configuration or inference related parameters can match that of AI models during applicable function reporting procedure. Proposal 1: Take the following modification (in red) for the feature of Rel-19 AI based beam management: 58. NR_AIML_air; 58-1-3; UE-side beam prediction for BM Case1 with predicted RSRP [for inference]; 1. Support of beam prediction, reporting of predicted beam index and predicted RSRP, for BM-Case1 [for inference] 2. Supported maximum number of predicted beams with RSRP in each reporting instance; 58-1-2; yes; n/a; UE-side beam prediction for BM Case 1 with predicted RSRP [for inference] is not supported; FFS; FFS; FFS; FFS; Component 2 candidate values: {1, 2, 3, 4} FFS: CPU/AIMLPU related information; Optional with capability signalling. |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] | For FG 58-1-2, the capability reporting for inference report should be updated from “per band” to “per UE” since inference report is only feasible in bands in FR2. Same update should be applied for FG 58-1-3, FG 58-1-4 and FG 58-1-5. Proposal 1. Update FG 58-1-2, FG 58-1-3, FG 58-1-4 and FG 58-1-5 from “per UE” to “per band”. |
| LG Electronics [12] |  |
| Apple [13] | Beam prediction (BM Case 1 and BM case 2) may or may not provide values depending carrier frequency and deployment scenarios. Whether to implement beam prediction based on AI on one band or another band should be a choice driven by market needs, engineering/test efforts. There is no justification on mandating its universal support even if only a single band may be found to benefit from one of those features. We have Proposal 1-1: Rectify the type designation for 58-1-1/58-1-2/58-1-3/58-1-4/58-1-5/58-1-6/58-1-7 to "per Band". |
| Qualcomm Incorporated [14] | Proposal 5: FG58-1-1, FG58-1-2, FG58-1-3, FG58-1-4, FG58-1-5, FG58-1-6, and FG58-1-7, the FGs should be per band, following a similar principle as the FGs for legacy beam management and reporting. 58. NR_AIML_air; 58-1-3; UE-side beam prediction for BM Case1 with predicted RSRP [for inference]; 1. Support of beam prediction, reporting of predicted beam index and predicted RSRP, for BM-Case1 [for inference] 2. Supported maximum number of predicted beams with RSRP in each reporting instance; 58-1-2; yes; n/a; UE-side beam prediction for BM Case 1 with predicted RSRP [for inference] is not supported; FFS Per UE Per band; FFS No; FFS No; FFS No; Component 2 candidate values: {1, 2, 3, 4}; Optional with capability signalling. |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 for inference | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 8. Supported CSI-RS resource types 9. Supported inference report types 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances 13. Supported maximum total number of reported predicted beams for predicted time instances in one report 14. Supported combinations of supported value(s) of valid time duration for each predicted time instance and number of predicted beams for each value of valid time duration 21. supported number of occupied CPU 22. supported number of occupied APU 23. supported value of d for the relaxation of Z3 timeline 24. supported value of d’ for the relaxation of Z’3 timeline 25. Index of the occupied APU pool | FFS | yes | n/a | UE-side beam prediction for BM-Case2 for inference is not supported | Per UE | No | No | No | Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report}FFS: candidate values for components | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] | Adopt FG 2-35 as the prerequisite for FG 58-1-4 |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] | For FG 58-1-2, it is acceptable to retain the purpose as "for inference", since the UE inference feature is designated exclusively for this function and cannot be repurposed for other purposes such as data collection or monitoring. - For components of FG 58-1-2, FG 58-1-3, FG 58-1-4 and FG 58-1-5, support to retain the purpose as "for inference". Components 7a and 7b of FG 58-1-2 and FG 58-1-4 offer a superior alternative to component 7. Within the UE capability reporting framework, it is mandatory that the UE declares its maximum capability. Critically, the adoption of component 7 would exponentially increase the combinatorial complexity of Set B and Set A dimensions, resulting in prohibitive signaling overhead. By leveraging components 7a and 7b, the UE can actively validate during the applicable report procedure whether the Set B size or Set A size within inference configuration parameters conforms to the deployed AI models. Concurrently, the minimum capability threshold of Set B must be reported to the network to prevent invalid configuration attempts and optimize resource allocation. - For components of FG 58-1-2 and FG 58-1-4 in addition to the agreed components: - Support 7a: supported maximum number of resources for Set B + 7b: supported maximum number of resources for Set A - Support 7c: supported minimum number of resources for Set B 58. NR_AIML_air; 58-1-4; UE-side beam prediction for BM Case2 [for inference] for inference; 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 [for inference] for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs [4. Maximum number of inference report(s) activated for BM-Case2 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case2 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case2 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case2 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 7c: Supported minimum number of resources for Set B 8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS 9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances 13. Supported maximum total number of reported predicted beams for predicted time instances in one report 14. Supported combinations of supported value(s) of valid time duration for each predicted time instance and number of predicted beams for each value of valid time duration 21. supported number of occupied CPU 22. supported number of occupied APU 23. supported value of d for the relaxation of Z3 timeline 24. supported value of d’ for the relaxation of Z’3 timeline 17. Index of the occupied APU pool; FFS; yes; UE-side beam prediction for BM-Case2 [for inference] for inference is not supported; FFS: candidate values for components. |
| Huawei/HiSilicon [5] | - Component 11/12: Candidate values refer to the RRC parameters of nrofreportedpredictedrs-r19 and nroftimeinstance-r19. - Component 13/14: Candidate values refer to the RRC parameters of nrofreportedpredictedrs-r19, nroftimeinstance-r19, and TimeGap-r19. The intention is to support all combinations. - Component 3/3a/7a/7B/21/22/23/24/25: Same values as 58-1-2. - Similar to 58-1-2, a note is added for CPU and APU to align with the notations in TS 38.214. 58. NR_AIML_air; 58-1-4; UE-side beam prediction for BM Case2 for inference; 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 8. Supported CSI-RS resource types 9. Supported inference report types 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances 13. Supported maximum total number of reported predicted beams for predicted time instances in one report 14. Supported combinations of supported value(s) of valid time duration for each predicted time instance and number of predicted beams for each value of valid time duration 21. supported number of occupied CPU 22. supported number of occupied APU 23. Supported value of di for the relaxation of Z3 timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 24. Supported value of d’ for the relaxation of Z’3 timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 25. Index of the occupied APU pool; FFS; yes; n/a; UE-sided beam prediction for BM Case2 for inference is not supported; Component 3 candidate values: {1, 2, 3, 4} Component 3a candidate values: {1, 2, 3, 4, 8} Component 7a candidate values: {4, 8, 16, 32} Component 7b candidate values: {8, 16, 32, 64} Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS } Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} Component 11 candidate values: {1, 2, 3, 4} Component 12 candidate values: {1, 2, 4, 8} Component 13 candidate values: {1, 2, 4, 6, 12, 16, 24, 32} Component 14 candidate values: any combination between: Number of predicted beams: {1,2,4,8} Time gap: {10ms, 20ms, 40ms, 80ms, 160ms} Component 21 candidate values: {0, 1, 2} Component 22 candidate values: {0, 1, 2} Note: The values of Component 21 and 22 are not allowed to be 0 simultaneously. Component 23 candidate values: d1 is {2, 4, 8, 14} d2 is {4, 8, 14, 28} d3 is {8,14, 28, 56} d4 is {14, 28, 56, 112} d5 is {56, 112, 224, 448} d6 is {112, 224, 448, 896} FFS: candidate values for components Component 25 candidate values: {1, 2} representing the first APU pool (i.e., CPU,2) and the second APU pool (i.e., CPU,3), respectively. Note: “CPU” corresponds to “CPU,1” in TS 38.214, and “APU” corresponds to “CPU,x” in TS 38.214, x = 2, 3.; Optional with capability signalling. |
| Xiaomi [6] | 58. NR_AIML_air; 58-1-4; UE-side beam prediction for BM Case2 for inference; 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs [4. Maximum number of inference report(s) activated for BM-Case2 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case2 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case2 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case2 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7. Supported combinations of the number of resources for Set B and the number of resources for Set A 8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS 9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances 13. Supported maximum total number of reported predicted beams for predicted time instances in one report 19. Supported combinations of time gaps between predicted time instances and value of set B periodicity. 20. Supported BM-Case 2 sub usecase(s): e.g., setB-equals-to-setA, setB-subset-of-setA, setB-different-from-setA, or merged version(s) 21. supported number of occupied CPU 22. supported number of occupied APU 23. supported value of d for the relaxation of Z3 timeline 24. supported value of d’ for the relaxation of Z’3 timeline 17. Index of the occupied APU pool; FFS; yes; n/a; UE-side beam prediction for BM-Case2 for inference is not supported; FFS; FFS; FFS; FFS; FFS: candidate values for components; Optional with capability signalling. |
| CMCC [7] | FG 58-1-4 and 58-1-5 share most components with FG 58-1-2 and 58-1-3. Components 20 is supported for BM-Case 2. Similar as supported BM-Case 1 sub-usecases component for BM-Case 1, BM-Case 2 sub-usecases component for BM-Case 2 is supported to report in UE capability. So that when gNB transmits inference configuration, gNB can know whether UE supports pure time domain beam prediction or spatial and time domain beam prediction. Proposal 2: Take the following modification (in red) for the feature of Rel-19 AI based beam management: 58. NR_AIML_air; 58-1-4; UE-side beam prediction for BM Case2 [for inference]; 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs [4. Maximum number of inference report(s) activated for BM-Case2 per BW[4. Maximum number of inference report(s) activated for BM-Case1 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case1 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case1 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case1 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] 8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS 9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances 13. Supported maximum total number of reported predicted beams for predicted time instances in one report 14. Supported combinations of supported value(s) of valid time duration for each predicted time instance and number of predicted beams for each value of valid time duration 20. Supported BM-Case 2 sub usecase(s): e.g., setB-equals-to-setA, setB-subset-of-setA, setB-different-from-setA, or merged version(s) 21. supported number of occupied CPU 22. supported number of occupied APU 23. supported value of d for the relaxation of Z3 timeline 24. supported value of d’ for the relaxation of Z’3 timeline 17. Index of the occupied APU pool; FFS; yes; n/a; UE-side beam prediction for BM-Case2 [for inference] is not supported; FFS; FFS; FFS; FFS; FFS: CPU/AIMLPU related information FFS: candidate values for components; Optional with capability signalling. |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] | Change a typo in Component 3, where maximum number CSI report configuration for BM-Case2 should be per CC, rather than per BWP. Suggest to remove Component 14, since we don’t quite understand the new concept of valid time duration. 58. NR_AIML_air; 58-1-4; UE-side beam prediction for BM Case2 [for inference]; 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per CC 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs [4. Maximum number of inference report(s) activated for BM-Case2 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case2 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case2 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case2 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] 8. Supported CSI-RS resource types: Periodic CSI-RS, Semi-persistent CSI-RS 9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances 13. Supported maximum total number of reported predicted beams for predicted time instances in one report 21. supported number of occupied CPU 22. supported number of occupied APU 23. supported value of d for the relaxation of Z3 timeline 24. supported value of d’ for the relaxation of Z’3 timeline 2517. Index of the occupied APU pool; FFS; yes; n/a; UE-side beam prediction for BM-Case2 [for inference] is not supported; FFS Per UE; FFS No; FFS No; FFS No; Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} FFS: candidate values for components; Optional with capability signalling. |
| Samsung [11] | For FG 58-1-2, the capability reporting for inference report should be updated from “per band” to “per UE” since inference report is only feasible in bands in FR2. Same update should be applied for FG 58-1-3, FG 58-1-4 and FG 58-1-5. Proposal 1. Update FG 58-1-2, FG 58-1-3, FG 58-1-4 and FG 58-1-5 from “per UE” to “per band”. |
| LG Electronics [12] | 58. NR_AIML_air; 58-1-4; UE-side beam prediction for BM Case2 for inference; 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 8. Supported CSI-RS resource types 9. Supported inference report types 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances 13. Supported maximum total number of reported predicted beams for predicted time instances in one report 14. Supported combinations of supported value(s) of valid time duration for each predicted time instance and number of predicted beams for each value of valid time duration 21. supported number of occupied CPU 22. supported number of occupied APU 23. supported value of d for the relaxation of Z3 timeline 24. supported value of d’ for the relaxation of Z’3 timeline 25. Index of the occupied APU pool; FFS; yes; n/a; UE-side beam prediction for BM-Case2 for inference is not supported; Per UE; No; No; No; Component 7a candidate values: {4, 8, 16, 32, 64} Component 7b candidate values: {4, 8, 16, 32, 64} Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} Component 11 candidate values: {1, 2, 3, 4} Component 12 candidate values: {1, 2, 4, 8} Component 13 candidate values: {1, 2, 3, 4, 8, 12, 16, 32} Component 21 candidate values: {0, 1, 2} Component 22 candidate values: {0, 1, 2} Component 25 candidate values: {1, 2} FFS: candidate values for components; Optional with capability signalling. |
| Apple [13] | Beam prediction (BM Case 1 and BM case 2) may or may not provide values depending carrier frequency and deployment scenarios. Whether to implement beam prediction based on AI on one band or another band should be a choice driven by market needs, engineering/test efforts. There is no justification on mandating its universal support even if only a single band may be found to benefit from one of those features. We have Proposal 1-1: Rectify the type designation for 58-1-1/58-1-2/58-1-3/58-1-4/58-1-5/58-1-6/58-1-7 to "per Band". |
| Qualcomm Incorporated [14] | Proposal 5: FG58-1-1, FG58-1-2, FG58-1-3, FG58-1-4, FG58-1-5, FG58-1-6, and FG58-1-7, the FGs should be per band, following a similar principle as the FGs for legacy beam management and reporting. 58. NR_AIML_air; 58-1-4; UE-side beam prediction for BM Case2 [for inference]; 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 [for inference] with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs [4. Maximum number of inference report(s) activated for BM-Case2 per BWP] [4a. Maximum number of inference report(s) activated for BM-Case2 across all CCs] [5. Maximum number of inference report(s) triggered for BM-Case2 per BWP] [5a. Maximum number of inference report(s) triggered for BM-Case2 across all CCs] 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A [7. Supported combinations of the number of resources for Set B and the number of resources for Set A] [7a: Supported maximum number of resources for Set B] [7b: Supported maximum number of resources for Set A] 8. Supported CSI-RS resource types for Set B: Periodic CSI-RS, Semi-persistent CSI-RS 9. Supported inference report types: Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances 13. Supported maximum total number of reported predicted beams for predicted time instances in one report 14. Supported combinations of supported value(s) of valid time duration for each predicted time instance and number of predicted beams for each value of valid time duration 15. Supported value(s) of time gap between predicted time instances and between reference time to the first future time instance 16. Supported value(s) of setB periodicity 17. Supported CSI-RS resource types for Set A [20. Supported BM-Case 2 sub usecase(s): e.g., setB-equals-to-setA, setB-subset-of-setA, setB-different-from-setA, or merged version(s)]-> agreed for BM-Case1 already 21. supported number of occupied CPU 22. supported number of occupied APU 23. supported value of d for the relaxation of Z3 timeline 24. supported value of d’ for the relaxation of Z’3 timeline 2517. Index of the occupied APU pool; FFS; yes; n/a; UE-side beam prediction for BM-Case2 [for inference] is not supported; FFS Per UE Per band; FFS No; FFS No; FFS No; Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} FFS: candidate values for components Component 11 candidate values: {1, 2, 4} Component 12 candidate values: {2, 4, 6, 8} Component 13 candidate values: {4, 8, 12, 16} Component 7a candidate values: {16, 32, 64} Component 7b candidate values: {16, 32, 64} Component 17 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS} Component 21 candidate values: INTEGER (1..8) Component 22 candidate values: INTEGER (0..8) Component 23 and Component 24 candidate values: for μ={0,1,2,3,4,5,6}, the corresponding values for d and d' are: {98,196,392,784,1568,3136,6272}, respectively.; Optional with capability signalling. |
| NTT DOCOMO, INC. [15] | At RAN1 #122 meeting, the issue about the terminology of APU or CPU,2/CPU,3 was discussed. Based on the discussions, there is no need to change the CPU,2/CPU,3 in the current specification. It is better to use the aligned terminology with the specification for the UE feature discussions. Therefore, we propose changing all APUs occurring in FG 58 to the term aligned with the specification. Proposal 1: Revise the following FGs about the usage of APU and change the APU and corresponding items to align with the term CPU,2/CPU,3. - FG 58-0-1, FG 58-1-2, FG 58-1-4, FG 58-3-1, FG 58-3-2. Besides the APU issues, the prerequisite FGs for FG 58-1-2 and 58-1-4 have not been decided yet. Similar to FG 58-1-1, the basic beam management framework (FG 2-24) should be one of the prerequisite FGs. One note is also added to clarify the UE behaviors further if FG 58-0-1 is not signalled based on Proposal 2. For FG 58-1-2, we also propose the candidate values for components 7a, 7b, 12, 13, 14, and 17. For component 14, we suggest considering a limited number of candidate values to reduce the NW operation burden in managing UE resources. Besides the value 0, we propose three candidates to reflect the UE choice on small, medium, and large-scale models. The same proposals are also applied to FG 58-1-4. 58. NR_AIML_air; 58-1-4; UE-side beam prediction for BM Case2 [for inference]; 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 8. Supported CSI-RS resource types 9. Supported inference report types 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances 13. Supported maximum total number of reported predicted beams for predicted time instances in one report 14. Supported combinations of supported value(s) of valid time duration for each predicted time instance and number of predicted beams for each value of valid time duration 21. supported number of occupied CPU 22. Supported number of occupied APU CPU,2/CPU,3 23. supported value of d for the relaxation of Z3 timeline 24. supported value of d’ for the relaxation of Z’3 timeline 25. Occupied resource pool between CPU,2 and CPU,3APU pool; FFS 2-24; yes; n/a; UE-side beam prediction for BM-Case2 [for inference] is not supported; Per UE; No; No; No; Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} FFS: candidate values for components Component 7a candidate values: {4, 8, 16, 32} Component 7b candidate values: {16, 32, 64, 128} Component 11 candidate values: {16, 32, 64, 128} Component 12 candidate values: {1,2,4} Component 13 candidate values: {6, 8} Component 21 candidate values: {0, 1, 8} Component 22 candidate values: {0, 2, 4, 8} Component 25 candidate values: {CPU,2 or CPU,3} Note: UE should not report non-zero value for Component 22 if FG 58-0-1 is not signalled.; Optional with capability signalling. |

| 58. NR_AIML_air | 58-1-5 | UE-side beam prediction for BM-Case2 with predicted RSRP for inference | 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (spatial and time domain beam prediction) for inference | 58-1-4 | yes | n/a | UE-side beam prediction for BM-Case2 for inference is not supported | Per UE | No | No | No |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] | For FG 58-1-2, it is acceptable to retain the purpose as "for inference", since the UE inference feature is designated exclusively for this function and cannot be repurposed for other purposes such as data collection or monitoring. - For components of FG 58-1-2, FG 58-1-3, FG 58-1-4 and FG 58-1-5, support to retain the purpose as "for inference". 58. NR_AIML_air; 58-1-5; UE-side beam prediction for BM-Case2 with predicted RSRP [for inference] for inference; 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (spatial and time domain beam prediction) [for inference] for inference; 58-1-4; yes; UE-side beam prediction for BM-Case2 [for inference] for inference is not supported. |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] | FG 58-1-4 and 58-1-5 share most components with FG 58-1-2 and 58-1-3. Components 20 is supported for BM-Case 2. Similar as supported BM-Case 1 sub-usecases component for BM-Case 1, BM-Case 2 sub-usecases component for BM-Case 2 is supported to report in UE capability. So that when gNB transmits inference configuration, gNB can know whether UE supports pure time domain beam prediction or spatial and time domain beam prediction. Proposal 2: Take the following modification (in red) for the feature of Rel-19 AI based beam management: 58. NR_AIML_air; 58-1-5; UE-side beam prediction for BM-Case2 with predicted RSRP [for inference]; 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (spatial and time domain beam prediction) [for inference]; 58-1-4; yes; n/a; UE-side beam prediction for BM-Case2 [for inference] is not supported; FFS; FFS; FFS; FFS; Optional with capability signalling. |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] | BM-Case2 is time domain beam prediction, whereas BM-Case1 is spatial domain beam prediction. 58. NR_AIML_air; 58-1-5; UE-side beam prediction for BM-Case2 with predicted RSRP [for inference]; 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (domain beam prediction) [for inference]; 58-1-4; yes; n/a; UE-side beam prediction for BM-Case2 [for inference] is not supported; FFS Per UE; FFS No; FFS No; FFS No; Optional with capability signalling. |
| Samsung [11] | For FG 58-1-2, the capability reporting for inference report should be updated from “per band” to “per UE” since inference report is only feasible in bands in FR2. Same update should be applied for FG 58-1-3, FG 58-1-4 and FG 58-1-5. Proposal 1. Update FG 58-1-2, FG 58-1-3, FG 58-1-4 and FG 58-1-5 from “per UE” to “per band”. |
| LG Electronics [12] | 58. NR_AIML_air; 58-1-5; UE-side beam prediction for BM-Case2 with predicted RSRP for inference; 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (spatial and time domain beam prediction) for inference 2. Supported maximum number of predicted beams in each predicted time instance 3. Supported maximum number of predicted time instances 4. Supported maximum total number of reported predicted beams for predicted time instances in one report; 58-1-4; yes; n/a; UE-side beam prediction for BM-Case2 for inference is not supported; Per UE; No; No; No; Component 2 candidate values: {1, 2, 3, 4} Component 3 candidate values: {1, 2, 4, 8} Component 4 candidate values: {1, 2, 3, 4, 8, 12, 16, 32}; Optional with capability signalling. |
| Apple [13] | Beam prediction (BM Case 1 and BM case 2) may or may not provide values depending carrier frequency and deployment scenarios. Whether to implement beam prediction based on AI on one band or another band should be a choice driven by market needs, engineering/test efforts. There is no justification on mandating its universal support even if only a single band may be found to benefit from one of those features. We have Proposal 1-1: Rectify the type designation for 58-1-1/58-1-2/58-1-3/58-1-4/58-1-5/58-1-6/58-1-7 to "per Band". |
| Qualcomm Incorporated [14] | Proposal 5: FG58-1-1, FG58-1-2, FG58-1-3, FG58-1-4, FG58-1-5, FG58-1-6, and FG58-1-7, the FGs should be per band, following a similar principle as the FGs for legacy beam management and reporting. 58. NR_AIML_air; 58-1-5; UE-side beam prediction for BM-Case2 with predicted RSRP [for inference]; 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (spatial and time domain beam prediction) [for inference]; 58-1-4; yes; n/a; UE-side beam prediction for BM-Case2 [for inference] is not supported; FFS Per UE Per band; FFS No; FFS No; FFS No; Optional with capability signalling. |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_air | 58-1-6 | Performance monitoring for UE-sided model | 1. Support of performance monitoring with RS-PAI of AI/ML model for beam prediction. 2. Maximum total number of the configured CSI-RS resources for monitoring RS resource set 3. Maximum number of monitoring report(s) configured per BWP 4. Maximum number of monitoring report(s) configured across all CCs 5. Maximum number of monitoring occasions for RS-PAI calculation | FFS | yes | n/a | Performance monitoring for UE-sided model is not supported | FFS | FFS | FFS | FFS | FFS: component candidate values | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] | Adopt FGs 58-1-2 or 58-1-4 as the prerequisite for FG 58-1-6 |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] | - Component 2: Candidate values refer to the Component 7b of 58-1-2. - Component 3/4: Candidate values refer to Component 3/3a of 58-1-2 (i.e., CSI report number for inference) and Component 12 of 58-1-4 (i.e., number of predicted time instances for BM-Case 2). - Component 5: Candidate values refer to the RRC parameters of nroftransmissionOccasion-r19. - Component 13/14: Candidate values refer to the RRC parameters of nrofreportedpredictedrs-r19, nroftimeinstance-r19, and TimeGap-r19. The intention is to support all combinations. - Component 3/3a/7a/7B/21/22/23/24/25: Same values as 58-1-2. - Similar to 58-1-2, a note is added for CPU and APU to align with the notations in TS 38.214. 58. NR_AIML_air; 58-1-6; Performance monitoring for UE-sided model; 1. Support of performance monitoring with RS-PAI of AI/ML model for beam prediction. 2. Maximum total number of the configured CSI-RS resources for monitoring RS resource set 3. Maximum number of monitoring report(s) configured per BWP 4. Maximum number of monitoring report(s) configured across all CCs 5. Maximum number of monitoring occasions for RS-PAI calculation; FFS; yes; n/a; Performance monitoring for UE-sided model is not supported; Component 2 candidate values: {8, 16, 32, 64} Component 3 candidate values: {1, 2, 4, 8} Component 4 candidate values: {1, 2, 4, 8} Component 5 candidate values: {1, 3, 7, 15} FFS: candidate values for components; Optional with capability signalling. |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] | 58. NR_AIML_air; 58-1-6; Performance monitoring for UE-sided model; 1. Support of performance monitoring with RS-PAI of AI/ML model for beam prediction. 2. Maximum total number of the configured CSI-RS resources for monitoring RS resource set 3. Maximum number of monitoring report(s) configured per BWP 4. Maximum number of monitoring report(s) configured across all CCs 5. Maximum number of monitoring occasions for RS-PAI calculation; FFS 58-1-2/58-1-4; yes; n/a; Performance monitoring for UE-sided model is not supported; FFS; FFS; FFS; FFS; FFS: component candidate values; Optional with capability signalling. |
| OPPO [10] | Add SSB as performance monitoring resources and change maximum number of monitoring reports from per BWP to per CC. 58. NR_AIML_air; 58-1-6; Performance monitoring for UE-sided model; 1. Support of performance monitoring with RS-PAI of AI/ML model for beam prediction. 2. Maximum total number of the configured SSB resources and/or CSI-RS resources for monitoring RS resource set 3. Maximum number of monitoring report(s) configured per CC 4. Maximum number of monitoring report(s) configured across all CCs 5. Maximum number of monitoring occasions for RS-PAI calculation; FFS; yes; n/a; Performance monitoring for UE-sided model is not supported; FFS; FFS; FFS; FFS; FFS: component candidate values; Optional with capability signalling. |
| Samsung [11] | For FG 58-1-6, similar principle should be applied, which means that the capability signalling for FG 58-1-6 is per band. Proposal 2. Update FG 58-1-6 to “per band”. |
| LG Electronics [12] | 58. NR_AIML_air; 58-1-6; Performance monitoring for UE-sided model; 1. Support of performance monitoring with RS-PAI of AI/ML model for beam prediction. 2. Maximum total number of the configured CSI-RS resources for monitoring RS resource set 3. Maximum number of monitoring report(s) configured per BWP 4. Maximum number of monitoring report(s) configured across all CCs 5. Maximum number of monitoring occasions for RS-PAI calculation; FFS FFS: X-1-2, [X-1-3], X-1-4, [X-1-5]; yes; n/a; Performance monitoring for UE-sided model is not supported; FFS Per UE; FFS No; FFS No; FFS No; Component 2 candidate values: {4, 8, 16, 32, 64} Component 5 candidate values: {1, 3, 7 15} FFS: component candidate values; Optional with capability signalling. |
| Apple [13] | Beam prediction (BM Case 1 and BM case 2) may or may not provide values depending carrier frequency and deployment scenarios. Whether to implement beam prediction based on AI on one band or another band should be a choice driven by market needs, engineering/test efforts. There is no justification on mandating its universal support even if only a single band may be found to benefit from one of those features. We have Proposal 1-1: Rectify the type designation for 58-1-1/58-1-2/58-1-3/58-1-4/58-1-5/58-1-6/58-1-7 to "per Band". |
| Qualcomm Incorporated [14] | Proposal 5: FG58-1-1, FG58-1-2, FG58-1-3, FG58-1-4, FG58-1-5, FG58-1-6, and FG58-1-7, the FGs should be per band, following a similar principle as the FGs for legacy beam management and reporting. 58. NR_AIML_air; 58-1-6; Performance monitoring for UE-sided model; 1. Support of performance monitoring with RS-PAI of AI/ML model for beam prediction. 2. Maximum total number of the configured CSI-RS resources for monitoring RS resource set 3. Maximum number of monitoring report(s) configured per BWP 4. Maximum number of monitoring report(s) configured across all CCs 5. Maximum number of monitoring occasions for RS-PAI calculation 6. Support of SSB as RS type for monitoring 7. Support of CSI-RS as RS type for monitoring 8. Supported monitoring resource types 9. Supported monitoring report types; FFS; yes; n/a; Performance monitoring for UE-sided model is not supported; FFS Per band; FFS; FFS; FFS; FFS: component candidate values Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report}; Optional with capability signalling. |
| NTT DOCOMO, INC. [15] | FG 58-1-6 is defined for the performance monitoring for BM-Case 1 and BM-Case 2. Therefore, its prerequisite FG should be at least one of FG 58-1-2 or FG 58-1-4. The granularity of the performance monitoring should be the same as the other ones for the same use case, which is per UE. We also propose the candidate values for the components. 58. NR_AIML_air; 58-1-6; Performance monitoring for UE-sided model; 1. Support of performance monitoring with RS-PAI of AI/ML model for beam prediction. 2. Maximum total number of the configured CSI-RS resources for monitoring RS resource set 3. Maximum number of monitoring report(s) configured per BWP 4. Maximum number of monitoring report(s) configured across all CCs 5. Maximum number of monitoring occasions for RS-PAI calculation; FFS Either one of 58-1-2 or 58-1-4; yes; n/a; Performance monitoring for UE-sided model is not supported; FFS Per UE; FFS No; FFS No; FFS No; FFS: component candidate values Component 2 candidate values: {16, 32, 64, 128} Component 3 candidate values: {1, 2} Component 4 candidate values: {1,2,4,8} Component 5 candidate values: {1,2,3,4}; Optional with capability signalling. |

| 58. NR_AIML_air | 58-1-7 | Data collection for UE-side beam prediction | 1. Support of data collection for UE-side beam prediction 2. Support of SS/PBCH block and 1-port CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection 3. Supported sub-use cases 6: Supported maximum number of resources for Set B 7: Supported maximum number of resources for Set A 8. Support of SSB as RS type for Set B 9. Support of CSI-RS as RS type for Set B 10. Support of SSB as RS type for Set A 11. Support of CSI-RS as RS type for Set A | FFS | yes | n/a | Data collection for UE-side beam prediction is not supported | Per UE | No | No | No | Component 3 candidate values: {‘Set B equal to Set A’, ‘Set B subset of Set A’,’Set B not a subset of Set A’} Component 6 candidate values: {4, 8, 16, 32, 64} Component 7 candidate values: {8, 16, 32, 64} Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] | Adopt FGs 58-1-2 or 58-1-4 as the prerequisite for FG 58-1-7 |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] | 58. NR_AIML_air; 58-1-7; Data collection for UE-side beam prediction; 1. Support of data collection for UE-side beam prediction 2. Support of SS/PBCH block and 1-port CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection 3. Supported sub-use cases 6: Supported maximum number of resources for Set B 7: Supported maximum number of resources for Set A 8. Support of SSB as RS type for Set B 9. Support of CSI-RS as RS type for Set B 10. Support of SSB as RS type for Set A 11. Support of CSI-RS as RS type for Set A; FFS; yes; n/a; Data collection for UE-side beam prediction is not supported; Per UE; No; No; No; Component 3 candidate values: {‘Set B equal to Set A’, ‘Set B subset of Set A’,’Set B not a subset of Set A’} Component 6 candidate values: {4, 8, 16, 32, 64} Component 7 candidate values: {8, 16, 32, 64} Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2; Optional with capability signalling. |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] | Beam prediction (BM Case 1 and BM case 2) may or may not provide values depending carrier frequency and deployment scenarios. Whether to implement beam prediction based on AI on one band or another band should be a choice driven by market needs, engineering/test efforts. There is no justification on mandating its universal support even if only a single band may be found to benefit from one of those features. We have Proposal 1-1: Rectify the type designation for 58-1-1/58-1-2/58-1-3/58-1-4/58-1-5/58-1-6/58-1-7 to "per Band". |
| Qualcomm Incorporated [14] | Proposal 5: FG58-1-1, FG58-1-2, FG58-1-3, FG58-1-4, FG58-1-5, FG58-1-6, and FG58-1-7, the FGs should be per band, following a similar principle as the FGs for legacy beam management and reporting. 58. NR_AIML_air; 58-1-7; Data collection for UE-side beam prediction [for BM case 1]; 1. Support of data collection for UE-side beam prediction [for BM case 1] 2. Support of SS/PBCH block and 1-port CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection 3. Supported sub-use cases 6: Supported maximum number of resources for Set B 7: Supported maximum number of resources for Set A 8. Support of SSB as RS type for Set B 9. Support of CSI-RS as RS type for Set B 10. Support of SSB as RS type for Set A 11. Support of CSI-RS as RS type for Set A; FFS; yes; n/a; Data collection for UE-side beam prediction is not supported [for BM case 1]; Per UE Per band; N/A; N/A; N/A; Component 3 candidate values: {‘Set B equal to Set A’, ‘Set B subset of Set A’, ’Set B not a subset of Set A’} Component 6 candidate values: {4, 8, 16, 32, 64} Component 7 candidate values: {8, 16, 32, 64} Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2; Optional with capability signalling. |
| NTT DOCOMO, INC. [15] | The only one remaining issue for FG 58-1-7 is the prerequisite FG. Because the data collection is based on the legacy CSI framework and it does not require UE to support any AI/ML feature, the prerequisite FG should be the legacy CSI framework, i.e., FG 2-35 only. 58. NR_AIML_air; 58-1-7; Data collection for UE-side beam prediction; 1. Support of data collection for UE-side beam prediction 2. Support of SS/PBCH block and 1-port CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection 3. Supported sub-use cases 6: Supported maximum number of resources for Set B 7: Supported maximum number of resources for Set A 8. Support of SSB as RS type for Set B 9. Support of CSI-RS as RS type for Set B 10. Support of SSB as RS type for Set A 11. Support of CSI-RS as RS type for Set A; FFS 2-35; yes; n/a; Data collection for UE-side beam prediction is not supported; Per UE; N/A; N/A; N/A; Component 3 candidate values: {‘Set B equal to Set A’, ‘Set B subset of Set A’, ’Set B not a subset of Set A’} Component 6 candidate values: {4, 8, 16, 32, 64} Component 7 candidate values: {8, 16, 32, 64} Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2; Optional with capability signalling. |

Others

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] | Proposal 6: Adopt the following values for d and d' for AP AI/ML CSI report: |
| NTT DOCOMO, INC. [15] |  |

## Specification support for positioning accuracy enhancement

| 58. NR_AIML_Air | 58-2-1 | UE-based positioning Case 1 | Support of UE-based positioning Case 1 | 58-2-4 | N/A | N/A | UE-based positioning Case 1 is not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] | - Update FG 58-2-1 and FG 58-2-3/a/b prerequisites with “13-1 or 58-2-4”. - For each FG, add the same note from FG 58-2-4 clarifying that “If UE does not provide 58-2-4 but the UE supports Case 1, FG 13-1 indicates the DL PRS processing capabilities common across all positioning methods including UE-based positioning Case 1”. |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-2-2 | Support reception of AD for UE-based positioning Case 1 | Indicates support of reception of AD for UE-based positioning Case 1 | 58-2-1 | N/A | N/A | Reception of AD for UE-based positioning Case 1 is not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: For specific ADs, UE indicate supported AD that can be received as in [FFS: FGs] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] | For the FFS, the supported assistance data is not subject to capability in our understanding, since all info to be provided by the network for case 1 is not subject to a separate capability according to the agreement. - Remove the following note from FG 58-2-2: - “Note: For specific ADs, UE indicate supported AD that can be received as in [FFS: FGs]” |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] | Regarding the support of reception of AD for UE-based positioning Case 1, the note can be removed because RAN1 does not have any agreements for specific capability signalling for AD components. 58-2-2; Support reception of AD for UE-based positioning Case 1; Indicates support of reception of AD for UE-based positioning Case 1; 58-2-1; N/A; N/A; Reception of AD for UE-based positioning Case 1 is not supported; Per UE; n/a; n/a; n/a; Need for location server to know if the feature is supported. Note: For specific ADs, UE indicate supported AD that can be received as in [FFS: FGs]; Optional with capability signalling. |

| 58. NR_AIML_Air | 58-2-3 | DL PRS Resources for UE-based positioning Case 1 | 1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE 2. Max number of TRPs across all positioning frequency layers per UE 3. Max number of positioning frequency layers UE supports | [13-1] | N/A | N/A |  | Per UE | No | No | No | Component 1 candidate values: {1, 2} Component 2 candidate values: {4, 6, 12, 16, 24, 32, 64, 128, 256} Component 3 candidate values: {1, 2, 3, 4} Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] | - Update FG 58-2-1 and FG 58-2-3/a/b prerequisites with “13-1 or 58-2-4”. - For each FG, add the same note from FG 58-2-4 clarifying that “If UE does not provide 58-2-4 but the UE supports Case 1, FG 13-1 indicates the DL PRS processing capabilities common across all positioning methods including UE-based positioning Case 1”. |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] | Since UE-based positioning Case 1 is a UE feature independent of other positioning method, it should also be independent of PRS capability designed for other Rel-16 related positioning methods. This means that a UE supporting UE-based positioning Case 1 does not need to support other Rel-16 related positioning methods, i.e., FG 13-1, as the prerequisite for FG 58-2-3, 58-2-5 and 58-2-6. 58. NR_AIML_Air; 58-2-3; DL PRS Resources for UE-based positioning Case 1; 1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE 2. Max number of TRPs across all positioning frequency layers per UE 3. Max number of positioning frequency layers UE supports; [13-1] N/A; N/A; N/A; Per UE; No; No; No; Component 1 candidate values: {1, 2} Component 2 candidate values: {4, 6, 12, 16, 24, 32, 64, 128, 256} Component 3 candidate values: {1, 2, 3, 4} Need for location server to know if the feature is supported.; Optional with capability signaling. |
| Xiaomi [6] |  |
| CMCC [7] | As discussed in AI 9.1.2, UE-based positioning Case 1 need performance monitoring, which is derived at least in UE side. To make sure the model works properly, model monitoring should be a mandatory feature. A UE performing model inference needs little effort to support model monitoring, by comparing model inference result with other positioning methods. Thus, it should be together supported for model inference and model monitoring. Compared with legacy positioning methods, Case 1, i.e. UE-based positioning with UE-side model, has different requirement for the input data. Thus, it is reasonable to define new FGs for L1 measurement for AI based positioning. The principle of the max number of PRS resource and PRS resource Set for DL-TDOA could be reused as a starting point. Since case 1 is a UE-standalone positioning method, FGs about L1 measurement reporting and SRS transmission is not necessary for this feature. Compared with legacy positioning methods, the channel measurements used for positioning may be different in AI-based positioning, and the number of PRS resources required is also different. Therefore, it is recommended to define a new UE capability for the number of PRS resources for AI-based positioning. Proposal 3: For AI based positioning, model inference and model monitoring should be together supported. Proposal 4: For AI based positioning, a new UE capability for the number of PRS resources should be defined. Proposal 5: Adopt the following FGs for AI based positioning case1. 58. NR_AIML_Air; 58-2-3; DL PRS Resources for UE-based positioning Case 1; [1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE. Values = {1, 2} 2. Max number of TRPs across all positioning frequency layers per UE. Values = {4, 6, 12, 16, 24, 32, 64, 128, 256} 3. Max number of positioning frequency layers UE supports Values = {1, 2, 3, 4}]; 13-1; N/A; N/A; FFS Per UE; FFS n/a; FFS n/a; FFS n/a; Component 1 candidate values: {1, 2} Component 2 candidate values: {4, 6, 12, 16, 24, 32, 64, 128, 256} Component 3 candidate values: {1, 2, 3, 4} Need for location server to know if the feature is supported.; Optional with capability signaling. |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] | Proposal 2-1: Update Prerequisite of FG 58-2-3 (DL PRS Resources for UE-based positioning Case 1), FG 58-2-3a (DL PRS Resources for UE-based positioning Case 1 on a band), FG 58-2-3b (DL PRS Resources for UE-based positioning Case 1 on a band combination), FG 58-2-5 (Support of SSB from neighbor cell as QCL source of a DL PRS for UE-based positioning Case 1), FG 58-2-6 (Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1, - Prerequisite feature groups: 58-2-4 |
| Qualcomm Incorporated [14] | 58. NR_AIML_Air; 58-2-3; DL PRS Resources for UE-based positioning Case 1; 1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE 2. Max number of TRPs across all positioning frequency layers per UE 3. Max number of positioning frequency layers UE supports; 58-2-4 [13-1]; N/A; N/A; Per UE; No; No; No; Component 1 candidate values: {1, 2} Component 2 candidate values: {4, 6, 12, 16, 24, 32, 64, 128, 256} Component 3 candidate values: {1, 2, 3, 4} Need for location server to know if the feature is supported.; Optional with capability signaling. |
| NTT DOCOMO, INC. [15] | In Rel-19 discussion, there is no agreement to enhance DL PRS Resources for AI/ML based positioning. Therefore, we think it is straightforward to follow the elements of existing FGs 13-2/3/4 series. We also think that the relationship between FG 58-2-4 and 58-2-3 series is similar to the relationship between FG13-1 and FG 13-3 series of basic capabilities for legacy DL-TDOA of L1 UE features. Based on them, prerequisite feature group is FG 58-2-4. 58-2-3; DL PRS Resources for UE-based positioning Case 1; 1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE 2. Max number of TRPs across all positioning frequency layers per UE 3. Max number of positioning frequency layers UE supports; [13-1] 58-2-4; N/A; N/A; Per UE; n/a; n/a; n/a; Component 1 candidate values: {1, 2} Component 2 candidate values: {4, 6, 12, 16, 24, 32, 64, 128, 256} Component 3 candidate values: {1, 2, 3, 4} Need for location server to know if the feature is supported.; Optional with capability signaling. |

| 58. NR_AIML_Air | 58-2-3a | DL PRS Resources for UE-based positioning Case 1 on a band | [1. Max number of DL PRS Resources per DL PRS Resource Set 2. Max number of DL PRS Resources per positioning frequency layer | [13-1] | N/A | N/A |  | Per band | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 4, 8, 16, 32, 64} Component 2 candidate values: {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: For component 1, the values 16, 32, 64 are only applicable to FR2 bands Note: For component 2, the value 6 is only applicable to FR1 bands Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] | - Update FG 58-2-1 and FG 58-2-3/a/b prerequisites with “13-1 or 58-2-4”. - For each FG, add the same note from FG 58-2-4 clarifying that “If UE does not provide 58-2-4 but the UE supports Case 1, FG 13-1 indicates the DL PRS processing capabilities common across all positioning methods including UE-based positioning Case 1”. |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] | Since UE-based positioning Case 1 is a UE feature independent of other positioning method, it should also be independent of PRS capability designed for other Rel-16 related positioning methods. This means that a UE supporting UE-based positioning Case 1 does not need to support other Rel-16 related positioning methods, i.e., FG 13-1, as the prerequisite for FG 58-2-3, 58-2-5 and 58-2-6. 58. NR_AIML_Air; 58-2-3a; DL PRS Resources for UE-based positioning Case 1 on a band; [1. Max number of DL PRS Resources per DL PRS Resource Set 2. Max number of DL PRS Resources per positioning frequency layer; [13-1] N/A; N/A; N/A; Per band; n/a; n/a; n/a; Component 1 candidate values: {1, 2, 4, 8, 16, 32, 64} Component 2 candidate values: {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: For component 1, the values 16, 32, 64 are only applicable to FR2 bands Note: For component 2, the value 6 is only applicable to FR1 bands Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination.; Optional with capability signaling. |
| Xiaomi [6] |  |
| CMCC [7] | As discussed in AI 9.1.2, UE-based positioning Case 1 need performance monitoring, which is derived at least in UE side. To make sure the model works properly, model monitoring should be a mandatory feature. A UE performing model inference needs little effort to support model monitoring, by comparing model inference result with other positioning methods. Thus, it should be together supported for model inference and model monitoring. Compared with legacy positioning methods, Case 1, i.e. UE-based positioning with UE-side model, has different requirement for the input data. Thus, it is reasonable to define new FGs for L1 measurement for AI based positioning. The principle of the max number of PRS resource and PRS resource Set for DL-TDOA could be reused as a starting point. Since case 1 is a UE-standalone positioning method, FGs about L1 measurement reporting and SRS transmission is not necessary for this feature. Compared with legacy positioning methods, the channel measurements used for positioning may be different in AI-based positioning, and the number of PRS resources required is also different. Therefore, it is recommended to define a new UE capability for the number of PRS resources for AI-based positioning. Proposal 3: For AI based positioning, model inference and model monitoring should be together supported. Proposal 4: For AI based positioning, a new UE capability for the number of PRS resources should be defined. Proposal 5: Adopt the following FGs for AI based positioning case1. 58. NR_AIML_Air; 58-2-3a; DL PRS Resources for UE-based positioning Case 1 on a band; [1. Max number of DL PRS Resources per DL PRS Resource Set Values = {1, 2, 4, 8, 16, 32, 64} Note: 16, 32, 64 are only applicable to FR2 bands 2. Max number of DL PRS Resources per positioning frequency layer. Values = {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: 6 is only applicable to FR1 bands]; 13-1; N/A; N/A; FFS Per band; FFS n/a; FFS n/a; FFS n/a; Component 1 candidate values: {1, 2, 4, 8, 16, 32, 64} Component 2 candidate values: {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: For component 1, the values 16, 32, 64 are only applicable to FR2 bands Note: For component 2, the value 6 is only applicable to FR1 bands Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination.; Optional with capability signaling. |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] | 58. NR_AIML_Air; 58-2-3a; DL PRS Resources for UE-based positioning Case 1 on a band; [1. Max number of DL PRS Resources per DL PRS Resource Set 2. Max number of DL PRS Resources per positioning frequency layer; 58-2-4 [13-1]; N/A; N/A; Per band; n/a; n/a; n/a; Component 1 candidate values: {1, 2, 4, 8, 16, 32, 64} Component 2 candidate values: {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: For component 1, the values 16, 32, 64 are only applicable to FR2 bands Note: For component 2, the value 6 is only applicable to FR1 bands Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination.; Optional with capability signaling. |
| NTT DOCOMO, INC. [15] | In Rel-19 discussion, there is no agreement to enhance DL PRS Resources for AI/ML based positioning. Therefore, we think it is straightforward to follow the elements of existing FGs 13-2/3/4 series. We also think that the relationship between FG 58-2-4 and 58-2-3 series is similar to the relationship between FG13-1 and FG 13-3 series of basic capabilities for legacy DL-TDOA of L1 UE features. Based on them, prerequisite feature group is FG 58-2-4. 58-2-3a; DL PRS Resources for UE-based positioning Case 1 on a band; 1. Max number of DL PRS Resources per DL PRS Resource Set 2. Max number of DL PRS Resources per positioning frequency layer; [13-1] 58-2-4; N/A; N/A; Per Band; n/a; n/a; n/a; Component 1 candidate values: {1, 2, 4, 8, 16, 32, 64} Component 2 candidate values: {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: For component 1, the values 16, 32, 64 are only applicable to FR2 bands Note: For component 2, the value 6 is only applicable to FR1 bands Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination.; Optional with capability signaling. |

| 58. NR_AIML_Air | 58-2-3b | DL PRS Resources for UE-based positioning Case 1 on a band combination | 1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation | [13-1] | N/A | N/A |  | Per BC | n/a | n/a | n/a | Component 1 candidate values: {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC Component 2 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC Component 3 candidate values: {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands Component 4 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands] Need for location server to know if the feature is supported. Note: the reported value is the total number across all bands in the corresponding BC Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] | - Update FG 58-2-1 and FG 58-2-3/a/b prerequisites with “13-1 or 58-2-4”. - For each FG, add the same note from FG 58-2-4 clarifying that “If UE does not provide 58-2-4 but the UE supports Case 1, FG 13-1 indicates the DL PRS processing capabilities common across all positioning methods including UE-based positioning Case 1”. |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] | Since UE-based positioning Case 1 is a UE feature independent of other positioning method, it should also be independent of PRS capability designed for other Rel-16 related positioning methods. This means that a UE supporting UE-based positioning Case 1 does not need to support other Rel-16 related positioning methods, i.e., FG 13-1, as the prerequisite for FG 58-2-3, 58-2-5 and 58-2-6. 58. NR_AIML_Air; 58-2-3b; DL PRS Resources for UE-based positioning Case 1 on a band combination; 1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation; [13-1] N/A; N/A; N/A; Per BC; n/a; n/a; n/a; Component 1 candidate values: {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC Component 2 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC Component 3 candidate values: {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands Component 4 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands] Need for location server to know if the feature is supported. Note: the reported value is the total number across all bands in the corresponding BC Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination.; Optional with capability signaling. |
| Xiaomi [6] |  |
| CMCC [7] | As discussed in AI 9.1.2, UE-based positioning Case 1 need performance monitoring, which is derived at least in UE side. To make sure the model works properly, model monitoring should be a mandatory feature. A UE performing model inference needs little effort to support model monitoring, by comparing model inference result with other positioning methods. Thus, it should be together supported for model inference and model monitoring. Compared with legacy positioning methods, Case 1, i.e. UE-based positioning with UE-side model, has different requirement for the input data. Thus, it is reasonable to define new FGs for L1 measurement for AI based positioning. The principle of the max number of PRS resource and PRS resource Set for DL-TDOA could be reused as a starting point. Since case 1 is a UE-standalone positioning method, FGs about L1 measurement reporting and SRS transmission is not necessary for this feature. Compared with legacy positioning methods, the channel measurements used for positioning may be different in AI-based positioning, and the number of PRS resources required is also different. Therefore, it is recommended to define a new UE capability for the number of PRS resources for AI-based positioning. Proposal 3: For AI based positioning, model inference and model monitoring should be together supported. Proposal 4: For AI based positioning, a new UE capability for the number of PRS resources should be defined. Proposal 5: Adopt the following FGs for AI based positioning case1. 58. NR_AIML_Air; 58-2-3b; DL PRS Resources for UE-based positioning Case 1 on a band combination; [1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only. Values = {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC. 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation. Values = {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation. Values = {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands]; 13-1; N/A; N/A; FFS Per BC; FFS n/a; FFS n/a; FFS n/a; Component 1 candidate values: {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC Component 2 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC Component 3 candidate values: {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands Component 4 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands] Need for location server to know if the feature is supported. Note: [the reported value is the total number across all bands in the corresponding BC] Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination.; Optional with capability signaling. |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] | 58. NR_AIML_Air; 58-2-3b; DL PRS Resources for UE-based positioning Case 1 on a band combination; 1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation; 58-2-4 [13-1]; N/A; N/A; Per BC; n/a; n/a; n/a; Component 1 candidate values: {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC Component 2 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC Component 3 candidate values: {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands Component 4 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands] Need for location server to know if the feature is supported. Note: the reported value is the total number across all bands in the corresponding BC Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination.; Optional with capability signaling. |
| NTT DOCOMO, INC. [15] | In Rel-19 discussion, there is no agreement to enhance DL PRS Resources for AI/ML based positioning. Therefore, we think it is straightforward to follow the elements of existing FGs 13-2/3/4 series. We also think that the relationship between FG 58-2-4 and 58-2-3 series is similar to the relationship between FG13-1 and FG 13-3 series of basic capabilities for legacy DL-TDOA of L1 UE features. Based on them, prerequisite feature group is FG 58-2-4. 58-2-3b; DL PRS Resources for UE-based positioning Case 1 on a band combination; 1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation; [13-1] 58-2-4; N/A; N/A; Per BC; n/a; n/a; n/a; Component 1 candidate values: {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC Component 2 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC Component 3 candidate values: {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands Component 4 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands] Need for location server to know if the feature is supported. Note: the reported value is the total number across all bands in the corresponding BC Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination.; Optional with capability signaling. |

| 58. NR_AIML_Air | 58-2-4 | DL PRS Processing Capability for UE-based positioning Case 1 | 1. Maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 2. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 3. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 4. Max number of DL PRS resources that UE can process in a slot under it Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than 30%. |  | No | N/A |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Component 1 candidate values: -FR1 bands values {5, 10, 20, 40, 50, 80, 100} -FR2 bands values {50, 100, 200, 400} Component 3 candidate values: -T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms -N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 4 candidate values: -FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz -FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Notes for component 3: a. UE reports one combination of (N, T) values per band, where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE b. UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value c. UE DL PRS processing capability is defined for a single positioning frequency layer. UE capability for simultaneous DL PRS processing across positioning frequency layers is not supported in Rel.16 (i.e. for a UE supporting multiple positioning frequency layers, a UE is expected to process one frequency layer at a time) d. UE DL PRS processing capability is agnostic to DL PRS comb factor configuration e. The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS Note: if the UE does not indicate this capability for a band or band combination, the UE does not support PRS processing in this band or band combination. Note: If UE does not provide [this FG] but the UE supports Case 1, FG 13-1 indicates the DL PRS processing capabilities common across all positioning methods including UE-based positioning Case 1.] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] | Confirm the notes for FG 58-2-4. |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] | As discussed in AI 9.1.2, UE-based positioning Case 1 need performance monitoring, which is derived at least in UE side. To make sure the model works properly, model monitoring should be a mandatory feature. A UE performing model inference needs little effort to support model monitoring, by comparing model inference result with other positioning methods. Thus, it should be together supported for model inference and model monitoring. Compared with legacy positioning methods, Case 1, i.e. UE-based positioning with UE-side model, has different requirement for the input data. Thus, it is reasonable to define new FGs for L1 measurement for AI based positioning. The principle of the max number of PRS resource and PRS resource Set for DL-TDOA could be reused as a starting point. Since case 1 is a UE-standalone positioning method, FGs about L1 measurement reporting and SRS transmission is not necessary for this feature. Compared with legacy positioning methods, the channel measurements used for positioning may be different in AI-based positioning, and the number of PRS resources required is also different. Therefore, it is recommended to define a new UE capability for the number of PRS resources for AI-based positioning. Proposal 3: For AI based positioning, model inference and model monitoring should be together supported. Proposal 4: For AI based positioning, a new UE capability for the number of PRS resources should be defined. Proposal 5: Adopt the following FGs for AI based positioning case1. 58. NR_AIML_Air; 58-2-z1; DL PRS Processing Capability for UE-based positioning Case 1; 1. Maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 2. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 3. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 4. Max number of DL PRS resources that UE can process in a slot under it Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than 30%.; FFS; No; N/A; Per band; n/a; n/a; n/a; Need for location server to know if the feature is supported. Component 1 candidate values: -FR1 bands values {5, 10, 20, 40, 50, 80, 100} -FR2 bands values {50, 100, 200, 400} Component 3 candidate values: -T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms -N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 4 candidate values: -FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz -FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Notes for component 3: a. UE reports one combination of (N, T) values per band, where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE b. UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value c. UE DL PRS processing capability is defined for a single positioning frequency layer. UE capability for simultaneous DL PRS processing across positioning frequency layers is not supported in Rel.16 (i.e. for a UE supporting multiple positioning frequency layers, a UE is expected to process one frequency layer at a time) d. UE DL PRS processing capability is agnostic to DL PRS comb factor configuration e. The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS Note: if the UE does not indicate this capability for a band or band combination, the UE does not support PRS processing in this band or band combination. Note: If the UE reports both FG 13-1 and this FG, then the UE is not expected to perform simultaneous processing of legacy methods and Case 1 positioning method. Note: If UE does not provide this FG but the UE supports Case 1, FG 13-1 indicates the DL PRS processing capabilities common across all positioning methods including UE-based positioning Case 1.; Optional with capability signaling. |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] | Regarding DL PRS Processing Capability for UE-based positioning Case 1, we think the last note can be removed. At the last meeting, it was agreed that the prerequisite feature group for FG 58-2-1 is only FG 58-2-4, which means UE always reports FG 58-2-4 when UE can use AI/ML based positioning for Case 1. Therefore, FG 58-2-4 can be always referred to and the note can be removed. 58-2-4; DL PRS Processing Capability for UE-based positioning Case 1; 1. Maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 2. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 3. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 4. Max number of DL PRS resources that UE can process in a slot under it Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than 30%.; No; N/A; Per band; n/a; n/a; n/a; Need for location server to know if the feature is supported. Component 1 candidate values: -FR1 bands values {5, 10, 20, 40, 50, 80, 100} -FR2 bands values {50, 100, 200, 400} Component 3 candidate values: -T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms -N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 4 candidate values: -FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz -FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Notes for component 3: a. UE reports one combination of (N, T) values per band, where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE b. UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value c. UE DL PRS processing capability is defined for a single positioning frequency layer. UE capability for simultaneous DL PRS processing across positioning frequency layers is not supported in Rel.16 (i.e. for a UE supporting multiple positioning frequency layers, a UE is expected to process one frequency layer at a time) d. UE DL PRS processing capability is agnostic to DL PRS comb factor configuration e. The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS Note: if the UE does not indicate this capability for a band or band combination, the UE does not support PRS processing in this band or band combination. Note: If UE does not provide [this FG] but the UE supports Case 1, FG 13-1 indicates the DL PRS processing capabilities common across all positioning methods including UE-based positioning Case 1.]; Optional with capability signaling. |

| 58. NR_AIML_Air | 58-2-7 | Simultaneous UE-based positioning Case 1 and DL-TDoA positioning | 1. Support of simultaneous operation for UE-based positioning Case 1 and DL TDoA | 58-2-3 and 13-3 | No | N/A |  | Per band | n/a | n/a | n/a | If it is not indicated, a UE is not expected to perform simultaneously the UE-based positioning Case 1 and DL TDoA positioning Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-2-8 | Simultaneous UE-based positioning Case 1 and DL-AoD positioning | 1. Support of simultaneous operation for UE-based positioning Case 1 and DL AoD | 58-2-3 and 13-2 | No | N/A | Simultaneous UE-based positioning Case 1 and DL-AoD positioning is not supported | Per band | n/a | n/a | n/a | If it is not indicated, a UE is not expected to perform simultaneously the UE-based Case 1 and DL AoD positioning Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-2-9 | DL PRS measurement outside MG and in a PRS processing window for UE-based positioning Case 1 | 1. Supported PRS processing types subject to the UE determining that DL PRS to be higher priority for PRS measurement outside MG and in a PRS processing window 2. Support of priority handing options of PRS: Option1, Option2 or Option3 − Option 1: Support of "st1" and "st3" defined in clause 5.1.6.5 of TS 38.214 [20] − Option 2: Support of "st1", "st2", and "st3" defined in clause 5.1.6.5 of TS 38.214 [20] - Option 3: Support of "st1" only defined in clause 5.1.6.5 of TS 38.214 [20] | 58-2-4 | N/A | N/A | DL PRS measurement outside MG and in a PRS processing window for UE-based positioning Case 1 is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: One or more of {Type 1A, Type 1B, Type 2} Component 2 candidate values: {option1, option2, option3} Need for location server to know if the feature is supported Note: Component 2 can be reported per supported band for each type supported by the UE, details left to RAN2 Note: - Type 1A refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR - Type 1B refers to the determination of prioritization between DL PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected - Type 2 refers to the determination of prioritization between DL PRS and other DL signals/channels only in DL PRS symbols within the PRS processing window Note: When the UE determines higher priority for other DL signals/channels over the PRS measurement/processing, the UE is not expected to measure/process DL PRS which is applicable to all of the above capability options Note: Within a PRS processing window, UE measurement is inside the active DL BWP with PRS having the same numerology as the active DL BWP Note: Support of configuration of PRS processing window in RRC and support of using DL MAC CE to activate/deactivate the PRS processing window for PRS measurements is part of the FG , but no dedicated signaling is required. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-2-10 | DL PRS Processing Capability outside MG - buffering capability for UE-based positioning Case 1 | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2a. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE 2b. Duration of DL PRS symbols N2 in units of ms a UE can process inT2 ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE 3. Max number of DL PRS resources that UE can process in a slot 4. Maximum DL PRS bandwidth in MHz, which is supported and reported by UE for PRS measurement outside MG within the PPW | 58-2-9 | N/A | N/A | DL PRS Processing Capability outside MG - buffering capability for UE-based positioning Case 1 is not supported | Per band | n/a | n/a | n/a | Component 1 candidate values: {Type 1, Type 2} Component 2a candidate values: a) T: {1, 2, 4, 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Candidate 2b component values: a) N2: {0.125, 0.25, 0.5, 1, 2, 3, 4, 5, 6, 8, 12} ms b) T2: {4, 5, 6, 8} ms Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Component 4 candidate values: FR1 bands: {5, 10, 20, 40, 50, 80, 100} FR2 bands: {50, 100, 200, 400} Need for location server to know if the feature is supported Note 1:The (N, T) UE capabilities are interpreted as legacy (N, T) in FG 58-2-4, and the UE is expected to receive the PRS within the PRS processing window and but the processing of the received PRS may be outside a PRS processing window. The (N2, T2) UE capabilities are interpreted such that the UE is capable of measuring up to N2 ms PRS within a PPW and is capable of completing the PRS processing within the PPW, e.g., if the time duration from the last symbol of the measured PRS resource(s) inside the PPW, to the end of PPW is not smaller than T2 ms Note 3: UE shall support either component 2a and component 2b , but not both for each supported type in a band Note 4: A UE shall declare PRS processing capabilities of each of the supported Type-1A, Type-1B, Type-2 capabilities in case it supports multiple types in a band Note: if UE supports same values for one or more components as in FG 27-3-3, then the UE can skip indicating these components in this FG and the values in corresponding FG 27-3-3 components indicate supported PRS processing capabilities outside MG for Case 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-2-11 | Support of more than one activated PRS processing windows across all active DL BWPs for UE-based positioning Case 1 | 1. Number of supported activated PRS processing windows | 58-2-9 | N/A | N/A | DL PRS measurement outside MG and in a PRS processing window for UE-based positioning Case 1 is not supported | Per UE | No | No | No | Candidate values: {2, 3, 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-2-15 | DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous within a MG for RRC_CONNECTED - UE-based positioning Case 1 | 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Maximum number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot | 58-2-4 | N/A | N/A | DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous within a MG for RRC_CONNECTED - UE-based positioning Case 1 is not supported | Per Band | n/a | n/a | n/a | Component 1 candidate values: a) FR1 bands: {10, 20, 40, 50, 80, 100, 160, 200} b) FR2 bands: {100, 200, 400, 800} Component 2 candidate values: a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in this FG follows buffering capability type reported in FG 58-2-4 Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by FG 58-2-4, or this value T should be equal or larger than the value T reported by FG 58-2-4 Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each two linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by FG 58-2-4 Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL)/measurement gap repetition period (MGRP) of no more than 30% Note: if UE supports same values for one or more components as in FG 41-4-1, then the UE can skip indicating these components in this FG and the values in corresponding FG 41-4-1 components indicate supported aggregated PRS processing of 2 PFLs in intra-band contiguous within a MG for RRC_CONNECTED for Case 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-2-15a | DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous within a MG for RRC_CONNECTED - UE-based positioning Case 1 | 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Maximum number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot | 58-2-15 | N/A | N/A | DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous within a MG for RRC_CONNECTED - UE-based positioning Case 1 is not supported | Per Band | n/a | n/a | n/a | Component 1 candidate values: a) FR1 bands: {15, 20, 30, 40, 50, 60, 80, 100, 120, 140, 150, 160, 180, 200, 240, 300}} b) FR2 bands: {150, 200, 300, 400, 600, 800, 1000, 1200} Component 2 candidate values: a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in this FG follows buffering capability type reported in FG 58-2-4 Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by FG 58-2-4 or this value T should be equal or larger than the value T reported by FG 58-2-4 Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each three linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by FG 58-2-4 Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL)/measurement gap repetition period (MGRP) of no more than 30% Note: if UE supports same values for one or more components as in FG 41-4-1a, then the UE can skip indicating these components in this FG and the values in corresponding FG 41-4-1a components indicate supported aggregated PRS processing of 3 PFLs in intra-band contiguous within a MG for RRC_CONNECTED for Case 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-2-16 | PRS bandwidth aggregation with two PFL combinations - UE-based positioning Case 1 | Support of PRS bandwidth aggregation with two PFL combinations for UE-based positioning Case 1 | 58-2-15 | N/A | N/A | PRS bandwidth aggregation with two PFL combinations for UE-based positioning Case 1 is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: More than one combination are measured in TDMed manner | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-2-17 | PRS bandwidth aggregation in RRC_CONNECTED UE-based positioning Case 1 | Support of PRS bandwidth aggregation in RRC_CONNECTED for UE-based positioning Case 1 | 58-2-3, 58-2-15 | N/A | N/A | PRS bandwidth aggregation in RRC_CONNECTED for UE-based positioning Case 1 is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-2-20 | Support of PRS TEG association information for UE-based positioning Case 1 | Support of reception of association between PRS and TRP Tx TEG for UE-based positioning Case 1 | 58-2-4 | N/A | N/A | Reception of PRS TEG association information for UE-based positioning Case 1 is not supported | Per UE | No | No | No | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-2-5 | Support of SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 | 1. Support of SSB from neighbour cell as QCL source of a DL PRS 2. Support of reuse SSB measurement from RRM for receiving PRS Note: Refers to Type-C for FR1 and Type-C & Type-D support for FR2 | [58-2-4; otherwise 13-1] | No | N/A | SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] | The prerequisite for QCL information for case 1 is currently either the new processing capability FG 58-2-4 or the legacy FG 13-1. in our view, if the legacy processing 13-1 is declared, the legacy QCL info FGs 13-7/a can also be re-used. Hence only FG 58-2-4 is needed as a prerequisite for 58-2-5/6. - For FG 58-2-5 and 58-2-6, only FG 58-2-4 is needed as a prerequisite. |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] | Since UE-based positioning Case 1 is a UE feature independent of other positioning method, it should also be independent of PRS capability designed for other Rel-16 related positioning methods. This means that a UE supporting UE-based positioning Case 1 does not need to support other Rel-16 related positioning methods, i.e., FG 13-1, as the prerequisite for FG 58-2-3, 58-2-5 and 58-2-6. 58. NR_AIML_Air; 58-2-5; Support of SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1; 1. Support of SSB from neighbour cell as QCL source of a DL PRS 2. Support of reuse SSB measurement from RRM for receiving PRS Note: Refers to Type-C for FR1 and Type-C & Type-D support for FR2; [58-2-4; otherwise N/A 13-1]; N/A; N/A; Per band; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. |
| Xiaomi [6] |  |
| CMCC [7] | As discussed in AI 9.1.2, UE-based positioning Case 1 need performance monitoring, which is derived at least in UE side. To make sure the model works properly, model monitoring should be a mandatory feature. A UE performing model inference needs little effort to support model monitoring, by comparing model inference result with other positioning methods. Thus, it should be together supported for model inference and model monitoring. Compared with legacy positioning methods, Case 1, i.e. UE-based positioning with UE-side model, has different requirement for the input data. Thus, it is reasonable to define new FGs for L1 measurement for AI based positioning. The principle of the max number of PRS resource and PRS resource Set for DL-TDOA could be reused as a starting point. Since case 1 is a UE-standalone positioning method, FGs about L1 measurement reporting and SRS transmission is not necessary for this feature. Compared with legacy positioning methods, the channel measurements used for positioning may be different in AI-based positioning, and the number of PRS resources required is also different. Therefore, it is recommended to define a new UE capability for the number of PRS resources for AI-based positioning. Proposal 3: For AI based positioning, model inference and model monitoring should be together supported. Proposal 4: For AI based positioning, a new UE capability for the number of PRS resources should be defined. Proposal 5: Adopt the following FGs for AI based positioning case1. 58. NR_AIML_Air; 58-2-x; Support of SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1; 1. Support of SSB from neighbour cell as QCL source of a DL PRS 2. Support of reuse SSB measurement from RRM for receiving PRS Note: Refers to Type-C for FR1 and Type-C & Type-D support for FR2; 58-2-z1; otherwise 13-1; No; N/A; SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported; Per band; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] | UE may support QCL source and relations same to existing legacy methods. If this is the case, to optimize the signaling overhead, the UE may skip those components or FGs that are common to existing legacy methods. We propose adding the following notes: - Note: if UE supports same values for one or more components as in FG 13-7, then the UE can skip indicating these components in this FG and the values in corresponding FG 13-7 components indicate supported QCL sources for Case 1 - Note: if UE supports same values for one or more components as in FG 13-7a, then the UE can skip indicating these components in this FG and the values in corresponding FG 13-7a components indicate supported QCL sources for Case 1 Proposal 13: Modify the Rel. 19 UE FG of PRS processing capabilities for UE-based positioning Case 1: 58. NR_AIML_Air; 58-2-5; Support of SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1; 1. Support of SSB from neighbour cell as QCL source of a DL PRS 2. Support of reuse SSB measurement from RRM for receiving PRS Note: Refers to Type-C for FR1 and Type-C & Type-D support for FR2; 58-2-4 [ otherwise 13-1]; No; N/A; SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported; Per band; n/a; n/a; n/a; Need for location server to know if the feature is supported. Note: if UE supports same values for one or more components as in FG 13-7, then the UE can skip indicating these components in this FG and the values in corresponding FG 13-7 components indicate supported QCL sources for Case 1; Optional with capability signaling. |
| NTT DOCOMO, INC. [15] | For FG58-2-5, following the legacy capability “Support of SSB from neighbour cell as QCL source of a DL PRS,” prerequisite feature group should be DL PRS Processing Capability. Following this principle, prerequisite feature group should be FG 58-2-4. FG 13-1 is not necessary because FG 58-2-4 can be always provided when UE can use UE-based positioning Case 1. 58-2-5; Support of SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1; 1. Support of SSB from neighbour cell as QCL source of a DL PRS 2. Support of reuse SSB measurement from RRM for receiving PRS Note: Refers to Type-C for FR1 and Type-C & Type-D support for FR2; [58-2-4; otherwise 13-1]; No; N/A; SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported; Per band; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signaling. |

| 58. NR_AIML_Air | 58-2-6 | Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 | 1. Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS Note 1: Refers to Type-D support for FR2 Note 2: A PRS from a PRS-only TP is treated as PRS from a non-serving cell | [58-2-4; otherwise 13-1] | No | N/A | DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. DL PRSs are in the same band | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] | The prerequisite for QCL information for case 1 is currently either the new processing capability FG 58-2-4 or the legacy FG 13-1. in our view, if the legacy processing 13-1 is declared, the legacy QCL info FGs 13-7/a can also be re-used. Hence only FG 58-2-4 is needed as a prerequisite for 58-2-5/6. For FG 58-2-5 and 58-2-6, only FG 58-2-4 is needed as a prerequisite. |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] | Since UE-based positioning Case 1 is a UE feature independent of other positioning method, it should also be independent of PRS capability designed for other Rel-16 related positioning methods. This means that a UE supporting UE-based positioning Case 1 does not need to support other Rel-16 related positioning methods, i.e., FG 13-1, as the prerequisite for FG 58-2-3, 58-2-5 and 58-2-6. 58. NR_AIML_Air; 58-2-6; Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1; 1. Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS Note 1: Refers to Type-D support for FR2 Note 2: A PRS from a PRS-only TP is treated as PRS from a non-serving cell; [58-2-4; otherwise N/A 13-1]; N/A; N/A; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported. DL PRSs are in the same band; Optional with capability signaling. |
| Xiaomi [6] |  |
| CMCC [7] | As discussed in AI 9.1.2, UE-based positioning Case 1 need performance monitoring, which is derived at least in UE side. To make sure the model works properly, model monitoring should be a mandatory feature. A UE performing model inference needs little effort to support model monitoring, by comparing model inference result with other positioning methods. Thus, it should be together supported for model inference and model monitoring. Compared with legacy positioning methods, Case 1, i.e. UE-based positioning with UE-side model, has different requirement for the input data. Thus, it is reasonable to define new FGs for L1 measurement for AI based positioning. The principle of the max number of PRS resource and PRS resource Set for DL-TDOA could be reused as a starting point. Since case 1 is a UE-standalone positioning method, FGs about L1 measurement reporting and SRS transmission is not necessary for this feature. Compared with legacy positioning methods, the channel measurements used for positioning may be different in AI-based positioning, and the number of PRS resources required is also different. Therefore, it is recommended to define a new UE capability for the number of PRS resources for AI-based positioning. Proposal 3: For AI based positioning, model inference and model monitoring should be together supported. Proposal 4: For AI based positioning, a new UE capability for the number of PRS resources should be defined. Proposal 5: Adopt the following FGs for AI based positioning case1. 58. NR_AIML_Air; 58-2-x; Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1; 1. Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS Note 1: Refers to Type-D support for FR2 Note 2: A PRS from a PRS-only TP is treated as PRS from a non-serving cell; 58-2-z1; otherwise 13-1; No; N/A; DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported. DL PRSs are in the same band; Optional with capability signaling. |
| Nokia [8] | Explicitly mentioning about the PRS-only TP is not supported in Rel. 19. Proposal 3: Remove support to PRS-only TP in FG index 58-2-6 for AIML positioning Case 1. 58. NR_AIML_Air; 58-2-6; Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1; 1. Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS Note 1: Refers to Type-D support for FR2 Note 2:A PRS from a PRS-only TP is not considered in Rel. 19; [58-2-4; otherwise 13-1]; No; N/A; DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported. DL PRSs are in the same band; Optional with capability signaling. |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] | UE may support QCL source and relations same to existing legacy methods. If this is the case, to optimize the signaling overhead, the UE may skip those components or FGs that are common to existing legacy methods. We propose adding the following notes: - Note: if UE supports same values for one or more components as in FG 13-7, then the UE can skip indicating these components in this FG and the values in corresponding FG 13-7 components indicate supported QCL sources for Case 1 - Note: if UE supports same values for one or more components as in FG 13-7a, then the UE can skip indicating these components in this FG and the values in corresponding FG 13-7a components indicate supported QCL sources for Case 1 Proposal 13: Modify the Rel. 19 UE FG of PRS processing capabilities for UE-based positioning Case 1: 58. NR_AIML_Air; 58-2-6; Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1; 1. Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS Note 1: Refers to Type-D support for FR2 Note 2: A PRS from a PRS-only TP is treated as PRS from a non-serving cell; 58-2-4 [ otherwise 13-1]; No; N/A; DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported. DL PRSs are in the same band Note: if UE supports same values for one or more components as in FG 13-7a, then the UE can skip indicating these components in this FG and the values in corresponding FG 13-7a components indicate supported QCL sources for Case 1; Optional with capability signaling. |
| NTT DOCOMO, INC. [15] | Same as FG 58-2-5, FG 58-2-6 follows the legacy capability “Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS,” i.e., prerequisite feature group is FG 58-2-4. The same as FG 58-2-5, FG 13-1 is not necessary. 58-2-6; Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1; 1. Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS Note 1: Refers to Type-D support for FR2 Note 2: A PRS from a PRS-only TP is treated as PRS from a non-serving cell; [58-2-4; otherwise 13-1]; No; N/A; DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported. DL PRSs are in the same band; Optional with capability signaling. |

Others

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] | - Introduce the following Rel. 19 UE FGs for AI/ML based positioning. X. NR_AIML_Air; X-2-1; AI/ML based positioning for UE-side model; 1. Support of AI/ML based positioning Case 1; FFS; N/A; AI/ML based positioning Case 1 is not supported; RAN1 kindly requests RAN2 to decide on the necessity for location server to know if the feature is supported. X. NR_AIML_Air; X-2-2; Support of model monitoring for AI/ML based positioning for UE-side model; 1. Support of performance monitoring for AI/ML based positioning Case 1; FFS; N/A; Performance monitoring of AI/ML based positioning Case 1 is not supported; FFS: The necessity and further partitioning of this FG depend on the future RAN1/2 agreements on the format of model monitoring outcome.. X. NR_AIML_Air; X-2-3; Support of data collection for AI/ML based positioning for UE-side model; 1. Support of Data collection for AI/ML based positioning Case 1 2. Support of Associated ID; FFS; N/A; Data collection of AI/ML based positioning Case 1 is not supported; FFS: The necessity and further partitioning of this FG depends on the future RAN1 agreements on Associated ID. |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] | Support Case 1 for implicit Associated ID For implicit associated ID handling in Case 1, the mechanism enables efficient management of assistance data, particularly for Info #7, which pertains to TRP locations. Unlike legacy methods that rely on explicit geographical coordinates, the implicit approach leverages an associated ID, such as NCGI-r15, to signal consistency in TRP locations without disclosing sensitive location data. This abstraction aligns with privacy considerations while ensuring that the UE can interpret TRP configurations effectively. The associated ID is configured per cell and serves as a streamlined alternative to explicit signaling, reducing complexity in data handling. Regarding the above aspect, we find the following need to be clarified. If UE does not provide FG for implicit associated ID handling in Case 1, then can LMF assume the legacy processing of associated IDs (e.g., explicit signaling of Info #7) applies to Case 1. When the UE receives the same implicit associated ID for a set of TRPs regardless of the time that associated ID being used within a cell area (e.g., represented by AreaID-cellList), which is associated with PRS transmissions, the UE may be able to assume similar physical properties for the set of TRPs. Proposal 1: Introduce the Rel.19 UE FG of assistance data for UE-based positioning Case 1. 58. NR_AIML_Air; 58-2-21; Implicit associated ID indication for UE-based positioning Case 1; Support reception of assistance data containing the implicit associated ID for UE-based positioning Case 1.; 58-2-1; N/A; N/A; Reception of Associated ID for UE-based positioning Case 1 is not supported; Per UE; n/a; n/a; n/a; Need for location server to know if the feature is supported. Note: The Associated ID is used to provide implicit information of Info #7 of the assistance information from legacy UE-based DL-TDOA. Note: For specific ADs, UE indicate supported AD that can be received as in [FFS: FGs]; Optional with capability signalling. Support Case 1 for Monitoring The inclusion of error causes in performance monitoring outcomes is critical for ensuring proper network operations, as emphasized by the RAN1#120-bis and RAN2#130 agreements. The RAN1#120-bis agreement resolves the ambiguity in monitoring outcome content by requiring that it must include an indication when the target UE cannot perform the Case 1 positioning method. This explicit feedback is essential for enabling network operators to identify and address failures proactively, ensuring service reliability and minimizing disruptions. Similarly, the RAN2#130 agreement introduces the error cause ‘DL AIML positioning not available’ for Case 1, which serves to indicate scenarios where the UE cannot perform positioning due to issues such as model unavailability or the absence of valid performance monitoring outcomes. However, challenges arise with the RAN2#130 agreement due to its lack of explicit detail regarding the underlying causes of the error. While the agreement introduces the error cause, it does not specify the conditions or parameters that trigger this designation, leaving ambiguity in its practical implementation. For example, it is unclear whether the error cause should be applied uniformly across all UEs or tailored based on specific device capabilities, network configurations, or operational contexts. This lack of granularity can lead to inconsistent interpretations, making it difficult for network operators to pinpoint the root causes of positioning failures and implement targeted corrective actions. Furthermore, the absence of detailed guidelines for monitoring error causes may hinder the ability to align troubleshooting processes across different network deployments, reducing the overall efficiency of network operations. Proposal 2: Establish a new UE FG for performance monitoring in Case 1 to explicitly define error causes, clarify simultaneous operation capabilities, and enhance network reliability through standardized monitoring mechanisms. 58. NR_AIML_Air; 58-2-22; Support performance monitoring for positioning Case 1; Indicates support of performance monitoring for positioning Case 1; FFS; N/A; N/A; Performance monitoring for positioning Case 1 is not supported; Per UE; n/a; n/a; n/a; FFS: The necessity and further components of this FG depend on the future RAN1/2 agreements on the format of model monitoring outcome.; Optional with capability signalling. |
| CATT/CICTCI [9] |  |
| OPPO [10] | As agreed in previous meetings, the measurements for determining model input are based on DL PRS and UL SRS as in non-AI procedure as in following agreements. Conclusion From RAN1 perspective, for Case 3a measurements, - The existing procedures can be reused in terms of SRS configuration. - Note: parameter values for SRS configuration can be further discussed - These measurements can be used for multiple aspects related to case 3a, e.g. training data collection, monitoring, or inference procedures. - Note: Purpose, such as the training data collection, monitoring, or inference procedures mentioned above, will not necessarily be specified in RAN 1 specifications Agreement For Rel-19 AI/ML based positioning, the measurements for determining model input are based on the DL PRS and UL SRS defined in TS38.211. - Note: The use of SRS for MIMO resource is transparent to UE.. Therefore, we suggest to reuse capability signalling as legacy methods. Proposal 2: Support information related to UL SRS as in the capability signaling of legacy methods, detailed capability value can be further discussed if needed. 58-2-6; SRS Resources for AI Positioning Case 3a/3b; 1. Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. 2. Max number of P/SP/AP SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of P/SP/AP SRS Resources including the SRS resources for positioning per BWP per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Note: Max number of P/SP/AP SRS Resources in Component 3 include both SRS resources configured by SRS-Resource and SRS resources configured by SRS-PosResource-r16 supported by UE 4. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 5. Max number of periodic SRS Resources for positioning per BWP per slot. Values = {1,2,3,4,5,6,8,10,12,14} OLPC for SRS for positioning based on SSB from serving cell is part of FG13-8 Note: no dedicated capability signaling is intended for this component; n/a; n/a; Note: if the UE does not indicate this capability for a band in a band combination, the UE does not support SRS for Positioning in this band in the band combination.; Optional with capability signaling. 58-2-7; Support of Aperiodic SRS Resources for AI positioning Case 3a/3b; 1. Max number of aperiodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 2. Max number of aperiodic SRS Resources for positioning per BWP per slot. Values = {1,2,3,4,5,6,8,10,12,14}; 58-2-6; n/a; n/a; Optional with capability signaling. 58-2-8; Support of Semi-persistent SRS Resources for AI positioning Case 3a/3b; 1. Max number of semi-persistent SRS Resources for positioning supported by UE per BWP. Values = {1,2,4,8,16,32,64} 2. Max number of semi-persistent SRS Resources for positioning supported by UE per BWP per slot. Values = {1,2,3,4,5,6,8,10,12,14}; 58-2-6; n/a; n/a; Optional with capability signaling. 58-2-9; SRS Resources for AI Positioning Case 3a/3b; 1. Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. 2. Max number of P/SP/AP SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64}; 58-2-6; n/a; n/a; Need for location server to know if the feature is supported UE only reports the number on bands for the current configured CA band combination; Optional with capability signaling. 58-2-10; Support of Aperiodic SRS Resources for AI positioning Case 3a/3b; 1. Max number of aperiodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64}; 58-2-7, 58-2-9; Need for location server to know if the feature is supported. UE only reports the number on bands for the current configured CA band combination.; Optional with capability signaling. 58-2-11; Support of Semi-persistent SRS Resources for AI positioning Case 3a/3b; 1. Max number of semi-persistent SRS Resources for positioning supported by UE per BWP. Values = {1,2,4,8,16,32,64}; 58-2-6, 58-2-9; n/a; n/a; Need for location server to know if the feature is supported. UE only reports the number on bands for the current configured CA band combination; Optional with capability signaling. |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] | Proposal 2-2: - For Case 1, From the Rel-17 UE feature list for NR Positioning FG 27-10a and 27-11 are applicable AI/ML-based positioning. A Case 1 specific FG should also be defined for each of these FGs. Old; New; Description. 27-10a; 58-x-y; Low latency MG activation request for PRS measurements. 27-11; 58-x-y; Support of DL MAC CE based MG activation for PRS measurements. |
| Qualcomm Incorporated [14] | One missing important feature is the maximum number of PFLs that UE can support for Case 1. We propose to introduce a corresponding FG to let UE indicate this. Proposal 7: Introduce the following Rel. 19 UE FGs for UE-based positioning Case 1: 58. NR_AIML_Air; 58-2-4a; Max number of positioning frequency layers UE supports all bands for UE-based for positioning Case 1; Max number of positioning frequency layers UE supports across all bands for UE-based positioning Case-1 Values: {1, 2, 3, 4}; N/A; N/A; Maximum number of PFLs for UE-based positioning Case 1 is not supported; FFS; FFS; FFS; FFS; Need for location server to know if the feature is supported.; Optional with capability signalling. For AI/ML Case 1, UE can also indicate whether it supports AI/ML positioning in IDLE or INACTIVE states along with corresponding PRS resource and processing features. We propose the following changes to existing FGs and new FGs: - Support of PRS measurement in RRC_INACTIVE state for UE-based positioning Case 1 - DL PRS processing capabilities in RRC inactive state for UE-based positioning Case 1 - Support of PRS measurement in RRC_IDLE for UE-based positioning Case 1 There was a confusion in the last meeting about whether Case 1 can be supported in IDLE or INACTIVE states. This confusion is probably due to an old discussion in Rel-18. RAN1 provided RAN2 LS on expected data collection size when considering data collection for training as part of RAN1#118 study item. RAN1 intention was that data collection during IDLE or INACTIVE may not be considered. However, the inference operation can still function during CONNECTED, IDLE, or INACTIVE states. The UE should not be expected to support same capabilities for Case 1 during INACTIVE or IDLE states as in the CONNECTED state. This is because AI/ML model and related processing can be different between these states. Equivalent treatment has been also considered for legacy methods. Capabilities of legacy methods in the IDLE or INACTIVE states are not necessarily same as those of CONNECTED state and that is the reason they already have separate indications. Therefore, equivalent treatment is needed for Case 1. For similarity between components between Case1 and legacy methods, same design can be applied as in other FGs to optimize UE indication overhead. If UE supports component values same to common values of existing legacy methods, then UE can skip indicating those component(s). It is beneficial to indicate the UE can skip them and those indicated in other corresponding FGs become the baseline. We propose adding a note to indicate it (see newly proposed FG 58-2-13 below). Proposal 8: Introduce the following Rel. 19 UE FGs for UE-based positioning Case 1: 58. NR_AIML_Air; 58-2-12; Support of UE-based positioning Case 1 in RRC_INACTIVE state; UE-based positioning Case 1 in RRC_INACTIVE state is supported; 58-2-3 and 58-2-13; N/A; N/A; UE-based positioning Case 1 in RRC_INACTIVE state is not supported; Per Band; FFS; FFS; FFS; Support of UE-based positioning Case 1 in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state; Optional with capability signalling. 58. NR_AIML_Air; 58-2-13; DL PRS processing capabilities in RRC inactive state for UE-based positioning Case 1; 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE 3. Max number of DL PRS resources that UE can process in a slot; 58-2-4; N/A; N/A; Per band; FFS; FFS; FFS; Component 1 candidate values: {Type 1, Type 2} Component 2 candidate values: T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: Having the PRS processing capabilities in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state Note: if UE supports same values for one or more components as in FG 27-6, then the UE can skip indicating these components in this FG and the values in corresponding FG 27-6 components indicate supported PRS processing capabilities in RRC inactive state for Case 1; Optional with capability signalling. 58. NR_AIML_Air; 58-2-14; Support of UE-based positioning Case 1 in RRC_IDLE; Support of UE-based positioning Case 1 in RRC_IDLE; 58-2-4, 58-2-12, 58-2-13; N/A; N/A; UE-based positioning Case 1 in RRC_IDLE is not supported; Per Band; FFS; FFS; FFS; Need for location server to know if the feature is supported; Optional with capability signalling. RAN1 agreed to support assistance data for PRS/PRS bandwidth aggregation similar to UE-based DL-TdoA. Therefore, it is expected that their corresponding UE features that are provided for UE-based DL-TdoA need to be stated for Case 1. We propose to introduce the following FGs corresponding to PRS bandwidth aggregation features for Case 1 when in INACTIVE or IDLE modes: - DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1 - DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1 - PRS bandwidth aggregation in RRC_ INACTIVE UE-based positioning Case 1 - PRS bandwidth aggregation in RRC_IDLE - UE-based positioning Case 1 In addition, if UE support component values same as common values of existing legacy methods (when applicable), then UE can skip indicating those component(s). It is beneficial to indicate the UE can skip them and those indicated in other corresponding FGs become the baseline. We propose adding a note to indicate it (see newly proposed FGs 58-2-15b/15c below). Proposal 9: Introduce the following Rel. 19 UE FGs for UE-based positioning Case 1: 58. NR_AIML_Air; 58-2-15b; DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1; 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Maximum number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot; 58-2-13; N/A; N/A; FFS; Per Band; FFS; FFS; FFS; Component 1 candidate values: a) FR1 bands: {10, 20, 40, 50, 80, 100, 160, 200} b) FR2 bands: {100, 200, 400, 800} Component 2 candidate values:a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in FG 58-2-15b (this FG) follows buffering capability type reported in FG 58-2-4 Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by FG 58-2-13 or this value T should be equal or larger than the value T reported by FG 58-2-13 Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each two linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by FG 58-2-13 Note: if UE supports same values for one or more components as in FG 41-4-1b, then the UE can skip indicating these components in this FG and the values in corresponding FG 41-4-1b components indicate supported for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE capabilities for Case 1; Optional with capability signalling. 58. NR_AIML_Air; 58-2-15c; DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1; 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Max number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot under it; 58-2-15b; N/A; N/A; FFS; Per Band; FFS; FFS; FFS; Component 1 candidate values: a) FR1 bands: {15, 20, 30, 40, 50, 60, 80, 100, 120, 140, 150, 160, 180, 200, 240, 300} b) FR2 bands: {150, 200, 300, 400, 600, 800, 1000, 1200} Component 2 candidate values: a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in FG 58-2-15c (this FG) follows buffering capability type reported in FG 58-2-4 Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by FG 58-2-13 or this value T should be equal or larger than the value T reported by FG 58-2-13 Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each three linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by FG 58-2-13 Note: if UE supports same values for one or more components as in FG 41-4-1c, then the UE can skip indicating these components in this FG and the values in corresponding FG 41-4-1c components indicate supported aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE for Case 1; Optional with capability signalling. 58. NR_AIML_Air; 58-2-18; PRS bandwidth aggregation in RRC_ INACTIVE UE-based positioning Case 1; Support of PRS bandwidth aggregation in RRC_ INACTIVE for UE-based positioning Case 1; 58-2-12, 58-2-15b; N/A; N/A; PRS bandwidth aggregation in RRC_ INACTIVE for UE-based positioning Case 1 is not supported; Per Band; FFS; FFS; FFS; Need for location server to know if the feature is supported.; Optional with capability signalling. 58. NR_AIML_Air; 58-2-19; PRS bandwidth aggregation in RRC_IDLE - UE-based positioning Case 1; Support of PRS bandwidth aggregation in RRC_IDLE for UE-based positioning Case 1; 58-2-14, 58-2-15b; N/A; N/A; PRS bandwidth aggregation in RRC_IDLE for UE-based positioning Case 1 is not supported; Per Band; FFS; FFS; FFS; Need for location server to know if the feature is supported.; Optional with capability signalling. For AI/ML Case 1, UE can also indicate whether it supports low latency MG activation, similar to other legacy positioning methods. We propose the following change to existing FG: Proposal 10: Introduce the following change for UE-based positioning Case 1: 27. NR_pos_enh; 27-10a; Low latency MG activation request for PRS measurements; support of low latency MG activation request for PRS measurements; 27-10, 27-11; mg-ActivationRequest-r17; LPP NR-DL-TDOA-ProvideCapabilities-r16 NR-DL-AoD-ProvideCapabilities-r16 NR-Multi-RTT-ProvideCapabilities-r16 NR-UE-based-PositioningCase1-ProvideCapabilities-r19; No; No; Need for location server to know if the feature is supported Note: RAN1 understands that FG 27-10a is intended only for the LMF to know, and that the current prerequisite FGs of FG 27-10a are capabilities only for the gNB to know. It is up to RAN2 to decide whether such a FG dependency is meaningful from signaling description perspective, and whether and how it can be captured in RAN2 specifications.; Optional with capability signaling. RAN1 agreed to support assistance data for LOS/NLOS indicator AD as in UE-based DL-TdoA. Therefore, it is expected that their corresponding RAN1 UE features to be provided for Case 1. We propose to modify the FG and include Case 1, as follows: Proposal 11: Introduce the following change for UE-based positioning Case 1: 27. NR_pos_enh; 27-12; LOS/NLOS indicator for UE-based positioning assistance data; Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type 2. LOS/NLOS indicator granularity; nr-los-nlos-IndicatorSupport-r17; NR-DL-TDOA-ProvideCapabilities-r16 NR-DL-AoD-ProvideCapabilities-r16 NR-Multi-RTT-ProvideCapabilities-r16 [NR-Case1-ProvideCapabilities]; No; No; Component 1 candidate values: {hardValue+softValue, hardValue} Component 2 candidate values: {resourceSpecific, trpSpecific} Need for location server to know if the feature is supported.; Optional with capability signaling.. |
| NTT DOCOMO, INC. [15] | At the last meeting, Rel-17/18 features for RRC_CONNECTED state for Case 1 were introduced (e.g., FG 58-2-15, 58-2-17) and UE features RRC_INACTIVE state and RRC_IDLE state for Case 1 were proposed. In our understanding, there is no agreement to support AI/ML based positioning for Case 1 only in RRC_CONNECTED state. Therefore, UE features for other RRC states for Case 1 should be introduced. Proposal 12: The following UE feature group for RRC_INACTIVE state and RRC_IDLE state should be supported. - FG 58-2-12: Support of UE-based positioning Case 1 in RRC_INACTIVE state - FG 58-2-13: DL PRS processing capabilities in RRC inactive state for UE-based positioning Case 1 - FG 58-2-14: Support of UE-based positioning Case 1 in RRC_IDLE - FG 58-2-15b: DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1 - FG 58-2-15c: DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1 - FG 58-2-18: PRS bandwidth aggregation in RRC_ INACTIVE UE-based positioning Case 1 - FG 58-2-19: PRS bandwidth aggregation in RRC_IDLE - UE-based positioning Case 1 58-2-12; Support of UE-based positioning Case 1 in RRC_INACTIVE state; UE-based positioning Case 1 in RRC_INACTIVE state is supported; [58-2-3 and 58-2-13]; N/A; N/A; UE-based positioning Case 1 in RRC_INACTIVE state is not supported; Per Band; n/a; n/a; n/a; Support of UE-based positioning Case 1 in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state; Optional with capability signalling. 58-2-13; DL PRS processing capabilities in RRC inactive state for UE-based positioning Case 1; 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE 3. Max number of DL PRS resources that UE can process in a slot; [58-2-4]; N/A; N/A; Per band; n/a; n/a; n/a; Component 1 candidate values: {Type 1, Type 2} Component 2 candidate values: T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: Having the PRS processing capabilities in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state Note: if UE supports same values for one or more components as in FG 27-6, then the UE can skip indicating these components in this FG and the values in corresponding FG 27-6 components indicate supported PRS processing capabilities in RRC inactive state for Case 1; Optional with capability signalling. 58-2-14; Support of UE-based positioning Case 1 in RRC_IDLE; Support of UE-based positioning Case 1 in RRC_IDLE; [ 58-2-4, 58-2-12, 58-2-13]; N/A; N/A; UE-based positioning Case 1 in RRC_IDLE is not supported; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported; Optional with capability signalling. 58-2-15b; DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1; 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Maximum number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot; [58-2-13]; N/A; N/A; DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1 is not supported; Per Band; n/a; n/a; n/a; Component 1 candidate values: a) FR1 bands: {10, 20, 40, 50, 80, 100, 160, 200} b) FR2 bands: {100, 200, 400, 800} Component 2 candidate values:a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in [FG 58-2-15b (this FG)] follows buffering capability type reported in [FG 58-2-4] Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by [FG 58-2-13] or this value T should be equal or larger than the value T reported by [FG 58-2-13] Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each two linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by [FG 58-2-13] Note: if UE supports same values for one or more components as in FG 41-4-1b, then the UE can skip indicating these components in this FG and the values in corresponding FG 41-4-1b components indicate supported for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE capabilities for Case 1; Optional with capability signalling. 58-2-15c; DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1; 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Max number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot under it; [58-2-15b]; N/A; N/A; DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1 is not suported; Per Band; n/a; n/a; n/a; Component 1 candidate values: a) FR1 bands: {15, 20, 30, 40, 50, 60, 80, 100, 120, 140, 150, 160, 180, 200, 240, 300} b) FR2 bands: {150, 200, 300, 400, 600, 800, 1000, 1200} Component 2 candidate values: a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in [FG 58-2-15c (this FG)] follows buffering capability type reported in [FG 58-2-4] Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by [FG 58-2-13] or this value T should be equal or larger than the value T reported by [FG 58-2-13] Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each three linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by [FG 58-2-13] Note: if UE supports same values for one or more components as in FG 41-4-1c, then the UE can skip indicating these components in this FG and the values in corresponding FG 41-4-1c components indicate supported aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE for Case 1; Optional with capability signalling. 58-2-18; PRS bandwidth aggregation in RRC_ INACTIVE UE-based positioning Case 1; Support of PRS bandwidth aggregation in RRC_ INACTIVE for UE-based positioning Case 1; [58-2-12, 58-2-15b]; N/A; N/A; PRS bandwidth aggregation in RRC_ INACTIVE for UE-based positioning Case 1 is not supported; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signalling. 58-2-19; PRS bandwidth aggregation in RRC_IDLE - UE-based positioning Case 1; Support of PRS bandwidth aggregation in RRC_IDLE for UE-based positioning Case 1; [58-2-14, 58-2-15b]; N/A; N/A; PRS bandwidth aggregation in RRC_IDLE for UE-based positioning Case 1 is not supported; Per Band; n/a; n/a; n/a; Need for location server to know if the feature is supported.; Optional with capability signalling. |

## Specification support for CSI prediction

| 58. NR_AIML_Air | 58-3-1 | CSI prediction for UE-sided inference when N4=1 | 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 11. Scaling factor for active resource counting Kp 12. supported value of t for the relaxation of Z and Z’ timeline 13. supported number of occupied CPU 14. supported number of occupied APU 15. Index of the occupied APU pool | 58-0-1 | yes | n/a | CSI prediction for N4=1 for inference is not supported | Per band and Per BC | n/a | n/a | n/a | Component 3 candidate values: a. {4,8,12,16,24,32} b. {2,3,4 … 64} c. {4, …, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] | To reduce the complexity for NW to handle many different combinations of PU occupancies, a limited number of values should be supported. For inference report, introducing positive values of t implies that AI based CSI prediction requires longer CSI processing time than legacy Rel-18 CSI prediction, which makes this AI feature less useful. In RAN1 discussions, companies have indicated that shortened inference latency can be achieved compared to non-AI based scheme if using dedicated AI hardware and high degree of parallelization for AI based CSI processing (e.g., matrix operations). Hence, negative values of t shall be supported. - Add the following components to the basic feature group 58-3-1 for Rel-19 CSI prediction using UE-sided model: - Limit the number of candidate values for APU occupation, - Limit the number of candidate values for CPU occupation, - Limit the number of candidate values for the relaxation of Z and Z’ timeline. At least negative value of t shall be supported. |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] | 58. NR_AIML_Air; 58-3-1; CSI prediction for UE-sided inference when N4=1; 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) [9. Value for CPU occupation, when P/SP-CSI-RS is configured for CMR] [10. Value for CPU occupation, when A-CSI-RS is configured for CMR] 11. Scaling factor for active resource counting Kp 12. Supported values of the maximum number of observation number; 2-35; yes; CSI prediction for N4=1 for inference is not supported; Component 9 and10 candidate values: FFS Component 12 candidate values: FFS. |
| Huawei/HiSilicon [5] | - Component 11: Candidate values refer to the Kp values of the legacy Rel-18 Doppler codebook. - Component 12: Similar to BM case, the maximum inference delay could be ms level. Moreover, since the legacy Rel-18 Doppler codebook has already introduced additional prediction latency, t=0 for AI/ML CSI prediction should also be considered, if the AI/ML based prediction latency is no larger than legacy CSI prediction. - Component 13: Different from BM, UE may need more CPU to perform PMI related inference. In legacy, the maximum value of CPU pool is 8. Thus, we think {0, 1, ..., 8} can considered as candidate values for the number of occupied CPU. - Component 14: Similar to BM Case, {0, 1, 2} can considered as candidate values for the number of occupied APU. - Component 15: Candidate values include Index 1 to represent the first APU pool and Index 2 to represent the second APU pool. - Similar to 58-1-2, a note is added for CPU and APU to align with the notations in TS 38.214. 58. NR_AIML_Air; 58-3-1; CSI prediction for UE-sided inference when N4=1; 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 11. Scaling factor for active resource counting Kp 12. supported value of ti for the relaxation of Z and Z’ timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 13. supported number of occupied CPU 14. supported number of occupied APU 15. Index of the occupied APU pool; 58-0-1; yes; n/a; CSI prediction for N4=1 for inference is not supported; Per band and Per BC; Component 3 candidate values: a. {4,8,12,16,24,32} b. {2,3,4 … 64} c. {4, …, 256} Component 11 candidate values: {1,2,4} Component 12 candidate values: d1 is {0, 2, 4, 8, 14} d2 is {0, 4, 8, 14, 28} d3 is {0, 8,14, 28, 56} d4 is {0, 14, 28, 56, 112} d5 is {0, 56, 112, 224, 448} d6 is {0, 112, 224, 448, 896} Component 13 candidate values: {0, …, 8} Component 14 candidate values: {0, 1, 2} Component 15 candidate values: {1, 2} representing the first APU pool (i.e., CPU,2) and the second APU pool (i.e., CPU,3), respectively. Note: “CPU” corresponds to “CPU,1” in TS 38.214, and “APU” corresponds to “CPU,x” in TS 38.214, x = 2, 3.; Optional with capability signaling. |
| Xiaomi [6] |  |
| CMCC [7] | For FG 59-3-1, there is an FFS part for CPU occupation, and we have the following agreement in last meeting [2] for that: [121] Agreement For CSI prediction using UE-side model, to calculate the inference report using Doppler codebook, - For PU occupancy, support - Dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied, - OAPU= 0 or N is reported by UE - OCPU=0 or M is reported by UE - Note: Detailed values of N and M can be further discussed in UE feature. - Note: Combination of OAPU= 0 and OCPU=0 is not allowed - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behaviour of exceeding the CPU limit, neither of the PUs are occupied - For occupancy duration of CPU and APU, - the same occupancy duration is used if both CPU and APU are reported non-zero value - if associated monitoring report is not configured, reuse following legacy occupancy duration - For semi-persistent CSI report on PUSCH with P/SP CSIRS CMR, occupation starts from the first symbol of KP-th latest consecutive P/SP-CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where KP is indicated by UE capability. - Note: Detailed values of KP can be further discussed in UE feature. - Aperiodic CSI report occupies PU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. - The total number of dedicated AI/ML PU for AI/ML is reported by UE capability. It is supported to make UE report the number of APU and CPU for AI/ML based CSI prediction, but it is not related with the time domain property. And combination of OAPU= 0 and OCPU=0 is not allowed. So, we think component 9 and 10 is basically needed with the following modifications: - 9. Value for APU occupation - 10. Value for CPU occupation As for the candidate value of APU and CPU, at least value 0 should be included. Besides, now that we support to include component 11 in FG 59-3-1, which can be seen as number of observation number, then the component 8 of FG 59-3-2 is not needed since the prerequisite of FG 59-3-2 is FG 59-3-1. Based on the above analyses, we suggest taking the following modifications (in red) for the feature of Rel-19 AI based CSI prediction. Proposal 6: Take the following modification (in red) for the feature of Rel-19 AI based CSI prediction: 58. NR_AIML_Air; 58-3-1; CSI prediction for UE-sided inference when N4=1; 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) [9. Value for APU occupation, when P/SP-CSI-RS is configured for CMR] [10. Value for CPU occupation, when A-CSI-RS is configured for CMR] 11. Scaling factor for active resource counting Kp; 2-35; yes; n/a; CSI prediction for N4=1 for inference is not supported; [Per band and Per BC]; n/a; n/a; n/a; Candidate value of number of APU and CPU: at least value 0 is supported Note: Combination of OAPU= 0 and OCPU=0 is not allowed; Optional with capability signalling. |
| Nokia [8] | 58. NR_AIML_Air; 58-3-1; CSI prediction for UE-sided inference when N4=1; 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 11. Scaling factor for active resource counting Kp 12. supported value of t for the relaxation of Z and Z’ timeline 13. supported number of occupied CPU 14. supported number of occupied APU 15. Index of the occupied APU pool; 2-35; yes; n/a; CSI prediction for N4=1 for inference is not supported; Per band and Per BC; n/a; n/a; n/a; Component 3 candidate values: a. {4,8,12,16,24,32} b. {2,3,4 … 64} c. {4, …, 256}; Optional with capability signalling. |
| CATT/CICTCI [9] |  |
| OPPO [10] | Regarding the 58-3-1 FG of CSI prediction on UE-sided inference when N4=1: - Component 9/10: besides CPU occupation, AI/ML PU occupation when P/SP/AP is configured for CMR should also be considered 58. NR_AIML_Air; 58-3-1; CSI prediction for UE-sided inference when N4=1; 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 9. Value for CPU/AIML PU occupation, when P/SP-CSI-RS is configured for CMR 10. Value for CPU/AIML PU occupation, when A-CSI-RS is configured for CMR] 11. Scaling factor for active resource counting Kp; 2-35; yes; n/a; CSI prediction for N4=1 for inference is not supported; [Per band and Per BC]; n/a; n/a; n/a; Candidate values for CPU/AIML PU: FFS; Optional with capability signalling. |
| Samsung [11] | Unlink CPU, the number of occupied APUs for CSI prediction does not linearly scale with the number of measured CSI-RS resources (K) or the number of prediction slot intervals (N4). Thus, it is better for the UE to report a single candidate value for the number of occupied APU irrespective of the values for K and N4. Thus we propose the following Proposal 5: For component 14 of 58-3-1 and component 8 of 58-3-1, support the candidate values {1,2} for the occupied number of CPUs for performance monitoring for CSI prediction model 58. NR_AIML_Air; 58-3-1; CSI prediction for UE-sided inference when N4=1; 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) [9. Value for CPU occupation, when P/SP-CSI-RS is configured for CMR] [10. Value for CPU occupation, when A-CSI-RS is configured for CMR] 11. Scaling factor for active resource counting Kp 12. supported value of t for the relaxation of Z and Z’ timeline 13. supported number of occupied CPU 14. supported number of occupied APU 15. Index of the occupied APU pool; 2-35 58-0-1; yes; n/a; CSI prediction for N4=1 for inference is not supported; Component 14: candidate values {1,2,}; Optional with capability signalling. |
| LG Electronics [12] | 58. NR_AIML_Air; 58-3-1; CSI prediction for UE-sided inference when N4=1; 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 911. Scaling factor for active resource counting Kp 1012. supported value of t for the relaxation of Z and Z’ timeline 1113. supported number of occupied CPU 1214. supported number of occupied APU 1315. Index of the occupied APU pool; 58-0-1; yes; n/a; CSI prediction for N4=1 for inference is not supported; Per band and Per BC; n/a; n/a; n/a; Component 3 candidate values: a. {4,8,12,16,24,32} b. {2,3,4 … 64} c. {4, …, 256} [Component 9 candidate values: {1, 2, 4}] [Component 11 candidate values: {1, 2,4,8}] [Component 12 candidate values: {1, 2,4,8}] Component 13 candidate values: {1, 2}; Optional with capability signalling. |
| Apple [13] | Proposal 3-2: For the UE feature group 58-3-1 with N4=1 the proposed candidate values are - Component 12: supported value of t for the relaxation of Z and Z’ timeline: 2ms, 3ms and 4ms. - Component 13: supported number of occupied CPU: 0, 1, 2, …. 8 - Component 14: supported number of occupied APU: 0, 1, 2, …, 8 - Component 15: Index of the occupied APU pool: {1, 2} Proposal 3-3: For the UE feature group 58-3-1 with N4>1 the proposed candidate values are - Component 12: supported value of t for the relaxation of Z and Z’ timeline: 2ms, 3ms and 4ms. - Component 13: supported number of occupied CPU: 0, 1, 2, …. 8 - Component 14: supported number of occupied APU: 0, 1, 2, …. 8 - Component 15: Index of the occupied APU pool: {1,2} |
| Qualcomm Incorporated [14] | Adopt the O_CPU and O_APU candidate values as O_CPU={1…8}, O_APU={0…8} in FG58-3-1 and FG58-3-2 Adopt FG2-35 as the pre-requisite FG for FG58-0-1 (APU), FG58-3-1 (CSI prediction inference) and FG58-3-4 (data collection) UE reporting max 16 CSI-RS triplets reported for FG58-3-1, FG58-3-2, FG58-3-4 and FG58-3-5 |
| NTT DOCOMO, INC. [15] | At RAN1 #122 meeting, the issue about the terminology of APU or CPU,2/CPU,3 was discussed. Based on the discussions, there is no need to change the CPU,2/CPU,3 in the current specification. It is better to use the aligned terminology with the specification for the UE feature discussions. Therefore, we propose changing all APUs occurring in FG 58 to the term aligned with the specification. Proposal 1: Revise the following FGs about the usage of APU and change the APU and corresponding items to align with the term CPU,2/CPU,3. - FG 58-0-1, FG 58-1-2, FG 58-1-4, FG 58-3-1, FG 58-3-2. As proposed in Proposal 1, the APU issue should also be solved for this FG. According to the discussions of the last meeting, FG 58-0-1 will not be signalled if UE has no processing resource pool dedicated to AI/ML. Therefore, FG 58-0-1 cannot be the prerequisite of FG 58-3-1. Instead, FG 2-35 (CSI framework) can be used. 58. NR_AIML_Air; 58-3-1; CSI prediction for UE-sided inference when N4=1; 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 11. Scaling factor for active resource counting Kp 12. supported value of t for the relaxation of Z and Z’ timeline 13. supported number of occupied CPU 14. Supported number of occupied APU CPU,2/CPU,3 15. Occupied resource pool between CPU,2 and CPU,3APU pool; 2-35 58-0-1; yes; n/a; CSI prediction for N4=1 for inference is not supported; Per band and Per BC; n/a; n/a; n/a; Component 3 candidate values: a. {4,8,12,16,24,32} b. {2,3,4 … 64} c. {4, …, 256} Component 14 candidate values: {0, 2, 4, 8} Component 15 candidate values: {CPU,2 or CPU,3} Note: UE should not report non-zero value for Component 14 if FG 58-0-1 is not signalled.; Optional with capability signalling. |

| 58. NR_AIML_Air | 58-3-1a-1 | DD unit size when A-CSI-RS is configured for CMR N4>1 for UE side inference of CSI prediction | Value of d=1 for the DD unit size when A-CSI-RS for inference is configured for CMR | 58-3-1 | Yes | N/A | DD unit size d=1 is not supported when A-CSI-RS is configured for CMR N4>1 for UE side inference of CSI prediction | Per-band and Per-BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-3-1b | Maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for Rel-16-based doppler measurement for UE side inference of CSI prediction | 1. Maximum number of aperiodic CSI-RS resources for inference that can be configured in the same CSI report setting for Rel-16-based doppler measurement | 58-3-1 | Yes | N/A | Maximum number of aperiodic CSI-RS resources that can be configured in the same CSI report setting for Rel-16-based doppler measurement for UE side inference of CSI prediction is not known | Per-band and Per-BC | N/A | N/A | N/A | Component 1 candidate values: {4, 8, 12} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-3-1-2 | Support R=2 for Rel-16-based doppler codebook for UE side inference of CSI prediction | A list of supported combinations {Max # of Tx ports in one resource for inference, Max # of resources for inference and total # of Tx ports}, across all CCs simultaneously, with R=2 | 58-3-1 | Yes | N/A | R=2 for Rel-16-based doppler codebook for UE side inference of CSI prediction is not supported | Per band and per BC | N/A | N/A | N/A | Candidate values for component 1: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-3-1-3 | Support X=1 based on first and last slot of WCSI, for Rel-16-based doppler codebook for UE side inference of CSI prediction | Support of TDCQI = ‘1-2’ | 58-3-1 | Yes | N/A | X=1 based on first and last slot of WCSI, for Rel-16-based doppler codebook for UE side inference of CSI prediction is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-3-1-3a | Support X=2 CQI based on 2 slots for Rel-16-based doppler codebook for UE side inference of CSI prediction | Support of TDCQI = ‘2’ | 58-3-1 | Yes | N/A | X=2 CQI based on 2 slots for Rel-16-based doppler for UE side inference of CSI prediction is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-3-1-4 | support of l = (n – nCSI,ref ) for CSI reference slot for Rel-16 based doppler codebook for UE side inference of CSI prediction | 1. Support of l = (n – nCSI,ref ) for CSI reference slot when N4=1 and d>1 2. Support of l = (n – nCSI,ref ) for CSI reference slot when N4>1 | 58-3-1 | Yes | N/A | l = (n – nCSI,ref ) for CSI reference slot for Rel-16 based doppler codebook for UE side inference of CSI prediction is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-3-1-5 | Support of L=6 for Rel-16 based doppler codebook for UE side inference of CSI prediction | Support of L=6 for Rel-16 based doppler codebook | 58-3-1 | Yes | N/A | L=6 for Rel-16-based doppler codebook for UE side inference of CSI prediction is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| X. NR_AIML_Air | 58-3-1-6 | Support of rank equals 3 and 4 for Rel-16 based doppler codebook for UE side inference of CSI prediction | Support of rank equals 3 and 4 for Rel-16 based doppler codebook | 58-3-1 | Yes | N/A | Rank equals 3 and 4 for Rel-16-based doppler codebook for UE side inference of CSI prediction is not supported | Per band and per BC | N/A | N/A | N/A |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-3-1-7 | Active CSI-RS resources and ports for mixed R16 based doppler codebook for CSI prediction via UE side model with other codebooks in any slot | 1. List of codebook combinations of two types 2. List of {max number of ports per resource, max number of resources, max number of total ports} for each codebook combination | 23-9-1, 16-3a, 2-36, 2-40, 2-41, 23-9-2, 23-9-4, 40-3-2-1, 40-3-2-1a, 40-3-2-2, X-1-1, X-1-1a, X-1-2 | Yes | n/a | Active CSI-RS resources and ports for mixed R16 based doppler codebook for CSI prediction via UE side model with other codebooks in any slot is not supported | Per band and Per BC | n/a | n/a | n/a | Component 1 candidate values: {Type I SP, CSI prediction for UE-sided inference when N4=1 and R=1} {Type I SP, CSI prediction for UE-sided inference when N4>1 and R=1} {eType II R=1, CSI prediction for UE-sided inference when N4=1 and R=1} {eType II R=1, CSI prediction for UE-sided inference when N4>1 and R=1} Component 2 candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Note: if a UE reports one or more codebook combinations in 58-3-1-7, then usage of active CSI-RS resources and ports for multiple codebooks in any slot is allowed only within those combinations Note: For coexisting of mixed codebooks in any slot, gNB need to honor 16-8, 23-9-5, 58-3-1-7 and per-codebook capability 2-36/40/41, 16-3a, and 23-9-1/23-9-2/23-9-4. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

| 58. NR_AIML_Air | 58-3-2 | CSI prediction for UE-sided inference when N4>1 | 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 5. Support for the size of DD-basis, N4>1 6. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting 7. Supported number of occupied CPU 8. Supported number of occupied APU 9. Index of the occupied APU pool | 58-3-1 | yes | n/a | CSI prediction for N4>1 for inference is not supported | Per band and Per BC | N/A | N/A | N/A | Component 3 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {2,3,4 … 64} d. {4, …, 256} Component 6 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {4,8,12} d.{4, …, 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] | 58. NR_AIML_Air; 58-3-2; CSI prediction for UE-sided inference when N4>1; 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 5. Support for the size of DD-basis, N4>1 7. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting [8. Supported values of the maximum number of observation number]; 58-3-1; yes; CSI prediction for N4>1 for inference is not supported; Component 8 candidate values: FFS. |
| Huawei/HiSilicon [5] | - Component 7/8: Same as analyzed for Component 13/14 of 58-3-1. - Similar to 58-1-2, a note is added for CPU and APU to align with the notations in TS 38.214. 58. NR_AIML_Air; 58-3-2; CSI prediction for UE-sided inference when N4>1; 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 5. Support for the size of DD-basis, N4>1 6. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting 7. Supported number of occupied CPU 8. Supported number of occupied APU 9. Index of the occupied APU pool; 58-3-1; yes; n/a; CSI prediction for N4>1 for inference is not supported; Per band and Per BC; Component 3 Candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {2,3,4 … 64} d. {4, …, 256} Component 6 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {4,8,12} d.{4, …, 256} Component 7 candidate values: {0, …, 8} Component 8 candidate values: {0, 1, 2} Component 9 candidate values: {1, 2} representing the first APU pool (i.e., CPU,2) and the second APU pool (i.e., CPU,3), respectively. Note: “CPU” corresponds to “CPU,1” in TS 38.214, and “APU” corresponds to “CPU,x” in TS 38.214, x = 2, 3.; Optional with capability signaling. |
| Xiaomi [6] |  |
| CMCC [7] | For FG 59-3-1, there is an FFS part for CPU occupation, and we have the following agreement in last meeting [2] for that: [121] Agreement For CSI prediction using UE-side model, to calculate the inference report using Doppler codebook, - For PU occupancy, support - Dedicated AI/ML PU (OAPU) and/or legacy CPU (OCPU) are occupied, - OAPU= 0 or N is reported by UE - OCPU=0 or M is reported by UE - Note: Detailed values of N and M can be further discussed in UE feature. - Note: Combination of OAPU= 0 and OCPU=0 is not allowed - Note: if any of the unoccupied PU cannot satisfy the corresponding required PU by the CSI report, the CSI report will follow the legacy behaviour of exceeding the CPU limit, neither of the PUs are occupied - For occupancy duration of CPU and APU, - the same occupancy duration is used if both CPU and APU are reported non-zero value - if associated monitoring report is not configured, reuse following legacy occupancy duration - For semi-persistent CSI report on PUSCH with P/SP CSIRS CMR, occupation starts from the first symbol of KP-th latest consecutive P/SP-CSI-RS occasions no later than CSI reference resource, until the last symbol of the PUSCH carrying the report, where KP is indicated by UE capability. - Note: Detailed values of KP can be further discussed in UE feature. - Aperiodic CSI report occupies PU(s) from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the report. - The total number of dedicated AI/ML PU for AI/ML is reported by UE capability. It is supported to make UE report the number of APU and CPU for AI/ML based CSI prediction, but it is not related with the time domain property. And combination of OAPU= 0 and OCPU=0 is not allowed. So, we think component 9 and 10 is basically needed with the following modifications: - 9. Value for APU occupation - 10. Value for CPU occupation As for the candidate value of APU and CPU, at least value 0 should be included. Besides, now that we support to include component 11 in FG 59-3-1, which can be seen as number of observation number, then the component 8 of FG 59-3-2 is not needed since the prerequisite of FG 59-3-2 is FG 59-3-1. Based on the above analyses, we suggest taking the following modifications (in red) for the feature of Rel-19 AI based CSI prediction. Proposal 7: Take the following modification (in red) for the feature of Rel-19 AI based CSI prediction: 58. NR_AIML_Air; 58-3-2; CSI prediction for UE-sided inference when N4>1; 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 5. Support for the size of DD-basis, N4>1 7. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting [8. Supported values of the maximum number of observation number]; 58-3-1; yes; n/a; CSI prediction for N4>1 for inference is not supported; [Per band and Per BC]; FFS; FFS; FFS; FFS: CPU/AIMLPU related information Candidate values: FFS; Optional with capability signalling. |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] | Regarding the 58-3-2 FG of CSI prediction on UE-sided inference when N4=4: - Component 58 is needed - Add CPU and AI/ML PU occupation when P/SP/AP is configured for CMR 58. NR_AIML_Air; 58-3-2; CSI prediction for UE-sided inference when N4>1; 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 5. Support for the size of DD-basis, N4>1 7. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting 8. Supported values of the maximum number of observation number 9. Value of CPU/AIML PU occupation, when P/SP CSI-RS is configured for CMR 10. Value of CPU/AIML PU occupation, when A-CSI-RS is configured for CMR; 58-3-1; yes; n/a; CSI prediction for N4>1 for inference is not supported; [Per band and Per BC]; FFS; FFS; FFS; Candidate values: FFS; Optional with capability signalling. |
| Samsung [11] | Unlink CPU, the number of occupied APUs for CSI prediction does not linearly scale with the number of measured CSI-RS resources (K) or the number of prediction slot intervals (N4). Thus, it is better for the UE to report a single candidate value for the number of occupied APU irrespective of the values for K and N4. Thus we propose the following Proposal 5: For component 14 of 58-3-1 and component 8 of 58-3-1, support the candidate values {1,2} for the occupied number of CPUs for performance monitoring for CSI prediction model 58. NR_AIML_Air; 58-3-2; CSI prediction for UE-sided inference when N4>1; 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 5. Support for the size of DD-basis, N4>1 76. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting 7. Supported number of occupied CPU 8. Supported number of occupied APU 9. Index of the occupied APU pool; 58-3-1; yes; n/a; CSI prediction for N4>1 for inference is not supported; [Per band and Per BC]; FFS N/A; FFS N/A; N/A; Component 3 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {2,3,4 … 64} d. {4, …, 256} Component 6 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {4,8,12} d.{4, …, 256} Component 8: candidate values {1,2,}; Optional with capability signalling. |
| LG Electronics [12] | 58. NR_AIML_Air; 58-3-2; CSI prediction for UE-sided inference when N4>1; 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 5. Support for the size of DD-basis, N4>1 6. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting 7. Supported number of occupied CPU 8. Supported number of occupied APU 9. Index of the occupied APU pool; 58-3-1; yes; n/a; CSI prediction for N4>1 for inference is not supported; Per band and Per BC; N/A; N/A; N/A; Component 3 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {2,3,4 … 64} d. {4, …, 256} Component 6 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {4,8,12} d.{4, …, 256} [Component 7 candidate values: {1,2,4,6,8}] [Component 8 candidate values: {1, 2,4,6,8}] Component 9 candidate values: {1, 2}; Optional with capability signalling. |
| Apple [13] |  |
| Qualcomm Incorporated [14] | Adopt the O_CPU and O_APU candidate values as O_CPU={1…8}, O_APU={0…8} in FG58-3-1 and FG58-3-2 UE reporting max 16 CSI-RS triplets reported for FG58-3-1, FG58-3-2, FG58-3-4 and FG58-3-5 |
| NTT DOCOMO, INC. [15] | At RAN1 #122 meeting, the issue about the terminology of APU or CPU,2/CPU,3 was discussed. Based on the discussions, there is no need to change the CPU,2/CPU,3 in the current specification. It is better to use the aligned terminology with the specification for the UE feature discussions. Therefore, we propose changing all APUs occurring in FG 58 to the term aligned with the specification. Proposal 1: Revise the following FGs about the usage of APU and change the APU and corresponding items to align with the term CPU,2/CPU,3. - FG 58-0-1, FG 58-1-2, FG 58-1-4, FG 58-3-1, FG 58-3-2. As proposed in Proposal 1, the APU issue should also be solved for this FG. According to the discussions of the last meeting, FG 58-0-1 will not be signalled if UE has no processing resource pool dedicated to AI/ML. Therefore, FG 58-0-1 cannot be the prerequisite of FG 58-3-1. Instead, FG 2-35 (CSI framework) can be used. 58. NR_AIML_Air; 58-3-2; CSI prediction for UE-sided inference when N4>1; 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 5. Support for the size of DD-basis, N4>1 6. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting 7. Supported number of occupied CPU 8. Supported number of occupied APU CPU,2/CPU,3 9. Occupied resource pool between CPU,2 and CPU,3APU pool; 58-3-1; yes; n/a; CSI prediction for N4>1 for inference is not supported; Per band and Per BC; N/A; N/A; N/A; Component 3 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {2,3,4 … 64} d. {4, …, 256} Component 6 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {4,8,12} d.{4, …, 256} Component 8 candidate values: {0, 2, 4, 8} Component 9 candidate values: {CPU,2 or CPU,3} Note: UE should not report non-zero value for Component 8 if FG 58-0-1 is not signalled; Optional with capability signalling. |

| 58. NR_AIML_Air | 58-3-4 | UE side data collection for CSI prediction | 1. Support of data collection for CSI prediction | FFS | yes | n/a | UE side data collection for CSI prediction is not supported | Per band and Per BC | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] | 58. NR_AIML_Air; 58-3-4; UE side data collection for CSI prediction; 1. Support of data collection for CSI prediction; 2-33; yes; n/a; UE side data collection for CSI prediction is not supported; Per band and Per BC; N/A; N/A; N/A; Optional with capability signalling. |
| CATT/CICTCI [9] | 58. NR_AIML_Air; 58-3-4; UE side data collection for CSI prediction; 1. Support of data collection for CSI prediction; FFS; yes; n/a; UE side data collection for CSI prediction is not supported; Per band and Per BC; N/A; N/A; N/A; Optional with capability signalling. |
| OPPO [10] | Regarding the 58-3-4 FG of UE side data collection for CSI prediction: - Add supported values of the maximum number of resources for measurement - Add supported values of the maximum number of future time instance 58. NR_AIML_Ai; 58-3-4; UE side data collection for CSI prediction; 1. Support of data collection for CSI prediction 2. Supported values of the maximum number of resources for measurement 3. Supported values of the maximum number of future time instance.; 58-3-1/2; yes; n/a; UE side data collection for CSI prediction is not supported; FFS; FFS; FFS; FFS; [Component 2 candidate values: {5,10,20}] [Component 3 candidate values: {1,2,4,8}]; Optional with capability signalling. |
| Samsung [11] |  |
| LG Electronics [12] | 58. NR_AIML_Air; 58-3-4; UE side data collection for CSI prediction; 1. Support of data collection for CSI prediction; FFS58-3-1; yes; n/a; UE side data collection for CSI prediction is not supported; Per band and Per BC; N/A; N/A; N/A; Optional with capability signalling. |
| Apple [13] |  |
| Qualcomm Incorporated [14] | Adopt FG2-35 as the pre-requisite FG for FG58-0-1 (APU), FG58-3-1 (CSI prediction inference) and FG58-3-4 (data collection) UE reporting max 16 CSI-RS triplets reported for FG58-3-1, FG58-3-2, FG58-3-4 and FG58-3-5 |
| NTT DOCOMO, INC. [15] | Like FG 58-1-6, the prerequisite FG for data collection should be FG 2-35, which resolves the only one remaining issue of this FG. Proposal 14: Update FG 58-3-4 as shown in the following table. 58. NR_AIML_Air; 58-3-4; UE side data collection for CSI prediction; 1. Support of data collection for CSI prediction; FFS 2-35; yes; n/a; UE side data collection for CSI prediction is not supported; Per band and Per BC; N/A; N/A; N/A; Optional with capability signalling. |

| 58. NR_AIML_Air | 58-3-5 | Performance monitoring for CSI prediction model | 1. Support of two performance metric SGCS 2. Support of one wideband frequency granularity SGCS per layer 3. Support of one configured time instance for N4>1 4. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously | 58-3-1 | Yes | n/a | Performance monitoring for CSI prediction model is not supported | Per Band and Per BC | N/A | N/A | N/A | Component 4 candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| Ericsson [2] |  |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] | - Component 5: In the RAN1#122 meeting, it is agreed that for CSI-PAI report, support OCPU=X, X is reported by UE capability. Thus, we support add Component 5 to capture the above agreement, and {1, 2} can considered as candidate values for the number of occupied CPU. 58. NR_AIML_Air; 58-3-5; Performance monitoring for CSI prediction model; 1. Support of two performance metric SGCS 2. Support of one wideband frequency granularity SGCS per layer 3. Support of one configured time instance for N4>1 4. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 5. Supported number of occupied CPU; 58-3-1; yes; n/a; Performance monitoring for CSI prediction model is not supported; Per band and Per BC; Component 4 Candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Component 5 candidate values: {1, 2}; Optional with capability signaling. |
| Xiaomi [6] |  |
| CMCC [7] |  |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] | Agreement For CSI-PAI report, support - OCPU =X, X is reported by UE capability - For AP monitoring report, CPU occupancy starts from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the CSI-PAI report - For SP monitoring report, CPU occupancy starts from the first symbol of the latest periodic/semi-persistent CSI-RS occasion of the inference report not later than CSI reference resource of the inference report, until the last symbol of the configured PUSCH carrying the CSI-PAI report RAN1#122 has agreed the UE to report the value for the occupied CPU by A CSI report for monitoring. This above can be reflected as component 5 of 58-3-5 as follows Proposal 4: Introduce component 5 for 58-3-5 for the occupied number of CPUs for performance monitoring for CSI prediction model 58. NR_AIML_Air; 58-3-5; Performance monitoring for CSI prediction model; 1. Supported of two performance metric SGCS 2. Support of one wideband frequency granularity SGCS per layer 3. Support of one configured time instance for N4>1 4. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 5. supported number of occupied CPU; 58-3-1; Yes; n/a; Performance monitoring for CSI prediction model is not supported; Per Band and Per BC; N/A; N/A; N/A; Component 4 candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Component 5 candidate values: {1,2}; Optional with capability signalling. |
| LG Electronics [12] | 58. NR_AIML_Air; 58-3-8; Performance monitoring for CSI prediction model; 1. Support of two performance metric SGCS 2. Support of one wideband frequency granularity SGCS per layer 3. Support of one configured time instance for N4>1 4. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 5. Supported number of occupied CPU; 58-3-1; Yes; n/a; Performance monitoring for CSI prediction model is not supported; Per Band and Per BC; N/A; N/A; N/A; Component 4 candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Component 5 candidate values: {1, 2}; Optional with capability signalling. |
| Apple [13] | Proposal 3-1: Add one additional component “supported number of occupied CPU” in UE feature group 58-3-5. |
| Qualcomm Incorporated [14] | UE reporting max 16 CSI-RS triplets reported for FG58-3-1, FG58-3-2, FG58-3-4 and FG58-3-5 |
| NTT DOCOMO, INC. [15] | This FG has no remaining FFS part. However, there is one maintenance agreement during RAN1 #122 that should be reflected in this FG [2]. Agreement: For CSI-PAI report, support - OCPU =X, X is reported by UE capability - For AP monitoring report, CPU occupancy starts from the first symbol after the PDCCH triggering the CSI report until the last symbol of the scheduled PUSCH carrying the CSI-PAI report - For SP monitoring report, CPU occupancy starts from the first symbol of the latest periodic/semi-persistent CSI-RS occasion of the inference report not later than CSI reference resource of the inference report, until the last symbol of the configured PUSCH carrying the CSI-PAI report. Based on the online discussions about this agreement, the intention to allow UE to report occupied CPU by UE capability is to ensure that the UE can support both inference and performance monitoring simultaneously. The value of X can be 0, 1, or 2 based on the online discussions. According to the agreement and the corresponding discussions, we added Component 5 into this FG, and its candidate values can be 0, 1, or 2. To reflect the intention of introducing this UE capability, a note is added to restrict that the UE must report a valid O_CPU combination for inference and performance monitoring. Proposal 15: Update FG 58-3-5 as shown in the following table. 58. NR_AIML_Air; 58-3-5; Performance monitoring for CSI prediction model; 1. Supported of two performance metric SGCS 2. Support of one wideband frequency granularity SGCS per layer 3. Support of one configured time instance for N4>1 4. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 5. Supported number of occupied CPU; 58-3-1; Yes; n/a; Performance monitoring for CSI prediction model is not supported; Per Band and Per BC; N/A; N/A; N/A; Component 4 candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Component 5 candidate values: {0, 1, 2} Note: The summation of the value reported by Component 5 and the one reported by Component 13 of FG58-3-1 (or Component 7 of FG58-3-2) should not significant than the N_CPU UE reported.; Optional with capability signalling. |

Other

| Company | Summary |
| --- | --- |
| Ericsson [2] | For semi-persistent inference report, one remaining issue is whether to support additional time relaxation, which may be needed for loading the model into the AI engine. In our view, allowing such relaxation will make this AI feature less useful, therefore we do not support adding new time relaxation. - Do not support adding new time relaxation to semi-persistent inference report for the CSI prediction use case. |
| ZTE Corporation/Sanechips [3] |  |
| Vivo [4] |  |
| Huawei/HiSilicon [5] |  |
| Xiaomi [6] |  |
| CMCC [7] | Besides, we have agreed to support performance monitoring for CSI prediction [3], we suggest introducing the corresponding Rel-19 UE FG for AI/ML based CSI prediction: [120bis] Agreement For CSI prediction using UE-side model, for performance monitoring, support UE assisted performance monitoring subject to an additional UE capability, and UE assisted performance monitoring is based on Type 3 performance monitoring. Proposal 8: Introduce the following Rel-19 UE FG for AI/ML based CSI prediction: 58. NR_AIML_Air; 58-3-14; Performance monitoring for CSI prediction; 1. Support of Type 3 performance monitoring based on SGCS for CSI prediction; 58-3-1; Yes; n/a; Type 3 performance monitoring based on SGCS for CSI prediction is not supported; FFS; FFS; FFS; FFS; Optional with capability signaling. |
| Nokia [8] |  |
| CATT/CICTCI [9] |  |
| OPPO [10] |  |
| Samsung [11] |  |
| LG Electronics [12] |  |
| Apple [13] |  |
| Qualcomm Incorporated [14] |  |
| NTT DOCOMO, INC. [15] |  |

# Discussion Items during RAN1 #122bis

After review of contributions submitted to RAN1 #122bis in this agenda item, the following topics were identified by the moderator for discussion during RAN1 #122bis.

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

## Specification support for beam management

After review of contributions submitted to RAN1 #122bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_air | 58-0-1 | CSI report framework for UE-side inference | 1. Number of APU CPU,x pools N 2. Maximum number of APU CPU,x in each APU CPU,x pool of UE-sided inference for CSI report(s) for simultaneously in a CC 3. Maximum number of APU CPU,x in each APU CPU,x pool of UE-sided inference for CSI report(s) simultaneously across all CCs 4. Maximum number of active AI/ML CSI-ReportConfigs across use cases | FFS 2-35 | yes | n/a | Maximum number of APUs for UE-sided inference is unknown to the network | Per UE Per band and per BC | No n/a | No n/a | No n/a | Component 1 candidate values: {1,2} Component 2candidate values: {1…8} Component 3 candidate values: {1…32} Component 4 candidate values: FFS Note: Component 2 and 3 candidate values are signalled separately for each pool [A UE that does not support this FG should not report non-zero occupied CPU,2 or CPU,3 values in any dependency FG reuses the CPU] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| NTT DOCOMO | About the granularity, we think it should be kept as Per UE since this FG is about the UE overall AI/ML capability across multiple use cases. It is fine to consider Per band for 58-1-x FGs but for this one, Per UE is more reasonable. |
| QC | - Need for Component 4 is not clear, suggest removing it. - We believe per band and per BC is the correct choice here, following a similar logic as the counterpart legacy components for CPU in 38.306. |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_air | 58-1-1 | Increased number of reported RSs for beam management | 1. Support of reporting format for L1-RSRP measurements not including CRI/SSBRI other than one for the largest measured L1-RSRP in a reporting instance, if the number of reported L1-RSRPs is equal to the size of the measurement resource set. 2. Support of reporting format for L1-RSRPs and corresponding beam information of Top M beam(s) with largest M measured value(s) of L1-RSRP(s) of a measurement resource set, where M is configured by gNB, if the number of reported L1-RSRPs is smaller than the size of the measurement resource set 3. Maximum number of M reported RSs, M>4 | FFS {2-21, 2-22} or {2-23, 2-23a} or 2-29 or 2-24 | yes | n/a | Increased number of reported beams for beam management is not supported | Per UE Per band | No n/a | No n/a | No n/a | Component 3 candidate values: {6,8} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_air | 58-1-2 | UE-side beam prediction for BM Case1 for inference | 1. Support of beam prediction with reporting of predicted beam index for BM-Case1 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case1 per BWP CC 3a. Maximum number of inference report(s) configured for BM-Case1 across all CCs 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 7c: Supported minimum number of resources for Set B 8. Supported CSI-RS resource types for Set B 9. Supported inference report types 11. Supported BM-Case 1 sub-usecase(s): {setB-subset-of-setA, setB-different-from-setA, both}] 12. Supported maximum number of predicted beams in each reporting instance FFS: whether/how to merge this FG with other FG(s) for performance monitoring and/or data collection 13. Supported number of occupied CPU 14. Supported number of occupied APU CPU,2/CPU,3 15. Supported value of d for the relaxation of Z3 timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 16. Supported value of d’ for the relaxation of Z’3 timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 17. Index of the Occupied APU resource pool between CPU,2 and CPU,3 18. Supported CSI-RS resource types for Set A | FFS 2-35 | yes | n/a | UE-sided beam prediction for BM Case 1 for inference is not supported | Per UE Per band | No n/a | No n/a | No n/a | Component 3 candidate values: {1, 2, 3, 4} Component 3a candidate values: {1, 2, 3, 4, 8} Component 7a candidate values: {4, 8, 16, 32, 64} Component 7b candidate values: {4, 8, 16, 32, 64} Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} Component 11 candidate values: {setB-subset-of-setA, setB-different-from-setA, both} Component 12 candidate values: {1, 2, 3, 4} Component 13 candidate values: {0, 1, 2} Component 14 candidate values: {0, 1, 2} Note: The values of Component 13 and 14 are not allowed to be 0 simultaneously Component 15 candidate values: d1 is {2, 4, 8, 14} d2 is {4, 8, 14, 28} d3 is {8,14, 28, 56} d4 is {14, 28, 56, 112} d5 is {56, 112, 224, 448} d6 is {112, 224, 448, 896} Component 17 candidate values: {1, 2} representing the first APU pool (i.e., CPU,2) and the second APU pool (i.e., CPU,3), respectively Component 18 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS, Aperiodic CSI-RS} Note: “CPU” corresponds to “CPU,1” in TS 38.214, and “APU” corresponds to “CPU,x” in TS 38.214, x = 2, 3 FFS: candidate values for components | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| QC | - For component 7a candidate values {32, 64} need to be justified, given the max size of 64 for Set A. - Need for Component 7c in not clear, suggest removing it. - For Components 13 and 14, the current range of candidate values are too restrictive. Suggest updating the values for Components 13 and 14 to INTEGER(1..8) and INTEGER(0..8), respectively. This set of candidate values should be determined based on the values agreed for N_APU, which is agreed to be INTEGER(1..8) [Similarly for N_CPU it is INTEGER(1..8)]. If a UE reports an N_APU budget of 8, then restricting the values to {0,1,2}, then this means UE has to support at least 4 simultaneous AI/ML reports which defeats the purpose of going into this direction of reporting O_CPU values via UE capability. The main motivation was to enable flexible implementation of UE-side AI/ML models given this is the first time we are specifying AI/ML features. - For components 15 and 16 the current provided values are too restrictive, and defeat the purpose of introducing this UE capability to enable this UE feature wit reasonable complexity. Our suggested values: for μ={0,1,2,3,4,5,6}, the corresponding values for d and d' are: {56,112,224,448,896,1792,3584}, respectively. For this timeline, we need to account for model loading latency, pre-and post-processing to AI/ML inputs/outputs, and finally the latency associated with generating the predictions (inference latency). |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_air | 58-1-3 | UE-side beam prediction for BM Case1 with predicted RSRP for inference | 1. Support of beam prediction, reporting of predicted beam index and predicted RSRP, for BM-Case1 for inference 2. Supported maximum number of predicted beams with RSRP in each reporting instance | 58-1-2 | yes | n/a | UE-side beam prediction for BM Case 1 with predicted RSRP for inference is not supported | Per UE Per band | No n/a | No n/a | No n/a | Component 2 candidate values: {1, 2, 3, 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_air | 58-1-4 | UE-side beam prediction for BM Case2 for inference | 1. Support of beam prediction with reporting of predicted beam index for BM-Case2 for inference with UE-side model 3. Maximum number of inference report(s) configured for BM-Case2 per BWP CC 3a. Maximum number of inference report(s) configured for BM-Case2 across all CCs 6. Support of SSB as RS type for Set B 6a. Support of CSI-RS as RS type for Set B 6b. Support of SSB as RS type for Set A 6c. Support of CSI-RS as RS type for Set A 7a: Supported maximum number of resources for Set B 7b: Supported maximum number of resources for Set A 8. Supported CSI-RS resource types for Set B 9. Supported inference report types 11. Supported maximum number of predicted beams in each predicted time instance 12. Supported maximum number of predicted time instances 13. Supported maximum total number of reported predicted beams for predicted time instances in one report 14. Supported combinations of supported value(s) of valid time duration for each predicted time instance and number of predicted beams for each value of valid time duration 15. Supported value(s) of time gap between predicted time instances and between reference time to the first future time instance 16. Supported value(s) of setB periodicity 17. Supported CSI-RS resource types for Set A 19. Supported combinations of time gaps between predicted time instances and value of set B periodicity. 20. Supported BM-Case 2 sub usecase(s) 21. supported number of occupied CPU 22. supported number of occupied APU CPU,2/CPU,3 23. supported value of d for the relaxation of Z3 timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 24. supported value of d’ for the relaxation of Z’3 timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 25. Index of the Occupied APU resource pool between CPU,2 and CPU,3 | FFS 2-35 | yes | n/a | UE-side beam prediction for BM-Case2 for inference is not supported | Per UE Per band | No n/a | No n/a | No n/a | Component 3 candidate values: {1, 2, 3, 4} Component 3a candidate values: {1, 2, 3, 4, 8} Component 7a candidate values: {4, 8, 16, 32, 64} Component 7b candidate values: {4, 8, 16, 32, 64} Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report}FFS: candidate values for components Component 11 candidate values: {1, 2, 3, 4} Component 12 candidate values: {1, 2, 4, 6, 8} Component 13 candidate values: {1, 2, 4, 6, 8, 12, 16, 24, 32} Component 14 candidate values: any combination between: Number of predicted beams: {1,2,4,8} Time gap: {10ms, 20ms, 40ms, 80ms, 160ms} Component 15 candidate values: FFS Component 16 candidate values: FFS Component 17 candidate values: FFS Component 19 candidate values: FFS Component 20 candidate values: FFS Component 21 candidate values: {0, 1, 2, …, 8} Component 22 candidate values: {0, 1, 2, …, 8} Note: The values of Component 21 and 22 are not allowed to be 0 simultaneously. Component 23 candidate values: d1 is {2, 4, 8, 14} d2 is {4, 8, 14, 28} d3 is {8,14, 28, 56} d4 is {14, 28, 56, 112} d5 is {56, 112, 224, 448} d6 is {112, 224, 448, 896} Component 25 candidate values: {1, 2} representing the first APU pool (i.e., CPU,2) and the second APU pool (i.e., CPU,3), respectively Note: “CPU” corresponds to “CPU,1” in TS 38.214, and “APU” corresponds to “CPU,x” in TS 38.214, x = 2, 3 Note: UE should not report non-zero value for Component 22 if FG 58-0-1 is not signalled | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| NTT DOCOMO | To reduce the NW management burden, we suggest reducing the number of candidates values for components 21/22, e.g., {0, 1, 2, 4, 8}. |
| QC | - For component 13, at least the values {24, 32} need to be justified. - For components 23 and 24 the current provided values are too restrictive, and defeat the purpose of introducing this UE capability to enable this UE feature wit reasonable complexity. Our suggested values: for μ={0,1,2,3,4,5,6}, the corresponding values for d and d' are: {56,112,224,448,896,1792,3584}, respectively. For this timeline, we need to account for model loading latency, pre-and post-processing to AI/ML inputs/outputs, and finally the latency associated with generating the predictions (inference latency). |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_air | 58-1-5 | UE-side beam prediction for BM-Case2 with predicted RSRP for inference | 1. Support of beam prediction, reporting of predicted beams and predicted RSRP, for BM-Case2 (spatial and time domain beam prediction) for inference 2. Supported maximum number of predicted beams in each predicted time instance 3. Supported maximum number of predicted time instances 4. Supported maximum total number of reported predicted beams for predicted time instances in one report | 58-1-4 | yes | n/a | UE-side beam prediction for BM-Case2 for inference is not supported | Per UE Per band | No n/a | No n/a | No n/a | Component 2 candidate values: {1, 2, 3, 4} Component 3 candidate values: {1, 2, 4, 8} Component 4 candidate values: {1, 2, 3, 4, 8, 12, 16, 32} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
|  | - For component 4 candidate values {32, 64} need to be justified, given the max size of 64 for Set A. |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_air | 58-1-6 | Performance monitoring for UE-sided model | 1. Support of performance monitoring with RS-PAI of AI/ML model for beam prediction. 2. Maximum total number of the configured SSB resources and/or CSI-RS resources for monitoring RS resource set 3. Maximum number of monitoring report(s) configured per BWP CC 4. Maximum number of monitoring report(s) configured across all CCs 5. Maximum number of monitoring occasions for RS-PAI calculation 6. Support of SSB as RS type for monitoring 7. Support of CSI-RS as RS type for monitoring 8. Supported monitoring resource types 9. Supported monitoring report types | FFS 58-1-2 or 58-1-4 | yes | n/a | Performance monitoring for UE-sided model is not supported | FFS Per band | FFS n/a | FFS n/a | FFS n/a | Component 2 candidate values: {4, 8, 16, 32, 64} Component 3 candidate values: {1, 2, 4, 8} Component 4 candidate values: {1, 2, 4, 8} Component 5 candidate values: {1, 3, 7, 15} Component 8 candidate values: {Periodic CSI-RS, Semi-persistent CSI-RS} Component 9 candidate values: {Periodic CSI report, Aperiodic CSI report, semi-persistent CSI report} FFS: candidate values for components | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_air | 58-1-7 | Data collection for UE-side beam prediction | 1. Support of data collection for UE-side beam prediction 2. Support of SS/PBCH block and 1-port CSI-RS based RSRP measurements for measurement RS resource sets (Set B and Set A) for data collection 3. Supported sub-use cases 6: Supported maximum number of resources for Set B 7: Supported maximum number of resources for Set A 8. Support of SSB as RS type for Set B 9. Support of CSI-RS as RS type for Set B 10. Support of SSB as RS type for Set A 11. Support of CSI-RS as RS type for Set A | FFS 58-1-2 or 58-1-4, 2-35 | yes | n/a | Data collection for UE-side beam prediction is not supported | Per UE Per band | No n/a | No n/a | No n/a | Component 3 candidate values: {‘Set B equal to Set A’, ‘Set B subset of Set A’,’Set B not a subset of Set A’} Component 6 candidate values: {4, 8, 16, 32, 64} Component 7 candidate values: {8, 16, 32, 64} Note: it is up to RAN2 whether this FG is merged into data collection FG defined by RAN2 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| NTT DOCOMO | The data collection does not rely on the UE AIML inference capability. It is a CSI report under the legacy CSI framework. Therefore, the prerequisites 58-1-x are unnecessary. |

## Specification support for positioning accuracy enhancement

After review of contributions submitted to RAN1 #122bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_Air | 58-2-1 | UE-based positioning Case 1 | Support of UE-based positioning Case 1 | 13-1 or 58-2-4 | N/A | N/A | UE-based positioning Case 1 is not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported Note: If UE does not provide 58-2-4 but the UE supports Case 1, FG 13-1 indicates the DL PRS processing capabilities common across all positioning methods including UE-based positioning Case 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_Air | 58-2-2 | Support reception of AD for UE-based positioning Case 1 | Indicates support of reception of AD for UE-based positioning Case 1 | 58-2-1 | N/A | N/A | Reception of AD for UE-based positioning Case 1 is not supported | Per UE | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: For specific ADs, UE indicate supported AD that can be received as in [FFS: FGs] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | The Note was meant as a place holder for adding other ADs (some needs RAN2 action). AD of Case 1 should have multiple bits (similar to other legacy positioning methods). If the note is deleted, then the whole FG 58-2-2 shall be deleted and individual AD FG shall be introduced. |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_Air | 58-2-3 | DL PRS Resources for UE-based positioning Case 1 | 1. Max number of DL PRS Resource Sets per TRP per frequency layer supported by UE 2. Max number of TRPs across all positioning frequency layers per UE 3. Max number of positioning frequency layers UE supports | [13-1 or 58-2-4] | N/A | N/A |  | Per UE | No | No | No | Component 1 candidate values: {1, 2} Component 2 candidate values: {4, 6, 12, 16, 24, 32, 64, 128, 256} Component 3 candidate values: {1, 2, 3, 4} Need for location server to know if the feature is supported. Note: If UE does not provide 58-2-4 but the UE supports Case 1, FG 13-1 indicates the DL PRS processing capabilities common across all positioning methods including UE-based positioning Case 1 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| DOCOMO | If prerequisite of FG 58-2-1 includes FG 13-1, this version is fine for us. If not, prerequisite of this FG should be only 58-2-4. |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_Air | 58-2-3a | DL PRS Resources for UE-based positioning Case 1 on a band | [1. Max number of DL PRS Resources per DL PRS Resource Set 2. Max number of DL PRS Resources per positioning frequency layer | [13-1 or 58-2-4] | N/A | N/A |  | Per band | n/a | n/a | n/a | Component 1 candidate values: {1, 2, 4, 8, 16, 32, 64} Component 2 candidate values: {6, 24, 32, 64, 96, 128, 256, 512, 1024} Note: For component 1, the values 16, 32, 64 are only applicable to FR2 bands Note: For component 2, the value 6 is only applicable to FR1 bands Need for location server to know if the feature is supported. Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination Note: If UE does not provide 58-2-4 but the UE supports Case 1, FG 13-1 indicates the DL PRS processing capabilities common across all positioning methods including UE-based positioning Case 1 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| DOCOMO | Same comment in FG 58-2-3. |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_Air | 58-2-3b | DL PRS Resources for UE-based positioning Case 1 on a band combination | 1. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1-only 2. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2-only 3. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR1 in FR1/FR2 mixed operation 4. Max number of DL PRS Resources supported by UE across all frequency layers, TRPs and DL PRS Resource Sets for FR2 in FR1/FR2 mixed operation | [13-1 or 58-2-4] | N/A | N/A |  | Per BC | n/a | n/a | n/a | Component 1 candidate values: {6, 24, 64, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR1 only BC Component 2 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for FR2 only BC Component 3 candidate values: {6, 24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands Component 4 candidate values: {24, 64, 96, 128, 192, 256, 512, 1024, 2048} Note this is reported for BC containing FR1 and FR2 bands] Need for location server to know if the feature is supported. Note: the reported value is the total number across all bands in the corresponding BC Note: if the UE does not indicate this capability for a band or band combination, the UE does not support this positioning method in this band or band combination Note: If UE does not provide 58-2-4 but the UE supports Case 1, FG 13-1 indicates the DL PRS processing capabilities common across all positioning methods including UE-based positioning Case 1 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| DOCOMO | Same comment in FG 58-2-3. |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_Air | 58-2-4 | DL PRS Processing Capability for UE-based positioning Case 1 | 1. Maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 2. DL PRS buffering capability: Type 1 or Type 2 a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 3. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE. 4. Max number of DL PRS resources that UE can process in a slot under it Note: The above parameters are reported assuming a configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) of no more than 30%. |  | No | N/A |  | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Component 1 candidate values: -FR1 bands values {5, 10, 20, 40, 50, 80, 100} -FR2 bands values {50, 100, 200, 400} Component 3 candidate values: -T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms -N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 4 candidate values: -FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz -FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Notes for component 3: a. UE reports one combination of (N, T) values per band, where N is a duration of DL PRS symbols in ms processed every T ms for a given maximum bandwidth (B) in MHz supported by UE b. UE is not expected to support DL PRS bandwidth that exceeds the reported DL PRS bandwidth value c. UE DL PRS processing capability is defined for a single positioning frequency layer. UE capability for simultaneous DL PRS processing across positioning frequency layers is not supported in Rel.16 (i.e. for a UE supporting multiple positioning frequency layers, a UE is expected to process one frequency layer at a time) d. UE DL PRS processing capability is agnostic to DL PRS comb factor configuration e. The reporting of (N, T) values for maximum BW in MHz is not dependent on SCS Note: if the UE does not indicate this capability for a band or band combination, the UE does not support PRS processing in this band or band combination. Note: If UE does not provide [this FG] but the UE supports Case 1, FG 13-1 indicates the DL PRS processing capabilities common across all positioning methods including UE-based positioning Case 1.] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_Air | 58-2-5 | Support of SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 | 1. Support of SSB from neighbour cell as QCL source of a DL PRS 2. Support of reuse SSB measurement from RRM for receiving PRS Note: Refers to Type-C for FR1 and Type-C & Type-D support for FR2 | [58-2-4; otherwise 13-1] | No | N/A | SSB from neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported | Per band | n/a | n/a | n/a | Need for location server to know if the feature is supported Note: Refers to Type-C for FR1 and Type-C & Type-D support for FR2 Note: if UE supports same values for one or more components as in FG 13-7, then the UE can skip indicating these components in this FG and the values in corresponding FG 13-7 components indicate supported QCL sources for Case 1 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_Air | 58-2-6 | Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 | 1. Support of DL PRS from serving/neighbour cell as QCL source of a DL PRS Note 1: Refers to Type-D support for FR2 Note 2: A PRS from a PRS-only TP is treated as PRS from a non-serving cell | [58-2-4; otherwise 13-1] | No | N/A | DL PRS from serving/neighbour cell as QCL source of a DL PRS for UE-based positioning Case 1 is not supported | Per Band | n/a | n/a | n/a | Need for location server to know if the feature is supported. Note: DL PRSs are in the same band Note: Refers to Type-D support for FR2 Note: A PRS from a PRS-only TP is not considered in Rel. 19 Note: if UE supports same values for one or more components as in FG 13-7a, then the UE can skip indicating these components in this FG and the values in corresponding FG 13-7a components indicate supported QCL sources for Case 1 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-X | AI/ML based positioning for UE-side model | 1. Support of AI/ML based positioning Case 1 |  | FFS | N/A | AI/ML based positioning Case 1 is not supported | FFS | FFS | FFS | FFS | RAN1 kindly requests RAN2 to decide on the necessity for location server to know if the feature is supported | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 58. NR_AIML_Air | 58-2-Y | Support of model monitoring for AI/ML based positioning for UE-side model | 1. Support of performance monitoring for AI/ML based positioning Case 1 |  | FFS | N/A | Performance monitoring of AI/ML based positioning Case 1 is not supported | FFS | FFS | FFS | FFS | FFS: The necessity and further partitioning of this FG depend on the future RAN1/2 agreements on the format of model monitoring outcome. | Optional with capability signalling |
| 58. NR_AIML_Air | 58-2-Z | Support of data collection for AI/ML based positioning for UE-side model | 1. Support of Data collection for AI/ML based positioning Case 1 2. Support of Associated ID |  | FFS | N/A | Data collection of AI/ML based positioning Case 1 is not supported | FFS | FFS | FFS | FFS | FFS: The necessity and further partitioning of this FG depends on the future RAN1 agreements on Associated ID | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm2 | For 58-2-X, there is an FG, i.e., 58-2-1, that indicates support for Case 1. It is not clear how this is different with 58-2-1 For 58-2-Z, there is no necessity to introduce this FG at this stage. There is rigorous RAN1 agreement for supporting this FG. Data collection solution is still ongoing work in Rel-20. For 58-2-Y, this will most likely be captured as an LPP error cause which is handled by RAN2. We propose to skip this one as UE already obtains relevant assistance data to use for monitoring using other FGs. |
|  | For 58-2-x Agree with Qualcomm For 58-2-y , in RAN1 we had the following agreements that support this UE capability Agreement(RAN1#119) For model performance monitoring of AI/ML positioning Case 1, support at least: - Option A. The target UE side performs monitoring metric calculation. - The target UE may signal the monitoring outcome to the LMF. - FFS: content of monitoring outcome - FFS: Option B Agreement(RAN1#120b) For model performance monitoring of AI/ML positioning Case 1, “FFS: content of monitoring outcome” in RAN1#119 agreement is resolved by: - the content of monitoring outcome includes at least an indication that the target UE cannot perform the Case 1 positioning method. Conclusion(RAN1#121) For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, - There is no consensus in RAN1 to support Option B (including Option B-1 and B-2). Based on the agreements and conclusion, from RAN1 point of view, it is expected that UE should be capable to provide Monitoring outcome. So, replying to QC when is said “this will most likely be captured as an LPP error cause which is handled by RAN2”, it is an assumption that is out of RAN1 scope that does not correspond to discuss in RAN1 forum. Our assumption is to map current RAN1 agreement which support this new UE Feature Group. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-21 | Implicit associated ID indication for UE-based positioning Case 1 | Support reception of assistance data containing the implicit associated ID for UE-based positioning Case 1. | 58-2-1 | N/A | N/A | Reception of Associated ID for UE-based positioning Case 1 is not supported | Per UE | No | No | No | Need for location server to know if the feature is supported. Note: The Associated ID is used to provide implicit information of Info #7 of the assistance information from legacy UE-based DL-TDOA. Note: For specific ADs, UE indicate supported AD that can be received as in [FFS: FGs] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | Our understanding RAN2 introducing features for both implicit associated ID and explicit indication of info#7. |
| Nokia | The proposed new UE Feature group is based on RAN1 agreements. It is out of RAN1 discussion on making assumptions on RAN2 UE Features. The following agreements were done in RAN1 that supports our view: Working Assumption(RAN1#121) For AI/ML based positioning Case 1, regarding info #7 in the assistance information from legacy UE-based DL-TDOA, it can be provided as in legacy UE-based DL-TDOA or implicitly. Agreement(RAN1#121) Above Working Assumption is confirmed. Agreement(RAN1#121) For AI/ML based positioning Case 1, regarding Info #7 in the assistance information from legacy UE-based DL-TDOA, - If implicitly provided, the implicit indication of Info #7 is via associated ID. - For given TRP(s), same associated ID implies that geographical coordinates of the TRP(s) can be understood as consistent by the UE. - The associated ID is not expected to provide the real value of Info #7 (i.e., geographical coordinates of the TRP(s) are not disclosed). - an associated ID is configured per-cell (e.g., NCGI-r15) - UE does not expect to receive different values of associated ID for TRPs belonging to the same NCGI-r15 - Associated ID can be realized by an identifier of N bits (e.g., 8 bits) |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-22 | Support performance monitoring for positioning Case 1 | Indicates support of performance monitoring for positioning Case 1 | FFS | N/A | N/A | Performance monitoring for positioning Case 1 is not supported | Per UE | No | No | No | FFS: The necessity and further components of this FG depend on the future RAN1/2 agreements on the format of model monitoring outcome | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm2 | This is repeated FG to the one preoposed in an earlier proposal (see FG-58-2-Y). This will most likely be captured as an LPP error cause which is handled by RAN2. We propose to skip this one as UE already obtains relevant assistance data to use for monitoring using other FGs. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-4a | Max number of positioning frequency layers UE supports all bands for UE-based for positioning Case 1 | Max number of positioning frequency layers UE supports across all bands for UE-based positioning Case-1 |  | N/A | N/A | Maximum number of PFLs for UE-based positioning Case 1 is not supported | Per Band | N/A | N/A | N/A | Candidate values: {1, 2, 3, 4} Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | We proposed this FG but now realizes component 3 in 58-2-3 provides equivalent coverage. We ask to skip presenting this FG. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-12 | Support of UE-based positioning Case 1 in RRC_INACTIVE state | UE-based positioning Case 1 in RRC_INACTIVE state is supported | 58-2-3 and 58-2-13 | N/A | N/A | UE-based positioning Case 1 in RRC_INACTIVE state is not supported | Per Band | N/A | N/A | N/A | Support of UE-based positioning Case 1 in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-13 | DL PRS processing capabilities in RRC inactive state for UE-based positioning Case 1 | 1. DL PRS buffering capability a) Type 1 – sub-slot/symbol level buffering b) Type 2 – slot level buffering 2. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum DL PRS bandwidth in MHz, which is supported and reported by UE 3. Max number of DL PRS resources that UE can process in a slot | 58-2-4 | N/A | N/A |  | Per band | N/A | N/A | N/A | Component 1 candidate values: {Type 1, Type 2} Component 2 candidate values: T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Component 3 candidate values: FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Need for location server to know if the feature is supported Note: Having the PRS processing capabilities in RRC_INACTIVE state does not imply that LMF is aware of or controlling UE RRC state Note: if UE supports same values for one or more components as in FG 27-6, then the UE can skip indicating these components in this FG and the values in corresponding FG 27-6 components indicate supported PRS processing capabilities in RRC inactive state for Case 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-14 | Support of UE-based positioning Case 1 in RRC_IDLE | Support of UE-based positioning Case 1 in RRC_IDLE | 58-2-4, 58-2-12, 58-2-13 | N/A | N/A | UE-based positioning Case 1 in RRC_IDLE is not supported | Per Band | N/A | N/A | N/A | Need for location server to know if the feature is supported | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-15b | DL PRS processing capabilities for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1 | 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Maximum number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot | 58-2-13 | N/A | N/A |  | Per Band | N/A | N/A | N/A | Component 1 candidate values: a) FR1 bands: {10, 20, 40, 50, 80, 100, 160, 200} b) FR2 bands: {100, 200, 400, 800} Component 2 candidate values:a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in FG 58-2-15b (this FG) follows buffering capability type reported in FG 58-2-4 Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by FG 58-2-13 or this value T should be equal or larger than the value T reported by FG 58-2-13 Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each two linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by FG 58-2-13 Note: if UE supports same values for one or more components as in FG 41-4-1b, then the UE can skip indicating these components in this FG and the values in corresponding FG 41-4-1b components indicate supported for aggregated PRS processing of 2 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE capabilities for Case 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-15c | DL PRS processing capabilities for aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE - UE-based positioning Case 1 | 1. Maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE 2. Maximum DL PRS bandwidth in MHz, per PFL 3. DL PRS buffering capability 4. Duration of DL PRS symbols N in units of ms a UE can process every T ms assuming maximum aggregated DL PRS bandwidth in MHz, which is supported and reported by UE. 5. Max number of aggregated DL PRS resources across aggregated PFLs that UE can process in a slot under it | 58-2-15b | N/A | N/A |  | Per Band | N/A | N/A | N/A | Component 1 candidate values: a) FR1 bands: {15, 20, 30, 40, 50, 60, 80, 100, 120, 140, 150, 160, 180, 200, 240, 300} b) FR2 bands: {150, 200, 300, 400, 600, 800, 1000, 1200} Component 2 candidate values: a) FR1 bands: {5, 10, 20, 40, 50, 80, 100} b) FR2 bands: {50, 100, 200, 400} Note: Component 3 in FG 58-2-15c (this FG) follows buffering capability type reported in FG 58-2-4 Component 4 candidate values: a) T: {8, 16, 20, 30, 40, 80, 160, 320, 640, 1280} ms b) N: {0.125, 0.25, 0.5, 1, 2, 4, 6, 8, 12, 16, 20, 25, 30, 32, 35, 40, 45, 50} ms Note: this value N should be equal or smaller than the value N reported by FG 58-2-13 or this value T should be equal or larger than the value T reported by FG 58-2-13 Component 5 candidate values: a. FR1 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 15kHz, 30kHz, 60kHz b. FR2 bands: {1, 2, 4, 6, 8, 12, 16, 24, 32, 48, 64} for each SCS: 60kHz, 120kHz Note: each three linked PRS resources are counted as 1 resource Note: this value should be equal or smaller than the value reported by FG 58-2-13 Note: if UE supports same values for one or more components as in FG 41-4-1c, then the UE can skip indicating these components in this FG and the values in corresponding FG 41-4-1c components indicate supported aggregated PRS processing of 3 PFLs in intra-band contiguous for RRC_IDLE and RRC_INACTIVE for Case 1 | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-18 | PRS bandwidth aggregation in RRC_ INACTIVE UE-based positioning Case 1 | Support of PRS bandwidth aggregation in RRC_ INACTIVE for UE-based positioning Case 1 | 58-2-12, 58-2-15b | N/A | N/A | PRS bandwidth aggregation in RRC_ INACTIVE for UE-based positioning Case 1 is not supported | Per Band | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-2-19 | PRS bandwidth aggregation in RRC_IDLE - UE-based positioning Case 1 | Support of PRS bandwidth aggregation in RRC_IDLE for UE-based positioning Case 1 | 58-2-14, 58-2-15b | N/A | N/A | PRS bandwidth aggregation in RRC_IDLE for UE-based positioning Case 1 is not supported | Per Band | N/A | N/A | N/A | Need for location server to know if the feature is supported. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 27. NR_pos_enh | 58-2-27-10a | Low latency MG activation request for PRS measurements | support of low latency MG activation request for PRS measurements | 27-10, 27-11 | mg-ActivationRequest-r17 | LPP NR-DL-TDOA-ProvideCapabilities-r16 NR-DL-AoD-ProvideCapabilities-r16 NR-Multi-RTT-ProvideCapabilities-r16 NR-UE-based-PositioningCase1-ProvideCapabilities-r19 | No | No | Need for location server to know if the feature is supported Note: RAN1 understands that FG 27-10a is intended only for the LMF to know, and that the current prerequisite FGs of FG 27-10a are capabilities only for the gNB to know. It is up to RAN2 to decide whether such a FG dependency is meaningful from signaling description perspective, and whether and how it can be captured in RAN2 specifications. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 27. NR_pos_enh | 58-2-27-12 | LOS/NLOS indicator for UE-based positioning assistance data | Support reception of the assistance data containing the LOS/NLOS indicator. 1. LOS/NLOS indicator type 2. LOS/NLOS indicator granularity |  | nr-los-nlos-IndicatorSupport-r17 | NR-DL-TDOA-ProvideCapabilities-r16 NR-DL-AoD-ProvideCapabilities-r16 NR-Multi-RTT-ProvideCapabilities-r16 [NR-Case1-ProvideCapabilities] | No | No | Component 1 candidate values: {hardValue+softValue, hardValue} Component 2 candidate values: {resourceSpecific, trpSpecific} Need for location server to know if the feature is supported. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 27. NR_pos_enh | 58-2-6 | SRS Resources for AI Positioning Case 3a/3b | 1. Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. 2. Max number of P/SP/AP SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of P/SP/AP SRS Resources including the SRS resources for positioning per BWP per slot. Values = {1, 2, 3, 4, 5, 6, 8, 10, 12, 14} Note: Max number of P/SP/AP SRS Resources in Component 3 include both SRS resources configured by SRS-Resource and SRS resources configured by SRS-PosResource-r16 supported by UE 4. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 5. Max number of periodic SRS Resources for positioning per BWP per slot. Values = {1,2,3,4,5,6,8,10,12,14} OLPC for SRS for positioning based on SSB from serving cell is part of FG13-8 Note: no dedicated capability signaling is intended for this component |  | N/A | N/A | SRS Resources for AI Positioning Case 3a/3b is not supported | Per Band | N/A | N/A | N/A | Note: if the UE does not indicate this capability for a band in a band combination, the UE does not support SRS for Positioning in this band in the band combination. | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 58-2-7 | Support of Aperiodic SRS Resources for AI positioning Case 3a/3b | 1. Max number of aperiodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 2. Max number of aperiodic SRS Resources for positioning per BWP per slot. Values = {1,2,3,4,5,6,8,10,12,14} | 58-2-6 | N/A | N/A | Support of Aperiodic SRS Resources for AI positioning Case 3a/3b is not supported | Per Band | N/A | N/A | N/A |  | Optional with capability signaling |
| 27. NR_pos_enh | 58-2-8 | Support of Semi-persistent SRS Resources for AI positioning Case 3a/3b | 1. Max number of semi-persistent SRS Resources for positioning supported by UE per BWP. Values = {1,2,4,8,16,32,64} 2. Max number of semi-persistent SRS Resources for positioning supported by UE per BWP per slot. Values = {1,2,3,4,5,6,8,10,12,14} | 58-2-6 | N/A | N/A | Support of Semi-persistent SRS Resources for AI positioning Case 3a/3b is not supported | Per Band | N/A | N/A | N/A |  | Optional with capability signaling |
| 27. NR_pos_enh | 58-2-9 | SRS Resources for AI Positioning Case 3a/3b | 1. Max number of SRS Resource Sets for positioning supported by UE per BWP. Values = {1, 2, 4, 8, 12, 16}. 2. Max number of P/SP/AP SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} 3. Max number of periodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} | 58-2-6 | N/A | N/A | SRS Resources for AI Positioning Case 3a/3b is not supported | Per Band | N/A | N/A | N/A | Need for location server to know if the feature is supported UE only reports the number on bands for the current configured CA band combination | Optional with capability signaling |
| 27. NR_pos_enh | 58-2-10 | Support of Aperiodic SRS Resources for AI positioning Case 3a/3b | 1. Max number of aperiodic SRS Resources for positioning per BWP. Values = {1,2,4,8,16,32,64} | 58-2-7, 58-2-9 | N/A | N/A | Support of Aperiodic SRS Resources for AI positioning Case 3a/3b is not supported | Per Band | N/A | N/A | N/A | Need for location server to know if the feature is supported. UE only reports the number on bands for the current configured CA band combination. | Optional with capability signaling |
| 27. NR_pos_enh | 58-2-11 | Support of Semi-persistent SRS Resources for AI positioning Case 3a/3b | 1. Max number of semi-persistent SRS Resources for positioning supported by UE per BWP. Values = {1,2,4,8,16,32,64} | 58-2-6, 58-2-9 | N/A | N/A | Support of Semi-persistent SRS Resources for AI positioning Case 3a/3b is not supported | Per Band | N/A | N/A | N/A | Need for location server to know if the feature is supported. UE only reports the number on bands for the current configured CA band combination | Optional with capability signaling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | For SRS capabilities, RAN1 did not distinguish SRS for Case 3a/3b from existing SRS configurations. Existing FGs are sufficient. No need to introduce new ones. |
| DOCOMO | Agree with QC. In my understanding, RAN1 does not make any agreement to enhance SRS for Case 3a/3b, which means UE transmits existing SRS whether legacy positioning method or Case 3a/3b is used. |

## Specification support for CSI prediction

After review of contributions submitted to RAN1 #122bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_Air | 58-3-1 | CSI prediction for UE-sided inference when N4=1 | 1. Support of CSI prediction for UE-sided inference when N4=1 2. Support for reporting predicted PMI with N4=1 3. A list of supported combinations, each combination is { Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Support of Rel-16 eType-II regular codebook refinement for predicted PMI with PMI subband R=1 5. Support parameter combinations with L=2,4 6. Support for rank = 1,2 7. Support for the size of DD-basis, N4=1 8. Support X=1 CQI based on the first/earliest slot of the CSI reporting window and the first/earliest predicted PMI (TDCQI=’1-1’) 11. Scaling factor for active resource counting Kp 12. supported value of t for the relaxation of Z and Z’ timeline, where i is the index of SCS, i=1,2,3,4,5,6 corresponding to 15,30,60,120,480,960 kHz SCS 13. supported number of occupied CPU 14. supported number of occupied APU CPU,2/CPU,3 15. Index of the Occupied APU resource pool between CPU,2 and CPU,3 16. Limit the number of candidate values for APU occupation 17. Limit the number of candidate values for CPU occupation 18. Limit the number of candidate values for the relaxation of Z and Z’ timeline. At least negative value of t shall be supported | 58-0-1 2-35 | yes | n/a | CSI prediction for N4=1 for inference is not supported | Per band and Per BC | n/a | n/a | n/a | Component 3 candidate values: a. {4,8,12,16,24,32} b. {2,3,4 … 64} c. {4, …, 256} Component 11 candidate values: {1,2,4} Component 12 candidate values: d1 is {0, 2, 4, 8, 14} d2 is {0, 4, 8, 14, 28} d3 is {0, 8,14, 28, 56} d4 is {0, 14, 28, 56, 112} d5 is {0, 56, 112, 224, 448} d6 is {0, 112, 224, 448, 896} Component 13 candidate values: {0, …, 8} Component 14 candidate values: {0, …, 8} Component 15 candidate values: {1, 2} representing the first APU pool (i.e., CPU,2) and the second APU pool (i.e., CPU,3), respectively. Note: “CPU” corresponds to “CPU,1” in TS 38.214, and “APU” corresponds to “CPU,x” in TS 38.214, x = 2, 3 Note: UE should not report non-zero value for Component 14 if FG 58-0-1 is not signalled | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | Suggest to delete 16-18. We should be open to all possible candidate values at this stage. It’s premature to do any down-selection until the phase of practical deployment or IODT trialed. Only the implementation team has a clear answer for that. For 18, the negative value of t is lack of justification. |
| NTT DOCOMO | Similar to FG 58-1-4, to reduce the NW management burden, we suggest reducing the number of candidate values for components 13/14, e.g., {0, 1, 2, 4, 8}. |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_Air | 58-3-2 | CSI prediction for UE-sided inference when N4>1 | 1. Support of CSI prediction for UE-sided inference when N4>1 2. Support for reporting predicted PMI with N4>1 3. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 4. Value of d=m for the DD unit size when A-CSI-RS is configured for CMR 5. Support for the size of DD-basis, N4>1 6. A list of supported combinations, each combination is {Max N4, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} for one CSI report setting 7. Supported number of occupied CPU 8. Supported number of occupied APU CPU,2/CPU,3 9. Index of the Occupied APU resource pool between CPU,2 and CPU,3 | 58-3-1 | yes | n/a | CSI prediction for N4>1 for inference is not supported | Per band and Per BC | N/A | N/A | N/A | Component 3 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {2,3,4 … 64} d. {4, …, 256} Component 6 candidate values: a. {1,2,4,8} b. {4,8,12,16,24,32} c. {4,8,12} d.{4, …, 256} Component 7 candidate values: {0, …, 8} Component 8 candidate values: {0, 1, 2, 4, 8} Component 9 candidate values: {1, 2} representing the first APU pool (i.e., CPU,2) and the second APU pool (i.e., CPU,3), respectively Note: “CPU” corresponds to “CPU,1” in TS 38.214, and “APU” corresponds to “CPU,x” in TS 38.214, x = 2, 3 Note: UE should not report non-zero value for Component 8 if FG 58-0-1 is not signalled | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| NTT DOCOMO | Similar to FG 58-1-4, to reduce the NW management burden, we suggest reducing the number of candidate values for components 7/8, e.g., {0, 1, 2, 4, 8}. |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_Air | 58-3-4 | UE side data collection for CSI prediction | 1. Support of data collection for CSI prediction | FFS 2-33 or 2-35 or 58-3-1 | yes | n/a | UE side data collection for CSI prediction is not supported | Per band and Per BC | N/A | N/A | N/A |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | The pre-requiste group should not be 58-3-1 as the UE supports data collection may not support the prediction inference. |
| NTT DOCOMO | We share the same view with QC. For data collection, the FG about the inference is unnecessary as a prerequisite. |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 58. NR_AIML_Air | 58-3-5 | Performance monitoring for CSI prediction model | 1. Support of two performance metric SGCS 2. Support of one wideband frequency granularity SGCS per layer 3. Support of one configured time instance for N4>1 4. A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs in a band when reported per band, and across all CCs in a band combination when reported per BC simultaneously 5. Supported number of occupied CPU | 58-3-1 | Yes | n/a | Performance monitoring for CSI prediction model is not supported | Per Band and Per BC | N/A | N/A | N/A | Component 4 candidate values: - Maximum 16 triplets - Max # of Tx ports in one resource: {4,8,12,16,24,32} - Max # resources: {1 to 64} - Max # total ports: {4 to 256} Component 5 candidate values: {0, 1, 2} Note: The summation of the value reported by Component 5 and the one reported by Component 13 of FG58-3-1 (or Component 7 of FG58-3-2) should not significant than the N_CPU UE reported | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| NTT DOCOMO | We want to emphasize that the note in this FG is essential. The UE should not report supporting this FG, but the N_CPU cannot accommodate both 58-3-1/2 and 58-3-5. |

Proposal: Introduce the following Rel. 19 UE FGs (yellow highlighting, if any, shows text that’s not yet agreed)

| 58. NR_AIML_Air | 58-3-14 | Performance monitoring for CSI prediction | 1. Support of Type 3 performance monitoring based on SGCS for CSI prediction | 58-3-1 | Yes | n/a | Type 3 performance monitoring based on SGCS for CSI prediction is not supported | FFS | FFS | FFS | FFS |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | Not sure what this FG means, is it overlapped with 58-3-5? |
| NTT DOCOMO | Same view with QC. This one is redundant. |

# Conclusion

Agreements reached during RAN1 #122bis as part of this agenda item are summarized in [16].

# References

- R1-2506627, Updated RAN1 UE features list for Rel-19 NR after RAN1 #122, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2506761, UE Features for Rel-19 AI/ML for NR Air Interface, Ericsson

- R1-2506772, Discussion on UE features for AI/ML for NR Air Interface, ZTE Corporation/Sanechips

- R1-2506881, UE features for AI/ML for NR Air Interface, vivo

- R1-2506935, UE features for AI/ML for NR air interface, Huawei/HiSilicon

- R1-2506972, Discussion on UE features for AI/ML for NR Air Interface, Xiaomi

- R1-2507004, Discussion on UE features for AI/ML for NR air Interface, CMCC

- R1-2507072, UE features for AI/ML for NR Air Interface, Nokia

- R1-2507100, UE features for AI/ML for NR Air Interface, CATT/CICTCI

- R1-2507156, UE features for AIML for NR air interface, OPPO

- R1-2507236, Remaining issues on UE features for AI/ML for NR air interface, Samsung

- R1-2507395, Discussion on UE features for AI/ML for NR Air Interface, LG Electronics

- R1-2507658, Views on UE features for Rel-19 AI/ML for NR air interface, Apple

- R1-2507704, UE features for AI/ML air interface, Qualcomm Incorporated

- R1-2507795, Discussion on UE features for AI/ML for NR Air Interface, NTT DOCOMO, INC.

- R1-25nnnnn, Session Notes of AI 9.1, Ad-Hoc Chair (AT&T)