3GPP TSG RAN WG1 #122bis	                                   					       												                           R1-2507740

Prague, Czech Republic, October 13th –17th, 2025

Agenda Item:	9.4

Source:	Moderator (AT&T)

Title:	Summary of UE features for enhancements of network energy savings for NR

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email discussion [122bis-R19-UE_features] during RAN1 #122bis. According to the Chair’s Notes:

[122bis-R19-UE_features] Email discussion on Rel-19 UE features – Ralf (AT&T), Naoya (DOCOMO) - To be used for sharing updates on online/offline schedule, details on what is to be discussed in online/offline sessions, tdoc number of the moderator summary for online session, etc

The following was discussed during RAN1 #122bis within the scope of [122bis-R19-UE_features]. All proposals are based on the latest RAN1 UE features list for Rel. 19 in [1].

# Summary of Contributions Submitted to RAN1 #122bis

The following is the moderator’s summary of contributions submitted to RAN1 #122bis in this agenda item.

## On-demand SSB SCell operation

| 61. Netw_Energy_NR_enh | 61-1 | On-demand SSB SCell operation indicated by RRC based signaling in Case #1 | 1. Support RRC based signalling to indicate activation and deactivation of on-demand SSB transmission on the SCell in Case #1 (No always-on SSB on the cell) |  | Yes | No | UE does not support on-demand SSB transmission indicated by RRC based signaling in Case #1 | Per band | n/a | n/a | n/a | Note: it is up to RAN2 whether/how to update this FG for RRC based deactivation | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| OPPO [2] | We first would like to provide some background information on the OD-SSB activation and deactivation. RAN1#116bis: Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. RAN1#120: Conclusion The following combination of scenarios and cases for indicating OD-SSB are not supported in Rel-19 - Scenario #3A and Case #1 - Scenario #3A and Case #2 Above does not impact discussion on SSB periodicity adaptation in time domain RAN1#120bis: Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. RAN1#120bis: Agreement For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell) - UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.. For OD-SSB activation In our understanding, from RAN1#120 and RAN1#120bis meeting agreements, the activation of OD-SSB by MAC-CE can only be performed in scenario 2 and 2A for both Case 1 and Case 2. Thus, we understand that the OD-SSB activation by MAC-CE is only on deactivated SCell. We suspect that this is also the case for RRC based OD-SSB activation although there is no explicit RAN1 agreement. If companies agree with our understanding, we would like to suggest a clarification on the FG description to correctly reflect the feature. For OD-SSB deactivation In our understanding, from RAN1#120bis meeting agreement, for Case 1, the OD-SSB deactivation by RRC/MAC-CE can only be performed when SCell is deactivated. If companies agree with our understanding, we would like to suggest a clarification on the FG description to correctly reflect the feature. With the above explanation, we suggest the following changes: 61. Netw_Energy_NR_enh; 61-1; On-demand SSB SCell operation indicated by RRC based signaling in Case #1; 1. Support RRC based signalling to indicate activation and deactivation of on-demand SSB transmission on the deactivated SCell in Case #1 (No always-on SSB on the cell); Yes; No; UE does not support on-demand SSB transmission indicated by RRC based signaling in Case #1; Per band; n/a; n/a; n/a; Note: it is up to RAN2 whether/how to update this FG for RRC based deactivation [Note: If UE supports both of FG 61-1 and FG 61-3, UE supports MAC CE based deactivation mechanism to deactivate the on-demand SSB indicated by RRC in Case #1]; Optional with capability signaling. |
| Ericsson [3] |  |

| 61. Netw_Energy_NR_enh | 61-1a | On-demand SSB SCell operation indicated to be activated by RRC based signaling and indicated to be adapted and deactivated by MAC CE signalling in Case #1 | 1. Support RRC based signalling to indicate activation and MAC CE based signalling to indicate adaptation and deactivation of on-demand SSB transmission on the SCell in Case #1 (No always-on SSB on the cell) | 61-1, 61-3 | Yes | No | UE does not support on-demand SSB transmission indicated to be activated by RRC based signaling and indicated to be adapted and deactivated by MAC CE signalling in Case #1 | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| OPPO [2] | We first would like to provide some background information on the OD-SSB activation and deactivation. RAN1#116bis: Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. RAN1#120: Conclusion The following combination of scenarios and cases for indicating OD-SSB are not supported in Rel-19 - Scenario #3A and Case #1 - Scenario #3A and Case #2 Above does not impact discussion on SSB periodicity adaptation in time domain RAN1#120bis: Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. RAN1#120bis: Agreement For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell) - UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.. For OD-SSB activation In our understanding, from RAN1#120 and RAN1#120bis meeting agreements, the activation of OD-SSB by MAC-CE can only be performed in scenario 2 and 2A for both Case 1 and Case 2. Thus, we understand that the OD-SSB activation by MAC-CE is only on deactivated SCell. We suspect that this is also the case for RRC based OD-SSB activation although there is no explicit RAN1 agreement. If companies agree with our understanding, we would like to suggest a clarification on the FG description to correctly reflect the feature. For OD-SSB deactivation In our understanding, from RAN1#120bis meeting agreement, for Case 1, the OD-SSB deactivation by RRC/MAC-CE can only be performed when SCell is deactivated. If companies agree with our understanding, we would like to suggest a clarification on the FG description to correctly reflect the feature. With the above explanation, we suggest the following changes: 61. Netw_Energy_NR_enh; 61-1a; On-demand SSB SCell operation indicated to be activated by RRC based signaling and indicated to be adapted and deactivated by MAC CE signalling in Case #1; 1. Support RRC based signalling to indicate activation and MAC CE based signalling to indicate adaptation of on-demand SSB transmission on the activated SCell and deactivation of on-demand SSB transmission on the deactivated SCell in Case #1 (No always-on SSB on the cell); 61-1, 61-3; Yes; No; UE does not support on-demand SSB transmission indicated to be activated by RRC based signaling and indicated to be adapted and deactivated by MAC CE signalling in Case #1; Per band; n/a; n/a; n/a; Optional with capability signaling. |
| Ericsson [3] |  |

| 61. Netw_Energy_NR_enh | 61-2 | On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for same center frequency | 1. Support RRC based signalling to indicate activation and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency between always-on SSB and on-demand SSB 2. Supported time domain relation between on-demand SSB and always-on SSB |  | Yes | No | UE does not support on-demand SSB transmission on the SCell indicated by RRC based signaling in Case #2 for same center frequency between always-on SSB and on-demand SSB | Per band | n/a | n/a | n/a | Candidate value of component 2 = {Time-C1, Time-C1nC2} Note: - Time-C1: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern (the interval between SSB bursts is even and supported in legacy specification) - Time-C1nC2 includes both Time-C1 and Time-C2 (Time-C2: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern) Note: it is up to RAN2 whether/how to update this FG for RRC based deactivation | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| OPPO [2] |  |
| Ericsson [3] |  |

| 61. Netw_Energy_NR_enh | 61-2b | On-demand SSB SCell operation indicated to be activated by RRC based signaling and indicated to be adapted and deactivated by MAC CE signalling in Case #2 for same center frequency | 1. Support RRC based signalling to indicate activation and MAC CE based signalling to indicate adaptation and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency | 61-2, 61-4 | Yes | No | UE does not support on-demand SSB transmission on the SCell indicated to be activated by RRC based signaling and indicated to be adapted and deactivated by MAC CE signalling in Case #2 for same center frequency | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| OPPO [2] |  |
| Ericsson [3] |  |

| 61. Netw_Energy_NR_enh | 61-2a | On-demand SSB SCell operation indicated by RRC based signaling in Case #2 for different center frequencies | 1. Support RRC based signalling to indicate activation and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequencies between always-on SSB and on-demand SSB | 61-2 | Yes | No | UE does not support on-demand SSB transmission on the SCell indicated by RRC based signaling in Case #2 for different center frequencies between always-on SSB and on-demand SSB | Per band | n/a | n/a | n/a | Note: it is up to RAN2 whether/how to update this FG for RRC based deactivation | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| OPPO [2] |  |
| Ericsson [3] |  |

| 61. Netw_Energy_NR_enh | 61-2c | On-demand SSB SCell operation indicated to be activated by RRC based signaling and indicated to be adapted and deactivated by MAC CE signalling in Case #2 for different center frequencies | 1. Support RRC based signalling to indicate activation and MAC CE based signalling to indicate adaptation and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequencies between always-on SSB and on-demand SSB | 61-2a, 61-4a | Yes | No | UE does not support on-demand SSB transmission on the SCell indicated to be activated by RRC based signaling and indicated to be adapted and deactivated by MAC CE signalling in Case #2 for different center frequencies | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| OPPO [2] |  |
| Ericsson [3] | Our views on remaining aspects of the FGs for Rel-19 NES are as shown below. ## FG 61-2c: (On-demand SSB SCell operation indicated to be activated by RRC based signaling and indicated to be adapted and deactivated by MAC CE signalling in Case #2 for different center frequencies) - Pre-requisite: Add “61-2b” as an additional pre-requisite. - 61-2b: “On-demand SSB SCell operation indicated to be activated by RRC based signaling and indicated to be adapted and deactivated by MAC CE signalling in Case #2 for same center frequency” |

| 61. Netw_Energy_NR_enh | 61-3 | On-demand SSB SCell operation indicated via MAC CE in Case #1 | 1. Support MAC CE based signalling to indicate activation, adaptation, and deactivation of on-demand SSB transmission on the SCell in Case #1 (No always-on SSB on the cell) 2. Supported on-demand SSB deactivation mechanisms: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indicationImplicit deactivation via od-ssb-nrofBurst of on-demand SSB bursts to be transmitted after on-demand SSB is indicated |  | Yes |  | UE does not support on-demand SSB transmission on the SCell indicated via MAC CE in Case #1 | Per band | n/a | n/a | n/a | Component 2 candidate value: {explicit deactivation, explicit and implicit deactivation} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| OPPO [2] | We first would like to provide some background information on the OD-SSB activation and deactivation. RAN1#116bis: Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. RAN1#120: Conclusion The following combination of scenarios and cases for indicating OD-SSB are not supported in Rel-19 - Scenario #3A and Case #1 - Scenario #3A and Case #2 Above does not impact discussion on SSB periodicity adaptation in time domain RAN1#120bis: Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. RAN1#120bis: Agreement For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell) - UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.. For OD-SSB activation In our understanding, from RAN1#120 and RAN1#120bis meeting agreements, the activation of OD-SSB by MAC-CE can only be performed in scenario 2 and 2A for both Case 1 and Case 2. Thus, we understand that the OD-SSB activation by MAC-CE is only on deactivated SCell. We suspect that this is also the case for RRC based OD-SSB activation although there is no explicit RAN1 agreement. If companies agree with our understanding, we would like to suggest a clarification on the FG description to correctly reflect the feature. For OD-SSB deactivation In our understanding, from RAN1#120bis meeting agreement, for Case 1, the OD-SSB deactivation by RRC/MAC-CE can only be performed when SCell is deactivated. If companies agree with our understanding, we would like to suggest a clarification on the FG description to correctly reflect the feature. With the above explanation, we suggest the following changes: 61. Netw_Energy_NR_enh; 61-3; On-demand SSB SCell operation indicated via MAC CE in Case #1; 1. Support MAC CE based signalling to indicate activation, [adaptation,] and deactivation of on-demand SSB transmission on the deactivated SCell, and adaptation of on-demand SSB transmission on the activated SCell in Case #1 (No always-on SSB on the cell) 2. Supported on-demand SSB deactivation mechanisms: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indicationImplicit deactivation via od-ssb-nrofBurst of on-demand SSB bursts to be transmitted after on-demand SSB is indicated; FFS; Yes; UE does not support on-demand SSB transmission on the SCell indicated via MAC CE in Case #1; Per band; n/a; n/a; n/a; Component 2 candidate value: {explicit deactivation, explicit and implicit deactivation}; Optional with capability signaling. |
| Ericsson [3] |  |

| 61. Netw_Energy_NR_enh | 61-4 | On-demand SSB SCell operation indicated via MAC CE in Case #2 for same center frequency | 1. Support MAC CE based signalling to indicate activation, adaptation, and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency between always-on SSB and on-demand SSB 2.Supported time domain relation between on-demand SSB and always-on SSB 3. Supported on-demand SSB deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Implicit deactivation via od-ssb-nrofBurst of on-demand SSB bursts to be transmitted after on-demand SSB is indicated |  | Yes |  | UE does not support on-demand SSB transmission on the SCell indicated via MAC CE in Case #2 for same center frequency between always-on SSB and on-demand SSB | Per band | n/a | n/a | n/a | Candidate value of component 2 = {Time-C1, Time-C1nC2} Note: - Time-C1: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern (the interval between SSB bursts is even and supported in legacy specification) - Time-C1nC2 includes both Time-C1 and Time-C2 (Time-C2: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern) Component 3 candidate value: {explicit deactivation, explicit and implicit deactivation} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| OPPO [2] | We first would like to provide some background information on the OD-SSB activation and deactivation. RAN1#116bis: Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. RAN1#120: Conclusion The following combination of scenarios and cases for indicating OD-SSB are not supported in Rel-19 - Scenario #3A and Case #1 - Scenario #3A and Case #2 Above does not impact discussion on SSB periodicity adaptation in time domain RAN1#120bis: Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. RAN1#120bis: Agreement For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell) - UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.. For OD-SSB activation In our understanding, from RAN1#120 and RAN1#120bis meeting agreements, the activation of OD-SSB by MAC-CE can only be performed in scenario 2 and 2A for both Case 1 and Case 2. Thus, we understand that the OD-SSB activation by MAC-CE is only on deactivated SCell. We suspect that this is also the case for RRC based OD-SSB activation although there is no explicit RAN1 agreement. If companies agree with our understanding, we would like to suggest a clarification on the FG description to correctly reflect the feature. For OD-SSB deactivation In our understanding, from RAN1#120bis meeting agreement, for Case 1, the OD-SSB deactivation by RRC/MAC-CE can only be performed when SCell is deactivated. If companies agree with our understanding, we would like to suggest a clarification on the FG description to correctly reflect the feature. With the above explanation, we suggest the following changes: 61. Netw_Energy_NR_enh; 61-4; On-demand SSB SCell operation indicated via MAC CE in Case #2 for same center frequency; 1. Support MAC CE based signalling to indicate activation of on-demand SSB transmission on deactivated SCell, [adaptation] of on-demand SSB transmission on activated SCell, and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency between always-on SSB and on-demand SSB 2.Supported time domain relation between on-demand SSB and always-on SSB 3. Supported on-demand SSB deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Implicit deactivation via od-ssb-nrofBurst of on-demand SSB bursts to be transmitted after on-demand SSB is indicated; FFS; Yes; UE does not support on-demand SSB transmission on the SCell indicated via MAC CE in Case #2 for same center frequency between always-on SSB and on-demand SSB; Per band; n/a; n/a; n/a; Candidate value of component 2 = {Time-C1, Time-C1nC2} Note: - Time-C1: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern (the interval between SSB bursts is even and supported in legacy specification) - Time-C1nC2 includes both Time-C1 and Time-C2 (Time-C2: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern) Component 3 candidate value: {explicit deactivation, explicit and implicit deactivation}; Optional with capability signaling. |
| Ericsson [3] |  |

| 61. Netw_Energy_NR_enh | 61-4a | On-demand SSB SCell operation indicated via MAC CE in Case #2 for different center frequencies | 1. Support MAC CE based signalling to indicate activation, adaptation, and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequencies between always-on SSB and on-demand SSB 2. Supported on-demand SSB deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Implicit deactivation via od-ssb-nrofBurst of on-demand SSB bursts to be transmitted after on-demand SSB is indicated | 61-4 | Yes |  | UE does not support on-demand SSB transmission on the SCell indicated via MAC CE in Case #2 for different center frequencies between always-on SSB and on-demand SSB | Per band | n/a | n/a | n/a | Component 3 candidate value: {explicit deactivation, explicit and implicit deactivation} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| OPPO [2] | We first would like to provide some background information on the OD-SSB activation and deactivation. RAN1#116bis: Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. RAN1#120: Conclusion The following combination of scenarios and cases for indicating OD-SSB are not supported in Rel-19 - Scenario #3A and Case #1 - Scenario #3A and Case #2 Above does not impact discussion on SSB periodicity adaptation in time domain RAN1#120bis: Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. RAN1#120bis: Agreement For a cell supporting on-demand SSB SCell operation, for Case #1 (i.e., No always-on SSB on the cell) - UE does not expect the OD-SSB transmission indicated by RRC/MAC-CE to be deactivated while the SCell is activated.. For OD-SSB activation In our understanding, from RAN1#120 and RAN1#120bis meeting agreements, the activation of OD-SSB by MAC-CE can only be performed in scenario 2 and 2A for both Case 1 and Case 2. Thus, we understand that the OD-SSB activation by MAC-CE is only on deactivated SCell. We suspect that this is also the case for RRC based OD-SSB activation although there is no explicit RAN1 agreement. If companies agree with our understanding, we would like to suggest a clarification on the FG description to correctly reflect the feature. For OD-SSB deactivation In our understanding, from RAN1#120bis meeting agreement, for Case 1, the OD-SSB deactivation by RRC/MAC-CE can only be performed when SCell is deactivated. If companies agree with our understanding, we would like to suggest a clarification on the FG description to correctly reflect the feature. With the above explanation, we suggest the following changes: 61. Netw_Energy_NR_enh; 61-4a; On-demand SSB SCell operation indicated via MAC CE in Case #2 for different center frequenciesy; 1. Support MAC CE based signalling to indicate activation of on-demand SSB transmission on deactivated SCell, [adaptation,] of on-demand SSB transmission on activated SCell and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequenciesy between always-on SSB and on-demand SSB 2. Supported on-demand SSB deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Implicit deactivation via od-ssb-nrofBurst of on-demand SSB bursts to be transmitted after on-demand SSB is indicated; 61-4; Yes; UE does not support on-demand SSB transmission on the SCell indicated via MAC CE in Case #2 for different center frequenciesy between always-on SSB and on-demand SSB; Per band; n/a; n/a; n/a; Component 3 candidate value: {explicit deactivation, explicit and implicit deactivation}; Optional with capability signaling. |
| Ericsson [3] |  |

## On-demand SIB1 for idle/inactive mode UEs

| 61. Netw_Energy_NR_enh | 61-5 | SIB1 request for idle/inactive UEs | 1. Reception of SIB1 request configuration associated with SIB1 request for a cell 2. Transmission of PRACH on the uplink to request SIB1 of the cell 3. Reception of SIB1 upon SIB1 request |  | No | No | UE does not camp on the cell | n/a | n/a | n/a | n/a | A UE indicates support of this FG if it transmits a SIB1 request | Optional without capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| OPPO [2] |  |
| Ericsson [3] |  |

## Adaptation of SSB transmissions

| 61. Netw_Energy_NR_enh | 61-6 | SSB burst periodicity adaptation for SCell operation | Support of adaptation of SSB burst periodicity for SCell by DCI format 2_9 |  | Yes |  | UE does not support adaptation of SSB burst periodicity for SCell | Per band | n/a | n/a | n/a | Note: the SSB for this FG is not cell defining SSB | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| OPPO [2] |  |
| Ericsson [3] |  |

## Adaptation of RACH transmissions

| 61. Netw_Energy_NR_enh | 61-7 | Adaptation of RACH in time domain based on additional RACH resources | 1. Support of adaptation of RACH in time domain based on additional RACH resources in RRC idle/inactive/connected mode 2. Configuration of additional PRACH resources via higher layer signaling 3. DCI-based indication of additional PRACH resources by DCI format 1_0 with P-RNTI 4. DCI-based indication of additional PRACH resources by DCI format 1_0 with C-RNTI for PDCCH-ordered PRACH 5. Support semi-static PRACH mask to identify the subset of additional resources |  | Yes |  | UE does not support adaptation of RACH in time domain based on additional RACH resources | Per band | n/a | n/a | n/a | A UE that transmits PRACH in additional RO based on configuration of additional PRACH resources via higher layer signaling supports this FG | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Summary |
| --- | --- |
| OPPO [2] |  |
| Ericsson [3] |  |

## Others

| Company | Summary |
| --- | --- |
| OPPO [2] |  |
| Ericsson [3] |  |

# Discussion Items during RAN1 #122bis

After review of contributions submitted to RAN1 #122bis in this agenda item, the following topics were identified by the moderator for discussion during RAN1 #122bis.

General comments

| Company | Comments/Questions/Suggestions |
| --- | --- |

## On-demand SSB SCell operation

After review of contributions submitted to RAN1 #122bis in this agenda item, the following is proposed by the moderator. Companies submitted the following views on the moderator’s proposals.

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 61. Netw_Energy_NR_enh | 61-1 | On-demand SSB SCell operation indicated by RRC based signaling in Case #1 | 1. Support RRC based signalling to indicate activation and deactivation of on-demand SSB transmission on the deactivated SCell in Case #1 (No always-on SSB on the cell) |  | Yes | No | UE does not support on-demand SSB transmission indicated by RRC based signaling in Case #1 | Per band | n/a | n/a | n/a | Note: it is up to RAN2 whether/how to update this FG for RRC based deactivation | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Samsung | We don’t think the change is needed. MAC CE/RRC based activation/deactivation can be used as a generic term in the UE capability description, and the details of how to use it can be up to RAN1/RAN2 detailed specification. |
| LG Electronics | We share the view with Samsung. Capturing signaling details in TS series specifications is sufficient. |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 61. Netw_Energy_NR_enh | 61-1a | On-demand SSB SCell operation indicated to be activated by RRC based signaling and indicated to be adapted and deactivated by MAC CE signalling in Case #1 | 1. Support RRC based signalling to indicate activation and MAC CE based signalling to indicate adaptation of on-demand SSB transmission on the activated SCell and deactivation of on-demand SSB transmission on the deactivated SCell in Case #1 (No always-on SSB on the cell) | 61-1, 61-3 | Yes | No | UE does not support on-demand SSB transmission indicated to be activated by RRC based signaling and indicated to be adapted and deactivated by MAC CE signalling in Case #1 | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Our understanding is that adaptation using MAC-CE can be also used for both activated and deactivated SCell, which is not aligned with the proposal. |
| Samsung | We don’t think the change is needed. MAC CE/RRC based activation/deactivation can be used as a generic term in the UE capability description, and the details of how to use it can be up to RAN1/RAN2 detailed specification. |
| LG Electronics | We share the view with Samsung. Capturing signaling details in TS series specifications is sufficient. |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 61. Netw_Energy_NR_enh | 61-2c | On-demand SSB SCell operation indicated to be activated by RRC based signaling and indicated to be adapted and deactivated by MAC CE signalling in Case #2 for different center frequencies | 1. Support RRC based signalling to indicate activation and MAC CE based signalling to indicate adaptation and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequencies between always-on SSB and on-demand SSB | 61-2a, 61-2b, 61-4a | Yes | No | UE does not support on-demand SSB transmission on the SCell indicated to be activated by RRC based signaling and indicated to be adapted and deactivated by MAC CE signalling in Case #2 for different center frequencies | Per band | n/a | n/a | n/a |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Samsung | OK with the change. |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 61. Netw_Energy_NR_enh | 61-3 | On-demand SSB SCell operation indicated via MAC CE in Case #1 | 1. Support MAC CE based signalling to indicate activation, adaptation, and deactivation of on-demand SSB transmission on the deactivated SCell and adaptation of on-demand SSB transmission on the activated SCell in Case #1 (No always-on SSB on the cell) 2. Supported on-demand SSB deactivation mechanisms: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indicationImplicit deactivation via od-ssb-nrofBurst of on-demand SSB bursts to be transmitted after on-demand SSB is indicated |  | Yes |  | UE does not support on-demand SSB transmission on the SCell indicated via MAC CE in Case #1 | Per band | n/a | n/a | n/a | Component 2 candidate value: {explicit deactivation, explicit and implicit deactivation} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Our understanding is that adaptation using MAC-CE can be also used for both activated and deactivated SCell, which is not aligned with the proposal. |
| Samsung | We don’t think the change is needed. MAC CE/RRC based activation/deactivation can be used as a generic term in the UE capability description, and the details of how to use it can be up to RAN1/RAN2 detailed specification. |
| LG Electronics | We share the view with Samsung. Capturing signaling details in TS series specifications is sufficient. |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 61. Netw_Energy_NR_enh | 61-4 | On-demand SSB SCell operation indicated via MAC CE in Case #2 for same center frequency | 1. Support MAC CE based signalling to indicate activation of on-demand SSB transmission on deactivated SCell, adaptation of on-demand SSB transmission on activated SCell, and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for same center frequency between always-on SSB and on-demand SSB 2.Supported time domain relation between on-demand SSB and always-on SSB 3. Supported on-demand SSB deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Implicit deactivation via od-ssb-nrofBurst of on-demand SSB bursts to be transmitted after on-demand SSB is indicated |  | Yes |  | UE does not support on-demand SSB transmission on the SCell indicated via MAC CE in Case #2 for same center frequency between always-on SSB and on-demand SSB | Per band | n/a | n/a | n/a | Candidate value of component 2 = {Time-C1, Time-C1nC2} Note: - Time-C1: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a periodic time domain pattern (the interval between SSB bursts is even and supported in legacy specification) - Time-C1nC2 includes both Time-C1 and Time-C2 (Time-C2: During OD-SSB transmission, the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern) Component 3 candidate value: {explicit deactivation, explicit and implicit deactivation} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Our understanding is that adaptation using MAC-CE can be also used for both activated and deactivated SCell, which is not aligned with the proposal. |
| Samsung | We don’t think the change is needed. MAC CE/RRC based activation/deactivation can be used as a generic term in the UE capability description, and the details of how to use it can be up to RAN1/RAN2 detailed specification. |
| LG Electronics | We share the view with Samsung. Capturing signaling details in TS series specifications is sufficient. |

Proposal: Adopt the following changes highlighted in chromatic fonts, while keeping the yellow highlighting, if any, as shown

| 61. Netw_Energy_NR_enh | 61-4a | On-demand SSB SCell operation indicated via MAC CE in Case #2 for different center frequencies | 1. Support MAC CE based signalling to indicate activation of on-demand SSB transmission on deactivated SCell, adaptation of on-demand SSB transmission on activated SCell, and deactivation of on-demand SSB transmission on the SCell in Case #2 (Always-on SSB is periodically transmitted on the cell) for different center frequencies between always-on SSB and on-demand SSB 2. Supported on-demand SSB deactivation mechanisms: - Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Implicit deactivation via od-ssb-nrofBurst of on-demand SSB bursts to be transmitted after on-demand SSB is indicated | 61-4 | Yes |  | UE does not support on-demand SSB transmission on the SCell indicated via MAC CE in Case #2 for different center frequencies between always-on SSB and on-demand SSB | Per band | n/a | n/a | n/a | Component 3 candidate value: {explicit deactivation, explicit and implicit deactivation} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Our understanding is that adaptation using MAC-CE can be also used for both activated and deactivated SCell, which is not aligned with the proposal. |
| Samsung | We don’t think the change is needed. MAC CE/RRC based activation/deactivation can be used as a generic term in the UE capability description, and the details of how to use it can be up to RAN1/RAN2 detailed specification. |
| LG Electronics | We share the view with Samsung. Capturing signaling details in TS series specifications is sufficient. |

## On-demand SIB1 for idle/inactive mode UEs

Void

## Adaptation of SSB transmissions

Void

## Adaptation of RACH transmissions

Void

# Conclusion

Agreements reached during RAN1 #122bis as part of this agenda item are summarized in [4].

# References

- R1-2506627, Updated RAN1 UE features list for Rel-19 NR after RAN1 #122, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2507135, Discussion on UE features for enhancements of network energy savings for NR, OPPO

- R1-2507640, UE features for R19 NES, Ericsson

- R1-25nnnnn, Session Notes of AI 9.4, Ad-Hoc Chair (AT&T)