3GPP TSG RAN WG2#131	R2-2505016

Bengaluru, India, 25th - 29thAugust 2025

3GPP TSG RAN WG1 #121			R1-2504861

St Julian’s, Malta, May 19th – 23rd, 2025

Title:	LS on TS38.300 TP for Multi-carrier enhancements in Rel-19

Response to:

Release:	Rel-19

Work Items:	NR_MC_enh2

Source:	RAN WG1

To:	RAN WG2

CC:

Contact Person:

#### Name:	Haipeng Lei

E-mail Address:	leihp1@lenovo.com

Attachment:

1. Overall Description:

RAN1 discussed on necessary updates for Rel-19 Multi-carrier enhancements in TS38.300, and following text proposal was agreed as RAN1 recommendation.

## 10.11 Multi-cell scheduling by a single DCI Multi-cell scheduling by a single DCI allows the PDCCH of a serving cell to schedule PDSCH(s)/PUSCH(s) on one or more serving cells with the single DCI but with the following restrictions: - When a serving cell is configured with a PDCCH which schedules PDSCH(s)/PUSCH(s) on a cell set, the PUSCH/PDSCH on serving cells in the cell set is always scheduled by a PDCCH on the serving cell; - When SpCell is configured with a PDCCH which schedules PDSCH(s)/PUSCH(s) on serving cells in a cell set, that SpCell's PDSCH and PUSCH cannot be scheduled by a PDCCH on an SCell; - When an SCell is configured with a PDCCH which schedules PDSCH(s)/PUSCH(s) on serving cells in a cell set, SpCell is not included in the cell set; - The scheduling PDCCH and the scheduled PDSCH(s)/PUSCH(s) can use the same or different numerologies; - The co-scheduled PDSCH(s) with a PDCCH can use the same or different numerologies; - The co-scheduled PUSCH(s) with a PDCCH can use the same or different numerologies.

2. Actions:

To RAN WG2

ACTION: RAN1 kindly asks RAN2 to take the above RAN1 recommendation into account.

3. Date of Next RAN WG1 Meetings:

RAN WG1 Meeting #122	August 25 - 29, 2025	Bengaluru (Bangalore), India

RAN WG1 Meeting #122 bis 	October 13 – 17, 2025	Prague, Czech Republic