3GPP TSG-RAN WG2 Meeting #131                                                        R2-2505175

Bengaluru, India, August 25th – 29th, 2025

Source:	CATT

Title:	Discussion on the Control Plane Procedures

Agenda Item:	8.13.3

Document for:	Discussion and Decision

Introduction

Based on the progress of discussion, most of issues had been well handled. In the current contribution, we further share some views on the key leftover issues for control plane procedure:

- Paging related leftover issues.

- Whether assistant information is needed for QoS split？

- [RRC-13] Fine tuning of extended timer values covering both Uu and PC5 hops.

Discussion

  - Paging related leftover issues

In RAN2#129 meeting, some agreements had been reached for Rel-19 multi-hop U2N relay case. Now, we focus on the below FFS:

Agreements:

An intermediate relay UE in RRC_CONNECTED releases its paging-related information in the parent UE at least for itself.  FFS whether it also releases it for the child UEs and how the child UEs receive the paging when the intermediate relay UE is in RRC_CONNECTED.

In RAN2#129bis meeting, the below agreements were reached:

Agreements:

Support the child UE relying on the parent relay UE for paging monitoring.

Strive to minimize spec impact to support intermediate relay UEs in coverage monitoring paging for a child UE on Uu interface, while avoiding duplicated paging delivery to the remote UE due to double-monitoring by upstream UEs.

The situation is similar as what we discussed in the system information part. For the RRC_IDLE/RRC_INACTIVE remote UE, there will be RRC_CONNECTED intermediate relay UE involved. Once the related intermediate relay UE enters into RRC_CONNECTED,  it can receive the paging-related information through the RRC signalling, that is to say, it should release its paging-related information in the parent UE for the child UEs if there is any. This approach can avoid the duplicated paging delivery issue easily.

Proposal 1: Once the intermediate relay UE enters into RRC_CONNECTED mode, it should release the paging-related information in the parent for its child UEs if there is any.

In Rel-17 U2N relay, the below agreement was reached:

When L2 Relay UE in RRC CONNECTED and L2 Remote UE(s) in RRC_IDLE/RRC_INACTIVE, the Relay UE can monitor PO of its PC5-RRC connected Remote UE(s) if the active DL BWP of Relay UE is configured with common CORESET and common search space.

That is to say, when the U2N relay UE is RRC_CONNECTED, it can monitor PO of its PC5-RRC connected Remote UE(s) if the active DL BWP of Relay UE is configured with common CORESET and common search space. Otherwise, the relay UE needs to provide the remote UE ID information in SidelinkUEInformation to request for paging. For Rel-19 multi-hop sidelink relay case, same principle should be followed:

Proposal 2:  When Last Relay UE in RRC CONNECTED and L2 Remote UE(s) or Intermediate Relay UE(s) in RRC_IDLE/RRC_INACTIVE, the Last Relay UE can monitor PO on behalf of each RRC_IDLE/RRC_INACTIVE intermediate relay UE or remote UE connected to it via multi-hop if the active DL BWP of Last Relay UE is configured with common CORESET and common search space.

QoS split for multi-hop relay

Regarding to the multi-hop U2N relay QoS split, in RAN2#128 meeting, the following agreement was reached for approach 1:

Agreements:

For the baseline procedure, the PDB for each hop is provided by the network according to gNB implementation.   This does not preclude the possibility of assistance information from the UE(s).

Based on the above agreement, it should further discuss whether the assistance information from the UE is needed to assistant the gNB to perform QoS split for multi-hop U2N relay.

In Rel-17 U2N Relay, the NW performs the QoS split based on its implementation, no UE assistant information is reported. In case of multi-hop, although the number of hops is higher than Rel-17 U2N relay, it is still unnecessary to report the assistance information. The main reason is that considering both the Remote UE and the Relay UE may be moving, the PC5 link quality between two UEs is not stable and can be changed dynamically. However, the QoS split is a semi-static procedure. Hence, the PC5 link channel quality is not stable and trustable to be used by gNB for QoS splitting. All in all, we don’t think UE assistance information is needed for Rel-19 multi-hop U2N relay QoS split.

Proposal 3:  UE assistance information is not needed for QoS split in Rel-19 multi-hop U2N relay.

[RRC-13] Fine tuning of extended timer values covering both Uu and PC5 hops

During the post email discussion, one leftover issue was raised that related to fine tuning of extended timer values covering both Uu and PC5 hops. There are three possible options to handle this issue shared by rapporteur:

- Option 1 -  Instead of multiplying the timers in UE-TimersAndConstantsRemoteUE  with the hop count we multiply the UE-TimersAndConstants with the hop count ( eg t300 * hop count) that includes only the Uu hop count.

- Option 2 – We introduce a new timer for a PC5 hop, e.g., t300-PC5/t301-PC5/t319-PC5 and calculate the value as t300 for n-hop relay = t300 + t300-PC5*n.

- Option 3 – We multiply the timers in  UE-TimersAndConstantsRemoteUE  with the hop count as currently captured in the CR.

Option 3 should be excluded, since this will cause unnecessarily prolonged timers in multi-hop configurations. Between Option1 and Option2, we prefer using option1, since this is more fit to use as a multiplier with simplification.Hence, we propose:

Proposal 4:  (RRC-13) RAN2 agrees that instead of multiplying the timers in UE-TimersAndConstantsRemoteUE  with the hop count RAN2 multiplies the UE-TimersAndConstants with the hop count ( eg t300 * hop count).

Conclusion

According to the analysis in section 2, it is proposed:

Paging related leftover issues

Proposal 1: Once the intermediate relay UE enters into RRC_CONNECTED mode, it should release the paging-related information in the parent for its child UEs if there is any.

Proposal 2:  When Last Relay UE in RRC CONNECTED and L2 Remote UE(s) or Intermediate Relay UE(s) in RRC_IDLE/RRC_INACTIVE, the Last Relay UE can monitor PO on behalf of each RRC_IDLE/RRC_INACTIVE intermediate relay UE or remote UE connected to it via multi-hop if the active DL BWP of Last Relay UE is configured with common CORESET and common search space.

QoS split for multi-hop relay

Proposal 3:  UE assistance information is not needed for QoS split in Rel-19 multi-hop U2N relay.

Fine tuning of extended timer values covering both Uu and PC5 hops

Proposal 4:  (RRC-13) RAN2 agrees that instead of multiplying the timers in UE-TimersAndConstantsRemoteUE  with the hop count RAN2 multiplies the UE-TimersAndConstants with the hop count ( eg t300 * hop count).

Reference

- R2-2504674	Report from session on positioning and sidelink relay	Session chair (MediaTek)	Report