3GPP TSG-RAN WG2 Meeting #131		R2-2505192

Bengaluru, India, 25th – 29th Aug, 2025

Source:	vivo

Title:		Discussion on LS on functionality activation and open issues on BM

Agenda Item:	8.1.2.2

Document for:	Discussion and Decision

# Introduction

At the RAN4#115 meeting, RAN4 discussed the AI/ML functionality activation delay and sent an LS [1] to RAN2 to clarify the interpretation.

RAN4 is discussing activation delay for AI/ML functionality activation based on the following RAN2 agreement: RAN2#129 agreements: - If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).. Companies in RAN4 have different interpretations about the following RAN2 agreement: “the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4”. - Interpretation 1: the applicable functionalities are already activated (i.e., the applicable functionalities activation is completed) before reporting applicable functionalities via RRCReconfigurationComplete in step 4 - Interpretation 2: UE starts to activate the applicable functionalities (i.e., the applicable functionalities activation is not completed) upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 RAN4 kindly requests RAN2 to clarify which interpretation is aligned with RAN2 agreements.

At the RAN2#130 meeting, RAN2 achieved the following agreements regarding LCM for the UE-sided model for the beam management use case:

Agreements - Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration (e.g. due to model in availability in the local device FFS reason to be specified) - Introduce a flag in OtherConfig indicating whether applicability reporting via UAI is enabled or disabled. Assume this applies to Option A and B, FFS if anything different needs to be done for option B (if specified) - The UE doesn’t need to measure the candidate data collection configuration(s). This will be specified in RAN2 specs. 4 For beam management, candidate data collection configuration includes at least: - CSI-ResourceConfigId of Set A - CSI-ResourceConfigId of Set B - One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements FFS the details of how this is signalled (e.g. CSIReport config or simplified signaling) Agreements 1. When UE indicates that a periodic CSI-ReportConfig is not applicable, the gNB is expected to release the configuration (i.e., UE autonomous release is not supported). 2. The UE continues to perform the inference and reporting until the configuration is released. It is up to network implementation what to do with UE reported beam predicted reporting after UE indicates that a CSI-ReportConfig is not applicable. 3. The UE shall report when CSI-ReportConfig becomes not applicable Agreements - (RRC 7) RAN2 assumes applicability report for Option B (sets of inference related parameters) can be included in both RRCReconfigurationComplete and UAI (i.e., same as Option A). This can be revisited based on RAN1 conclusions/final signaling design. - (RRC6) On how to handle RRC configuration in IDLE/INACTIVE/RLF, follow the legacy UE behaviour in TS 38.331 on whether to release or keep the RRC configuration in CSI-MeasConfig (for inference configuration) and OtherConfig (for applicability reporting and UE data collection preference configurations). FFS Whether applicability reporting via RRCReestablishmentComplete and RRCResumeComplete is supported (if it comes for free). - (RRC8) RAN2 confirm that option A and option B can be configured in the same RRCReconfiguration message with the unified applicability report procedure.

In this contribution, we will further discuss:

- Which interpretation of functionality activation is more reasonable from RAN2’s perspective, and then propose a reply to RAN4

- Remaining open issues on LCM for UE-sided model for beam management use case based on RAN2 agreements.

# Discussion

## 2.1 LS on functionality activation

At the R4#115 meeting, R4 discussed the definition related to activation delay to specify the requirements for activation latency.

Activation delay: - Activation delay requirements created for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction - It is FFS for CSI prediction - For Option A, RAN4 will send LS to RAN2 to clarify the related mechanism. - For semi-persistent CSI reporting - Activation delay starts at the reception of the MAC-CE/DCI - For aperiodic CSI reporting - Activation delay starts at the reception of the DCI

It can be observed that there is an additional activation delay from when the UE starts activation until activation is completed. For semi-persistent and aperiodic CSI reporting, the activation delay starts at the reception of the activation command (i.e., MAC CE/DCI).

- There is an additional functionality activation delay from when the UE starts activation until the activation is completed.

The R2 agreement states that ‘the UE autonomously activates the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete.’ This means that the transmission of the RRCReconfigurationComplete message is equivalent to the UE receiving the activation command, and this time point serves as the start of activation (i.e., Interpretation 2).

Furthermore, the existing RRC specification already specifies the processing delay requirements for RRC procedures. For Reconfiguration, the duration from when the UE receives the RRCReconfiguration to when it is ready to send the RRCReconfigurationComplete message shall not exceed the processing delay requirement (10 ms for RRCReconfiguration procedure). If Interpretation 1 is adopted, the processing delay requirement of RRCReconfiguration would need to be extended to include the functionality activation delay.

Table 12.1-1: UE performance requirements for RRC procedures for UEs

| Procedure title: | Network -> UE | UE -> Network | Value [ms] | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC reconfiguration | RRCReconfiguration | RRCReconfigurationComplete | 10 |  |

- If Interpretation 1 in the LS is adopted, the processing delay requirement of RRCReconfiguration procedure in RRC spec would need to be extended to include the functionality activation delay.

Based on the above analysis, we believe that Interpretation 2 is more aligned with the previous RAN2 agreement. Besides, additional spec impact in RAN2 can be avoided.

Therefore, we propose:

- Reply to RAN4 that interpretation 2 is aligned with RAN2 agreements. That is, UE starts to activate the applicable functionalities (i.e., the applicable functionalities activation is not completed) upon reporting applicable functionalities via RRCReconfigurationComplete.

## 2.2 Open issue RRC-1: Cause of inapplicability

At the RAN2#130 meeting, RAN2 reached the following agreement regarding the inapplicability cause:

Introduce “release configuration” flag instead of inapplicability cause to indicate UEs preference to release a configuration (e.g. due to model in availability in the local device FFS reason to be specified)

In the latest TS 38.331 running CR in [2], it still remains FFS:

releaseConfigurationPreference Indicates the UE’s preference to release the configuration associated to applicabilityReportConfigId (e.g. due to model unavailability), if the applicabilityStatus is set to 'inapplicable'. Editor's Note: FFS whether it is needed to capture the reason for this preference in the spec .

The purpose of the “release configuration” flag or cause is to guide the subsequent behavior of the network. We think that “release configuration” flag is sufficient to achieve the purpose, and the inapplicability cause is not needed unless it is justified.

- (RRC-1) “release configuration” flag is sufficient, and the inapplicability cause is not needed.

## 2.3 Open issue RRC-15: The time duration for an AI functionality to become available for inference when the UE reports applicability

In the email discussion [3], the RRC open issue-15 was presented by the rapporteur:

Issue description: The UE needs to synchronize the time duration for an AI functionality to become available for inference with the network when the UE reports applicability for either a full inference configuration or a set of inference parameters. Unlike conventional non-AI features, where the algorithm size is fixed, AI algorithms can vary significantly in size depending on their generalization performance and specific use cases. These sizes can range from several kilobytes to tens of megabytes. Additionally, UE implementations may use various types of memory to balance cost and performance, and these different memory types and constraints can affect the access speed of stored models. The UE should indicate the time duration for the AI functionality to become available when reporting applicability to the network. This time duration refers to how long it takes for the UE to load the AI models into RAM or another accessible memory for inference. By providing this information, the network can better understand the timeframe within which the configured radio resources and AI model can be utilized, enabling more efficient and effective resource management, especially for periodic CSI. Another option is to let RAN4 address this time duration in terms of requirements. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

The rapporteur thinks that the UE should indicate the time duration for the AI functionality to become available when reporting applicability to the network, which benefits for more efficient and effective resource management, especially for periodic CSI-RS. We understand that the intention is that the network does not transmit the periodic CSI-RS configured in the full inference configuration in the time duration for resource efficiency since the AI model is not ready. If our understanding is correct, it is strange that the NW does not transmit the periodic CSI-RS although the periodic CSI-RS has been configured to the UE. Moremove, the UE can’t immediately perform the inference when AI model is ready, because there are no measurement results of Set B, which causes the delay of performing inference, as shown in Figure 1.

Figure 1: NW does not transmit periodic CSI-RS during AI model loading

If the network transmits the periodic CSI-RS during AI model loading, the UE can measure Set B and obtain the measurement results of Set B. Once AI model is ready for inference, the inference can be immediately performed using the measurement results of Set B as model input.

Figure 2: UE measures Set B during AI model loading

Therefore, we think that the enhancement described in the open issue RRC-15 is unnecessary.

- (RRC-15) After receiving the periodic CSI-RS in the full inference configuration, the UE starts performing the measurement of set B. Once AI model is ready for inference, the inference can be immediately performed using the measurement results of Set B as model input.

If the above proposal is agreed, it is unnecessary that the network obtains the exact time duration unless it is justified.

- (RRC-15) Not support that the network obtains the exact time duration from UE reporting applicability to the AI functionality becoming available for inference, unless it is justified.

Different UE vendors may implement different time durations, e.g., due to different model sizes or using various types of memory. It is unclear whether to need to define the related requirement, e.g., the maximum time duration. However, it is RAN4’s responsibility to discuss the issue. We think that RAN2 can inform RAN4 the issue.

- (RRC-15) Send an LS to RAN4 to ask whether to need to define the requirement for the time duration.

## 2.4 Open issue RRC-16: UE behaviour when the associated ID is not provided by the network

When the associated ID is not provided by the network, RAN2 needs to discuss how the UE determines the applicability. In this paper, for the convenience of discussion, The factors influencing applicability decision can be divided into two categories: one is associated ID, the other is other conditions, including, e.g., UE-side additional conditions (internally known by UE), model availability in device and the other provided information in the configuration. If the other conditions are not fulfilled, it is very clear that the UE feedbacks inapplicable regardless of whether associated ID is provided by the network or not.

- (RRC-16) If the other conditions (e.g., UE-side additional conditions, model availability in device and the other provided information in the configuration) are not fulfilled, the UE feedbacks inapplicable regardless of whether associated ID is provided by the network or not.

The main point is how to feedback the applicability when the other conditions except associated ID are fulfilled and the associated ID is not provided by the network. We think that it is possible that a AI model (e.g., with huge size) has a great generalization performance and is applicable to all the associated ID. In this case, if the other conditions except associated ID are fulfilled and the associated ID is not provided by the network, the UE feedbacks applicable if there is at least one AI model of the functionality which is applicable to all the associated IDs.

- (RRC-16) If the other conditions are fulfilled and the associated ID is not provided by the network, the UE feedbacks applicable if there is at least one AI model of the functionality which is applicable to all the associated IDs.

If the other conditions except associated ID are fulfilled, the associated ID is not provided by the network and there is no AI model of the functionality is applicable to all the associated IDs, there are 3 alternatives:

- Alt1: The UE always feedbacks inapplicable.

- Alt2: The UE feedbacks inapplicable (in order not to activate the functionality by the UE) with indicating the applicable associated ID(s). Then the network may reconfigure the inference configuration with one of the applicable associated ID(s).

- Alt3: The UE feedbacks applicable and also activates the functionality. In this case, the network performs the management based on performance monitoring.

- (RRC-16) If the other conditions are fulfilled, the associated ID is not provided by the network and there is no AI model of the functionality which is applicable to all the associated IDs, RAN2 to discuss the following 3 alternatives:

- Alt1: The UE always feedbacks inapplicable.

- Alt2: The UE feedbacks inapplicable (in order not to activate the functionality by the UE) with indicating the applicable associated ID(s). Then the network may reconfigure the inference configuration with one of the applicable associated ID(s).

- Alt3: The UE feedbacks applicable and also activates the functionality. In this case, the network performs the management based on performance monitoring.

## 2.5 Open issue RRC-44: Whether RRCReconfigurationComplete contains the applicability/inapplicability of all inference configurations

In the email discussion [3], the rapporteur listed two interpretations:

- The RRCReconfigurationComplete contains the applicable/inapplicable status for all inference configurations. Only UAI contains updates. - The RRCReconfigurationComplete contains the applicable/inapplicable status for the inference configurations sent in the corresponding immediate previous RRCReconfiguration message and only applicability changes for those inference configurations received in earlier RRCReconfiguration messages, for which the UE already reported applicable/inapplicable at least once. UAI contains updates.

In principle, the RRCReconfigurationComplete is the reply to the configuration in the corresponding RRCReconfiguration. In other words, the UE handles the RRCReconfiguration message to produce the corresponding RRCReconfigurationComplete. Therefore,the RRCReconfigurationComplete should be only for the reply to the configuration in the corresponding RRCReconfiguration. The monitoring of applicability change should be another procedure. The two procedures should be independent and decoupled. Othewises, the complexity will increase.

- The RRCReconfigurationComplete should be only for the reply to the configuration in the corresponding RRCReconfiguration. The reconfiguration procedure and UAI procedure should be two independent/decoupled procedures.

Based on the above observation, we suggest:

- (RRC-44) The RRCReconfigurationComplete is only for initial applicability report, i.e., it only contains the applicable/inapplicable status for the inference configurations sent in the corresponding immediate previous RRCReconfiguration message. The UAI message is for applicability changes.

## 2.6 Open issue RRC-45: How/where to capture activation of periodic inference CSI-ReportConfig in specifications

In the email discussion [3], the rapporteur listed two issues:

- Where exactly in RRC to capture the solution: - In Section 5.3.5.3 when setting the content of RRCReconfigurationComplete, or - In Section 5.3.5.3 when sending RRCReconfigurationComplete to the lower layers - What to write in RRC specifications: - Upon including the applicable status in RRCReconfigurationComplete, indicate to lower layers to activate the configuration, or - Submit/do not submit the configuration to lower layers, which would be the equivalent of activation/no activation.

In legacy, there is no concept of activation/deactivation for periodic CSI-RS in both RRC layer and PHY layer, but submitting/configuring the configuration to lower layers. We think that it is unnecessary to introduce the concept of activation/deactivation for periodic CSI-RS configuration since submitting/not submitting can achieve the same effect.

- (RRC-45) Not introduce the concept of activation/deactivation for periodic CSI-RS, but use submitting/not submitting the configuration as legacy handling.

In the TS38.331, the IE levels of CSI-ReportConfig is as below:

sPCell: RRCReconfiguration->CellGroupConfig->SpCellConfig->ServingCellConfig(spCellConfigDedicated)->CSI-MeasConfig->CSI-ReportConfig

SCell: RRCReconfiguration->CellGroupConfig->SCellConfig->ServingCellConfig(sCellConfigDedicated)->CSI-MeasConfig->CSI-ReportConfig

There is no text procedure dedicated to the handling of CSI-ReportConfig, but just the general handling of the upper-level IE ServingCellConfig (the spCellConfigDedicated field for SpCell and the sCellConfigDedicated for SCell) as below:

##### 5.3.5.5.7 SpCell Configuration <ignore unrelated part> 2> if the SpCellConfig contains spCellConfigDedicated: 3> configure the SpCell in accordance with the spCellConfigDedicated; ##### 5.3.5.5.9 SCell Addition/Modification The UE shall: 1> for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition): 2> add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated; <ignore unrelated part> 1> for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification): 2> modify the SCell configuration in accordance with the sCellConfigDedicated;

To our understanding, the above yellow-highlighted part implicitly contains the action of submiting the periodic CSI-RS configuration to lower layers.

- There is no text procedure dedicated to the handling of CSI-ReportConfig, but just the general handling of the upper-level IE ServingCellConfig (spCellConfigDedicated/ sCellConfigDedicated), which implicitly contains the action of submiting the periodic CSI-RS configuration to lower layers.

If Proposal 10 is agreed, we think that the related specification impacts should be captured in Section 5.3.5.5.7 and 5.3.5.5.9 separately for SpCell and SCell. The following example is for reference:

##### 5.3.5.5.7 SpCell Configuration <ignore unrelated part> 2> if the SpCellConfig contains spCellConfigDedicated: 3> configure the SpCell in accordance with the spCellConfigDedicated with the following exception: 4> if a CSI-ReportConfig including reportQuantity-r19 set to p-cri-r19 or p-ssb-index-r19 or p-cri-RSRP-r19 or p-ssb-index-RSRP-r19, or including csi-InferencePrediction, is considered as inapplicable: 5> not configure the CSI-ReportConfig; ##### 5.3.5.5.9 SCell Addition/Modification The UE shall: 1> for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition): 2> add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated with the following exception: 3> if a CSI-ReportConfig including reportQuantity-r19 set to p-cri-r19 or p-ssb-index-r19 or p-cri-RSRP-r19 or p-ssb-index-RSRP-r19, or including csi-InferencePrediction, is considered as inapplicable: 4> not configure the CSI-ReportConfig; <ignore unrelated part> 1> for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification): 2> modify the SCell configuration in accordance with the sCellConfigDedicated with the following exception: 3> if a CSI-ReportConfig including reportQuantity-r19 set to p-cri-r19 or p-ssb-index-r19 or p-cri-RSRP-r19 or p-ssb-index-RSRP-r19, or including csi-InferencePrediction, is considered as inapplicable: 4> not configure the CSI-ReportConfig;

- (RRC-45) Specification impacts of activation of periodic inference CSI-ReportConfig are captured in Section 5.3.5.5.7 and 5.3.5.5.9 separately for SpCell and SCell. The following example is for reference:

##### 5.3.5.5.7 SpCell Configuration <ignore unrelated part> 2> if the SpCellConfig contains spCellConfigDedicated: 3> configure the SpCell in accordance with the spCellConfigDedicated with the following exception: 4> if a CSI-ReportConfig including reportQuantity-r19 set to p-cri-r19 or p-ssb-index-r19 or p-cri-RSRP-r19 or p-ssb-index-RSRP-r19, or including csi-InferencePrediction, is considered as inapplicable: 5> not configure the CSI-ReportConfig; ##### 5.3.5.5.9 SCell Addition/Modification The UE shall: 1> for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition): 2> add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated with the following exception: 3> if a CSI-ReportConfig including reportQuantity-r19 set to p-cri-r19 or p-ssb-index-r19 or p-cri-RSRP-r19 or p-ssb-index-RSRP-r19, or including csi-InferencePrediction, is considered as inapplicable: 4> not configure the CSI-ReportConfig; <ignore unrelated part> 1> for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification): 2> modify the SCell configuration in accordance with the sCellConfigDedicated with the following exception: 3> if a CSI-ReportConfig including reportQuantity-r19 set to p-cri-r19 or p-ssb-index-r19 or p-cri-RSRP-r19 or p-ssb-index-RSRP-r19, or including csi-InferencePrediction, is considered as inapplicable: 4> not configure the CSI-ReportConfig;

## 2.7 Open issue RRC-46: Whether the associated ID is cell specific or multi-cell specific

It is still FFS whether the associated ID is cell specific or multi-cell specific. Nevertheless, we think that RAN2 should not make the conclusion instead of RAN1. Now RAN2 needs to define the value range for associated ID. For the sake of assurance, we understand that RAN2 can assume multi-cell specific associated ID so far to define the value range before RAN1 does not reach the conclusion.

- (RRC-46) It should be RAN1, not RAN2 to determine whether the associated ID is cell specific or multi-cell specific. RAN2 can assume multi-cell specific associated ID so far to define the value range before RAN1 does not reach the conclusion.

## 2.8 Open issue RRC-48: Applicability reporting for inference related parameter sets after the corresponding full inference configuration is provided

In the email discussion [3], the issue is as below:

Issue description: Based on current agreements, the UE reports the applicability of both full inference configurations and corresponding inference related parameter sets, even if some of them may correspond to each other. The issue was raised that once the network provides full inference configuration for applicable inference related parameter set, it would be highly inefficient if UE keeps reporting both full inference configuration ID and inference related parameter set config ID upon change in applicability. One solution is to allow the UE to either provide the applicability of inference related parameter set or inference configuration once the inference configuration is provided based on the applicability of inference related parameter set.

As known, applicability report of full inference configuration and inference related parameter set separately corresponds to two options: Option A and Option B. It is likely that a full inference configuration is still applicable, but the corresponding inference related parameter sets become inapplicable.

- The applicability of a full inference configuration is not always consistent with the applicability of the corresponding inference related parameter sets.

Therefore, we think that the solution of providing the  applicability  of either of inference related parameter set or full inference configuration is problematic. Morever, the gain of signaling overhead is quite limited, and also it increases the complexity.

- (RRC-48) No enhancement is needed, i.e., not support “allow the UE to either provide the applicability of inference related parameter set or inference configuration”.

# Conclusion

In the contribution, we have the following observations and proposals:

Addressing RAN4 Ls on functionality activativation

- There is an additional functionality activation delay from when the UE starts activation until the activation is completed.

- If Interpretation 1 in the LS is adopted, the processing delay requirement of RRCReconfiguration procedure in RRC spec would need to be extended to include the functionality activation delay.

- Reply to RAN4 that interpretation 2 is aligned with RAN2 agreements. That is, UE starts to activate the applicable functionalities (i.e., the applicable functionalities activation is not completed) upon reporting applicable functionalities via RRCReconfigurationComplete.

Open issue RRC-1: Cause of inapplicability

- (RRC-1) “release configuration” flag is sufficient, and the inapplicability cause is not needed.

Open issue RRC-15: The time duration for an AI functionality to become available for inference

- (RRC-15) After receiving the periodic CSI-RS in the full inference configuration, the UE starts performing the measurement of set B. Once AI model is ready for inference, the inference can be immediately performed using the measurement results of Set B as model input.

- (RRC-15) Not support that the network obtains the exact time duration from UE reporting applicability to the AI functionality becoming available for inference, unless it is justified.

- (RRC-15) Send an LS to RAN4 to ask whether to need to define the requirement for the time duration.

Open issue RRC-16: UE behaviour when the associated ID is not provided by the network

- (RRC-16) If the other conditions (e.g., UE-side additional conditions, model availability in device and the other provided information in the configuration) are not fulfilled, the UE feedbacks inapplicable regardless of whether associated ID is provided by the network or not.

- (RRC-16) If the other conditions are fulfilled and the associated ID is not provided by the network, the UE feedbacks applicable if there is at least one AI model of the functionality which is applicable to all the associated IDs.

- (RRC-16) If the other conditions are fulfilled, the associated ID is not provided by the network and there is no AI model of the functionality which is applicable to all the associated IDs, RAN2 to discuss the following 3 alternatives:

- Alt1: The UE always feedbacks non-applicable.

- Alt2: The UE feedbacks non-applicable (in order not to activate the functionality by the UE) with indicating the applicable associated ID(s). Then the network may reconfigure the inference configuration with one of the applicable associated ID(s).

- Alt3: The UE feedbacks applicable and also activates the functionality. In this case, the network performs the management based on performance monitoring.

Open issue RRC-44: Whether RRCReconfigurationComplete contains the applicability/inapplicability

- The RRCReconfigurationComplete should be only for the reply to the configuration in the corresponding RRCReconfiguration. The reconfiguration procedure and UAI procedure should be two independent/decoupled procedures.

- (RRC-44) The RRCReconfigurationComplete is only for initial applicability report, i.e., it only contains the applicable/inapplicable status for the inference configurations sent in the corresponding immediate previous RRCReconfiguration message. The UAI message is for applicability changes.

Open issue RRC-45: How/where to capture activation of periodic inference CSI-ReportConfig in specifications

- (RRC-45) Not introduce the concept of activation/deactivation for periodic CSI-RS, but use submitting/not submitting the configuration as legacy handling.

- There is no text procedure dedicated to the handling of CSI-ReportConfig, but just the general handling of the upper-level IE ServingCellConfig (spCellConfigDedicated/ sCellConfigDedicated), which implicitly contains the action of submiting the periodic CSI-RS configuration to lower layers.

- (RRC-45) Specification impacts of activation of periodic inference CSI-ReportConfig are captured in Section 5.3.5.5.7 and 5.3.5.5.9 separately for SpCell and SCell. The following example is for reference:

##### 5.3.5.5.7 SpCell Configuration <ignore unrelated part> 2> if the SpCellConfig contains spCellConfigDedicated: 3> configure the SpCell in accordance with the spCellConfigDedicated with the following exception: 4> if a CSI-ReportConfig including reportQuantity-r19 set to p-cri-r19 or p-ssb-index-r19 or p-cri-RSRP-r19 or p-ssb-index-RSRP-r19, or including csi-InferencePrediction, is considered as inapplicable: 5> not configure the CSI-ReportConfig; ##### 5.3.5.5.9 SCell Addition/Modification The UE shall: 1> for each sCellIndex value included in the sCellToAddModList that is not part of the current UE configuration (SCell addition): 2> add the SCell, corresponding to the sCellIndex, in accordance with the sCellConfigCommon and sCellConfigDedicated with the following exception: 3> if a CSI-ReportConfig including reportQuantity-r19 set to p-cri-r19 or p-ssb-index-r19 or p-cri-RSRP-r19 or p-ssb-index-RSRP-r19, or including csi-InferencePrediction, is considered as inapplicable: 4> not configure the CSI-ReportConfig; <ignore unrelated part> 1> for each sCellIndex value included in the sCellToAddModList that is part of the current UE configuration (SCell modification): 2> modify the SCell configuration in accordance with the sCellConfigDedicated with the following exception: 3> if a CSI-ReportConfig including reportQuantity-r19 set to p-cri-r19 or p-ssb-index-r19 or p-cri-RSRP-r19 or p-ssb-index-RSRP-r19, or including csi-InferencePrediction, is considered as inapplicable: 4> not configure the CSI-ReportConfig;

Open issue RRC-46: Whether the associated ID is cell specific or multi-cell specific

- (RRC-46) It should be RAN1, not RAN2 to determine whether the associated ID is cell specific or multi-cell specific. RAN2 can assume multi-cell specific associated ID so far to define the value range before RAN1 does not reach the conclusion.

Open issue RRC-48: Applicability reporting for inference related parameter sets

- The applicability of a full inference configuration is not always consistent with the applicability of the corresponding inference related parameter sets.

- (RRC-48) No enhancement is needed, i.e., not support “allow the UE to either provide the applicability of inference related parameter set or inference configuration”.

# Reference

- R4-2508085 LS on AI/ML functionality activation

- R2-2505777 Introduction of AIML for NR air interface

- R2-2505778 RRC open issues for AIML for NR air interface