3GPP TSG-RAN WG2 Meeting#131	R2-2505194

Bengaluru, India, 25th – 29th Aug, 2025

Source:	vivo

Title:	Discussion on LPP-6a for AI based positioning Case1

Agenda Item:	8.1.2.3

Document for:	Discussion and Decision

- Introduction

In this contribution, with the guidance listed by Stage 3 Rapporteur, we mainly discussed LPP-6a [1] as follows.

- Discussion

Issue description: Currently, the IE NR-PRU-DL-Info provides PRU measurements for a single PRU. However, the issue was raised at RAN2#130 whether more PRUs are needed to be included for training purposes. If more PRUs are needed, a LMF can always provide multiple Provide Assistance Data messages. However, the Request Assistance Data message may need to include an indication of e.g., number of PRUs requested (if needed). Proposed resolution: Companies to provide contributions to the following meeting on whether more PRUs are needed to be included for training purposes, and if so, the corresponding signalling details.

In our understanding, it can be useful for UE to be allowed to request the exact number of PRUs for not only training but also monitoring purposes. For example, UE’s capability of model training or performance monitoring is only to handle a constant value of PRUs’ ground truth label, in this sense, it can indicate the network the exact number of PRUs it expects. The corresponding TP is provided based on the current running CR [2] as in Annex.

- (LPP-6a) UE is allowed to request the exact number of PRUs that it expects the PRU information, the value range can be FFS.

What is more, considering the signalling overhead caused by barely sending PRU information for training purpose, we think it is necessary to introduce a list of PRU information in LPP Provide Assistance data message. The possible extensions are illustrated in Annex.

- (LPP-6a) RAN2 to adopt that the NW can provide a list of PRUs’ information in one Provide Assistance Data message, considering signalling overhead to repeatedly sending LPP message.

- Conclusion

Based on the above analysis, we come to the following proposals:

- (LPP-6a) UE is allowed to request the exact number of PRUs that it expects the PRU information.

- (LPP-6a) RAN2 to adopt that the NW can provide a list of PRUs’ information in one Provide Assistance Data message, considering signalling overhead to repeatedly sending LPP message.

- Reference

- R2-2505702	LPP open issues for feature "AI/ML for NR air interface"		Qualcomm Incorporated

- R2-2505704	Introduction of AI/ML Positioning Accuracy Enhancements	Qualcomm Incorporated

- Annex

#### –	NR-DL-AIML-ProvideAssistanceData

The IE NR-DL-AIML-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-based DL AI/ML positioning. It may also be used to provide NR DL AI/ML positioning specific error reason.

-- ASN1START

NR-DL-AIML-ProvideAssistanceData-r19 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r19		NR-DL-PRS-AssistanceData-r16		OPTIONAL,	-- Need ON

nr-SelectedDL-PRS-IndexList-r19		NR-SelectedDL-PRS-IndexList-r16		OPTIONAL,	-- Need ON

nr-On-Demand-DL-PRS-Configurations-r19

NR-On-Demand-DL-PRS-Configurations-r17

OPTIONAL,	-- Need ON

nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r19

NR-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17																				OPTIONAL,	-- Need ON

assistanceDataValidityArea-r19		AreaID-CellList-r17					OPTIONAL,	-- Need ON

nr-PositionCalculationAssistance-r19

NR-PositionCalculationAssistance-r16

OPTIONAL,	-- Need ON

nr-DL-AIML-Positioning-Error-r19	NR-DL-AIML-Positioning-Error-r19	OPTIONAL,	-- Need ON

nr-PRU-InfoList-r19					NR-PRU-InfoList-r19				OPTIONAL,	-- Need ON

...

}

NR-PRU-InfoList-r19::= SEQUENCE (SIZE(1..maxNrofPRUPerList-r19)) OF NR-PRU-DL-Info-r18	OPTIONAL

-- ASN1STOP

#### –	NR-DL-AIML-RequestAssistanceData

The IE NR-DL-AIML-RequestAssistanceData is used by the target device to request assistance data from a location server for NR DL AI/ML positioning.

-- ASN1START

NR-DL-AIML-RequestAssistanceData-r19 ::= SEQUENCE {

nr-PhysCellID-r19								NR-PhysCellID-r16					OPTIONAL,

nr-DL-PRS-AssistanceDataReq-r19					ENUMERATED { requested }			OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Req-r19			ENUMERATED { eAoD, eAoA }			OPTIONAL,

nr-on-demand-DL-PRS-Req-r19						NR-On-Demand-DL-PRS-Request-r17		OPTIONAL,

pre-configured-AssistanceDataReq-r19			ENUMERATED { true }					OPTIONAL,

nr-PositionCalculationAssistanceReq-r19			BIT STRING {

trpLoc					(0),

beamInfo				(1),

rtdInfo					(2),

losNlosInfo				(3),

trpTEG-Info				(4),

nr-IntegrityBounds		(5),

pruInfo					(6),

trpLoc-Implicit			(7)

} (SIZE (1..8))						OPTIONAL,

nr-IntegrityAssistanceReq-r19					BIT STRING {

serviceParameters		(0),

serviceAlert			(1),

riskParameters			(2),

integrityParaTRP-Loc	(3),

integrityParaBeamInfo	(4),

integrityParaRTD-Info	(5)

} (SIZE (1..8))						OPTIONAL,

NrofPruReq-r19									Integer (1..x)				OPTIONAL,

...

}

-- ASN1STOP