3GPP TSG-RAN WG2 Meeting #131                                                                     	R2-2505214

Bengaluru, India, Aug 25th – 29th, 2025

Source:	CATT

Title:	Issues to address for AI/ML Positioning stage-2

Agenda Item:	8.1.2.3

Document for:	Discussion and Decision

- Introduction

During the preparation of 38.305 AI/ML positioning running CR, some issues are identified for further RAN2 discussion. In this contribution, the corresponding resolutions are provided from our perspective.

- Discussion

  - Open issue 1: applicability reporting

Regarding applicability reporting discussed for BM use case, RAN2 has made the following agreements:

RAN2#127 Agreements on procedures - Step 1: Network sends UECapabilityEnqiry message to initiate the procedure to a UE reporting its AI/ML supported functionalities. - Step 2: UE sends UECapablityInformation message to network, containing supported functionalities at the UE side. - “Step 3”: Following configurations are provided from NW to UE: 1) UE is allowed to do UAI reporting via OtherConfig. 2) Network may provide NW-side additional condition. FFS on the RRC signalling and whether it is mandatory or optional. 3) FFS on configuration (e.g. inference configuration) of supported functionalities. FFS on the content of configuration. - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration). FFS how the applicable functionality is decided if NW-side additional condition is not provided in step 3. <other part is omitted>

- RAN2#129 Agreements applicability reporting and management Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. FFS if we report explicit cause

Although there are some progress in the aspect of applicability reporting for positioning Case 1, i.e., the baseline procedure includes proactive and reactive reporting as shown below. The detail of how UE determines whether a functionality is applicable, and the detailed content in initial report and subsequent reports, are still missing.

RAN2#128 Agreements

1	For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline.

- 	Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF .

- 	Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF.  Configuration details are FFS

Since the RAN2 intention is to have the same design for BM case and positioning case as much as possible, the agreements made for BM applicability reporting can also be applicable to positioning case 1.

(LPP-19)Proposal 1: the following agreements made for BM applicability reporting is also applicable to positioning case 1:

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

- Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.

  - Open issue 2: RAN3 stage-2 input for AI/ML positioning Case 1

As part of the UE-based AI/ML positioning method (i.e., AI/ML positioning Case 1), the Information that may be transferred from the gNB to LMF and Procedures between LMF and gNB need to be specified. Since currently RAN3 is working on Case 3a/3b, and doesn’t focus on Case 1, in current running CR we just reuse the corresponding description for DL-TDOA. But it’s necessary to contact RAN3 to let them check whether there is any necessary change, so a RAN2 LS may be needed.

Proposal 2: check with RAN3 whether reusing DL-TDOA “Information that may be transferred from the gNB to LMF” and “Procedures between LMF and gNB” for AI/ML positioning Case 1 is doable.

- Conclusion

In this contribution, we provide our views on the open issues for AI/ML positioning Case 1, and the corresponding proposals are as below:

(LPP-19)Proposal 1: the following agreements made for BM applicability reporting is also applicable to positioning case 1:

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

- Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.

Proposal 2: check with RAN3 whether reusing DL-TDOA “Information that may be transferred from the gNB to LMF” and “Procedures between LMF and gNB” for AI/ML positioning Case 1 is doable.

- References

- RP-243244, Revised WID for NR_AIML_air, Qualcomm