3GPP TSG-RAN WG2 Meeting #131	R2-2505243

Bengaluru, India, Aug. 25th– 29th, 2025

Agenda Item:	8.11.2

Source:	OPPO, ZTE Corporation, Sharp, NEC, NTT DOCOMO INC., Qualcomm Incorporated, CATT

Title:	Power ramping issue on the RO type fallback

Document for:	Discussion, Decision

# Introduction

According to the discussion on the RACH configuration for SBFD symbols, the network can provide two separate RACH configurations, i.e. one for the legacy RO and another for the additional RO. In this contribution, we discuss the issue on the uplink transmission power upon the RO type fallback.

# Discussion

## Power ramping issue

TS 38.331: ### 5.1.1a Initialization of variables specific to Random Access type … 2> if RA_TYPE is switched from 2-stepRA to 4-stepRA during this Random Access procedure: 3> set POWER_OFFSET_2STEP_RA to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (MSGA_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP). ### 5.1.3 Random Access Preamble transmission … 1> set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA;

According to the MAC specification [1] as quote above for the fallback between 2-step RACH and 4-step RACH, when the UE is configured with two different power ramping steps, and the preamble transmission switches between two different power ramping steps, the UE needs to add a power offset (i.e. POWER_OFFSET_2STEP_RA) to compensate the uplink transmission power difference caused by the different power ramping steps.

Running 38.331 CR of SBFD:

According to the running 38.331 CR [1] as quote above, when two separate RACH configurations (i.e. rach-ConfigCommon for the legacy RO and sbfd-AdditionalRACH-Config for the additional RO) of RACH configuration Option 2 for SBFD are provided, the UE is configured with two different powerRamingStep(s) (i.e. PreamblePowerRampingSteplegacy-RO  and preambleReceivedTargetPoweradditional-RO for legacy RO and additional RO respectively). When the UE fallbacks between the legacy RO and the additional RO, the power offset needs to be added to compensate the preamble transmission power difference between the legacy RO and the additional RO, similar as the fallback from 2-step RACH to 4-step RACH.

Poffset=（N-1）×powerRampingStepadditional-RO-powerRampingSteplegacy-RO for fallback from additional RO to legacy RO or

Poffset=（N-1）×powerRampingSteplegacy-RO-powerRampingStepadditional-RO for fallback from legacy RO to additional RO

Proposal: For the RO type fallback between legacy RO and additional RO, a power offset given by the difference between the two quantities of preamble power ramping steps is added.

# Conclusion

In this contribution, we analyse the issue on the preamble transmission power upon the fallback between the legacy RO and additional RO, when two RACH configurations with two separate powerRampingStep configurations are provided. To avoid the issues on the coverage and the interference caused by the RO type fallback, we think that a power offset given by the difference between the two quantities of preamble power ramping steps needs to be added.

Proposal: For the RO type fallback between legacy RO and additional RO, a power offset given by the difference between the two quantities of preamble power ramping steps is added.

# Reference

[1] 3GPP TS38.321, “Technical Specification Group Radio Access Network; NR; Medium Access Control (MAC) protocol specification”.