3GPP TSG-RAN WG2 Meeting #131	R2-2505278

Bangalore, India, 25th – 29th Aug. 2025

Source: 	Xiaomi

Title:  	Remaining issues of conditional LTM

Agenda Item:	8.6.4

Document for:	Discussion and Decision

# Introduction

In the WID [1], one of the objectives is to specify support for conditional Intra-CU LTM, as shown below.

- Specify support of conditional Intra-CU LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Limit specifying the conditional LTM to the scenario where the UE is in non-DC

In this contribution, we provide our understandings on conditional intra-CU LTM.

# Discussion

  - MAC open issues

In the list of MAC open issues for mobility [2], following open issues are provided for CLTM, as shown below:

| MAC-14 | FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received - for case 1: LTM cell switch target cell is CLTM candidate cell Editor’s NOTE: FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received. [Rapp]: New added based on Anil’s comments. | Issue Type: Not essential but important How to address it: based on companies’ contribution |
| --- | --- | --- |
| MAC-28 | During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, if UE based TA is valid, whether/how to fallback. [Rapp]: New added based on Lenovo’s comments. | Issue Type: Not essential How to address it: based on companies’ contribution |
| MAC-29 | When CLTM candidate configuration is reconfigured/released, whether/how to handle the stored TA value and the running TAT for the corresponding candidate cell. [Rapp]: New added based on Offinno’s comments. | Issue Type: Not essential How to address it: based on companies’ contribution |
| MAC-30 | If RACH-less CLTM fallback to RACH-based CLTM is declared due to PTAG expiration, how does UE obtain the MAC PDU from HARQ buffer to Msg3/A buffer. [Rapp]: New added based on OPPO’s comments. | Issue Type: Not essential How to address it: based on companies’ contribution |
| MAC-31 | Upon the CLTM execution, whether activated candidate TCI state(s), other than the TCI state associated with the triggered beam, should be deactivated or not. [Rapp]: New added based on vivo’s comments. | Issue Type: Not essential How to address it: based on companies’ contribution |

(MAC-14) FFS for the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received.

For the case when CLTM candidate TAT timer is running, an (Enhanced) LTM Cell switch Command MAC CE is received, one open issue is whether the valid TA stored by UE for the candidate cell can be used for MAC CE triggered LTM, and two options can be discussed:

- Option 1: The valid CLTM TA stored by UE can be used for MAC CE triggered LTM, e.g., if candidate LTM TAT timer is running; and TAC is set as FFF in (Enhanced) LTM Cell Switch Command MAC CE; and the UE has not measured the Timing Advance, the UE can perform RACH-less LTM Cell switch (using the CLTM TA value stored by UE).

- Option 2: The valid CLTM TA stored by UE cannot be used for MAC CE triggered LTM, e.g., if candidate LTM TAT timer is running; and TAC is set as FFF in (Enhanced) LTM Cell Switch Command MAC CE; and the UE has not measured the Timing Advance, the UE performs RACH-based LTM Cell switch.

For option 1, there are three methods (i.e., TA indicated in LTM CSC, UE measured TA, and TA stored for CLTM) for the candidate TA acquisition for LTM Cell switch. If we support option 1, RAN2 needs to discuss the priorities of these methods, which may result in additional work. Furthermore, both CLTM TA and TA in LTM CSC are indicated by the source cell. The source cell and the UE have the same understanding on the validity of candidate TA. It is simple to include the same TA value in LTM CSC as that maintained by the UE for the candidate cell. Hence, to support option 1 is not necessary.

Option 2 is aligned with the legacy UE behaviour of MAC CE triggered LTM. If an (Enhanced) LTM Cell switch Command MAC CE is received, the UE shall ignore the stored candidate TA. We prefer to support option 2, since it has no specification impact.

Proposal 1: (MAC-14) The valid CLTM candidate TA stored by UE cannot be used for MAC CE triggered LTM.

(MAC-28) During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, if UE based TA is valid, whether/how to fallback.

For RACH-less CLTM, RAN2 has greed “During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, fallback to RACH-based CLTM.” Based on the agreement, whether the UE based TA is valid or not, the UE shall fallback to RACH-based CLTM. It seems unnecessary to carry out further optimization for the case, i.e.., during CLTM is ongoing, after the first transmission, the UE doesn’t need to consider whether the UE based TA is valid or not.

Proposal 2: (MAC-28) During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, irrespective of whether the UE based TA is valid or not, fallback to RACH-based CLTM.

(MAC-29) When CLTM candidate configuration is reconfigured/released, whether/how to handle the stored TA value and the running TAT for the corresponding candidate cell.

For the open issue, when CLTM candidate configuration is released, the associated candidate PCell has been removed. When CLTM candidate configuration is reconfigured, the associated candidate PCell might have been changed. For these cases, the stored TA value and the running TAT for the corresponding candidate cell are useless. Hence, the UE shall remove the stored TA value and stop the TAT.

Proposal 3: (MAC-29) When CLTM candidate configuration is reconfigured/released, the UE shall remove the stored TA value and stop the running TAT for the corresponding candidate cell.

(MAC-30) If RACH-less CLTM fallback to RACH-based CLTM is declared due to PTAG expiration, how does UE obtain the MAC PDU from HARQ buffer to Msg3/A buffer.

During the RACH-less CLTM execution, the Msg3/A buffer is empty. And the MAC PDU from the HARQ buffer consists of RRCReconfigurationComplete message. When the UE fallbacks to RACH-based CLTM, the Msg3/A buffer may not have enough size to include the RRCReconfigurationComplete message and the RRCReconfigurationComplete message can be re-transmitted after the competition of the RACH procedure.

Proposal 4: (MAC-30) If RACH-less CLTM fallback to RACH-based CLTM is declared due to PTAG expiration, the RRCReconfigurationComplete message can be re-transmitted after the competition of the RACH procedure.

(MAC-31) Upon the CLTM execution, whether activated candidate TCI state(s), other than the TCI state associated with the triggered beam, should be deactivated or not.

In Rel-18, upon LTM cell switch execution, activated candidate TCI state(s), other than the TCI state associated with the triggered beam, should be deactivated. For Rel-19 CLTM, UE shall follow the same behaviours.

Proposal 5: (MAC-31) Upon the CLTM execution, activated candidate TCI state(s), other than the TCI state associated with the triggered beam, should be deactivated.

  - RRC open issues

In the list of RRC open issues for mobility [3], two open issues are provided for CLTM, as shown below:

| Company (Name) | Open issue | Justification | Rapporteur comment |
| --- | --- | --- | --- |
| Fujitsu, NTT DOCOMO | NW should be able to distinguish between CLTM and fast recovery after CLTM failure in order to deal with SN gap caused by SRB COUNT continuation when UE is performing fast recovery. If reordering timer (t-Reordering) on NW-side is set for a long duration, because NW must wait for the duration before receiving RRCReconfigurationComplete in RRC layer and sending contention resolution to the UE, T304 in the UE may expire, which results in fast recovery failure. In Rel-18, RAN2 agreed to deal with SN gap by NW implementation, and that can be done because LTM and fast recovery can be distinguished. However, in Rel-19, CLTM and fast recovery after CLTM failure cannot be distinguished, which is an open issue. | This issue is critical for operators which configures long reordering timer. If this is not resolved, this leads to failure of fast recovery after CLTM and interruption time longer than the duration of reordering timer. | The understanding is that this could be an issue only when the value of the reordering timer is long, but now I am wondering if we really need to fix this. Anyhow, I would encorauge the companies to bring a contribution to the next meeting and also clarify which are the solutions on the table. |
| LGE | Based on the lateast RRC running CR (i.e., v22), L1 condition for CLTM evaluation is associated with an event for L1 MR (i.e., EventLTM3/4/5). This implies that both L1 MR and CLTM execution are simultaneously triggered if the event is satisfied. To avoid simultaneous trigger of L1 MR and CLTM execution, it is good to introduce separate events for CLTM evaluation as in CHO (i.e., CondEventLTM3/4/5). | If this open issue is not addressed, UE triggers both L1 MR and CLTM execution simultaneously when the event associated with L1 execution condition is being satisfied. | This issue was already discussed in the WI and we have agreed to not introduce separate events for the case of CLTM. I think this can be easily solved in the MAC CR discussion. I don’t expect any contribution about this in RRC, because this is not really an RRC problem but a MAC issue (i.e., how to fix the procedure in MAC). |

(Issue form Fujitsu, NTT DOCOMO) How to deal with SN gap during CLTM based failure recovery.

The issue can be solved by network implementation.

Based on the network implementation, the network may know whether the UE is performing CLTM based failure recovery. For CLTM based failure recovery, only CBRA is supported. If the selected candidate cell has been configured with CFRA resource or has the valid TA in network, but the UE is performing CBRA to access the selected candidate cell, the network can infer the procedure is for recovery. When the CFRA resource was not configured for the CLTM candidate cell, at least, the network can know the UE is performing CBRA for reconfiguration with sync procedure. In network side, the delivery of PDCP SDU(s) to RRC layer is up to implementation. When the network (PDCP layer) has realized the PDCP packet is for handover (reconfiguration with sync), the PDCP layer can provide the data to RRC layer rapidly and ignore the reordering timer (t-Reordering) on NW-side. Hence, as in Rel-18, the issue can also be solved by network implementation.

Proposal 6: It is up to network implementation to deal with SN gap during CLTM based failure recovery.

(Issue form LGE) How to avoid simultaneous trigger of L1 MR and CLTM execution.

As the description for the open issue, based on the RRC and MAC running CR. when a L1 CLTM execution condition is fulfilled, the associated L1 MR will be initiated. However, when CLTM execution procedure is ongoing, it is unnecessary to perform the corresponding L1 MR.

After the triggering of the CLTM execution, the UE detaches from the source cell, so the UE cannot provide the triggered L1 MR to the source cell. During the CLTM execution, the UE performs the MAC Reset, and the triggered L1 MR is cancelled. Hence, the UE cannot provide the triggered L1 MR to the target cell.

Based on the above analysis, even if the L1 MR and CLTM are triggered simultaneously, the UE cannot report the L1 MR to the network. It seems that this open issue has been resolved in the MAC.

Observation 1: The open issue “how to avoid simultaneous trigger of L1 MR and CLTM execution” has been handled by MAC CR as part of MAC reset. Based on the MAC CR, when triggering the CLTM execution, the triggered L1 MR cannot be reported to the network, because the UE performs the MAC Reset and the triggered L1 MR is cancelled during the CLTM execution.

  - CLTM TA

The issue when the number of TAs maintained by UE is up to the UE capability

In RAN2#129bis meeting, RAN2 has agreed to define a new per UE capability for UE support of early TA MAC CE reception for CLTM and the max number of maintaining TA values. The value range is (1~8), which has been captured in the running CR of UE capability [4][5].

In Draft 306 [4]: In Draft 331[5]:

As captured in the current Running MAC CR [6], when an LTM Candidate Timing Advance Command MAC CE is received, the UE shall store the TA value and start or restart the associated ltm-Candidate-TimeAlignmentTimer.

The maximum number of CLTM candidate is 8 and CLTM with Rel-18 MIMO 2TA is supported. So, the most extreme case is the network would like to provide the TA value for each TAG id of each candidate via LTM Candidate Timing Advance Command MAC CE and the UE may need to store up to 16 TA values.

However, the maximum number of TA values that the UE can store is up to the UE capability cltm-EarlyTA-Indication-r19 and the value range of cltm-EarlyTA-Indication-r19 is 1~8. Moreover, C-LTM TAT for target cell and candidate cell is not stopped upon C-LTM execution. Therefore, it is highly possible that the number of TAs that the UE stores is up to the value of cltm-EarlyTA-Indication-r19 very soon.

In this case (when the number of TAs that the UE stores is up to cltm-EarlyTA-Indication-r19), if the network want to add a new CLTM TA to be maintained by UE (the UE needs to start a new CLTM TAT for the new TA), how to handle it.

As shown in the Figure 1. The maximum number of maintaining TA values is 4 and UE has stored 4 TAs for cell 1~4. The network wants to add a new TA for Cell 5 to UE, but now, UE cannot store 5 TAs at the same time. How to handle it.

Figure 1

The issue can be solved from two different perspectives.

For the network-based method：

The network can avoid transmitting the LTM Candidate TAC to UE when the number of stored CLTM TAs is equal to cltm-EarlyTA-Indication-r19. Before transmitting the new TA value, the network can remove CLTM TA and then send the LTM Candidate TAC with the new TA value. Without the network triggered CLTM TA remove procedure, if the network intends to send a new TA value for a better candidate, the network must wait for one CLTM TAT to expire.

As shown in the Figure 2. The maximum number of maintaining TA values is 4 and UE has stored 4 TAs for cell 1~4. The network wants to add a new TA for Cell 5 to UE. First, the network can send a message to UE to remove Cell1 TA, and then send a message to add Cell5 TA.

Figure 2

In current Running MAC CR [6], the network can only add or modify the CLTM TA stored by UE, but the network cannot remove the CLTM TA stored by UE. For Rel-18 LTM, the TA value in LTM CSC can be set to “FFF”. If the TA value is set to “FFF”, this field indicates that no valid timing adjustment is available for the PTAG of the LTM target cell. The similar principle can be used for the network to remove CLTM TA, the network can set the TA value in LTM Candidate TAC to “FFF” to indicate that no valid TA is available for the indicated candidate cell (for the indicated TAG). If the UE receives the LTM Candidate Timing Advance Command MAC CE and the TA value in it is “FFF”, the UE shall remove the TA value for the indicated candidate cell (for the indicated TAG) and stop associated CLTM TAT.

For the UE-based method:

In this case, the UE shall automatically remove one maintained CLTM TA and stop associated TAT, and then maintain the new TA value indicated in the received LTM Candidate TAC.

For the UE based method, the key issue is which CLTM TA shall be removed. One simple method is to remove the oldest CLTM TA and the oldest TA is the one among all the TAs stored by UE that received the associated CLTM TAC earliest.

As shown in the Figure 2. The maximum number of maintaining TA values is 4 and UE has stored 4 TAs for cell 1~4. The network wants to add a new TA for Cell 5 to UE. First, the network can send a message to UE to add Cell5 TA. When the UE receives the message, the UE removes the oldest TA and stores the Cell5 TA.

Figure 3

Based on the above analysis, both the network-based method and the UE-based method can be considered, and we provided the P7.

Proposal 7: When the number of CLTM TAs that the UE stores is up to cltm-EarlyTA-Indication-r19, to adding a new CLTM TA to be maintained by the UE, the following options can be considered.

- Option 1: (Network-based method) Before sending the LTM Candidate Timing Advance Command to add a new CLTM TA, the network can remove one CLTM TA by setting the TAC field to “FFF” in LTM Candidate Timing Advance Command MAC CE.

- Option 2: (UE-based method) Upon receiving the LTM Candidate Timing Advance Command MAC CE for adding a new CLTM TA, the UE shall remove the oldest CLTM TA and stop associated CLTM TAT.

  - CLTM based recovery

Whether to support RACH-less and CFRA for CLTM based recovery:

For CLTM based failure recovery, RAN2 has reached the following agreement:

•	Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

Compared to the network triggered LTM, when the UE performs CLTM based failure recovery, the UE may have the valid TA or CFRA resource for the selected candidate cell. Technically, RACH-less and CFRA for CLTM based recovery may work. Fast failure recovery is optimization for mobility and we may not have enough time to support these enhancements and discuss some details. Hence, for CLTM based recovery, only support CBRA based failure recovery.

Proposal 8: In Rel-19, RAN2 doesn’t support RACH-less and CFRA for CLTM based recovery.

Whether to define a separate UE capability for CLTM fast recovery:

In the report for UE capabilities [7], one open issue is whether to define a separate UE capability for CLTM fast recovery.

If P8 is agreed, the UE behaviour to apply CLTM candidate configuration for CLTM recovery seems to be the same as Rel-18 LTM recovery. And we don’t need to define a per UE capability(ies) for CLTM fast recovery.

Proposal 9: No need to define a separate UE capability for CLTM fast recovery (i.e., reuse the UE capability for Rel-18 intra-CU LTM fast recovery).

  - The coexistence of CLTM and other mobility cases

In Rel-18 intra-CU LTM and Rel-19 inter-CU LTM, RAN2 has reached the following agreements for the coexistence of LTM and L3 Mobility：

For LTM and L3 HO/CHO: RAN2 #127 (Rel-19 MOB) agreement [9]: L3 mobility (including both the network triggered L3 HO and CHO) can be configured to UE, while the inter-CU LTM is configured (w/o DC), and the following items can be considered (follow Rel-18 intra-CU LTM): - When performing the L3 mobility (HO or CHO), the UE does not autonomously release inter-CU LTM configurations, unless these are explicitly released by the network. - The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations. - For the execution order between CHO and LTM, Rel-18 principle is applied. RAN2#128(Rel-18 MOB) agreement [10]: Clarify in RRC that the conditional reconfiguration cannot include the ltm-config. For DAPS: RAN2#125(Rel-18 MOB) agreement [11]: - The coexistence of LTM and DAPS HO is not supported.

For CLTM and L3 HO:

For the coexistence of CLTM and L3 HO, we can follow the Rel-19 inter-CU LTM agreement and support the coexistence of CLTM and L3 HO. When performing the L3 HO, the UE does not autonomously release CLTM configurations, unless these are explicitly released by the network and the L3 HO Command (RRCReconfiguration message) may reconfigure CLTM configurations.

Proposal 10: The coexistence of CLTM and L3 HO is supported in Rel-19.

For CLTM and CHO:

In Rel-18 and Rel-19, the coexistence of MAC CE triggered LTM and CHO is supported, and RAN2 has agreed “Network can send an LTM Cell Switch Command MAC CE indicating a CLTM candidate configuration”. So, the coexistence of CLTM and LTM is supported. To follow the regular LTM, the coexistence of CLTM and CHO can be considered.

But for the coexistence of LTM and CHO, the agreement in RAN2 #127 (Rel-19 MOB) and the agreement in RAN2#128(Rel-18 MOB) are in conflict.

Observation 2: For the coexistence of LTM/CLTM and CHO, the agreement in RAN2 #127 (Rel-19 MOB) and the agreement in RAN2#128(Rel-18 MOB) are in conflict:

- The agreement in RAN2 #127 (Rel-19 MOB): The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- The agreement in RAN2#128(Rel-18 MOB): Clarify in RRC that the conditional reconfiguration cannot include the ltm-config.

Because the Rel-18 LTM agreement is newer than the Rel-19 LTM agreement, we need to follow the clarification for Rel-18 LTM that the conditional reconfiguration cannot include the ltm-config. Upon CHO execution, the serving cell changed and the Rel-18 ID of serving cell, Rel-19 ID of serving cell, UE based TA ID of serving cell and CLTM execution conditions need to be updated according to the CHO target cell. Without the LTM configuration update, the LTM and CLTM cannot work correctly. But the ltm-config cannot be updated during CHO execution based on the Rel-18 LTM agreement.

Observation 3: Because the Rel-18 LTM agreement is newer than the Rel-19 LTM agreement, we need to follow the clarification for Rel-18 LTM that the conditional reconfiguration cannot include the ltm-config.

For the coexistence of CHO and MAC CE triggered LTM, there is no big problem. The network can send a new RRCReconfiguration to reconfigure the LTM configuration after CHO execution. Before the LTM configuration update, the network shall not trigger LTM Cell Switch.

For the coexistence of CHO and CLTM, the issue has been solved by the procedure in RRC Running CR. In RRC CR, upon L3 handover, CHO and LTM cell switch execution, the UE shall stop CLTM evaluation for all candidates. After CHO execution, the network can reconfigure the CLTM execution condition(s), and the UE can restart the evaluation of CLTM. Hence, RAN2 can confirm the coexistence of CLTM and CHO is supported

Proposal 11: The coexistence of CLTM and CHO is supported.

For CLTM and DAPS:

For the coexistence case of CLTM and DAPS, we can follow the Rel-18 LTM.

Proposal 12: The coexistence of CLTM and DAPS HO is not supported.

# Conclusions

According to the analysis given above, we have the following observations and proposals:

2.1	MAC open issues

Proposal 1: (MAC-14) The valid CLTM candidate TA stored by UE cannot be used for MAC CE triggered LTM.

Proposal 2: (MAC-28) During CLTM is ongoing, after the first transmission, if TAT timer expires while RACH-less LTM is ongoing, irrespective of whether the UE based TA is valid or not, fallback to RACH-based CLTM.

Proposal 3: (MAC-29) When CLTM candidate configuration is reconfigured/released, the UE shall remove the stored TA value and stop the running TAT for the corresponding candidate cell.

Proposal 4: (MAC-30) If RACH-less CLTM fallback to RACH-based CLTM is declared due to PTAG expiration, the RRCReconfigurationComplete message can be re-transmitted after the competition of the RACH procedure.

Proposal 5: (MAC-31) Upon the CLTM execution, activated candidate TCI state(s), other than the TCI state associated with the triggered beam, should be deactivated.

2.2	RRC open issues

Proposal 6: It is up to network implementation to deal with SN gap during CLTM based failure recovery.

Observation 1: The open issue “how to avoid simultaneous trigger of L1 MR and CLTM execution” has been handled by MAC CR as part of MAC reset. Based on the MAC CR, when triggering the CLTM execution, the triggered L1 MR cannot be reported to the network, because the UE performs the MAC Reset and the triggered L1 MR is cancelled during the CLTM execution.

2.3 CLTM TA

Proposal 7: When the number of CLTM TAs that the UE stores is up to cltm-EarlyTA-Indication-r19, to adding a new CLTM TA to be maintained by the UE, the following options can be considered.

- Option 1: (Network-based method) Before sending the LTM Candidate Timing Advance Command to add a new CLTM TA, the network can remove one CLTM TA by setting the TAC field to “FFF” in LTM Candidate Timing Advance Command MAC CE.

- Option 2: (UE-based method) Upon receiving the LTM Candidate Timing Advance Command MAC CE for adding a new CLTM TA, the UE shall remove the oldest CLTM TA and stop associated CLTM TAT.

2.4 CLTM based recovery

Proposal 8: In Rel-19, RAN2 doesn’t support RACH-less and CFRA for CLTM based recovery.

Proposal 9: No need to define a separate UE capability for CLTM fast recovery (i.e., reuse the UE capability for Rel-18 intra-CU LTM fast recovery).

2.4 The coexistence of CLTM and other mobility cases

Proposal 10: The coexistence of CLTM and L3 HO is supported in Rel-19.

Observation 2: For the coexistence of LTM/CLTM and CHO, the agreement in RAN2 #127 (Rel-19 MOB) and the agreement in RAN2#128(Rel-18 MOB) are in conflict:

- The agreement in RAN2 #127 (Rel-19 MOB): The RRCReconfiguration message to execute an L3 mobility (HO or CHO) procedure may reconfigure inter-CU LTM configurations.

- The agreement in RAN2#128(Rel-18 MOB): Clarify in RRC that the conditional reconfiguration cannot include the ltm-config.

Observation 3: Because the Rel-18 LTM agreement is newer than the Rel-19 LTM agreement, we need to follow the clarification for Rel-18 LTM that the conditional reconfiguration cannot include the ltm-config.

Proposal 11: The coexistence of CLTM and CHO is supported.

Proposal 12: The coexistence of CLTM and DAPS HO is not supported.

# Reference

- RP-242356, “Revised WID: NR mobility enhancements Phase 4”, Apple Inc, China Telecom

- R2-25xxxxx, “List of open issues for mobility phase4”, Ericsson

- R2-25xxxxx, “Discussion summary and list of MAC open issue for Mob Ph4”, vivo

- R2-250xxxx, “Draft 306 running CR for UE capability for Mob Ph4”, CATT

- R2-250xxxx, “Draft 331 running CR for UE capability for Mob Ph4”, CATT

- R2-250xxxx, “Running MAC CR for enhanced mobility Ph4”, vivo

- R2-25xxxxx, “Report of [POST130][120][MOB] (CATT)”, CATT

- 3GPP TS 38.321

- R2-2409210, “Report of 3GPP TSG RAN WG2 meeting #127”

- R2-2500002, “Report of 3GPP TSG RAN WG2 meeting #128”

- R2-2403994, “Report of 3GPP TSG RAN WG2 meeting #125”