3GPP TSG-RAN WG2 Meeting #131	R2-2505315

Bangalore, India 25th –29th , 2025

Agenda Item:	8.5.2

Source: 	Xiaomi

Title:  	Remaining open issues on on-demand SSB

Document for:	Discussion and Decision

1	Introduction

In last meeting, we discussed OD-SSB and some agreements were achieved. During CR implementation, some open issues are identified, as listed below.

-	Issue 1: Number of OD-SSB configuration

-	Issue 2: RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst

-	Issue 3: Granularity of servingCellMO

In this contribution, we would like to further discuss the remaining open issues and have corresponding observations and proposals.

2	Discussion

2.1 RRC-1: Number of OD-SSB configuration

In last meeting, we reached the following agreement on OD-SSB configuration index.

- (RRC-1) The configuration index is associated with a set of parameters, i.e., od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

According to the parameter list from RAN1, the candidate values of the three parameters are listed as below.

| od-ssb-Periodicity | ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 } |
| --- | --- |
| od-ssb-PositionsInBurst | Same as ssb-PositionsInBurst provided in ServingCellConfigCommon, i.e., choice of 4-bit shortBitmap, 8-bit mediumBitmap, or 64-bit longBitmap |
| od-ssb-nrofBurst | INTEGER (1..8) |

Therefore, for od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst, the number of candidate values are 6, 264 and 8. In this case there are at most 6*264*8 candidate combinations, which requires nearly 270 configuration indexes, i.e., 70 bits, and is too many. Therefore, we suggest to send LS to ask RAN1 about the possible candidate combinations before determining the number of the configuration index.

Proposal 1: (RRC-1) RAN2 to send LS to RAN1 to ask about the possible candidate value combinations of od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

In addition, in the existing running CR, the configuration index is configured within OD-SSB-Config but associated with all the configured parameters within OD-SSB-Config. Actually, based on RAN1 agreement, only the above three parameters can be adapted by MAC CE, however, the existing structure allows to adapt all the other parameters configured within OD-SSB-Config, which conflicts with the intention from RAN1. A proposed TP1 is attached in the Annex.

Proposal 2: RAN2 to confirm configuration index is only associated with od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

2.2 RRC-2: RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst

According to the following RAN1 agreement, if an OD-SSB configuration includes od-ssb-nrofBurst, this OD-SSB configuration is not allowed to be activated via RRC. This is because due to the different RRC processing delay which is related to UE capability, there may be some un-sync between the NW and the UE on the starting/ending point of the OD-SSB transmission.

Agreement For a cell supporting on-demand SSB SCell operation, the following combinations are supported. - For OD-SSB transmission activation (OD-Tact) and OD-SSB transmission adaptation (OD-TA), - Case A1: RRC-based OD-Tact without N (i.e., od-ssb-nrofBurst) configured + MAC CE-based OD-TA; - Subject to UE capability - Case B1: MAC CE-based OD-Tact without N configured + MAC CE-based OD-TA; - Case B2: MAC CE-based OD-Tact with N configured + MAC CE-based OD-TA. - For OD-SSB transmission deactivation (OD-TD), - Case X1: RRC-based OD-Tact without N configured + MAC CE-based OD-TD; - Subject to UE capability - Case Y1: MAC CE-based OD-Tact or OD-TA without N configured + MAC CE-based OD-TD; - Case Y2: MAC CE-based OD-Tact or OD-TA with N configured + implicit OD-TD; - Case Y3: MAC CE-based OD-Tact or OD-TA with N configured + MAC CE-based OD-TD. - Conclusion: There is no RAN1 consensus to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst. - Note: “Implicit OD-TD” above implies that the on-demand SSB is deactivated based on the value for od-ssb-nrofBurst according to NW indication.

According to the existing running CR, parameter od-ssb-ActivationStatus is defined to indicate the activation/deactivation status of the OD-SSB pattern upon configuration and is configured within OD-SSB-Config. Since this parameter has ENUMERATED value “activated”, if absent, it indicates the deactivation of the OD-SSB pattern upon configuration. In addition, parameter od-ssb-nrofBurst is also configured within OD-SSB-Config. In this case, to reflect the above RAN1 agreement, we think one solution is to clarify in the FD of od-ssb-ActivationStatus that this parameter is absent if od-ssb-nrofBurst is configured. A TP is proposed as below. A TP2 is attached in the annex.

Proposal 3: (RRC-2) Clarify in the field description of od-ssb-ActivationStatus that this field is absent if od-ssb-nrofBurst is configured.

According to the existing running CR, od-ssb-ActivationStatus is mandatory present. Since this parameter has ENUMERATED value “activated”, it is not correct to configure it as mandatory, otherwise all the OD-SSB patterns are activated upon configuration, which is not aligned with the limitation in FD that only one OD-SSB pattern can be activated. in addition, as we proposed above, to reflect the RAN1 conclusion, this IE should be absent if od-ssb-nrofBurst is configured, which also requires this parameter to be optionally present.

Proposal 4: (RRC-2) od-ssb-ActivationStatus is an optional IE instead of mandatory.

2.3 RRC-3: Granularity of servingCellMO

In last meeting, we discussed the MO for OD-SSB when OD-SSB has different frequency than AO-SSB and achieved the following agreement.

- RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.

- RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported.

- When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

According to the agreement, there should be at most one OD-SSB with different frequency for a given SCell, thus at most one new servingCellMO. However, according to the existing running CR, this servingCellMO is configured within OD-SSB-Config which is not aligned with the agreed per serving cell granularity.

Although rapporteur clarified only one OD-SSB pattern will be activated and the servingCellMO associated with the OD-SSB pattern is active, at least from signaling point of view, for each OD-SSB configuration of which the OD-SSB has different frequency than the AO-SSB, there could be a new servingCellMO configured. We think we should move this parameter outside the OD-SSB-Config and put it under SCelConfig due to

- Considering servingCellMO is per frequency, actually all the OD-SSB configurations associated with the OD-SSB frequency should be configured with the same servingCellMO, which is not necessary and a waste of signaling.

- With the existing per OD-SSB config granularity, we need to clarify in the field description that all the OD-SSB configurations associated with the OD-SSB frequency should be configured with the same servingCellMO, otherwise it is not aligned with the agreement.

Proposal 5: (RRC-3) RAN2 to agree to configure the new servingCellMO (i.e., servingCellMO-OD) within ServingCellConfig instead of within OD-SSB-Config.

2.4 New issue: L3 measurement

In last meeting, we discussed L3 mesaurement for OD-SSB and achieved the following agreement.

- In L3 measurement in OD-SSB case 1, if MAC-CE/RRC based activation / deactivation:

• The UE starts L3 measurement towards the activated OD-SSB based on configured servingCellMO after reception of the activation.

• The UE stops L3 measurements after it determines the OD-SSB is deactivated implicitly or explicitly.

Spec impact can be further discussed in running CR preparation.

- When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO.

- The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.

- RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.

- RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported.

- When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

For case 1, we already agree to start L3 measurement when OD-SSB is activated while stop measurement when OD-SSB is deactivated. Since there is only OD-SSB, single servingCellMO should be configured and this servingCellMO should associates with the OD-SSB.

Proposal 6: For case 1, servingCellMO-OD is configured and L3 measurement is based on the configured servingCellMO-OD.

Then in general, how to perform L3 measuremnet can be summarized as below.

| Case | Frequency of OD-SSB and AO-SSB | servingCellMO | A/D of OD-SSB | L3 measurement |
| --- | --- | --- | --- | --- |
| Case 1 | / | Single | Activation | - Starts measurement based on new servingCellMO - L3 measurement is based on OD-SSB |
|  |  | Single | Deactivation | - Stops measurement |
| Case 2 | Same | Single | Activation | - L3 measurement is based on legacy servingCellMO - Whether both OD-SSB and AO-SSB are measured is up to RAN4 |
|  |  | Single | Deactivation | - L3 measurement is based on legacy servingCellMO - L3 measurement is based on AO-SSB |
| Case 2 | Different | Two | Activation | - L3 measurement is based on new servingCellMO - L3 measurement is based on OD-SSB |
|  |  | Two | Deactivation | - L3 measurement is based on legacy servingCellMO - L3 measurement is based on AO-SSB |

According to the existing specification, whenever the UE has a measurement configuration, the UE shall perform RSRP and RSRQ measurements for each serving cell for which servingCellMO is configured, this can still apply to case 2, but for case 1, some update is needed to reflect the agreement. A proposed TP is attached in the Annex.

Proposal 7: RAN2 to agree to use the proposed TP4 on L3 measurment of OD-SSB as a baseline.

2.5 New issue: Applied scenario of OD-SSB transmission adaptation

According to the following RAN1 agreement, for scenario#3B, after SCell is activated, the MAC CE is only used for updating the transmission parameters. However, there is no restriction that OD-SSB transmission adaptation cannot apply to scenario#2 and scenario#2A, i.e., after SCell is configured and before/during SCell activation. In our understanding, even before SCell activation, it is possible the NW may update the transmission parameters with the MAC CE.

Agreement For a cell supporting on-demand SSB SCell operation, the following combinations of scenarios and cases are supported for indicating OD-SSB using a MAC-CE. - Scenario #3B and Case #1 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication. - Scenario #3B and Case #2 - In the above combinations of scenarios and cases, the MAC-CE is used only for updating the transmission parameter of a transmitted OD-SSB for the cell since the OD-SSB has been transmitted according to NW indication.

Proposal 8: RAN2 to clarify that the OD-SSB activation/deactivation MAC CE can be used for updating the transmission parameter of a transmitted OD-SSB for scenario#2 and scenario#2A (i.e., scenario#2 refers to a scenario after SCell is configured and before SCell is activated while scenario#2A refers to a scenario when UE receives SCell activation command).

2.6 New issue: Neighbour cell measurement

For neighbour cells measurements, we think according to the WID, the impact on neighbour cell measurement with OD-SSB is out of scope. From RAN2 perspective, there is no need to discuss this aspect and we can assume the SSB status of neighbour cell is transparent to UE and how to handle the reported neighbour cell measurement is up to gNB implementation.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

Proposal 9: There is no impact on neighbour cell measurement with OD-SSB, i.e., the OD-SSB status of neighbour cell is transparent to UE.

3	Conclusion

In this contribution, we discussed the procedures and signalling to support on-demand SSB and have corresponding observations and proposals:

RRC-1: Number of OD-SSB configuration

Proposal 1: (RRC-1) RAN2 to send LS to RAN1 to ask about the possible candidate value combinations of od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

Proposal 2: RAN2 to confirm configuration index is only associated with od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

RRC-2: RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst

Proposal 3: (RRC-2) Clarify in the field description of od-ssb-ActivationStatus that this field is absent if od-ssb-nrofBurst is configured.

Proposal 4: (RRC-2) od-ssb-ActivationStatus is an optional IE instead of mandatory.

RRC-3: Granularity of servingCellMO

Proposal 5: (RRC-3) RAN2 to agree to configure the new servingCellMO (i.e., servingCellMO-OD) within ServingCellConfig instead of within OD-SSB-Config.

New issue: L3 measurement

Proposal 6: For case 1, servingCellMO-OD is configured and L3 measurement is based on the configured servingCellMO-OD.

Proposal 7: RAN2 to agree to use the proposed TP4 on L3 measurment of OD-SSB as a baseline.

New issue: Applied scenario of OD-SSB transmission adaptation

Proposal 8: RAN2 to clarify that the OD-SSB activation/deactivation MAC CE can be used for updating the transmission parameter of a transmitted OD-SSB for scenario#2 and scenario#2A (i.e., scenario#2 refers to a scenario after SCell is configured and before SCell is activated while scenario#2A refers to a scenario when UE receives SCell activation command).

New issue: Neighbour cell measurement

Proposal 9: There is no impact on neighbour cell measurement with OD-SSB, i.e., the OD-SSB status of neighbour cell is transparent to UE.

4	Reference

[1] RAN1#120bis Chair notes.

[2] RAN2#129bis meeting minutes

- Annex

TP1 (proposal 2):

–	OD-SSB-Config

The IE OD-SSB-Config is used to configure the OD-SSB activated by a MAC CE see TS 38.321 [3], clause XXX

OD-SSB-Config information element

-- ASN1START

-- TAG-OD-SSB-CONFIG-START

OD-SSB-Config-r19 ::= SEQUENCE {

od-ssb-ActivationStatus-r19            ENUMERATED {activated},

od-ssb-Periodicity-r19                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL, -- Cond ODssbAOssb

od-ssb-sfn-Offset-r19                   INTEGER (0..15)                                                         OPTIONAL, -- Cond ODssbAOssb

od-ssb-halfFrameIndex-r19              ENUMERATED {zero, one}                                                  OPTIONAL, -- Cond ODssbAOssb

od-ssb-absoluteFrequency-r19           ARFCN-ValueNR                                                           OPTIONAL, -- Cond ODssbAOssb

od-ssbSubcarrierSpacing-r19            SubcarrierSpacing                                                       OPTIONAL, -- Cond ODssbOnly

od-ss-PBCH-BlockPower-r19              INTEGER (-60..50)                                                       OPTIONAL, -- Cond ODssbOnly

servingCellMO-r19                      MeasObjectId                                                            OPTIONAL, -- Cond InterFreq

smtc1-r19                            SSB-MTC                                                                 OPTIONAL, -- Cond IntraFreq

od-ssb-AdaptedConfig-r19               OD-SSB-AdaptedConfig-r19                                                OPTIONAL,

...

}

OD-SSB-AdaptedConfig-r19 ::= SEQUENCE {

od-ssb-ConfigId-r19                    OD-SSB-ConfigId-r19,

od-ssb-Periodicity-r19                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL,

od-ssb-PositionsInBurst-r19            CHOICE {

shortBitmap                            BIT STRING (SIZE (4)),

mediumBitmap                           BIT STRING (SIZE (8)),

longBitmap                             BIT STRING (SIZE (64))

}                                                                                                              OPTIONAL,

od-ssb-nrofBurst-r19                   INTEGER (1..8)                                                          OPTIONAL, -- Need R

}

OD-SSB-ConfigId-r19   ::= INTEGER (0.. maxNrofOD-SSB-1-r19)

-- TAG-OD-SSB-CONFIG-STOP

-- ASN1STOP

TP2 (proposal 3):

#### –	OD-SSB-Config

The IE OD-SSB-Config is used to configure the OD-SSB activated by a MAC CE see TS 38.321 [3], clause XXX

OD-SSB-Config information element

-- ASN1START

-- TAG-OD-SSB-CONFIG-START

OD-SSB-Config-r19 ::= SEQUENCE {

od-ssb-ConfigId-r19                    OD-SSB-ConfigId-r19,

od-ssb-ActivationStatus-r19            ENUMERATED {activated},

od-ssb-Periodicity-r19                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL, -- Cond ODssbAOssb

od-ssb-sfn-Offset-r19                   INTEGER (0..15)                                                         OPTIONAL, -- Cond ODssbAOssb

od-ssb-halfFrameIndex-r19              ENUMERATED {zero, one}                                                  OPTIONAL, -- Cond ODssbAOssb

od-ssb-absoluteFrequency-r19           ARFCN-ValueNR                                                           OPTIONAL, -- Cond ODssbAOssb

od-ssb-PositionsInBurst-r19            CHOICE {

shortBitmap                            BIT STRING (SIZE (4)),

mediumBitmap                           BIT STRING (SIZE (8)),

longBitmap                             BIT STRING (SIZE (64))

}                                                                                                              OPTIONAL, -- Cond ODssbAOssb

od-ssbSubcarrierSpacing-r19            SubcarrierSpacing                                                       OPTIONAL, -- Cond ODssbOnly

od-ss-PBCH-BlockPower-r19              INTEGER (-60..50)                                                       OPTIONAL, -- Cond ODssbOnly

od-ssb-nrofBurst-r19                   INTEGER (1..8)                                                          OPTIONAL, -- Need R

servingCellMO-r19                      MeasObjectId                                                            OPTIONAL, -- Cond InterFreq

smtc1-r19                            SSB-MTC                                                                 OPTIONAL, -- Cond IntraFreq

...

}

OD-SSB-ConfigId-r19   ::= INTEGER (0.. maxNrofOD-SSB-1-r19)

-- TAG-OD-SSB-CONFIG-STOP

-- ASN1STOP

OD-SSB-Config field descriptions

smtc1 Primary measurement timing configuration (see clause 5.5.2.10) to be used instead of smtc1 configured in servingCellMO in IE servingCellConfig when this OD-SSB is activated and the serving cell is activated

od-ssb-absoluteFrequency Indicates the frequency of the OD-SSB when the frequency is different from absoluteFrequencySSB configured in IE FrequencyInfoDL for this serving cell. Additional restrictions as described in subclause 4.4. of TS 38.213.

od-ssb-ActivationStatus Indicates the activation status of this OD-SSB pattern upon configuration. Only one OD-SSB pattern can be activated. This field is absent if od-ssb-nrofBurst is configured.

od-ssb-halfFrameIndex Indicates whether OD-SSB is in the first half or the second half of the frame. If the field is absent, the UE applies the value 0.

od-ssb-sfn-Offset Indicates SFN offset from the SFN which satisfies (SFN index *10) modulo (OD-SSB periodicity) = 0. The network configures this field according to the field od-ssb-Periodicity such that the indicated system frame does not exceed the OD-SSB periodicity. If the field is absent, the UE applies the value 0.

od-ssb-nrofBurst Indicates the number of OD-SSB bursts to be transmitted after OD-SSB is activated.

od-ssb-Periodicity The SSB periodicity in ms. If the field is absent, the UE applies the value ms5. (see TS 38.213 [13], clause 4.1).

od-ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks for OD-SSB in a half frame with SS/PBCH blocks as defined in TS 38.213 [13], clause 4.1. If absent, od-ssb-PositionsInBurst is the same as ssb-PositionsInBurst provided in ServingCellConfigCommon.

od-ss-PBCH-BlockPower Indicates average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for OD-SSB transmission, see TS 38.213 [13], clause 7.

od-ssbSubcarrierSpacing Indicates subcarrier spacing of OD-SSB, for Case #1, i.e., no always-on SSB on this serving cell. Only the following values are applicable depending on the used frequency: FR1: 15 or 30 kHz FR2-1/FR2-NTN: 120 or 240 kHz FR2-2: 120, 480, or 960 kHz

servingCellMO measObjectId of the MeasObjectNR in MeasConfig which is associated to the serving cell when this OD-SSB is activated instead of servingCellMO in IE ServingCellConfig.

| Conditional Presence | Explanation |
| --- | --- |
| ODssbOnly | The field is optionally present, Need R, when absoluteFrequencySSB of the serving cell is absent. It is absent otherwise. |
| ODssbAOssb | The field is mandatory present, Need R, when absoluteFrequencySSB of the serving cell is absent. It is optionally present otherwise. |
| InterFreq | This field is mandatory present if od-ssb-absoluteFrequency indicates different frequency than absoluteFrequencySSB of the serving cell. It is absent otherwise. FFS SSB-less SCell |
| IntraFreq | This field is mandatory present if od-ssb-absoluteFrequency indicates same frequency as absoluteFrequencySSB of the serving cell. It is absent otherwise. FFS SSB-less SCell |

TP3 (proposal 4):

#### –	OD-SSB-Config

The IE OD-SSB-Config is used to configure the OD-SSB activated by a MAC CE see TS 38.321 [3], clause XXX

OD-SSB-Config information element

-- ASN1START

-- TAG-OD-SSB-CONFIG-START

OD-SSB-Config-r19 ::= SEQUENCE {

od-ssb-ConfigId-r19                    OD-SSB-ConfigId-r19,

od-ssb-ActivationStatus-r19            ENUMERATED {activated}                                                  OPTIONAL,

od-ssb-Periodicity-r19                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL, -- Cond ODssbAOssb

od-ssb-sfn-Offset-r19                   INTEGER (0..15)                                                         OPTIONAL, -- Cond ODssbAOssb

od-ssb-halfFrameIndex-r19              ENUMERATED {zero, one}                                                  OPTIONAL, -- Cond ODssbAOssb

od-ssb-absoluteFrequency-r19           ARFCN-ValueNR                                                           OPTIONAL, -- Cond ODssbAOssb

od-ssb-PositionsInBurst-r19            CHOICE {

shortBitmap                            BIT STRING (SIZE (4)),

mediumBitmap                           BIT STRING (SIZE (8)),

longBitmap                             BIT STRING (SIZE (64))

}                                                                                                              OPTIONAL, -- Cond ODssbAOssb

od-ssbSubcarrierSpacing-r19            SubcarrierSpacing                                                       OPTIONAL, -- Cond ODssbOnly

od-ss-PBCH-BlockPower-r19              INTEGER (-60..50)                                                       OPTIONAL, -- Cond ODssbOnly

od-ssb-nrofBurst-r19                   INTEGER (1..8)                                                          OPTIONAL, -- Need R

servingCellMO-r19                      MeasObjectId                                                            OPTIONAL, -- Cond InterFreq

smtc1-r19                            SSB-MTC                                                                 OPTIONAL, -- Cond IntraFreq

...

}

OD-SSB-ConfigId-r19   ::= INTEGER (0.. maxNrofOD-SSB-1-r19)

-- TAG-OD-SSB-CONFIG-STOP

-- ASN1STOP

TP3 (proposal 5):

#### –	OD-SSB-Config

The IE OD-SSB-Config is used to configure the OD-SSB activated by a MAC CE see TS 38.321 [3], clause XXX

OD-SSB-Config information element

-- ASN1START

-- TAG-OD-SSB-CONFIG-START

OD-SSB-Config-r19 ::= SEQUENCE {

od-ssb-ConfigId-r19                    OD-SSB-ConfigId-r19,

od-ssb-ActivationStatus-r19            ENUMERATED {activated},

od-ssb-Periodicity-r19                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL, -- Cond ODssbAOssb

od-ssb-sfn-Offset-r19                   INTEGER (0..15)                                                         OPTIONAL, -- Cond ODssbAOssb

od-ssb-halfFrameIndex-r19              ENUMERATED {zero, one}                                                  OPTIONAL, -- Cond ODssbAOssb

od-ssb-absoluteFrequency-r19           ARFCN-ValueNR                                                           OPTIONAL, -- Cond ODssbAOssb

od-ssb-PositionsInBurst-r19            CHOICE {

shortBitmap                            BIT STRING (SIZE (4)),

mediumBitmap                           BIT STRING (SIZE (8)),

longBitmap                             BIT STRING (SIZE (64))

}                                                                                                              OPTIONAL, -- Cond ODssbAOssb

od-ssbSubcarrierSpacing-r19            SubcarrierSpacing                                                       OPTIONAL, -- Cond ODssbOnly

od-ss-PBCH-BlockPower-r19              INTEGER (-60..50)                                                       OPTIONAL, -- Cond ODssbOnly

od-ssb-nrofBurst-r19                   INTEGER (1..8)                                                          OPTIONAL, -- Need R

smtc1-r19                            SSB-MTC                                                                 OPTIONAL, -- Cond IntraFreq

...

}

OD-SSB-ConfigId-r19   ::= INTEGER (0.. maxNrofOD-SSB-1-r19)

-- TAG-OD-SSB-CONFIG-STOP

-- ASN1STOP

–	CellGroupConfig

The CellGroupConfig IE is used to configure a master cell group (MCG) or secondary cell group (SCG). A cell group comprises of one MAC entity, a set of logical channels with associated RLC entities and of a primary cell (SpCell) and one or more secondary cells (SCells). For an NCR-MT, the CellGroupConfig IE is also used to provide the configuration of side control information for the NCR-Fwd access link.

CellGroupConfig information element

SCellConfig ::=                     SEQUENCE {

sCellIndex                          SCellIndex,

sCellConfigCommon                   ServingCellConfigCommon                                     OPTIONAL,   -- Cond SCellAdd

sCellConfigDedicated                ServingCellConfig                                           OPTIONAL,   -- Cond SCellAddMod

...,

[[

smtc                                SSB-MTC                                                     OPTIONAL    -- Need S

]],

[[

sCellState-r16                  ENUMERATED {activated}                                          OPTIONAL,   -- Cond SCellAddSync

secondaryDRX-GroupConfig-r16    ENUMERATED {true}                                               OPTIONAL    -- Need S

]],

[[

preConfGapStatus-r17             BIT STRING (SIZE (maxNrofGapId-r17))                           OPTIONAL,   -- Cond PreConfigMG

goodServingCellEvaluationBFD-r17 GoodServingCellEvaluation-r17                                  OPTIONAL,   -- Need R

sCellSIB20-r17                   SetupRelease { SCellSIB20-r17 }                                OPTIONAL    -- Need M

]],

[[

plmn-IdentityInfoList-r17       SetupRelease {PLMN-IdentityInfoList}                            OPTIONAL,   -- Cond SCellSIB20-Opt

npn-IdentityInfoList-r17        SetupRelease {NPN-IdentityInfoList-r16}                         OPTIONAL    -- Cond SCellSIB20-Opt

]],

[[

od-SSB-ConfigToAddModList-r19                     SEQUENCE (SIZE (1.. maxNrofOD-SSB-r19)) OF OD-SSB-Config-r19    OPTIONAL,   -- Need N

od-SSB-ConfigToReleaseList-r19                    SEQUENCE (SIZE (1.. maxNrofOD-SSB-r19)) OF OD-SSB-ConfigId-r19    OPTIONAL,   -- Need N

servingCellMO-r19                      				MeasObjectId                                                      OPTIONAL -- Cond InterFreq

]]

}

TP4 (proposal 7):

#### 5.5.3.1	General

An RRC_CONNECTED UE shall derive cell measurement results by measuring one or multiple beams associated per cell as configured by the network, as described in 5.5.3.3. For all cell measurement results, except for RSSI, and CLI measurement results in RRC_CONNECTED, the UE applies the layer 3 filtering as specified in 5.5.3.2, before using the measured results for evaluation of reporting criteria, measurement reporting or the criteria to trigger conditional reconfiguration execution. For cell measurements, the network can configure RSRP, RSRQ, SINR, RSCP or EcN0 as trigger quantity. For CLI measurements, the network can configure SRS-RSRP or CLI-RSSI as trigger quantity. For cell and beam measurements, reporting quantities can be any combination of quantities (i.e. only RSRP; only RSRQ; only SINR; RSRP and RSRQ; RSRP and SINR; RSRQ and SINR; RSRP, RSRQ and SINR; only RSCP; only EcN0; RSCP and EcN0), irrespective of the trigger quantity, and for CLI measurements, reporting quantities can be either SRS-RSRP or CLI-RSSI. For conditional reconfiguration execution, the network can configure up to 2 quantities, both using same RS type. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the CBR measurements. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the Rx-Tx time difference measurements. The UE does not apply the layer 3 filtering as specified in 5.5.3.2 to derive the altitude measurements.

The network may also configure the UE to report measurement information per beam (which can either be measurement results per beam with respective beam identifier(s) or only beam identifier(s)), derived as described in 5.5.3.3a. If beam measurement information is configured to be included in measurement reports, the UE applies the layer 3 beam filtering as specified in 5.5.3.2. On the other hand, the exact L1 filtering of beam measurements used to derive cell measurement results is implementation dependent.

The UE shall:

- whenever the UE has a measConfig, for each serving cell for which servingCellMO is configured and OD-SSB-Config is not configured; or

- whenever the UE has a measConfig, for each serving cell for which servingCellMO, OD-SSB-Config and absoluteFrequencySSB are configured; or

- whenever the UE has a measConfig, for each serving cell for which OD-SSB-Config is configured, absoluteFrequencySSB is not configured and OD-SSB transmission is activated as specified in [xxx];

2>	perform RSRP and RSRQ measurements as follows:

3>	if servingCellMO is configured and OD-SSB-Config is not configured;

4>	apply the servingCellMO;

3>	if servingCellMO, OD-SSB-Config and absoluteFrequencySSB are configured;

4>	if od-ssb-absoluteFrequency indicates same frequency as absoluteFrequencySSB of the serving cell;

5>	apply the servingCellMO;

4>	else if od-ssb-absoluteFrequency indicates different frequency as absoluteFrequencySSB of the serving cell;

5>	if OD-SSB transmission is activated as specified in [xxx];

6>	apply the servingCellMO-OD;

5>	else;

6>	apply the servingCellMO;

3>	if OD-SSB-Config is configured and absoluteFrequencySSB is not configured;

4>	apply the servingCellMO-OD;

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to ssb and ssb-ConfigMobility is configured in the measObject indicated by the servingCellMO or servingCellMO-OD:

4>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to ssb:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on SS/PBCH block, as described in 5.5.3.3;

3>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains an rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the measObject indicated by the servingCellMO or servingCellMO-OD:

4>	if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport and contains an rsType set to csi-rs:

5>	derive layer 3 filtered RSRP and RSRQ per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

4>	derive serving cell measurement results based on CSI-RS, as described in 5.5.3.3;

1>	for each serving cell for which servingCellMO or servingCellMO-OD is configured, if the reportConfig associated with at least one measId included in the measIdList within VarMeasConfig contains SINR as trigger quantity and/or reporting quantity:

2>	if the reportConfig contains rsType set to ssb and ssb-ConfigMobility is configured in the servingCellMO or servingCellMO-OD:

3>	if the reportConfigcontains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

4>	derive layer 3 filtered SINR per beam for the serving cell based on SS/PBCH block, as described in 5.5.3.3a;

3>	derive serving cell SINR based on SS/PBCH block, as described in 5.5.3.3;

2>	if the reportConfig contains rsType set to csi-rs and CSI-RS-ResourceConfigMobility is configured in the servingCellMO or servingCellMO-OD:

3>	if the reportConfigcontains a reportQuantityRS-Indexes and maxNrofRS-IndexesToReport:

4>	derive layer 3 filtered SINR per beam for the serving cell based on CSI-RS, as described in 5.5.3.3a;

3>	derive serving cell SINR based on CSI-RS, as described in 5.5.3.3;