3GPP TSG-RAN WG2 Meeting#131                               	R2-2505370

Bangalore, India, Aug 25th – 29th, 2025

Agenda item:	8.9.2

Source: 	Google

Title: 	Leftover issues on the satellite S&F operation

Document for:	Discussion and Decision

# Introduction

In Rel-19, the benefits of enabling the Store & Forward operation for IoT-NTN are identified and the following scopes are included in the revised WID of Rel-19 IoT-NTN [1].

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - Specify necessary enhancements for full eNB as regenerative payload e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required.

RAN2 has agreed to introduce and signal an S&F mode transition time (i.e., t-ModeSwitching) in SIB31. This transition time indicates when the serving cell will switch between S&F and normal modes, depending on the presence or absence of sf-OperationMode in SIB1. The UE forwards this time to the upper layers upon acquisition from SIB31. This paper discusses remaining issues concerning this S&F mode transition time, including its usage in the AS layer.

# Discussion

## (RRC-4) Whether to forward mode information to the upper layer

In the current RRC running CR [2], the UE forwards t-ModeSwitching to the upper layers upon acquiring it from SIB31. During the RRC running CR discussion, some companies suggested that this time information should be forwarded together with the S&F mode indication (i.e., sf-OperationMode in SIB1). Their reasoning was that without this, the upper layers would not know whether the serving cell is switching to normal or S&F mode. Although these arguments are valid, the upper layer (at least the UE NAS layer) should already be aware of the serving cell's current operation mode (i.e., S&F or normal mode), as per the following statement captured from 3GPP TS 23.401 [3].

4.13.9 Support of Store and Forward Satellite operation #### 4.13.9.1 General [irrelevant parts skipped] When an on-board MME supports Store and Forward Satellite operation and is operating in S&F Mode, the on-board eNodeB broadcasts an indication of operating in S&F Mode as described in TS 36.300 [5], which the UE uses to determine the current operation mode of the satellite. If a satellite is operating in S&F Mode and if a UE is enabled for Store and Forward Satellite operation, then the UE indicates the S&F capability during Attach and Tracking Area Update.

(RRC-4) The UE NAS needs to know from the RRC layer the current operation mode of the serving satellite, in order to decide whether to indicate the S&F capability in an Attach or in a TAU procedure.

However, the current RRC running CR does not specify the UE behavior regarding interaction with the upper layers upon receiving the S&F mode indication from SIB1. Therefore, it may be necessary to capture this missing UE behavior in the RRC specification, specifically in sub-clause 5.2.2.7 “Actions upon reception of the SystemInformationBlockType1 message.” Without this, the UE NAS will be unable to determine the satellite's current operation mode, and consequently, unable to execute the action highlighted above in 3GPP TS 23.401 [3].

(RRC-4) UE forwards the S&F mode indication (i.e., sf-OperationMode) to upper layers upon acquiring it from SIB1(-NB). RAN2 to capture this UE behavior in 3GPP TS 36.331, sub-clause 5.2.2.7.

## The impact of t-ModeSwitching on the UE AS

Since the satellite operation mode of a serving cell impacts both the UE AS (regarding cell access allowance) and UE NAS (regarding S&F capability indication when connecting to the cell), it is crucial for UEs to know the exact timing of operational mode changes. Given that the S&F mode indication (i.e., sf-OperationMode) and legacy barring indications can only change at SI modification period boundaries, the S&F mode transition time (i.e., t-ModeSwitching) signaled in SIB31 provides UEs with a more precise indication of when the serving cell changes its operational mode.

As the S&F mode indication (i.e., sf-OperationMode) and the legacy barring indications can only change at the boundaries of SI modification periods, the S&F mode transition time (i.e., t-ModeSwitching) signaled in SIB31 allows UEs to know more precisely when the serving cell changes its operational mode.

RAN2#129bis agreed that the transition time from the normal to the S&F mode is useful to the UE NAS. However, if the UE AS can properly utilize this transition time, it would also benefit Rel-19 UEs not supporting the S&F operation. These UEs could determine whether to establish, reestablish, or resume an RRC connection based on whether there is sufficient time to fulfill the access purpose. Without this information, non-S&F-capable UEs might initiate an RRC connection procedure just before the satellite enters the S&F mode, and then will be released by the network soon after the satellite enters the S&F mode, which consumes considerable radio resources and UE energy. Utilizing the transition time information to assist RRC connection initiation can be left to UE implementation, with no specification impact.

Non-S&F-capable UE can utilize the transition time from the normal mode to the S&F mode to determine whether to establish/reestablish/resume an RRC connection (no spec impact).

For UEs that don't support S&F, the goal is to keep them on normal-mode cells for as long as possible, even if they could camp on an S&F-mode cell. Therefore, when a non-S&F-capable UE is camping on a normal-mode cell and receives an indication that S&F mode is about to start, it should trigger intra-/inter-frequency measurements. Similar to the t-service mechanism in Rel-18, these measurements should be triggered regardless of other conditions, such as serving cell quality or distance to the reference location.

Non-S&F-capable UE can utilize the transition time from the normal mode to the S&F mode to determine when to trigger the measurement for cell reselection.

If P3 above can be agreed, the TP appended in the Annex at the end of this paper can be considered as the baseline for updating the current 36.304 running CR [4].

If P3 is agreed, RAN2 to adopt the TP in Annex into the current 36.304 running CR.

## S&F mode indications for neighbour cells

To increase the likelihood for the UE to detect, measure, and reselect a normal-mode cell, it is beneficial if the network can provide/signal the S&F mode indication (i.e., sf-OperationMode) even for the neighbor cell. This aspect has been discussed in the RAN2#130 meeting and a working assumption is built based on strong interest from the majority of companies.

Working Assumption: - In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

Although the working assumption from RAN2#130 was confirmed during the 36.304 running CR discussion, the specific impacts on the specification remain unconfirmed because of the different views among the companies. As the next meeting will be the final one for developing this concept, we need to first decide on the signaling details, such as which SIB and IE should include the S&F mode indication for neighbor cells, before discussing UE behavior further. We believe the neighbor satellite list in SIB33 is a suitable place for these indications, provided they are signaled for each neighbor satellite. We also propose signaling the S&F mode transition time along with the S&F mode indication for each neighbor satellite. This would prevent the UE from constantly checking for these indications.

The S&F mode indication (i.e., sf-OperationMode) and the S&F mode transition time (i.e., t-ModeSwitching) of the neighbor satellite are signaled in SIB33 per neighbor satellite.

We believe the most effective and straightforward solution for UE behavior is to allow non-S&F-capable UEs to deprioritize neighbor cells operating in the S&F mode. This can be achieved by having UEs decrease the cell rankings of these neighbor cells by a common delta value during the cell reselection evaluation process.

Non-S&F-capable UE deprioritizes the neighbor cells operating in the S&F mode by decreasing the cell rankings of these neighbor cells by a common delta value.

# Conclusion

In this paper, we discuss several leftover issues on the S&F mode indication and the S&F mode transition time. Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

(RRC-4) Whether to forward mode information to the upper layer

Observation 1	(RRC-4) The UE NAS needs to know from the RRC layer the current operation mode of the serving satellite, in order to decide whether to indicate the S&F capability in an Attach or in a TAU procedure.

Proposal 1	(RRC-4) UE forwards the S&F mode indication (i.e., sf-OperationMode) to upper layers upon acquiring it from SIB1(-NB). RAN2 to capture this UE behavior in 3GPP TS 36.331, sub-clause 5.2.2.7.

The impact of t-ModeSwitching on the UE AS

Observation 2	As the S&F mode indication (i.e., sf-OperationMode) and the legacy barring indications can only change at the boundaries of SI modification periods, the S&F mode transition time (i.e., t-ModeSwitching) signaled in SIB31 allows UEs to know more precisely when the serving cell changes its operational mode.

Proposal 2	Non-S&F-capable UE can utilize the transition time from the normal mode to the S&F mode to determine whether to establish/reestablish/resume an RRC connection (no spec impact).

Proposal 3	Non-S&F-capable UE can utilize the transition time from the normal mode to the S&F mode to determine when to trigger the measurement for cell reselection.

Proposal 4	If P3 is agreed, RAN2 to adopt the TP in Annex into the current 36.304 running CR.

S&F mode indications for neighbour cells

Proposal 5	The S&F mode indication (i.e., sf-OperationMode) and the S&F mode transition time (i.e., t-ModeSwitching) of the neighbor satellite are signaled in SIB33 per neighbor satellite.

Proposal 6	Non-S&F-capable UE deprioritizes the neighbor cells operating in the S&F mode by decreasing the cell rankings of these neighbor cells by a common delta value.

# Reference

RP-243278, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3.

[Post130][306][R19 IoT NTN], RRC Runing CR for IoT NTN, Huawei.

3GPP TS 23.401 v19.2.0, General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network.

[Post130][308][R19 IoT NTN], Running CR for TS36.304 for IoT-NTN, Nokia.

R2-250XXXX, Remaining Open issues for Idle mode IoT-NTN Operation, Nokia.

# Annex: TP for triggering the measurement for cell reselection

--------------------------------------------- TP on the running CR for TS 36.304[4] ---------------------------------------------------

#### 5.2.4.2	Measurement rules for cell re-selection

For NB-IoT measurement rules for cell re-selection is defined in clause 5.2.4.2.a.

When evaluating Srxlev and Squal of non-serving cells for reselection purposes, the UE shall use parameters provided by the serving cell.

Following rules are used by the UE to limit needed measurements:

-	If the measurements are performed using RSS as specified in [10] and the serving cell fulfils Srxlev > SIntraSearchP:

-	If distanceThresh and referenceLocation are broadcast in SystemInformationBlockType31, and if the UE has obtained its location information:

-	If referenceLocation is set to fixedReferenceLocation and if the UE supports location-based measurement initiation (quasi-)Earth for fixed cell, referenceLocation is used as serving cell reference location.

-	If the distance between the UE and the serving cell reference location is shorter than distanceThresh, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE shall perform intra-frequency measurements.

-	If referenceLocation is set to movingReferenceLocation and if the UE supports location-based measurement initiation for Earth moving cell, the UE derives the serving cell reference location based on ephemeris, epochTime and referenceLocation.

-	If the distance between the UE and the serving cell reference location is shorter than distanceThresh, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE shall perform intra-frequency measurements.

-	Else, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE may choose not to perform intra-frequency measurements.

-	Else if the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ:

-	If distanceThresh and referenceLocation are broadcast in SystemInformationBlockType31, and if the UE has obtained its location information:

-	If referenceLocation is set to fixedReferenceLocation and if the UE supports location-based measurement initiation for (quasi-)Earth fixed cell, the referenceLocation is used as serving cell reference location.

-	If the distance between the UE and the serving cell reference location, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE shall perform intra-frequency measurements.

-	If referenceLocation is set to movingReferenceLocation and if the UE supports location-based measurement initiation for Earth moving cell, the UE derives the serving cell reference location based on ephemeris, epochTime and referenceLocation.

-	If the distance between the UE and the serving cell reference location is shorter than distanceThresh, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE shall perform intra-frequency measurements.

-	Else, the UE may choose not to perform intra-frequency measurements.

-	Else, the UE may choose not to perform intra-frequency measurements.

-	Otherwise, the UE shall perform intra-frequency measurements.

-	The UE shall apply the following rules for E-UTRAN inter-frequencies and inter-RAT frequencies which are indicated in system information and for which the UE has priority provided as defined in 5.2.4.1:

-	For an E-UTRAN inter-frequency or inter-RAT frequency with a reselection priority higher than the reselection priority of the current E-UTRA frequency the UE shall perform measurements of higher priority E-UTRAN inter-frequency or inter-RAT frequencies according to TS 36.133 [10].

-	For an E-UTRAN inter-frequency with an equal or lower reselection priority than the reselection priority of the current E-UTRA frequency and for inter-RAT frequency with lower reselection priority than the reselection priority of the current E-UTRAN frequency:

-	If the measurements are performed using RSS as specified in [10] and the serving cell fulfils Srxlev > SnonIntraSearchP:

-	If distanceThresh and referenceLocation are broadcast in SystemInformationBlockType31, and if the UE has obtained its location:

-	If referenceLocation is set to fixedReferenceLocation and if the UE supports location-based measurement initiation for (quasi-)Earth fixed cell, the referenceLocation is used as serving cell reference location. The referenceLocation is used as serving cell reference location.

-	If the distance between the UE and serving cell reference location is shorter than distanceThresh the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 36.133 [10].

-	If referenceLocation is set to movingReferenceLocation and if the UE supports location-based measurement initiation for Earth moving cell, the UE derives the serving cell reference location based on ephemeris, epochTime and referenceLocation.

-	If the distance between the UE and serving cell reference location is shorter than distanceThresh the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 36.133 [10].

-	Else, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else if the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ:

-	If distanceThresh and referenceLocation are broadcast in SystemInformationBlockType31, and if the UE supports location-based measurement initiation and has obtained its location:

-	If referenceLocation is set to fixedReferenceLocation and UE supports location-based measurement initiation for (quasi-)Earth fixed cell, the referenceLocation is used as serving cell reference location.

-	If the distance between the UE and serving cell reference location is shorter than distanceThresh, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 36.133 [10].

-	If referenceLocation is set to movingReferenceLocation and UE supports location-based measurement initiation for Earth moving cell, the UE derives the serving cell reference location based on ephemeris, epochTime and referenceLocation.

-	If the distance between the UE and serving cell reference location is shorter than distanceThresh, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 36.133 [10].

-	Else, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Else, the UE may choose not to perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority unless the UE is triggered to measure an E-UTRAN inter-frequency which is configured with redistributionInterFreqInfo.

-	Otherwise, the UE shall perform measurements of E-UTRAN inter-frequencies or inter-RAT frequency cells of equal or lower priority according to TS 36.133 [10].

-	If the UE supports relaxed monitoring and s-SearchDeltaP is present in SystemInformationBlockType3, the UE may further limit the needed measurements, as specified in clause 5.2.4.12.

If t-Service is present in SystemInformationBlockType3 of the serving cell, UE shall perform intra-frequency, inter-frequency or inter-RAT measurements, before the time t-Service regardless of the distance between the UE and serving cell reference location, and regardless whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ. The exact time to start measurements before t-Service is up to UE implementation and t-ServiceStartNeigh if present in SystemInformationBlockType33 may be used to decide on when to start measurements. UE shall perform measurements of higher priority inter-frequencies or inter-RAT frequencies regardless of the remaining service time of the serving cell.

If t-ModeSwitching is present in SystemInformationBlockType31 of the serving cell and sf-OperationMode is absent in SystemInformationBlockType1 of the serving cell, UE not supporting the satellite S&F operation shall perform intra-frequency, inter-frequency or inter-RAT measurements, before the time t-ModeSwitching regardless of the distance between the UE and serving cell reference location, and regardless whether the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, or Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ. The exact time to start measurements before t-ModeSwitching is up to UE implementation and t-ServiceStartNeigh if present in SystemInformationBlockType33 may be used to decide on when to start measurements.